3GPP TSG-RAN5 Meeting #94-e	R5-221551

# Online, , 21st Feb 2022 - 4th Mar 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-6 | CR | 0042 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New MCVideo TC Selected Group Change of Targeted User CO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2022-03-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The client originated case of the feature "Remote change of selected group" was not being tested |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test On-network / Remote Change of Selected Group / Selected Group Change of Targeted User / Client Originated (CO) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The client originated case of the feature "Remote change of selected group" will not be tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.4, 6.1.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the final version of R5-220512r1. |  |  |  |  |  |  |  |  |

<START OF CHANGES>

### 6.1.4	Remote change of selected group

#### 6.1.4.1	On-network / Remote Change of Selected Group / Selected Group Change of Targeted User / Client Originated (CO)

6.1.4.1.1	Test Purpose (TP)

(1)

with { UE (MCVideo Client) registered and authorised for MCVIDEO Service }

ensure that {

when { the MCVideo User requests to send a group selection change request to change the selected group of a targeted MCVIDEO user to a specific MCVideo group }

then { UE (MCVideo Client) sends a SIP MESSAGE message and responds to a SIP MESSAGE message with a SIP 200 (OK) message }

}

6.1.4.1.2	Conformance Requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.281 clause 9.2.4.2.1. Unless otherwise stated these are Rel-15 requirements.

[TS 24.281, clause 9.2.4.2.1]

Upon receiving a request from the MCVideo user to send a group selection change request to change the selected group of a targeted MCVideo user to a specific MCVideo group, the MCVideo client:

1)	if:

a)	the <RemoteGroupSelectionURIList> element does not exist in the MCVideo user profile document with one or more <entry> elements (see the MCVideo user profile document in 3GPP TS 24.484 [25]); or

b)	the <RemoteGroupSelectionURIList> element exists in the MCVideo user profile document and the MCVideo ID of the targeted MCVideo user does not match with one of the <entry> elements of the <RemoteGroupSelectionURIList> element of the MCVideo user profile document (see the MCVideo user profile document in 3GPP TS 24.484 [25]);

then:

a)	should indicate to the requesting MCVideo user that they are not authorised to change the selected MCVideo group of the targeted MCVideo user; and

b)	shall skip the rest of the steps of the present clause;

2)	shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [11] and IETF RFC 3428 [17] with the following clarifications:

a)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcvideo" (coded as specified in 3GPP TS 24.229 [11]), in a P-Preferred-Service header field according to IETF RFC 6050 [14] in the SIP MESSAGE request;

b)	shall include an Accept-Contact header field with the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcvideo" along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [20];

c)	may include a P-Preferred-Identity header field in the SIP MESSAGE request containing a public user identity as specified in 3GPP TS 24.229 [11]; and

d)	shall include an application/vnd.3gpp.mcvideo-info+xml MIME body as specified in clause F.1 with the <mcvideoinfo> element containing the <mcvideo-Params> element with the <anyExt> element containing:

i)	the <mcvideo-request-uri>  set to the MCVideo group identity to be selected by the targeted MCVideo user; and

ii)	the <request-type> element set to a value of "group-selection-change-request"; and

e)	shall insert in the SIP MESSAGE request a MIME resource-lists body with the MCVideo ID of the targeted MCVideo user, according to rules and procedures of IETF RFC 5366 [37];

3)	shall set the Request-URI to the public service identity identifying the participating MCVideo function serving the MCVideo user; and

4)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [11].

Upon receipt of a SIP 4xx, 5xx or 6xx response to the SIP MESSAGE request, should indicate to the MCVideo user the failure of the sent group selection change request and not continue with the rest of the steps.

Upon receiving a "SIP MESSAGE request for group selection change response for terminating client", the MCVideo client:

1)	shall determine the success or failure of the sent group selection change request from the value of the <selected-group-change-outcome> element contained in the <anyExt> element of the <mcvideo-Params> element  of the <mcvideoinfo> element of the application/vnd.3gpp.mcvideo-info+xml MIME body included in the received SIP MESSAGE request; and

2)	should indicate to the MCVideo user the success or failure of the sent group selection change request.

6.1.4.1.3	Test description

6.1.4.1.3.1	Pre-test conditions

System Simulator:

-	SS (MCVideo server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCVideo operation in the MCVideo configuration document).

IUT:

-	UE (MCVideo client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The UE has performed the Generic Test Procedure for MCVideo UE registration as specified in TS 36.579-1 [2], subclause 5.4.2A.

-	The MCVideo User performs the Generic Test Procedure for MCVideo Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2A.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCVideo Client Application has been activated and User has registered-in as the MCVideo User with the Server as active user at the Client

6.1.4.1.3.2	Test procedure sequence

Table 6.1.4.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the MCVideo User request to send a group selection change request to change the selected group of a targeted MCVideo user to a specific MCVideo group. (NOTE 1) | - | - | - | - |
| 2 | Check: Does the UE (MCVIDEO Client) correctly perform Generic Test Procedure for MCX SIP MESSAGE Request - Accept CO as described in TS 36.579-1 [2] Table 5.3.30.3-1 to change the selected group of a targeted MCVideo user to a specific MCVideo group? | - | - | 1 | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.1.4.1.3.3	Specific message contents

Table 6.1.4.1.3.3-1: SIP MESSAGE (Step 2, Table 6.1.4.1.3.2-1; Step 2, TS 36.579-1 [2] Table 5.3.30.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCVideo Info |  |  |
| MIME-part-body | MCVideo-Info as described in Table 6.1.4.1.3.3-2 |  |  |  |
| MIME body part |  | Resource lists |  |  |
| MIME-part-body | Resource-lists as described in Table 6.1.4.1.3.3-3 |  |  |  |

Table 6.1.4.1.3.3-2: MCVideo-Info in SIP MESSAGE (Table 6.1.4.1.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoinfo |  |  |  |  |
| mcvideo-Params |  |  |  |  |
| anyExt |  |  |  |  |
| mcvideo-request-uri | px_MCVideo_Group_A_ID | The MCVideo group identity to be selected by the targeted MCVideo user | TS 24.281 [26] clause 9.2.4.2.1 |  |
| request-type | "group-selection-change-request" |  | TS 24.281 [26] clause 9.2.4.2.1 |  |

Table 6.1.4.1.3.3-3: Resource lists in SIP MESSAGE (Table 6.1.4.1.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.3.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| resource-lists |  |  |  |  |
| list |  |  |  |  |
| name attribute |  |  |  |  |
| display-name |  |  |  |  |
| entry |  |  |  |  |
| uri attribute | px_MCVideo_ID_User_C | The targeted MCVideo user |  |  |

Table 6.1.4.1.3.3-4: SIP MESSAGE (Step 2, Table 6.1.4.1.3.2-1; Step 4, TS 36.579-1 [2] Table 5.3.30.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| P-Asserted-Identity |  |  |  |  |
| name-addr | px_MCVideo_ID_User_C |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCVideo Info |  |  |
| MIME-part-body | MCVideo-Info as described in Table 6.1.4.1.3.3-5 |  |  |  |

Table 6.1.4.1.3.3-5: MCVideo-Info in SIP MESSAGE (Table 6.1.4.1.3.3-4)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideoinfo |  |  |  |  |
| mcvideo-Params |  |  |  |  |
| mcvideo-request-uri | px_MCVideo_ID_User_A |  | TS 24.281 [26] clause 9.2.4.4 |  |
| mcvideo-calling-user-id | not present |  |  |  |
| mcvideo-calling-group-id | px_MCVideo_Group_A_ID |  | TS 24.281 [26] clause 9.2.4.4 |  |
| anyExt |  |  |  |  |
| mcvideo-request-uri | px_MCVideo_Group_A_ID | The MCVideo group identity to be selected by the targeted MCVIDEO user | TS 24.281 [26] clause 9.2.4.4 |  |
| response-type | "group-selection-change-response" |  | TS 24.281 [26] clause 9.2.4.4 |  |
| selected-group-change-outcome | "success" |  | TS 24.281 [26] clause 9.2.4.4 |  |

<END OF CHANGES>