3GPP TSG-RAN5 Meeting #94-e	R5-221577

# Electronic Meeting, 21st Feb– 04th March 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2786 | rev | 1 | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MDT TC 8.1.6.3.3.3-Inter System_RLF_Sensor |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To minimize the applicability surported by UEs for this test case. - We could further check that the IEs sensor-MeasurementInformation-r16 and sensor-MotionInformation-r16 are decodable as per the 37.355 ASN.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modify Pre-test conditions to minimize the applicability surported by UEs - Further check that the IEs sensor-MeasurementInformation-r16 and sensor-MotionInformation-r16 are decodable as per the 37.355 ASN.1. - Editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-220618r1 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

8.1.6.3.3.3	Inter-System MDT / Radio Link Failure / Logging and reporting / sensor information

8.1.6.3.3.3.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with the radio link failure information available and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

ensure that {

when { UE receives the UEInformationRequest message with rlf-ReportReq }

then { UE sends the UEInformationResponse message with rlf-Report-r16 including LocationInfo-r16 which includes Sensor-LocationInfo-r16 }

}

(2)

with { UE in RRC_CONNECTED state with the radio link failure information available and plmn-Identity stored in VarRLF-Report is equal to the RPLMN }

ensure that {

when { UE receives the UEInformationRequest message with rlf-ReportReq and a sensor type is not included in sensor-NameListConfig }

then { UE sends the UEInformationResponse message not including the measurement result of the sensor not included in sensor-NameListConfig }

}

8.1.6.3.3.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 clauses 5.3.5.9, 5.3.10.5, 5.3.3.7, 5.7.10.3.

[TS 38.331, clause 5.3.5.9]

The UE shall:

1>	if the received otherConfig includes the wlanNameList:

2>	if wlanNameList is set to setup, include available WLAN measurement results for any subsequent measurement report or any subsequent RLF report, CEF report and SCGFailureInformation;

1>	if the received otherConfig includes the sensorNameList:

2>	if sensorNameList is set to setup, include available Sensor measurement results for any subsequent measurement report or any subsequent RLF report, CEF report and SCGFailureInformation;

NOTE 2:	The UE is requested to attempt to have valid Bluetooth measurements, WLAN measurements and Sensor measurements whenever sending a measurement report for which it is configured to include these measurements. The UE may not succeed e.g. because the user manually disabled the WLAN or Bluetooth or Sensor hardware. Further details, e.g. regarding when to activate WLAN or Bluetooth or Sensor, are up to UE implementation.

[TS 38.331, clause 5.3.10.5]

The UE shall determine the content in the VarRLF-Report as follows:

1>	clear the information included in VarRLF-Report, if any;

1>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN);

…

1>	else if the failure is detected due to radio link failure as described in 5.3.10.3, set the fields in VarRLF-report as follows:

2>	set the connectionFailureType to rlf;

2>	set the rlf-Cause to the trigger for detecting radio link failure in accordance with clause 5.3.10.4;

2>	set the nrFailedPCellId in failedPCellId to the global cell identity and the tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected;

2>	if an RRCReconfiguration message including the reconfigurationWithSync was received before the connection failure:

3>	if the last RRCReconfiguration message including the reconfigurationWithSync concerned an intra NR handover:

4>	include the nrPreviousCell in previousPCellId and set it to the global cell identity and the tracking area code of the PCell where the last RRCReconfiguration message including reconfigurationWithSync was received;

4>	set the timeConnFailure to the elapsed time since reception of the last RRCReconfiguration message including the reconfigurationWithSync;

…

1>	if available, set the locationInfo as in 5.3.3.7.

[TS 38.331, clause 5.3.3.7]

The UE shall:

1>	if timer T300 expires:

…

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

…

3>	if available, set the locationInfo as follows:

4>	if available, set the commonLocationInfo to include the detailed location information;

4>	if available, set the bt-LocationInfo to include the Bluetooth measurement results, in order of decreasing RSSI for Bluetooth beacons;

4>	if available, set the wlan-LocationInfo to include the WLAN measurement results, in order of decreasing RSSI for WLAN APs;

4>	if available, set the sensor-LocationInfo to include the sensor measurement results as follows;

5>	if available, include the sensor-MeasurementInformation;

5>	if available, include the sensor-MotionInformation;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if rlf-ReportReq is set to true:

2>	if the UE has radio link failure information or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report:

3>	set timeSinceFailure in VarRLF-Report to the time that elapsed since the last radio link failure or handover failure in NR;

3>	set the rlf-Report in the UEInformationResponse message to the value of rlf-Report in VarRLF-Report;

3>	discard the rlf-Report from VarRLF-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

8.1.6.3.3.3.3	Test description

8.1.6.3.3.3.3.1	Pre-test conditions

System Simulator:

- NR Cell 1, NR Cell 2

UE:

-	UE supports at least one of pc_barometer_r16, pc_orientation_r16 and pc_speed_r16.

Preamble:

- The UE is in state 1N-A as defined in TS 38.508-1 [4], subclause 4.4A.

8.1.6.3.3.3.3.2	Test procedure sequence

Table 8.1.6.3.3.3.3.2-1 illustrates the downlink power levels. Row marked "T0" denotes the conditions after the preamble.

Table 8.1.6.3.3.3.3.2-1: Time instances of cell power level and parameter changes (FR1)

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | The power level values are such that measurement results for NR Cell 1 (M1) and NR Cell 2 (M2) satisfy exit condition for event A3 (M2 < M1). |
|  | BeaconRSSI | dBm |  |  |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | “Off” | -79 | Only NR Cell 2 is available. (NOTE 1) |
|  | BeaconRSSI | dBm |  |  |  |

Table 8.1.6.3.3.3.3.2-2: Time instances of cell power level and parameter changes (FR2)

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | FFS | FFS | The power level values are such that measurement results for NR Cell 1 (M1) and NR Cell 2 (M2) satisfy exit condition for event A3 (M2 < M1). |
|  | BeaconRSSI | dBm |  |  |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | “Off” | FFS | Only NR Cell 2 is available. (NOTE 1) |
|  | BeaconRSSI | dBm |  |  |  |

Table 8.1.6.3.3.3.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message on NR Cell 1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message on NR Cell 1. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS changes NR Cell 1 parameter according to the row "T1" in Table 8.1.6.3.3.3.3.2-1. | - | - | - | - |
| 4 | The UE transmits an RRCReestablishmentRequest message on NR Cell 2. | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 5 | The SS transmits an RRCReestablishment message on NR Cell 2. | <-- | NR RRC: RRCReestablishment | - | - |
| 6 | UE transmit an RRCReestablishmentComplete message on NR Cell 2. | --> | NR RRC: RRCReestablishmentComplete | - | - |
| 7 | The SS transmits an RRCReconfiguration message on NR Cell 2. | <-- | NR RRC: RRCReconfiguration | - | - |
| 8 | The UE transmits an RRCReconfigurationComplete message on NR Cell 2. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 9 | The SS transmits a UEInformationRequest message on NR Cell 2. | <-- | NR RRC: UEInformationRequest | - | - |
| 10 | Check: Does the UE transmit a UEInformationResponse message with Sensor-LocationInfo- including the measurement result entry specified in the sensor-namelist in step 1? | --> | NR RRC: UEInformationResponse | 1,2 | P |

8.1.6.3.3.3.3.3	Specific message contents

Table 8.1.6.3.3.3.3.3-1: RRCReconfiguration (step 1, Table 8.1.6.3.3.3.3.2-3)

| Derivation path: 38.508-1 [4], Table 4.6.1-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| RRC-TransactionIdentifier |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| RRCReconfiguration-IEs SEQUENCE { |  |  |  |
| RRCReconfiguration-v1530-IEs SEQUENCE { |  |  |  |
| OtherConfig-v1610 SEQUENCE { |  |  |  |
| Setup sensor-NameList-r16 SEQUENCE (SIZE (1..maxsensor-Name-r16)) OF | 1 entry |  |  |
| sensor-Name-r16 [1] SEQUENCE { |  |  |  |
| measUncomBarPre-r16 | true |  | pc_barometer_r16 |
|  | Not present |  |  |
| measUeSpeed | true |  | pc_speed_r16 |
|  | Not present |  |  |
| measUeOrientation | true |  | pc_orientation_r16 |
|  | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| reportType CHOICE { |  |  |  |
| Periodical SEQUENCE { |  |  |  |
| loggingInterval-r16 | ms320 | 0.32 second |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.3.3.3.3.3-2: UEInformationResponse (step 10, Table 8.1.6.3.3.3.3.2-3)

| Derivation Path: 38.508-1 clause 4.6.1 table 4.6.1-32B UEInformationResponse |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| RLF-Report-r16 CHOICE { |  |  |  |
| nr-RLF-Report-r16 SEQUENCE { |  |  |  |
| measResultLastServCell-r16 | Not checked |  |  |
| measResultNeighCells-r16 | Not checked |  |  |
| c-RNTI-r16 | Not checked |  |  |
| previousPCellId-r16 | Not checked |  |  |
| failedPCellId-r16 | Not checked |  |  |
| reconnectCellId-r16 | Not checked |  |  |
| timeUntilReconnection-16 | Not checked |  |  |
| reestablishmentCellId-r16 | Not checked |  |  |
| timeConnFailure-r16 | Not checked |  |  |
| timeSinceFailure-r16 | Not checked |  |  |
| connectionFailureType-r16 | Not checked |  |  |
| rlf-Cause-r16 | Not checked |  |  |
| LocationInfo-r16 SEQUENCE { | Not checked |  |  |
| commonLocationInfo-r16 | Not checked |  |  |
| bt-LocationInfo-r16 | Not checked |  |  |
| wlan-LocationInfo-r16 | Not checked |  |  |
| sensor-LocationInfo-r16 SEQUENCE { |  |  |  |
| sensor-MeasurementInformation-r16 | Not present |  |  |
| sensor-MeasurementInformation-r16 | OCTET STRING including Sensor-MeasurementInformation-r13 according to TS 37.355 cl 6.5.5.2. |  | pc_barometer_r16 |
| sensor-MotionInformation-r16 | Not present |  |  |
| sensor-MotionInformation-r16 SEQUENCE | OCTET STRING including Sensor-MotionInformation-r15 according to TS 37.355 cl 6.5.5.2. |  | pc_orientation_r16 OR pc_speed_r16 |
| } |  |  |  |
| } |  |  |  |
| noSuitableCellFound-r16 | Not checked |  |  |
| ra-InformationCommon-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 1>