3GPP TSG-RAN5 Meeting #94-e	R5-221613

# Electronic Meeting, 21st Feb – 04th Mar. 2022

Title:	LS on Additional RF requirements for NS_03U, NS_05U and NS_43U

Response to:

Release:	Release 15

Work Item:	5GS_NR_LTE-UEConTest

Source:	TSG RAN WG5

To:	TSG RAN WG4

Cc:	N/A

Contact Person:

#### Name:	Yuxin Hao

E-mail Address:	haoyuxin@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

In TS 38.101-1 clause 6.2.3, UE shall meet additional emission requirements on specific NR bands if NS_03U, NS_05U, NS_43U are indicated by RRC signaling. But the specific additional emission requirements for these network signaling values are actually absent in current specification.

Taking NS_05U as an example. According to TS 38.101-1 clause 6.2.3.1, UE should meet the additional emission requirement specified in clause 6.5.3.3.4 for NR band n1, n65 and n84 when NS_05U is indicated in the field additionalSpectrumEmission. But in clause 6.5.3.3.4, there’s only additional requirements for NS_05 specified. Additional requirements for NS_05U are not included. And no instructions can be found throughout specification whether the requirements in table 6.5.3.3.4-1 can be applied for NS_05U or not.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | Table 5.3.2-1 | N/A |
| NS_03 | 6.5.2.3.3 | n2, n25, n66, n70, n86 |  |  | Clause 6.2.3.7 |
| NS_03U | 6.5.2.3.3, 6.5.2.4.2 | n2, n25, n66, n86 |  |  | Clause 6.2.3.7 |
| NS_04 | 6.5.2.3.2, 6.5.3.3.1 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | Clause 6.2.3.2 |
| NS_05 | 6.5.3.3.4 | n1, n65, n84 | 5, 10, 15, 20 (NOTE 2) |  | Clause 6.2.3.4 |
| NS_05U | 6.5.3.3.4, 6.5.2.4.2 | n1, n65, n84 | 5, 10, 15, 20 |  | Clause 6.2.3.4 |
| … | … | … | … | … | … |
| NS_43 | 6.5.3.3.5 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.6 |
| NS_43U | 6.5.3.3.5, 6.5.2.4.2 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.6 |

##### 6.5.3.3.4	Requirement for network signalling value "NS_05"

When "NS_05" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.4-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.4-1: Additional requirements for "NS_05"

| Frequency band (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |  |
| --- | --- | --- | --- |
|  | 5, 10, 15, 20 |  |  |
| 1884.5 f 1915.7 | -41 | 300 kHz |  |

Similar issues also exist for NS_03U and NS_43U. The additional requirements in clause 6.5.2.3.3 for NS_03U and 6.5.3.3.5 for NS_43U are absent in TS 38.101-1.

The additional emission requirements for NS_03U, NS_05U and NS_43U need to be clarified in TS 38.101-1. RAN5 respectfully requests RAN4 to reply with clarification of the additional emission requirements for NS_03U, NS_05U and NS_43U in TS 38.101-1 subclauses 6.5.2.3.3, 6.5.3.3.4 and 6.5.3.3.5 so that RAN5 could correctly design the testing of these requirements.

2. Actions:

To RAN4 group.

ACTION: 	RAN5 respectfully requests RAN4 to reply with clarification of the additional emission requirements for NS_03U, NS_05U and NS_43U in TS 38.101-1 subclauses 6.5.2.3.3, 6.5.3.3.4 and 6.5.3.3.5.

3. Date of Next TSG-RAN WG5 Meetings:

TSG-RAN5 Meeting#95e 	 16rd – 27th May 2022		Electronic Meeting

TSG-RAN5 Meeting#96	 	 22nd – 26th August 2022	Toulouse, FR

# References

- R4-1811246, NS numbering for NR, Qualcomm Incorporated

- R4-1811457, NS numbering, TS 38.101-1, Qualcomm Incorporated