3GPP TSG-RAN5 Meeting #95-e	R5-222392

# Electronic Meeting, 9th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0254 | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of clause 5.5.3.15 - Conference-info |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Conference-info is used at several places in MCPTT and MCVideo test case 6.1.3.1 and has many more fields as specified in the specific message contents so far |  |  |  |  |  |  |  |  |
| Summary of change: |  | Default message content added for conference-info |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.15 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.3.15	Conference-info

Table 5.5.3.15-1: Conference-info from the SS

| Derivation Path: RFC 4575 [127] clause 6 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| conference-info |  |  |  |  |
| entity attribute | Encrypted URI (NOTE 1) with value set to px_MCPTT_Group_A_ID | The URI of the group |  | MCPTT |
|  | Encrypted URI (NOTE 1) with value set to px_MCVideo_Group_A_ID |  |  | MCVIDEO |
| state attribute | not present |  |  |  |
| version attribute | not present |  |  |  |
| conference-description | not present |  |  |  |
| host-info | not present |  |  |  |
| conference-state | not present |  |  |  |
| users |  |  |  |  |
| user [1] |  |  |  |  |
| entity attribute | Encrypted URI (NOTE 1) with value set to px_MCPTT_ID_User_A |  |  | MCPTT |
|  | Encrypted URI (NOTE 1) with value set to px_MCVideo_ID_User_A |  |  | MCVIDEO |
| state attribute | not present |  |  |  |
| display-text | not present |  |  |  |
| associated-aors | not present |  |  |  |
| roles | not present |  |  |  |
| languages | not present |  |  |  |
| cascaded-focus | not present |  |  |  |
| endpoint |  |  |  |  |
| entity attribute | px_MCX_SIP_PublicUserId_A_1 | Contact URI of the participant | RFC 4575 [127] clause 5.7 |  |
| status attribute | not present |  |  |  |
| display-text | not present |  |  |  |
| referred | not present |  |  |  |
| status | connected |  |  |  |
| joining-method | not present |  |  |  |
| joining-info | not present |  |  |  |
| disconnection-method | not present |  |  |  |
| disconnection-info | not present |  |  |  |
| media | not present |  |  |  |
| call-info | not present |  |  |  |
| user [2] |  |  |  |  |
| entity attribute | Encrypted URI (NOTE 1) with value set to px_MCPTT_ID_User_B |  |  | MCPTT |
|  | Encrypted URI (NOTE 1) with value set to px_MCVideo_ID_User_B |  |  | MCVIDEO |
| state attribute | not present |  |  |  |
| display-text | not present |  |  |  |
| associated-aors | not present |  |  |  |
| roles | not present |  |  |  |
| languages | not present |  |  |  |
| cascaded-focus | not present |  |  |  |
| endpoint |  |  |  |  |
| entity attribute | px_MCX_SIP_PublicUserId_B | Contact URI of the participant | RFC 4575 [127] clause 5.7 |  |
| status attribute | not present |  |  |  |
| display-text | not present |  |  |  |
| referred | not present |  |  |  |
| status | connected |  |  |  |
| joining-method | not present |  |  |  |
| joining-info | not present |  |  |  |
| disconnection-method | not present |  |  |  |
| disconnection-info | not present |  |  |  |
| media | not present |  |  |  |
| call-info | not present |  |  |  |
| user [3] |  |  |  |  |
| entity attribute | Encrypted URI (NOTE 1) with value set to px_MCPTT_ID_User_C |  |  | MCPTT |
|  | Encrypted URI (NOTE 1) with value set to px_MCVideo_ID_User_C |  |  | MCVIDEO |
| state attribute | not present |  |  |  |
| display-text | not present |  |  |  |
| associated-aors | not present |  |  |  |
| roles | not present |  |  |  |
| languages | not present |  |  |  |
| cascaded-focus | not present |  |  |  |
| endpoint |  |  |  |  |
| entity attribute | px_MCX_SIP_PublicUserId_C | Contact URI of the participant | RFC 4575 [127] clause 5.7 |  |
| status attribute | not present |  |  |  |
| display-text | not present |  |  |  |
| referred | not present |  |  |  |
| status | connected |  |  |  |
| joining-method | not present |  |  |  |
| joining-info | not present |  |  |  |
| disconnection-method | not present |  |  |  |
| disconnection-info | not present |  |  |  |
| media | not present |  |  |  |
| call-info | not present |  |  |  |
| sidebars-by-ref | not present |  |  |  |
| sidebars-by-val | not present |  |  |  |
| NOTE 1: Encrypted attribute as described in Table 5.5.13.3-1 |  |  |  |  |