3GPP TSG-RAN5 Meeting #95-e	R5-222529

# Electronic Meeting, 9th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1776 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification to the asynchronous / synchronous cells conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Annex Tables H.3.4-6 and H.3.1-3 define conditions “asynchronous” and “synchronous”. The only use of these conditions is to determine whether deriveSSB-IndexFromCell is true or false. deriveSSB-IndexFromCell The field indicates whether the UE may use, to derive the SSB index of a cell on the indicated SSB frequency and subcarrier spacing, the timing of the NR serving cell with the same SSB frequency and subcarrier spacing if configured. Otherwise, the field indicates whether the UE may use the timing of any detected cell with the same SSB frequency and subcarrier spacing. The parameter is only required for test cases with multiple NR cells, to determine whether the SSB Index of a neighbour cell could be derived. Therefore, this parameter is not needed in single-cell test cases Furthermore, there is no need to define both conditions “asynchronous” and “synchronous” separately. A better practice is to define only one of the conditions and leave the other one as default behavior. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For Table H.3.4-6: added a third option (default option) for deriveSSB-IndexFromCell-r15 to be “Not present”. This is the default option for test cases unless they explicitely request something. For Table H.3.1-3, since the deriveSSB-IndexFromCell is mandatory, set it by default to false (removing “Asynchronous” condition). Only test cases requiring to set it to true need to explicitly state “Synchronous”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be superfluous conditions and confusion, since the conditions are used in all test cases and they are in most cases not even needed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | H.3.1, H.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

# H.3	RRC message content exceptions

## H.3.1	RRC messages and information elements contents exceptions for NR measurement configuration

< Unchanged parts omitted >

MeasObjectNR-DEFAULT

NR measurement object configuration.

Table H.3.1-3: MeasObjectNR-DEFAULT: NR intra-frequency measurement object configuration

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR of the SSB associated to serving cell | frequency of the serving cell | INTRA-FREQ MO |
|  | ARFCN-ValueNR of the SSB associated to SCell |  | Deactivated SCell |
|  | ARFCN-ValueNR of the SSB associated to inter-frequency neighbour cell |  | INTER-FREQ MO |
| referenceSignalConfig SEQUENCE { |  |  |  |
| ssb-ConfigMobility SEQUENCE { |  |  |  |
| ssb-ToMeasure | Not present |  |  |
| deriveSSB-IndexFromCell | false |  |  |
|  | true |  | Synchronous cells OR RLM |
| } |  |  |  |
| } |  |  |  |
| absThreshSS-BlocksConsolidation SEQUENCE { |  |  |  |
| thresholdRSRP | 0 | SS-RSRP < -156dB | NOT SS-SINR |
| thresholdSINR | 0 | SS-SINR<-23 | SS-SINR |
| } |  |  |  |
| measCycleSCell-v1530 | sf640 |  | Deactivated SCell |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| INTRA-FREQ MO | Configuration for NR MO associated to intra-frequency carrier |
| INTER-FREQ MO | Configuration for NR MO associated to inter-frequency carrier |
| Synchronous cells | SSB indices of neighbourcells can be derived from timing of serving cell |
| RLM | Configuration for RLM tests |
| Deactivated SCell | Configuration for SCell activation and deactivation tests |
| SS-SINR | Configuration for SS-SINR tests |

# < Unchanged sections omitted >

## H.3.4	E-UTRA RRC messages and information elements contents exceptions for NR measurement configuration

< Unchanged parts omitted >

Table H.3.4-6: MeasObjectNR

| Derivation Path: TS 36.508 [25] Table 4.6.6-2B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR-GENERIC(Freq) ::= SEQUENCE { |  |  |  |
| rs-ConfigSSB-r15 ::= SEQUENCE { |  |  |  |
| measTimingConfig-r15 ::= SEQUENCE { |  |  |  |
| periodicityAndOffset-r15 CHOICE { |  |  |  |
| sf20-r15 | 0 |  | SMTC.1 or SMTC.2 |
|  | 10 |  | SMTC.4 or SMTC.5 |
|  | 17 |  | SMTC.6 |
| sf160-r15 | 0 |  | SMTC.3 |
| } |  |  |  |
| ssb-Duration-r15 | sf1 |  | SMTC.1, SMTC.3 or SMTC.4 |
|  | sf5 |  | SMTC.2, SMTC.5 or SMTC.6 |
| } |  |  |  |
| subcarrierSpacingSSB-r15 | kHz15 |  | SSB.1 FR1, SSB.3 FR1 or SSB.5 FR1 |
|  | kHz30 |  | SSB.2 FR1, SSB.4 FR1 or SSB.6 FR1 |
|  | kHz120 |  | SSB.1 FR2, SSB.3 FR2, SSB.5 FR2 or SSB.7 FR2 |
|  | KHz240 |  | SSB.2 FR2, SSB.4 FR2, SSB.6 FR2 or SSB.8 FR2 |
| } |  |  |  |
| deriveSSB-IndexFromCell-r15 | Not present |  |  |
|  | true |  | Synchronous cells |
|  | false |  | Asynchronous cells |
| } |  |  |  |

< End of changes >