3GPP TSG-RAN5 Meeting #95-e	R5-222135

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-2 | CR | 0289 | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of 36.579-2 TC 6.2.22 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCenhUEConTest |  |  |  |  |  | Date: |  |  | 2022-04-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The SDP for the no floor control scenario was updated in the core spec |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the message contents of the affected messages to include the new conditions |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A device will not properly configure the calls to use the new fmtp attribute and fail the test |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.22 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<START OF CHANGES>

### 6.2.22	On-network / First-to-answer call / Pre-established session / Client Originated (CO)

6.2.22.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client) registered and authorized for MCPTT Service, including authorized to initiate first-to-answer calls, and, having established a pre-established session }

ensure that {

when { MCPTT User requests the establishment of an MCPTT first-to-answer call within the pre-established session }

then { UE (MCPTT Client) sends a SIP REFER request outside a dialog requesting an MCPTT first-to-answer call and providing a MIME resource-lists body with the MCPTT IDs of the potential target MCPTT users }

}

(2)

with { UE (MCPTT Client) registered and authorized for MCPTT Service, including authorized to initiate first-to-answer calls, and, having established a pre-established session, and, having send a SIP REFER request outside a dialog requesting an MCPTT first-to-answer call and providing a MIME resource-lists body with the MCPTT IDs of the potential target MCPTT users }

ensure that {

when { UE (MCPTT Client) receives a Connect message from the participating MCPTT function }

then { UE (MCPTT Client) sends an Acknowledge message indicating that the connection to the pre-established session is accepted }

}

(3)

with { UE (MCPTT Client) registered and authorized for MCPTT Service, including authorized to initiate first-to-answer calls, and, having established a pre-established session, and, having send a SIP REFER request outside a dialog requesting an MCPTT first-to-answer call and providing a MIME resource-lists body with the MCPTT IDs of the potential target MCPTT users, and, having accepted the connection to the pre-established session }

ensure that {

when { UE (MCPTT Client) receives a re-INVITE within the pre-established session }

then { UE (MCPTT Client) sends OK confirming the establishment of the first-to-answer call and notifies the user }

}

6.2.22.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.379 clauses 11.1.2.2, 11.1.1.2.2.1, 11.1.1.2.2.2, TS 24.380 clauses 4.1.2.2, 9.2.2.3.2. Unless otherwise stated these are Rel-15 requirements.

[TS 24.379, clause 11.1.2.2]

When the MCPTT user wants to make a private call without floor control or first-to-answer call without floor control using a pre-established session, the MCPTT client shall follow the procedures in subclause 11.1.1.2.2.1 with the following exceptions:

...

2)	step 9a) is re-written as: if the SDP parameters of the pre-established session contain a media-level section of a media-floor control entity, an application/sdp MIME body containing the SDP parameters of the pre-established session according to 3GPP TS 24.229 [4] with the clarifications given in subclause 6.2.1. If the pre-established session was established with implicit floor control, then the application/sdp MIME body shall not contain the implicit floor request as specified in subclause 6.4.

[TS 24.379, clause 11.1.1.2.2.1]

Upon receiving a request from an MCPTT user to establish an MCPTT private call within a pre-established session the MCPTT client shall generate a SIP REFER request outside a dialog in accordance with the procedures specified in 3GPP TS 24.229 [4], IETF RFC 4488 [22] and IETF RFC 3515 [25] as updated by IETF RFC 6665 [26] and IETF RFC 7647 [27], with the clarifications given below.

...

The MCPTT client populates the SIP REFER request as follows:

1)	shall include the Request-URI set to the public service identity identifying the pre-established session on the MCPTT server serving the MCPTT user;

2)	shall include the Refer-Sub header field with value "false" according to rules and procedures of IETF RFC 4488 [22];

3)	shall include the Supported header field with value "norefersub" according to rules and procedures of IETF RFC 4488 [22];

4)	shall include the option tag "multiple-refer" in the Require header field;

5)	may include a P-Preferred-Identity header field in the SIP REFER request containing a public user identity as specified in 3GPP TS 24.229 [4];

6)	shall include a P-Preferred-Service header field set to the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), according to IETF RFC 6050 [9];

7)	shall set the Refer-To header field of the SIP REFER request as specified in IETF RFC 3515 [25] with a Content-ID ("cid") Uniform Resource Locator (URL) as specified in IETF RFC 2392 [62] that points to an application/resource-lists MIME body as specified in IETF RFC 5366 [20], and with the Content-ID header field set to this "cid" URL.

...

9)	for an initiation of a first-to-answer call, shall include in the application/resource-lists MIME body an <entry> element for each of the targeted MCPTT users, with each <entry> element containing a "uri" attribute set to the MCPTT ID of the targeted user, extended with hname "body" URI header field containing:

NOTE 2:	Characters that are not formatted as ASCII characters are escaped in the following URI header fields

a)	if the SDP parameters of the pre-established session do not contain a media-level section of a media-floor control entity, an application/sdp MIME body containing the SDP parameters of the pre-established session according to 3GPP TS 24.229 [4] with the clarification given in subclause 6.2.1. If implicit floor control is required and the pre-established session was not established with an implicit floor request, then the application/sdp MIME body shall contain an implicit floor request as specified in subclause 6.4; and

b)	an application/vnd.3gpp.mcptt-info MIME body with the <session-type> element set to "first-to-answer";

...

12)	shall include a Target-Dialog header field as specified in IETF RFC 4538 [23] identifying the pre-established session; and

13)	if:

a)	implicit floor control is required;

b)	the pre-established session was not established with an implicit floor request; and

c)	location information has not yet been included in the SIP REFER request;

then shall include an application/vnd.3gpp.mcptt-location-info+xml MIME body with a <Report> element included in the <location-info> root element.

The MCPTT client shall send the SIP REFER request towards the MCPTT server according to 3GPP TS 24.229 [4].

Upon receiving a final SIP 2xx response to the SIP REFER request the MCPTT client shall interact with media plane as specified in 3GPP TS 24.380 [5].

...

Upon receipt of a SIP re-INVITE request within the pre-established session targeted by the sent SIP REFER request, the MCPTT client:

1)	if the sent SIP REFER request was a request to originate a first-to-answer call:

a)	if the received SIP re-INVITE request contains an SDP offer including an a=key-mgmt attribute field with a "mikey" attribute value containing a MIKEY-SAKKE I_MESSAGE:

i)	shall extract the MCPTT ID of the sender of the SIP 200 (OK) response from the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.180 [78];

ii)	shall convert the MCPTT ID to a UID as described in 3GPP TS 33.180 [78];

iii)	shall use the UID to validate the signature of the MIKEY-SAKKE I_MESSAGE as described in 3GPP TS 33.180 [78];

...

vii)	if the signature of the MIKEY-SAKKE I_MESSAGE was successfully validated:

A)	shall extract and decrypt the encapsulated PCK using the originating user's (KMS provisioned) UID key as described in 3GPP TS 33.180 [78]; and

B)	shall extract the PCK-ID, from the payload as specified in 3GPP TS 33.180 [78];

NOTE 3:	With the PCK successfully shared between the originating MCPTT client and the terminating MCPTT client, both clients are able to use SRTP/SRTCP to create an end-to-end secure session;

...

4)	shall accept the SIP re-INVITE request and generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [4];

5)	shall include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP re-INVITE request according to 3GPP TS 24.229 [4], based upon the parameters already negotiated for the pre-established session; and

6)	shall send the SIP 200 (OK) response towards the participating MCPTT function according to rules and procedures of 3GPP TS 24.229 [4].

[TS 24.380, clause 4.1.2.2]

For a pre-arranged group call, when the originator initiates the call setup indicating the use of a pre-established session using SIP messages as specified in 3GPP TS 24.379 [2], the participating MCPTT function (which serves the originating MCPTT client) sends to the originating MCPTT client a Connect message after the controlling MCPTT function accepts the initiation of this call. After the reception of this Connect message the originating MCPTT client sends an Acknowledgment message indicating that the connection is accepted or indicating that the connection is not accepted. If the connection is accepted by the originating MCPTT client, the floor control for this call continues as specified in clause 6.

For a pre-arranged group call if the controlling MCPTT function as triggered by an originating group member initiates a call as specified in 3GPP TS 24.379 [2], the participating MCPTT function which serves the terminating MCPTT client sends a Connect message to all affiliated MCPTT clients of this group. After the reception of the Connect message the terminating MCPTT client sends an Acknowledgment message indicating that the connection is accepted or indicating that the connection is not accepted. If the connection is accepted by the terminating MCPTT client, the floor control for this call continues as specified in clause 6.

NOTE:	If a terminating client does not have an available pre-established session, the call setup proceeds as in on-demand call setup as specified in 3GPP TS 24.379 [2].

...

For a private call the procedures for the originator are the same as for the originator initiating a call for a pre-arranged call setup over a pre-established session, with the difference that the recipient of the call is a private user and not a pre-arranged group.

For a private call if the controlling MCPTT function as triggered by the originator initiates a call as specified in 3GPP TS 24.379 [2], the participating MCPTT function (which serves the terminating MCPTT client) sends a Connect message to the terminating MCPTT client served by the participating MCPTT function if this MCPTT client has an available pre-established session and the commencement mode is automatic. If the commencement mode is manual the terminating MCPTT client is invited using SIP procedures as specified in 3GPP TS 24.379 [2].

[TS 24.380, clause 9.2.2.3.2]

Upon reception of a Connect message:

1.	if the MCPTT client accepts the incoming call the MCPTT client:

a.	shall send the Acknowledgement message with Reason Code field set to 'Accepted';

b.	shall use only the media streams of the pre-established session which are indicated as used in the associated call session Media Streams field, if the Connect contains a Media Streams field;

c.	shall create an instance of the 'Floor participant state transition diagram for basic operation' as specified in subclause 6.2.4; and

d. shall enter the 'U: Pre-established session in use' state; or

6.2.22.3	Test description

6.2.22.3.1	Pre-test conditions

System Simulator:

-	SS (MCPTT server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCPTT operation in the MCPTT configuration document).

IUT:

-	UE (MCPTT client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The UE has performed the Generic Test Procedure for MCPTT UE registration as specified in TS 36.579-1 [2], subclause 5.4.2.

-	The MCPTT User performs the procedure for MCPTT Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

-	The MCPTT User is authorized to initiate first-to-answer call: <allow-request-first-to-answer-call> element of the <ruleset> element of the <ruleset> element is present in the MCPTT user profile document and is set to "true".

-	The MCPTT User performs the procedure for MCPTT pre-established session establishment CO as specified in TS 36.579-1 [2], subclause 5.3.3.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCPTT Client Application has been activated and User has registered-in as the MCPTT User with the Server as active user at the Client.

6.2.22.3.2	Test procedure sequence

Table 6.2.22.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCPTT User) request the establishment of an MCPTT first-to-answer call with targeted users B and C, within a pre-established session. (NOTE 1) | - | - | - | - |
| 2a1-4 | Check: Does the UE (MCPTT client) perform steps 1a1-3 of MCPTT CO call establishment using a pre-established session according to TS 36.579-1 [2], clause 5.3A.3? NOTE: The UE sends a SIP REFER which is responded with a SIP 200 OK. | - | - | 1 | - |
| 5-6 | Void | - | - | - | - |
| 7 | The SS (MCPTT server) sends SIP re-INVITE containing the a=key-mgmt attribute. NOTE: The SS simulates the case when the first-to-answer Client sent a SIP 200 (OK) response containing an SDP answer including an a=key-mgmt "mikey" attribute value containing a MIKEY-SAKKE I_MESSAGE. | <-- | SIP INVITE | - | - |
| - | EXCEPTION: Step 8a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE responds to a SIP INVITE with a SIP 100 (Trying) | - | - | - | - |
| 8a1 | The UE (MCPTT client) sends SIP 100 (Trying). | --> | SIP 100 (Trying) | - | - |
| 9 | Check: Does the UE (MCPTT client) send a SIP 200 (OK)? | --> | SIP 200 (OK) | 3 | P |
| 10 | The SS (MCPTT server) sends a SIP ACK to acknowledge the session establishment | <-- | SIP ACK | - | - |
| 10A-10B | Check: Does the UE (MCPTT client) perform steps 4-5 of MCPTT CO call establishment using a pre-established session according to TS 36.579-1 [2], clause 5.3A.3? NOTE: The UE acknowledges the Connect sent by the SS. | - | - | 2 | - |
| 11 | Check: Does the UE (MCPTT Client) notifies the User that the call was established? (NOTE 1) | - | - | 3 | P |
| 12 | Make the UE (MCPTT User) request termination of the MCPTT first-to-answer call. (NOTE 1) | - | - | - | - |
| 13 | The UE (MCPTT Client) performs the procedure for MCX CO call release as described in TS 36.579-1 [2] Table 5.3.10.3-1 to end the private call. | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: The media plane control messages related to call setup/release over a pre-established session are sent over the channel used for media plane control. |  |  |  |  |  |

6.2.22.3.3	Specific message contents

Table 6.2.22.3.3-1: SIP REFER (Step 3, Table 6.2.22.3.2-1; step 2, TS 36.579-1 [2] Table 5.3A.3.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.12-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| MIME body part |  | Resource list |  |  |
| MIME-part-body | As described in Table 6.2.22.3.3-2 |  |  |  |

Table 6.2.22.3.3-2: Resource list in SIP REFER (Table 6.2.22.3.3-1)

Derivation Path: TS 36.579-1 [2], table 5.5.3.3.1-1, condition PRE-ESTABLISH AND FIRST-TO-ANSWER with the uri attribute of each entry extended with the SIP URI header fields as specified in Table 6.2.22.3.3-3.

Table 6.2.22.3.3-3: SIP header fields extending the uri attributes of the resource-lists' entries (Table 6.2.22.3.3-1)

| Derivation Path: TS 36.579-1 [2], table 5.5.2.12-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Answer-Mode | any value if present | NOTE 1 |  |  |
| body |  |  |  |  |
| MIME body part | optional | SDP Message |  |  |
| MIME-part-body | SDP Message as described in Table 6.2.22.3.3-4 if present |  |  |  |
| MIME body part |  | MCPTT Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.2.22.3.3-5 |  |  |  |
| NOTE 1: In case of a first-to-answer call there is no use for the client sending any Answer-Mode as according to TS 24.379 [9] clause 11.1.1.4.1 the MCPTT server sends the INVITE to the invited clients with Priv-Answer-Mode "Manual" independent from what the originating client has sent. In general the originating client should not include any Answer-Mode in the INVITE but this is not checked. |  |  |  |  |

Table 6.2.22.3.3-4: SDP Message in SIP REFER (Table 6.2.22.3.3-3)

Same SDP offer as sent during establishment of the pre-established session (TS 36.579-1 [2], clause 5.3.3)

Table 6.2.22.3.3-5: MCPTT-Info in SIP REFER (Table 6.2.22.3.3-3)

Derivation Path: TS 36.579-1 [2], table 5.5.3.2.1-1, condition INVITE_REFER, FIRST-TO-ANSWER

Table 6.2.22.3.3-6: SIP INVITE (Step 7, Table 6.2.22.3.2-1)

| Derivation Path: TS 36.579-1 [2], table 5.5.2.5.2-1, condition re_INVITE, MO_CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type |  |  |  |  |
| media-type | "application/sdp" |  |  |  |
| Message-body |  |  |  |  |
| SDP Message | SDP message as described in Table 6.2.22.3.3-6A |  |  |  |

Table 6.2.22.3.3-6A: SDP Message in SIP INVITE (Table 6.2.22.3.3-6)

| Derivation Path: TS 36.579-1 [2], table 5.5.3.1.2-1, condition PRE_ESTABLISHED_SESSION, WITH_SECURITY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Media description[2] |  | Media description for media control |  |  |
| media attribute |  | a= line attribute = fmtp |  |  |
| fmtp |  |  |  |  |
| format | "MCPTT" |  |  |  |
| format specific parameters | same parameters as sent to the UE in the SDP answer during establishment of the pre-established session (step 10 of table 5.3.3.3-1 in TS 36.579-1 [2]) |  |  |  |

Table 6.2.22.3.3-7: SIP 200 (OK) (Step 8, Table 6.2.22.3.2-1)

| Derivation Path: TS 36.579-1 [2], table 5.5.2.17.1.1-1 condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Content-Type | Not present |  |  |  |
| Message-body | Not present |  |  |  |

Table 6.2.22.3.3-8: Connect (Step 5, Table 6.2.22.3.2-1)

Derivation Path: 36.579-1 [2], Table 5.5.6.12-1, condition PRIVATE-CALL (NOTE 1), WITHOUT_FLOORCONTROL, ACK

NOTE 1: There is no <Session Type> value for "first-to-answer" in TS 24.380 [10] clause 8.3.3.3, but "private" only.

<END OF CHANGES>