3GPP TSG-RAN5 Meeting #95-e	R5-222340
Electronic Meeting, 9th May – 20th May 2022

Agenda item:	5.4.17

Source:	Keysight Technologies

Title:	Discussion on TT and testability proposal for FR2 EVM

Document for:	Discussion and Endorsement

# Introduction

The aim of this document is to progress in the TT definition for the FR2 EVM test case.

# Discussion

In [1], the following proposal to define TT and testability was made:

Proposal 3. If the increase of the measured EVM, as defined in [2] equation (1), is: - Below 7.5% of the minimum conformance requirement, no TT is applied. - Equal or greater than 7.5% of the minimum conformance requirement, set TT = 1*(the increase of the measured EVM). - If resulting TT is above 50% of the minimum conformance requirement, skip the test as not testable.

In that formula the factor between the influence of agreed MU and the proposed TT is k=1, what avoids any risk of failing a conformance UE applying the permitted MPR. There was a concern on the resulting testability limitations so it was considered that a relaxation in the factor k could be made in order to increase the test coverage.

In this document, previous proposal is reformulated to apply a factor k < 1 only when original proposal drives into an untestable test point.

Proposal 1. If the increase of the measured EVM, as defined in [2] equation (1), is:

- Below 7.5% of the minimum conformance requirement, no TT is applied.

- Equal or greater than 7.5% and equal or smaller than 50% of the minimum conformance requirement, set TT = 1*(the increase of the measured EVM).

- Else

  - Set TT = k*(the increase of the measured EVM) if resulting TT is equal or smaller than 50% of the minimum conformance requirement,

  - Otherwise, skip the test as not testable.

In Annex A it is analyzed the resulting TT and testability for different values of factor k. For FR2a it is observed that no testability enhancements are achieved for k<0.8 while for FR2b testability enhancements are achieved with k=0.6.

The analysis is limited down to k=0.6 assuming the risk of failing a conformance UE can already be too high.

Proposal 2. In proposal 1, use k=0.8 for FR2a and k=0.6 for FR2b.

Associated change requests can be found in [3], [4].

# Conclusion

The following observations and conclusions were made in this contribution.

Proposal 1. If the increase of the measured EVM, as defined in [2] equation (1), is:

- Below 7.5% of the minimum conformance requirement, no TT is applied.

- Equal or greater than 7.5% and equal or smaller than 50% of the minimum conformance requirement, set TT = 1*(the increase of the measured EVM).

- Else

  - Set TT = k*(the increase of the measured EVM) if resulting TT is equal or smaller than 50% of the minimum conformance requirement,

  - Otherwise, skip the test as not testable.

Proposal 2. In proposal 1, use k=0.8 for FR2a and k=0.6 for FR2b.

# References

- R5-221625: "Discussion on MU and TT proposal for FR2 EVM". 3GPP TSG-RAN5 Meeting #94-e, Electronic Meeting, 21st February - 4th March 2022

- R5-215476: "On the MU of Tx Modulation Quality test cases". 3GPP TSG-RAN5 Meeting #92-e, Electronic Meeting, 16th Aug 2021 - 27th Aug 2021.

- R5-222341: “FR2 SA EVM test case update based on TT analysis”, 3GPP TSG-RAN5 Meeting #95-e, Electronic Meeting, 9th May – 20th May 2022

- R5-222343: “FR2 NSA test case editor notes update”, 3GPP TSG-RAN5 Meeting #95-e, Electronic Meeting, 9th May – 20th May 2022

# Annex A Resulting TT and testability vs factor K

Changes vs the case k=1 highlighted in orange. Case with k=1 equivalent to proposal 3 in [1].

## K=1

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 0.00% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 0.00% | 0.00% | 0.00% | 0.00% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 1.61% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 0.00% | 0.00% | 0.00% | 2.18% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 0.00% | 0.00% | 1.53% | 4.29% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 0.00% | 0.00% | 1.67% | 4.29% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 1.06% | 1.97% | 3.61% | NA |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 1.44% | 2.68% | NA | NA |
| 9 | CP-OFDM QPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 3.66% |
| 10 | CP-OFDM QPSK | Outer_Full | 0.00% | 0.00% | 1.37% | 3.66% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 0.00% | 1.35% | 2.57% | NA |
| 12 | CP-OFDM 16 QAM | Outer_Full | 0.00% | 1.35% | 2.57% | NA |
| 13 | CP-OFDM 64 QAM | Inner_Full | 2.19% | 3.97% | NA | NA |
| 14 | CP-OFDM 64 QAM | Outer_Full | 2.19% | 3.97% | NA | NA |

Table 1. Resulting TT for EVM in PUSCH, PC3, FR2a for k=1 in proposal 1.

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 0.00% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 0.00% | 0.00% | 0.00% | 2.50% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 0.00% | 0.00% | 1.31% | 2.49% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 0.00% | 0.00% | 1.79% | 4.01% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 0.00% | 1.48% | 2.85% | NA |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 1.00% | 1.92% | 3.60% | NA |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 2.49% | NA | NA | NA |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 3.35% | NA | NA | NA |
| 9 | CP-OFDM QPSK | Inner_Full | 0.00% | 1.42% | 2.73% | 8.42% |
| 10 | CP-OFDM QPSK | Outer_Full | 0.00% | 1.58% | 3.04% | 8.42% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 1.72% | 3.25% | 5.92% | NA |
| 12 | CP-OFDM 16 QAM | Outer_Full | 1.72% | 3.25% | 5.92% | NA |
| 13 | CP-OFDM 64 QAM | Inner_Full | NA | NA | NA | NA |
| 14 | CP-OFDM 64 QAM | Outer_Full | NA | NA | NA | NA |

Table 2. Resulting TT for EVM in PUSCH, PC3, FR2b for k=1 in proposal 1.

## K=0.9

No testability enhancements versus k=1.

## K=0.8

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 0.00% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 0.00% | 0.00% | 0.00% | 0.00% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 1.61% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 0.00% | 0.00% | 0.00% | 2.18% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 0.00% | 0.00% | 1.53% | 4.29% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 0.00% | 0.00% | 1.67% | 4.29% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 1.06% | 1.97% | 3.61% | NA |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 1.44% | 2.68% | 3.84% | NA |
| 9 | CP-OFDM QPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 3.66% |
| 10 | CP-OFDM QPSK | Outer_Full | 0.00% | 0.00% | 1.37% | 3.66% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 0.00% | 1.35% | 2.57% | 6.15% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 0.00% | 1.35% | 2.57% | 6.15% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 2.19% | 3.97% | NA | NA |
| 14 | CP-OFDM 64 QAM | Outer_Full | 2.19% | 3.97% | NA | NA |

Table 3. Resulting TT for EVM in PUSCH, PC3, FR2a for k=0.8 in proposal 1.

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 0.00% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 0.00% | 0.00% | 0.00% | 2.50% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 0.00% | 0.00% | 1.31% | 2.49% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 0.00% | 0.00% | 1.79% | 4.01% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 0.00% | 1.48% | 2.85% | NA |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 1.00% | 1.92% | 3.60% | NA |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 2.49% | 3.58% | NA | NA |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 3.35% | NA | NA | NA |
| 9 | CP-OFDM QPSK | Inner_Full | 0.00% | 1.42% | 2.73% | 8.42% |
| 10 | CP-OFDM QPSK | Outer_Full | 0.00% | 1.58% | 3.04% | 8.42% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 1.72% | 3.25% | 5.92% | NA |
| 12 | CP-OFDM 16 QAM | Outer_Full | 1.72% | 3.25% | 5.92% | NA |
| 13 | CP-OFDM 64 QAM | Inner_Full | 3.93% | NA | NA | NA |
| 14 | CP-OFDM 64 QAM | Outer_Full | 3.93% | NA | NA | NA |

Table 4. Resulting TT for EVM in PUSCH, PC3, FR2b for k=0.8 in proposal 1.

## K=0.7

No testability enhancements versus k=0.8.

## K=0.6

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 0.00% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 0.00% | 0.00% | 0.00% | 0.00% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 1.61% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 0.00% | 0.00% | 0.00% | 2.18% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 0.00% | 0.00% | 1.53% | 4.29% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 0.00% | 0.00% | 1.67% | 4.29% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 1.06% | 1.97% | 3.61% | NA |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 1.44% | 2.68% | 2.88% | NA |
| 9 | CP-OFDM QPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 3.66% |
| 10 | CP-OFDM QPSK | Outer_Full | 0.00% | 0.00% | 1.37% | 3.66% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 0.00% | 1.35% | 2.57% | 4.61% |
| 12 | CP-OFDM 16 QAM | Outer_Full | 0.00% | 1.35% | 2.57% | 4.61% |
| 13 | CP-OFDM 64 QAM | Inner_Full | 2.19% | 3.97% | NA | NA |
| 14 | CP-OFDM 64 QAM | Outer_Full | 2.19% | 3.97% | NA | NA |

Table 5. Resulting TT for EVM in PUSCH, PC3, FR2a for k=0.6 in proposal 1.

| Test ID | Modulation | RB alloc. | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner_Full | 0.00% | 0.00% | 0.00% | 0.00% |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer_Full | 0.00% | 0.00% | 0.00% | 2.50% |
| 3 | DFT-s-OFDM QPSK | Inner_Full | 0.00% | 0.00% | 1.31% | 2.49% |
| 4 | DFT-s-OFDM QPSK | Outer_Full | 0.00% | 0.00% | 1.79% | 4.01% |
| 5 | DFT-s-OFDM 16 QAM | Inner_Full | 0.00% | 1.48% | 2.85% | 5.68% |
| 6 | DFT-s-OFDM 16 QAM | Outer_Full | 1.00% | 1.92% | 3.60% | 5.68% |
| 7 | DFT-s-OFDM 64 QAM | Inner_Full | 2.49% | 2.69% | NA | NA |
| 8 | DFT-s-OFDM 64 QAM | Outer_Full | 3.35% | 3.55% | NA | NA |
| 9 | CP-OFDM QPSK | Inner_Full | 0.00% | 1.42% | 2.73% | 8.42% |
| 10 | CP-OFDM QPSK | Outer_Full | 0.00% | 1.58% | 3.04% | 8.42% |
| 11 | CP-OFDM 16 QAM | Inner_Full | 1.72% | 3.25% | 5.92% | NA |
| 12 | CP-OFDM 16 QAM | Outer_Full | 1.72% | 3.25% | 5.92% | NA |
| 13 | CP-OFDM 64 QAM | Inner_Full | 2.95% | NA | NA | NA |
| 14 | CP-OFDM 64 QAM | Outer_Full | 2.95% | NA | NA | NA |

Table 6. Resulting TT for EVM in PUSCH, PC3, FR2b for k=0.6 in proposal 1.