3GPP TSG-RAN5 Meeting #95-e	R5-222378

# Online, , 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2514 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G V2X: Test Model updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - NR V2X uses a complete new radio interface, so the test model is moved from clause 5.2.1.6 to clause 5.3. - The test model for out of coverage scenario needs to be specified. - A new pixit is required to indicate the V2X operating bands for con-current operation. - Other typo. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Changed clause 5.2.1.6 to void and added clause 5.3. - Added out of coverage test model in clause 5.3.1. - Added px_NR_V2X_ConcurrentOperationBandCombination. - Corrected RRCReconfigurationSidelinkComplete to RRCReconfigurationCompleteSidelink. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Test specification structure will remain unclear. - The test model will remain incomplete. - The concurrent operating band cannot be selected by test operator. - The message name will not be as per TS 38.331. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.6, 5.3(added), 7.4.3.2.2.1, 7.4.3.2.2.2, 9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.2.1.6	Void

< TEXT SKIPPED >

##### 5.2.2.4.2	NR carrier aggregation

Figure 5.2.2.4.2-1: Test model for NR/5GC MAC CA testing

The NR/5GC MAC CA test model builds on top of the NR/5GC MAC test model, with the differences specified hereafter.

On the SS NR side, there is one PCell and one SCell configured:

-	PCell only: On DRBs the NR RLC is configured in transparent mode. Hence with this configuration PDUs out of SS RLC are same as the SDUs in it. There is no NR PDCP and SDAP configured in the SS. The ports are directly above NR RLC.

-	PCell / SCell: Layer 1 is configured in the normal way. NR MAC is configured in a special mode, where it does not add any MAC headers in DL and /or not remove any MAC headers in UL directions respectively at DRB port. In this case, the TTCN shall provide the final MAC PDU, including padding. Except for this, the MAC layer shall perform all of its other functions.

The UL Scheduling Grant and DL Scheduling assignments are configured from TTCN over system control port. SS reports PUCCH scheduling information reception in PCell over system indication port, if configured. In a similar way the reception of PRACH preambles in PCell / SCell is reported by SS over the same port, if configured.

The NR data routing between the RLC layer of PCell and the lower layers of either PCell or SCell shall be provided to/by SS in the common part of the data ASP using the MacBearerRouting field.

## 5.3	NR sidelink

#### 5.3.1	Layer 3

Figure 5.3.1-1: Test model for Layer 3 NR sidelink PC5 broadcast and groupcast in coverage

Figure 5.3.1-2: Test model for Layer 3 NR sidelink PC5 unicast in coverage

Figure 5.3.1-3: Test model for Layer 3 NR sidelink PC5 broadcast and groupcast out of coverage

Figure 5.3.1-4: Test model for Layer 3 NR sidelink PC5 unicast out of coverage

The UE is configured in normal mode or in Test Loop Mode E. On the UE side on the Uu interface, ciphering and integrity (PDCP and NAS) are enabled and ROHC is not configured. On the UE side on NR-PC5 interface, the NULL integrity algorithm and NULL ciphering is applied and ROHC is not configured.

For testing NR sidelink, the system simulator (SS) can implement one or several simulated NR cells as specified in clause 5.1.1.1 and one or several simulated UEs, called hereafter NR-SS-UE. The NR-SS-UE is used to send/receive data with the UE under test over the PC5 interface. A GNSS Simulator is configured when GNSS synchronisation source or a geographical position is required in the test. The requirements for the GNSS simulator (also referred as positioning simulator) are specified in TS 37.571-4 [28]. When UE is in coverage of an NR cell, this is depicted in the NR sidelink test model of Figure 5.3.1-1 for PC5 broadcast and groupcast and Figure 5.3.1-2 for PC5 unicast. When UE is out of coverage of an NR cell, this is depicted in the NR sidelink test model of Figure 5.3.1-3 for PC5 broadcast and groupcast and Figure 5.3.1-4 for PC5 unicast.

The NR-SS-UE is controlled by TTCN in the NR Sidelink PTC and is configured for NR sidelink by TTCN over sidelink system control NR_SL_SYS port. The NR-SS-UE can be configured in coverage of an NR cell, out of coverage, UTC synchronised or synchronised on the UE under test. The NR-SS-UE can also be configured for transmission/reception of an SLSS/PSBCH. The NULL ciphering and integrity algorithms are applied. L1, MAC, RLC, PDCP and SDAP are configured in normal way; they shall perform all of their functions. The NR_SL_DATA port for transmission and reception of STCH data is above SDAP. The STCH data is considered as raw data.

Editor’s note: MAC, RLC, PDCP and SDAP transparent modes are FFS

<TEXT SKIPPED >

###### 7.4.3.2.2.1	UE initiated PC5 unicast direct link communication

In general, the UE initiating PC5 unicast direct link communication procedure is done without activation time.

1.	NR-SS-UE is previously configured with Tx and Rx pools. SL-SRB0/1/2/3 are configured with UE layer-2 ID and NR-SS-UE layer-2 ID set to ‘AnyValue’.

2.	Receive DIRECT LINK ESTABLISHMENT REQUEST message on SL-SRB0.

NOTE 1:  The Source and destination Layer-2 IDs used to transmit the message are reported in the common part of NR_SL_SYSTEM_IND.

3.	Reconfigure SL-SRB1/2/3: set the UE layer-2 ID with the source Layer-2 ID received with the DIRECT LINK ESTABLISHMENT REQUEST message. Set the NR-SS-UE Id to a constant.

4.	Start ciphering and integrity on SL-SRB1/2/3.

5.	Send DIRECT LINK SECURITY MODE COMMNAND on SL-SRB1.

6.	Receive DIRECT LINK SECURITY MODE COMPLETE on SL-SRB1.

NOTE 2: Retrieve the PQFI from DIRECT LINK SECURITY MODE COMPLETE.

7.	Send DIRECT LINK ESTABLISHMENT ACCEPT on SL-SRB2.

8.	Receive RRCReconfigurationSidelink on SL-SRB3.

NOTE 3: Retrieve parameters to configure SL-DRB.

9.	Configure SL-DRB.

10.	Activate ciphering and integrity on SL-DRB.

11.	Send RRCReconfigurationCompleteSidelink on SL-SRB3.

###### 7.4.3.2.2.2	NR-SS-UE initiated PC5 unicast direct link communication

In general, the NR-SS-UE initiating PC5 unicast direct link communication procedure is done without activation time.

1.	NR-SS-UE is previously configured with Tx and Rx pools. SL-SRB0/1/2/3 configured with UE layer-2 ID set to the Destination layer-2 ID for unicast initial signalling (TS 38.508-1 [5] Table 4.7.5.5-42) and NR-SS-UE layer-2 ID set to a constant value.

2.	Start ciphering and integrity on SL-SRB1/2/3.

3.	Send DIRECT LINK ESTABLISHMENT REQUEST message on SL-SRB0.

4.	Receive DIRECT LINK SECURITY MODE COMMNAND on SL-SRB1.

NOTE 1: The Source Layer-2 ID and destination Layer-2 ID used to transmit the message is reported in the common part of NR_SL_SYSTEM_IND.

5.	Reconfigure SL-SRB1/2/3: set the UE layer-2 ID to the source Layer-2 ID used by UE to transmit the DIRECT LINK SECURITY MODE COMMNAND message.

6.	Send DIRECT LINK SECURITY MODE COMPLETE on SL-SRB1.

7.	Receive DIRECT LINK ESTABLISHMENT ACCEPT on SL-SRB2.

8.	Configure SL-DRB.

9.	Activate ciphering and integrity on SL-DRB.

10.	Send RRCReconfigurationSidelink on SL-SRB3.

11.	Receive RRCReconfigurationCompleteSidelink on SL-SRB3.

<TEXT SKIPPED>

## 9.2	E-UTRA and NR PIXIT

Table 9.2-1: E-UTRA and NR PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_ENDC_BandCombination | ENDC_BandCombination_Type | DC_1A_n28A |  | EN-DC Band Combination |
| px_ENDC_CA_BandCombination | ENDC_CA_BandCombination_Type | DC_1A_n28A_n78A |  | EN-DC CA Band Combination |
| px_ENDC_SecondaryBandCombination | ENDC_BandCombination_Type | DC_1A_n77A |  | Secondary EN-DC Band Combination |
| px_IPv4_Address_Other | charstring |  |  | Other IPv4 Address (see NOTE 4) |
| px_IPv6_Address_Other | charstring |  |  | Other IPv6 Address (see NOTE 4) |
| px_IPv4_Address4_NW | charstring |  |  | IPv4 Gateway Address in PDN4 |
| px_IPv6_Address4_NW | charstring |  |  | IPv6 Gateway Address in PDN4 |
| px_IPv4_Address4_UE | charstring |  |  | IPv4 Address connected to PDN4 |
| px_IPv6_Address4_UE | charstring |  |  | IPv6 Address connected to PDN4 |
| px_IPv4_Address5_NW | charstring |  |  | IPv4 Gateway Address in PDN5 |
| px_IPv6_Address5_NW | charstring |  |  | IPv6 Gateway Address in PDN5 |
| px_IPv4_Address5_UE | charstring |  |  | IPv4 Address connected to PDN5 |
| px_IPv6_Address5_UE | charstring |  |  | IPv6 Address connected to PDN5 |
| px_IPv4_Address6_NW | charstring |  |  | IPv4 Gateway Address in PDN6 |
| px_IPv6_Address6_NW | charstring |  |  | IPv6 Gateway Address in PDN6 |
| px_IPv4_Address6_UE | charstring |  |  | IPv4 Address connected to PDN6 |
| px_IPv6_Address6_UE | charstring |  |  | IPv6 Address connected to PDN6 |
| px_NEDC_BandCombination | NEDC_BandCombination_Type | DC_n1A_28A |  | NE-DC Band Combination |
| px_NR_CA_BandCombination | NR_CA_BandCombination_Type | CA_n3A_n77A |  | NR CA Band Combination |
| px_NR_DC_BandCombination | NR_DC_BandCombination_Type | DC_n78A_n257A |  | NR-DC Band Combination |
| px_NR_DC_CA_BandCombination | NR_DC_CA_BandCombination_Type | DC_n78A_n257G |  | NR-DC CA Band Combination |
| px_NR_CipheringAlgorithm | CipheringAlgorithm | nea2 |  | Ciphering Algorithm (see Note 1) |
| px_NR_IntegrityProtAlgorithm | IntegrityProtAlgorithm | nia2 |  | Integrity Algorithm (see Note 1) |
| px_NR_OverlappingNotSupportedBand_MFBI | integer | 1 |  | A not supported NR band that is overlapping with a supported band (px_NR_PrimaryBand). Applied to MFBI test case scenario. |
| px_NR_PrimaryBand | integer | 1 |  | NR primary band |
| px_NR_SecondaryBand | integer | 2 |  | NR secondary band. Applied to inter-band and SUL test cases. |
| px_NR_SidelinkBand | integer | 47 |  | NR sidelink band |
| px_NR_RATComb_Tested | NR_RATComb_Tested_Type | NR_UTRA | NR_UTRA, NR_GERAN | This parameter represents the network RAT capability / preference and indicates which, if any is supported, RAT combination is to be tested. |
| px_NR_V2X_ConcurrentOperationBandCombination | NR_V2X_ConcurrentOperationBandCombination_Type | V2X_n71_n47 |  | V2X operating bands for con-current operation |
| NOTE 1: Unless specified otherwise in the test case prose, the null algorithm shall not be used for verification. NOTE 2: Void. NOTE 3: Void. NOTE 4: IP address used for specific IP-address-related test requirements. The only requirements applying to this IP address is that its value shall be different from the values of all other IP address PIXITs (see TS 36.523-3 [12] clause 9) and it shall not be used within more than one PDN (PDU session) within a test case. In the present version of this specification, it is only used within PDN1 NW. |  |  |  |  |