3GPP TSG-RAN5 Meeting #95-e	R5-222441
Electronic Meeting, 9th May – 20th May 2022

Title:	Draft LS on ModifiedMPRbehaviour clarification for different power classes

Response to:

Release:	Release 16

Work Item:	NR_RF_FR2_req_enh-UEConTest

Source:	TSG RAN WG5

To:	TSG RAN WG4

Cc:	N/A

Contact Person:

#### Name:	Flores Fernández

E-mail Address:	flores_fernandez@keysight.com

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN5 has been discussing the introduction of Rel-16 FR2 MPR enhancements added in NR_RF_FR2_req_enh work item. Analysing Rel-16 core requirements, the following observations were made:

- Power class 2, 4 and 5 FR2 MPR requirements refer to power class 3 FR2 MPR core requirement.

- FR2 MPR enhancements requirements were added to power class 3 FR2 MPR core requirement (extension of Inner Region 1 and additional granularity in MPRnarrow requirements).

- Annex H.1 contains the definitions of the bits in the field modifiedMPRbehavior indicated per supported NR band in the IE RF-Parameters by a UE supporting an MPR or A-MPR modified in a given version of this specification.

  - Bit 0 for each band is defined for FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 v16.2.0.

  - This annex was firstly added in [1].

- modifiedMPRbehaviour is defined per band in 38.306.

- modifiedMPRbehaviour is used either in FR1 and in FR2

- modifiedMPRbehaviour bits have different meaning in FR1 (Annex L.1 in 38.101-1) and in FR2 (Annex H.1 in 38.101-2)

- The only reference to modifiedMPRbehaviour in 38.307 in Annex B.4.1 refers to FR1.

- modifiedMPRbehaviour is defined in Rel-15 signalling as BIT STRING (SIZE (8)) in 38.331 [11].

When defining test specification for FR2 MPR enhancements, RAN5 faced some concerns as listed below:

- [Are FR2 MPR enhancements defined subject to modifiedMPRbehavior bit 0 UE capability described in Annex H.1?]

- [Are FR2 MPR enhancements applicable to Rel-16 and onwards PC3 devices supporting modifiedMPRbehavior bit 0 UE capability described in Annex H.1 or is it also applicable to Rel-15 PC3 devices supporting modifiedMPRbehavior bit 0 UE capability?]

- Are FR2 MPR enhancements applicable to Power class 2, 4 and 5 devices?

- Is modifiedMPRbehavior bit 0 capability defined exclusively for power class 3 devices?

2. Actions:

To RAN4 group.

ACTION: 	RAN5 respectfully requests RAN4 group to provide feedback on the questions raised above.

3. Date of Next TSG-RAN WG5 Meetings:

TSG-RAN5 Meeting#96	 22nd – 26th August 2022	Toulouse, FR

TSG-RAN5 Meeting#97  14h – 18th November 2022	US, US

# References

- R4-2009169, “Indication of modified MPR behaviour for FR2 MPR enhancement”, Nokia, Nokia Shanghai Bell, RAN4#95, May 2020