3GPP TSG-RAN5 Meeting #95-e	R5-222641

# Electronic Meeting, 09th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2332 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of test frequency for NR SL concurrent |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To add missing test frequencies for NR SL concurrent operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Test frequencies for NR SL concurrent operation are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.8.2(new), 6.2.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.3.1.8	Test frequencies for NR Sidelink operating bands

4.3.1.8.1	Test frequencies for NR Sidelink operating bands in FR1

4.3.1.8.1.1	Reference test frequencies for NR Sidelink operating band n38

FFS

4.3.1.8.1.2	Reference test frequencies for NR Sidelink operating band n47

Table 4.3.1.8.1.2-1: Test frequencies for NR operating band n47 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | sl-absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | sl-absoluteFrequencySSB [ARFCN] (NOTE) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | S-SSB Low | S-SSB Mid | S-SSB High |
| 10 | 52 | Low | 5860.005 | 790667 | 5855.325 | 790355 | 0 | 790421 | 790673 | 790913 |
|  |  | Mid | 5889.99 | 792666 | 5794.59 | 786306 | 504 | 792420 | 792672 | 792912 |
|  |  | High | 5919.99 | 794666 | 5914.23 | 794282 | 6 | 794420 | 794672 | 794912 |
| 20 | 106 | Low | 5865 | 791000 | 5855.46 | 790364 | 0 | 790430 | 791006 | 791570 |
|  |  | Mid | 5889.99 | 792666 | 5789.73 | 785982 | 504 | 792096 | 792672 | 793236 |
|  |  | High | 5914.995 | 794333 | 5904.375 | 793625 | 6 | 793763 | 794339 | 794903 |
| 30 | 160 | Low | 5870.01 | 791334 | 5855.61 | 790374 | 0 | 790440 | 791340 | 792228 |
|  |  | Mid | 5889.99 | 792666 | 5784.87 | 785658 | 504 | 791772 | 792672 | 793560 |
|  |  | High | 5910 | 794000 | 5894.52 | 792968 | 6 | 793106 | 794006 | 794894 |
| 40 | 216 | Low | 5875.005 | 791667 | 5855.565 | 790371 | 0 | 790437 | 791673 | 792897 |
|  |  | Mid | 5889.99 | 792666 | 5779.83 | 785322 | 504 | 791436 | 792672 | 793896 |
|  |  | High | 5904.99 | 793666 | 5884.47 | 792298 | 6 | 792436 | 793672 | 794896 |
| NOTE: S-SSB Mid is the default S-SSB frequency unless explicitly indicated. |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.8.1.2-2: Test frequencies for NR operating band n47 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | sl-absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | sl-absoluteFrequencySSB [ARFCN] (NOTE) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | S-SSB Low | S-SSB Mid | S-SSB High |
| 10 | 24 | Low | 5860.005 | 790667 | 5855.685 | 790379 | 0 | 790511 | 790679 | 790823 |
|  |  | Mid | 5889.99 | 792666 | 5704.23 | 780282 | 504 | 792510 | 792678 | 792822 |
|  |  | High | 5919.99 | 794666 | 5913.51 | 794234 | 6 | 794510 | 794678 | 794822 |
| 20 | 51 | Low | 5865 | 791000 | 5855.82 | 790388 | 0 | 790520 | 791000 | 791480 |
|  |  | Mid | 5889.99 | 792666 | 5699.37 | 779958 | 504 | 792186 | 792666 | 793146 |
|  |  | High | 5914.995 | 794333 | 5903.655 | 793577 | 6 | 793853 | 794333 | 794813 |
| 30 | 78 | Low | 5870.01 | 791334 | 5855.97 | 790398 | 0 | 790530 | 791346 | 792138 |
|  |  | Mid | 5889.9 | 792666 | 5694.51 | 779634 | 504 | 791862 | 792678 | 793470 |
|  |  | High | 5910 | 794000 | 5893.8 | 792920 | 6 | 793196 | 794012 | 794804 |
| 40 | 106 | Low | 5875.005 | 791667 | 5855.925 | 790395 | 0 | 790527 | 791679 | 792807 |
|  |  | Mid | 5889.99 | 792666 | 5689.47 | 779298 | 504 | 791526 | 792678 | 793806 |
|  |  | High | 5904.99 | 793666 | 5883.75 | 792250 | 6 | 792526 | 793678 | 794806 |
| NOTE: S-SSB Mid is the default S-SSB frequency unless explicitly indicated. |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.8.1.2-3: Test frequencies for NR operating band n47 and SCS 60 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | sl-absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | sl-absoluteFrequencySSB [ARFCN] (NOTE) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | S-SSB Low | S-SSB Mid | S-SSB High |
| 10 | 11 | Low | 5860.005 | 790667 | 5856.045 | 790403 | 0 | 790667 | 790667 | 790667 |
|  |  | Mid | 5889.99 | 792666 | 5523.15 | 768210 | 504 | 792666 | 792666 | 792666 |
|  |  | High | 5919.99 | 794666 | 5911.71 | 794114 | 6 | 794666 | 794666 | 794666 |
| 20 | 24 | Low | 5865 | 791000 | 5856.36 | 790424 | 0 | 790688 | 791024 | 791312 |
|  |  | Mid | 5889.99 | 792666 | 5518.47 | 767898 | 504 | 792354 | 792690 | 792978 |
|  |  | High | 5914.995 | 794333 | 5902.035 | 793469 | 6 | 794021 | 794357 | 794645 |
| 30 | 38 | Low | 5870.01 | 791334 | 5856.33 | 790422 | 0 | 790686 | 791358 | 791982 |
|  |  | Mid | 5889.99 | 792666 | 5513.43 | 767562 | 504 | 792018 | 792690 | 793314 |
|  |  | High | 5910 | 794000 | 5892 | 792800 | 6 | 793352 | 794024 | 794648 |
| 40 | 51 | Low | 5875.005 | 791667 | 5856.645 | 790443 | 0 | 790707 | 791667 | 792627 |
|  |  | Mid | 5889.99 | 792666 | 5508.75 | 767250 | 504 | 791706 | 792666 | 793626 |
|  |  | High | 5904.99 | 793666 | 5882.31 | 792154 | 6 | 792706 | 793666 | 794626 |
| NOTE: S-SSB Mid is the default S-SSB frequency unless explicitly indicated. |  |  |  |  |  |  |  |  |  |  |

4.3.1.8.2	Test frequencies for concurrent NR sidelink operation

4.3.1.8.2.1	Reference test frequencies for Inter-band concurrent NR sidelink configurations

For inter-band concurrent NR sidelink configurations as listed in this clause, the following apply:

For the NR Uu band, test frequencies as specified in clause 4.3.1.1 for FR1 are used.

For the NR PC5 band, test frequencies as specified in clause 4.3.1.8.1 for FR1 are used.

Table 4.3.1.8.2.1-1: Test frequencies for Inter-band concurrent NR sidelink operating bands

| Inter-band concurrent NR sidelink configuration | Operating band for NR uplink | Operating band for NR downlink | Operating band for NR sidelink | Applicable for protocol testing |
| --- | --- | --- | --- | --- |
| V2X_n71-n47 | n71 | n71 | n47 | Yes |

<End of modified section 1>

<Start of modified section 2>

6.2.3.7	Test frequencies for NR sidelink configurations for signalling testing

The default channel bandwidths for NR sidelink signalling test are specified per NR sidelink operation band. The test frequencies are defined so that no frequency overlapping takes place, in order to avoid unnecessary inter-frequency interference.

For signalling test cases, the mapping of frequency ranges to NR sidelink test frequencies are as follows:

-	for band with only one test frequency: Low Range (NRf1);

-	for band with up to two test frequencies: Low Range (NRf1) and High Range (NRf2);

-	for band with up to three test frequencies: Low Range (NRf1), Mid Range (NRf2) and High Range (NRf3);

The test frequencies, subcarrier spacing, default channel bandwidth and S-SSBparameters for signalling is specified in Table 6.2.3.7-1 for PC5-only operations and Table 6.2.3.7-2 for concurrent operations..

Table 6.2.3.7-1: Test frequencies for NR Sidelink operating bands using 10 MHz channel bandwidth

| NR Band | SCS [kHz] | CBW [MHz] | carrierBandwidth [PRBs] | Range | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | sl-absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | sl-absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n47 | 15 | 10 | 52 | Low, Mid, High | Same values as for Low, Mid and High range in clause 4.3.1.8.1.2 for bandwidth=10 MHz and SCS=15 kHz. For sl-absoluteFrequencySSB, same value as for S-SSB Low in clause 4.3.1.8.1.2 for bandwidth=10 MHz and SCS=15 kHz is used. |  |  |  |  |  |

Table 6.2.3.7-2: Test frequencies for concurrent NR sidelink operating band configuration

| concurrent operating band configuration | NR Band | SCS [kHz] | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] Note 2 | offsetToCarrier [Carrier PRBs] | absoluteFrequencySSB [ARFCN] Note 3 | SS block SCS [kHz] | GSCN | kssb | Offset Carrier CORESET#0 [RBs] | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA(SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_n71A-n47A | n71 | 15 | 5 | 25 | Downlink | Low,High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Mid-Low | Same values as for Low and High, Mid-Low and Mid-High range in Table 6.2.3.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Mid-High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Uplink | Low,High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Mid-Low | Same values as for Low and High, Mid-Low and Mid-High range in Table 6.2.3.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Mid-High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n47 | 15 | 10 | 52 | Sidelink | Low |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Mid | Same values as for Low, Mid and High range in Table 6.2.3.7-1 |  |  |  |  |  | N/A |  |  |  |  |  |
|  |  |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: For NR downlink and uplink, absolute frequency of Point A is given by absoluteFrequencyPointA. For NR sidelink, absolute frequency of Point A is given by sl-absoluteFrequencyPointA. Note 3: For NR downlink and uplink, absolute frequency of SSB is given by absoluteFrequencySSB. For NR sidelink, absolute frequency of SL SSB is given by sl-absoluteFrequencySSB. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<End of modified section 1>