3GPP TSG-RAN5 Meeting #95-e	R5-222867

# Online, ,9th May 2022 - 20th May 2022

| Text Proposal |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.918 | CR | n/a | rev | - | Current version: | 0.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Text Proposal on Test Procedure A.3.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NR_Slice_Test |  |  |  |  |  | Date: |  |  | 2022-04-24 |
| Category: | n/a |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Test procedure of 5G NR / Service Performance / Multiple Applications with Single Network Slicing needs to be defined according to the work plan |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test procedure A.3.1.2 5G NR / Service Performance / Multiple Applications with Single Network Slicing |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TR 38.918 will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

A.3.1.2	5G NR / Service Performance / Multiple Applications with Single Network Slicing

A.3.1.2.1	Definition

The service performance of 5G NR UE with network slicing is determined by the UE operating system and communication unit.

A.3.1.2.2	Test Purpose

To measure the performance of multiple services in application layer of the 5G NR UE with single network slicing.

A.3.1.2.3	Test Parameters

FFS

A.3.1.2.4	Test Description

A.3.1.2.4.1	Initial Conditions

System Simulator:

-	NGC Cell A is configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in 38.508-1 [6].

UE:

-	Empty URSP Configuration

Preamble:

-	The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [6].

A.3.1.2.4.2	Test Procedure

1.	Steps 1 to 6 of the test procedure specified in subclause A.2.1.1.4.2 are performed except the  MANAGE UE POLICY COMMAND message is defined in Table A.3.2.1.4.2-2

2.	Using the Application Client Simulator to simulate APP A, generate traffic matching DNN value in Traffic descriptor in URSP rule 1 provisioned to the UE in step 1

3.	The UE transmits an UL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT REQUEST message. Observe the right S-NSSAI is selected.

4.	The SS transmits a DL NAS TRANSPORT message and PDU SESSION ESTABLISHMENT ACCEPT message.

5.	Using the simulated APP A, begin uplink data transfer from UE to the Application Server Simulator

6.	Wait for 15 seconds and then start to measure the throughput result in application layer(This is iteration 1). Continue data transfer for 1 minute.

7.	Repeat step 3 for 3 iterations within the same call as the first iteration. Wait for at least 5 seconds between each iteration of the data transfer

8.	Calculate and record the average application layer data throughput across three iterations.

9.	Using the Application Client Simulator to simulate APP B, generate traffic matching DNN value in Traffic descriptor in URSP rule 2 provisioned to the UE in step 1

10.	Using the simulated APP B, begin uplink data transfer from UE to the Application Server Simulator

11.	Repeat step 6-8 to get the average application layer data throughput of the data traffic of simulated APP A

12.	Observe that the throughput in step 11 is not lower than the throughput benchmark in step 8

Table A.3.1.2.4.2-1: MANAGE UE POLICY COMMAND (step 1)

| Derivation Path: Table 5.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UE policy part |  |  |  |  |
| URSP rule 1 |  |  |  |  |
| Precedence value of URSP rule |  | 0 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | ‘10100000’B | OS App Id type |  |
| Traffic descriptor component |  |  |  |  |
| OS App Id length |  | Set to the actual length of ' OS App Id value ' in bytes |  |  |
| OS App Id value |  | pc_OS_App_ID |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | ‘00000010’B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000010’B |  |  |
| SD |  | 0x000001 |  |  |
| URSP rule 2 |  |  |  |  |
| Precedence value of URSP rule |  | 1 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | ‘10100000’B | OS App Id type |  |
| Traffic descriptor component |  |  |  |  |
| OS App Id length |  | Set to the actual length of ' OS App Id value ' in bytes |  |  |
| OS App Id value |  | pc_OS_App_ID_2nd |  |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | ‘00000010’B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000010’B |  |  |
| SD |  | 0x000001 |  |  |
| URSP rule 3 |  |  |  |  |
| Precedence value of URSP rule |  | 2 |  |  |
| Traffic descriptor |  |  |  |  |
| Traffic descriptor component type identifier |  | ‘00000001’B | Match-all type |  |
| Route selection descriptor list |  |  |  |  |
| Route selection descriptor 1 |  |  |  |  |
| Precedence value of route selection descriptor |  | 0 |  |  |
| Route selection descriptor contents |  |  |  |  |
| Route selection descriptor component type |  | ‘00000010’B | S-NSSAI type |  |
| Route selection descriptor component |  |  |  |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000010’B |  |  |
| SD |  | 0x000002 |  |  |

Table A.3.1.2.4.2-2: UL NAS Transport (step 3)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Payload container type |  | ‘0001’B | N1 SM information |  |
| PDU session ID |  | PSI-1 |  |  |
| Request type |  | ‘001’B | Initial request |  |
| S-NSSAI |  | Not Present |  |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000010’B | 1 |  |
| SD |  | 0x000001 |  |  |

Table A.3.1.2.4.2-3: PDU SESSION ESTABLISHMENT ACCEPT (step 4)

| Derivation path: TS 38.508-1 clause 4.7.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| S-NSSAI |  |  |  |  |
| Length of S-NSSAI contents |  | ‘00000100’B | SST and SD |  |
| SST |  | ‘00000010’B | 1 |  |
| SD |  | 0x000001 |  |  |

<<< END OF CHANGES >>>