3GPP TSG-RAN5 Meeting #95-e	R5-222897

# Electronic Meeting, 9th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0542 | rev | - | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to URLLC test case 7.2.2.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test case 7.2.2.2.2 is still not completed yet. The test applicability is also not clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Fixing the open issues in 7.2.2.2.2. Clarifying the test applicability. The approval of this CR would depend on output of discussion paper R5-222891. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1_1, 7.2.2.2.2_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

7.1.1_1	Applicability of test requirements due to maximum achievable SNR

Editor’s note: The applicability of test requirements with a modulation above 64 QAM is FFS.

Table 7.1.1_1-1 specifies the current assumption of maximum testable SNRBB for indirect farfield (IFF), PC3, Max device size ≤ 30 cm under fading conditions.

Table 7.1.1_1-1: maximum testable SNRBB under fading conditions for modulations up to 64 QAM

| Operating Band / Frequency | Maximum testable SNRBB (dB) |  |  |
| --- | --- | --- | --- |
|  | CHBW 50 MHz | CHBW 100 MHz | CHBW 200 MHz |
| n257 mid | 30.6 | 27.5 | 24.4 |
| n258 mid | 30.6 | 27.5 | 24.4 |
| n259 mid | 20.4 | 17.2 | 14.1 |
| n260 mid | 24.4 | 21.2 | 18.2 |
| n261 mid | 30.6 | 27.5 | 24.4 |

Based on the current assumption of maximum testable SNRBB, the applicability of test points is defined in Table 7.1.1_1-2 for indirect farfield (IFF), PC3, Max device size size ≤ 30 cm under fading conditions.

Table 7.1.1_1-2: Testability of test requirements due to maximum achievable SNR per band

| Test Case | Test point | CHBW / MHz | Fading | SNR test requirement | Test Point Applicability |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | n257 | n258 | n259 | n260 | n261 |
| 7.2.2.2.1_1 | 1-1 | 100 | Yes | 1.4 | x | x | x | x | x |
|  | 1-2 | 100 | Yes | 3.6 | x | x | x | x | x |
|  | 1-3 | 100 | Yes | 14.2 | x | x | x | x | x |
|  | 2-1 | 100 | Yes | 5.8 | x | x | x | x | x |
|  | 2-2 | 100 | Yes | 16.0 | x | x | x | x | x |
|  | 2-3 | 50 | Yes | 15.7 | x | x | x | x | x |
|  | 2-4 | 200 | Yes | 15.8 | x | x | - | x | x |
|  | 2-5 | 50 | Yes | 16 | x | x | x | x | x |
|  | 2-6 | 100 | Yes | 20.3 | x | x | - | x | x |
| 7.2.2.2.1_2 | 3-1 | 100 | Yes | 20.7 | x | x | - | x | x |
| 7.2.2.2.1_3 | 1-4 | 50 | Yes | 22.0 | TBD | TBD | TBD | TBD | TBD |
| 7.2.2.2.2_1 | 1-1 | 100 | Yes | 0.7 | x | x | x | x | x |
| 7.2.2.2.3_1 | 1-1 | 100 | Yes | 3.1 | x | x | x | x | X |
| 7.3.2.2.1 | 1-1 | 100 | Yes | 7.7 | x | x | x | x | x |
|  | 1-2 | 100 | Yes | 4.3 | x | x | x | x | x |
| 7.3.2.2.2 | 2-1 | 100 | Yes | 3.2 | x | x | x | x | x |
|  | 2-2 | 100 | Yes | 0.2 | x | x | x | x | x |
| 7.3.2.2.3 | 1-1 | 100 | Yes | 4.7 | x | x | x | x | x |

<Unchanged Text Skipped>

7.2.2.2.2	2Rx TDD FR2 PDSCH repetitions over multiple slots

7.2.2.2.2_0	Minimum conformance requirements

For PDSCH with slot aggregation, the requirements are specified in Table 7.2.2.2.2_0-3, additional parameters in Table 7.2.2.2.2_0-2 and the downlink physical channel setup according to Annex C.5.1.

The test purpose is specified in Table 7.2.2.2.2_0-1.

Table 7.2.2.2.2_0-1: Test purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH repetitions over multiple slots performance under 2 receive antenna conditions | 1-1 |

Table 7.2.2.2.2_0-2: Test Parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 |
|  | PDSCH aggregation factor |  | 2 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Number of HARQ Processes |  |  | 2 |
| The number of slots between final repetition of PDSCH and corresponding HARQ-ACK information |  |  | As defined in Annex A.1.3 (Note 1) |
| Note 1: ACK/NACK feedback is generated for PDSCH on slot i, where mod(i,4) = 1, where i is the slot index per frame; i = {0,...,79} |  |  |  |

Table 7.2.2.2.2_0-3: Minimum performance for Rank 1 (FRC)

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Target BLER | SNR (dB) |
| 1-1 | R.PDSCH. 5-11.1 TDD | 100 / 120 | 16QAM, 0.37 | FR2.120-2 | TDLA30-75 | 2x2 ULA Low | 1% (Note 1) | -1.1 |
| Note 1: BLER is defined as residual BLER; i.e. ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block. |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 7.2.2.2.2.

7.2.2.2.2_1	2Rx TDD FR2 PDSCH repetitions over multiple slots - 2x2 MIMO with baseline receiver for SA and NSA

7.2.2.2.2_1.1	Test Purpose

Verify the PDSCH repetitions over multiple slots performance under 2 receive antenna conditions.

7.2.2.2.2_1.2	Test Applicability

This test applies to all types of NR UE release 16 and forward supporting capability IE pdsch-RepetitionMultiSlots-r16.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC supporting capability IE pdsch-RepetitionMultiSlots-r16.

7.2.2.2.2_1.3	Test Description

7.2.2.2.2_1.3.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.2-1 of TS 38.521-2 [8].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.2.

Only sub tests shall be tested which are testable according to Table 7.1.1_1-2.

For EN-DC within FR2 operation, setup the LTE radiated link according to Annex D:

1.	Connection between SS, the faders, AWGN noise source and the UE is shown in TS 38.508-1 [6] Annex A, Figure A.3.3.2 for TE diagram and Figure A.3.4.2 for UE diagram.

2.	The parameter settings for the NR cell are set up according to Table 7.2-1 and Table 7.2.2.2.2.0-2 and as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2, and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-2 [8].

4.	Propagation conditions for NR cell are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 7.2.2.2.2_1.3.3.

7.2.2.2.2_1.3.2	Test Procedure

1.	Set the UE in a direction that satisfies the 3 normative criteria specified in Annex H.0.If no direction found, mark the test as inconclusive.

2.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 7.2.2.2.2_1.4-1. The SS sends downlink MAC padding bits on the DL RMC. The UE may expect that the TB is repeated with same symbol allocation among each of the pdsch-AggregationFactor consecutive slots.

3.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 7.2.2.2.2_1.4-1 as appropriate.

4.	Measure the BLER for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of correctly and incorrectly received transport bloks based on ACK/NACK feedback on the UL during each subtest and decide pass or fail according to Tables G.1.5-1a in Annex G.

7.2.2.2.2_1.3.3	Message Contents

Message contents are according to TS 38.508-1 [6] subclause 4.6.1 and 5.4.2 with the following exceptions:

7.2.2.2.2_1.3.3_1	Message exceptions for SA

Table 7.2.2.2.2_1.3.3_1-1: SchedulingRequestResourceConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-157 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestResourceConfig ::= SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl80 | 7 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.2.2.2.2_1.3.3_1-2: PDSCH-Config

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-100 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| pdsch-AggregationFactor | 2 |  |  |
| } |  |  |  |

Table 7.2.2.2.2_1.3.3_1-3: PDSCH-ServingCellConfig

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-102 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| nrofHARQ-ProcessesForPDSCH | 4 |  |  |
| } |  |  |  |

7.2.2.2.2_1.3.3_2	Message exceptions for NSA

Same as 7.2.2.2.2_1.3.3_1.

7.2.2.2.2_1.4	Test Requirements

Table 7.2.2.2.2_1.4-1 defines the primary level settings.

The target BLER percentage for the downlink reference measurement channels specified in Annex A.3.2.2 for each BLER test shall meet or exceed the specified value in Table 7.2.2.2.2_1.4-1 for the specified SNR including test tolerances for all BLER tests.

Table 7.2.2.2.2_1.4-1: Test Requirement for Rank 1

| Test num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Target BLER | SNR (dB) |
| 1-1 | R.PDSCH. 5-11.1 TDD | 100 / 120 | 16QAM, 0.37 | FR2.120-2 | TDLA30-75 | 2x2 ULA Low | 1% (Note 1) | 0.7 |
| Note 1: BLER is defined as residual BLER; i.e. ratio of incorrectly received transport blocks / sent transport blocks, independently of the number HARQ transmission(s) for each transport block. |  |  |  |  |  |  |  |  |

<End of Changes >