3GPP TSG-RAN5 Meeting #95-e	R5-223306

# Electronic Meeting, 9th – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1500 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to default INVITE in A.2.1 for checking the absence of geolocation information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF 160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2022-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is no possibility to specify explicitly that geolocation information is not to be provided by the UE in INVITE request when such information is not available at the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Geolocation information was specified to be ‘Not present’ in INVITE request if geolocation information is not available to the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Prose specification will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Re-issued from R5-222389 because of title change. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## A.2.1	INVITE for MO Call Setup

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  |  |
| Method |  | INVITE |  | RFC 3261 [15] |
| Request-URI | NOT A5 AND NOT A32 | px_IMS_CalleeUri px_IMS_CalleeURI may be either SIP or Tel URI. It may contain a dialstring and phone-context parameter, when calling to dialstring. When calling to dialstring SIP URI must also contain user=phone or user=dialstring parameter. The dialstring, if used, may be global, home local number or geo-local number. For home local numbers the value of phone-context parameter must equal the home domain name i.e. px_IMS_HomeDomainName. For geo-local numbers the home domain name must be prefixed by string “geo-local.” or access technology specific prefix, if the UE supports that option. Note: The way how the UE determines whether numbers in a non-international format are geo-local, home-local or relating to another network, is UE implementation specific. For instance the UE might have a UI setting |  | TS 24.229 [10] cl 5.1.2A.1.3, 5.1.2A.1.5, 7.2A.10 |
|  | A5 OR A32 | px_IMS_CalleeContactUri |  |  |
|  | A6, A7 AND NOT (A20 OR A21) | emergency service URN beginning with urn:service:sos |  | RFC 5031 [97] |
|  | A20 AND (NOT A25) | urn:service:sos.ecall.manual | Rel-14 | RFC 8147 [149] |
|  | A21 AND (NOT A25) | urn:service:sos:ecall.automatic | Rel-14 | RFC 8147 [149] |
|  | A25 | The Test URI as per the generic “eCall test URI” which uses EFSDNURI from table 4.9.3.5-1 for “eCall capable” UEs or EFFDNURI from table 4.9.3.5-2 for “eCall only” UEs as specified in 3GPP TS 36.508 [94] | Rel-14 | RFC 8147 [149] |
| SIP-Version |  | SIP/2.0 |  |  |
| Via |  |  |  | RFC 3261 [15] |
| sent-protocol |  | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) |  |  |
| sent-by | A1,A7 | IP address or FQDN and protected server port of the UE |  |  |
|  | A2,A19 | IP address or FQDN, port (optional) and not checked |  |  |
|  | A6 | IP address and, when using UDP, unprotected server port of the UE |  |  |
|  | A17 | IP address and unprotected server port of the UE |  |  |
| response-port | A6 | rport (when using UDP) |  | RFC 3581 [96] |
| via-branch |  | value starting with ‘z9hG4bK’ |  |  |
| Route |  | order of the parameters in this header must be like in the respective rows |  | RFC 3261 [15] |
| route-param | A1 | <sip:SS P-CSCF address: protected server port of SS;lr>, <sip:scscf.3gpp.org;lr> |  |  |
|  | A2,A17 | <sip:SS P-CSCF address: unprotected server port of SS (optional);lr>, <sip:scscf.3gpp.orgf;lr |  |  |
|  | A5 OR A32 | MO call has been established: URIs of the Record-Route header of 183 response in reverse order (or any other response creating the dialog according to RFC 3261 clause 12.1 [15]) MT call has been established: same value as defined for the Record-Route header in A.2.9 |  |  |
|  | A6,A19 | <sip:SS P-CSCF address: unprotected server port of SS;lr> |  |  |
|  | A7 | <sip:SS P-CSCF address: protected server port of SS;lr> |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec | A6 | Any SIP URI with display name as “Anonymous” or anonymous |  |  |
|  | A7,A19 | emergency public user identity (NOTE 3) |  |  |
|  | A4 | any SIP URI being subscribed and registered as listed in the XML body of the NOTIFY request; additionally when there is a P-Preferred-Identity header within the INVITE request the SIP URI shall match the URI within the P-Preferred-Identity header |  |  |
| tag | A4 | must be present, value not checked |  |  |
| addr-spec | A5 OR A32 | local SIP URI of the UE as used in any previous request in the same dialog (In the earlier requests within the same dialog this URI appears in From header within requests sent by the UE and in To header within requests sent by the SS) |  |  |
| tag | A5 OR A32 | local tag of the dialog ID (In the earlier requests within the same dialog this tag appears in From header within requests sent by the UE and in To header within requests sent by the SS) |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec | A6, A7 AND NOT (A20 OR A21) | emergency service URN beginning as urn:service:sos |  | RFC 5031 [97] |
|  | A20 AND (NOT A25) | urn:service:sos.ecall.manual | Rel-14 | RFC 8147 [149] |
|  | A21 AND (NOT A25) | urn:service:sos:ecall:automatic | Rel-14 | RFC 8147 [149] |
|  | A25 | The Test URI as per the generic “eCall test URI” which uses EFSDNURI from table 4.9.3.5-1 for “eCall capable” UEs or EFFDNURI from table 4.9.3.5-2 for “eCall only” UEs as specified in 3GPP TS 36.508 [94] | Rel-14 | RFC 8147 [149] |
|  | A4 | px_IMS_CalleeUri |  |  |
| tag | A4 | not present |  |  |
| addr-spec | A5 OR A32 | remote SIP URI of SS (i.e. the remote UE) as used in any previous request in the same dialog (In the earlier requests within the same dialog this URI appears in To header within requests sent by the UE and in From header within requests sent by the SS) |  |  |
| tag | A5 OR A32 | remote tag of the dialog ID (In the earlier requests within the same dialog this tag appears in To header within requests sent by the UE and in From header within requests sent by the SS) |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid | A4 | value different to that received in REGISTER message |  |  |
|  | A5 OR A32 | value of Call-ID as in any previous request in the same dialog |  |  |
| Call-Info | A20,A21 |  | Rel-14 | RFC 8147 [149] |
| cid URL |  | any URL |  |  |
| purpose |  | EmergencyCallData.eCall.MSD |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value | A4 | must be present, value not checked |  |  |
|  | A5 OR A32 | value of CSeq sent by the UE within its previous request in the same dialog but increased by one |  |  |
| method |  | INVITE |  |  |
| Supported |  | The option tags defined below shall be included additionally to any option tags defined in any specific message content, unless specified otherwise in this specific message content. |  | RFC 3261 [15] |
| option-tag | A4 OR A5 | 100rel |  |  |
|  | A13 OR A14 | norefersub | Rel-11 | RFC 4488 [126] |
|  | A4 AND A26 AND NOT (A6 OR A7 OR A19 OR A20 OR A21) | timer |  | RFC 4028 [146] |
| P-Early-Media | A16 AND NOT (A5 OR A32) |  |  | RFC 5009 [138] IR.92 [133] |
| em-param |  | supported |  |  |
| Geolocation | A8 |  | Rel-9 | RFC 6442 [98] |
| locationURI |  |  |  | cid-url indicating the Content-Id of the PIDF-LO within the multipart MIME body of INVITE request. (Note that location-by-reference URI is not allowed as the SS does not provide any external storage for location info for the UE to refer.) |  |  |  |  |  |
| Geolocation |  | NOT A8 |  | not present |  | Rel-9 |  | RFC 6442 [98] |  |
| Geolocation-Routing | A8 | “yes” | Rel-9 | RFC 6442 [98] |
| Geolocation-Routing | NOT A8 | not present | Rel-9 | RFC 6442 [98] |
| Require | A1,A7 |  |  | RFC 3261 [15] |
|  | A6 | not present |  |  |
| option-tag | A1,A7 | sec-agree |  | RFC 3329 [21] |
| Proxy-Require | A1,A7 |  |  | RFC 3261 [15] RFC 3329 [21] |
|  | A6 | not present |  |  |
| option-tag | A1,A7 | sec-agree |  |  |
| Security-Verify | A1,A7 |  |  | RFC 3329 [21] |
|  | A2,A6 | not present |  |  |
| sec-mechanism | A1,A7 | same value as Security-Server header sent by SS |  |  |
| Contact |  |  |  | RFC 3261 [15] |
| addr-spec | (A1 OR A7) AND NOT A15 | SIP URI with IP address or FQDN and protected server port of UE |  |  |
|  | (A2 OR A19) AND NOT A15 | SIP URI with IP address or FQDN and unprotected server port of UE |  |  |
|  | A15 AND NOT A6 | Public GRUU as obtained during registration as pub-gruu contact parameter of the 200 OK for REGISTER response |  | RFC 5627 [61] |
|  | A6 | SIP URI with IP address and unprotected server port of UE |  |  |
| c-p-instance | A6 | +sip.instance="<urn:gsma:imei: (gsma-specifier-defined-substring)>” where gsma-specifier-defined-substring shall be the IMEI code of the UE, coded as specified in RFC 7254 [122], without optional parameters | Rel-10 | RFC 5626 [109] RFC 7254 [122] |
|  | A3,A17,A29 | +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mmtel" (see NOTE 2, 4) |  |  |
| feature-param | A10,A31 | video |  | RFC 3840 [63] |
|  | A12 | +g.3gpp.srvcc-alerting |  | RFC 3840 [63] |
|  | A30 OR (A22 AND (A23 OR A24)) | audio |  | RFC 3840 [63] |
|  | A18 AND NOT (A5 OR A32) | +g.3gpp.ps2cs-srvcc-orig-pre-alerting |  | RFC 3840 [63] |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | non-zero value |  |  |
| P-Access-Network-Info | NOT A2 |  |  | RFC 7315 [132] RFC 7913 [154] |
| access-net-spec | A1 AND A27 | access network information and, if applicable, the cell ID |  |  |
|  | A28 | access network information for NR, containing access-class parameter with value "3GPP-NR" or access-type parameter with value "3GPP-NR-FDD" or "3GPP-NR-TDD", and also containing the cell ID | Rel-15 |  |
| P-Access-Network-Info | A2(o) | header optional |  |  |
| access-net-spec |  | access network information and, if applicable, the cell ID |  |  |
| Accept | NOT (A5 OR A32) |  | Rel-7 | RFC 3261 [15] |
|  | A5(o) , A32(o) | header optional |  |  |
| media-range | A4 | application/sdp,application/3gpp-ims+xml (additional medias can be added in any order) |  |  |
|  | A13 | application/vnd.3gpp.mid-call+xml | Rel-11 |  |
|  | A14 | application/vnd.3gpp.state-and-event-info+xml | Rel-11 |  |
|  | A20,A21 | application/EmergencyCallData.Control+xml | Rel-14 | RFC 8147 [149] |
| P-Preferred-Service |  |  |  | RFC 6050 [68] |
| Service-ID | A3 AND A4 | urn:urn-7:3gpp-service.ims.icsi.mmtel |  |  |
| P-Preferred-Identity |  |  |  | RFC 3325 [89] |
| PPreferredID-value | A7 | emergency public user identity (NOTE 3) |  |  |
| Recv-Info |  |  |  | RFC 6086 [139] |
| Info-package-type | A14 | g.3gpp.state-and-event |  |  |
|  | A20,A21 | EmergencyCallData.eCall.MSD | Rel-14 | RFC 8147 [149] |
| Accept-Contact |  |  |  | RFC 3841 [64] |
| ac-value | A3 AND A4 | +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mmtel" (see NOTE 2, 4) |  | TS 24.173 [65] |
|  | A10 AND A11 | video |  | IR.94 [134] NG.114 [157] |
| Proxy-Authorization | A17 |  |  | RFC 2617 [16] RFC 3310 [17] |
| username | A17 | preconfigured in the UE |  |  |
| realm | A17 | same value as received in the realm directive in the Proxy-Authorization header sent by SS |  |  |
| nonce | A17 | same value as in Proxy-Authorization header sent by SS |  |  |
| digest-uri | A17 | preconfigured in the UE |  |  |
| qop-value | A17 | auth |  |  |
| cnonce-value | A17 | value assigned by UE affecting the response calculation |  |  |
| nonce-count | A17 | counter to indicate how many times UE has sent the same value of nonce within successive INVITESs, initial value shall be 1 |  |  |
| response | A17 | response calculated by UE |  |  |
| algorithm | A17 | MD5 |  |  |
| Content-Type |  |  |  | RFC 3261 [15] |
| media-type | NOT A8 AND NOT A20 AND NOT A21 AND NOT A25 | application/sdp |  |  |
|  | A8, A20, A21, A25 | multipart/mixed;boundary=any value |  | RFC 6442 [98] RFC 8147 [149] |
| Content-Length |  | header shall be present if UE uses TCP to send this message and if there is a message body |  | RFC 3261 [15] |
| Value |  | length of message-body |  |  |
| Message-body |  | consists of one or several parts as indicated by Content-Type, and each part having actual contents as follows (SDP contents, if any, is specified in dedicated sections) |  |  |
|  | A8 | a PIDF-LO element mapped to the same Content-ID which can be found from the Geolocation header The PIDF-LO shall contain at least the following elements: - One or more 'geopriv' elements, each containing: - One 'location-info' element describing the location of the UE; and - One 'usage-rules' element describing the limitations of the usage of the location info |  |  |
|  | A20 OR A21 OR A25 | --boundary value (as provided in SIP hdr Content-Type) Content-Type: application/EmergencyCallData.eCall.MSD Content-ID: same URL as in Call-Info header Content-Disposition: by-reference;handling=optional MSD in ASN.1 PER encoding --boundary value (as provided in SIP hdr Content-Type) | Rel-14 | RFC 8147 [149] |

| Condition |  |  | Explanation |  |  |
| --- | --- | --- | --- | --- | --- |
| A1 |  |  | IMS security (A.6a/2 3GPP TS 34.229-2 [5]) |  |  |
| A2 |  |  | GIBA (A.6a/1 3GPP TS 34.229-2 [5]) |  |  |
| A3 |  |  | UE supports MTSI (A.3A/50 3GPP TS 34.229-2 [5]) |  |  |
| A4 |  |  | INVITE creating a dialog |  |  |
| A5 |  |  | re-INVITE within a dialog for session modification |  |  |
| A6 |  |  | INVITE for creating an emergency session in case of no registration |  |  |
| A7 |  |  | INVITE for creating an emergency session within an emergency registration using IMS security |  |  |
| A8 |  |  | UE is capable of obtaining location information, has obtained its location and is setting up an emergency session |  |  |
| A9 |  |  | Void |  |  |
| A10 |  |  | UE supports video feature tag (A.12/32 3GPP TS 34.229-2 [5]) |  |  |
| A11 |  |  | INVITE for creating a video call |  |  |
| A12 |  |  | INVITE for creating a voice or video call and UE supports g.3gpp.srvcc-alerting media feature tag (A.12/34 3GPP TS 34.229-2 [5]) |  |  |
| A13 |  |  | INVITE for creating a voice call during rSRVCC and UE CS to PS SRVCC with the MSC assisted mid-call feature (A.12/42 3GPP TS 34.229-2 [5]) |  |  |
| A14 |  |  | INVITE for creating a voice call and UE supports CS to PS SRVCC for calls in alerting phase (A.12/41 3GPP TS 34.229-2 [5]) |  |  |
| A15 |  |  | obtaining and using GRUUs in the Session Initiation Protocol (SIP) (A.4/53 3GPP TS 34.229-2 [5]) |  |  |
| A16 |  |  | UE supports early media (A.12/45 3GPP TS 34.229-2 [5]) |  |  |
| A17 |  |  | SIP Digest without TLS for Fixed Broadband Access (SIP Digest without TLS, A.6a/5 3GPP TS 34.229-2 [5]) |  |  |
| A18 |  |  | UE indicates g.3gpp.ps2cs-srvcc-orig-pre-alerting media feature tag in INVITE request (A.12/36 3GPP TS 34.229-2 [5]) |  |  |
| A19 |  |  | INVITE for creating an emergency session within an emergency registration using GIBA |  |  |
| A20 |  |  | INVITE for creating an eCall over IMS session manually |  |  |
| A21 |  |  | INVITE for creating an eCall over IMS session automatically |  |  |
| A22 |  |  | UE supports audio media feature tag (A.12/56 3GPP TS 34.229-2 [5]) |  |  |
| A23 |  |  | UE uses E-UTRAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.301 [150], clauses 8.2.1 and 9.9.3.12A |  |  |
| A24 |  |  | UE uses UTRAN/GERAN access and has received IMS voice over PS Session Supported Indication in the NAS ATTACH ACCEPT message as described in TS 24.008 [12], clauses 9.4.2 and 10.5.5.23 |  |  |
| A25 |  |  | INVITE for creating a test eCall over IMS session |  |  |
| A26 |  |  | UE supports Session Timer (A.12/57 3GPP TS 34.229-2 [5]) |  |  |
| A27 |  |  | UE uses E-UTRAN access (A.18/1 3GPP TS 34.229-2 [5]) |  |  |
| A28 |  |  | UE uses NR access (A.18/5 3GPP TS 34.229-2 [5]) |  |  |
| A29 |  |  | UE includes mmtel ICSI value as per NG.114 V1.0 [157] (A.22/1 3GPP TS 34.229-2 [5]) |  |  |
| A30 |  |  | UE includes audio feature tag as per NG.114 V1.0 [157] (A.22/2 3GPP TS 34.229-2 [5]) |  |  |
| A31 |  |  | UE includes video feature tag as per NG.114 V1.0 [157] (A.22/3 3GPP TS 34.229-2 [5]) |  |  |
| A32 |  |  | re-INVITE within a dialog for purposes other than session modification |  |  |

NOTE 1:	All choices for applicable conditions are described for each header.

NOTE 2:	The “=” may include optional linear white spaces according to the EQUAL definition in chapter 25.1, RFC 3261 [15].

NOTE 3:	According to TS 24.229 clause 5.1.1.1A and 5.1.6.2 [10] when the UE is using ISIM the emergency public user identity is the first public user identity in the list stored in the ISIM; when there is no ISIM it is the default public user id if the UE registered or the temporary user id (derived from IMSI) else.

NOTE 4:	URN is the outcome of URL encoding (“Percent-Encoding” according to RFC 3986 [129]) of urn:urn-7:3gpp-service.ims.icsi.mmtel.