3GPP TSG-RAN5 Meeting #96-e	R5-224040

# Electronic Meeting, 15th Aug - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-2 | CR | 0237 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Condition C175 for TC7.1.1.3.2b in 38.523-1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SGS Wireless |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2022-07-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Condition C175 of 38.523-1 incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Condition C175 of 38.523-1 corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases couldn’t be excuted correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged sections skipped>

4.2	Protocol conformance test cases Applicability Condition

Table 4.2-1: Applicability of Protocol conformance test cases Conditions

| Condition | Test case Selection Expression | Comment |
| --- | --- | --- |
| C01 | IF A.4.1-3/2 THEN R ELSE N/A | UEs supporting EN-DC |
| C02 | IF (A.4.3.4-1/2 OR A.4.3.4-1/3) THEN R ELSE N/A | UEs supporting 5GS and RLC UM Mode |
| C03 | IF A.4.3.5-1/1 THEN R ELSE N/A | UEs supporting 5GS and Long DRX Cycle |
| C04 | IF A.4.3.5-1/2 THEN R ELSE N/A | UEs supporting 5GS and short DRX cycle |
| C05 | IF A.4.3.4-1/3 THEN R ELSE N/A | UEs supporting 5GS and RLC UM with 6-bit length of RLC sequence number |
| C06 | IF A.4.3.4-1/2 THEN R ELSE N/A | UEs supporting 5GS and RLC UM with 12-bit length of RLC sequence number |
| C07 | IF A.4.3.4-1/1 THEN R ELSE N/A | UEs supporting 5GS and RLC AM with 12-bit length of RLC sequence number |
| C08 | IF A.4.3.3-1/1 THEN R ELSE N/A | UEs supporting 5GS and 12-bit length of PDCP sequence number |
| C09 | IF [10] A.4.4-1/99 THEN R ELSE N/A | UEs supporting 5GS and ZUC Algorithm |
| C10 | IF A.4.1-3/2 AND A.4.3.7-1/2 THEN R ELSE N/A | UEs supporting EN-DC and UL transmission via both MCG path and SCG path for the split DRB |
| C11 | IF (A.4.3.2-1/2 OR A.4.3.2-1/3) THEN R ELSE N/A | UEs supporting 5GS and 256QAM for PDSCH for FR1/FR2 |
| C12 | IF (A.4.3.2-1/4) THEN R ELSE N/A | UEs supporting 5GS and 256QAM for PUSCH |
| C13 | IF A.4.1-3/2 AND A.4.3.6-1/1 THEN R ELSE N/A | UEs supporting EN-DC and NR measurements and Event A triggered reporting |
| C14 | IF A.4.1-3/2 AND A.4.3.6-1/1 AND A.4.3.6-1/3 THEN R ELSE N/A | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR Intra-frequency and NR-Inter frequency measurements and at least periodical reporting) |
| C15 | IF A.4.1-3/2 AND A.4.3.6-1/1 AND A.4.3.6-1/3 AND (A.4.3.6-1/4 OR A.4.3.6-1/40) THEN R ELSE N/A | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR Intra-frequency and Inter frequency measurements and at least periodical reporting) and CSI-RSRP and CSI-RSRQ measurement |
| C16 | IF A.4.1-3/2 AND [10] A.4.4-1/18 AND [10] A.4.4-1/19 THEN R ELSE N/A | UEs supporting EN-DC and UE requested bearer resource allocation and modification procedures |
| C17 | IF A.4.3.2-1/1 THEN R ELSE N/A | UEs supporting 5GS and PDSCH reception based on semi-persistent scheduling |
| C18 | IF A.4.3.2-1/10 THEN R ELSE N/A | UEs supporting 5GS and Type 1 PUSCH transmissions with configured grant |
| C19 | IF A.4.3.2-1/11 THEN R ELSE N/A | UEs supporting 5GS and Type 2 PUSCH transmissions with configured grant |
| C20 | IF A.4.3.2-1/12 THEN R ELSE N/A | UEs supporting 5GS and PDSCH aggregation |
| C21 | IF A.4.1-5/1 THEN R ELSE N/A | UEs supporting 5G Core |
| C21A | IF A.4.1-5/1 AND A.4.3.7-1/4 THEN R ELSE N/A | UEs supporting 5G Core and reflective QoS |
| C22 | IF A.4.1-3/2 AND A.4.3.7-1/3 THEN R ELSE N/A | UEs supporting EN-DC and SRB3 |
| C23 | IF A.4.1-3/2 AND A.4.3.7-1/3 AND A.4.3.7-1/1 THEN R ELSE N/A | UEs supporting EN-DC and SRB3 and (UL transmission via either MCG path or SCG path for the split SRB) |
| C24 | IF A.4.1-3/2 AND A.4.3.6-1/3 AND A.4.3.6-1/2 AND A.4.1-4/3 THEN R ELSE N/A | UEs supporting EN-DC and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) and (two independent measurement gap configurations for FR1 and FR2) and Inter-Band EN-DC within FR1 |
| C25 | IF A.4.1-3/2 AND A.4.3.6-1/3 AND A.4.3.6-1/2 AND A.4.1-4/4 THEN R ELSE N/A | UEs supporting EN-DC and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) and (two independent measurement gap configurations for FR1 and FR2) and Inter-Band EN-DC including FR2 |
| C26 | IF ([10] A.4.1-1/1 OR [10] A.4.1-1/2) THEN R ELSE N/A | UEs supporting 5GS and E-UTRA |
| C27 | IF A.4.1-5/1 AND A.4.3.6-1/1 THEN R ELSE N/A | UEs supporting 5G Core and NR measurements and Event A triggered reporting |
| C28 | IF A.4.3.2-1/13 THEN R ELSE N/A | UEs supporting 5GS and supplemental uplink with dynamic switch |
| C29 | IF A.4.1-5/2 AND [10] A.4.1-1/5 THEN R ELSE N/A | UEs supporting 5G core over non-3GPP Access Network and WLAN |
| C30 | IF A.4.1-5/2 AND A.4.3.7-1/6 AND [10] A.4.1-1/5 THEN R ELSE N/A | UEs supporting 5G core over non-3GPP Access Network and SMS over NAS and WLAN |
| C31 | IF A.4.1-5/1 AND A.4.3.6-1/5 THEN R ELSE N/A | UEs supporting 5G Core and Inter-RAT E-UTRA measurements and Event B triggered reporting |
| C32 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA |
| C33 | IF A.4.1-5/1 AND A.4.3.7-1/6 AND NOT [10] A.4.4-2/32 THEN R ELSE N/A | UEs supporting 5G Core and SMS over NAS and UE configured to not use SMSoIP |
| C34 | IF A.4.1-5/1 AND [10] A.4.4-1/84 THEN R ELSE N/A | UEs supporting 5G Core and MinimumPeriodicSearchTimer |
| C35 | IF A.4.1-5/1 AND (A.4.3.7-1/8 OR A.4.3.7-1/7) THEN R ELSE N/A | UEs supporting 5G Core and (ETWS reception or CMAS reception) |
| C36 | IF A.4.1-5/1 AND [10] A.4.4-1/69 THEN R ELSE N/A | UEs supporting 5G Core and user initiated PLMN reselection in automatic mode on NR |
| C37 | IF A.4.1-5/1 AND (A.4.1-2/1 OR A.4.1-2/2) THEN R ELSE N/A | UEs supporting 5G Core and more than 1 FDD or TDD NR band |
| C38 | IF A.4.1-5/1 AND A.4.1-1/1 AND A.4.1-1/2 THEN R ELSE N/A | UEs supporting 5G Core and NR FDD and NR TDD |
| C39 | IF A.4.1-5/1 AND A.4.3.7-1/9 THEN R ELSE N/A | UEs supporting 5G Core and additional UE-requested PDU establishment |
| C40 | IF A.4.1-5/1 AND A.4.3.6-1/6 THEN R ELSE N/A | UEs supporting 5G Core and SS-SINR measurements |
| C41 | IF A.4.1-5/1 AND (A.4.1-4A/1 OR A.4.1-4A/3) THEN R ELSE N/A | UEs supporting 5G Core and intra-band contiguous CA |
| C42 | IF A.4.1-5/1 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) THEN R ELSE N/A | UEs supporting 5G Core and inter-band CA |
| C43 | IF A.4.1-5/1 AND (A.4.1-4A/2 OR A.4.1-4A/4) THEN R ELSE N/A | UEs supporting 5G Core and intra-band non-contiguous CA |
| C44 | IF (A.4.1-4A/1 OR A.4.1.4A/3) THEN R ELSE N/A | UEs supporting 5GS and intra-band contiguous CA |
| C45 | IF (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) THEN R ELSE N/A | UEs supporting 5GS and inter-band CA |
| C46 | IF (A.4.1-4A/2 OR A.4.1.4A/4) THEN R ELSE N/A | UEs supporting 5GS and intra-band non-contiguous CA |
| C47 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.2.1.1-1/4 AND A.4.3.7-1/11 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and Emergency Services Fallback in NR connected to 5GCN |
| C48 | Void |  |
| C49 | IF A.4.1-5/1 AND A.4.3.6-1/2 THEN R ELSE N/A | UE supporting 5G Core and two independent measurement gap configurations for FR1 and FR2 |
| C50 | IF A.4.1-5/1 AND A.4.3.6-1/5 AND A.4.3.6-1/42 THEN R ELSE N/A | UEs supporting 5G Core and Inter-RAT E-UTRA measurements and Event B triggered reporting and E-UTRA RS-SINR measurements |
| C51 | IF A.4.3.2-1/21 THEN R ELSE N/A | UEs supporting 5GS and PUSCH aggregation |
| C52 | IF A.4.1-5/1 AND A.4.3.6-1/1 AND A.4.3.6-1/3 AND (A.4.3.6-1/4 OR A.4.3.6-1/40) THEN R ELSE N/A | UEs supporting 5G Core and NR measurements and Event A triggered reporting and (NR Intra-frequency and Inter frequency measurements and at least periodical reporting) and CSI-RSRP and CSI-RSRQ measurement |
| C53 | IF A.4.3.5-1/4 THEN R ELSE N/A | UEs supporting 5GS and Logical Channel SR-Delay Timer |
| C54 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.4-1/33 AND A.4.3.7-1/12 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback |
| C55 | IF A.4.1-3/2 AND A.4.3.6-1/1 AND (A.4.1-4A/1 OR A.4.1-4A/3) THEN R ELSE N/A | UEs supporting EN-DC and NR measurements and Event A triggered reporting and intra-band contiguous CA |
| C56 | IF A.4.1-3/2 AND A.4.3.6-1/1 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) THEN R ELSE N/A | UEs supporting EN-DC and NR measurements and Event A triggered reporting and inter-band CA |
| C57 | IF A.4.1-3/2 AND A.4.3.6-1/1 AND (A.4.1-4A/2 OR A.4.1-4A/4) THEN R ELSE N/A | UEs supporting EN-DC and NR measurements and Event A triggered reporting and intra-band non-contiguous CA |
| C58 | IF A.4.1-5/2 AND [10] A.4.1-1/5.AND A.4.4-1/1 | UEs supporting 5G core over non-3GPP Access Network, WLAN and (ICMP or ICMP IPv6) |
| C59 | IF A.4.1-5/1 AND A.4.3.6-1/8 THEN R ELSE N/A | UEs supporting 5G Core and Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when EN-DC is not configured. |
| C60 | IF A.4.1-5/1 AND A.4.3.6-1/7 THEN R ELSE N/A | UEs supporting 5G Core and Support acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the EN-DC is not configured. |
| C61 | IF A.4.1-3/2 AND A.4.3.3-1/6 THEN R ELSE N/A | UEs supporting EN-DC and PDCP duplication over split SRB1/2 |
| C62 | IF A.4.1-3/2 AND A.4.3.3-1/4 THEN R ELSE N/A | UEs supporting EN-DC and PDCP duplication over split DRB |
| C63 | IF A.4.1-5/1 AND A.4.3.7-1/13 THEN R ELSE N/A | UEs supporting 5G Core and UE requested PDU session modification procedure |
| C64 | IF A.4.3.2-1/23 THEN R ELSE N/A | UEs supporting 5GS and The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE doesn’t support MIMO on this carrier |
| C65 | IF A.4.3.2-1/23 AND (A.4.3.2-1/4) THEN R ELSE N/A | UEs supporting 5GS and The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE doesn’t support MIMO on this carrier |
| C66 | IF (A.4.3.2-1/24 OR A.4.3.2-1/24A) AND (A.4.3.2-1/24 OR A.4.3.2-1/24A) THEN R ELSE N/A | UEs supporting 5GS and (DCI and timer based active BWP switching delay type1 or type2) and (Support of BWP adaptation up to 2 or up to 4) |
| C67 | IF A.4.1-3/2 AND (A.4.1-4A/1 OR A.4.1-4A/3) THEN R ELSE N/A | UEs supporting EN-DC and Intra-Band Contiguous CA |
| C68 | IF A.4.1-3/2 AND (A.4.1-4A/2 OR A.4.1-4A/4) THEN R ELSE N/A | UEs supporting EN-DC and Intra-Band Non-Contiguous CA |
| C69 | IF A.4.1-3/2 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) THEN R ELSE N/A | UEs supporting EN-DC and Inter-Band CA |
| C70 | IF A.4.3.5-1/1 AND A.4.3.5-1/2 THEN R ELSE N/A | UEs supporting 5GS and Long DRX Cycle and Short DRX Cycle |
| C71 | IF A.4.1-3/2 AND A.4.3.7-1/3 AND A.4.3.6-1/3 THEN R ELSE N/A | UEs supporting EN-DC and SRB3 and NR intra-frequency and inter-frequency measurements and at least periodical reporting |
| C72 | IF A.4.1-5/1 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A | UEs supporting 5G Core and intra-band contiguous CA and CA-based PDCP duplication over MCG or SCG DRB anf UL NR CA with 2 carriers |
| C73 | IF A.4.1-5/1 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A | UEs supporting 5G Core and inter-band contiguous CA and CA-based PDCP duplication over MCG or SCG DRB anf UL NR CA with 2 carriers |
| C74 | IF A.4.1-5/1 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A | UEs supporting 5G Core and intra-band non-contiguous CA and CA-based PDCP duplication over MCG or SCG DRB anf UL NR CA with 2 carriers |
| C75 | IF A.4.1-3/2 AND A.4.3.7-1/3 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A | UEs supporting EN-DC and SRB3 and intra-band contiguous CA and CA-based PDCP duplication over MCG or SCG DRB anf UL NR CA with 2 carriers |
| C76 | IF A.4.1-3/2 AND A.4.3.7-1/3 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A | UEs supporting EN-DC and SRB3 and inter-band CA and CA-based PDCP duplication over MCG or SCG DRB anf UL NR CA with 2 carriers |
| C77 | IF A.4.1-3/2 AND A.4.3.7-1/3 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A | UEs supporting EN-DC and SRB3 and intra-band non-contiguous CA and CA-based PDCP duplication over MCG or SCG DRB anf UL NR CA with 2 carriers |
| C78 | IF A.4.1-5/1 AND [9] A.3A/50 AND [9] A.4/2B AND [9] A.15/1 AND [9] A.3A/61 THEN R ELSE N/A | UEs supporting 5G Core and Initiating session and MTSI speech and SMS over IP |
| C79 | IF A.4.1-5/1 AND [9] A.3A/50 AND [9] A.4/2B AND [9] A.15/3 THEN R ELSE N/A | UEs supporting 5G Core and Initiating session and MTSI video |
| C80 | IF A.4.1-4/6 THEN R ELSE N/A | UEs supporting NR-DC |
| C81 | IF (A.4.1-4A/1 OR A.4.1.4A/3) AND A.4.3.2A.1-2/1 THEN R ELSE N/A | UEs supporting 5GS and intra-band contiguous CA and UL NR CA with 2 carriers |
| C82 | IF (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) AND A.4.3.2A.1-2/1 THEN R ELSE N/A | UEs supporting 5GS and inter-band CA and UL NR CA with 2 carriers |
| C83 | IF (A.4.1-4A/2 OR A.4.1.4A/4) AND A.4.3.2A.1-2/1 THEN R ELSE N/A | UEs supporting 5GS and intra-band non-contiguous CA and UL NR CA with 2 carriers |
| C84 | IF A.4.1-5/1 AND [10] A.4.4-1/99 THEN R ELSE N/A | UEs supporting 5G Core and ZUC algorithm |
| C85 | Void |  |
| C85A | IF (A.4.1-5/1 AND A.4.4-2/9) AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.2.1.1-1/4 AND A.4.3.7-1/14 THEN R ELSE N/A | UEs supporting 5G core and Emergency PDN connection transfer from S1 mode to N1 mode when network does not support N26 interface, and, E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and emergency services in NR connected to 5GCN |
| C86 | IF A.4.1-4/6 AND A.4.3.7-1/3 THEN R ELSE N/A | UEs supporting NR-DC and SRB3 |
| C87 | IF A.4.1-4/6 AND A.4.3.7-1/3 AND A.4.3.6-1/3 THEN R ELSE N/A | UEs supporting NR-DC and SRB3 and NR intra-frequency and inter-frequency measurements and at least periodical reporting |
| C88 | IF A.4.1-4/6 AND A.4.3.7-1/3 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A | UEs supporting NR-DC and SRB3 and intra-band contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |
| C89 | IF A.4.1-4/6 AND A.4.3.7-1/3 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A | UEs supporting NR-DC and SRB3 and inter-band CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |
| C90 | IF A.4.1-4/6 AND A.4.3.7-1/3 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.3-1/3 AND A.4.3.2A.1-2/1 THEN R ELSE N/A | UEs supporting NR-DC and SRB3 and intra-band non-contiguous CA and CA-based PDCP duplication over MCG or SCG DRB and UL NR CA with 2 carriers |
| C91 | IF A.4.1-5/1 AND [10] A.4.4-1/98 THEN R ELSE N/A | UEs supporting 5G Core and ManualModeNetworkSelectionException |
| C92 | IF A.4.1-5/1 AND A.4.3.7-1/14 THEN R ELSE N/A | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| C93 | IF A.4.1-3/2 AND A.4.3.6-1/1 AND A.4.3.6-1/3 AND (A.4.1-2/1 OR A.4.1-2/2 OR (A.4.1-1/1 AND A.4.1-1/2)) THEN R ELSE N/A | UEs supporting EN-DC and NR measurements and Event A triggered reporting and (NR Intra-frequency and NR-Inter frequency measurements and at least periodical reporting) and multiple NR bands. |
| C94 | IF A.4.1-5/1 AND (A.4.1-2/1 OR A.4.1-2/2 OR (A.4.1-1/1 AND A.4.1-1/2)) THEN R ELSE N/A | UEs supporting 5G Core and multiple NR bands |
| C95 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.4-1/33 AND A.4.3.7-1/12 AND A.4.3.7-1/15 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback and voiceFallbackIndication |
| C96 | IF A.4.1-5/1 AND A.4.1-3/2 AND A.4.3.8-1/10 THEN R ELSE N/A | UEs supporting 5G Core and EN-DC and inter-RAT Handover from NR to EN-DC |
| C97 | IF A.4.1-4/6 AND A.4.3.7-1/2 THEN R ELSE N/A | UEs supporting NR-DC and UL transmission via both MCG path and SCG path for the split DRB |
| C98 | IF A.4.1-4/6 AND A.4.3.3-1/4 THEN R ELSE N/A | UEs supporting NR-DC and PDCP duplication over split DRB |
| C99 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND (A.4.3.8-1/6 OR A.4.3.8-1/7 OR A.4.3.8-1/8)THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and (inter-RAT Handover to NR FR1 TDD from EUTRA connected to EPC or inter-RAT Handover to NR FR1 FDD from EUTRA connected to EPC or inter-RAT Handover to NR FR2 TDD from EUTRA connected to EPC) |
| C100 | IF A.4.1-5/1 AND [9] A.15/1 AND A.4.3.5-1/9 THEN R ELSE N/A | UEs supporting 5G Core and MTSI speech and bit rate recommendation query message |
| C101 | IF A.4.1-5/1 AND A.4.3.8-1/9 THEN R ELSE N/A | UEs supporting 5G Core and intra-frequency DAPS handover |
| C102 | IF A.4.3.2-1/30 THEN R ELSE N/A | UEs supporting 5GS and cross slot scheduling |
| C103 | IF A.4.3.5-1/1 AND A.4.3.5-1/5 THEN R ELSE N/A | UEs supporting 5GS and Long DRX Cycle and DRX adaptation |
| C104 | IF (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.2A.1-1/2 AND A.4.3.2A.1-2/2 AND A.4.3.3-1/5 THEN R ELSE N/A | UEs supporting 5GC and Intra-band contiguous CA and DL and UL NR CA with 3 carriers and PDCP duplication with more than two RLC entities |
| C105 | IF (A.4.3.4-1/2 OR A.4.3.4-1/3) AND A.4.3.3-1/7 THEN R ELSE N/A | UEs supporting 5GS and RLC UM mode and PDCP ethernet header compression |
| C106 | IF A.4.1-5/1 AND A.4.3.10-1/1 THEN R ELSE N/A | UE supporting 5G core and NR sidelink mode 1 transmission |
| C107 | IF A.4.3.2-1/32 THEN R ELSE N/A | UE’s supporting multi-DCI based multi-TRP |
| C108 | IF A.4.1-5/1 AND A.4.3.7-1/17 THEN R ELSE N/A | UEs supporting 5G Core and RACS |
| C109 | IF A.4.1-5/1 AND A.4.3.7-1/19 THEN R ELSE N/A | UEs supporting 5G Core and RRC_INACTIVE |
| C110 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.3.7-1/19 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and RRC_INACTIVE |
| C111 | IF A.4.1-5/1 AND (A.4.3.7-1/8 OR A.4.3.7-1/7) AND A.4.3.7-1/19 THEN R ELSE N/A | UEs supporting 5G Core and (ETWS reception or CMAS reception) and RRC_INACTIVE |
| C112 | Void |  |
| C113 | IF A.4.1-5/1 AND A.4.3.2/1 THEN R ELSE N/A | UEs 5GS and PDSCH reception based on multiple semi-persistent scheduling |
| C114 | IF A.4.1-5/1 AND A.4.3.5-1/6 THEN R ELSE N/A | UEs supporting 5GS and LCH-based UL grant prioritization |
| C115 | IF A.4.1-5/1 AND A.4.3.8-1/11 THEN R ELSE N/A | UEs supporting 5G Core and conditional handover |
| C116 | IF A.4.1-5/1 AND A.4.3.8-1/11 AND A.4.3.8-1/13 THEN R ELSE N/A | UEs supporting 5G Core and conditional handover and supporting 2 trigger events for same execution condition |
| C117 | IF A.4.1-5/1 AND A.4.3.8-1/11 AND A.4.3.8-1/12 THEN R ELSE N/A | UEs supporting 5G Core and conditional handover and conditional handover during re-establishment procedure when the selected cell is configured as candidate cell for condition handover |
| C118 | IF A.4.3.5-1/1 AND A.4.3.5-1/5 AND A.4.3.2-1/35 AND (A.4.1-4A/1 OR A.4.1-4A/3) THEN R ELSE N/A | UEs supporting 5GS and Long DRX Cycle and DRX adaptation and SCell Dormancy indication outside active time and intra-band contiguous CA |
| C119 | IF A.4.3.5-1/1 AND A.4.3.5-1/5 AND A.4.3.2-1/35 AND (A.4.1-4A/2 OR A.4.1-4A/4) THEN R ELSE N/A | UEs supporting 5GS and Long DRX Cycle and DRX adaptation and SCell Dormancy indication outside active time and intra-band non-contiguous CA |
| C120 | IF A.4.3.5-1/1 AND A.4.3.5-1/5 AND A.4.3.2-1/35 AND (A.4.1-4A/5 OR A.4.1-4A/6 OR A.4.1-4A/7) THEN R ELSE N/A | UEs supporting 5GS and Long DRX Cycle and DRX adaptation and SCell Dormancy indication outside active time and inter-band CA |
| C121 | Void |  |
| C122 | IF A.4.1-5/1 AND A.4.4-1/5 THEN R ELSE N/A | UEs supporting 5G Core and UL PDCP Packet Delay per DRB |
| C123 | IF A.4.1-5/1 AND A.4.4-1/6 THEN R ELSE N/A | UEs supporting 5G core and logged measurements in RRC_IDLE and RRC_INACTIVE. |
| C124 | IF A.4.1-5/1 AND A.4.4-1/4 AND A.4.4-1/6 THEN R ELSE N/A | UEs supporting 5G core and logged measurements in RRC_IDLE and RRC_INACTIVE and equipped with a GNSS receiver to provide detailed location information |
| C125 | IF A.4.1-5/1 AND A.4.4-1/6 AND A.4.3.7-1/19 THEN R ELSE N/A | UEs supporting 5G core and RRC_INACTIVE and logged measurements in RRC_IDLE and RRC_INACTIVE. |
| C126 | IF A.4.1-5/1 AND A.4.4-1/4 THEN R ELSE N/A | UEs supporting 5G Core and equipped with a GNSS or A-GNSS receiver to provide detailed location information. |
| C127 | IF A.4.1-5/1 AND [10] A.4.1-1/6 AND A.4.3.8-1/11 THEN R ELSE N/A | UEs supporting 5G Core and UTRA and NR to UTRA-FDD CELL_DCH CS handover |
| C128 | IF A.4.1-5/1 AND A.4.1-1/3 THEN R ELSE N/A | UE supporting 5G core and NR sidelink |
| C129 | IF A.4.1-5/1 AND A.4.3.7-1/18 AND A.4.3.7-1/25 THEN R ELSE N/A | UEs supporting 5G Core and RRC message Segmentation in the UL and Support of test function for using a preconfigured UE capability container over NR |
| C130 | IF A.4.1-5/1 AND A.4.3.8-1/15 THEN R ELSE N/A | UEs supporting 5G Core and inter-frequency DAPS handover |
| C131 | IF A.4.1-5/1 AND A.4.3.7-1/24 THEN R ELSE N/A | UEs supporting 5G Core and SNPN |
| C132 | IF A.4.1-5/1 AND A.4.3.7-1/23 THEN R ELSE N/A | UEs supporting 5G Core and CAG |
| C133 | IF A.4.1-5/1 AND A.4.3.7-1/21 THEN R ELSE N/A | UEs supporting 5G Core and RRC connection release with Deprioritisation |
| C134 | IF A.4.3.2-1/45 THEN R ELSE N/A | UEs supporting PUSCH repetition type B |
| C135 | IF A.4.3.2-1/46 THEN R ELSE N/A | UEs supporting 2-Step RACH |
| C136 | IF A.4.1-5/1 AND A.4.4-1/3 THEN R ELSE N/A | UEs supporting 5G Core and delivery of rachReport upon request from the network. |
| C137 | IF A.4.1-5/1 AND A.4.4-1/12 THEN R ELSE N/A | UEs supporting 5G core and Bluetooth measurements in RRC_IDLE and RRC_INACTIVE state |
| C138 | IF A.4.1-5/1 AND A.4.4-1/13 THEN R ELSE N/A | UEs supporting 5G core and WLAN measurements in RRC_IDLE and RRC_INACTIVE state |
| C139 | IF A.4.1-5/1 AND (A.4.4-1/7 OR A.4.4-1/8 OR A.4.4-1/9) THEN R ELSE N/A | UEs supporting 5G Core and collection of sensor information such as Barometric pressure, UE speed, and UE orientation information as defined in TS 37.355. |
| C140 | IF A.4.1-5/1 AND A.4.4-1/10 THEN R ELSE N/A | UEs supporting 5G core and Bluetooth Measurement Collection in Immediate MDT |
| C141 | IF A.4.1-5/1 AND A.4.4-1/11 THEN R ELSE N/A | UEs supporting 5G core and WLAN Measurement Collection in Immediate MDT |
| C142 | IF A.4.1-5/1 AND A.4.3.5-1/10 THEN R ELSE N/A | UEs supporting 5GS and PUSCH transmissions on multiple configured uplink grants |
| C143 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.4-1/4 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and standalone GNSS receiver to provide detailed location information |
| C144 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.4-1/6 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and logged measurements in RRC_IDLE and RRC_INACTIVE |
| C145 | IF A.4.1-5/1 AND A.4.3.7-1/29 THEN R ELSE N/A | UEs supporting 5G Core and release preference assistance information |
| C146 | IF A.4.3.2-1/52 THEN R ELSE N/A | UEs supporting monitoring DCI format 1_2 for DL scheduling and monitoring DCI format 0_2 for UL scheduling |
| C147 | IF A.4.1-5/1 AND A.4.3.7-1/26 AND A.4.3.7-1/27 THEN R ELSE N/A | UEs supporting 5G Core and NSSAA and EAP-AKA’ for NSSAA |
| C148 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.3.7-1/21 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and RRC connection release with Deprioritisation |
| C149 | IF A.4.1-4/6 AND A.4.3.6-1/2 THEN R ELSE N/A | UEs supporting NR-DC and two independent measurement gap configurations for FR1 and FR2 |
| C150 | IF A.4.1-5/1 AND (A.4.3.6-1/48 OR A.4.3.6-1/49) THEN R ELSE N/A | UEs supporting 5G Core and SFTD measurements between NR PCell and NR neighbour cell |
| C151 | IF A.4.1-3/2 AND (A.4.3.6-1/43 OR A.4.3.6-1/44) AND (A.4.3.6-1/46 OR A.4.3.6-1/47) THEN R ELSE N/A | UEs supporting EN-DC and SFTD measurement between E-UTRA PCell and an NR neighbour cell, and SFTD measurement between E-UTRA PCell and NR PSCell |
| C152 | IF A.4.1-4/6 AND (A.4.3.6-1/48 OR A.4.3.6-1/49) AND (A.4.3.6-1/50 OR A.4.3.6-1/51) THEN R ELSE N/A | UEs supporting NR-DC and SFTD measurement between NR PCell and an NR neighbour cell, and SFTD measurement between NR PCell and NR PSCell |
| C153 | IF A.4.1-3/2 AND A.4.3.8-1/19 THEN R ELSE N/A | UEs supporting EN-DC and conditional PSCell change |
| C154 | IF A.4.1-5/1 AND (A.4.1-4A/1 OR A.4.1-4A/3) AND A.4.3.7-1/19 THEN R ELSE N/A | UEs supporting 5G Core and intra-band contiguous CA and RRC_INACTIVE |
| C155 | IF A.4.1-5/1 AND (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.7-1/19 THEN R ELSE N/A | UEs supporting 5G Core and intra-band non-contiguous CA and RRC_INACTIVE |
| C156 | IF A.4.1-5/1 AND (A.4.1-4A/5 OR A.4.1-4A/6) AND A.4.3.7-1/19 THEN R ELSE N/A | UEs supporting 5G Core and inter-band CA and RRC_INACTIVE |
| C157 | IF A.4.1-4/6 AND A.4.3.7-1/3 AND A.4.3.7-1/1 THEN R ELSE N/A | UEs supporting NR-DC and SRB3 and (UL transmission via either MCG path or SCG path for the split SRB) |
| C158 | IF A.4.1-5/1 AND A.4.1-4/6 AND A.4.3.7-1/19 THEN R ELSE N/A | UEs supporting 5G Core and NR-DC and RRC_INACTIVE |
| C159 | IF A.4.1-5/2 AND [10] A.4.1-1/5 THEN R ELSE N/A | UEs supporting 5G core over non-3GPP Access Network and WLAN and additional UE-requested PDU establishment |
| C160 | IF A.4.1-3/3 THEN R ELSE N/A | UEs supporting NE-DC |
| C161 | IF A.4.1-5/1 AND A.4.3.7-1/21 AND [10] A.4.4-1/98 THEN R ELSE N/A | UEs supporting 5G Core and RRC connection release with Deprioritisation and ManualModeNetworkSelectionException |
| C162 | IF A.4.1-5/1 AND [9] A.22/8 AND A.4.3.7-1/36 AND [9] A.3A/50 AND [9] A.15/1 THEN R ELSE N/A | UEs supporting 5G Core and NG.114 v1.0 default configuration voice exempt and 3GPP PS data off and Initiating session and MTSI speech |
| C162A | IF A.4.1-5/1 AND [9] A.22/8 AND A.4.3.7-1/36 AND [9] A.3A/50 AND [9] A.3A/61 THEN R ELSE N/A | UEs supporting 5G Core and NG.114 v1.0 default configuration voice exempt and 3GPP PS data off and Initiating session and SMS over IP |
| C163 | IF A.4.1-5/1 AND A.4.1-1/3 AND A.4.3.10-1/3 THEN R ELSE N/A | UE supporting 5G core and NR sidelink and Sidelink CSI report |
| C164 | IF A.4.1-5/1 AND A.4.3.10-1/1 AND A.4.3.10-1/3 THEN R ELSE N/A | UE supporting 5G core and NR sidelink mode 1 transmission and Sidelink CSI report |
| C165 | IF A.4.1-5/1 AND A.4.3.7-1/33 THEN R ELSE N/A | UE supporting 5G Core and V2X communication |
| C166 | IF A.4.1-5/1 AND A.4.3.7-1/34 THEN R ELSE N/A | UE supporting 5G Core and V2X communication over NR-PC5 |
| C167 | IF A.4.1-5/1 AND A.4.3.7-1/24 AND A.4.3.7-1/30 THEN R ELSE N/A | UEs supporting 5G Core and SNPN and user initiated SNPN reselection in automatic mode on NR |
| C168 | IF A.4.1-5/1 AND A.4.3.7-1/23 AND A.4.3.7-1/31 THEN R ELSE N/A | UEs supporting 5G Core and CAG and Autonomous search function on NR |
| C169 | IF A.4.1-5/1 AND A.4.3.7-1/23 AND A.4.3.7-1/52 THEN R ELSE N/A | UEs supporting 5G Core and CAG and acquisition of CGI information from neighbour NR NPN cell |
| C170 | IF A.4.1-5/1 AND [10](A.4.1-1/1 OR A.4.1-1/2) AND [9]A.12/64 AND [11]A.10/16 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation |
| C171 | IF A.4.1-5/1 AND [10](A.4.1-1/1 OR A.4.1-1/2) AND [9]A.12/64 AND [11]A.10/16 AND [11]A.10/19 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation and capable of triggering a Test eCall |
| C172 | IF A.4.1-5/1 AND [9] A.22/9 AND A.4.3.7-1/36 AND [9] A.3A/50 AND [9] A.15/3 THEN R ELSE N/A | UEs supporting 5G Core and NG.114 v2.0 default configuration video exempt and 3GPP PS data off and Initiating session and MTSI video |
| C173 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [9] A.21/2 | UEs supporting 5G Core and E-UTRA and NG.114 v2.0 |
| C174 | IF A.4.1-5/1 AND [9]A.12/64 AND [11]A.10/16 AND [11]A.10/19 THEN R ELSE N/A | UEs supporting 5G Core and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation and capable of triggering a Test eCall |
| C175 | IF A.4.3.5-1/11 THEN R ELSE N/A | UEs supporting 5GS and selection of logical channels for each UL grant based on RRC configured restriction |
| C176 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND [10] A.4.2.1.1-1/4 THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") |
| C177 | IF A.4.1-5/1 AND A.4.3.7-1/17 AND A.4.3.7-1/35 THEN R ELSE N/A | UEs supporting 5G Core and RACS and Manufacturer assigned Radio Capability ID |
| C178 | IF A.4.1-5/1 AND [10](A.4.1-1/1 OR A.4.1-1/2) AND A.4.3.7-1/17 AND [10]A.4.4-1/215THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and RACS |
| C179 | IF A.4.3.2-1/80 THEN R ELSE N/A | UEs supporting DCI DL Priority Indicator |
| C180 | IF A.4.3.2-1/81 AND A.4.3.2-1/82 THEN R ELSE N/A | UEs supporting DCI UL Priority Indicator and LCH grant prioritisation |
| C181 | IF (A.4.1-4A/2 OR A.4.1-4A/4) AND A.4.3.2A.1-1/2 AND A.4.3.2A.1-2/2 AND A.4.3.3-1/5 THEN R ELSE N/A | UEs supporting 5GC and Intra-band non-contiguous CA and DL and UL NR CA with 3 carriers and PDCP duplication with more than two RLC entities |
| C182 | IF A.4.1-3/3 AND A.4.3.6-1/1 AND A.4.3.6-1/3 THEN R ELSE N/A | UEs supporting NE-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting). |
| C183 | IF A.4.1-3/3 AND A.4.3.6-1/1 AND A.4.3.6-1/3 AND (A.4.1-2/1 OR A.4.1-2/2 OR (A.4.1-1/1 AND A.4.1-1/2)) THEN R ELSE N/A | UEs supporting NE-DC and NR measurements and Event A triggered reporting and (NR intra-frequency and inter-frequency measurements and at least periodical reporting) and multiple NR bands. |
| C184 | IF A.4.1-5/1 AND [9]A.12/64 AND [11]A.10/17 THEN R ELSE N/A | UEs supporting 5G Core and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| C185 | IF A.4.1-5/1 AND [9]A.12/63 AND [11]A.10/17 AND A.4.3.7-1/14 THEN R ELSE N/A | UEs supporting 5G Core and IMS eCall type of emergency services over 5GS and Automatic type of eCall initiation and emergency services in NR connected to 5GCN |
| C186 | IF A.4.1-5/1 AND [10]A.4.1-1/6 AND [9]A.12/64 AND [11]A.10/16 AND A.4.3.8-1/14 THEN R ELSE N/A | UEs supporting 5G Core and UTRA and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation and NR to UTRA-FDD CELL_DCH CS handover |
| C187 | IF A.4.1-5/1 AND ([10]A.4.1-1/6 OR [10]A.4.1-1/7) AND [9]A.12/64 AND [11]A.10/16 THEN R ELSE N/A | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation |
| C188 | IF A.4.1-5/1 AND ([10]A.4.1-1/6 OR [10]A.4.1-1/7) AND [9]A.12/64 AND [11]A.10/17 THEN R ELSE N/A | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| C189 | IF A.4.1-5/1 AND ([10]A.4.1-1/6 OR [10]A.4.1-1/7) AND [9]A.12/63 AND [11]A.10/16 THEN R ELSE N/A | UEs supporting 5G Core and (UTRA OR GERAN) and eCall type of emergency services over 5GS and Manual type of eCall initiation |
| C190 | IF A.4.1-5/1 AND A.4.3.6-1/aa THEN R ELSE N/A | UEs supporting 5G Core and Idle/Inactive Measurements |
| C191 | IF A.4.1-5/1 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.3.6-1/bb THEN R ELSE N/A | UEs supporting 5G Core and E-UTRA and Idle/Inactive Measurements |
| C192 | IF A.4.1-5/1 AND A.4.3.7-1/19 AND A.4.3.6-1/aa THEN R ELSE N/A | UEs supporting 5G Core and RRC_INACTIVE and Idle/Inactive Measurements |
| C193 | IF A.4.1-5/1 AND A.4.3.7-1/19 AND ([10] A.4.1-1/1 OR [10] A.4.1-1/2) AND A.4.3.6-1/bb THEN R ELSE N/A | UEs supporting 5G Core and RRC_INACTIVE and E-UTRA and Idle/Inactive Measurements |
| C194 | IF A.4.1-3/3 AND A.4.3.7-1/2 THEN R ELSE N/A | UEs supporting NE-DC and UL transmission via both MCG path and SCG path for the split DRB |

<Unchanged sections skipped>