3GPP TSG-RAN5 Meeting #96-e	R5-224070
Electronic Meeting, 15th August – 26th August 2022

Title: 	Discussion on Rel-16 Power Boost measurement uncertainties and test tolerances

Source:	        Keysight Technologies UK Ltd

Agenda Item:	5.3.15.8

Document for:	Discussion and Endorsement

# 1.	Introduction

As part of the Rel-16 FR2 RF enhancements, RAN4 has defined a mechanism to enable UL power boosting in exchange for suspended In-Band Emissions requirements ([1], [2], [3], [4]).

[5] provides an analysis on how to introduce such core requirements into test specifications ([6]).

[7], [8], [9] provides CRs to 38.521-2 ([6]) and 38.905 ([10]) proposing concrete changes in test specifications to enable testing on Rel-16 FR2 RF enhancements for Power Boost.

This document makes a brief analysis on how measurement uncertainties and test tolerances could be defined for the new test proposed in [7] (6.2.4_1 Configured transmitted power with Power Boost).

# 2.	Discussion

According to [7], measurements performed in new test case 6.2.4_1 Configured transmitted power with Power Boost will be very similar to the ones performed for EIRP in test case 6.2.1.1 maximum output power- EIRP and TRP. The main difference is that the minimum EIRP power value measured now will be ΔPIBE=1.0 dB higher than in test 6.2.1.1. This implies that noise impact will be reduced when compared to noise impact of test case 6.2.1.1.

Proposal 1: Define measurement uncertainties for new test case 6.2.4_1 Configured transmitted power with Power Boost similar to the ones defined for 6.2.1.1 maximum output power- EIRP and TRP, except for the noise impact where it should be considered that SNR available will be now ΔPIBE=1.0 dB higher.

On the other hand, test tolerances formula for new test case 6.2.4_1 Configured transmitted power with Power Boost can be leveraged from the one defined for 6.2.1.1 maximum output power- EIRP and TRP, i.e. TT = 0.60 x (MTSU - 0.1).

Proposal 2: Define test tolerances for new test case 6.2.4_1 Configured transmitted power with Power Boost as TT = 0.60 x (MTSU – influence of noise).

# 3. 	Conclusion

This document makes a brief analysis on how measurement uncertainties and test tolerances could be defined for the new test proposed in [7] (6.2.4_1 Configured transmitted power with Power Boost) and makes the following proposals:

Proposal 1: Define measurement uncertainties for new test case 6.2.4_1 Configured transmitted power with Power Boost similar to the ones defined for 6.2.1.1 maximum output power- EIRP and TRP, except for the noise impact where it should be considered that SNR available will be now ΔPIBE=1.0 dB higher.

Proposal 2: Define test tolerances for new test case 6.2.4_1 Configured transmitted power with Power Boost as TT = 0.60 x (MTSU - 0.1).

All above proposals are implemented in [7], [11] and [12].

Similar approach has been followed for corresponding EN-DC test case in [13].

# 4.	References

[1] 3GPP TS 38.101-2 v16.12.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 16)

[2] R4-2011905, “CR to 38.101-2: FR2 UE EIRP increase with IBE relaxation”, Qualcomm, Nokia, AT&T, T-Mobile, Verizon, RAN4#96-e

[3] R4-1912975, “WF on UL Power Boost and IBE Relaxation”, Qualcomm, Skyworks, RAN4#92-bis, Chongqing, China

[4] R4-2006782, “FR2 UE EIRP increase with IBE relaxation”, Qualcomm, Nokia, AT&T, T-Mobile, Verizon, RAN4#95-e.

[5] R5-224046, “Discussion on Rel-16 Power Boost test definition”, Keysight Technologies UK, RAN5#96-e

[6] 3GPP TS 38.521-2 v16.12.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone

[7] R5-224071, “CR to 38.521-2 on New test case addition: 6.2.4_1 Configured transmitted power with Power Boost”, Keysight Technologies UK, RAN5#96-e.

[8] R5-224072, “CR to 38.521-2: In-band emissions minimum conformance requirements update”, Keysight Technologies UK, RAN5#96-e.

[9] R5-224073, “CR to 38.905: Test point analysis for new test 6.2.4_1 Configured transmitted power with Power Boost”, Keysight Technologies UK, RAN5#96-e.

[10] 3GPP TS 38.905 v17.5.0 Derivation of test points for radio transmission and reception User Equipment (UE) conformance test cases

[11] R5-224076, “CR to 38.521-2 on Measurement uncertainties and test tolerances for New test case addition: 6.2.4_1 Configured transmitted power with Power Boost”, Keysight Technologies UK, RAN5#96-e.

[12] R5-224075, “CR to 38.903 on Measurement uncertainties for New test case addition: 6.2.4_1 Configured transmitted power with Power Boost”, Keysight Technologies UK, RAN5#96-e.

[13] R5-224077, “CR to 38.521-3 on Measurement uncertainties and test tolerances for New test case addition: 6.2B.4.1.4_1 Configured Output Power with Power Boost for Inter-Band EN-DC including FR2 (1 NR CC)”, Keysight Technologies UK, RAN5#96-e.