3GPP TSG-RAN5 Meeting #96-e	R5-224109
Electronic Meeting, 15th August – 26th August 2022

Title: 	Discussion on measurement uncertainties for enhanced beam correspondence

Source:	        Keysight Technologies UK Ltd

Agenda Item:	5.3.15.8

Document for:	Discussion and Endorsement

# 1.	Introduction

In RAN5#95-e, enhanced beam correspondence core requirements part of FR2 RF Enhancement work item was added into TS 38.521-2 ([1]) via CR in [2].

Missing aspects to complete this test case are the definition of the measurement uncertainties and test tolerances.

This document makes proposals to define measurement uncertainties and test tolerances for this test case.

# 2.	Discussion

According to current TS 38.521-2 specification ([1]):

- Enhanced beam correspondence test procedure is similar to Rel-15 beam correspondence test procedure, except for the configuration of some initial conditions depending on UE capabilities.

- Enhanced beam correspondence test requirements are similar to Rel-15 beam correspondence test requirements

On the other hand, when Rel-15 beam correspondence measurement uncertainties were defined in RAN5#94-e ([3] and [4]), inputs considered were the test procedure and the test requirements. Initial conditions were assumed not to have a major impact on measurement uncertainties.

Similarly, test tolerances were defined as TT = 0.60 x (MTSUIFF - ΔSNRmr), being ΔSNRmr: Systematic offset due to noise when measuring at minimum requirement level

Hence, it seems reasonable to define Enhanced beam correspondence test measurement uncertainties and test tolerances equal to those for Rel-15 Beam correspondence test.

Proposal 1: Define Enhanced beam correspondence test measurement uncertainties and test tolerances equal to those for Rel-15 Beam correspondence test.

The same reasoning applies to the 2 new test cases introduced in [2]:

- 6.2.1.1_1 	UE maximum output power - EIRP and TRP (Rel16 and forward), whose title should be updated to remove TRP

- 6.2.1.2_1 	UE maximum output power - Spherical coverage (Rel16 and forward)

Proposal 2: Define 6.2.1.1_1 UE maximum output power - EIRP (Rel16 and forward) test measurement uncertainties and test tolerances equal to 6.2.1.1 UE maximum output power (EIRP).

Proposal 3: Define 6.2.1.2_1 UE maximum output power – Spherical coverage (Rel16 and forward) test measurement uncertainties and test tolerances equal to 6.2.1.2 UE maximum output power (Spherical coverage).

Proposal 4: Update 6.2.1.1_1 test title to remove TRP.

Proposal 5: Remove editor’s notes related to PC3 MU and TT for test cases 6.2.1.1_1, 6.2.1.2_1 and 6.6.2.

# 3. 	Conclusion

This document makes proposals to define measurement uncertainties and test tolerances for enhanced beam correspondence test. The following proposals are made:

Proposal 1: Define Enhanced beam correspondence test measurement uncertainties and test tolerances equal to those for Rel-15 Beam correspondence test.

Proposal 2: Define 6.2.1.1_1 UE maximum output power - EIRP (Rel16 and forward) test measurement uncertainties and test tolerances equal to 6.2.1.1 UE maximum output power (EIRP).

Proposal 3: Define 6.2.1.2_1 UE maximum output power – Spherical coverage (Rel16 and forward) test measurement uncertainties and test tolerances equal to 6.2.1.2 UE maximum output power (Spherical coverage).

Proposal 4: Update 6.2.1.1_1 test title to remove TRP.

Proposal 5: Remove editor’s notes related to PC3 MU and TT for test cases 6.2.1.1_1, 6.2.1.2_1 and 6.6.2.

These proposals are implemented in [5].

# 4.	References

- 3GPP TS 38.521-2 v16.12.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone

- R5-223750, “FR2 Enhanced Beam Correspondence test updates”, Apple Portugal, RAN5#95-e

- R5-221629, “38.903 Beam correspondence Measurement Uncertainties”, Keysight Technologies UK Ltd, RAN5#94-e

- R5-221657, “38.521-2 Beam correspondence Measurement Uncertainties and test tolerances”, Keysight Technologies UK Ltd, RAN5#94-e

- R5-224110, “38.521-2 Enhanced Beam correspondence Measurement Uncertainties and test tolerances”, Keysight Technologies UK Ltd, RAN5#96-e