3GPP TSG-RAN5 Meeting #96-e	R5-224178

# Electronic meeting, 15 – 26 Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0354 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of configurations for Inter-band NR-DC within FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Configurations for NR-DC within FR1 Physical Layer Baseline Implementation Capabilities are missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Configurations for NR-DC Physical Layer Baseline Implementation Capabilities within FR1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inter-band NR-DC within FR1 Physical Layer Baseline Implementation Capabilities will not be specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.2B.1.0a.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

### A.4.3.2B	NR-DC, EN-DC and NE-DC Physical Layer Baseline Implementation Capabilities

#### A.4.3.2B.1	NR-DC Physical Layer Baseline Implementation Capabilities

#### A.4.3.2B.1.0	General NR-DC capabilities

Table A.4.3.2B.1.0-1: Downlink NR-DC capabilities (for one or more of the supported NR-DC configurations)

| Item | Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | DL NR-DC with 2 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_2CC |  |
| 2 | DL NR-DC with 3 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_3CC |  |
| 3 | DL NR-DC with 4 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_4CC |  |
| 4 | DL NR-DC with 5 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_5CC |  |
| 5 | DL NR-DC with 6 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_6CC |  |
| 6 | DL NR-DC with 7 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_7CC |  |
| 7 | DL NR-DC with 8 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_8CC |  |
| 8 | DL NR-DC with 9 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_9CC |  |
| 9 | DL NR-DC with 10 carriers | 38.101-3, 5.5B | pc_DL_NR_DC_10CC |  |

Table A.4.3.2B.1.0-2: Uplink NR-DC capabilities (for one or more of the supported NR-DC configurations)

| Item | Bandwidth Class | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | UL NR-DC with 2 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_2CC |  |
| 2 | UL NR-DC with 3 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_3CC |  |
| 3 | UL NR-DC with 4 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_4CC |  |
| 4 | UL NR-DC with 5 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_5CC |  |
| 5 | UL NR-DC with 6 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_6CC |  |
| 6 | UL NR-DC with 7 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_7CC |  |
| 7 | UL NR-DC with 8 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_8CC |  |
| 8 | UL NR-DC with 9 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_9CC |  |
| 9 | UL NR-DC with 10 carriers | 38.101-3, 5.5B | pc_UL_NR_DC_10CC |  |

#### A.4.3.2B.1.0a	NR-DC within FR1

##### A.4.3.2B.1.0a.1	NR-DC within FR1 (two bands)

Table A.4.3.2B.1.0a.1-1: Downlink NR-DC Bandwidth Class Combination capabilities within FR1 and two bands (for one or more of the supported DC configurations in Table A.4.3.2B.1.0a.1-3)

| Item | DL NR-DC FR1 Bandwidth Class (two bands) | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | DL NR-DC FR1 BW Class Combination A-A (two bands) | 38.101-1, 5.5B | pc_DL_NR_DC_FR1_2B_Class_A-A |  |

Table A.4.3.2B.1.0a.1-2: Uplink NR-DC Bandwidth Class Combination capabilities within FR1 and two bands (for one or more of the supported DC configurations in Table A.4.3.2B.1.0a.1-3)

| Item | UL NR-DC FR1 Bandwidth Class (two bands) | Ref. | Mnemonic | Comments |
| --- | --- | --- | --- | --- |
| 1 | UL NR-DC FR1 BW Class Combination A-A (two bands) | 38.101-1, 5.5B | pc_UL_NR_DC_FR1_2B_Class_A-A |  |

Table A.4.3.2B.1.0a.1-3: Supported NR-DC configurations within FR1 (two bands)

| NR FR1 Inter-band NR-DC configuration / Item (Note 1, 6) | Release | Supported | Supported NR-DC Bandwidth Class(es) in UL (Note 2,3) | Supported Bandwidth Combination Set(s) | Supported ULTxSwitching Band Pair (Note 4, 5) |
| --- | --- | --- | --- | --- | --- |
| DC_n48A_n70A | Rel-17 |  |  |  |  |
| Note 1: Notation used for inter-band NR-DC Bands is according to TS 38.101-1 [23] Table 5.5B.1-1, e.g. ‘DC_n2A-n48C’ indicates NR-DC operation on NR band n2 and n48 with DL CA Bandwidth Class A and C respectively. Note 2: The UL NR-DC capabilities as per Table A.4.3.2B.1.0a.1-2 can be supported on a single or multiple CA Band(s). The UE supplier shall indicate all supported UL NR-DC Bandwidth Class(es), in uplink of the supported NR-DC Band(s), as per TS 38.101-1 [23] Table 5.5B.1-1. For this release of specification valid choices are ’N’, ‘nXA-nYA’, ‘nX(2A)’, ‘nXB’ and ‘nXC’, where both nX and nY are the NR bands. For example, for DC_n48A-n70A, ‘N’ would mean only DL NR_DC, ‘n48A-n70A’ would mean both DL and UL NR-DC. Note 3: See UL(table_index) in Note 1 of Table 4.0-3 and UL_nCC(table_index) in Note 2 of Table 4.0-3 in TS 38.522 [9]. Note 4: The ULTxSwitching capability can be reported on inter-band NR-DC band combinations. The UE supplier shall indicate inter-band NR-DC band pairs on which it supports ULTxSwitching. For this release of specification valid choices are ’N’ and ‘nX-nY’, where both nX and nY are NR bands. For example, for DC_n48A-n70A, ‘N’ would mean not supporting ULTxSwitching, ‘n48-n70’ would mean supporting of ULTxSwitching on this band pair. If UE supplier indicates supporting of ULTxSwitching on a band pair, they shall indicate at least one inter-band UL NR-DC configuration on the same band pair in the column “Supported NR-DC Bandwidth Class(es) in UL”. The ULTxSwitching is only tested with 2 UL CCs, so UE is allowed to report ‘N’ by default for NR-DC configuration with > 2 component carriers. Note 5: See ULTxSwitching(table_index) Note 6 of Table 4.0-3 in TS 38.522 [9]. Note 6: See DL_nCC(table_index) in Note 4 of Table 4.0-3 in TS 38.522 [9]. |  |  |  |  |  |

-------- End of changes --------