3GPP TSG-RAN5 Meeting #96-e	R5-224235

# Electronic Meeting, 15th – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2443 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of the styles of the subclauses in 4.3.1.1 and 4.3.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The styles of the subclauses in 4.3.1.1 and 4.3.1.2 are not consistent, some are visible in the navigation pane while some are not. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The styles of some subclauses in 4.3.1.1 and 4.3.1.2 were corrected to make them visible in the navigation pane. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The styles of the subclauses in 4.3.1.1 and 4.3.1.2 will remain inconsistent and be inconvinient for the spec reader to navigate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1, 4.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

4.3.1.1	Test frequencies for NR operating bands in FR1

4.3.1.1.1	NR operating bands in FR1

4.3.1.1.1.1	Reference test frequencies for NR operating band n1

Table 4.3.1.1.1.1-1: Test frequencies for NR operating band n1 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2112.5 | 422500 | 2110.25 | 422050 | 0 | 15 | 5279 | 422410 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2140 | 428000 | 2119.39 | 423878 | 102 |  | 5350 | 427970 | 8 | 1 | 0 (0) | 103 |
|  |  |  | High | 2167.5 | 433500 | 2074.53 | 414906 | 504 |  | 5418 | 433470 | 8 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 1922.5 | 384500 | 1920.25 | 384050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1857.03 | 371406 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1977.5 | 395500 | 1974.17 | 394834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 2115 | 423000 | 2110.32 | 422064 | 0 | 15 | 5280 | 422430 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2140 | 428000 | 2116.96 | 423392 | 102 |  | 5344 | 427490 | 10 | 1 | 0 (0) | 103 |
|  |  |  | High | 2165 | 433000 | 2069.6 | 413920 | 504 |  | 5405 | 432490 | 10 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 1925 | 385000 | 1920.32 | 384064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1854.6 | 370920 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1975 | 395000 | 1969.24 | 393848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 2117.5 | 423500 | 2110.39 | 422078 | 0 | 15 | 5281 | 422450 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2140 | 428000 | 2114.53 | 422906 | 102 |  | 5338 | 427010 | 0 | 0 | 1 (2) | 104 |
|  |  |  | High | 2162.5 | 432500 | 2064.67 | 412934 | 504 |  | 5395 | 431570 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1927.5 | 385500 | 1920.39 | 384078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1852.17 | 370434 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1972.5 | 394500 | 1964.31 | 392862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2140 | 428000 | 2112.1 | 422420 | 102 |  | 5332 | 426530 | 2 | 0 | 1 (2) | 104 |
|  |  |  | High | 2160 | 432000 | 2059.74 | 411948 | 504 |  | 5382 | 430590 | 10 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1930 | 386000 | 1920.46 | 384092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1849.74 | 369948 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1970 | 394000 | 1959.38 | 391876 | 6 |  | - | - | - | - | - | - |
| 25 | 133 | Downlink | Low | 2122.5 | 424500 | 2110.53 | 422106 | 0 | 15 | 5283 | 422670 | 8 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2140 | 428000 | 2109.67 | 421934 | 102 |  | 5326 | 426050 | 4 | 0 | 1 (2) | 104 |
|  |  |  | High | 2157.5 | 431500 | 2054.81 | 410962 | 504 |  | 5369 | 429610 | 0 | 0 | 2 (4) | 508 |
|  |  | Uplink | Low | 1932.5 | 386500 | 1920.53 | 384106 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1847.31 | 369462 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1967.5 | 393500 | 1954.45 | 390890 | 6 |  | - | - | - | - | - | - |
| 30 | 160 | Downlink | Low | 2125 | 425000 | 2110.6 | 422120 | 0 | 15 | 5284 | 422690 | 10 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2140 | 428000 | 2107.24 | 421448 | 102 |  | 5320 | 425570 | 6 | 0 | 1 (2) | 104 |
|  |  |  | High | 2155 | 431000 | 2049.88 | 409976 | 504 |  | 5359 | 428690 | 10 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 1935 | 387000 | 1920.6 | 384120 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1844.88 | 368976 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1965 | 393000 | 1949.52 | 389904 | 6 |  | - | - | - | - | - | - |
| 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2140 | 428000 | 2102.2 | 420440 | 102 |  | 5308 | 424610 | 10 | 1 | 1 (2) | 105 |
|  |  |  | High | 2150 | 430000 | 2039.84 | 407968 | 504 |  | 5330 | 426490 | 6 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1940 | 388000 | 1920.56 | 384112 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1839.84 | 367968 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1960 | 392000 | 1939.48 | 387896 | 6 |  | - | - | - | - | - | - |
| 45 | 242 | Downlink | Low | 2132.5 | 426500 | 2110.72 | 422144 | 0 | 15 | 5284 | 422690 | 2 | 1 | 2(4) | 5 |
|  |  |  | Mid | 2140 | 428000 | 2099.86 | 419972 | 102 |  | 5302 | 424130 | 6 | 1 | 1(2) | 105 |
|  |  |  | High | 2147.5 | 429500 | 2035 | 407000 | 504 |  | 5320 | 425570 | 10 | 1 | 0(0) | 505 |
|  |  | Uplink | Low | 1942.5 | 388500 | 1920.72 | 384144 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1837.5 | 367500 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1957.5 | 391500 | 1934.64 | 386928 | 6 |  | - | - | - | - | - | - |
| 50 | 270 | Downlink | Low | 2135 | 427000 | 2110.7 | 422140 | 0 | 15 | 5282 | 422650 | 2 | 0 | 2 (4) | 4 |
|  |  |  | Mid | 2140 | 428000 | 2097.34 | 419468 | 102 |  | 5296 | 423650 | 2 | 0 | 2 (4) | 106 |
|  |  |  | High | 2145 | 429000 | 2029.98 | 405996 | 504 |  | 5307 | 424590 | 6 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 1945 | 389000 | 1920.7 | 384140 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1834.98 | 366996 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1955 | 391000 | 1929.62 | 385924 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.1-2: Test frequencies for NR operating band n1 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2115 | 423000 | 2110.68 | 422136 | 0 | 15 | 5286 | 422910 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2140 | 428000 | 2098.96 | 419792 | 102 |  | 5350 | 427970 | 14 | 0 | 1 (6) | 216 |
|  |  |  | High | 2165 | 433000 | 1979.24 | 395848 | 504 |  | 5411 | 432970 | 14 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1925 | 385000 | 1920.68 | 384136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1764.24 | 352848 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1975 | 395000 | 1968.52 | 393704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 2117.5 | 423500 | 2110.66 | 422132 | 0 | 15 | 5287 | 422930 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 2140 | 428000 | 2096.44 | 419288 | 102 |  | 5344 | 427490 | 22 | 0 | 1 (6) | 216 |
|  |  |  | High | 2162.5 | 432500 | 1974.22 | 394844 | 504 |  | 5401 | 432050 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1927.5 | 385500 | 1920.66 | 384132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1761.72 | 352344 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1972.5 | 394500 | 1963.5 | 392700 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 2120 | 424000 | 2110.82 | 422164 | 0 | 15 | 5285 | 422890 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2140 | 428000 | 2094.1 | 418820 | 102 |  | 5338 | 427010 | 18 | 0 | 1 (6) | 216 |
|  |  |  | High | 2160 | 432000 | 1969.38 | 393876 | 504 |  | 5388 | 431070 | 14 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1930 | 386000 | 1920.82 | 384164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1759.38 | 351876 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1970 | 394000 | 1958.66 | 391732 | 6 |  | - | - | - | - | - | - |
| 25 | 65 | Downlink | Low | 2122.5 | 424500 | 2110.8 | 422160 | 0 | 15 | 5286 | 422910 | 10 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2140 | 428000 | 2091.58 | 418316 | 102 |  | 5332 | 426530 | 2 | 0 | 2 (7) | 218 |
|  |  |  | High | 2157.5 | 431500 | 1964.36 | 392872 | 504 |  | 5375 | 430090 | 22 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1932.5 | 386500 | 1920.8 | 384160 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1756.86 | 351372 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1967.5 | 393500 | 1953.64 | 390728 | 6 |  | - | - | - | - | - | - |
| 30 | 78 | Downlink | Low | 2125 | 425000 | 2110.96 | 422192 | 0 | 15 | 5287 | 422930 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2140 | 428000 | 2089.24 | 417848 | 102 |  | 5326 | 426050 | 22 | 0 | 1 (6) | 216 |
|  |  |  | High | 2155 | 431000 | 1959.52 | 391904 | 504 |  | 5362 | 428930 | 6 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1935 | 387000 | 1920.96 | 384192 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1754.52 | 350904 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1965 | 393000 | 1948.8 | 389760 | 6 |  | - | - | - | - | - | - |
| 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2140 | 428000 | 2084.2 | 416840 | 102 |  | 5314 | 425090 | 14 | 0 | 2 (7) | 218 |
|  |  |  | High | 2150 | 430000 | 1949.48 | 389896 | 504 |  | 5336 | 426970 | 22 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1940 | 388000 | 1920.92 | 384184 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1749.48 | 349896 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1960 | 392000 | 1938.76 | 387752 | 6 |  | - | - | - | - | - | - |
| 45 | 119 | Downlink | Low | 2132.5 | 426500 | 2111.08 | 422216 | 0 | 15 | 5290 | 423170 | 6 | 0 | 3(8) | 16 |
|  |  |  | Mid | 2140 | 428000 | 2081.86 | 416372 | 102 |  | 5308 | 424610 | 10 | 0 | 2(7) | 218 |
|  |  |  | High | 2147.5 | 429500 | 1944.64 | 388928 | 504 |  | 5326 | 426050 | 14 | 0 | 1(6) | 1020 |
|  |  | Uplink | Low | 1942.5 | 388500 | 1921.08 | 384216 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1747.14 | 349428 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1957.5 | 391500 | 1933.92 | 386784 | 6 |  | - | - | - | - | - | - |
| 50 | 133 | Downlink | Low | 2135 | 427000 | 2111.06 | 422212 | 0 | 15 | 5288 | 423130 | 18 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 2140 | 428000 | 2079.34 | 415868 | 102 |  | 5302 | 424130 | 18 | 0 | 2 (7) | 218 |
|  |  |  | High | 2145 | 429000 | 1939.62 | 387924 | 504 |  | 5313 | 425070 | 22 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1945 | 389000 | 1921.06 | 384212 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1744.62 | 348924 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1955 | 391000 | 1928.9 | 385780 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.1-3: Test frequencies for NR operating band n1 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2115 | 423000 | 2111.04 | 422208 | 0 | 15 | - | 422568 |
|  |  |  | Mid | 2140 | 428000 | 2062.6 | 412520 | 102 |  | - | 427568 |
|  |  |  | High | 2165 | 433000 | 1798.16 | 359632 | 504 |  | - | 432568 |
|  |  | Uplink | Low | 1925 | 385000 | 1921.04 | 384208 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1583.16 | 316632 | 504 |  | - | - |
|  |  |  | High | 1975 | 395000 | 1966.72 | 393344 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 2117.5 | 423500 | 2111.02 | 422204 | 0 | 15 | - | 422564 |
|  |  |  | Mid | 2140 | 428000 | 2060.08 | 412016 | 102 |  | - | 427064 |
|  |  |  | High | 2162.5 | 432500 | 1793.14 | 358628 | 504 |  | - | 431564 |
|  |  | Uplink | Low | 1927.5 | 385500 | 1921.02 | 384204 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1580.64 | 316128 | 504 |  | - | - |
|  |  |  | High | 1972.5 | 394500 | 1961.7 | 392340 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 2120 | 424000 | 2111.36 | 422272 | 0 | 15 | - | 422632 |
|  |  |  | Mid | 2140 | 428000 | 2057.92 | 411584 | 102 |  | - | 426632 |
|  |  |  | High | 2160 | 432000 | 1788.48 | 357696 | 504 |  | - | 430632 |
|  |  | Uplink | Low | 1930 | 386000 | 1921.36 | 384272 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1578.48 | 315696 | 504 |  | - | - |
|  |  |  | High | 1970 | 394000 | 1957.04 | 391408 | 6 |  | - | - |
| 25 | 31 | Downlink | Low | 2122.5 | 424500 | 2111.34 | 422268 | 0 | 15 | - | 422628 |
|  |  |  | Mid | 2140 | 428000 | 2055.4 | 411080 | 102 |  | - | 426128 |
|  |  |  | High | 2157.5 | 431500 | 1783.46 | 356692 | 504 |  | - | 429628 |
|  |  | Uplink | Low | 1932.5 | 386500 | 1921.34 | 384268 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1575.96 | 315192 | 504 |  | - | - |
|  |  |  | High | 1967.5 | 393500 | 1952.02 | 390404 | 6 |  | - | - |
| 30 | 38 | Downlink | Low | 2125 | 425000 | 2111.32 | 422264 | 0 | 15 | - | 422624 |
|  |  |  | Mid | 2140 | 428000 | 2052.88 | 410576 | 102 |  | - | 425624 |
|  |  |  | High | 2155 | 431000 | 1778.44 | 355688 | 504 |  | - | 428624 |
|  |  | Uplink | Low | 1935 | 387000 | 1921.32 | 384264 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1573.44 | 314688 | 504 |  | - | - |
|  |  |  | High | 1965 | 393000 | 1947 | 389400 | 6 |  | - | - |
| 40 | 51 | Downlink | Low | 2130 | 426000 | 2111.64 | 422328 | 0 | 15 | - | 422688 |
|  |  |  | Mid | 2140 | 428000 | 2048.2 | 409640 | 102 |  | - | 424688 |
|  |  |  | High | 2150 | 430000 | 1768.76 | 353752 | 504 |  | - | 426688 |
|  |  | Uplink | Low | 1940 | 388000 | 1921.64 | 384328 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1568.76 | 313752 | 504 |  | - | - |
|  |  |  | High | 1960 | 392000 | 1937.32 | 387464 | 6 |  | - | - |
| 45 | 58 | Downlink | Low | 2132.5 | 426500 | 2111.62 | 422324 | 0 | 15 | - | 422684 |
|  |  |  | Mid | 2140 | 428000 | 2045.68 | 409136 | 102 |  | - | 424184 |
|  |  |  | High | 2147.5 | 429500 | 1763.74 | 352748 | 504 |  | - | 425684 |
|  |  | Uplink | Low | 1942.5 | 388500 | 1921.62 | 384324 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1566.24 | 313248 | 504 |  | - | - |
|  |  |  | High | 1957.5 | 391500 | 1932.3 | 386460 | 6 |  | - | - |
| 50 | 65 | Downlink | Low | 2135 | 427000 | 2111.6 | 422320 | 0 | 15 | - | 422680 |
|  |  |  | Mid | 2140 | 428000 | 2043.16 | 408632 | 102 |  | - | 423680 |
|  |  |  | High | 2145 | 429000 | 1758.72 | 351744 | 504 |  | - | 424680 |
|  |  | Uplink | Low | 1945 | 389000 | 1921.6 | 384320 | 0 | - | - | - |
|  |  |  | Mid | 1950 | 390000 | 1563.72 | 312744 | 504 |  | - | - |
|  |  |  | High | 1955 | 391000 | 1927.28 | 385456 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.2	 Reference test frequencies for NR operating band n2

Table 4.3.1.1.1.2-1: Test frequencies for NR operating band n2 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1932.5 | 386500 | 1930.25 | 386050 | 0 | 15 | 4829 | 386410 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1960 | 392000 | 1939.39 | 387878 | 102 |  | 4900 | 391970 | 8 | 1 | 0 (0) | 103 |
|  |  |  | High | 1987.5 | 397500 | 1894.53 | 378906 | 504 |  | 4968 | 397470 | 8 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 1852.5 | 370500 | 1850.25 | 370050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1787.03 | 357406 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1907.5 | 381500 | 1904.17 | 380834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 1935 | 387000 | 1930.32 | 386064 | 0 | 15 | 4830 | 386430 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1960 | 392000 | 1936.96 | 387392 | 102 |  | 4894 | 391490 | 10 | 1 | 0 (0) | 103 |
|  |  |  | High | 1985 | 397000 | 1889.6 | 377920 | 504 |  | 4955 | 396490 | 10 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 1855 | 371000 | 1850.32 | 370064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1784.6 | 356920 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1905 | 381000 | 1899.24 | 379848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 1937.5 | 387500 | 1930.39 | 386078 | 0 | 15 | 4831 | 386450 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1960 | 392000 | 1934.53 | 386906 | 102 |  | 4888 | 391010 | 0 | 0 | 1 (2) | 104 |
|  |  |  | High | 1982.5 | 396500 | 1884.67 | 376934 | 504 |  | 4945 | 395570 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1857.5 | 371500 | 1850.39 | 370078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1782.17 | 356434 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1902.5 | 380500 | 1894.31 | 378862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 1940 | 388000 | 1930.46 | 386092 | 0 | 15 | 4832 | 386650 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1960 | 392000 | 1932.1 | 386420 | 102 |  | 4882 | 390530 | 2 | 0 | 1 (2) | 104 |
|  |  |  | High | 1980 | 396000 | 1879.74 | 375948 | 504 |  | 4932 | 394590 | 10 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1860 | 372000 | 1850.46 | 370092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1779.74 | 355948 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1900 | 380000 | 1889.38 | 377876 | 6 |  | - | - | - | - | - | - |
| 25 | 133 | Downlink | Low | 1942.5 | 388500 | 1930.53 | 386106 | 0 | 15 | 4833 | 386670 | 8 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1960 | 392000 | 1929.67 | 385934 | 102 |  | 4876 | 390050 | 4 | 0 | 1 (2) | 104 |
|  |  |  | High | 1977.5 | 395500 | 1874.81 | 374962 | 504 |  | 4919 | 393610 | 0 | 0 | 2 (4) | 508 |
|  |  | Uplink | Low | 1862.5 | 372500 | 1850.53 | 370106 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1777.31 | 355462 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1897.5 | 379500 | 1884.45 | 376890 | 6 |  | - | - | - | - | - | - |
| 30 | 160 | Downlink | Low | 1945 | 389000 | 1930.6 | 386120 | 0 | 15 | 4834 | 386690 | 10 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1960 | 392000 | 1927.24 | 385448 | 102 |  | 4870 | 389570 | 6 | 0 | 1 (2) | 104 |
|  |  |  | High | 1975 | 395000 | 1869.88 | 373976 | 504 |  | 4909 | 392690 | 10 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 1865 | 373000 | 1850.6 | 370120 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1774.88 | 354976 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1895 | 379000 | 1879.52 | 375904 | 6 |  | - | - | - | - | - | - |
| 40 | 216 | Downlink | Low | 1950 | 390000 | 1930.56 | 386112 | 0 | 15 | 4833 | 386670 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1960 | 392000 | 1922.2 | 384440 | 102 |  | 4858 | 388610 | 10 | 1 | 1 (2) | 105 |
|  |  |  | High | 1970 | 394000 | 1859.84 | 371968 | 504 |  | 4880 | 390490 | 6 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1870 | 374000 | 1850.56 | 370112 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1769.84 | 353968 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1890 | 378000 | 1869.48 | 373896 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.2-2: Test frequencies for NR operating band n2 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 1935 | 387000 | 1930.68 | 386136 | 0 | 15 | 4836 | 386910 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1960 | 392000 | 1918.96 | 383792 | 102 |  | 4900 | 391970 | 14 | 0 | 1 (6) | 216 |
|  |  |  | High | 1985 | 397000 | 1799.24 | 359848 | 504 |  | 4961 | 396970 | 14 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1855 | 371000 | 1850.68 | 370136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1694.24 | 338848 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1905 | 381000 | 1898.52 | 379704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 1937.5 | 387500 | 1930.66 | 386132 | 0 | 15 | 4837 | 386930 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1960 | 392000 | 1916.44 | 383288 | 102 |  | 4894 | 391490 | 22 | 0 | 1 (6) | 216 |
|  |  |  | High | 1982.5 | 396500 | 1794.22 | 358844 | 504 |  | 4951 | 396050 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1857.5 | 371500 | 1850.66 | 370132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1691.72 | 338344 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1902.5 | 380500 | 1893.5 | 378700 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 1940 | 388000 | 1930.82 | 386164 | 0 | 15 | 4835 | 386890 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1960 | 392000 | 1914.1 | 382820 | 102 |  | 4888 | 391010 | 18 | 0 | 1 (6) | 216 |
|  |  |  | High | 1980 | 396000 | 1789.38 | 357876 | 504 |  | 4938 | 395070 | 14 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1860 | 372000 | 1850.82 | 370164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1689.38 | 337876 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1900 | 380000 | 1888.66 | 377732 | 6 |  | - | - | - | - | - | - |
| 25 | 65 | Downlink | Low | 1942.5 | 388500 | 1930.8 | 386160 | 0 | 15 | 4836 | 386910 | 10 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1960 | 392000 | 1911.58 | 382316 | 102 |  | 4882 | 390530 | 2 | 0 | 2 (7) | 218 |
|  |  |  | High | 1977.5 | 395500 | 1784.36 | 356872 | 504 |  | 4925 | 394090 | 22 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1862.5 | 372500 | 1850.8 | 370160 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1686.86 | 337372 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1897.5 | 379500 | 1883.64 | 376728 | 6 |  | - | - | - | - | - | - |
| 30 | 78 | Downlink | Low | 1945 | 389000 | 1930.96 | 386192 | 0 | 15 | 4837 | 386930 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1960 | 392000 | 1909.24 | 381848 | 102 |  | 4876 | 390050 | 22 | 0 | 1 (6) | 216 |
|  |  |  | High | 1975 | 395000 | 1779.52 | 355904 | 504 |  | 4912 | 392930 | 6 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1865 | 373000 | 1850.96 | 370192 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1684.52 | 336904 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1895 | 379000 | 1878.8 | 375760 | 6 |  | - | - | - | - | - | - |
| 40 | 106 | Downlink | Low | 1950 | 390000 | 1930.92 | 386184 | 0 | 15 | 4836 | 386910 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1960 | 392000 | 1904.2 | 380840 | 102 |  | 4864 | 389090 | 14 | 0 | 2 (7) | 218 |
|  |  |  | High | 1970 | 394000 | 1769.48 | 353896 | 504 |  | 4886 | 390970 | 22 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1870 | 374000 | 1850.92 | 370184 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1679.48 | 335896 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1890 | 378000 | 1868.76 | 373752 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.2-3: Test frequencies for NR operating band n2 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 1935 | 387000 | 1931.04 | 386208 | 0 | 15 | - | 386568 |
|  |  |  | Mid | 1960 | 392000 | 1882.6 | 376520 | 102 |  | - | 391568 |
|  |  |  | High | 1985 | 397000 | 1618.16 | 323632 | 504 |  | - | 396568 |
|  |  | Uplink | Low | 1855 | 371000 | 1851.04 | 370208 | 0 | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1513.16 | 302632 | 504 |  | - | - |
|  |  |  | High | 1905 | 381000 | 1896.72 | 379344 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 1937.5 | 387500 | 1931.02 | 386204 | 0 | 15 | - | 386564 |
|  |  |  | Mid | 1960 | 392000 | 1880.08 | 376016 | 102 |  | - | 391064 |
|  |  |  | High | 1982.5 | 396500 | 1613.14 | 322628 | 504 |  | - | 395564 |
|  |  | Uplink | Low | 1857.5 | 371500 | 1851.02 | 370204 | 0 | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1510.64 | 302128 | 504 |  | - | - |
|  |  |  | High | 1902.5 | 380500 | 1891.7 | 378340 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 1940 | 388000 | 1931.36 | 386272 | 0 | 15 | - | 386632 |
|  |  |  | Mid | 1960 | 392000 | 1877.92 | 375584 | 102 |  | - | 390632 |
|  |  |  | High | 1980 | 396000 | 1608.48 | 321696 | 504 |  | - | 394632 |
|  |  | Uplink | Low | 1860 | 372000 | 1851.36 | 370272 | 0 | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1508.48 | 301696 | 504 |  | - | - |
|  |  |  | High | 1900 | 380000 | 1887.04 | 377408 | 6 |  | - | - |
| 25 | 31 | Downlink | Low | 1942.5 | 388500 | 1931.34 | 386268 | 0 | 15 | - | 386628 |
|  |  |  | Mid | 1960 | 392000 | 1875.4 | 375080 | 102 |  | - | 390128 |
|  |  |  | High | 1977.5 | 395500 | 1603.46 | 320692 | 504 |  | - | 393628 |
|  |  | Uplink | Low | 1862.5 | 372500 | 1851.34 | 370268 | 0 | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1505.96 | 301192 | 504 |  | - | - |
|  |  |  | High | 1897.5 | 379500 | 1882.02 | 376404 | 6 |  | - | - |
| 30 | 38 | Downlink | Low | 1945 | 389000 | 1931.32 | 386264 | 0 | 15 | - | 386624 |
|  |  |  | Mid | 1960 | 392000 | 1872.88 | 374576 | 102 |  | - | 389624 |
|  |  |  | High | 1975 | 395000 | 1598.44 | 319688 | 504 |  | - | 392624 |
|  |  | Uplink | Low | 1865 | 373000 | 1851.32 | 370264 | 0 | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1503.44 | 300688 | 504 |  | - | - |
|  |  |  | High | 1895 | 379000 | 1877 | 375400 | 6 |  | - | - |
| 40 | 51 | Downlink | Low | 1950 | 390000 | 1931.64 | 386328 | 0 | 15 | - | 386688 |
|  |  |  | Mid | 1960 | 392000 | 1868.2 | 373640 | 102 |  | - | 388688 |
|  |  |  | High | 1970 | 394000 | 1588.76 | 317752 | 504 |  | - | 390688 |
|  |  | Uplink | Low | 1870 | 374000 | 1851.64 | 370328 | 0 | - | - | - |
|  |  |  | Mid | 1880 | 376000 | 1498.76 | 299752 | 504 |  | - | - |
|  |  |  | High | 1890 | 378000 | 1867.32 | 373464 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.3	Reference test frequencies for NR operating band n3

Table 4.3.1.1.1.3-1: Test frequencies for NR operating band n3 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1807.5 | 361500 | 1805.25 | 361050 | 0 | 15 | 4518 | 361470 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 1842.5 | 368500 | 1821.89 | 364378 | 102 |  | 4604 | 368410 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 1877.5 | 375500 | 1784.53 | 356906 | 504 |  | 4693 | 375410 | 0 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1654.53 | 330906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1782.5 | 356500 | 1779.17 | 355834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 1810 | 362000 | 1805.32 | 361064 | 0 | 15 | 4519 | 361490 | 10 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 1842.5 | 368500 | 1819.46 | 363892 | 102 |  | 4598 | 367930 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 1875 | 375000 | 1779.6 | 355920 | 504 |  | 4680 | 374430 | 2 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1652.1 | 330420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1780 | 356000 | 1774.24 | 354848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 1812.5 | 362500 | 1805.39 | 361078 | 0 | 15 | 4517 | 361450 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1842.5 | 368500 | 1817.03 | 363406 | 102 |  | 4592 | 367450 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 1872.5 | 374500 | 1774.67 | 354934 | 504 |  | 4667 | 373450 | 4 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1717.5 | 343500 | 1710.39 | 342078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1649.67 | 329934 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1777.5 | 355500 | 1769.31 | 353862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 1815 | 363000 | 1805.46 | 361092 | 0 | 15 | 4518 | 361470 | 6 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1842.5 | 368500 | 1814.6 | 362920 | 102 |  | 4586 | 366970 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 1870 | 374000 | 1769.74 | 353948 | 504 |  | 4657 | 372530 | 2 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1647.24 | 329448 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1775 | 355000 | 1764.38 | 352876 | 6 |  | - | - | - | - | - | - |
| 25 | 133 | Downlink | Low | 1817.5 | 363500 | 1805.53 | 361106 | 0 | 15 | 4519 | 361490 | 8 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1842.5 | 368500 | 1812.17 | 362434 | 102 |  | 4580 | 366490 | 8 | 0 | 0 (0) | 102 |
|  |  |  | High | 1867.5 | 373500 | 1764.81 | 352962 | 504 |  | 4644 | 371550 | 4 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 1722.5 | 344500 | 1710.53 | 342106 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1644.81 | 328962 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1772.5 | 354500 | 1759.45 | 351890 | 6 |  | - | - | - | - | - | - |
| 30 | 160 | Downlink | Low | 1820 | 364000 | 1805.6 | 361120 | 0 | 15 | 4520 | 361690 | 10 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1842.5 | 368500 | 1809.74 | 361948 | 102 |  | 4574 | 366010 | 10 | 0 | 0 (0) | 102 |
|  |  |  | High | 1865 | 373000 | 1759.88 | 351976 | 504 |  | 4631 | 370570 | 6 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 1725 | 345000 | 1710.6 | 342120 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1642.38 | 328476 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1770 | 354000 | 1754.52 | 350904 | 6 |  | - | - | - | - | - | - |
| 40 | 216 | Downlink | Low | 1825 | 365000 | 1805.56 | 361112 | 0 | 15 | 4519 | 361490 | 6 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1842.5 | 368500 | 1804.7 | 360940 | 102 |  | 4562 | 365050 | 2 | 0 | 1 (2) | 104 |
|  |  |  | High | 1860 | 372000 | 1749.84 | 349968 | 504 |  | 4608 | 368670 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 1730 | 346000 | 1710.56 | 342112 | 0 | - | - |  | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1637.34 | 327468 | 504 |  | - |  | - | - | - | - |
|  |  |  | High | 1765 | 353000 | 1744.48 | 348896 | 6 |  | - |  | - | - | - | - |
| 50 | 270 | Downlink | Low | 1830 | 366000 | 1805.7 | 361140 | 0 | 15 | 4521 | 361710 | 10 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1842.5 | 368500 | 1799.84 | 359968 | 102 |  | 4550 | 364090 | 6 | 0 | 1 (2) | 104 |
|  |  |  | High | 1855 | 371000 | 1739.98 | 347996 | 504 |  | 4582 | 366530 | 10 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1735 | 347000 | 1710.7 | 342140 | 0 | - | - |  | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1632.48 | 326496 | 504 |  | - |  | - | - | - | - |
|  |  |  | High | 1760 | 352000 | 1734.62 | 346924 | 6 |  | - |  | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.3-2: Test frequencies for NR operating band n3 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 1810 | 362000 | 1805.68 | 361136 | 0 | 15 | 4525 | 361970 | 14 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1842.5 | 368500 | 1801.46 | 360292 | 102 |  | 4604 | 368410 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 1875 | 375000 | 1689.24 | 337848 | 504 |  | 4686 | 374910 | 18 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1561.74 | 312348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1780 | 356000 | 1773.52 | 354704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 1812.5 | 362500 | 1805.66 | 361132 | 0 | 15 | 4523 | 361930 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1842.5 | 368500 | 1798.94 | 359788 | 102 |  | 4598 | 367930 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 1872.5 | 374500 | 1684.22 | 336844 | 504 |  | 4673 | 373930 | 2 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1717.5 | 343500 | 1710.66 | 342132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1559.22 | 311844 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1777.5 | 355500 | 1768.5 | 353700 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 1815 | 363000 | 1805.82 | 361164 | 0 | 15 | 4524 | 361950 | 22 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1842.5 | 368500 | 1796.6 | 359320 | 102 |  | 4592 | 367450 | 22 | 0 | 0 (5) | 214 |
|  |  |  | High | 1870 | 374000 | 1679.38 | 335876 | 504 |  | 4663 | 373010 | 18 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1556.88 | 311376 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1775 | 355000 | 1763.66 | 352732 | 6 |  | - | - | - | - | - | - |
| 25 | 65 | Downlink | Low | 1817.5 | 363500 | 1805.8 | 361160 | 0 | 15 | 4525 | 361970 | 6 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1842.5 | 368500 | 1794.08 | 358816 | 102 |  | 4586 | 366970 | 6 | 0 | 1 (6) | 216 |
|  |  |  | High | 1867.5 | 373500 | 1674.36 | 334872 | 504 |  | 4650 | 372030 | 2 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1722.5 | 344500 | 1710.8 | 342160 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1554.36 | 310872 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1772.5 | 354500 | 1758.64 | 351728 | 6 |  | - | - | - | - | - | - |
| 30 | 78 | Downlink | Low | 1820 | 364000 | 1805.96 | 361192 | 0 | 15 | 4523 | 361930 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1842.5 | 368500 | 1791.74 | 358348 | 102 |  | 4580 | 366490 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 1865 | 373000 | 1669.52 | 333904 | 504 |  | 4637 | 371050 | 22 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1725 | 345000 | 1710.96 | 342192 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1552.02 | 310404 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1770 | 354000 | 1753.8 | 350760 | 6 |  | - | - | - | - | - | - |
| 40 | 106 | Downlink | Low | 1825 | 365000 | 1805.92 | 361184 | 0 | 15 | 4525 | 361970 | 22 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1842.5 | 368500 | 1786.7 | 357340 | 102 |  | 4568 | 365530 | 18 | 0 | 1 (6) | 216 |
|  |  |  | High | 1860 | 372000 | 1659.48 | 331896 | 504 |  | 4611 | 368910 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1730 | 346000 | 1710.92 | 342184 | 0 | - | - |  | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1546.98 | 309396 | 504 |  | - |  | - | - | - | - |
|  |  |  | High | 1765 | 353000 | 1743.76 | 348752 | 6 |  | - |  | - | - | - | - |
| 50 | 133 | Downlink | Low | 1830 | 366000 | 1806.06 | 361212 | 0 | 15 | 4524 | 361950 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1842.5 | 368500 | 1781.84 | 356368 | 102 |  | 4556 | 364570 | 22 | 0 | 1 (6) | 216 |
|  |  |  | High | 1855 | 371000 | 1649.62 | 329924 | 504 |  | 4588 | 367010 | 2 | 0 | 1(6) | 1020 |
|  |  | Uplink | Low | 1735 | 347000 | 1711.06 | 342212 | 0 | - | - |  | - | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1542.12 | 308424 | 504 |  | - |  | - | - | - | - |
|  |  |  | High | 1760 | 352000 | 1733.9 | 346780 | 6 |  | - |  | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.3-3: Test frequencies for NR operating band n3 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 1810 | 362000 | 1806.04 | 361208 | 0 | 15 | - | 361568 |
|  |  |  | Mid | 1842.5 | 368500 | 1765.1 | 353020 | 102 |  | - | 368068 |
|  |  |  | High | 1875 | 375000 | 1508.16 | 301632 | 504 |  | - | 374568 |
|  |  | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1380.66 | 276132 | 504 |  | - | - |
|  |  |  | High | 1780 | 356000 | 1771.72 | 354344 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 1812.5 | 362500 | 1806.02 | 361204 | 0 | 15 | - | 361564 |
|  |  |  | Mid | 1842.5 | 368500 | 1762.58 | 352516 | 102 |  | - | 367564 |
|  |  |  | High | 1872.5 | 374500 | 1503.14 | 300628 | 504 |  | - | 373564 |
|  |  | Uplink | Low | 1717.5 | 343500 | 1711.02 | 342204 | 0 | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1378.14 | 275628 | 504 |  | - | - |
|  |  |  | High | 1777.5 | 355500 | 1766.7 | 353340 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 1815 | 363000 | 1806.36 | 361272 | 0 | 15 | - | 361632 |
|  |  |  | Mid | 1842.5 | 368500 | 1760.42 | 352084 | 102 |  | - | 367132 |
|  |  |  | High | 1870 | 374000 | 1498.48 | 299696 | 504 |  | - | 372632 |
|  |  | Uplink | Low | 1720 | 344000 | 1711.36 | 342272 | 0 | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1375.98 | 275196 | 504 |  | - | - |
|  |  |  | High | 1775 | 355000 | 1762.04 | 352408 | 6 |  | - | - |
| 25 | 31 | Downlink | Low | 1817.5 | 363500 | 1806.34 | 361268 | 0 | 15 | - | 361628 |
|  |  |  | Mid | 1842.5 | 368500 | 1757.9 | 351580 | 102 |  | - | 366628 |
|  |  |  | High | 1867.5 | 373500 | 1493.46 | 298692 | 504 |  | - | 371628 |
|  |  | Uplink | Low | 1722.5 | 344500 | 1711.34 | 342268 | 0 | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1373.46 | 274692 | 504 |  | - | - |
|  |  |  | High | 1772.5 | 354500 | 1757.02 | 351404 | 6 |  | - | - |
| 30 | 38 | Downlink | Low | 1820 | 364000 | 1806.32 | 361264 | 0 | 15 | - | 361624 |
|  |  |  | Mid | 1842.5 | 368500 | 1755.38 | 351076 | 102 |  | - | 366124 |
|  |  |  | High | 1865 | 373000 | 1488.44 | 297688 | 504 |  | - | 370624 |
|  |  | Uplink | Low | 1725 | 345000 | 1711.32 | 342264 | 0 | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1370.94 | 274188 | 504 |  | - | - |
|  |  |  | High | 1770 | 354000 | 1752 | 350400 | 6 |  | - | - |
| 40 | 51 | Downlink | Low | 1825 | 365000 | 1806.64 | 361328 | 0 | 15 | - | 361688 |
|  |  |  | Mid | 1842.5 | 368500 | 1750.7 | 350140 | 102 |  | - | 365188 |
|  |  |  | High | 1860 | 372000 | 1478.76 | 295752 | 504 |  | - | 368688 |
|  |  | Uplink | Low | 1730 | 346000 | 1711.64 | 342328 | 0 | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1366.26 | 273252 | 504 |  | - | - |
|  |  |  | High | 1765 | 353000 | 1742.32 | 348464 | 6 |  | - | - |
| 50 | 65 | Downlink | Low | 1830 | 366000 | 1806.6 | 361320 | 0 | 15 | - | 361680 |
|  |  |  | Mid | 1842.5 | 368500 | 1745.66 | 349132 | 102 |  | - | 364180 |
|  |  |  | High | 1855 | 371000 | 1468.72 | 293744 | 504 |  | - | 366680 |
|  |  | Uplink | Low | 1735 | 347000 | 1711.6 | 342320 | 0 | - | - | - |
|  |  |  | Mid | 1747.5 | 349500 | 1361.22 | 272244 | 504 |  | - | - |
|  |  |  | High | 1760 | 352000 | 1732.28 | 346456 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.4	FFS

4.3.1.1.1.5	Reference test frequencies for NR operating band n5

Table 4.3.1.1.1.5-1: Test frequencies for NR operating band n5 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 871.5 | 174300 | 869.25 | 173850 | 0 | 15 | 2178 | 174270 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 881.5 | 176300 | 860.89 | 172178 | 102 |  | 2203 | 176210 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 891.5 | 178300 | 798.53 | 159706 | 504 |  | 2228 | 178330 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 826.5 | 165300 | 824.25 | 164850 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 743.53 | 148706 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 846.5 | 169300 | 843.17 | 168634 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 874 | 174800 | 869.32 | 173864 | 0 | 15 | 2179 | 174290 | 10 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 881.5 | 176300 | 858.46 | 171692 | 102 |  | 2197 | 175730 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 889 | 177800 | 793.6 | 158720 | 504 |  | 2218 | 177410 | 2 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 829 | 165800 | 824.32 | 164864 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 741.1 | 148220 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 844 | 168800 | 838.24 | 167648 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 876.5 | 175300 | 869.39 | 173878 | 0 | 15 | 2177 | 174250 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 881.5 | 176300 | 856.03 | 171206 | 102 |  | 2191 | 175250 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 886.5 | 177300 | 788.67 | 157734 | 504 |  | 2205 | 176430 | 4 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 831.5 | 166300 | 824.39 | 164878 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 738.67 | 147734 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 841.5 | 168300 | 833.31 | 166662 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 879 | 175800 | 869.46 | 173892 | 0 | 15 | 2178 | 174270 | 6 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 881.5 | 176300 | 853.6 | 170720 | 102 |  | 2185 | 174770 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 884 | 176800 | 783.74 | 156748 | 504 |  | 2192 | 175450 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 834 | 166800 | 824.46 | 164892 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 736.24 | 147248 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 839 | 167800 | 828.38 | 165676 | 6 |  | - | - | - | - | - | - |
| 25 | 133 | Downlink | Low | 881.5 | 176300 | 869.53 | 173906 | 0 | 15 | 2179 | 174290 | 8 | 0 | 0 (0) | 0 |
| Note 3 |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.5-2: Test frequencies for NR operating band n5 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 874 | 174800 | 869.68 | 173936 | 0 | 30 | 2185 | 174770 | 14 | 0 | 1 (1) | 2 |
|  |  |  | Mid | 881.5 | 176300 | 840.46 | 168092 | 102 |  | 2203 | 176210 | 18 | 0 | 0 (0) | 204 |
|  |  |  | High | 889 | 177800 | 703.24 | 140648 | 504 |  | 2224 | 177890 | 6 | 0 | 3 (3) | 1014 |
|  |  | Uplink | Low | 829 | 165800 | 824.68 | 164936 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 650.74 | 130148 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 844 | 168800 | 837.52 | 167504 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 876.5 | 175300 | 869.66 | 173932 | 0 | 30 | 2183 | 174730 | 2 | 0 | 1 (1) | 2 |
|  |  |  | Mid | 881.5 | 176300 | 837.94 | 167588 | 102 |  | 2197 | 175730 | 2 | 0 | 1 (1) | 206 |
|  |  |  | High | 886.5 | 177300 | 698.22 | 139644 | 504 |  | 2208 | 176670 | 6 | 0 | 0 (0) | 1008 |
|  |  | Uplink | Low | 831.5 | 166300 | 824.66 | 164932 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 648.22 | 129644 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 841.5 | 168300 | 832.5 | 166500 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 879 | 175800 | 869.82 | 173964 | 0 | 30 | 2184 | 174750 | 22 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 881.5 | 176300 | 835.6 | 167120 | 102 |  | 2191 | 175250 | 22 | 0 | 0 (0) | 204 |
|  |  |  | High | 884 | 176800 | 693.38 | 138676 | 504 |  | 2195 | 175690 | 2 | 0 | 0 (0) | 1008 |
|  |  | Uplink | Low | 834 | 166800 | 824.82 | 164964 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 836.5 | 167300 | 645.88 | 129176 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 839 | 167800 | 827.66 | 165532 | 6 |  | - | - | - | - | - | - |
| 25 | 65 | Downlink | Low | 881.5 | 176300 | 869.8 | 173960 | 0 | 30 | 2185 | 174770 | 6 | 0 | 1 (1) | 2 |
| Note 3 |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: This UE channel bandwidth is applicable only to downlink. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.6	FFS

4.3.1.1.1.7	Reference test frequencies for NR operating band n7

Table 4.3.1.1.1.7-1: Test frequencies for NR operating band n7 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2622.5 | 524500 | 2620.25 | 524050 | 0 | 15 | 6554 | 524410 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2655 | 531000 | 2634.39 | 526878 | 102 |  | 6636 | 530910 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 2687.5 | 537500 | 2594.53 | 518906 | 504 |  | 6718 | 537410 | 0 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 2502.5 | 500500 | 2500.25 | 500050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2442.03 | 488406 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2567.5 | 513500 | 2564.17 | 512834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 2625 | 525000 | 2620.32 | 524064 | 0 | 15 | 6555 | 524430 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2655 | 531000 | 2631.96 | 526392 | 102 |  | 6630 | 530430 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 2685 | 537000 | 2589.6 | 517920 | 504 |  | 6705 | 536430 | 2 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 2505 | 501000 | 2500.32 | 500064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2439.6 | 487920 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2565 | 513000 | 2559.24 | 511848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 2627.5 | 525500 | 2620.39 | 524078 | 0 | 15 | 6556 | 524450 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2655 | 531000 | 2629.53 | 525906 | 102 |  | 6624 | 529950 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 2682.5 | 536500 | 2584.67 | 516934 | 504 |  | 6692 | 535450 | 4 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 2507.5 | 501500 | 2500.39 | 500078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2437.17 | 487434 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2562.5 | 512500 | 2554.31 | 510862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 2630 | 526000 | 2620.46 | 524092 | 0 | 15 | 6557 | 524650 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2655 | 531000 | 2627.1 | 525420 | 102 |  | 6618 | 529470 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 2680 | 536000 | 2579.74 | 515948 | 504 |  | 6682 | 534530 | 2 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 2510 | 502000 | 2500.46 | 500092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2434.74 | 486948 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2560 | 512000 | 2549.38 | 509876 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.7-2: Test frequencies for NR operating band n7 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2625 | 525000 | 2620.68 | 524136 | 0 | 15 | 6561 | 524910 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2655 | 531000 | 2613.96 | 522792 | 102 |  | 6636 | 530910 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 2685 | 537000 | 2499.24 | 499848 | 504 |  | 6711 | 536910 | 18 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 2505 | 501000 | 2500.68 | 500136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2349.24 | 469848 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2565 | 513000 | 2558.52 | 511704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 2627.5 | 525500 | 2620.66 | 524132 | 0 | 15 | 6562 | 524930 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 2655 | 531000 | 2611.44 | 522288 | 102 |  | 6630 | 530430 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 2682.5 | 536500 | 2494.22 | 498844 | 504 |  | 6698 | 535930 | 2 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 2507.5 | 501500 | 2500.66 | 500132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2346.72 | 469344 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2562.5 | 512500 | 2553.5 | 510700 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 2630 | 526000 | 2620.82 | 524164 | 0 | 15 | 6560 | 524890 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2655 | 531000 | 2609.1 | 521820 | 102 |  | 6624 | 529950 | 22 | 0 | 0 (5) | 214 |
|  |  |  | High | 2680 | 536000 | 2489.38 | 497876 | 504 |  | 6688 | 535010 | 18 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 2510 | 502000 | 2500.82 | 500164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2344.38 | 468876 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2560 | 512000 | 2548.66 | 509732 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.7-3: Test frequencies for NR operating band n7 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2625 | 525000 | 2621.04 | 524208 | 0 | 15 | - | 524568 |
|  |  |  | Mid | 2655 | 531000 | 2577.6 | 515520 | 102 |  | - | 530568 |
|  |  |  | High | 2685 | 537000 | 2318.16 | 463632 | 504 |  | - | 536568 |
|  |  | Uplink | Low | 2505 | 501000 | 2501.04 | 500208 | 0 | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2168.16 | 433632 | 504 |  | - | - |
|  |  |  | High | 2565 | 513000 | 2556.72 | 511344 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 2627.5 | 525500 | 2621.02 | 524204 | 0 | 15 | - | 524564 |
|  |  |  | Mid | 2655 | 531000 | 2575.08 | 515016 | 102 |  | - | 530064 |
|  |  |  | High | 2682.5 | 536500 | 2313.14 | 462628 | 504 |  | - | 535564 |
|  |  | Uplink | Low | 2507.5 | 501500 | 2501.02 | 500204 | 0 | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2165.64 | 433128 | 504 |  | - | - |
|  |  |  | High | 2562.5 | 512500 | 2551.7 | 510340 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 2630 | 526000 | 2621.36 | 524272 | 0 | 15 | - | 524632 |
|  |  |  | Mid | 2655 | 531000 | 2572.92 | 514584 | 102 |  | - | 529632 |
|  |  |  | High | 2680 | 536000 | 2308.48 | 461696 | 504 |  | - | 534632 |
|  |  | Uplink | Low | 2510 | 502000 | 2501.36 | 500272 | 0 | - | - | - |
|  |  |  | Mid | 2535 | 507000 | 2163.48 | 432696 | 504 |  | - | - |
|  |  |  | High | 2560 | 512000 | 2547.04 | 509408 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.8	Reference test frequencies for NR operating band n8

Table 4.3.1.1.1.8-1: Test frequencies for NR operating band n8 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 927.5 | 185500 | 925.25 | 185050 | 0 | 15 | 2318 | 185530 | 4 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 942.5 | 188500 | 921.89 | 184378 | 102 |  | 2354 | 188410 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 957.5 | 191500 | 864.53 | 172906 | 504 |  | 2393 | 191530 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 882.5 | 176500 | 880.25 | 176050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 804.53 | 160906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 912.5 | 182500 | 909.17 | 181834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 930 | 186000 | 925.32 | 185064 | 0 | 15 | 2319 | 185550 | 6 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 942.5 | 188500 | 919.46 | 183892 | 102 |  | 2348 | 187930 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 955 | 191000 | 859.6 | 171920 | 504 |  | 2383 | 190610 | 2 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 885 | 177000 | 880.32 | 176064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 802.1 | 160420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 904.24 | 180848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 932.5 | 186500 | 925.39 | 185078 | 0 | 15 | 2320 | 185570 | 8 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 942.5 | 188500 | 917.03 | 183406 | 102 |  | 2342 | 187450 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 952.5 | 190500 | 854.67 | 170934 | 504 |  | 2370 | 189630 | 4 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 887.5 | 177500 | 880.39 | 176078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 799.67 | 159934 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 907.5 | 181500 | 899.31 | 179862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 935 | 187000 | 925.46 | 185092 | 0 | 15 | 2318 | 185530 | 2 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 942.5 | 188500 | 914.6 | 182920 | 102 |  | 2336 | 186970 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 950 | 190000 | 849.74 | 169948 | 504 |  | 2357 | 188650 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 890 | 178000 | 880.46 | 176092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 797.24 | 159448 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 905 | 181000 | 894.38 | 178876 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.8-2: Test frequencies for NR operating band n8 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 930 | 186000 | 925.68 | 185136 | 0 | 15 | 2325 | 186030 | 10 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 942.5 | 188500 | 901.46 | 180292 | 102 |  | 2354 | 188410 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 955 | 191000 | 769.24 | 153848 | 504 |  | 2389 | 191090 | 6 | 0 | 3 (8) | 1024 |
|  |  | Uplink | Low | 885 | 177000 | 880.68 | 176136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 711.74 | 142348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 910 | 182000 | 903.52 | 180704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 932.5 | 186500 | 925.66 | 185132 | 0 | 15 | 2326 | 186050 | 18 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 942.5 | 188500 | 898.94 | 179788 | 102 |  | 2348 | 187930 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 952.5 | 190500 | 764.22 | 152844 | 504 |  | 2373 | 189870 | 6 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 887.5 | 177500 | 880.66 | 176132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 709.22 | 141844 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 907.5 | 181500 | 898.5 | 179700 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 935 | 187000 | 925.82 | 185164 | 0 | 15 | 2324 | 186010 | 18 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 942.5 | 188500 | 896.6 | 179320 | 102 |  | 2342 | 187450 | 22 | 0 | 0 (5) | 214 |
|  |  |  | High | 950 | 190000 | 759.38 | 151876 | 504 |  | 2360 | 188890 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 890 | 178000 | 880.82 | 176164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 897.5 | 179500 | 706.88 | 141376 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 905 | 181000 | 893.66 | 178732 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.9 to 4.3.1.1.1.11	FFS

4.3.1.1.1.12	Reference test frequencies for NR operating band n12

Table 4.3.1.1.1.12-1: Test frequencies for NR operating band n12 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 731.5 | 146300 | 729.25 | 145850 | 0 | 15 | 1828 | 146210 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 737.5 | 147500 | 716.89 | 143378 | 102 |  | 1843 | 147410 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 743.5 | 148700 | 650.53 | 130106 | 504 |  | 1858 | 148610 | 0 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 701.5 | 140300 | 699.25 | 139850 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707.5 | 141500 | 614.53 | 122906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 713.5 | 142700 | 710.17 | 142034 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 734 | 146800 | 729.32 | 145864 | 0 | 15 | 1829 | 146410 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 737.5 | 147500 | 714.46 | 142892 | 102 |  | 1837 | 146930 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 741 | 148200 | 645.6 | 129120 | 504 |  | 1845 | 147630 | 2 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 704 | 140800 | 699.32 | 139864 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707.5 | 141500 | 612.1 | 122420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 711 | 142200 | 705.24 | 141048 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 736.5 | 147300 | 729.39 | 145878 | 0 | 15 | 1830 | 146430 | 4 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 737.5 | 147500 | 712.03 | 142406 | 102 |  | 1831 | 146450 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 738.5 | 147700 | 640.67 | 128134 | 504 |  | 1832 | 146650 | 4 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 706.5 | 141300 | 699.39 | 139878 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707.5 | 141500 | 609.67 | 121934 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 708.5 | 141700 | 700.31 | 140062 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.12-2: Test frequencies for NR operating band n12 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 734 | 146800 | 729.68 | 145936 | 0 | 15 | 1835 | 146890 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid | 737.5 | 147500 | 696.46 | 139292 | 102 |  | 1843 | 147410 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 741 | 148200 | 555.24 | 111048 | 504 |  | 1851 | 148110 | 18 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 704 | 140800 | 699.68 | 139936 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707.5 | 141500 | 521.74 | 104348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 711 | 142200 | 704.52 | 140904 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 736.5 | 147300 | 729.66 | 145932 | 0 | 15 | 1833 | 146670 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 737.5 | 147500 | 693.94 | 138788 | 102 |  | 1837 | 146930 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 738.5 | 147700 | 550.22 | 110044 | 504 |  | 1838 | 147130 | 2 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 706.5 | 141300 | 699.66 | 139932 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 707.5 | 141500 | 519.22 | 103844 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 708.5 | 141700 | 699.5 | 139900 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.13	FFS

4.3.1.1.1.14	Reference test frequencies for NR operating band n14

Table 4.3.1.1.1.14-1: Test frequencies for NR operating band n14 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 760.5 | 152100 | 758.25 | 151650 | 0 | 15 | 1902 | 152190 | 0 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 763 | 152600 | 742.39 | 148478 | 102 |  | 1909 | 152690 | 0 | 1 | 2 (4) | 107 |
|  |  |  | High | 765.5 | 153100 | 672.53 | 134506 | 504 |  | 1913 | 153130 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 790.5 | 158100 | 788.25 | 157650 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 793 | 158600 | 700.03 | 140006 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 795.5 | 159100 | 792.17 | 158434 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 763 | 152600 | 758.32 | 151664 | 0 | 15 | 1903 | 152210 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 793 | 158600 | 788.32 | 157664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcchConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET0Carrier in Annex C expressed in number of common RBs. Note 3: 10 MHz test channel bandwidth is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.14-2: Test frequencies for NR operating band n14 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 763 | 152600 | 758.68 | 151736 | 0 | 15 | 1909 | 152690 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | Low | 793 | 158600 | 788.68 | 157736 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled controlResourceSetZero (pdcchConfigSIB1 in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET0Carrier in Annex C expressed in number of common RBs. Note 3: 10 MHz test channel bandwidth is tested with Low range test frequency only. Low range test frequency shall be used instead of Mid range and High range test frequencies. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.15 to 4.3.1.1.1.19	FFS

4.3.1.1.1.20	Reference test frequencies for NR operating band n20

Table 4.3.1.1.1.20-1: Test frequencies for NR operating band n20 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 793.5 | 158700 | 791.25 | 158250 | 0 | 15 | 1983 | 158670 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 806 | 161200 | 785.39 | 157078 | 102 |  | 2015 | 161290 | 0 | 1 | 2 (4) | 107 |
|  |  |  | High | 818.5 | 163700 | 725.53 | 145106 | 504 |  | 2047 | 163730 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 834.5 | 166900 | 832.25 | 166450 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 754.03 | 150806 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 859.5 | 171900 | 856.17 | 171234 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 796 | 159200 | 791.32 | 158264 | 0 | 15 | 1984 | 158690 | 10 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 806 | 161200 | 782.96 | 156592 | 102 |  | 2009 | 160810 | 2 | 1 | 2 (4) | 107 |
|  |  |  | High | 816 | 163200 | 720.6 | 144120 | 504 |  | 2034 | 162750 | 6 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 837 | 167400 | 832.32 | 166464 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 751.6 | 150320 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 857 | 171400 | 851.24 | 170248 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 798.5 | 159700 | 791.39 | 158278 | 0 | 15 | 1982 | 158650 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 806 | 161200 | 780.53 | 156106 | 102 |  | 2003 | 160330 | 4 | 1 | 2 (4) | 107 |
|  |  |  | High | 813.5 | 162700 | 715.67 | 143134 | 504 |  | 2021 | 161770 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 839.5 | 167900 | 832.39 | 166478 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 749.17 | 149834 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 854.5 | 170900 | 846.31 | 169262 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 801 | 160200 | 791.46 | 158292 | 0 | 15 | 1983 | 158670 | 6 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 806 | 161200 | 778.1 | 155620 | 102 |  | 1997 | 159850 | 6 | 1 | 2 (4) | 107 |
|  |  |  | High | 811 | 162200 | 710.74 | 142148 | 504 |  | 2011 | 160850 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 842 | 168400 | 832.46 | 166492 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 746.74 | 149348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 852 | 170400 | 841.38 | 168276 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.20-2: Test frequencies for NR operating band n20 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 796 | 159200 | 791.68 | 158336 | 0 | 15 | 1990 | 159170 | 14 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 806 | 161200 | 764.96 | 152992 | 102 |  | 2015 | 161290 | 6 | 0 | 3 (8) | 220 |
|  |  |  | High | 816 | 163200 | 630.24 | 126048 | 504 |  | 2040 | 163230 | 10 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 837 | 167400 | 832.68 | 166536 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 661.24 | 132248 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 857 | 171400 | 850.52 | 170104 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 798.5 | 159700 | 791.66 | 158332 | 0 | 15 | 1988 | 159130 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 806 | 161200 | 762.44 | 152488 | 102 |  | 2006 | 160570 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 813.5 | 162700 | 625.22 | 125044 | 504 |  | 2027 | 162250 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 839.5 | 167900 | 832.66 | 166532 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 658.72 | 131744 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 854.5 | 170900 | 845.5 | 169100 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 801 | 160200 | 791.82 | 158364 | 0 | 15 | 1989 | 159150 | 22 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 806 | 161200 | 760.1 | 152020 | 102 |  | 2000 | 160090 | 2 | 0 | 0 (5) | 214 |
|  |  |  | High | 811 | 162200 | 620.38 | 124076 | 504 |  | 2014 | 161090 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 842 | 168400 | 832.82 | 166564 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 847 | 169400 | 656.38 | 131276 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 852 | 170400 | 840.66 | 168132 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled controlResourceSetZero (pdcch-ConfigSIB1 in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.21 to 4.3.1.1.1.23	FFS

4.3.1.1.1.24	Reference test frequencies for NR operating band n24

Table 4.3.1.1.1.24-1a: Test frequencies for NR operating band n24 and SCS 15 kHz

| UL/DL CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 3 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 25 | Downlink | Low | 1528.5 | 305700 | 1526.25 | 305250 | 0 | 15 | 3822 | 305790 | 0 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1533.5 | 306700 | 1512.89 | 302578 | 102 |  | 3833 | 306730 | 4 | 1 | 1 (2) | 105 |
|  |  |  | High | 1533.5 | 306700 | 1440.53 | 288106 | 504 |  | 3833 | 306730 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1630.0 | 326000 | 1627.75 | 325550 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1635.0 | 327000 | 1542.03 | 308406 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1654.0 | 330800 | 1650.67 | 330134 | 6 |  | - | - | - | - | - | - |
| 10/5 (Note 4) | 25 | Downlink | Low Mid | 1528.5 | 305700 | 1526.25 | 305250 | 0 | 15 | 3822 | 305790 | 0 | 1 | 2 (4) | 5 |
|  |  |  | High | 1533.5 | 306700 | 1440.53 | 288106 | 504 |  | 3833 | 306730 | 4 | 1 | 1 (2) | 507 |
|  | 52 | Uplink | Low Mid | 1632.5 | 326500 | 1627.82 | 325564 | 0 | - | - | - | - | - | - | - |
|  |  |  | High | 1651.5 | 330300 | 1645.74 | 329148 | 6 |  | - | - | - | - | - | - |
| 10/10 | 52 | Downlink | Low Mid | 1531.0 | 306200 | 1526.32 | 305264 | 0 | 15 | 3823 | 305810 | 2 | 1 | 2 (4) | 5 |
|  |  |  | High | 1531.0 | 306200 | 1435.6 | 287120 | 504 |  | 3823 | 305810 | 2 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low Mid | 1632.5 | 326500 | 1627.82 | 325564 | 0 | - | - | - | - | - | - | - |
|  |  |  | High | 1651.5 | 330300 | 1645.74 | 329148 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: For symmetric CBW combinations, Low and Mid test frequencies are specified using Tx-Rx spacing of -101.5 MHz and High test frequency is specified using Tx-Rx spacing of -120.5 MHz Note 4: For asymmetric CBW combination UL=10MHz and DL=5MHz, Low and Mid test frequencies are specified using Tx-Rx spacing of -104 MHz and High test frequency is specified using Tx-Rx spacing of -118 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.24-2a: Test frequencies for NR operating band n24 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 3 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low Mid | 1531.0 | 306200 | 1526.68 | 305336 | 0 | 30 | 3829 | 306290 | 6 | 0 | 3 (3) | 6 |
|  |  |  | High | 1531.0 | 306200 | 1345.24 | 269048 | 504 |  | 3829 | 306290 | 6 | 0 | 3 (3) | 1014 |
|  |  | Uplink | Low Mid | 1632.5 | 326500 | 1628.18 | 325636 | 0 | - - | - | - | - | - | - | - |
|  |  |  | High | 1651.5 | 330300 | 1645.02 | 329004 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: Low and Mid test frequencies are specified using Tx-Rx spacing of -101.5 MHz and High test frequency is specified using Tx-Rx spacing of -120.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.24-3a: Test frequencies for NR operating band n24 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low Mid | 1531.0 | 306200 | 1527.04 | 305408 | 0 | 15 | - | 305768 |
|  |  |  | High | 1531.0 | 306200 | 1164.16 | 232832 | 504 |  | - | 305768 |
|  |  | Uplink | Low Mid | 1632.5 | 326500 | 1628.54 | 325708 | 0 | - | - | - |
|  |  |  | High | 1651.5 | 330300 | 1643.22 | 328644 | 6 |  | - | - |
| Note 1: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 2: Low and Mid test frequencies are specified using Tx-Rx spacing of -101.5 MHz and High test frequency is specified using Tx-Rx spacing of -120.5 MHz |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.25	Reference test frequencies for NR operating band n25

Table 4.3.1.1.1.25-1: Test frequencies for NR operating band n25 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1932.5 | 386500 | 1930.25 | 386050 | 0 | 15 | 4829 | 386410 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1962.5 | 392500 | 1941.89 | 388378 | 102 |  | 4904 | 392410 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 1992.5 | 398500 | 1899.53 | 379906 | 504 |  | 4979 | 398410 | 0 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1852.5 | 370500 | 1850.25 | 370050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1789.53 | 357906 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1912.5 | 382500 | 1909.17 | 381834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 1935 | 387000 | 1930.32 | 386064 | 0 | 15 | 4830 | 386430 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1962.5 | 392500 | 1939.46 | 387892 | 102 |  | 4898 | 391930 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 1990 | 398000 | 1894.6 | 378920 | 504 |  | 4969 | 397490 | 10 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 1855 | 371000 | 1850.32 | 370064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1787.1 | 357420 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1910 | 382000 | 1904.24 | 380848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 1937.5 | 387500 | 1930.39 | 386078 | 0 | 15 | 4831 | 386450 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1962.5 | 392500 | 1937.03 | 387406 | 102 |  | 4892 | 391450 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 1987.5 | 397500 | 1889.67 | 377934 | 504 |  | 4956 | 396510 | 0 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 1857.5 | 371500 | 1850.39 | 370078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1784.67 | 356934 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1907.5 | 381500 | 1899.31 | 379862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 1940 | 388000 | 1930.46 | 386092 | 0 | 15 | 4832 | 386650 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1962.5 | 392500 | 1934.6 | 386920 | 102 |  | 4886 | 390970 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 1985 | 397000 | 1884.74 | 376948 | 504 |  | 4943 | 395530 | 2 | 0 | 1 (2) | 506 |
|  |  | Uplink | Low | 1860 | 372000 | 1850.46 | 370092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1782.24 | 356448 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1905 | 381000 | 1894.38 | 378876 | 6 |  | - | - | - | - | - | - |
| 25 | 133 | Downlink | Low | 1942.5 | 388500 | 1930.53 | 386106 | 0 | 15 | 4833 | 386670 | 8 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1962.5 | 392500 | 1932.17 | 386434 | 102 |  | 4880 | 390490 | 8 | 0 | 0 (0) | 102 |
|  |  |  | High | 1982.5 | 396500 | 1879.81 | 375962 | 504 |  | 4933 | 394610 | 0 | 0 | 2 (4) | 508 |
|  |  | Uplink | Low | 1862.5 | 372500 | 1850.53 | 370106 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1779.81 | 355962 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1902.5 | 380500 | 1889.45 | 377890 | 6 |  | - | - | - | - | - | - |
| 30 | 160 | Downlink | Low | 1945 | 389000 | 1930.6 | 386120 | 0 | 15 | 4834 | 386690 | 10 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1962.5 | 392500 | 1929.74 | 385948 | 102 |  | 4874 | 390010 | 10 | 0 | 0 (0) | 102 |
|  |  |  | High | 1980 | 396000 | 1874.88 | 374976 | 504 |  | 4920 | 393630 | 2 | 0 | 2 (4) | 508 |
|  |  | Uplink | Low | 1865 | 373000 | 1850.6 | 370120 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1777.38 | 355476 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1900 | 380000 | 1884.52 | 376904 | 6 |  | - | - | - | - | - | - |
| 40 | 216 | Downlink | Low | 1950 | 390000 | 1930.56 | 386112 | 0 | 15 | 4833 | 386670 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 1962.5 | 392500 | 1924.7 | 384940 | 102 |  | 4862 | 389050 | 2 | 0 | 1 (2) | 104 |
|  |  |  | High | 1975 | 395000 | 1864.84 | 372968 | 504 |  | 4894 | 391490 | 6 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 1870 | 374000 | 1850.56 | 370112 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1772.34 | 354468 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1895 | 379000 | 1874.48 | 374896 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.25-2: Test frequencies for NR operating band n25 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 1935 | 387000 | 1930.68 | 386136 | 0 | 15 | 4836 | 386910 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1962.5 | 392500 | 1921.46 | 384292 | 102 |  | 4904 | 392410 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 1990 | 398000 | 1804.24 | 360848 | 504 |  | 4975 | 397970 | 14 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1855 | 371000 | 1850.68 | 370136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1696.74 | 339348 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1910 | 382000 | 1903.52 | 380704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 1937.5 | 387500 | 1930.66 | 386132 | 0 | 15 | 4837 | 386930 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1962.5 | 392500 | 1918.94 | 383788 | 102 |  | 4898 | 391930 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 1987.5 | 397500 | 1799.22 | 359844 | 504 |  | 4962 | 396990 | 22 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1857.5 | 371500 | 1850.66 | 370132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1694.22 | 338844 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1907.5 | 381500 | 1898.5 | 379700 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 1940 | 388000 | 1930.82 | 386164 | 0 | 15 | 4835 | 386890 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1962.5 | 392500 | 1916.6 | 383320 | 102 |  | 4892 | 391450 | 22 | 0 | 0 (5) | 214 |
|  |  |  | High | 1985 | 397000 | 1794.38 | 358876 | 504 |  | 4949 | 396010 | 18 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1860 | 372000 | 1850.82 | 370164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1691.88 | 338376 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1905 | 381000 | 1893.66 | 378732 | 6 |  | - | - | - | - | - | - |
| 25 | 65 | Downlink | Low | 1942.5 | 388500 | 1930.8 | 386160 | 0 | 15 | 4836 | 386910 | 10 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1962.5 | 392500 | 1914.08 | 382816 | 102 |  | 4886 | 390970 | 6 | 0 | 1 (6) | 216 |
|  |  |  | High | 1982.5 | 396500 | 1789.36 | 357872 | 504 |  | 4939 | 395090 | 22 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1862.5 | 372500 | 1850.8 | 370160 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1689.36 | 337872 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1902.5 | 380500 | 1888.64 | 377728 | 6 |  | - | - | - | - | - | - |
| 30 | 78 | Downlink | Low | 1945 | 389000 | 1930.96 | 386192 | 0 | 15 | 4837 | 386930 | 6 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1962.5 | 392500 | 1911.74 | 382348 | 102 |  | 4880 | 390490 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 1980 | 396000 | 1784.52 | 356904 | 504 |  | 4926 | 394110 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1865 | 373000 | 1850.96 | 370192 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1687.02 | 337404 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1900 | 380000 | 1883.8 | 376760 | 6 |  | - | - | - | - | - | - |
| 40 | 106 | Downlink | Low | 1950 | 390000 | 1930.92 | 386184 | 0 | 15 | 4836 | 386910 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1962.5 | 392500 | 1906.7 | 381340 | 102 |  | 4868 | 389530 | 18 | 0 | 1 (6) | 216 |
|  |  |  | High | 1975 | 395000 | 1774.48 | 354896 | 504 |  | 4900 | 391970 | 22 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1870 | 374000 | 1850.92 | 370184 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1681.98 | 336396 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1895 | 379000 | 1873.76 | 374752 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.25-3: Test frequencies for NR operating band n25 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 1935 | 387000 | 1931.04 | 386208 | 0 | 15 | - | 386568 |
|  |  |  | Mid | 1962.5 | 392500 | 1885.1 | 377020 | 102 |  | - | 392068 |
|  |  |  | High | 1990 | 398000 | 1623.16 | 324632 | 504 |  | - | 397568 |
|  |  | Uplink | Low | 1855 | 371000 | 1851.04 | 370208 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1515.66 | 303132 | 504 |  | - | - |
|  |  |  | High | 1910 | 382000 | 1901.72 | 380344 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 1937.5 | 387500 | 1931.02 | 386204 | 0 | 15 | - | 386564 |
|  |  |  | Mid | 1962.5 | 392500 | 1882.58 | 376516 | 102 |  | - | 391564 |
|  |  |  | High | 1987.5 | 397500 | 1618.14 | 323628 | 504 |  | - | 396564 |
|  |  | Uplink | Low | 1857.5 | 371500 | 1851.02 | 370204 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1513.14 | 302628 | 504 |  | - | - |
|  |  |  | High | 1907.5 | 381500 | 1896.7 | 379340 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 1940 | 388000 | 1931.36 | 386272 | 0 | 15 | - | 386632 |
|  |  |  | Mid | 1962.5 | 392500 | 1880.42 | 376084 | 102 |  | - | 391132 |
|  |  |  | High | 1985 | 397000 | 1613.48 | 322696 | 504 |  | - | 395632 |
|  |  | Uplink | Low | 1860 | 372000 | 1851.36 | 370272 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1510.98 | 302196 | 504 |  | - | - |
|  |  |  | High | 1905 | 381000 | 1892.04 | 378408 | 6 |  | - | - |
| 25 | 31 | Downlink | Low | 1942.5 | 388500 | 1931.34 | 386268 | 0 | 15 | - | 386628 |
|  |  |  | Mid | 1962.5 | 392500 | 1877.9 | 375580 | 102 |  | - | 390628 |
|  |  |  | High | 1982.5 | 396500 | 1608.46 | 321692 | 504 |  | - | 394628 |
|  |  | Uplink | Low | 1862.5 | 372500 | 1851.34 | 370268 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1508.46 | 301692 | 504 |  | - | - |
|  |  |  | High | 1902.5 | 380500 | 1887.02 | 377404 | 6 |  | - | - |
| 30 | 38 | Downlink | Low | 1945 | 389000 | 1931.32 | 386264 | 0 | 15 | - | 386624 |
|  |  |  | Mid | 1962.5 | 392500 | 1875.38 | 375076 | 102 |  | - | 390124 |
|  |  |  | High | 1980 | 396000 | 1603.44 | 320688 | 504 |  | - | 393624 |
|  |  | Uplink | Low | 1865 | 373000 | 1851.32 | 370264 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1505.94 | 301188 | 504 |  | - | - |
|  |  |  | High | 1900 | 380000 | 1882 | 376400 | 6 |  | - | - |
| 40 | 51 | Downlink | Low | 1950 | 390000 | 1931.64 | 386328 | 0 | 15 | - | 386688 |
|  |  |  | Mid | 1962.5 | 392500 | 1870.7 | 374140 | 102 |  | - | 389188 |
|  |  |  | High | 1975 | 395000 | 1593.76 | 318752 | 504 |  | - | 391688 |
|  |  | Uplink | Low | 1870 | 374000 | 1851.64 | 370328 | 0 | - | - | - |
|  |  |  | Mid | 1882.5 | 376500 | 1501.26 | 300252 | 504 |  | - | - |
|  |  |  | High | 1895 | 379000 | 1872.32 | 374464 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.26	Reference test frequencies for NR operating band n26

Table 4.3.1.1.1.26-1: Test frequencies for NR operating band n26 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 861.5 | 172300 | 859.25 | 171850 | 0 | 15 | 2153 | 172330 | 4 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 876.5 | 175300 | 855.89 | 171178 | 102 |  | 2189 | 175210 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 891.5 | 178300 | 798.53 | 159706 | 504 |  | 2228 | 178330 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 816.5 | 163300 | 814.25 | 162850 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 738.53 | 147706 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 846.5 | 169300 | 843.17 | 168634 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 864 | 172800 | 859.32 | 171864 | 0 | 15 | 2154 | 172350 | 6 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 876.5 | 175300 | 853.46 | 170692 | 102 |  | 2183 | 174730 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 889 | 177800 | 793.6 | 158720 | 504 |  | 2218 | 177410 | 2 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 819 | 163800 | 814.32 | 162864 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 736.1 | 147220 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 844 | 168800 | 838.24 | 167648 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 866.5 | 173300 | 859.39 | 171878 | 0 | 15 | 2155 | 172370 | 8 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 876.5 | 175300 | 851.03 | 170206 | 102 |  | 2177 | 174250 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 886.5 | 177300 | 788.67 | 157734 | 504 |  | 2205 | 176430 | 4 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 821.5 | 164300 | 814.39 | 162878 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 733.67 | 146734 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 841.5 | 168300 | 833.31 | 166662 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 869 | 173800 | 859.46 | 171892 | 0 | 15 | 2153 | 172330 | 2 | 0 | 1 (2) | 2 |
|  |  |  | Mid | 876.5 | 175300 | 848.6 | 169720 | 102 |  | 2171 | 173770 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 884 | 176800 | 783.74 | 156748 | 504 |  | 2192 | 175450 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 824 | 164800 | 814.46 | 162892 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 731.24 | 146248 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 839 | 167800 | 828.38 | 165676 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.26-2: Test frequencies for NR operating band n26 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 864 | 172800 | 859.68 | 171936 | 0 | 15 | 2160 | 172830 | 10 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 876.5 | 175300 | 835.46 | 167092 | 102 |  | 2189 | 175210 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 889 | 177800 | 703.24 | 140648 | 504 |  | 2224 | 177890 | 6 | 0 | 3 (8) | 1024 |
|  |  | Uplink | Low | 819 | 163800 | 814.68 | 162936 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 645.74 | 129148 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 844 | 168800 | 837.52 | 167504 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 866.5 | 173300 | 859.66 | 171932 | 0 | 15 | 2161 | 172850 | 18 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 876.5 | 175300 | 832.94 | 166588 | 102 |  | 2183 | 174730 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 886.5 | 177300 | 698.22 | 139644 | 504 |  | 2208 | 176670 | 6 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 821.5 | 164300 | 814.66 | 162932 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 643.22 | 128644 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 841.5 | 168300 | 832.5 | 166500 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 869 | 173800 | 859.82 | 171964 | 0 | 15 | 2159 | 172810 | 18 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 876.5 | 175300 | 830.6 | 166120 | 102 |  | 2177 | 174250 | 22 | 0 | 0 (5) | 214 |
|  |  |  | High | 884 | 176800 | 693.38 | 138676 | 504 |  | 2195 | 175690 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 824 | 164800 | 814.82 | 162964 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 831.5 | 166300 | 640.88 | 128176 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 839 | 167800 | 827.66 | 165532 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.27	FFS

4.3.1.1.1.28	Reference test frequencies for NR operating band n28

Table 4.3.1.1.1.28-1: Test frequencies for NR operating band n28 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 760.5 | 152100 | 758.25 | 151650 | 0 | 15 | 1902 | 152190 | 0 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 780.5 | 156100 | 759.89 | 151978 | 102 |  | 1949 | 156010 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 800.5 | 160100 | 707.53 | 141506 | 504 |  | 2002 | 160130 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 705.5 | 141100 | 703.25 | 140650 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 632.53 | 126506 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 745.5 | 149100 | 742.17 | 148434 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 763 | 152600 | 758.32 | 151664 | 0 | 15 | 1903 | 152210 | 2 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 780.5 | 156100 | 757.46 | 151492 | 102 |  | 1943 | 155530 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 798 | 159600 | 702.6 | 140520 | 504 |  | 1989 | 159150 | 6 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 708 | 141600 | 703.32 | 140664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 630.1 | 126020 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 743 | 148600 | 737.24 | 147448 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 765.5 | 153100 | 758.39 | 151678 | 0 | 15 | 1901 | 152170 | 8 | 1 | 1 (2) | 3 |
|  |  |  | Mid | 780.5 | 156100 | 755.03 | 151006 | 102 |  | 1937 | 155050 | 4 | 0 | 0 (0) | 102 |
|  |  |  | High | 795.5 | 159100 | 697.67 | 139534 | 504 |  | 1976 | 158170 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 710.5 | 142100 | 703.39 | 140678 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 627.67 | 125534 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 740.5 | 148100 | 732.31 | 146462 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 768 | 153600 | 758.46 | 151692 | 0 | 15 | 1902 | 152190 | 10 | 1 | 1 (2) | 3 |
| (Note 2) |  |  | Mid | 783 | 156600 | 755.1 | 151020 | 102 |  | 1938 | 155070 | 6 | 0 | 0 (0) | 102 |
|  |  |  | High | 793 | 158600 | 692.74 | 138548 | 504 |  | 1966 | 157250 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 713 | 142600 | 703.46 | 140692 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 728 | 145600 | 627.74 | 125548 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 738 | 147600 | 727.38 | 145476 | 6 |  | - | - | - | - | - | - |
| 30 | 160 | Downlink | Low | 773 | 154600 | 758.6 | 151720 | 0 | 15 | 1901 | 152170 | 6 | 0 | 1 (2) | 2 |
| (Note 4) |  |  | High | 788 | 157600 | 682.88 | 136576 | 504 |  | 1940 | 155290 | 10 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 718 | 143600 | 703.6 | 140720 | 0 | - | - | - | - | - | - | - |
|  |  |  | High | 733 | 146600 | 717.52 | 143504 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: Carrier centre frequency moved for Mid Range and CBW=20 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: No carrier centre frequency specified for Mid Range and CBW=30 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. For test cases specifying Mid range and CBW=30 MHz to be tested, use Low range and CBW=30 MHz instead. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.28-2: Test frequencies for NR operating band n28 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 763 | 152600 | 758.68 | 151736 | 0 | 15 | 1909 | 152690 | 6 | 0 | 3 (8) | 16 |
|  |  |  | Mid | 780.5 | 156100 | 739.46 | 147892 | 102 |  | 1949 | 156010 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 798 | 159600 | 612.24 | 122448 | 504 |  | 1995 | 159630 | 10 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 708 | 141600 | 703.68 | 140736 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 539.74 | 107948 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 743 | 148600 | 736.52 | 147304 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 765.5 | 153100 | 758.66 | 151732 | 0 | 15 | 1907 | 152650 | 18 | 0 | 2 (7) | 14 |
|  |  |  | Mid | 780.5 | 156100 | 736.94 | 147388 | 102 |  | 1943 | 155530 | 2 | 0 | 1 (6) | 216 |
|  |  |  | High | 795.5 | 159100 | 607.22 | 121444 | 504 |  | 1982 | 158650 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 710.5 | 142100 | 703.66 | 140732 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 725.5 | 145100 | 537.22 | 107444 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 740.5 | 148100 | 731.5 | 146300 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 768 | 153600 | 758.82 | 151764 | 0 | 15 | 1908 | 152670 | 14 | 0 | 2 (7) | 14 |
| (Note 2) |  |  | Mid | 783 | 156600 | 737.1 | 147420 | 102 |  | 1944 | 155550 | 22 | 0 | 0 (5) | 214 |
|  |  |  | High | 793 | 158600 | 602.38 | 120476 | 504 |  | 1969 | 157490 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 713 | 142600 | 703.82 | 140764 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 728 | 145600 | 537.38 | 107476 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 738 | 147600 | 726.66 | 145332 | 6 |  | - | - | - | - | - | - |
| 30 | 78 | Downlink | Low | 773 | 154600 | 758.96 | 151792 | 0 | 15 | 1907 | 152650 | 22 | 0 | 1 (6) | 12 |
| (Note 4) |  |  | High | 788 | 157600 | 592.52 | 118504 | 504 |  | 1943 | 155530 | 6 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 718 | 143600 | 703.96 | 140792 | 0 | - | - | - | - | - | - | - |
|  |  |  | High | 733 | 146600 | 716.8 | 143360 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: Carrier centre frequency moved for Mid Range and CBW=20 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: No carrier centre frequency specified for Mid Range and CBW=30 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.29	Reference test frequencies for NR operating band n29 (SDL)

Table 4.3.1.1.1.29-1: Test frequencies for NR operating band n29 and SCS 15 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 719.5 | 143900 | 717.25 | 143450 | 0 | 15 | - | 143810 |  |
|  |  |  | Mid | 722.5 | 144500 | 701.89 | 140378 | 102 |  | - | 144410 |  |
|  |  |  | High | 725.5 | 145100 | 632.53 | 126506 | 504 |  | - | 145010 |  |
| 10 | 52 | Downlink | Low | 722 | 144400 | 717.32 | 143464 | 0 | 15 | - | 143824 |  |
|  |  |  | Mid | 722.5 | 144500 | 699.46 | 139892 | 102 |  | - | 143924 |  |
|  |  |  | High | 723 | 144600 | 627.6 | 125520 | 504 |  | - | 144024 |  |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.29-2: Test frequencies for NR operating band n29 and SCS 30 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 722 | 144400 | 717.68 | 143536 | 0 | 15 | - | 143896 |  |
|  |  |  | Mid | 722.5 | 144500 | 681.46 | 136292 | 102 |  | - | 143996 |  |
|  |  |  | High | 723 | 144600 | 537.24 | 107448 | 504 |  | - | 144096 |  |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.30	Reference test frequencies for NR operating band n30

Table 4.3.1.1.1.30-1: Test frequencies for NR operating band n30 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2352.5 | 470500 | 2350.25 | 470050 | 0 | 15 | 5879 | 470410 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2355 | 471000 | 2334.39 | 466878 | 102 |  | 5886 | 470910 | 0 | 0 | 0 (0) | 102 |
|  |  |  | High | 2357.5 | 471500 | 2264.53 | 452906 | 504 |  | 5893 | 471410 | 0 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 2307.5 | 461500 | 2305.25 | 461050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 2310 | 462000 | 2217.03 | 443406 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2312.5 | 462500 | 2309.17 | 461834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 2355 | 471000 | 2350.32 | 470064 | 0 | 15 | 5880 | 470430 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2355 | 471000 | 2331.96 | 466392 | 102 |  | 5880 | 470430 | 2 | 0 | 0 (0) | 102 |
|  |  |  | High | 2355 | 471000 | 2259.6 | 451920 | 504 |  | 5880 | 470430 | 2 | 0 | 0 (0) | 504 |
|  |  | Uplink | Low | 2310 | 462000 | 2305.32 | 461064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.30-2: Test frequencies for NR operating band n30 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2355 | 471000 | 2350.68 | 470136 | 0 | 15 | 5886 | 470910 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2355 | 471000 | 2313.96 | 462792 | 102 |  | 5886 | 470910 | 18 | 0 | 0 (5) | 214 |
|  |  |  | High | 2355 | 471000 | 2169.24 | 433848 | 504 |  | 5886 | 470910 | 18 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 2310 | 462000 | 2305.68 | 461136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.31 to 4.3.1.1.1.33	FFS

4.3.1.1.1.34	Reference test frequencies for NR operating band n34

Table 4.3.1.1.1.34-1: Test frequencies for NR operating band n34 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2012.5 | 402500 | 2010.25 | 402050 | 0 | 15 | 5032 | 402530 | 4 | 1 | 1 (2) | 3 |
|  |  | & | Mid | 2017.5 | 403500 | 1996.89 | 399378 | 102 |  | 5043 | 403470 | 8 | 1 | 0 (0) | 103 |
|  |  | Uplink | High | 2022.5 | 404500 | 1929.53 | 385906 | 504 |  | 5054 | 404410 | 0 | 0 | 0 (0) | 504 |
| 10 | 52 | Downlink | Low | 2015 | 403000 | 2010.32 | 402064 | 0 | 30 | 5036 | 402970 | 2 | 3 | 0 (2) | 5 |
|  |  | & | Mid | 2017.5 | 403500 | 1994.46 | 398892 | 102 |  | 5043 | 403470 | 2 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2020 | 404000 | 1924.6 | 384920 | 504 |  | 5050 | 403970 | 2 | 3 | 0 (2) | 509 |
| 15 | 79 | Downlink | Low | 2017.5 | 403500 | 2010.39 | 402078 | 0 | 30 | 5037 | 402990 | 4 | 3 | 0 (2) | 5 |
|  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.34-2: Test frequencies for NR operating band n34 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2015 | 403000 | 2010.68 | 402136 | 0 | 30 | 5036 | 402970 | 14 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2017.5 | 403500 | 1976.46 | 395292 | 102 |  | 5043 | 403470 | 14 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2020 | 404000 | 1834.24 | 366848 | 504 |  | 5050 | 403970 | 14 | 0 | 1 (1) | 1010 |
| 15 | 38 | Downlink | Low | 2017.5 | 403500 | 2010.66 | 402132 | 0 | 30 | 5037 | 402990 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.34-3: Test frequencies for NR operating band n34 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2015 | 403000 | 2011.04 | 402208 | 0 | 15 | - | 402568 |  |
|  |  | & | Mid | 2017.5 | 403500 | 1940.1 | 388020 | 102 |  | - | 403068 |  |
|  |  | Uplink | High | 2020 | 404000 | 1653.16 | 330632 | 504 |  | - | 403568 |  |
| 15 | 18 | Downlink | Low | 2017.5 | 403500 | 2011.02 | 402204 | 0 | 15 | - | 402564 |  |
|  |  | & | Mid |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | High |  |  |  |  |  |  |  |  |  |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.35 to 4.3.1.1.1.37	FFS

4.3.1.1.1.38	Reference test frequencies for NR operating band n38

Table 4.3.1.1.1.38-1: Test frequencies for NR operating band n38 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2572.5 | 514500 | 2570.25 | 514050 | 0 | 15 | 6432 | 514590 | 0 | 1 | 2 (4) | 5 |
|  |  | & | Mid | 2592.5 | 518500 | 2571.89 | 514378 | 102 |  | 6479 | 518410 | 0 | 0 | 0 (0) | 102 |
|  |  | Uplink | High | 2617.5 | 523500 | 2524.53 | 504906 | 504 |  | 6543 | 523470 | 8 | 1 | 0 (0) | 505 |
| 10 | 52 | Downlink | Low | 2575 | 515000 | 2570.32 | 514064 | 0 | 30 | 6439 | 515090 | 6 | 2 | 1 (6) | 8 |
|  |  | & | Mid | 2595 | 519000 | 2571.96 | 514392 | 102 |  | 6486 | 518910 | 6 | 1 | 0 (2) | 105 |
|  |  | Uplink | High | 2615 | 523000 | 2519.6 | 503920 | 504 |  | 6536 | 522970 | 2 | 3 | 0 (2) | 509 |
| 15 | 79 | Downlink | Low | 2577.5 | 515500 | 2570.39 | 514078 | 0 | 30 | 6437 | 515050 | 0 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2595 | 519000 | 2569.53 | 513906 | 102 |  | 6480 | 518430 | 8 | 1 | 0 (2) | 105 |
|  |  | Uplink | High | 2612.5 | 522500 | 2514.67 | 502934 | 504 |  | 6526 | 522050 | 0 | 1 | 1 (6) | 511 |
| 20 | 106 | Downlink | Low | 2580 | 516000 | 2570.46 | 514092 | 0 | 30 | 6438 | 515070 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2595 | 519000 | 2567.1 | 513420 | 102 |  | 6474 | 517950 | 10 | 1 | 0 (2) | 105 |
|  |  | Uplink | High | 2610 | 522000 | 2509.74 | 501948 | 504 |  | 6513 | 521070 | 2 | 1 | 1 (6) | 511 |
| 25 | 133 | Downlink | Low | 2582.5 | 516500 | 2570.53 | 514106 | 0 | 30 | 6439 | 515090 | 4 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2595 | 519000 | 2564.67 | 512934 | 102 |  | 6468 | 517470 | 0 | 2 | 0 (2) | 106 |
|  |  | Uplink | High | 2607.5 | 521500 | 2504.81 | 500962 | 504 |  | 6500 | 520090 | 4 | 1 | 1 (6) | 511 |
| 30 | 160 | Downlink | Low | 2585 | 517000 | 2570.6 | 514120 | 0 | 30 | 6437 | 515050 | 10 | 3 | 0 (2) | 5 |
|  |  | & | Mid | 2595 | 519000 | 2562.24 | 512448 | 102 |  | 6462 | 516990 | 2 | 2 | 0 (2) | 106 |
|  |  | Uplink | High | 2605 | 521000 | 2499.88 | 499976 | 504 |  | 6487 | 518930 | 6 | 0 | 0 (2) | 506 |
| 40 | 216 | Downlink | Low | 2590 | 518000 | 2570.56 | 514112 | 0 | 30 | 6439 | 515090 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2595 | 519000 | 2557.2 | 511440 | 102 |  | 6450 | 516030 | 6 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2600 | 520000 | 2489.84 | 497968 | 504 |  | 6461 | 516970 | 10 | 1 | 0 (2) | 507 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 for SSB_SCS = 15kHz and Table 13-3 for SSB_SCS=30kHz in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.38-2: Test frequencies for NR operating band n38 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2575 | 515000 | 2570.68 | 514136 | 0 | 30 | 6439 | 515090 | 6 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 2595 | 519000 | 2553.96 | 510792 | 102 |  | 6486 | 518910 | 18 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2615 | 523000 | 2429.24 | 485848 | 504 |  | 6536 | 522970 | 14 | 0 | 1 (1) | 1010 |
| 15 | 38 | Downlink | Low | 2577.5 | 515500 | 2570.66 | 514132 | 0 | 30 | 6437 | 515050 | 18 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2595 | 519000 | 2551.44 | 510288 | 102 |  | 6480 | 518430 | 2 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2612.5 | 522500 | 2424.22 | 484844 | 504 |  | 6526 | 522050 | 18 | 0 | 2 (2) | 1012 |
| 20 | 51 | Downlink | Low | 2580 | 516000 | 2570.82 | 514164 | 0 | 30 | 6438 | 515070 | 14 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2595 | 519000 | 2549.1 | 509820 | 102 |  | 6474 | 517950 | 22 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2610 | 522000 | 2419.38 | 483876 | 504 |  | 6513 | 521070 | 14 | 0 | 2 (2) | 1012 |
| 25 | 65 | Downlink | Low | 2582.5 | 516500 | 2570.8 | 514160 | 0 | 30 | 6439 | 515090 | 22 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2595 | 519000 | 2546.58 | 509316 | 102 |  | 6468 | 517470 | 6 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2607.5 | 521500 | 2414.36 | 482872 | 504 |  | 6500 | 520090 | 22 | 0 | 2 (2) | 1012 |
| 30 | 78 | Downlink | Low | 2585 | 517000 | 2570.96 | 514192 | 0 | 30 | 6437 | 515050 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2595 | 519000 | 2544.24 | 508848 | 102 |  | 6462 | 516990 | 2 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2605 | 521000 | 2409.52 | 481904 | 504 |  | 6487 | 518930 | 6 | 0 | 0 (0) | 1008 |
| 40 | 106 | Downlink | Low | 2590 | 518000 | 2570.92 | 514184 | 0 | 30 | 6439 | 515090 | 14 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2595 | 519000 | 2539.2 | 507840 | 102 |  | 6450 | 516030 | 18 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2600 | 520000 | 2399.48 | 479896 | 504 |  | 6461 | 516970 | 22 | 0 | 0 (0) | 1008 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.38-3: Test frequencies for NR operating band n38 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2575 | 515000 | 2571.04 | 514208 | 0 | 30 | - | 514928 |
|  |  | & | Mid | 2595 | 519000 | 2517.6 | 503520 | 102 |  | - | 518928 |
|  |  | Uplink | High | 2615 | 523000 | 2248.16 | 449632 | 504 |  | - | 522928 |
| 15 | 18 | Downlink | Low | 2577.5 | 515500 | 2571.02 | 514204 | 0 | 30 | - | 514924 |
|  |  | & | Mid | 2595 | 519000 | 2515.08 | 503016 | 102 |  | - | 518424 |
|  |  | Uplink | High | 2612.5 | 522500 | 2243.14 | 448628 | 504 |  | - | 521924 |
| 20 | 24 | Downlink | Low | 2580 | 516000 | 2571.36 | 514272 | 0 | 30 | - | 514992 |
|  |  | & | Mid | 2595 | 519000 | 2512.92 | 502584 | 102 |  | - | 517992 |
|  |  | Uplink | High | 2610 | 522000 | 2238.48 | 447696 | 504 |  | - | 520992 |
| 25 | 31 | Downlink | Low | 2582.5 | 516500 | 2571.34 | 514268 | 0 | 30 | - | 514988 |
|  |  | & | Mid | 2595 | 519000 | 2510.4 | 502080 | 102 |  | - | 517488 |
|  |  | Uplink | High | 2607.5 | 521500 | 2233.46 | 446692 | 504 |  | - | 519988 |
| 30 | 38 | Downlink | Low | 2585 | 517000 | 2571.32 | 514264 | 0 | 30 | - | 514984 |
|  |  | & | Mid | 2595 | 519000 | 2507.88 | 501576 | 102 |  | - | 516984 |
|  |  | Uplink | High | 2605 | 521000 | 2228.44 | 445688 | 504 |  | - | 518984 |
| 40 | 51 | Downlink | Low | 2590 | 518000 | 2571.64 | 514328 | 0 | 15 | - | 514688 |
|  |  | & | Mid | 2595 | 519000 | 2503.2 | 500640 | 102 |  | - | 515688 |
|  |  | Uplink | High | 2600 | 520000 | 2218.76 | 443752 | 504 |  | - | 516688 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.39	Reference test frequencies for NR operating band n39

Table 4.3.1.1.1.39-1: Test frequencies for NR operating band n39 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1882.5 | 376500 | 1880.25 | 376050 | 0 | 15 | 4707 | 376590 | 0 | 1 | 2 (4) | 5 |
|  |  | & | Mid | 1899.1 | 379820 | 1878.49 | 375698 | 102 |  | 4747 | 379730 | 0 | 0 | 0 (0) | 102 |
|  |  | Uplink | High | 1917.5 | 383500 | 1824.53 | 364906 | 504 |  | 4793 | 383530 | 4 | 1 | 1 (2) | 507 |
| 10 | 52 | Downlink | Low | 1885 | 377000 | 1880.32 | 376064 | 0 | 30 | 4714 | 377090 | 6 | 2 | 1 (6) | 8 |
|  |  | & | Mid | 1900 | 380000 | 1876.96 | 375392 | 102 |  | 4750 | 379970 | 2 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 1915 | 383000 | 1819.6 | 363920 | 504 |  | 4789 | 383090 | 6 | 2 | 1 (6) | 512 |
| 15 | 79 | Downlink | Low | 1887.5 | 377500 | 1880.39 | 376078 | 0 | 30 | 4712 | 377050 | 0 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 1900 | 380000 | 1874.53 | 374906 | 102 |  | 4744 | 379490 | 4 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 1912.5 | 382500 | 1814.67 | 362934 | 504 |  | 4773 | 381870 | 0 | 0 | 0 (2) | 506 |
| 20 | 106 | Downlink | Low | 1890 | 378000 | 1880.46 | 376092 | 0 | 30 | 4713 | 377070 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 1900 | 380000 | 1872.1 | 374420 | 102 |  | 4738 | 379010 | 6 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 1910 | 382000 | 1809.74 | 361948 | 504 |  | 4760 | 380890 | 2 | 0 | 0 (2) | 506 |
| 25 | 133 | Downlink | Low | 1892.5 | 378500 | 1880.53 | 376106 | 0 | 30 | 4714 | 377090 | 4 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 1900 | 380000 | 1869.67 | 373934 | 102 |  | 4732 | 378530 | 8 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 1907.5 | 381500 | 1804.81 | 360962 | 504 |  | 4750 | 379970 | 0 | 2 | 0 (2) | 508 |
| 30 | 160 | Downlink | Low | 1895 | 379000 | 1880.6 | 376120 | 0 | 30 | 4712 | 377050 | 10 | 3 | 0 (2) | 5 |
|  |  | & | Mid | 1900 | 380000 | 1867.24 | 373448 | 102 |  | 4726 | 378050 | 10 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 1905 | 381000 | 1799.88 | 359976 | 504 |  | 4737 | 378990 | 2 | 2 | 0 (2) | 508 |
| 40 | 216 | Downlink | Low | 1900 | 380000 | 1880.56 | 376112 | 0 | 30 | 4714 | 377090 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 for SSB_SCS = 15kHz and Table 13-3 for SSB_SCS=30kHz in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.39-2: Test frequencies for NR operating band n39 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 1885 | 377000 | 1880.68 | 376136 | 0 | 30 | 4714 | 377090 | 6 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 1900 | 380000 | 1858.96 | 371792 | 102 |  | 4750 | 379970 | 14 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 1915 | 383000 | 1729.24 | 345848 | 504 |  | 4789 | 383090 | 6 | 0 | 3 (3) | 1014 |
| 15 | 38 | Downlink | Low | 1887.5 | 377500 | 1880.66 | 376132 | 0 | 30 | 4712 | 377050 | 18 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 1900 | 380000 | 1856.44 | 371288 | 102 |  | 4744 | 379490 | 22 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 1912.5 | 382500 | 1724.22 | 344844 | 504 |  | 4773 | 381870 | 6 | 0 | 0 (0) | 1008 |
| 20 | 51 | Downlink | Low | 1890 | 378000 | 1880.82 | 376164 | 0 | 30 | 4713 | 377070 | 14 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 1900 | 380000 | 1854.1 | 370820 | 102 |  | 4738 | 379010 | 18 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 1910 | 382000 | 1719.38 | 343876 | 504 |  | 4760 | 380890 | 2 | 0 | 0 (0) | 1008 |
| 25 | 65 | Downlink | Low | 1892.5 | 378500 | 1880.8 | 376160 | 0 | 30 | 4714 | 377090 | 22 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 1900 | 380000 | 1851.58 | 370316 | 102 |  | 4732 | 378530 | 2 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 1907.5 | 381500 | 1714.36 | 342872 | 504 |  | 4750 | 379970 | 6 | 0 | 1 (1) | 1010 |
| 30 | 78 | Downlink | Low | 1895 | 379000 | 1880.96 | 376192 | 0 | 30 | 4712 | 377050 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 1900 | 380000 | 1849.24 | 369848 | 102 |  | 4726 | 378050 | 22 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 1905 | 381000 | 1709.52 | 341904 | 504 |  | 4737 | 378990 | 2 | 0 | 1 (1) | 1010 |
| 40 | 106 | Downlink | Low | 1900 | 380000 | 1880.92 | 376184 | 0 | 30 | 4714 | 377090 | 14 | 0 | 2 (2) | 4 |
|  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.39-3: Test frequencies for NR operating band n39 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 1885 | 377000 | 1881.04 | 376208 | 0 | 30 | - | 376928 |  |
|  |  | & | Mid | 1900 | 380000 | 1822.6 | 364520 | 102 |  | - | 379928 |  |
|  |  | Uplink | High | 1915 | 383000 | 1548.16 | 309632 | 504 |  | - | 382928 |  |
| 15 | 18 | Downlink | Low | 1887.5 | 377500 | 1881.02 | 376204 | 0 | 30 | - | 376924 |  |
|  |  | & | Mid | 1900 | 380000 | 1820.08 | 364016 | 102 |  | - | 379424 |  |
|  |  | Uplink | High | 1912.5 | 382500 | 1543.14 | 308628 | 504 |  | - | 381924 |  |
| 20 | 24 | Downlink | Low | 1890 | 378000 | 1881.36 | 376272 | 0 | 30 | - | 376992 |  |
|  |  | & | Mid | 1900 | 380000 | 1817.92 | 363584 | 102 |  | - | 378992 |  |
|  |  | Uplink | High | 1910 | 382000 | 1538.48 | 307696 | 504 |  | - | 380992 |  |
| 25 | 31 | Downlink | Low | 1892.5 | 378500 | 1881.34 | 376268 | 0 | 30 | - | 376988 |  |
|  |  | & | Mid | 1900 | 380000 | 1815.4 | 363080 | 102 |  | - | 378488 |  |
|  |  | Uplink | High | 1907.5 | 381500 | 1533.46 | 306692 | 504 |  | - | 379988 |  |
| 30 | 38 | Downlink | Low | 1895 | 379000 | 1881.32 | 376264 | 0 | 30 | - | 376984 |  |
|  |  | & | Mid | 1900 | 380000 | 1812.88 | 362576 | 102 |  | - | 377984 |  |
|  |  | Uplink | High | 1905 | 381000 | 1528.44 | 305688 | 504 |  | - | 378984 |  |
| 40 | 51 | Downlink | Low | 1900 | 380000 | 1881.64 | 376328 | 0 | 30 | - | 377048 |  |
|  |  | & | Mid |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | High |  |  |  |  |  |  |  |  |  |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.40	Reference test frequencies for NR operating band n40

Table 4.3.1.1.1.40-1: Test frequencies for NR operating band n40 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2302.5 | 460500 | 2300.25 | 460050 | 0 | (Note 3) | - | 460770 | - | - | - | - |
|  |  | & | Mid | 2350 | 470000 | 2329.39 | 465878 | 102 |  | - | 470270 | - | - | - | - |
|  |  | Uplink | High | 2397.5 | 479500 | 2304.53 | 460906 | 504 |  | - | 479770 | - | - | - | - |
| 10 | 52 | Downlink | Low | 2305 | 461000 | 2300.32 | 460064 | 0 | 30 | 5764 | 461090 | 6 | 2 | 1 (6) | 8 |
|  |  | & | Mid | 2350 | 470000 | 2326.96 | 465392 | 102 |  | 5875 | 469970 | 2 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2395 | 479000 | 2299.6 | 459920 | 504 |  | 5989 | 479090 | 6 | 2 | 1 (6) | 512 |
| 15 | 79 | Downlink | Low | 2307.5 | 461500 | 2300.39 | 460078 | 0 | 30 | 5762 | 461050 | 0 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2350 | 470000 | 2324.53 | 464906 | 102 |  | 5869 | 469490 | 4 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2392.5 | 478500 | 2294.67 | 458934 | 504 |  | 5973 | 477870 | 0 | 0 | 0 (2) | 506 |
| 20 | 106 | Downlink | Low | 2310 | 462000 | 2300.46 | 460092 | 0 | 30 | 5763 | 461070 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2350 | 470000 | 2322.1 | 464420 | 102 |  | 5863 | 469010 | 6 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2390 | 478000 | 2289.74 | 457948 | 504 |  | 5960 | 476890 | 2 | 0 | 0 (2) | 506 |
| 25 | 133 | Downlink | Low | 2312.5 | 462500 | 2300.53 | 460106 | 0 | 30 | 5764 | 461090 | 4 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2350 | 470000 | 2319.67 | 463934 | 102 |  | 5857 | 468530 | 8 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2387.5 | 477500 | 2284.81 | 456962 | 504 |  | 5950 | 475970 | 0 | 2 | 0 (2) | 508 |
| 30 | 160 | Downlink | Low | 2315 | 463000 | 2300.6 | 460120 | 0 | 30 | 5762 | 461050 | 10 | 3 | 0 (2) | 5 |
|  |  | & | Mid | 2350 | 470000 | 2317.24 | 463448 | 102 |  | 5851 | 468050 | 10 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 2385 | 477000 | 2279.88 | 455976 | 504 |  | 5937 | 474990 | 2 | 2 | 0 (2) | 508 |
| 40 | 216 | Downlink | Low | 2320 | 464000 | 2300.56 | 460112 | 0 | 30 | 5764 | 461090 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 2350 | 470000 | 2312.2 | 462440 | 102 |  | 5839 | 467090 | 2 | 1 | 1 (6) | 109 |
|  |  | Uplink | High | 2380 | 476000 | 2269.84 | 453968 | 504 |  | 5914 | 473090 | 2 | 1 | 1 (6) | 511 |
| 50 | 270 | Downlink | Low | 2325 | 465000 | 2300.7 | 460140 | 0 | 30 | 5763 | 461070 | 10 | 3 | 0 (2) | 5 |
|  |  | & | Mid | 2350 | 470000 | 2307.34 | 461468 | 102 |  | 5827 | 466130 | 6 | 1 | 1 (6) | 109 |
|  |  | Uplink | High | 2375 | 475000 | 2259.98 | 451996 | 504 |  | 5888 | 471130 | 6 | 1 | 1 (6) | 511 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: No SS/PBCH block fits within the channel bandwidth. The channel bandwidth can only be used as SCell |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.40-2: Test frequencies for NR operating band n40 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2305 | 461000 | 2300.68 | 460136 | 0 | 30 | 5764 | 461090 | 6 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 2350 | 470000 | 2308.96 | 461792 | 102 |  | 5875 | 469970 | 14 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2395 | 479000 | 2209.24 | 441848 | 504 |  | 5989 | 479090 | 6 | 0 | 3 (3) | 1014 |
| 15 | 38 | Downlink | Low | 2307.5 | 461500 | 2300.66 | 460132 | 0 | 30 | 5762 | 461050 | 18 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2306.44 | 461288 | 102 |  | 5869 | 469490 | 22 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2392.5 | 478500 | 2204.22 | 440844 | 504 |  | 5973 | 477870 | 6 | 0 | 0 (0) | 1008 |
| 20 | 51 | Downlink | Low | 2310 | 462000 | 2300.82 | 460164 | 0 | 30 | 5763 | 461070 | 14 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2304.1 | 460820 | 102 |  | 5863 | 469010 | 18 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2390 | 478000 | 2199.38 | 439876 | 504 |  | 5960 | 476890 | 2 | 0 | 0 (0) | 1008 |
| 25 | 65 | Downlink | Low | 2312.5 | 462500 | 2300.8 | 460160 | 0 | 30 | 5764 | 461090 | 22 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2301.58 | 460316 | 102 |  | 5857 | 468530 | 2 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2387.5 | 477500 | 2194.36 | 438872 | 504 |  | 5950 | 475970 | 6 | 0 | 1 (1) | 1010 |
| 30 | 78 | Downlink | Low | 2315 | 463000 | 2300.96 | 460192 | 0 | 30 | 5762 | 461050 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2350 | 470000 | 2299.24 | 459848 | 102 |  | 5851 | 468050 | 22 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2385 | 477000 | 2189.52 | 437904 | 504 |  | 5937 | 474990 | 2 | 0 | 1 (1) | 1010 |
| 40 | 106 | Downlink | Low | 2320 | 464000 | 2300.92 | 460184 | 0 | 30 | 5764 | 461090 | 14 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2294.2 | 458840 | 102 |  | 5839 | 467090 | 14 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2380 | 476000 | 2179.48 | 435896 | 504 |  | 5914 | 473090 | 14 | 0 | 2 (2) | 1012 |
| 50 | 133 | Downlink | Low | 2325 | 465000 | 2301.06 | 460212 | 0 | 30 | 5763 | 461070 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2350 | 470000 | 2289.34 | 457868 | 102 |  | 5827 | 466130 | 18 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2375 | 475000 | 2169.62 | 433924 | 504 |  | 5888 | 471130 | 18 | 0 | 2 (2) | 1012 |
| 60 | 162 | Downlink | Low | 2330 | 466000 | 2300.84 | 460168 | 0 | 30 | 5762 | 461050 | 6 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2284.12 | 456824 | 102 |  | 5812 | 464930 | 14 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2370 | 474000 | 2159.4 | 431880 | 504 |  | 5862 | 468990 | 10 | 0 | 1 (1) | 1010 |
| 80 | 217 | Downlink | Low | 2340 | 468000 | 2300.94 | 460188 | 0 | 30 | 5763 | 461070 | 6 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 2350 | 470000 | 2274.22 | 454844 | 102 |  | 5788 | 463010 | 10 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 2360 | 472000 | 2139.5 | 427900 | 504 |  | 5813 | 465130 | 2 | 0 | 3 (3) | 1014 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.40-3: Test frequencies for NR operating band n40 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2305 | 461000 | 2301.04 | 460208 | 0 | 30 | - | 460928 |  |
|  |  | & | Mid | 2350 | 470000 | 2272.6 | 454520 | 102 |  | - | 469928 |  |
|  |  | Uplink | High | 2395 | 479000 | 2028.16 | 405632 | 504 |  | - | 478928 |  |
| 15 | 18 | Downlink | Low | 2307.5 | 461500 | 2301.02 | 460204 | 0 | 30 | - | 460924 |  |
|  |  | & | Mid | 2350 | 470000 | 2270.08 | 454016 | 102 |  | - | 469424 |  |
|  |  | Uplink | High | 2392.5 | 478500 | 2023.14 | 404628 | 504 |  | - | 477924 |  |
| 20 | 24 | Downlink | Low | 2310 | 462000 | 2301.36 | 460272 | 0 | 30 | - | 460992 |  |
|  |  | & | Mid | 2350 | 470000 | 2267.92 | 453584 | 102 |  | - | 468992 |  |
|  |  | Uplink | High | 2390 | 478000 | 2018.48 | 403696 | 504 |  | - | 476992 |  |
| 25 | 31 | Downlink | Low | 2312.5 | 462500 | 2301.34 | 460268 | 0 | 30 | - | 460988 |  |
|  |  | & | Mid | 2350 | 470000 | 2265.4 | 453080 | 102 |  | - | 468488 |  |
|  |  | Uplink | High | 2387.5 | 477500 | 2013.46 | 402692 | 504 |  | - | 475988 |  |
| 30 | 38 | Downlink | Low | 2315 | 463000 | 2301.32 | 460264 | 0 | 30 | - | 460984 |  |
|  |  | & | Mid | 2350 | 470000 | 2262.88 | 452576 | 102 |  | - | 467984 |  |
|  |  | Uplink | High | 2385 | 477000 | 2008.44 | 401688 | 504 |  | - | 474984 |  |
| 40 | 51 | Downlink | Low | 2320 | 464000 | 2301.64 | 460328 | 0 | 30 | - | 461048 |  |
|  |  | & | Mid | 2350 | 470000 | 2258.2 | 451640 | 102 |  | - | 467048 |  |
|  |  | Uplink | High | 2380 | 476000 | 1998.76 | 399752 | 504 |  | - | 473048 |  |
| 50 | 65 | Downlink | Low | 2325 | 465000 | 2301.6 | 460320 | 0 | 30 | - | 461040 |  |
|  |  | & | Mid | 2350 | 470000 | 2253.16 | 450632 | 102 |  | - | 466040 |  |
|  |  | Uplink | High | 2375 | 475000 | 1988.72 | 397744 | 504 |  | - | 471040 |  |
| 60 | 79 | Downlink | Low | 2330 | 466000 | 2301.56 | 460312 | 0 | 30 | - | 461032 |  |
|  |  | & | Mid | 2350 | 470000 | 2248.12 | 449624 | 102 |  | - | 465032 |  |
|  |  | Uplink | High | 2370 | 474000 | 1978.68 | 395736 | 504 |  | - | 469032 |  |
| 80 | 107 | Downlink | Low | 2340 | 468000 | 2301.48 | 460296 | 0 | 30 | - | 461016 |  |
|  |  | & | Mid | 2350 | 470000 | 2238.04 | 447608 | 102 |  | - | 463016 |  |
|  |  | Uplink | High | 2360 | 472000 | 1958.6 | 391720 | 504 |  | - | 465016 |  |
| Note: FR1 carrier without CORSESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.41	Reference test frequencies for NR operating band n41

Table 4.3.1.1.1.41-1: Test frequencies for NR operating band n41, SCS 15 kHz and ΔFRaster 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 | Downlink | Low | 2501.01 | 500202 | 2496.33 | 499266 | 0 | 15 | 6246 | 499710 | 4 | 0 | 1 (2) | 2 |
|  |  | & | Mid | 2593.005 | 518601 | 2569.965 | 513993 | 102 |  | 6477 | 518190 | 7 | 0 | 2 (4) | 106 |
|  |  | Uplink | High | 2685 | 537000 | 2589.6 | 517920 | 504 |  | 6705 | 536430 | 2 | 0 | 0 (0) | 504 |
| 15 | 79 | Downlink | Low | 2503.5 | 500700 | 2496.39 | 499278 | 0 | 15 | 6246 | 499710 | 0 | 0 | 1 (2) | 2 |
|  |  | & | Mid | 2593.005 | 518601 | 2567.535 | 513507 | 102 |  | 6471 | 517710 | 9 | 0 | 2 (4) | 106 |
|  |  | Uplink | High | 2682.495 | 536499 | 2584.665 | 516933 | 504 |  | 6693 | 535470 | 11 | 0 | 0 (0) | 504 |
| 20 | 106 | Downlink | Low | 2506.005 | 501201 | 2496.465 | 499293 | 0 | 15 | 6246 | 499710 | 7 | 1 | 0 (0) | 1 |
|  |  | & | Mid | 2593.005 | 518601 | 2565.105 | 513021 | 102 |  | 6465 | 517230 | 11 | 0 | 2 (4) | 106 |
|  |  | Uplink | High | 2679.99 | 535998 | 2579.73 | 515946 | 504 |  | 6681 | 534510 | 8 | 1 | 0 (0) | 505 |
| 30 | 160 | Downlink | Low | 2511 | 502200 | 2496.6 | 499320 | 0 | 15 | 6246 | 499710 | 10 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 2593.005 | 518601 | 2560.245 | 512049 | 102 |  | 6453 | 516270 | 3 | 1 | 2 (4) | 107 |
|  |  | Uplink | High | 2674.995 | 534999 | 2569.875 | 513975 | 504 |  | 6657 | 532590 | 1 | 1 | 1 (2) | 507 |
| 40 | 216 | Downlink | Low | 2516.01 | 503202 | 2496.57 | 499314 | 0 | 15 | 6246 | 499710 | 0 | 1 | 0 (0) | 1 |
|  |  | & | Mid | 2593.005 | 518601 | 2555.205 | 511041 | 102 |  | 6441 | 515310 | 7 | 2 | 2 (4) | 108 |
|  |  | Uplink | High | 2670 | 534000 | 2559.84 | 511968 | 504 |  | 6633 | 530670 | 6 | 1 | 2 (4) | 509 |
| 50 | 270 | Downlink | Low | 2521.005 | 504201 | 2496.705 | 499341 | 0 | 15 | 6246 | 499710 | 3 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 2593.005 | 518601 | 2550.345 | 510069 | 102 |  | 6426 | 514110 | 3 | 0 | 0 (0) | 102 |
|  |  | Uplink | High | 2664.99 | 532998 | 2549.97 | 509994 | 504 |  | 6606 | 528510 | 4 | 0 | 0 (0) | 504 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.41-2: Test frequencies for NR operating band n41, SCS 30 kHz and ΔFRaster 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2501.01 | 500202 | 2496.69 | 499338 | 0 | 30 | 6252 | 500190 | 20 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2551.95 | 510390 | 102 |  | 6483 | 518670 | 0 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 2685 | 537000 | 2499.24 | 499848 | 504 |  | 6711 | 536910 | 18 | 0 | 0 (0) | 1008 |
| 15 | 38 | Downlink | Low | 2503.5 | 500700 | 2496.66 | 499332 | 0 | 30 | 6252 | 500190 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2549.43 | 509886 | 102 |  | 6474 | 517950 | 0 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2682.48 | 536496 | 2494.2 | 498840 | 504 |  | 6699 | 535950 | 10 | 0 | 1 (1) | 1010 |
| 20 | 51 | Downlink | Low | 2506.02 | 501204 | 2496.84 | 499368 | 0 | 30 | 6252 | 500190 | 10 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2547.09 | 509418 | 102 |  | 6471 | 517710 | 4 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 2679.99 | 535998 | 2489.37 | 497874 | 504 |  | 6687 | 534990 | 12 | 0 | 1 (1) | 1010 |
| 30 | 78 | Downlink | Low | 2511 | 502200 | 2496.96 | 499392 | 0 | 30 | 6252 | 500190 | 2 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2542.23 | 508446 | 102 |  | 6456 | 516510 | 0 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2674.98 | 534996 | 2479.5 | 495900 | 504 |  | 6663 | 533070 | 6 | 0 | 2 (2) | 1012 |
| 40 | 106 | Downlink | Low | 2516.01 | 503202 | 2496.93 | 499386 | 0 | 30 | 6252 | 500190 | 4 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2537.19 | 507438 | 102 |  | 6444 | 515550 | 16 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2670 | 534000 | 2469.48 | 493896 | 504 |  | 6636 | 530910 | 2 | 0 | 0 (0) | 1008 |
| 50 | 133 | Downlink | Low | 2521.02 | 504204 | 2497.08 | 499416 | 0 | 30 | 6252 | 500190 | 18 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 2592.99 | 518598 | 2532.33 | 506466 | 102 |  | 6432 | 514590 | 20 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2664.99 | 532998 | 2459.61 | 491922 | 504 |  | 6612 | 528990 | 20 | 0 | 0 (0) | 1008 |
| 60 | 162 | Downlink | Low | 2526 | 505200 | 2496.84 | 499368 | 0 | 30 | 6252 | 500190 | 10 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2527.11 | 505422 | 102 |  | 6420 | 513630 | 0 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2659.98 | 531996 | 2449.38 | 489876 | 504 |  | 6588 | 527070 | 14 | 0 | 2 (2) | 1012 |
| 70 | 189 | Downlink | Low | 2531.01 | 506202 | 2496.99 | 499398 | 0 | 30 | 6252 | 500190 | 0 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2522.25 | 504450 | 102 |  | 6408 | 512670 | 4 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2655 | 531000 | 2439.54 | 487908 | 504 |  | 6564 | 525150 | 6 | 0 | 3 (3) | 1014 |
| 80 | 217 | Downlink | Low | 2536.02 | 507204 | 2496.96 | 499392 | 0 | 30 | 6252 | 500190 | 2 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2517.21 | 503442 | 102 |  | 6396 | 511710 | 20 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 2649.99 | 529998 | 2429.49 | 485898 | 504 |  | 6537 | 522990 | 4 | 0 | 1 (1) | 1010 |
| 90 | 245 | Downlink | Low | 2541 | 508200 | 2496.9 | 499380 | 0 | 30 | 6252 | 500190 | 6 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2512.17 | 502434 | 102 |  | 6381 | 510510 | 4 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2644.98 | 528996 | 2419.44 | 483888 | 504 |  | 6513 | 521070 | 10 | 0 | 2 (2) | 1012 |
| 100 | 273 | Downlink | Low | 2546.01 | 509202 | 2496.87 | 499374 | 0 | 30 | 6252 | 500190 | 8 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 2592.99 | 518598 | 2507.13 | 501426 | 102 |  | 6369 | 509550 | 20 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 2640 | 528000 | 2409.42 | 481884 | 504 |  | 6486 | 518910 | 6 | 0 | 0 (0) | 1008 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.41-3: Test frequencies for NR operating band n41, SCS 60 kHz and ΔFRaster 15 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2501.01 | 500202 | 2497.05 | 499410 | 0 | 15 | - | 499770 |
|  |  | & | Mid | 2593.005 | 518601 | 2515.605 | 503121 | 102 |  | - | 518169 |
|  |  | Uplink | High | 2685 | 537000 | 2318.16 | 463632 | 504 |  | - | 536568 |
| 15 | 18 | Downlink | Low | 2503.5 | 500700 | 2497.02 | 499404 | 0 | 15 | - | 499764 |
|  |  | & | Mid | 2593.005 | 518601 | 2513.085 | 502617 | 102 |  | - | 517665 |
|  |  | Uplink | High | 2682.495 | 536499 | 2313.135 | 462627 | 504 |  | - | 535563 |
| 20 | 24 | Downlink | Low | 2506.005 | 501201 | 2497.365 | 499473 | 0 | 15 | - | 499833 |
|  |  | & | Mid | 2593.005 | 518601 | 2510.925 | 502185 | 102 |  | - | 517233 |
|  |  | Uplink | High | 2679.99 | 535998 | 2308.47 | 461694 | 504 |  | - | 534630 |
| 30 | 38 | Downlink | Low | 2511 | 502200 | 2497.32 | 499464 | 0 | 15 | - | 499824 |
|  |  | & | Mid | 2593.005 | 518601 | 2505.885 | 501177 | 102 |  | - | 516225 |
|  |  | Uplink | High | 2674.995 | 534999 | 2298.435 | 459687 | 504 |  | - | 532623 |
| 40 | 51 | Downlink | Low | 2516.01 | 503202 | 2497.65 | 499530 | 0 | 15 | - | 499890 |
|  |  | & | Mid | 2593.005 | 518601 | 2501.205 | 500241 | 102 |  | - | 515289 |
|  |  | Uplink | High | 2670 | 534000 | 2288.76 | 457752 | 504 |  | - | 530688 |
| 50 | 65 | Downlink | Low | 2521.005 | 504201 | 2497.605 | 499521 | 0 | 15 | - | 499881 |
|  |  | & | Mid | 2593.005 | 518601 | 2496.165 | 499233 | 102 |  | - | 514281 |
|  |  | Uplink | High | 2664.99 | 532998 | 2278.71 | 455742 | 504 |  | - | 528678 |
| 60 | 79 | Downlink | Low | 2526 | 505200 | 2497.56 | 499512 | 0 | 15 | - | 499872 |
|  |  | & | Mid | 2593.005 | 518601 | 2491.125 | 498225 | 102 |  | - | 513273 |
|  |  | Uplink | High | 2659.995 | 531999 | 2268.675 | 453735 | 504 |  | - | 526671 |
| 70 | 93 | Downlink | Low | 2531.01 | 506202 | 2497.53 | 499506 | 0 | 15 | - | 499866 |
|  |  | & | Mid | 2593.005 | 518601 | 2486.085 | 497217 | 102 |  | - | 512265 |
|  |  | Uplink | High | 2655 | 531000 | 2258.64 | 451728 | 504 |  | - | 524664 |
| 80 | 107 | Downlink | Low | 2536.005 | 507201 | 2497.485 | 499497 | 0 | 15 | - | 499857 |
|  |  | & | Mid | 2593.005 | 518601 | 2481.045 | 496209 | 102 |  | - | 511257 |
|  |  | Uplink | High | 2649.99 | 529998 | 2248.59 | 449718 | 504 |  | - | 522654 |
| 90 | 121 | Downlink | Low | 2541 | 508200 | 2497.44 | 499488 | 0 | 15 | - | 499848 |
|  |  | & | Mid | 2593.005 | 518601 | 2476.005 | 495201 | 102 |  | - | 510249 |
|  |  | Uplink | High | 2644.995 | 528999 | 2238.555 | 447711 | 504 |  | - | 520647 |
| 100 | 135 | Downlink | Low | 2546.01 | 509202 | 2497.41 | 499482 | 0 | 15 | - | 499842 |
|  |  | & | Mid | 2593.005 | 518601 | 2470.965 | 494193 | 102 |  | - | 509241 |
|  |  | Uplink | High | 2640 | 528000 | 2228.52 | 445704 | 504 |  | - | 518640 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.42 to 4.3.1.1.1.47	FFS

4.3.1.1.1.48	Reference test frequencies for NR operating band n48

Table 4.3.1.1.1.48-1: Test frequencies for NR operating band n48 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] | (Note 4) | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 3552.51 | 636834 | 3550.26 | 636684 | 0 | (Note 3) | - | - | - | - | - | - |
|  |  | & | Mid | 3625.005 | 641667 | 3604.395 | 640293 | 102 |  | - | - | - | - | - | - |
|  |  | Uplink | High | 3697.5 | 646500 | 3604.53 | 640302 | 504 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 3555 | 637000 | 3550.32 | 636688 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (2) | 2 |
|  |  | & | Mid | 3624.99 | 641666 | 3601.95 | 640130 | 102 |  | 7933 | 641664 | 10 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 3694.98 | 646332 | 3599.58 | 639972 | 504 |  | 7982 | 646368 | 0 | 3 | 1 (6) | 513 |
| 15 | 79 | Downlink | Low | 3557.52 | 637168 | 3550.41 | 636694 | 0 | 30 | 7884 | 636960 | 2 | 0 | 0 (2) | 2 |
|  |  | & | Mid | 3624.99 | 641666 | 3599.52 | 639968 | 102 |  | 7931 | 641472 | 4 | 1 | 0 (2) | 105 |
|  |  | Uplink | High | 3692.49 | 646166 | 3594.66 | 639644 | 504 |  | 7978 | 645984 | 4 | 2 | 0 (2) | 508 |
| 20 | 106 | Downlink | Low | 3560.01 | 637334 | 3550.47 | 636698 | 0 | 30 | 7885 | 637056 | 10 | 3 | 1 (6) | 9 |
|  |  | & | Mid | 3624.99 | 641666 | 3597.09 | 639806 | 102 |  | 7930 | 641376 | 10 | 2 | 1 (6) | 110 |
|  |  | Uplink | High | 3690 | 646000 | 3589.74 | 639316 | 504 |  | 7975 | 645696 | 8 | 1 | 1 (6) | 511 |
| 30 | 160 | Downlink | Low | 3565.02 | 637668 | 3550.62 | 636708 | 0 | 30 | 7885 | 637056 | 0 | 3 | 1 (6) | 9 |
|  |  | & | Mid | 3624.99 | 641666 | 3592.23 | 639482 | 102 |  | 7926 | 640992 | 10 | 1 | 0 (2) | 105 |
|  |  | Uplink | High | 3684.99 | 645666 | 3579.87 | 638658 | 504 |  | 7968 | 645024 | 6 | 0 | 1 (6) | 510 |
| 40 | 216 | Downlink | Low | 3570 | 638000 | 3550.56 | 636704 | 0 | 30 | 7885 | 637056 | 4 | 3 | 1 (6) | 9 |
|  |  | & | Mid | 3624.99 | 641666 | 3587.19 | 639146 | 102 |  | 7923 | 640704 | 10 | 1 | 1 (6) | 109 |
|  |  | Uplink | High | 3679.98 | 645332 | 3569.82 | 637988 | 504 |  | 7961 | 644352 | 4 | 0 | 1 (6) | 510 |
| 50 | 270 | Downlink | Low | 3575.01 | 638334 | 3550.71 | 636714 | 0 | 30 |  | 637056 | 31 |  |  |  |
|  |  | (Note 4) | Mid | 3624.99 | 641666 | 3582.33 | 638822 | 102 |  |  | 640320 | 31 |  |  |  |
|  |  | Uplink | High | 3675 | 645000 | 3559.98 | 637332 | 504 |  |  | 643680 | 31 |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-3 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: No SS/PBCH block fits within the channel bandwidth. The channel bandwidth can only be used as SCell. Note 4: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink only SCell part of a CA configuration. As the bandwidth is limited to SCell then absence of CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.48-2: Test frequencies for NR operating band n48 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 3555 | 637000 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 3624.99 | 641666 | 3583.95 | 638930 | 102 |  | 7933 | 641664 | 22 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 3694.98 | 646332 | 3509.22 | 633948 | 504 |  | 7982 | 646368 | 12 | 0 | 3 (3) | 1014 |
| 15 | 38 | Downlink | Low | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 3624.99 | 641666 | 3581.43 | 638762 | 102 |  | 7931 | 641472 | 22 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 3692.49 | 646166 | 3504.21 | 633614 | 504 |  | 7978 | 645984 | 10 | 0 | 1 (1) | 1010 |
| 20 | 51 | Downlink | Low | 3560.01 | 637334 | 3550.83 | 636722 | 0 | 30 | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 3624.99 | 641666 | 3579.09 | 638606 | 102 |  | 7930 | 641376 | 10 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 3690 | 646000 | 3499.38 | 633292 | 504 |  | 7975 | 645696 | 20 | 0 | 2 (2) | 1012 |
| 30 | 78 | Downlink | Low | 3565.02 | 637668 | 3550.98 | 636732 | 0 | 30 | 7885 | 637056 | 12 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 3624.99 | 641666 | 3574.23 | 638282 | 102 |  | 7926 | 640992 | 22 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 3684.99 | 645666 | 3489.51 | 632634 | 504 |  | 7968 | 645024 | 6 | 0 | 2 (2) | 1012 |
| 40 | 106 | Downlink | Low | 3570 | 638000 | 3550.92 | 636728 | 0 | 30 | 7885 | 637056 | 16 | 0 | 3 (3) | 6 |
|  |  | & | Mid | 3624.99 | 641666 | 3569.19 | 637946 | 102 |  | 7923 | 640704 | 22 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 3679.98 | 645332 | 3479.46 | 631964 | 504 |  | 7961 | 644352 | 4 | 0 | 2 (2) | 1012 |
| 50 | 133 | Downlink | Low | 3575.01 | 638334 | 3551.07 | 636738 | 0 | 30 |  | 637056 | 31 |  |  |  |
|  |  | (Note 3) | Mid | 3624.99 | 641666 | 3564.33 | 637622 | 102 |  |  | 640320 | 31 |  |  |  |
|  |  | (Note 3) | High | 3675 | 645000 | 3469.62 | 631308 | 504 |  |  | 643680 | 31 |  |  |  |
| 60 | 162 | Downlink | Low | 3580.02 | 638668 | 3550.86 | 636724 | 0 | 30 |  | 637056 | 31 |  |  |  |
|  |  | (Note 3) | Mid | 3624.99 | 641666 | 3559.11 | 637274 | 102 |  |  | 640032 | 31 |  |  |  |
|  |  | (Note 3) | High | 3669.99 | 644666 | 3459.39 | 630626 | 504 |  |  | 643008 | 31 |  |  |  |
| 70 | 189 | Downlink (Note 3) | Low | 3585 | 639000 | 3550.98 | 636732 | 0 | 30 | - | 637056 | 31 | - | - | - |
|  |  |  | Mid | 3624.99 | 641666 | 3554.25 | 636950 | 102 |  | - | 639648 | 31 | - | - | - |
|  |  |  | High | 3664.98 | 644332 | 3449.52 | 629968 | 504 |  | - | 642336 | 31 | - | - | - |
| 80 | 217 | Downlink | Low | 3590.01 | 639334 | 3550.95 | 636730 | 0 | 30 |  | 637056 | 31 |  |  |  |
|  |  | (Note 3) | Mid | 3624.99 | 641666 | 3549.21 | 636614 | 102 |  |  | 639360 | 31 |  |  |  |
|  |  | (Note 3) | High | 3660 | 644000 | 3439.5 | 629300 | 504 |  |  | 641664 | 31 |  |  |  |
| 90 | 245 | Downlink | Low | 3595.02 | 639668 | 3550.92 | 636728 | 0 | 30 |  | 637056 | 31 |  |  |  |
|  |  | (Note 3) | Mid | 3624.99 | 641666 | 3544.17 | 636278 | 102 |  |  | 638976 | 31 |  |  |  |
|  |  | (Note 3) | High | 3654.99 | 643666 | 3429.45 | 628630 | 504 |  |  | 640992 | 31 |  |  |  |
| 100 | 273 | Downlink | Low | 3600 | 640000 | 3550.86 | 636724 | 0 | 30 |  | 637056 | 31 |  |  |  |
|  |  | (Note 3) | Mid | 3624.99 | 641666 | 3539.13 | 635942 | 102 |  |  | 638688 | 31 |  |  |  |
|  |  | (Note 3) | High | 3649.98 | 643332 | 3419.4 | 627960 | 504 |  |  | 640320 | 31 |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink only SCell part of a CA configuration. As the bandwidth is limited to SCell then absence of CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.48-3: Test frequencies for NR operating band n48 and SCS 60 kHz without CORESET#0 (Note 1)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 3555 | 637000 | 3551.04 | 636736 | 0 | 30 | - | 636976 |
|  |  | & | Mid | 3624.99 | 641666 | 3547.59 | 636506 | 102 |  | - | 641642 |
|  |  | Uplink | High | 3694.98 | 646332 | 3328.14 | 621876 | 504 |  | - | 646308 |
| 15 | 18 | Downlink | Low | 3557.52 | 637168 | 3551.04 | 636736 | 0 | 30 | - | 636976 |
|  |  | & | Mid | 3624.99 | 641666 | 3545.07 | 636338 | 102 |  | - | 641474 |
|  |  | Uplink | High | 3692.49 | 646166 | 3323.13 | 621542 | 504 |  | - | 645974 |
| 20 | 24 | Downlink | Low | 3560.01 | 637334 | 3551.37 | 636758 | 0 | 30 | - | 636998 |
|  |  | & | Mid | 3624.99 | 641666 | 3542.91 | 636194 | 102 |  | - | 641330 |
|  |  | Uplink | High | 3690 | 646000 | 3318.48 | 621232 | 504 |  | - | 645664 |
| 30 | 78 | Downlink | Low | 3565.02 | 637668 | 3550.98 | 636732 | 0 | 30 | 7885 | 637056 |
|  |  | & | Mid | 3624.99 | 641666 | 3574.23 | 638282 | 102 |  | 7926 | 640992 |
|  |  | Uplink | High | 3684.99 | 645666 | 3489.51 | 632634 | 504 |  | 7968 | 645024 |
| 40 | 51 | Downlink | Low | 3570 | 638000 | 3551.64 | 636776 | 0 | 30 | - | 637016 |
|  |  | & | Mid | 3624.99 | 641666 | 3533.19 | 635546 | 102 |  | - | 640682 |
|  |  | Uplink | High | 3679.98 | 645332 | 3298.74 | 619916 | 504 |  | - | 644348 |
| 50 | 65 | Downlink(Note 2) | Low | 3575.01 | 638334 | 3551.61 | 636774 | 0 | 30 | - | 637014 |
|  |  |  | Mid | 3624.99 | 641666 | 3528.15 | 635210 | 102 |  | - | 640346 |
|  |  |  | High | 3675 | 645000 | 3288.72 | 619248 | 504 |  | - | 643680 |
| 60 | 79 | Downlink(Note 2) | Low | 3580.02 | 638668 | 3551.58 | 636772 | 0 | 30 | - | 637012 |
|  |  |  | Mid | 3624.99 | 641666 | 3523.11 | 634874 | 102 |  | - | 640010 |
|  |  |  | High | 3669.99 | 644666 | 3278.67 | 618578 | 504 |  | - | 643010 |
| 70 | 93 | Downlink (Note 2) | Low | 3585 | 639000 | 3551.52 | 636768 | 0 | 30 | - | 637008 |
|  |  |  | Mid | 3624.99 | 641666 | 3518.07 | 634538 | 102 |  | - | 639674 |
|  |  |  | High | 3664.98 | 644332 | 3268.62 | 617908 | 504 |  | - | 642340 |
| 80 | 107 | Downlink(Note 2) | Low | 3590.01 | 639334 | 3551.49 | 636766 | 0 | 30 | - | 637006 |
|  |  |  | Mid | 3624.99 | 641666 | 3513.03 | 634202 | 102 |  | - | 639338 |
|  |  |  | High | 3660 | 644000 | 3258.6 | 617240 | 504 |  | - | 641672 |
| 90 | 121 | Downlink(Note 2) | Low | 3595.02 | 639668 | 3551.46 | 636764 | 0 | 30 | - | 637004 |
|  |  |  | Mid | 3624.99 | 641666 | 3507.99 | 633866 | 102 |  | - | 639002 |
|  |  |  | High | 3654.99 | 643666 | 3248.55 | 616570 | 504 |  | - | 641002 |
| 100 | 135 | Downlink(Note 2) | Low | 3600 | 640000 | 3551.4 | 636760 | 0 | 30 | - | 637000 |
|  |  |  | Mid | 3624.99 | 641666 | 3502.95 | 633530 | 102 |  | - | 638666 |
|  |  |  | High | 3649.98 | 643332 | 3238.5 | 615900 | 504 |  | - | 640332 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 2: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink only SCell part of a CA configuration. |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.49	FFS

4.3.1.1.1.50	Reference test frequencies for NR operating band n50

Table 4.3.1.1.1.50-1: Test frequencies for NR operating band n50 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset[RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1434.5 | 286900 | 1432.25 | 286450 | 0 | (Note 3) | - | 287170 | - | - | - | - |
|  |  | & | Mid | 1474.5 | 294900 | 1453.89 | 290778 | 102 |  | - | 295170 | - | - | - | - |
|  |  | Uplink | High | 1514.5 | 302900 | 1421.53 | 284306 | 504 |  | - | 303170 | - | - | - | - |
| 10 | 52 | Downlink | Low | 1437 | 287400 | 1432.32 | 286464 | 0 | 30 | 3591 | 287310 | 6 | 1 | 0 (2) | 3 |
|  |  | & | Mid | 1474.5 | 294900 | 1451.46 | 290292 | 102 |  | 3687 | 294990 | 6 | 2 | 1 (6) | 110 |
|  |  | Uplink | High | 1512 | 302400 | 1416.6 | 283320 | 504 |  | 3780 | 302430 | 10 | 0 | 1 (6) | 510 |
| 15 | 79 | Downlink | Low | 1439.5 | 287900 | 1432.39 | 286478 | 0 | 30 | 3592 | 287330 | 8 | 1 | 0 (2) | 3 |
|  |  | & | Mid | 1474.5 | 294900 | 1449.03 | 289806 | 102 |  | 3678 | 294270 | 0 | 0 | 0 (2) | 104 |
|  |  | Uplink | High | 1509.5 | 301900 | 1411.67 | 282334 | 504 |  | 3767 | 301450 | 0 | 1 | 1 (6) | 511 |
| 20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 30 | 3590 | 287290 | 2 | 0 | 0 (2) | 2 |
|  |  | & | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | 3672 | 293790 | 2 | 0 | 0 (2) | 104 |
|  |  | Uplink | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | 3754 | 300290 | 2 | 0 | 0 (2) | 506 |
| 40 | 216 | Downlink | Low | 1452 | 290400 | 1432.56 | 286512 | 0 | 30 | 3591 | 287310 | 2 | 0 | 0 (2) | 2 |
|  |  | & | Mid | 1474.5 | 294900 | 1436.7 | 287340 | 102 |  | 3648 | 291870 | 10 | 1 | 0 (2) | 105 |
|  |  | Uplink | High | 1497 | 299400 | 1386.84 | 277368 | 504 |  | 3705 | 296430 | 6 | 3 | 0 (2) | 509 |
| 50 | 270 | Downlink | Low | 1457 | 291400 | 1432.7 | 286540 | 0 | 30 | 3593 | 287530 | 6 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 1474.5 | 294900 | 1431.84 | 286368 | 102 |  | 3636 | 290910 | 2 | 2 | 0 (2) | 106 |
|  |  | Uplink | High | 1492 | 298400 | 1376.98 | 275396 | 504 |  | 3682 | 294530 | 6 | 1 | 1 (6) | 511 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: No SS/PBCH block fits within the channel bandwidth. The channel bandwidth can only be used as SCell |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.50-2: Test frequencies for NR operating band n50 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 1437 | 287400 | 1432.68 | 286536 | 0 | 30 | 3591 | 287310 | 18 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 1474.5 | 294900 | 1433.46 | 286692 | 102 |  | 3687 | 294990 | 6 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 1512 | 302400 | 1326.24 | 265248 | 504 |  | 3780 | 302430 | 10 | 0 | 2 (2) | 1012 |
| 15 | 38 | Downlink | Low | 1439.5 | 287900 | 1432.66 | 286532 | 0 | 30 | 3592 | 287330 | 2 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 1474.5 | 294900 | 1430.94 | 286188 | 102 |  | 3678 | 294270 | 6 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 1509.5 | 301900 | 1321.22 | 264244 | 504 |  | 3767 | 301450 | 18 | 0 | 2 (2) | 1012 |
| 20 | 51 | Downlink | Low | 1442 | 288400 | 1432.82 | 286564 | 0 | 30 | 3590 | 287290 | 2 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 1474.5 | 294900 | 1428.6 | 285720 | 102 |  | 3672 | 293790 | 2 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 1507 | 301400 | 1316.38 | 263276 | 504 |  | 3754 | 300290 | 2 | 0 | 0 (0) | 1008 |
| 40 | 106 | Downlink | Low | 1452 | 290400 | 1432.92 | 286584 | 0 | 30 | 3591 | 287310 | 2 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 1474.5 | 294900 | 1418.7 | 283740 | 102 |  | 3648 | 291870 | 22 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 1497 | 299400 | 1296.48 | 259296 | 504 |  | 3705 | 296430 | 18 | 0 | 1 (1) | 1010 |
| 50 | 133 | Downlink | Low | 1457 | 291400 | 1433.06 | 286612 | 0 | 30 | 3593 | 287530 | 18 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 1474.5 | 294900 | 1413.84 | 282768 | 102 |  | 3636 | 290910 | 2 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 1492 | 298400 | 1286.62 | 257324 | 504 |  | 3682 | 294530 | 18 | 0 | 2 (2) | 1012 |
| 60 | 162 | Downlink | Low | 1462 | 292400 | 1432.84 | 286568 | 0 | 30 | 3592 | 287330 | 14 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 1474.5 | 294900 | 1408.62 | 281724 | 102 |  | 3624 | 289950 | 6 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 1487 | 297400 | 1276.4 | 255280 | 504 |  | 3653 | 292330 | 14 | 0 | 0 (0) | 1008 |
| 80 | 217 | Downlink | Low | 1472 | 294400 | 1432.94 | 286588 | 0 | 30 | 3593 | 287530 | 2 | 0 | 3 (3) | 6 |
| (Note 3) |  |  | Mid | 1474.5 | 294900 | 1398.72 | 279744 | 102 |  | 3600 | 288030 | 2 | 0 | 3 (3) | 210 |
|  |  |  | High | 1477 | 295400 | 1256.5 | 251300 | 504 |  | 3607 | 288530 | 2 | 0 | 3 (3) | 1014 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: This UE channel bandwidth is applicable only to downlink (TS 38.101-1 table 5.3.5-1). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.50-3: Test frequencies for NR operating band n50 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 1437 | 287400 | 1433.04 | 286608 | 0 | 30 | - | 287328 |
|  |  | & | Mid | 1474.5 | 294900 | 1397.1 | 279420 | 102 |  | - | 294828 |
|  |  | Uplink | High | 1512 | 302400 | 1145.16 | 229032 | 504 |  | - | 302328 |
| 15 | 18 | Downlink | Low | 1439.5 | 287900 | 1433.02 | 286604 | 0 | 30 | - | 287324 |
|  |  | & | Mid | 1474.5 | 294900 | 1394.58 | 278916 | 102 |  | - | 294324 |
|  |  | Uplink | High | 1509.5 | 301900 | 1140.14 | 228028 | 504 |  | - | 301324 |
| 20 | 24 | Downlink | Low | 1442 | 288400 | 1433.36 | 286672 | 0 | 30 | - | 287392 |
|  |  | & | Mid | 1474.5 | 294900 | 1392.42 | 278484 | 102 |  | - | 293892 |
|  |  | Uplink | High | 1507 | 301400 | 1135.48 | 227096 | 504 |  | - | 300392 |
| 40 | 51 | Downlink | Low | 1452 | 290400 | 1433.64 | 286728 | 0 | 30 | - | 287448 |
|  |  | & | Mid | 1474.5 | 294900 | 1382.7 | 276540 | 102 |  | - | 291948 |
|  |  | Uplink | High | 1497 | 299400 | 1115.76 | 223152 | 504 |  | - | 296448 |
| 50 | 65 | Downlink | Low | 1457 | 291400 | 1433.6 | 286720 | 0 | 30 | - | 287440 |
|  |  | & | Mid | 1474.5 | 294900 | 1377.66 | 275532 | 102 |  | - | 290940 |
|  |  | Uplink | High | 1492 | 298400 | 1105.72 | 221144 | 504 |  | - | 294440 |
| 60 | 79 | Downlink | Low | 1462 | 292400 | 1433.56 | 286712 | 0 | 30 | - | 287432 |
|  |  | & | Mid | 1474.5 | 294900 | 1372.62 | 274524 | 102 |  | - | 289932 |
|  |  | Uplink | High | 1487 | 297400 | 1095.68 | 219136 | 504 |  | - | 292432 |
| 80 | 107 | Downlink | Low | 1472 | 294400 | 1433.48 | 286696 | 0 | 30 | - | 287416 |
| (Note 2) |  |  | Mid | 1474.5 | 294900 | 1362.54 | 272508 | 102 |  | - | 287916 |
|  |  |  | High | 1477 | 295400 | 1075.6 | 215120 | 504 |  | - | 288416 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 2: This UE channel bandwidth is applicable only to downlink (TS 38.101-1 table 5.3.5-1). |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.51	Reference test frequencies for NR operating band n51

Table 4.3.1.1.1.51-1: Test frequencies for NR operating band n51 and SCS 15 kHz

| [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1429.5 | 285900 | 1427.25 | 285450 | 0 | 15 | 3573 | 285870 | 8 | 1 | 0 (0) | 1 |
|  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Uplink | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.52	FFS

4.3.1.1.1.53	Reference test frequencies for NR operating band n53

Table 4.3.1.1.1.53-1: Test frequencies for NR operating band n53 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2486 | 497200 | 2483.75 | 496750 | 0 | 15 | 6215 | 497290 | 0 | 1 | 2 (4) | 5 |
|  |  | & | Mid | 2489.3 | 497860 | 2468.69 | 493738 | 102 |  | 6221 | 497770 | 0 | 0 | 0 (0) | 102 |
|  |  | Uplink | High | 2492.5 | 498500 | 2399.53 | 479906 | 504 |  | 6232 | 498530 | 4 | 1 | 1 (2) | 507 |
| 10 | 52 | Downlink | Low | 2488.5 | 497700 | 2483.82 | 496764 | 0 | 15 | 6216 | 497310 | 2 | 1 | 2 (4) | 5 |
|  |  | & | Mid | 2489.3 | 497860 | 2466.6 | 493252 | 102 |  | 6215 | 497290 | 2 | 0 | 0 (0) | 102 |
|  |  | Uplink | High | 2490 | 498000 | 2394.6 | 478920 | 504 |  | 6219 | 497550 | 6 | 1 | 1 (2) | 507 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.53-2: Test frequencies for NR operating band n53 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2488.5 | 497700 | 2484.18 | 496836 | 0 | 15 | 6222 | 497790 | 6 | 0 | 3 (8) | 16 |
|  |  | & | Mid | 2489.3 | 497860 | 2448.26 | 489652 | 102 |  | 6221 | 497770 | 18 | 0 | 0 (5) | 214 |
|  |  | Uplink | High | 2490 | 498000 | 2304.24 | 460848 | 504 |  | 6225 | 498030 | 10 | 0 | 2 (7) | 1022 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.53-3: Test frequencies for NR operating band n53 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2488.5 | 497700 | 2484.54 | 496908 | 0 | 15 | - | 497268 |
|  |  | & | Mid | 2489.3 | 497860 | 2411.9 | 482380 | 102 |  | - | 497428 |
|  |  | Uplink | High | 2490 | 498000 | 2123.16 | 424632 | 504 |  | - | 497568 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.54 to 4.3.1.1.1.64	FFS

4.3.1.1.1.65	Reference test frequencies for NR operating band n65

Table 4.3.1.1.1.65-1: Test frequencies for NR operating band n65 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 2112.5 | 422500 | 2110.25 | 422050 | 0 | 15 | 5279 | 422410 | 0 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2155 | 431000 | 2134.39 | 426878 | 102 |  | 5389 | 431090 | 0 | 1 | 2 (4) | 107 |
|  |  |  | High | 2197.5 | 439500 | 2104.53 | 420906 | 504 |  | 5493 | 439470 | 8 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 1922.5 | 384500 | 1920.25 | 384050 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1872.03 | 374406 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2007.5 | 401500 | 2004.17 | 400834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 2115 | 423000 | 2110.32 | 422064 | 0 | 15 | 5280 | 422430 | 2 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2155 | 431000 | 2131.96 | 426392 | 102 |  | 5383 | 430610 | 2 | 1 | 2 (4) | 107 |
|  |  |  | High | 2195 | 439000 | 2099.6 | 419920 | 504 |  | 5480 | 438490 | 10 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 1925 | 385000 | 1920.32 | 384064 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1869.6 | 373920 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2005 | 401000 | 1999.24 | 399848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 2117.5 | 423500 | 2110.39 | 422078 | 0 | 15 | 5281 | 422450 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 2155 | 431000 | 2129.53 | 425906 | 102 |  | 5377 | 430130 | 4 | 1 | 2 (4) | 107 |
|  |  |  | High | 2192.5 | 438500 | 2094.67 | 418934 | 504 |  | 5470 | 437570 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1927.5 | 385500 | 1920.39 | 384078 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1867.17 | 373434 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2002.5 | 400500 | 1994.31 | 398862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
|  |  |  | Mid | 2155 | 431000 | 2127.1 | 425420 | 102 |  | 5371 | 429650 | 6 | 1 | 2 (4) | 107 |
|  |  |  | High | 2190 | 438000 | 2089.74 | 417948 | 504 |  | 5457 | 436590 | 10 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1930 | 386000 | 1920.46 | 384092 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1864.74 | 372948 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2000 | 400000 | 1989.38 | 397876 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.65-2: Test frequencies for NR operating band n65 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 2115 | 423000 | 2110.68 | 422136 | 0 | 15 | 5286 | 422910 | 18 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2155 | 431000 | 2113.96 | 422792 | 102 |  | 5389 | 431090 | 6 | 0 | 3 (8) | 220 |
|  |  |  | High | 2195 | 439000 | 2009.24 | 401848 | 504 |  | 5486 | 438970 | 14 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 1925 | 385000 | 1920.68 | 384136 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1779.24 | 355848 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2005 | 401000 | 1998.52 | 399704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 2117.5 | 423500 | 2110.66 | 422132 | 0 | 15 | 5287 | 422930 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 2155 | 431000 | 2111.44 | 422288 | 102 |  | 5380 | 430370 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 2192.5 | 438500 | 2004.22 | 400844 | 504 |  | 5476 | 438050 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1927.5 | 385500 | 1920.66 | 384132 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1776.72 | 355344 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2002.5 | 400500 | 1993.5 | 398700 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 2120 | 424000 | 2110.82 | 422164 | 0 | 15 | 5285 | 422890 | 2 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 2155 | 431000 | 2109.1 | 421820 | 102 |  | 5374 | 429890 | 2 | 0 | 0 (5) | 214 |
|  |  |  | High | 2190 | 438000 | 1999.38 | 399876 | 504 |  | 5463 | 437070 | 14 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 1930 | 386000 | 1920.82 | 384164 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1774.38 | 354876 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 2000 | 400000 | 1988.66 | 397732 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.65-3: Test frequencies for NR operating band n65 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 2115 | 423000 | 2111.04 | 422208 | 0 | 15 | - | 422568 |
|  |  |  | Mid | 2155 | 431000 | 2077.6 | 415520 | 102 |  | - | 430568 |
|  |  |  | High | 2195 | 439000 | 1828.16 | 365632 | 504 |  | - | 438568 |
|  |  | Uplink | Low | 1925 | 385000 | 1921.04 | 384208 | 0 | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1598.16 | 319632 | 504 |  | - | - |
|  |  |  | High | 2005 | 401000 | 1996.72 | 399344 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 2117.5 | 423500 | 2111.02 | 422204 | 0 | 15 | - | 422564 |
|  |  |  | Mid | 2155 | 431000 | 2075.08 | 415016 | 102 |  | - | 430064 |
|  |  |  | High | 2192.5 | 438500 | 1823.14 | 364628 | 504 |  | - | 437564 |
|  |  | Uplink | Low | 1927.5 | 385500 | 1921.02 | 384204 | 0 | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1595.64 | 319128 | 504 |  | - | - |
|  |  |  | High | 2002.5 | 400500 | 1991.7 | 398340 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 2120 | 424000 | 2111.36 | 422272 | 0 | 15 | - | 422632 |
|  |  |  | Mid | 2155 | 431000 | 2072.92 | 414584 | 102 |  | - | 429632 |
|  |  |  | High | 2190 | 438000 | 1818.48 | 363696 | 504 |  | - | 436632 |
|  |  | Uplink | Low | 1930 | 386000 | 1921.36 | 384272 | 0 | - | - | - |
|  |  |  | Mid | 1965 | 393000 | 1593.48 | 318696 | 504 |  | - | - |
|  |  |  | High | 2000 | 400000 | 1987.04 | 397408 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.66	Reference test frequencies for NR operating band n66

Table 4.3.1.1.1.66-1: Test frequencies for NR operating band n66, uplink and downlink channel bandwidth combinations and SCS 15 kHz

| UL/DLCBW Combination (Asymmetric BCS) Note 3 | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 5 | 25 | Downlink | Low | 2112.5 | 422500 | 2110.25 | 422050 | 0 | 15 | 5279 | 422410 | 0 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid | 2145 | 429000 | 2124.39 | 424878 | 102 |  | 5361 | 428910 | 0 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2177.5 | 435500 | 2084.53 | 416906 | 504 |  | 5443 | 435410 | 0 | 0 | 0 (0) | 504 |
|  | 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1652.03 | 330406 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1777.5 | 355500 | 1774.17 | 354834 | 6 |  | - | - | - | - | - | - |
| 5/20 | 20 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
| (0,1) |  |  |  | Mid | 2152.5 | 430500 | 2124.6 | 424920 | 102 |  | 5364 | 429150 | 6 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2185 | 437000 | 2084.74 | 416948 | 504 |  | 5446 | 435650 | 6 | 1 | 2 (4) | 509 |
|  | 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1652.03 | 330406 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1777.5 | 355500 | 1774.17 | 354834 | 6 |  | - | - | - | - | - | - |
| 5/25 | 25 | 133 | Downlink | Low | 2122.5 | 424500 | 2110.53 | 422106 | 0 | 15 | 5283 | 422670 | 8 | 1 | 2 (4) | 5 |
| (1) |  |  |  | Mid | 2155 | 431000 | 2124.67 | 424934 | 102 |  | 5365 | 429170 | 8 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2187.5 | 437500 | 2084.81 | 416962 | 504 |  | 5444 | 435610 | 0 | 0 | 2 (4) | 508 |
|  | 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1652.03 | 330406 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1777.5 | 355500 | 1774.17 | 354834 | 6 |  | - | - | - | - | - | - |
| 5/30 | 30 | 160 | Downlink | Low | 2125 | 425000 | 2110.6 | 422120 | 0 | 15 | 5284 | 422690 | 10 | 1 | 2 (4) | 5 |
| (1) |  |  |  | Mid | 2155 | 431000 | 2122.24 | 424448 | 102 |  | 5359 | 428690 | 10 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2185 | 437000 | 2079.88 | 415976 | 504 |  | 5434 | 434690 | 10 | 1 | 2 (4) | 509 |
|  | 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1742.5 | 348500 | 1649.53 | 329906 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1772.5 | 354500 | 1769.17 | 353834 | 6 |  | - | - | - | - | - | - |
| 5/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
| (0,1) |  |  |  | Mid | 2155 | 431000 | 2117.2 | 423440 | 102 |  | 5344 | 427490 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2180 | 436000 | 2069.84 | 413968 | 504 |  | 5405 | 432490 | 6 | 0 | 0 (0) | 504 |
|  | 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1737.5 | 347500 | 1644.53 | 328906 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1762.5 | 352500 | 1759.17 | 351834 | 6 |  | - | - | - | - | - | - |
| 10/10 | 10 | 52 | Downlink | Low | 2115 | 423000 | 2110.32 | 422064 | 0 | 15 | 5280 | 422430 | 2 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid | 2145 | 429000 | 2121.96 | 424392 | 102 |  | 5355 | 428430 | 2 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2175 | 435000 | 2079.6 | 415920 | 504 |  | 5430 | 434430 | 2 | 0 | 0 (0) | 504 |
|  | 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  | - | - | - | - | - | - |
| 10/20 | 20 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
| (0,1) |  |  |  | Mid | 2150 | 430000 | 2122.1 | 424420 | 102 |  | 5357 | 428650 | 6 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2180 | 436000 | 2079.74 | 415948 | 504 |  | 5432 | 434650 | 6 | 1 | 2 (4) | 509 |
|  | 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  | - | - | - | - | - | - |
| 10/25 | 25 | 133 | Downlink | Low | 2122.5 | 424500 | 2110.53 | 422106 | 0 | 15 | 5283 | 422670 | 8 | 1 | 2 (4) | 5 |
| (1) |  |  |  | Mid | 2152.5 | 430500 | 2122.17 | 424434 | 102 |  | 5358 | 428670 | 8 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2182.5 | 436500 | 2079.81 | 415962 | 504 |  | 5433 | 434670 | 8 | 1 | 2 (4) | 509 |
|  | 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  | - | - | - | - | - | - |
| 10/30 | 30 | 160 | Downlink | Low | 2125 | 425000 | 2110.6 | 422120 | 0 | 15 | 5284 | 422690 | 10 | 1 | 2 (4) | 5 |
| (1) |  |  |  | Mid | 2155 | 431000 | 2122.24 | 424448 | 102 |  | 5359 | 428690 | 10 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2185 | 437000 | 2079.88 | 415976 | 504 |  | 5434 | 434690 | 10 | 1 | 2 (4) | 509 |
|  | 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  | - | - | - | - | - | - |
| 10/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
| (0,1) |  |  |  | Mid | 2155 | 431000 | 2117.2 | 423440 | 102 |  | 5344 | 427490 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2180 | 436000 | 2069.84 | 413968 | 504 |  | 5405 | 432490 | 6 | 0 | 0 (0) | 504 |
|  | 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1740 | 348000 | 1644.6 | 328920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1759.24 | 351848 | 6 |  | - | - | - | - | - | - |
| 15/15 | 15 | 79 | Downlink | Low | 2117.5 | 423500 | 2110.39 | 422078 | 0 | 15 | 5281 | 422450 | 4 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid | 2145 | 429000 | 2119.53 | 423906 | 102 |  | 5349 | 427950 | 4 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2172.5 | 434500 | 2074.67 | 414934 | 504 |  | 5417 | 433450 | 4 | 0 | 0 (0) | 504 |
|  | 15 | 79 | Uplink | Low | 1717.5 | 343500 | 1710.39 | 342078 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1647.17 | 329434 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1772.5 | 354500 | 1764.31 | 352862 | 6 |  | - | - | - | - | - | - |
| 20/20 | 20 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2145 | 429000 | 2117.1 | 423420 | 102 |  | 5343 | 427470 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2170 | 434000 | 2069.74 | 413948 | 504 |  | 5407 | 432530 | 2 | 0 | 1 (2) | 506 |
|  | 20 | 106 | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1644.74 | 328948 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1770 | 354000 | 1759.38 | 351876 | 6 |  | - | - | - | - | - | - |
| 20/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
| (0,1) |  |  |  | Mid | 2155 | 431000 | 2117.2 | 423440 | 102 |  | 5344 | 427490 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2180 | 436000 | 2069.84 | 413968 | 504 |  | 5405 | 432490 | 6 | 0 | 0 (0) | 504 |
|  | 20 | 106 | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1644.74 | 328948 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1770 | 354000 | 1759.38 | 351876 | 6 |  | - | - | - | - | - | - |
| 25/25 | 25 | 133 | Downlink | Low | 2122.5 | 424500 | 2110.53 | 422106 | 0 | 15 | 5283 | 422670 | 8 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2145 | 429000 | 2114.67 | 422934 | 102 |  | 5337 | 426990 | 8 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2167.5 | 433500 | 2064.81 | 412962 | 504 |  | 5394 | 431550 | 4 | 0 | 1 (2) | 506 |
|  | 25 | 133 | Uplink | Low | 1722.5 | 344500 | 1710.53 | 342106 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1642.31 | 328462 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1754.45 | 350890 | 6 |  | - | - | - | - | - | - |
| 25/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
| (1) |  |  |  | Mid | 2152.5 | 430500 | 2114.7 | 422940 | 102 |  | 5337 | 426990 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2175 | 435000 | 2064.84 | 412968 | 504 |  | 5394 | 431550 | 2 | 0 | 1 (2) | 506 |
|  | 25 | 133 | Uplink | Low | 1722.5 | 344500 | 1710.53 | 342106 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1642.31 | 328462 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1754.45 | 350890 | 6 |  | - | - | - | - | - | - |
| 30/30 | 30 | 160 | Downlink | Low | 2125 | 425000 | 2110.6 | 422120 | 0 | 15 | 5284 | 422690 | 10 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2145 | 429000 | 2112.24 | 422448 | 102 |  | 5331 | 426510 | 10 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2165 | 433000 | 2059.88 | 411976 | 504 |  | 5381 | 430570 | 6 | 0 | 1 (2) | 506 |
|  | 30 | 160 | Uplink | Low | 1725 | 345000 | 1710.6 | 342120 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1639.88 | 327976 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1749.52 | 349904 | 6 |  | - | - | - | - | - | - |
| 30/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
| (1) |  |  |  | Mid | 2150 | 430000 | 2112.2 | 422440 | 102 |  | 5330 | 426490 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2170 | 434000 | 2059.84 | 411968 | 504 |  | 5383 | 430610 | 10 | 1 | 1 (2) | 507 |
|  | 30 | 160 | Uplink | Low | 1725 | 345000 | 1710.6 | 342120 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1639.88 | 327976 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1749.52 | 349904 | 6 |  | - | - | - | - | - | - |
| 40/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2145 | 429000 | 2107.2 | 421440 | 102 |  | 5319 | 425550 | 2 | 0 | 1 (2) | 104 |
|  |  |  |  | High | 2160 | 432000 | 2049.84 | 409968 | 504 |  | 5358 | 428670 | 6 | 1 | 2 (4) | 509 |
|  | 40 | 216 | Uplink | Low | 1730 | 346000 | 1710.56 | 342112 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1634.84 | 326968 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1760 | 352000 | 1739.48 | 347896 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3 Asymmetric CBW BCS refers to applicable asymmetric UL and DL channel bandwidth combination set as specified in TS 38.101-1 [7], Table 5.3.6-1 for NR band n66. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.66-1A: Void

Table 4.3.1.1.1.66-2: Test frequencies for NR operating band n66, uplink and downlink channel bandwidth combinations and SCS 30 kHz

| UL/DLCBW Combination (Asymmetric BCS) Note 3 | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 10 | 24 | Downlink | Low | 2115 | 423000 | 2110.68 | 422136 | 0 | 15 | 5286 | 422910 | 18 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2145 | 429000 | 2103.96 | 420792 | 102 |  | 5361 | 428910 | 18 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2175 | 435000 | 1989.24 | 397848 | 504 |  | 5436 | 434910 | 18 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  | - | - | - | - | - | - |
| 10/20 | 20 | 51 | Downlink | Low | 2120 | 424000 | 2110.82 | 422164 | 0 | 15 | 5285 | 422890 | 2 | 0 | 0 (5) | 10 |
| (0,1) |  |  |  | Mid | 2150 | 430000 | 2104.1 | 420820 | 102 |  | 5360 | 428890 | 2 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2180 | 436000 | 1989.38 | 397876 | 504 |  | 5435 | 434890 | 2 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  | - | - | - | - | - | - |
| 10/25 | 25 | 65 | Downlink | Low | 2122.5 | 424500 | 2110.8 | 422160 | 0 | 15 | 5286 | 422910 | 10 | 0 | 0 (5) | 10 |
| (1) |  |  |  | Mid | 2152.5 | 430500 | 2104.08 | 420816 | 102 |  | 5361 | 428910 | 10 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2182.5 | 436500 | 1989.36 | 397872 | 504 |  | 5436 | 434910 | 10 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  | - | - | - | - | - | - |
| 10/30 | 30 | 78 | Downlink | Low | 2125 | 425000 | 2110.96 | 422192 | 0 | 15 | 5287 | 422930 | 6 | 0 | 0 (5) | 10 |
| (1) |  |  |  | Mid | 2155 | 431000 | 2104.24 | 420848 | 102 |  | 5362 | 428930 | 6 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2185 | 437000 | 1989.52 | 397904 | 504 |  | 5437 | 434930 | 6 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  | - | - | - | - | - | - |
| 10/40 | 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
| (0,1) |  |  |  | Mid | 2155 | 431000 | 2099.2 | 419840 | 102 |  | 5350 | 427970 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2180 | 436000 | 1979.48 | 395896 | 504 |  | 5411 | 432970 | 22 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1740 | 348000 | 1554.24 | 310848 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1758.52 | 351704 | 6 |  | - | - | - | - | - | - |
| 15/15 | 15 | 38 | Downlink | Low | 2117.5 | 423500 | 2110.66 | 422132 | 0 | 15 | 5287 | 422930 | 2 | 0 | 1 (6) | 12 |
|  |  |  |  | Mid | 2145 | 429000 | 2101.44 | 420288 | 102 |  | 5355 | 428430 | 2 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 2172.5 | 434500 | 1984.22 | 396844 | 504 |  | 5423 | 433930 | 2 | 0 | 1 (6) | 1020 |
|  | 15 | 38 | Uplink | Low | 1717.5 | 343500 | 1710.66 | 342132 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1556.72 | 311344 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1772.5 | 354500 | 1763.5 | 352700 | 6 |  | - | - | - | - | - | - |
| 20/20 | 20 | 51 | Downlink | Low | 2120 | 424000 | 2110.82 | 422164 | 0 | 15 | 5285 | 422890 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2145 | 429000 | 2099.1 | 419820 | 102 |  | 5349 | 427950 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2170 | 434000 | 1979.38 | 395876 | 504 |  | 5413 | 433010 | 18 | 0 | 1 (6) | 1020 |
|  | 20 | 51 | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1554.38 | 310876 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1770 | 354000 | 1758.66 | 351732 | 6 |  | - | - | - | - | - | - |
| 20/40 | 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
| (0,1) |  |  |  | Mid | 2155 | 431000 | 2099.2 | 419840 | 102 |  | 5350 | 427970 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2180 | 436000 | 1979.48 | 395896 | 504 |  | 5411 | 432970 | 22 | 0 | 0 (5) | 1018 |
|  | 20 | 51 | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1554.38 | 310876 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1770 | 354000 | 1758.66 | 351732 | 6 |  | - | - | - | - | - | - |
| 25/25 | 25 | 65 | Downlink | Low | 2122.5 | 424500 | 2110.8 | 422160 | 0 | 15 | 5286 | 422910 | 10 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2145 | 429000 | 2096.58 | 419316 | 102 |  | 5343 | 427470 | 6 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 2167.5 | 433500 | 1974.36 | 394872 | 504 |  | 5400 | 432030 | 2 | 0 | 2 (7) | 1022 |
|  | 25 | 65 | Uplink | Low | 1722.5 | 344500 | 1710.8 | 342160 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1551.86 | 310372 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1753.64 | 350728 | 6 |  | - | - | - | - | - | - |
| 25/40 | 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
| (1) |  |  |  | Mid | 2152.5 | 430500 | 2096.7 | 419340 | 102 |  | 5343 | 427470 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2175 | 435000 | 1974.48 | 394896 | 504 |  | 5400 | 432030 | 18 | 0 | 1 (6) | 1020 |
|  | 25 | 65 | Uplink | Low | 1722.5 | 344500 | 1710.8 | 342160 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1551.86 | 310372 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1753.64 | 350728 | 6 |  | - | - | - | - | - | - |
| 30/30 | 30 | 78 | Downlink | Low | 2125 | 425000 | 2110.96 | 422192 | 0 | 15 | 5287 | 422930 | 6 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2145 | 429000 | 2094.24 | 418848 | 102 |  | 5337 | 426990 | 2 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 2165 | 433000 | 1969.52 | 393904 | 504 |  | 5387 | 431050 | 22 | 0 | 1 (6) | 1020 |
|  | 30 | 78 | Uplink | Low | 1725 | 345000 | 1710.96 | 342192 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1549.52 | 309904 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1748.8 | 349760 | 6 |  | - | - | - | - | - | - |
| 30/40 | 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
| (1) |  |  |  | Mid | 2150 | 430000 | 2094.2 | 418840 | 102 |  | 5336 | 426970 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2170 | 434000 | 1969.48 | 393896 | 504 |  | 5389 | 431090 | 14 | 0 | 2 (7) | 1022 |
|  | 30 | 78 | Uplink | Low | 1725 | 345000 | 1710.96 | 342192 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1549.52 | 309904 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1748.8 | 349760 | 6 |  | - | - | - | - | - | - |
| 40/40 | 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2145 | 429000 | 2089.2 | 417840 | 102 |  | 5325 | 426030 | 18 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 2160 | 432000 | 1959.48 | 391896 | 504 |  | 5361 | 428910 | 2 | 0 | 0 (5) | 1018 |
|  | 40 | 106 | Uplink | Low | 1730 | 346000 | 1710.92 | 342184 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1544.48 | 308896 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1760 | 352000 | 1738.76 | 347752 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3 Asymmetric CBW BCS refers to applicable asymmetric UL and DL channel bandwidth combination set as specified in TS 38.101-1 [7], Table 5.3.6-1 for NR band n66. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.66-2A: Void

Table 4.3.1.1.1.66-3: Test frequencies for NR operating band n66, uplink and downlink channel bandwidth combinations and SCS 60 kHz without CORESET#0

| UL/DLCBW (Asymmetric BCS) Note 2 | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 10 | 11 | Downlink | Low | 2115 | 423000 | 2111.04 | 422208 | 0 | 15 | - | 422568 |
|  |  |  |  | Mid | 2145 | 429000 | 2067.6 | 413520 | 102 |  | - | 428568 |
|  |  |  |  | High | 2175 | 435000 | 1808.16 | 361632 | 504 |  | - | 434568 |
|  | 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1378.16 | 275632 | 504 |  | - | - |
|  |  |  |  | High | 1775 | 355000 | 1766.72 | 353344 | 6 |  | - | - |
| 10/20 | 20 | 24 | Downlink | Low | 2120 | 424000 | 2111.36 | 422272 | 0 | 15 | - | 422632 |
|  |  |  |  | Mid | 2150 | 430000 | 2067.92 | 413584 | 102 |  | - | 428632 |
|  |  |  |  | High | 2180 | 436000 | 1808.48 | 361696 | 504 |  | - | 434632 |
|  | 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1378.16 | 275632 | 504 |  | - | - |
|  |  |  |  | High | 1775 | 355000 | 1766.72 | 353344 | 6 |  | - | - |
| 10/25 | 25 | 31 | Downlink | Low | 2122.5 | 424500 | 2111.34 | 422268 | 0 | 15 | - | 422628 |
| (1) |  |  |  | Mid | 2152.5 | 430500 | 2067.9 | 413580 | 102 |  | - | 428628 |
|  |  |  |  | High | 2182.5 | 436500 | 1808.46 | 361692 | 504 |  | - | 434628 |
|  | 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1378.16 | 275632 | 504 |  | - | - |
|  |  |  |  | High | 1775 | 355000 | 1766.72 | 353344 | 6 |  | - | - |
| 10/30 | 30 | 38 | Downlink | Low | 2125 | 425000 | 2111.32 | 422264 | 0 | 15 | - | 422624 |
| (1) |  |  |  | Mid | 2155 | 431000 | 2067.88 | 413576 | 102 |  | - | 428624 |
|  |  |  |  | High | 2185 | 437000 | 1808.44 | 361688 | 504 |  | - | 434624 |
|  | 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1378.16 | 275632 | 504 |  | - | - |
|  |  |  |  | High | 1775 | 355000 | 1766.72 | 353344 | 6 |  | - | - |
| 10/40 | 40 | 51 | Downlink | Low | 2130 | 426000 | 2111.64 | 422328 | 0 | 15 | - | 422688 |
|  |  |  |  | Mid | 2155 | 431000 | 2063.2 | 412640 | 102 |  | - | 427688 |
|  |  |  |  | High | 2180 | 436000 | 1798.76 | 359752 | 504 |  | - | 432688 |
|  | 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 | - | - | - |
|  |  |  |  | Mid | 1740 | 348000 | 1373.16 | 274632 | 504 |  | - | - |
|  |  |  |  | High | 1765 | 353000 | 1756.72 | 351344 | 6 |  | - | - |
| 15/15 | 15 | 18 | Downlink | Low | 2117.5 | 423500 | 2111.02 | 422204 | 0 | 15 | - | 422564 |
|  |  |  |  | Mid | 2145 | 429000 | 2065.08 | 413016 | 102 |  | - | 428064 |
|  |  |  |  | High | 2172.5 | 434500 | 1803.14 | 360628 | 504 |  | - | 433564 |
|  | 15 | 18 | Uplink | Low | 1717.5 | 343500 | 1711.02 | 342204 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1375.64 | 275128 | 504 |  | - | - |
|  |  |  |  | High | 1772.5 | 354500 | 1761.7 | 352340 | 6 |  | - | - |
| 20/20 | 20 | 24 | Downlink | Low | 2120 | 424000 | 2111.36 | 422272 | 0 | 15 | - | 422632 |
|  |  |  |  | Mid | 2145 | 429000 | 2062.92 | 412584 | 102 |  | - | 427632 |
|  |  |  |  | High | 2170 | 434000 | 1798.48 | 359696 | 504 |  | - | 432632 |
|  | 20 | 24 | Uplink | Low | 1720 | 344000 | 1711.36 | 342272 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1373.48 | 274696 | 504 |  | - | - |
|  |  |  |  | High | 1770 | 354000 | 1757.04 | 351408 | 6 |  | - | - |
| 20/40 | 40 | 51 | Downlink | Low | 2130 | 426000 | 2111.64 | 422328 | 0 | 15 | - | 422688 |
|  |  |  |  | Mid | 2155 | 431000 | 2063.2 | 412640 | 102 |  | - | 427688 |
|  |  |  |  | High | 2180 | 436000 | 1798.76 | 359752 | 504 |  | - | 432688 |
|  | 20 | 24 | Uplink | Low | 1720 | 344000 | 1711.36 | 342272 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1373.48 | 274696 | 504 |  | - | - |
|  |  |  |  | High | 1770 | 354000 | 1757.04 | 351408 | 6 |  | - | - |
| 25/25 | 25 | 31 | Downlink | Low | 2122.5 | 424500 | 2111.34 | 422268 | 0 | 15 | - | 422628 |
|  |  |  |  | Mid | 2145 | 429000 | 2060.4 | 412080 | 102 |  | - | 427128 |
|  |  |  |  | High | 2167.5 | 433500 | 1793.46 | 358692 | 504 |  | - | 431628 |
|  | 25 | 31 | Uplink | Low | 1722.5 | 344500 | 1711.34 | 342268 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1370.96 | 274192 | 504 |  | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1752.02 | 350404 | 6 |  | - | - |
| 25/40 | 40 | 51 | Downlink | Low | 2130 | 426000 | 2111.64 | 422328 | 0 | 15 | - | 422688 |
| (1) |  |  |  | Mid | 2152.5 | 430500 | 2060.7 | 412140 | 102 |  | - | 427188 |
|  |  |  |  | High | 2175 | 435000 | 1793.76 | 358752 | 504 |  | - | 431688 |
|  | 25 | 31 | Uplink | Low | 1722.5 | 344500 | 1711.34 | 342268 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1370.96 | 274192 | 504 |  | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1752.02 | 350404 | 6 |  | - | - |
| 30/30 | 30 | 38 | Downlink | Low | 2125 | 425000 | 2111.32 | 422264 | 0 | 15 | - | 422624 |
|  |  |  |  | Mid | 2145 | 429000 | 2057.88 | 411576 | 102 |  | - | 426624 |
|  |  |  |  | High | 2165 | 433000 | 1788.44 | 357688 | 504 |  | - | 430624 |
|  | 30 | 38 | Uplink | Low | 1725 | 345000 | 1711.32 | 342264 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1368.44 | 273688 | 504 |  | - | - |
|  |  |  |  | High | 1765 | 353000 | 1747 | 349400 | 6 |  | - | - |
| 30/40 | 40 | 51 | Downlink | Low | 2130 | 426000 | 2111.64 | 422328 | 0 | 15 | - | 422688 |
| (1) |  |  |  | Mid | 2150 | 430000 | 2058.2 | 411640 | 102 |  | - | 426688 |
|  |  |  |  | High | 2170 | 434000 | 1788.76 | 357752 | 504 |  | - | 430688 |
|  | 30 | 38 | Uplink | Low | 1725 | 345000 | 1711.32 | 342264 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1368.44 | 273688 | 504 |  | - | - |
|  |  |  |  | High | 1765 | 353000 | 1747 | 349400 | 6 |  | - | - |
| 40/40 | 40 | 51 | Downlink | Low | 2130 | 426000 | 2111.64 | 422328 | 0 | 15 | - | 422688 |
|  |  |  |  | Mid | 2145 | 429000 | 2053.2 | 410640 | 102 |  | - | 425688 |
|  |  |  |  | High | 2160 | 432000 | 1778.76 | 355752 | 504 |  | - | 428688 |
|  | 40 | 51 | Uplink | Low | 1730 | 346000 | 1711.64 | 342328 | 0 | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1363.76 | 272752 | 504 |  | - | - |
|  |  |  |  | High | 1760 | 352000 | 1737.32 | 347464 | 6 |  | - | - |
| Note 1: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 2 Asymmetric CBW BCS refers to applicable asymmetric UL and DL channel bandwidth combination set as specified in TS 38.101-1 [7], Table 5.3.6-1 for NR band n66. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.66-3A: Void

4.3.1.1.1.67 – 4.3.1.1.1.69	FFS

4.3.1.1.1.70	Reference test frequencies for NR operating band n70

Editor’s note:	Test frequencies for the Tx-RX frequency separation of 295 Mhz option as specified in TS 38.101-1, Table 5.4.4-1 is FFS.

Table 4.3.1.1.1.70-1: Test frequencies for NR operating band n70, default Tx-RX frequency separation 300MHz, uplink and downlink channel bandwidth combinations and SCS 15 kHz

| UL/DLBandwidth combination | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 5 | 25 | Downlink | Low | 1997.5 | 399500 | 1995.25 | 399050 | 0 | 15 | 4993 | 399410 | 0 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid | 2002.5 | 400500 | 1981.89 | 396378 | 102 |  | 5007 | 400590 | 0 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2007.5 | 401500 | 1914.53 | 382906 | 504 |  | 5018 | 401530 | 4 | 1 | 1 (2) | 507 |
|  | 5 | 25 | Uplink | Low | 1697.5 | 339500 | 1695.25 | 339050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1609.53 | 321906 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1707.5 | 341500 | 1704.17 | 340834 | 6 |  | - | - | - | - | - | - |
| 5/10 | 10 | 52 | Downlink | Low | 2000 | 400000 | 1995.32 | 399064 | 0 | 15 | 4994 | 399610 | 2 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2005 | 401000 | 1981.96 | 396392 | 102 |  | 5008 | 400610 | 2 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2010 | 402000 | 1914.6 | 382920 | 504 |  | 5019 | 401550 | 6 | 1 | 1 (2) | 507 |
|  | 5 | 25 | Uplink | Low | 1697.5 | 339500 | 1695.25 | 339050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1609.53 | 321906 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1707.5 | 341500 | 1704.17 | 340834 | 6 |  | - | - | - | - | - | - |
| 5/15 | 15 | 79 | Downlink | Low | 2002.5 | 400500 | 1995.39 | 399078 | 0 | 15 | 4995 | 399630 | 4 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2007.5 | 401500 | 1982.03 | 396406 | 102 |  | 5006 | 400570 | 8 | 1 | 1 (2) | 105 |
|  |  |  |  | High | 2012.5 | 402500 | 1914.67 | 382934 | 504 |  | 5020 | 401570 | 8 | 1 | 1 (2) | 507 |
|  | 5 | 25 | Uplink | Low | 1697.5 | 339500 | 1695.25 | 339050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1609.53 | 321906 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1707.5 | 341500 | 1704.17 | 340834 | 6 |  | - | - | - | - | - | - |
| 5/20 | 20 | 106 | Downlink | Low | 2005 | 401000 | 1995.46 | 399092 | 0 | 15 | 4996 | 399650 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2007.5 | 401500 | 1979.6 | 395920 | 102 |  | 5000 | 400090 | 10 | 1 | 1 (2) | 105 |
|  |  |  |  | High | 2010 | 402000 | 1909.74 | 381948 | 504 |  | 5007 | 400590 | 10 | 1 | 1 (2) | 507 |
|  | 5 | 25 | Uplink | Low | 1697.5 | 339500 | 1695.25 | 339050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1700 | 340000 | 1607.03 | 321406 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1702.5 | 340500 | 1699.17 | 339834 | 6 |  | - | - | - | - | - | - |
| 5/25 | 25 | 133 | Downlink | Low | 2007.5 | 401500 | 1995.53 | 399106 | 0 | 15 | 4994 | 399610 | 0 | 0 | 2 (4) | 4 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 | 25 | Uplink | Low | 1697.5 | 339500 | 1695.25 | 339050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 10/10 | 10 | 52 | Downlink | Low | 2000 | 400000 | 1995.32 | 399064 | 0 | 15 | 4994 | 399610 | 2 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2002.5 | 400500 | 1979.46 | 395892 | 102 |  | 5001 | 400110 | 2 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2005 | 401000 | 1909.6 | 381920 | 504 |  | 5008 | 400610 | 2 | 1 | 2 (4) | 509 |
|  | 10 | 52 | Uplink | Low | 1700 | 340000 | 1695.32 | 339064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1607.1 | 321420 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1705 | 341000 | 1699.24 | 339848 | 6 |  | - | - | - | - | - | - |
| 10/20 | 20 | 106 | Downlink | Low | 2005 | 401000 | 1995.46 | 399092 | 0 | 15 | 4996 | 399650 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2007.5 | 401500 | 1979.6 | 395920 | 102 |  | 5000 | 400090 | 10 | 1 | 1 (2) | 105 |
|  |  |  |  | High | 2010 | 402000 | 1909.74 | 381948 | 504 |  | 5007 | 400590 | 10 | 1 | 1 (2) | 507 |
|  | 10 | 52 | Uplink | Low | 1700 | 340000 | 1695.32 | 339064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1607.1 | 321420 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1705 | 341000 | 1699.24 | 339848 | 6 |  | - | - | - | - | - | - |
| 10/25 | 25 | 133 | Downlink | Low | 2007.5 | 401500 | 1995.53 | 399106 | 0 | 15 | 4994 | 399610 | 0 | 0 | 2 (4) | 4 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 | 52 | Uplink | Low | 1700 | 340000 | 1695.32 | 339064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/15 | 15 | 79 | Downlink | Low | 2002.5 | 400500 | 1995.39 | 399078 | 0 | 15 | 4995 | 399630 | 4 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 | 79 | Uplink | Low | 1702.5 | 340500 | 1695.39 | 339078 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/20 | 20 | 106 | Downlink | Low | 2005 | 401000 | 1995.46 | 399092 | 0 | 15 | 4996 | 399650 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 | 79 | Uplink | Low | 1702.5 | 340500 | 1695.39 | 339078 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/25 | 25 | 133 | Downlink | Low | 2007.5 | 401500 | 1995.53 | 399106 | 0 | 15 | 4994 | 399610 | 0 | 0 | 2 (4) | 4 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 | 79 | Uplink | Low | 1702.5 | 340500 | 1695.39 | 339078 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.70-2: Test frequencies for NR operating band n70, default Tx-RX frequency separation 300MHz, uplink and downlink channel bandwidth combinations and SCS 30 kHz

| UL/DLBandwidth combination | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 10 | 24 | Downlink | Low | 2000 | 400000 | 1995.68 | 399136 | 0 | 15 | 5000 | 400090 | 6 | 0 | 3 (8) | 16 |
|  |  |  |  | Mid | 2002.5 | 400500 | 1961.46 | 392292 | 102 |  | 5007 | 400590 | 6 | 0 | 3 (8) | 220 |
|  |  |  |  | High | 2005 | 401000 | 1819.24 | 363848 | 504 |  | 5014 | 401090 | 6 | 0 | 3 (8) | 1024 |
|  | 10 | 24 | Uplink | Low | 1700 | 340000 | 1695.68 | 339136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1516.74 | 303348 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1705 | 341000 | 1698.52 | 339704 | 6 |  | - | - | - | - | - | - |
| 10/20 | 20 | 51 | Downlink | Low | 2005 | 401000 | 1995.82 | 399164 | 0 | 15 | 4999 | 399890 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2007.5 | 401500 | 1961.6 | 392320 | 102 |  | 5006 | 400570 | 14 | 0 | 2 (7) | 218 |
|  |  |  |  | High | 2010 | 402000 | 1819.38 | 363876 | 504 |  | 5013 | 401070 | 14 | 0 | 2 (7) | 1022 |
|  | 10 | 24 | Uplink | Low | 1700 | 340000 | 1695.68 | 339136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1516.74 | 303348 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1705 | 341000 | 1698.52 | 339704 | 6 |  | - | - | - | - | - | - |
| 10/25 | 25 | 65 | Downlink | Low | 2007.5 | 401500 | 1995.8 | 399160 | 0 | 15 | 5000 | 400090 | 22 | 0 | 2 (7) | 14 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 | 24 | Uplink | Low | 1700 | 340000 | 1695.68 | 339136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/15 | 15 | 38 | Downlink | Low | 2002.5 | 400500 | 1995.66 | 399132 | 0 | 15 | 4998 | 399870 | 6 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 | 38 | Uplink | Low | 1702.5 | 340500 | 1695.66 | 339132 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/20 | 20 | 51 | Downlink | Low | 2005 | 401000 | 1995.82 | 399164 | 0 | 15 | 4999 | 399890 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 | 38 | Uplink | Low | 1702.5 | 340500 | 1695.66 | 339132 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| 15/25 | 25 | 65 | Downlink | Low | 2007.5 | 401500 | 1995.8 | 399160 | 0 | 15 | 5000 | 400090 | 22 | 0 | 2 (7) | 14 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 15 | 38 | Uplink | Low | 1702.5 | 340500 | 1695.66 | 339132 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1 in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.70-3: Test frequencies for NR operating band n70, default Tx-RX frequency separation 300MHz, uplink and downlink channel bandwidth combinations and SCS 60 kHz without CORESET#0

| UL/DLBandwidth combination | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 10 | 11 | Downlink | Low | 2000 | 400000 | 1996.04 | 399208 | 0 | 15 | - | 399568 |
|  |  |  |  | Mid | 2002.5 | 400500 | 1925.1 | 385020 | 102 |  | - | 400068 |
|  |  |  |  | High | 2005 | 401000 | 1638.16 | 327632 | 504 |  | - | 400568 |
|  | 10 | 11 | Uplink | Low | 1700 | 340000 | 1696.04 | 339208 | 0 | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1335.66 | 267132 | 504 |  | - | - |
|  |  |  |  | High | 1705 | 341000 | 1696.72 | 339344 | 6 |  | - | - |
| 10/20 | 20 | 24 | Downlink | Low | 2005 | 401000 | 1996.36 | 399272 | 0 | 15 | - | 399632 |
|  |  |  |  | Mid | 2007.5 | 401500 | 1925.42 | 385084 | 102 |  | - | 400132 |
|  |  |  |  | High | 2010 | 402000 | 1638.48 | 327696 | 504 |  | - | 400632 |
|  | 10 | 11 | Uplink | Low | 1700 | 340000 | 1696.04 | 339208 | 0 | - | - | - |
|  |  |  |  | Mid | 1702.5 | 340500 | 1335.66 | 267132 | 504 |  | - | - |
|  |  |  |  | High | 1705 | 341000 | 1696.72 | 339344 | 6 |  | - | - |
| 10/25 | 25 | 31 | Downlink | Low | 2007.5 | 401500 | 1996.34 | 399268 | 0 | 15 | - | 399628 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |
|  | 10 | 11 | Uplink | Low | 1700 | 340000 | 1696.04 | 339208 | 0 | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |
| 15/15 | 15 | 18 | Downlink | Low | 2002.5 | 400500 | 1996.02 | 399204 | 0 | 15 | - | 399564 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |
|  | 15 | 18 | Uplink | Low | 1702.5 | 340500 | 1696.02 | 339204 | 0 | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |
| 15/20 | 20 | 24 | Downlink | Low | 2005 | 401000 | 1996.36 | 399272 | 0 | 15 | - | 399632 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |
|  | 15 | 18 | Uplink | Low | 1702.5 | 340500 | 1696.02 | 339204 | 0 | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |
| 15/25 | 25 | 31 | Downlink | Low | 2007.5 | 401500 | 1996.34 | 399268 | 0 | 15 | - | 399628 |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |
|  | 15 | 18 | Uplink | Low | 1702.5 | 340500 | 1696.02 | 339204 | 0 | - | - | - |
|  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  |  |  |  | High |  |  |  |  |  |  |  |  |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.71	Reference test frequencies for NR operating band n71

Table 4.3.1.1.1.71-1: Test frequencies for NR operating band n71 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 619.5 | 123900 | 617.25 | 123450 | 0 | 15 | 1548 | 123870 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 634.5 | 126900 | 613.89 | 122778 | 102 |  | 1587 | 126990 | 0 | 1 | 2 (4) | 107 |
|  |  |  | High | 649.5 | 129900 | 556.53 | 111306 | 504 |  | 1623 | 129870 | 8 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 665.5 | 133100 | 663.25 | 132650 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 587.53 | 117506 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 695.5 | 139100 | 692.17 | 138434 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 622 | 124400 | 617.32 | 123464 | 0 | 15 | 1549 | 123890 | 10 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 634.5 | 126900 | 611.46 | 122292 | 102 |  | 1581 | 126510 | 2 | 1 | 2 (4) | 107 |
|  |  |  | High | 647 | 129400 | 551.6 | 110320 | 504 |  | 1610 | 128890 | 10 | 1 | 0 (0) | 505 |
|  |  | Uplink | Low | 668 | 133600 | 663.32 | 132664 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 585.1 | 117020 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 693 | 138600 | 687.24 | 137448 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 624.5 | 124900 | 617.39 | 123478 | 0 | 15 | 1547 | 123850 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 634.5 | 126900 | 609.03 | 121806 | 102 |  | 1575 | 126030 | 4 | 1 | 2 (4) | 107 |
|  |  |  | High | 644.5 | 128900 | 546.67 | 109334 | 504 |  | 1600 | 127970 | 8 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 670.5 | 134100 | 663.39 | 132678 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 582.67 | 116534 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 690.5 | 138100 | 682.31 | 136462 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 627 | 125400 | 617.46 | 123492 | 0 | 15 | 1548 | 123870 | 6 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 634.5 | 126900 | 606.6 | 121320 | 102 |  | 1569 | 125550 | 6 | 1 | 2 (4) | 107 |
|  |  |  | High | 642 | 128400 | 541.74 | 108348 | 504 |  | 1587 | 126990 | 10 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 673 | 134600 | 663.46 | 132692 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 580.24 | 116048 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 688 | 137600 | 677.38 | 135476 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.71-2: Test frequencies for NR operating band n71 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 622 | 124400 | 617.68 | 123536 | 0 | 15 | 1555 | 124370 | 14 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 634.5 | 126900 | 593.46 | 118692 | 102 |  | 1587 | 126990 | 6 | 0 | 3 (8) | 220 |
|  |  |  | High | 647 | 129400 | 461.24 | 92248 | 504 |  | 1616 | 129370 | 14 | 0 | 1 (6) | 1020 |
|  |  | Uplink | Low | 668 | 133600 | 663.68 | 132736 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 494.74 | 98948 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 693 | 138600 | 686.52 | 137304 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 624.5 | 124900 | 617.66 | 123532 | 0 | 15 | 1553 | 124330 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 634.5 | 126900 | 590.94 | 118188 | 102 |  | 1578 | 126270 | 6 | 0 | 0 (5) | 214 |
|  |  |  | High | 644.5 | 128900 | 456.22 | 91244 | 504 |  | 1606 | 128450 | 18 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 670.5 | 134100 | 663.66 | 132732 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 492.22 | 98444 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 690.5 | 138100 | 681.5 | 136300 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 627 | 125400 | 617.82 | 123564 | 0 | 15 | 1554 | 124350 | 22 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 634.5 | 126900 | 588.6 | 117720 | 102 |  | 1572 | 125790 | 2 | 0 | 0 (5) | 214 |
|  |  |  | High | 642 | 128400 | 451.38 | 90276 | 504 |  | 1593 | 127470 | 14 | 0 | 2 (7) | 1022 |
|  |  | Uplink | Low | 673 | 134600 | 663.82 | 132764 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 680.5 | 136100 | 489.88 | 97976 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 688 | 137600 | 676.66 | 135332 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.72 – 4.3.1.1.1.73

4.3.1.1.1.74	Reference test frequencies for NR operating band n74

Table 4.3.1.1.1.74-1: Test frequencies for NR operating band n74 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1477.5 | 295500 | 1475.25 | 295050 | 0 | 15 | 3693 | 295470 | 8 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 1496.5 | 299300 | 1475.89 | 295178 | 102 |  | 3742 | 299330 | 4 | 1 | 1 (2) | 105 |
|  |  |  | High | 1515.5 | 303100 | 1422.53 | 284506 | 504 |  | 3788 | 303130 | 4 | 1 | 1 (2) | 507 |
|  |  | Uplink | Low | 1429.5 | 285900 | 1427.25 | 285450 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1355.53 | 271106 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1467.5 | 293500 | 1464.17 | 292834 | 6 |  | - | - | - | - | - | - |
| 10 | 52 | Downlink | Low | 1480 | 296000 | 1475.32 | 295064 | 0 | 15 | 3694 | 295490 | 10 | 1 | 0 (0) | 1 |
|  |  |  | Mid | 1496.5 | 299300 | 1473.46 | 294692 | 102 |  | 3736 | 298850 | 6 | 1 | 1 (2) | 105 |
|  |  |  | High | 1513 | 302600 | 1417.6 | 283520 | 504 |  | 3778 | 302210 | 2 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 1432 | 286400 | 1427.32 | 285464 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1353.1 | 270620 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1465 | 293000 | 1459.24 | 291848 | 6 |  | - | - | - | - | - | - |
| 15 | 79 | Downlink | Low | 1482.5 | 296500 | 1475.39 | 295078 | 0 | 15 | 3692 | 295450 | 4 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1496.5 | 299300 | 1471.03 | 294206 | 102 |  | 3730 | 298370 | 8 | 1 | 1 (2) | 105 |
|  |  |  | High | 1510.5 | 302100 | 1412.67 | 282534 | 504 |  | 3765 | 301230 | 4 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 1434.5 | 286900 | 1427.39 | 285478 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1350.67 | 270134 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1462.5 | 292500 | 1454.31 | 290862 | 6 |  | - | - | - | - | - | - |
| 20 | 106 | Downlink | Low | 1485 | 297000 | 1475.46 | 295092 | 0 | 15 | 3693 | 295470 | 6 | 0 | 0 (0) | 0 |
|  |  |  | Mid | 1496.5 | 299300 | 1468.6 | 293720 | 102 |  | 3724 | 297890 | 10 | 1 | 1 (2) | 105 |
|  |  |  | High | 1508 | 301600 | 1407.74 | 281548 | 504 |  | 3752 | 300250 | 6 | 1 | 2 (4) | 509 |
|  |  | Uplink | Low | 1437 | 287400 | 1427.46 | 285492 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1348.24 | 269648 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1460 | 292000 | 1449.38 | 289876 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.74-2: Test frequencies for NR operating band n74 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 1480 | 296000 | 1475.68 | 295136 | 0 | 15 | 3700 | 295970 | 14 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1496.5 | 299300 | 1455.46 | 291092 | 102 |  | 3742 | 299330 | 10 | 0 | 2 (7) | 218 |
|  |  |  | High | 1513 | 302600 | 1327.24 | 265448 | 504 |  | 3784 | 302690 | 6 | 0 | 3 (8) | 1024 |
|  |  | Uplink | Low | 1432 | 286400 | 1427.68 | 285536 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1262.74 | 252548 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1465 | 293000 | 1458.52 | 291704 | 6 |  | - | - | - | - | - | - |
| 15 | 38 | Downlink | Low | 1482.5 | 296500 | 1475.66 | 295132 | 0 | 15 | 3698 | 295930 | 2 | 0 | 1 (6) | 12 |
|  |  |  | Mid | 1496.5 | 299300 | 1452.94 | 290588 | 102 |  | 3736 | 298850 | 18 | 0 | 2 (7) | 218 |
|  |  |  | High | 1510.5 | 302100 | 1322.22 | 264444 | 504 |  | 3768 | 301470 | 6 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1434.5 | 286900 | 1427.66 | 285532 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1260.22 | 252044 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1462.5 | 292500 | 1453.5 | 290700 | 6 |  | - | - | - | - | - | - |
| 20 | 51 | Downlink | Low | 1485 | 297000 | 1475.82 | 295164 | 0 | 15 | 3699 | 295950 | 22 | 0 | 0 (5) | 10 |
|  |  |  | Mid | 1496.5 | 299300 | 1450.6 | 290120 | 102 |  | 3730 | 298370 | 14 | 0 | 2 (7) | 218 |
|  |  |  | High | 1508 | 301600 | 1317.38 | 263476 | 504 |  | 3755 | 300490 | 2 | 0 | 0 (5) | 1018 |
|  |  | Uplink | Low | 1437 | 287400 | 1427.82 | 285564 | 0 | - | - | - | - | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1257.88 | 251576 | 504 |  | - | - | - | - | - | - |
|  |  |  | High | 1460 | 292000 | 1448.66 | 289732 | 6 |  | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1 in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.74-3: Test frequencies for NR operating band n74 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 1480 | 296000 | 1476.04 | 295208 | 0 | 15 | - | 295568 |
|  |  |  | Mid | 1496.5 | 299300 | 1419.1 | 283820 | 102 |  | - | 298868 |
|  |  |  | High | 1513 | 302600 | 1146.16 | 229232 | 504 |  | - | 302168 |
|  |  | Uplink | Low | 1432 | 286400 | 1428.04 | 285608 | 0 | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1081.66 | 216332 | 504 |  | - | - |
|  |  |  | High | 1465 | 293000 | 1456.72 | 291344 | 6 |  | - | - |
| 15 | 18 | Downlink | Low | 1482.5 | 296500 | 1476.02 | 295204 | 0 | 15 | - | 295564 |
|  |  |  | Mid | 1496.5 | 299300 | 1416.58 | 283316 | 102 |  | - | 298364 |
|  |  |  | High | 1510.5 | 302100 | 1141.14 | 228228 | 504 |  | - | 301164 |
|  |  | Uplink | Low | 1434.5 | 286900 | 1428.02 | 285604 | 0 | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1079.14 | 215828 | 504 |  | - | - |
|  |  |  | High | 1462.5 | 292500 | 1451.7 | 290340 | 6 |  | - | - |
| 20 | 24 | Downlink | Low | 1485 | 297000 | 1476.36 | 295272 | 0 | 15 | - | 295632 |
|  |  |  | Mid | 1496.5 | 299300 | 1414.42 | 282884 | 102 |  | - | 297932 |
|  |  |  | High | 1508 | 301600 | 1136.48 | 227296 | 504 |  | - | 300232 |
|  |  | Uplink | Low | 1437 | 287400 | 1428.36 | 285672 | 0 | - | - | - |
|  |  |  | Mid | 1448.5 | 289700 | 1076.98 | 215396 | 504 |  | - | - |
|  |  |  | High | 1460 | 292000 | 1447.04 | 289408 | 6 |  | - | - |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.75	Reference test frequencies for NR operating band n75 (SDL)

Table 4.3.1.1.1.75-1: Test frequencies for NR operating band n75 and SCS 15 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1434.5 | 286900 | 1432.25 | 286450 | 0 | 15 | - | 286810 |  |
|  |  |  | Mid | 1474.5 | 294900 | 1453.89 | 290778 | 102 |  | - | 294810 |  |
|  |  |  | High | 1514.5 | 302900 | 1421.53 | 284306 | 504 |  | - | 302810 |  |
| 10 | 52 | Downlink | Low | 1437 | 287400 | 1432.32 | 286464 | 0 | 15 | - | 286824 |  |
|  |  |  | Mid | 1474.5 | 294900 | 1451.46 | 290292 | 102 |  | - | 294324 |  |
|  |  |  | High | 1512 | 302400 | 1416.6 | 283320 | 504 |  | - | 301824 |  |
| 15 | 79 | Downlink | Low | 1439.5 | 287900 | 1432.39 | 286478 | 0 | 15 | - | 286838 |  |
|  |  |  | Mid | 1474.5 | 294900 | 1449.03 | 289806 | 102 |  | - | 293838 |  |
|  |  |  | High | 1509.5 | 301900 | 1411.67 | 282334 | 504 |  | - | 300838 |  |
| 20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 15 | - | 286852 |  |
|  |  |  | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | - | 293352 |  |
|  |  |  | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | - | 299852 |  |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.75-2: Test frequencies for NR operating band n75 and SCS 30 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 1437 | 287400 | 1432.68 | 286536 | 0 | 15 | - | 286896 |  |
|  |  |  | Mid | 1474.5 | 294900 | 1433.46 | 286692 | 102 |  | - | 294396 |  |
|  |  |  | High | 1512 | 302400 | 1326.24 | 265248 | 504 |  | - | 301896 |  |
| 15 | 38 | Downlink | Low | 1439.5 | 287900 | 1432.66 | 286532 | 0 | 15 | - | 286892 |  |
|  |  |  | Mid | 1474.5 | 294900 | 1430.94 | 286188 | 102 |  | - | 293892 |  |
|  |  |  | High | 1509.5 | 301900 | 1321.22 | 264244 | 504 |  | - | 300892 |  |
| 20 | 51 | Downlink | Low | 1442 | 288400 | 1432.82 | 286564 | 0 | 15 | - | 286924 |  |
|  |  |  | Mid | 1474.5 | 294900 | 1428.6 | 285720 | 102 |  | - | 293424 |  |
|  |  |  | High | 1507 | 301400 | 1316.38 | 263276 | 504 |  | - | 299924 |  |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.75-3: Test frequencies for NR operating band n75 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 1437 | 287400 | 1433.04 | 286608 | 0 | 15 | - | 286968 |  |
|  |  |  | Mid | 1474.5 | 294900 | 1397.1 | 279420 | 102 |  | - | 294468 |  |
|  |  |  | High | 1512 | 302400 | 1145.16 | 229032 | 504 |  | - | 301968 |  |
| 15 | 18 | Downlink | Low | 1439.5 | 287900 | 1433.02 | 286604 | 0 | 15 | - | 286964 |  |
|  |  |  | Mid | 1474.5 | 294900 | 1394.58 | 278916 | 102 |  | - | 293964 |  |
|  |  |  | High | 1509.5 | 301900 | 1140.14 | 228028 | 504 |  | - | 300964 |  |
| 20 | 24 | Downlink | Low | 1442 | 288400 | 1433.36 | 286672 | 0 | 15 | - | 287032 |  |
|  |  |  | Mid | 1474.5 | 294900 | 1392.42 | 278484 | 102 |  | - | 293532 |  |
|  |  |  | High | 1507 | 301400 | 1135.48 | 227096 | 504 |  | - | 300032 |  |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.76	Reference test frequencies for NR operating band n76 (SDL)

Table 4.3.1.1.1.76-1: Test frequencies for NR operating band n76 and SCS 15 kHz without CORESET#0

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low, Mid, High | 1429.5 | 285900 | 1427.25 | 285450 | 0 | 15 | - | 285810 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.77	Reference test frequencies for NR operating band n77

Table 4.3.1.1.1.77-1: Test frequencies for NR operating band n77, SCS 15 kHz and ΔFRaster 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 | Downlink | Low | 3305.01 | 620334 | 3300.33 | 620022 | 0 | 30 | 7711 | 620352 | 6 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 3750 | 650000 | 3726.96 | 648464 | 102 |  | 8020 | 650016 | 4 | 1 | 1 (6) | 109 |
|  |  | Uplink | High | 4194.99 | 679666 | 4099.59 | 673306 | 504 |  | 8329 | 679680 | 2 | 1 | 1 (6) | 511 |
| 15 | 79 | Downlink | Low | 3307.5 | 620500 | 3300.39 | 620026 | 0 | 30 | 7711 | 620352 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 3750 | 650000 | 3724.53 | 648302 | 102 |  | 8018 | 649824 | 10 | 2 | 0 (2) | 106 |
|  |  | Uplink | High | 4192.5 | 679500 | 4094.67 | 672978 | 504 |  | 8325 | 679296 | 6 | 0 | 0 (2) | 506 |
| 20 | 106 | Downlink | Low | 3310.02 | 620668 | 3300.48 | 620032 | 0 | 30 | 7711 | 620352 | 8 | 0 | 1 (6) | 6 |
|  |  | & | Mid | 3750 | 650000 | 3722.1 | 648140 | 102 |  | 8016 | 649632 | 4 | 0 | 0 (2) | 104 |
|  |  | Uplink | High | 4189.98 | 679332 | 4089.72 | 672648 | 504 |  | 8322 | 679008 | 0 | 0 | 1 (6) | 510 |
| 25 | 133 | Downlink | Low | 3312.51 | 620834 | 3300.54 | 620036 | 0 | 30 | 7711 | 620352 | 4 | 0 | 1 (6) | 6 |
|  |  | & | Mid | 3750 | 650000 | 3719.67 | 647978 | 102 |  | 8015 | 649536 | 10 | 1 | 1 (6) | 109 |
|  |  | Uplink | High | 4187.49 | 679166 | 4084.8 | 672320 | 504 |  | 8319 | 678720 | 4 | 3 | 1 (6) | 513 |
| 30 | 160 | Downlink | Low | 3315 | 621000 | 3300.6 | 620040 | 0 | 30 | 7711 | 620352 | 0 | 0 | 1 (6) | 6 |
|  |  | & | Mid | 3750 | 650000 | 3717.24 | 647816 | 102 |  | 8013 | 649344 | 4 | 3 | 0 (2) | 107 |
|  |  | Uplink | High | 4185 | 679000 | 4079.88 | 671992 | 504 |  | 8315 | 678336 | 8 | 2 | 0 (2) | 508 |
| 40 | 216 | Downlink | Low | 3320.01 | 621334 | 3300.57 | 620038 | 0 | 30 | 7711 | 620352 | 2 | 0 | 1 (6) | 6 |
|  |  | & | Mid | 3750 | 650000 | 3712.2 | 647480 | 102 |  | 8010 | 649056 | 4 | 3 | 1 (6) | 111 |
|  |  | Uplink | High | 4179.99 | 678666 | 4069.83 | 671322 | 504 |  | 8308 | 677664 | 6 | 2 | 0 (2) | 508 |
| 50 | 270 | Downlink | Low | 3325.02 | 621668 | 3300.72 | 620048 | 0 | 30 | 7711 | 620352 | 4 | 3 | 0 (2) | 5 |
|  |  | & | Mid | 3750 | 650000 | 3707.34 | 647156 | 102 |  | 8006 | 648672 | 4 | 2 | 0 (2) | 106 |
|  |  | Uplink | High | 4174.98 | 678332 | 4059.96 | 670664 | 504 |  | 8301 | 676992 | 4 | 1 | 0 (2) | 507 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-3 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.77-2: Test frequencies for NR operating band n77, SCS 30 kHz and ΔFRaster 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 3305.01 | 620334 | 3300.69 | 620046 | 0 | 30 | 7711 | 620352 | 18 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3708.96 | 647264 | 102 |  | 8020 | 650016 | 16 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 4194.99 | 679666 | 4009.23 | 667282 | 504 |  | 8329 | 679680 | 14 | 0 | 2 (2) | 1012 |
| 15 | 38 | Downlink | Low | 3307.5 | 620500 | 3300.66 | 620044 | 0 | 30 | 7711 | 620352 | 20 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3706.44 | 647096 | 102 |  | 8018 | 649824 | 16 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 4192.5 | 679500 | 4004.22 | 666948 | 504 |  | 8325 | 679296 | 12 | 0 | 0 (0) | 1008 |
| 20 | 51 | Downlink | Low | 3310.02 | 620668 | 3300.84 | 620056 | 0 | 30 | 7711 | 620352 | 8 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3704.1 | 646940 | 102 |  | 8016 | 649632 | 4 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 4189.98 | 679332 | 3999.36 | 666624 | 504 |  | 8322 | 679008 | 0 | 0 | 2 (2) | 1012 |
| 25 | 65 | Downlink | Low | 3312.51 | 620834 | 3300.81 | 620054 | 0 | 30 | 7711 | 620352 | 10 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3701.58 | 646772 | 102 |  | 8015 | 649536 | 4 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 4187.49 | 679166 | 3994.35 | 666290 | 504 |  | 8319 | 678720 | 22 | 0 | 3 (3) | 1014 |
| 30 | 78 | Downlink | Low | 3315 | 621000 | 3300.96 | 620064 | 0 | 30 | 7711 | 620352 | 0 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3699.24 | 646616 | 102 |  | 8013 | 649344 | 16 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 4185 | 679000 | 3989.52 | 665968 | 504 |  | 8315 | 678336 | 8 | 0 | 1 (1) | 1010 |
| 40 | 106 | Downlink | Low | 3320.01 | 621334 | 3300.93 | 620062 | 0 | 30 | 7711 | 620352 | 2 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3694.2 | 646280 | 102 |  | 8010 | 649056 | 16 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 4179.99 | 678666 | 3979.47 | 665298 | 504 |  | 8308 | 677664 | 6 | 0 | 1 (1) | 1010 |
| 50 | 133 | Downlink | Low | 3325.02 | 621668 | 3301.08 | 620072 | 0 | 30 | 7711 | 620352 | 16 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 3750 | 650000 | 3689.34 | 645956 | 102 |  | 8006 | 648672 | 4 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 4174.98 | 678332 | 3969.6 | 664640 | 504 |  | 8301 | 676992 | 16 | 0 | 0 (0) | 1008 |
| 60 | 162 | Downlink | Low | 3330 | 622000 | 3300.84 | 620056 | 0 | 30 | 7711 | 620352 | 8 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3684.12 | 645608 | 102 |  | 8003 | 648384 | 16 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 4170 | 678000 | 3959.4 | 663960 | 504 |  | 8294 | 676320 | 0 | 0 | 1 (1) | 1010 |
| 70 | 189 | Downlink | Low | 3335.01 | 622334 | 3300.99 | 620066 | 0 | 30 | 7711 | 620352 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 3750 | 650000 | 3679.26 | 645284 | 102 |  | 7999 | 648000 | 4 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 4164.99 | 677666 | 3949.53 | 663302 | 504 |  | 8287 | 675648 | 10 | 0 | 0 (0) | 1008 |
| 80 | 217 | Downlink | Low | 3340.02 | 622668 | 3300.96 | 620064 | 0 | 30 | 7711 | 620352 | 0 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3674.22 | 644948 | 102 |  | 7996 | 647712 | 4 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 4159.98 | 677332 | 3939.48 | 662632 | 504 |  | 8280 | 674976 | 8 | 0 | 0 (0) | 1008 |
| 90 | 245 | Downlink | Low | 3345 | 623000 | 3300.9 | 620060 | 0 | 30 | 7711 | 620352 | 4 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3669.18 | 644612 | 102 |  | 7992 | 647328 | 4 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 4155 | 677000 | 3929.46 | 661964 | 504 |  | 8273 | 674304 | 4 | 0 | 0 (0) | 1008 |
| 100 | 273 | Downlink | Low | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3750 | 650000 | 3664.14 | 644276 | 102 |  | 7989 | 647040 | 4 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 4149.99 | 676666 | 3919.41 | 661294 | 504 |  | 8266 | 673632 | 2 | 0 | 0 (0) | 1008 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.77-3: Test frequencies for NR operating band n77, SCS 60 kHz and ΔFRaster 30 kHz without CORESET#0

| CBW [MHz] |  | carrierBandwidth [PRBs] |  | Range |  |  |  | Carrier centre [MHz] |  | Carrier centre [ARFCN] |  | point A [MHz] |  | absoluteFrequencyPointA [ARFCN] |  | offsetToCarrier [PRBs] |  | SS block SCS [kHz] |  | GSCN |  | absoluteFrequencySSB [ARFCN] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 |  | 11 |  | Downlink |  | Low |  | 3305.01 |  | 620334 |  | 3301.05 |  | 620070 |  | 0 |  | 30 |  | - |  | 620310 |  |
|  |  |  |  | & |  | Mid |  | 3750 |  | 650000 |  | 3672.6 |  | 644840 |  | 102 |  |  |  | - |  | 649976 |  |
|  |  |  |  | Uplink |  | High |  | 4194.99 |  | 679666 |  | 3828.15 |  | 655210 |  | 504 |  |  |  | - |  | 679642 |  |
| 15 |  | 18 |  | Downlink |  | Low |  | 3307.5 |  | 620500 |  | 3301.02 |  | 620068 |  | 0 |  | 30 |  | - |  | 620308 |  |
|  |  |  |  | & |  | Mid |  | 3750 |  | 650000 |  | 3670.08 |  | 644672 |  | 102 |  |  |  | - |  | 649808 |  |
|  |  |  |  | Uplink |  | High |  | 4192.5 |  | 679500 |  | 3823.14 |  | 654876 |  | 504 |  |  |  | - |  | 679308 |  |
| 20 |  | 24 |  | Downlink |  | Low |  | 3310.02 |  | 620668 |  | 3301.38 |  | 620092 |  | 0 |  | 30 |  | - |  | 620332 |  |
|  |  |  |  | & |  | Mid |  | 3750 |  | 650000 |  | 3667.92 |  | 644528 |  | 102 |  |  |  | - |  | 649664 |  |
|  |  |  |  | Uplink |  | High |  | 4189.98 |  | 679332 |  | 3818.46 |  | 654564 |  | 504 |  |  |  | - |  | 678996 |  |
| 25 |  | 31 |  | Downlink |  | Low |  | 3312.51 |  | 620834 |  | 3301.35 |  | 620090 |  | 0 |  | 30 |  | - |  | 620330 |  |
|  |  |  |  | & |  | Mid |  | 3750 |  | 650000 |  | 3665.4 |  | 644360 |  | 102 |  |  |  | - |  | 649496 |  |
|  |  |  |  | Uplink |  | High |  | 4187.49 |  | 679166 |  | 3813.45 |  | 654230 |  | 504 |  |  |  | - |  | 678662 |  |
| 30 |  | 38 |  | Downlink |  | Low |  | 3315 |  | 621000 |  | 3301.32 |  | 620088 |  | 0 |  | 30 |  | - |  | 620328 |  |
|  |  |  |  | & |  | Mid |  | 3750 |  | 650000 |  | 3662.88 |  | 644192 |  | 102 |  |  |  | - |  | 649328 |  |
|  |  |  |  | Uplink |  | High |  | 4185 |  | 679000 |  | 3808.44 |  | 653896 |  | 504 |  |  |  | - |  | 678328 |  |
| 40 |  | 51 |  | Downlink |  | Low |  | 3320.01 |  | 621334 |  | 3301.65 |  | 620110 |  | 0 |  | 30 |  | - |  | 620350 |  |
|  |  |  |  | & |  | Mid |  | 3750 |  | 650000 |  | 3658.2 |  | 643880 |  | 102 |  |  |  | - |  | 649016 |  |
|  |  |  |  | Uplink |  | High |  | 4179.99 |  | 678666 |  | 3798.75 |  | 653250 |  | 504 |  |  |  | - |  | 677682 |  |
| 50 |  | 65 |  | Downlink |  | Low |  | 3325.02 |  | 621668 |  | 3301.62 |  | 620108 |  | 0 |  | 30 |  | - |  | 620348 |  |
|  |  |  |  | & |  | Mid |  | 3750 |  | 650000 |  | 3653.16 |  | 643544 |  | 102 |  |  |  | - |  | 648680 |  |
|  |  |  |  | Uplink |  | High |  | 4174.98 |  | 678332 |  | 3788.7 |  | 652580 |  | 504 |  |  |  | - |  | 677012 |  |
| 60 |  | 79 |  | Downlink |  | Low |  | 3330 |  | 622000 |  | 3301.56 |  | 620104 |  | 0 |  | 30 |  | - |  | 620344 |  |
|  |  |  |  | & |  | Mid |  | 3750 |  | 650000 |  | 3648.12 |  | 643208 |  | 102 |  |  |  | - |  | 648344 |  |
|  |  |  |  | Uplink |  | High |  | 4170 |  | 678000 |  | 3778.68 |  | 651912 |  | 504 |  |  |  | - |  | 676344 |  |
| 70 |  | 93 |  | Downlink |  | Low |  | 3335.01 |  | 622334 |  | 3301.53 |  | 620102 |  | 0 |  | 30 |  | - |  | 620342 |  |
|  |  |  |  | & |  | Mid |  | 3750 |  | 650000 |  | 3643.08 |  | 642872 |  | 102 |  |  |  | - |  | 648008 |  |
|  |  |  |  | Uplink |  | High |  | 4164.99 |  | 677666 |  | 3768.63 |  | 651242 |  | 504 |  |  |  | - |  | 675674 |  |
| 80 |  | 107 |  | Downlink |  | Low |  | 3340.02 |  | 622668 |  | 3301.5 |  | 620100 |  | 0 |  | 30 |  | - |  | 620340 |  |
|  |  |  |  | & |  | Mid |  | 3750 |  | 650000 |  | 3638.04 |  | 642536 |  | 102 |  |  |  | - |  | 647672 |  |
|  |  |  |  | Uplink |  | High |  | 4159.98 |  | 677332 |  | 3758.58 |  | 650572 |  | 504 |  |  |  | - |  | 675004 |  |
| 90 |  | 121 |  | Downlink |  | Low |  | 3345 |  | 623000 |  | 3301.44 |  | 620096 |  | 0 |  | 30 |  | - |  | 620336 |  |
|  |  |  |  | & |  | Mid |  | 3750 |  | 650000 |  | 3633 |  | 642200 |  | 102 |  |  |  | - |  | 647336 |  |
|  |  |  |  | Uplink |  | High |  | 4155 |  | 677000 |  | 3748.56 |  | 649904 |  | 504 |  |  |  | - |  | 674336 |  |
| 100 |  | 135 |  | Downlink |  | Low |  | 3350.01 |  | 623334 |  | 3301.41 |  | 620094 |  | 0 |  | 30 |  | - |  | 620334 |  |
|  |  |  |  | & |  | Mid |  | 3750 |  | 650000 |  | 3627.96 |  | 641864 |  | 102 |  |  |  | - |  | 647000 |  |
|  |  |  |  | Uplink |  | High |  | 4149.99 |  | 676666 |  | 3738.51 |  | 649234 |  | 504 |  |  |  | - |  | 673666 |  |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.78	Reference test frequencies for NR operating band n78

Table 4.3.1.1.1.78-1: Test frequencies for NR operating band n78, SCS 15 kHz and ΔFRaster 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 | Downlink | Low | 3305.01 | 620334 | 3300.33 | 620022 | 0 | 30 | 7711 | 620352 | 6 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 3549.99 | 636666 | 3526.95 | 635130 | 102 |  | 7881 | 636672 | 6 | 0 | 1 (6) | 108 |
|  |  | Uplink | High | 3795 | 653000 | 3699.6 | 646640 | 504 |  | 8051 | 652992 | 4 | 3 | 0 (2) | 509 |
| 15 | 79 | Downlink | Low | 3307.5 | 620500 | 3300.39 | 620026 | 0 | 30 | 7711 | 620352 | 2 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 3549.99 | 636666 | 3524.52 | 634968 | 102 |  | 7879 | 636480 | 0 | 2 | 0 (2) | 106 |
|  |  | Uplink | High | 3792.48 | 652832 | 3694.65 | 646310 | 504 |  | 8048 | 652704 | 10 | 2 | 1 (6) | 512 |
| 20 | 106 | Downlink | Low | 3310.02 | 620668 | 3300.48 | 620032 | 0 | 30 | 7711 | 620352 | 8 | 0 | 1 (6) | 6 |
|  |  | & | Mid | 3549.99 | 636666 | 3522.09 | 634806 | 102 |  | 7878 | 636384 | 6 | 3 | 1 (6) | 111 |
|  |  | Uplink | High | 3789.99 | 652666 | 3689.73 | 645982 | 504 |  | 8044 | 652320 | 2 | 2 | 0 (2) | 508 |
| 25 | 133 | Downlink | Low | 3312.51 | 620834 | 3300.54 | 620036 | 0 | 30 | 7711 | 620352 | 4 | 0 | 1 (6) | 6 |
|  |  | & | Mid | 3549.99 | 636666 | 3519.66 | 634644 | 102 |  | 7876 | 636192 | 0 | 1 | 1 (6) | 109 |
|  |  | Uplink | High | 3787.5 | 652500 | 3684.81 | 645654 | 504 |  | 8041 | 652032 | 6 | 1 | 1 (6) | 511 |
| 30 | 160 | Downlink | Low | 3315 | 621000 | 3300.6 | 620040 | 0 | 30 | 7711 | 620352 | 0 | 0 | 1 (6) | 6 |
|  |  | & | Mid | 3549.99 | 636666 | 3517.23 | 634482 | 102 |  | 7874 | 636000 | 6 | 2 | 0 (2) | 106 |
|  |  | Uplink | High | 3784.98 | 652332 | 3679.86 | 645324 | 504 |  | 8037 | 651648 | 0 | 1 | 0 (2) | 507 |
| 40 | 216 | Downlink | Low | 3320.01 | 621334 | 3300.57 | 620038 | 0 | 30 | 7711 | 620352 | 2 | 0 | 1 (6) | 6 |
|  |  | & | Mid | 3549.99 | 636666 | 3512.19 | 634146 | 102 |  | 7871 | 635712 | 6 | 2 | 1 (6) | 110 |
|  |  | Uplink | High | 3780 | 652000 | 3669.84 | 644656 | 504 |  | 8030 | 650976 | 8 | 0 | 0 (2) | 506 |
| 50 | 270 | Downlink | Low | 3325.02 | 621668 | 3300.72 | 620048 | 0 | 30 | 7711 | 620352 | 4 | 3 | 0 (2) | 5 |
|  |  | & | Mid | 3549.99 | 636666 | 3507.33 | 633822 | 102 |  | 7867 | 635328 | 6 | 1 | 0 (2) | 105 |
|  |  | Uplink | High | 3774.99 | 651666 | 3659.97 | 643998 | 504 |  | 8024 | 650400 | 6 | 3 | 1 (6) | 513 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-3 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.78-2: Test frequencies for NR operating band n78, SCS 30 kHz and ΔFRaster 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 3305.01 | 620334 | 3300.69 | 620046 | 0 | 30 | 7711 | 620352 | 18 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3508.95 | 633930 | 102 |  | 7881 | 636672 | 6 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 3795 | 653000 | 3609.24 | 640616 | 504 |  | 8051 | 652992 | 16 | 0 | 1 (1) | 1010 |
| 15 | 38 | Downlink | Low | 3307.5 | 620500 | 3300.66 | 620044 | 0 | 30 | 7711 | 620352 | 20 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3506.43 | 633762 | 102 |  | 7879 | 636480 | 6 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 3792.48 | 652832 | 3604.2 | 640280 | 504 |  | 8048 | 652704 | 16 | 0 | 3 (3) | 1014 |
| 20 | 51 | Downlink | Low | 3310.02 | 620668 | 3300.84 | 620056 | 0 | 30 | 7711 | 620352 | 8 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3504.09 | 633606 | 102 |  | 7878 | 636384 | 18 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 3789.99 | 652666 | 3599.37 | 639958 | 504 |  | 8044 | 652320 | 2 | 0 | 1 (1) | 1010 |
| 25 | 65 | Downlink | Low | 3312.51 | 620834 | 3300.81 | 620054 | 0 | 30 | 7711 | 620352 | 10 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3501.57 | 633438 | 102 |  | 7876 | 636192 | 18 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 3787.5 | 652500 | 3594.36 | 639624 | 504 |  | 8041 | 652032 | 0 | 0 | 3 (3) | 1014 |
| 30 | 78 | Downlink | Low | 3315 | 621000 | 3300.96 | 620064 | 0 | 30 | 7711 | 620352 | 0 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3499.23 | 633282 | 102 |  | 7874 | 636000 | 6 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 3784.98 | 652332 | 3589.5 | 639300 | 504 |  | 8037 | 651648 | 12 | 0 | 0 (0) | 1008 |
| 40 | 106 | Downlink | Low | 3320.01 | 621334 | 3300.93 | 620062 | 0 | 30 | 7711 | 620352 | 2 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3494.19 | 632946 | 102 |  | 7871 | 635712 | 6 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 3780 | 652000 | 3579.48 | 638632 | 504 |  | 8030 | 650976 | 8 | 0 | 0 (0) | 1008 |
| 50 | 133 | Downlink | Low | 3325.02 | 621668 | 3301.08 | 620072 | 0 | 30 | 7711 | 620352 | 16 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 3549.99 | 636666 | 3489.33 | 632622 | 102 |  | 7867 | 635328 | 18 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 3774.99 | 651666 | 3569.61 | 637974 | 504 |  | 8024 | 650400 | 18 | 0 | 3 (3) | 1014 |
| 60 | 162 | Downlink | Low | 3330 | 622000 | 3300.84 | 620056 | 0 | 30 | 7711 | 620352 | 8 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3484.11 | 632274 | 102 |  | 7864 | 635040 | 6 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 3769.98 | 651332 | 3559.38 | 637292 | 504 |  | 8016 | 649632 | 4 | 0 | 0 (0) | 1008 |
| 70 | 189 | Downlink | Low | 3335.01 | 622334 | 3300.99 | 620066 | 0 | 30 | 7711 | 620352 | 22 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 3549.99 | 636666 | 3479.25 | 631950 | 102 |  | 7860 | 634656 | 18 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 3765 | 651000 | 3549.54 | 636636 | 504 |  | 8010 | 649056 | 12 | 0 | 3 (3) | 1014 |
| 80 | 217 | Downlink | Low | 3340.02 | 622668 | 3300.96 | 620064 | 0 | 30 | 7711 | 620352 | 0 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3474.21 | 631614 | 102 |  | 7857 | 634368 | 18 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 3759.99 | 650666 | 3539.49 | 635966 | 504 |  | 8003 | 648384 | 10 | 0 | 3 (3) | 1014 |
| 90 | 245 | Downlink | Low | 3345 | 623000 | 3300.9 | 620060 | 0 | 30 | 7711 | 620352 | 4 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3469.17 | 631278 | 102 |  | 7853 | 633984 | 18 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 3754.98 | 650332 | 3529.44 | 635296 | 504 |  | 7996 | 647712 | 8 | 0 | 3 (3) | 1014 |
| 100 | 273 | Downlink | Low | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 | 7711 | 620352 | 6 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 3549.99 | 636666 | 3464.13 | 630942 | 102 |  | 7850 | 633696 | 18 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 |  | 7989 | 647040 | 4 | 0 | 3 (3) | 1014 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.78-3: Test frequencies for NR operating band n78, SCS 60 kHz and ΔFRaster 30 kHz without CORESET#0.

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 3305.01 | 620334 | 3301.05 | 620070 | 0 | 30 | - | 620310 |
|  |  | & | Mid | 3549.99 | 636666 | 3472.59 | 631506 | 102 |  | - | 636642 |
|  |  | Uplink | High | 3795 | 653000 | 3428.16 | 628544 | 504 |  | - | 652976 |
| 15 | 18 | Downlink | Low | 3307.5 | 620500 | 3301.02 | 620068 | 0 | 30 | - | 620308 |
|  |  | & | Mid | 3549.99 | 636666 | 3470.07 | 631338 | 102 |  | - | 636474 |
|  |  | Uplink | High | 3792.48 | 652832 | 3423.12 | 628208 | 504 |  | - | 652640 |
| 20 | 24 | Downlink | Low | 3310.02 | 620668 | 3301.38 | 620092 | 0 | 30 | - | 620332 |
|  |  | & | Mid | 3549.99 | 636666 | 3467.91 | 631194 | 102 |  | - | 636330 |
|  |  | Uplink | High | 3789.99 | 652666 | 3418.47 | 627898 | 504 |  | - | 652330 |
| 25 | 31 | Downlink | Low | 3312.51 | 620834 | 3301.35 | 620090 | 0 | 30 | - | 620330 |
|  |  | & | Mid | 3549.99 | 636666 | 3465.39 | 631026 | 102 |  | - | 636162 |
|  |  | Uplink | High | 3787.5 | 652500 | 3413.46 | 627564 | 504 |  | - | 651996 |
| 30 | 38 | Downlink | Low | 3315 | 621000 | 3301.32 | 620088 | 0 | 30 | - | 620328 |
|  |  | & | Mid | 3549.99 | 636666 | 3462.87 | 630858 | 102 |  | - | 635994 |
|  |  | Uplink | High | 3784.98 | 652332 | 3408.42 | 627228 | 504 |  | - | 651660 |
| 40 | 51 | Downlink | Low | 3320.01 | 621334 | 3301.65 | 620110 | 0 | 30 | - | 620350 |
|  |  | & | Mid | 3549.99 | 636666 | 3458.19 | 630546 | 102 |  | - | 635682 |
|  |  | Uplink | High | 3780 | 652000 | 3398.76 | 626584 | 504 |  | - | 651016 |
| 50 | 65 | Downlink | Low | 3325.02 | 621668 | 3301.62 | 620108 | 0 | 30 | - | 620348 |
|  |  | & | Mid | 3549.99 | 636666 | 3453.15 | 630210 | 102 |  | - | 635346 |
|  |  | Uplink | High | 3774.99 | 651666 | 3388.71 | 625914 | 504 |  | - | 650346 |
| 60 | 79 | Downlink | Low | 3330 | 622000 | 3301.56 | 620104 | 0 | 30 | - | 620344 |
|  |  | & | Mid | 3549.99 | 636666 | 3448.11 | 629874 | 102 |  | - | 635010 |
|  |  | Uplink | High | 3769.98 | 651332 | 3378.66 | 625244 | 504 |  | - | 649676 |
| 70 | 93 | Downlink & Uplink | Low | 3335.01 | 622334 | 3301.53 | 620102 | 0 | 30 |  | 620342 |
|  |  |  | Mid | 3549.99 | 636666 | 3443.07 | 629538 | 102 |  |  | 634674 |
|  |  |  | High | 3765 | 651000 | 3368.64 | 624576 | 504 |  |  | 649008 |
| 80 | 107 | Downlink | Low | 3340.02 | 622668 | 3301.5 | 620100 | 0 | 30 | - | 620340 |
|  |  | & | Mid | 3549.99 | 636666 | 3438.03 | 629202 | 102 |  | - | 634338 |
|  |  | Uplink | High | 3759.99 | 650666 | 3358.59 | 623906 | 504 |  | - | 648338 |
| 90 | 121 | Downlink | Low | 3345 | 623000 | 3301.44 | 620096 | 0 | 30 | - | 620336 |
|  |  | & | Mid | 3549.99 | 636666 | 3432.99 | 628866 | 102 |  | - | 634002 |
|  |  | Uplink | High | 3754.98 | 650332 | 3348.54 | 623236 | 504 |  | - | 647668 |
| 100 | 135 | Downlink | Low | 3350.01 | 623334 | 3301.41 | 620094 | 0 | 30 | - | 620334 |
|  |  | & | Mid | 3549.99 | 636666 | 3427.95 | 628530 | 102 |  | - | 633666 |
|  |  | Uplink | High | 3750 | 650000 | 3338.52 | 622568 | 504 |  | - | 647000 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.79	Reference test frequencies for NR operating band n79

Table 4.3.1.1.1.79-1: Test frequencies for NR operating band n79, SCS 15 kHz and ΔFRaster 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40 | 216 | Downlink | Low | 4420.02 | 694668 | 4400.58 | 693372 | 0 | 30 | 8480 | 694176 | 0 | 43 | 0 (4) | 47 |
|  |  | & | Mid | 4699.98 | 713332 | 4662.18 | 710812 | 102 |  | 8672 | 712608 | 8 | 23 | 0 (4) | 129 |
|  |  | Uplink | High | 4980 | 732000 | 4869.84 | 724656 | 504 |  | 8864 | 731040 | 0 | 4 | 0 (4) | 512 |
| 50 | 270 | Downlink | Low | 4425 | 695000 | 4400.7 | 693380 | 0 | 30 | 8480 | 694176 | 4 | 42 | 0 (4) | 46 |
|  |  | & | Mid | 4699.98 | 713332 | 4657.32 | 710488 | 102 |  | 8672 | 712608 | 8 | 50 | 0 (4) | 156 |
|  |  | Uplink | High | 4974.99 | 731666 | 4859.97 | 723998 | 504 |  | 8864 | 731040 | 10 | 58 | 0 (4) | 566 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-5 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.79-2: Test frequencies for NR operating band n79, SCS 30 kHz and ΔFRaster 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40 | 106 | Downlink | Low | 4420.02 | 694668 | 4400.94 | 693396 | 0 | 30 | 8480 | 694176 | 12 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4644.21 | 709614 | 102 |  | 8672 | 712608 | 18 | 8 | 1 (4) | 228 |
|  |  | Uplink | High | 4980 | 732000 | 4779.48 | 718632 | 504 |  | 8864 | 731040 | 0 | 3 | 0 (0) | 1014 |
| 50 | 133 | Downlink | Low | 4425 | 695000 | 4401.06 | 693404 | 0 | 30 | 8480 | 694176 | 4 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4639.35 | 709290 | 102 |  | 8672 | 712608 | 6 | 22 | 1 (4) | 256 |
|  |  | Uplink | High | 4974.99 | 731666 | 4769.61 | 717974 | 504 |  | 8864 | 731040 | 10 | 26 | 1 (4) | 1068 |
| 60 | 162 | Downlink | Low | 4430.01 | 695334 | 4400.85 | 693390 | 0 | 30 | 8480 | 694176 | 18 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4634.13 | 708942 | 102 |  | 8672 | 712608 | 18 | 36 | 1 (4) | 284 |
|  |  | Uplink | High | 4969.98 | 731332 | 4759.38 | 717292 | 504 |  | 8864 | 731040 | 20 | 54 | 1 (4) | 1124 |
| 80 | 217 | Downlink | Low | 4440 | 696000 | 4400.94 | 693396 | 0 | 30 | 8480 | 694176 | 12 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4624.23 | 708282 | 102 |  | 8656 | 711072 | 6 | 0 | 1 (4) | 212 |
|  |  | Uplink | High | 4959.99 | 730666 | 4739.49 | 715966 | 504 |  | 8848 | 729504 | 2 | 46 | 1 (4) | 1108 |
| 100 | 273 | Downlink | Low | 4450.02 | 696668 | 4400.88 | 693392 | 0 | 30 | 8480 | 694176 | 16 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4614.15 | 707610 | 102 |  | 8656 | 711072 | 6 | 28 | 1 (4) | 268 |
|  |  | Uplink | High | 4950 | 730000 | 4719.42 | 714628 | 504 |  | 8832 | 727968 | 20 | 37 | 1 (4) | 1090 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-6 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.79-3: Test frequencies for NR operating band n79, SCS 60 kHz and ΔFRaster 30 kHz without CORESET#0.

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40 | 51 | Downlink | Low | 4420.02 | 694668 | 4401.66 | 693444 | 0 | 30 | - | 693684 |
|  |  | & | Mid | 4700.01 | 713334 | 4608.21 | 707214 | 102 |  | - | 712350 |
|  |  | Uplink | High | 4980 | 732000 | 4598.76 | 706584 | 504 |  | - | 731016 |
| 50 | 65 | Downlink | Low | 4425 | 695000 | 4401.6 | 693440 | 0 | 30 | - | 693680 |
|  |  | & | Mid | 4700.01 | 713334 | 4603.17 | 706878 | 102 |  | - | 712014 |
|  |  | Uplink | High | 4974.99 | 731666 | 4588.71 | 705914 | 504 |  | - | 730346 |
| 60 | 79 | Downlink | Low | 4430.01 | 695334 | 4401.57 | 693438 | 0 | 30 | - | 693678 |
|  |  | & | Mid | 4700.01 | 713334 | 4598.13 | 706542 | 102 |  | - | 711678 |
|  |  | Uplink | High | 4969.98 | 731332 | 4578.66 | 705244 | 504 |  | - | 729676 |
| 80 | 107 | Downlink | Low | 4440 | 696000 | 4401.48 | 693432 | 0 | 30 | - | 693672 |
|  |  | & | Mid | 4700.01 | 713334 | 4588.05 | 705870 | 102 |  | - | 711006 |
|  |  | Uplink | High | 4959.99 | 730666 | 4558.59 | 703906 | 504 |  | - | 728338 |
| 100 | 135 | Downlink | Low | 4450.02 | 696668 | 4401.42 | 693428 | 0 | 30 | - | 693668 |
|  |  | & | Mid | 4700.01 | 713334 | 4577.97 | 705198 | 102 |  | - | 710334 |
|  |  | Uplink | High | 4950 | 730000 | 4538.52 | 702568 | 504 |  | - | 727000 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.1.80	Reference test frequencies for NR operating band n80 (SUL)

Table 4.3.1.1.1.80-1: Test frequencies for NR operating band n80 and SCS 15 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1654.53 | 330906 | 504 |
|  |  |  | High | 1782.5 | 356500 | 1779.17 | 355834 | 6 |
| 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1652.1 | 330420 | 504 |
|  |  |  | High | 1780 | 356000 | 1774.24 | 354848 | 6 |
| 15 | 79 | Uplink | Low | 1717.5 | 343500 | 1710.39 | 342078 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1649.67 | 329934 | 504 |
|  |  |  | High | 1777.5 | 355500 | 1769.31 | 353862 | 6 |
| 20 | 106 | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1647.24 | 329448 | 504 |
|  |  |  | High | 1775 | 355000 | 1764.38 | 352876 | 6 |
| 25 | 133 | Uplink | Low | 1722.5 | 344500 | 1710.53 | 342106 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1644.81 | 328962 | 504 |
|  |  |  | High | 1772.5 | 354500 | 1759.45 | 351890 | 6 |
| 30 | 160 | Uplink | Low | 1725 | 345000 | 1710.6 | 342120 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1642.38 | 328476 | 504 |
|  |  |  | High | 1770 | 354000 | 1754.52 | 350904 | 6 |

Table 4.3.1.1.1.80-2: Test frequencies for NR operating band n80 and SCS 30 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1561.74 | 312348 | 504 |
|  |  |  | High | 1780 | 356000 | 1773.52 | 354704 | 6 |
| 15 | 38 | Uplink | Low | 1717.5 | 343500 | 1710.66 | 342132 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1559.22 | 311844 | 504 |
|  |  |  | High | 1777.5 | 355500 | 1768.5 | 353700 | 6 |
| 20 | 51 | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1556.88 | 311376 | 504 |
|  |  |  | High | 1775 | 355000 | 1763.66 | 352732 | 6 |
| 25 | 65 | Uplink | Low | 1722.5 | 344500 | 1710.8 | 342160 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1554.36 | 310872 | 504 |
|  |  |  | High | 1772.5 | 354500 | 1758.64 | 351728 | 6 |
| 30 | 78 | Uplink | Low | 1725 | 345000 | 1710.96 | 342192 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1552.02 | 310404 | 504 |
|  |  |  | High | 1770 | 354000 | 1753.8 | 350760 | 6 |

Table 4.3.1.1.1.80-3: Test frequencies for NR operating band n80 and SCS 60 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1380.66 | 276132 | 504 |
|  |  |  | High | 1780 | 356000 | 1771.72 | 354344 | 6 |
| 15 | 18 | Uplink | Low | 1717.5 | 343500 | 1711.02 | 342204 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1378.14 | 275628 | 504 |
|  |  |  | High | 1777.5 | 355500 | 1766.7 | 353340 | 6 |
| 20 | 24 | Uplink | Low | 1720 | 344000 | 1711.36 | 342272 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1375.98 | 275196 | 504 |
|  |  |  | High | 1775 | 355000 | 1762.04 | 352408 | 6 |
| 25 | 31 | Uplink | Low | 1722.5 | 344500 | 1711.34 | 342268 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1373.46 | 274692 | 504 |
|  |  |  | High | 1772.5 | 354500 | 1757.02 | 351404 | 6 |
| 30 | 38 | Uplink | Low | 1725 | 345000 | 1711.32 | 342264 | 0 |
|  |  |  | Mid | 1747.5 | 349500 | 1370.94 | 274188 | 504 |
|  |  |  | High | 1770 | 354000 | 1752 | 350400 | 6 |

4.3.1.1.1.81	Reference test frequencies for NR operating band n81 (SUL)

Table 4.3.1.1.1.81-1: Test frequencies for NR operating band n81 and SCS 15 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 882.5 | 176500 | 880.25 | 176050 | 0 |
|  |  |  | Mid | 897.5 | 179500 | 804.53 | 160906 | 504 |
|  |  |  | High | 912.5 | 182500 | 909.17 | 181834 | 6 |
| 10 | 52 | Uplink | Low | 885 | 177000 | 880.32 | 176064 | 0 |
|  |  |  | Mid | 897.5 | 179500 | 802.1 | 160420 | 504 |
|  |  |  | High | 910 | 182000 | 904.24 | 180848 | 6 |
| 15 | 79 | Uplink | Low | 887.5 | 177500 | 880.39 | 176078 | 0 |
|  |  |  | Mid | 897.5 | 179500 | 799.67 | 159934 | 504 |
|  |  |  | High | 907.5 | 181500 | 899.31 | 179862 | 6 |
| 20 | 106 | Uplink | Low | 890 | 178000 | 880.46 | 176092 | 0 |
|  |  |  | Mid | 897.5 | 179500 | 797.24 | 159448 | 504 |
|  |  |  | High | 905 | 181000 | 894.38 | 178876 | 6 |

Table 4.3.1.1.1.81-2: Test frequencies for NR operating band n81 and SCS 30 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 885 | 177000 | 880.68 | 176136 | 0 |
|  |  |  | Mid | 897.5 | 179500 | 711.74 | 142348 | 504 |
|  |  |  | High | 910 | 182000 | 903.52 | 180704 | 6 |
| 15 | 38 | Uplink | Low | 887.5 | 177500 | 880.66 | 176132 | 0 |
|  |  |  | Mid | 897.5 | 179500 | 709.22 | 141844 | 504 |
|  |  |  | High | 907.5 | 181500 | 898.5 | 179700 | 6 |
| 20 | 51 | Uplink | Low | 890 | 178000 | 880.82 | 176164 | 0 |
|  |  |  | Mid | 897.5 | 179500 | 706.88 | 141376 | 504 |
|  |  |  | High | 905 | 181000 | 893.66 | 178732 | 6 |

4.3.1.1.1.82	Reference test frequencies for NR operating band n82 (SUL)

Table 4.3.1.1.1.82-1: Test frequencies for NR operating band n82 and SCS 15 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 834.5 | 166900 | 832.25 | 166450 | 0 |
|  |  |  | Mid | 847 | 169400 | 754.03 | 150806 | 504 |
|  |  |  | High | 859.5 | 171900 | 856.17 | 171234 | 6 |
| 10 | 52 | Uplink | Low | 837 | 167400 | 832.32 | 166464 | 0 |
|  |  |  | Mid | 847 | 169400 | 751.6 | 150320 | 504 |
|  |  |  | High | 857 | 171400 | 851.24 | 170248 | 6 |
| 15 | 79 | Uplink | Low | 839.5 | 167900 | 832.39 | 166478 | 0 |
|  |  |  | Mid | 847 | 169400 | 749.17 | 149834 | 504 |
|  |  |  | High | 854.5 | 170900 | 846.31 | 169262 | 6 |
| 20 | 106 | Uplink | Low | 842 | 168400 | 832.46 | 166492 | 0 |
|  |  |  | Mid | 847 | 169400 | 746.74 | 149348 | 504 |
|  |  |  | High | 852 | 170400 | 841.38 | 168276 | 6 |

Table 4.3.1.1.1.82-2: Test frequencies for NR operating band n82 and SCS 30 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 837 | 167400 | 832.68 | 166536 | 0 |
|  |  |  | Mid | 847 | 169400 | 661.24 | 132248 | 504 |
|  |  |  | High | 857 | 171400 | 850.52 | 170104 | 6 |
| 15 | 38 | Uplink | Low | 839.5 | 167900 | 832.66 | 166532 | 0 |
|  |  |  | Mid | 847 | 169400 | 658.72 | 131744 | 504 |
|  |  |  | High | 854.5 | 170900 | 845.5 | 169100 | 6 |
| 20 | 51 | Uplink | Low | 842 | 168400 | 832.82 | 166564 | 0 |
|  |  |  | Mid | 847 | 169400 | 656.38 | 131276 | 504 |
|  |  |  | High | 852 | 170400 | 840.66 | 168132 | 6 |

4.3.1.1.1.83	Reference test frequencies for NR operating band n83 (SUL)

Table 4.3.1.1.1.83-1: Test frequencies for NR operating band n83 and SCS 15 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 705.5 | 141100 | 703.25 | 140650 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 632.53 | 126506 | 504 |
|  |  |  | High | 745.5 | 149100 | 742.17 | 148434 | 6 |
| 10 | 52 | Uplink | Low | 708 | 141600 | 703.32 | 140664 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 630.1 | 126020 | 504 |
|  |  |  | High | 743 | 148600 | 737.24 | 147448 | 6 |
| 15 | 79 | Uplink | Low | 710.5 | 142100 | 703.39 | 140678 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 627.67 | 125534 | 504 |
|  |  |  | High | 740.5 | 148100 | 732.31 | 146462 | 6 |
| 20 | 106 | Uplink | Low | 713 | 142600 | 703.46 | 140692 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 625.24 | 125048 | 504 |
|  |  |  | High | 738 | 147600 | 727.3 | 145476 | 6 |
| 30 | 160 | Uplink | Low | 718 | 143600 | 703.6 | 140720 | 0 |
| (Note 1) |  |  | High | 733 | 146600 | 717.52 | 143504 | 6 |
| Note 1: No carrier centre frequency specified for Mid Range and CBW=30 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1 |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.83-2: Test frequencies for NR operating band n83 and SCS 30 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 708 | 141600 | 703.68 | 140736 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 539.7 | 107948 | 504 |
|  |  |  | High | 743 | 148600 | 736.52 | 147304 | 6 |
| 15 | 38 | Uplink | Low | 710.5 | 142100 | 703.66 | 140732 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 537.22 | 107444 | 504 |
|  |  |  | High | 740.5 | 148100 | 731.5 | 146300 | 6 |
| 20 | 51 | Uplink | Low | 713 | 142600 | 703.82 | 140764 | 0 |
|  |  |  | Mid | 725.5 | 145100 | 534.88 | 106976 | 504 |
|  |  |  | High | 738 | 147600 | 726.66 | 145332 | 6 |
| 30 | 78 | Uplink | Low | 718 | 143600 | 703.96 | 140792 | 0 |
| (Note 2) |  |  | High | 733 | 146600 | 716.8 | 143360 | 6 |
| Note 2: No carrier centre frequency specified for Mid Range and CBW=30 MHz due to Note 7 in TS 38.101-1 [7], Table 5.3.5-1 |  |  |  |  |  |  |  |  |

4.3.1.1.1.84	Reference test frequencies for NR operating band n84 (SUL)

Table 4.3.1.1.1.84-1: Test frequencies for NR operating band n84 and SCS 15 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 1922.5 | 384500 | 1920.25 | 384050 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1857.03 | 371406 | 504 |
|  |  |  | High | 1977.5 | 395500 | 1974.17 | 394834 | 6 |
| 10 | 52 | Uplink | Low | 1925 | 385000 | 1920.32 | 384064 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1854.6 | 370920 | 504 |
|  |  |  | High | 1975 | 395000 | 1969.24 | 393848 | 6 |
| 15 | 79 | Uplink | Low | 1927.5 | 385500 | 1920.39 | 384078 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1852.17 | 370434 | 504 |
|  |  |  | High | 1972.5 | 394500 | 1964.31 | 392862 | 6 |
| 20 | 106 | Uplink | Low | 1930 | 386000 | 1920.46 | 384092 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1849.74 | 369948 | 504 |
|  |  |  | High | 1970 | 394000 | 1959.38 | 391876 | 6 |
| 25 | 133 | Uplink | Low | 1932.5 | 386500 | 1920.53 | 384106 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1847.31 | 369462 | 504 |
|  |  |  | High | 1967.5 | 393500 | 1954.45 | 390890 | 6 |
| 30 | 160 | Uplink | Low | 1935 | 387000 | 1920.6 | 384120 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1844.88 | 368976 | 504 |
|  |  |  | High | 1965 | 393000 | 1949.52 | 389904 | 6 |
| 40 | 216 | Uplink | Low | 1940 | 388000 | 1920.56 | 384112 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1839.84 | 367968 | 504 |
|  |  |  | High | 1960 | 392000 | 1939.48 | 387896 | 6 |
| 50 | 270 | Uplink | Low | 1945 | 389000 | 1920.7 | 384140 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1834.98 | 366996 | 504 |
|  |  |  | High | 1955 | 391000 | 1929.62 | 385924 | 6 |

Table 4.3.1.1.1.84-2: Test frequencies for NR operating band n84 and SCS 30 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 1925 | 385000 | 1920.68 | 384136 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1764.24 | 352848 | 504 |
|  |  |  | High | 1975 | 395000 | 1968.52 | 393704 | 6 |
| 15 | 38 | Uplink | Low | 1927.5 | 385500 | 1920.66 | 384132 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1761.72 | 352344 | 504 |
|  |  |  | High | 1972.5 | 394500 | 1963.5 | 392700 | 6 |
| 20 | 51 | Uplink | Low | 1930 | 386000 | 1920.82 | 384164 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1759.38 | 351876 | 504 |
|  |  |  | High | 1970 | 394000 | 1958.66 | 391732 | 6 |
| 25 | 65 | Uplink | Low | 1932.5 | 386500 | 1920.8 | 384160 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1756.86 | 351372 | 504 |
|  |  |  | High | 1967.5 | 393500 | 1953.64 | 390728 | 6 |
| 30 | 78 | Uplink | Low | 1935 | 387000 | 1920.96 | 384192 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1754.52 | 350904 | 504 |
|  |  |  | High | 1965 | 393000 | 1948.8 | 389760 | 6 |
| 40 | 106 | Uplink | Low | 1940 | 388000 | 1920.92 | 384184 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1749.48 | 349896 | 504 |
|  |  |  | High | 1960 | 392000 | 1938.76 | 387752 | 6 |
| 50 | 133 | Uplink | Low | 1945 | 389000 | 1921.06 | 384212 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1744.62 | 348924 | 504 |
|  |  |  | High | 1955 | 391000 | 1928.9 | 385780 | 6 |

Table 4.3.1.1.1.84-3: Test frequencies for NR operating band n84 and SCS 60 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Uplink | Low | 1925 | 385000 | 1921.04 | 384208 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1583.16 | 316632 | 504 |
|  |  |  | High | 1975 | 395000 | 1966.72 | 393344 | 6 |
| 15 | 18 | Uplink | Low | 1927.5 | 385500 | 1921.02 | 384204 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1580.64 | 316128 | 504 |
|  |  |  | High | 1972.5 | 394500 | 1961.7 | 392340 | 6 |
| 20 | 24 | Uplink | Low | 1930 | 386000 | 1921.36 | 384272 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1578.48 | 315696 | 504 |
|  |  |  | High | 1970 | 394000 | 1957.04 | 391408 | 6 |
| 25 | 31 | Uplink | Low | 1932.5 | 386500 | 1921.34 | 384268 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1575.96 | 315192 | 504 |
|  |  |  | High | 1967.5 | 393500 | 1952.02 | 390404 | 6 |
| 30 | 38 | Uplink | Low | 1935 | 387000 | 1921.32 | 384264 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1573.44 | 314688 | 504 |
|  |  |  | High | 1965 | 393000 | 1947 | 389400 | 6 |
| 40 | 51 | Uplink | Low | 1940 | 388000 | 1921.64 | 384328 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1568.76 | 313752 | 504 |
|  |  |  | High | 1960 | 392000 | 1937.32 | 387464 | 6 |
| 50 | 65 | Uplink | Low | 1945 | 389000 | 1921.6 | 384320 | 0 |
|  |  |  | Mid | 1950 | 390000 | 1563.72 | 312744 | 504 |
|  |  |  | High | 1955 | 391000 | 1927.28 | 385456 | 6 |

4.3.1.1.1.85	FFS

4.3.1.1.1.86	Reference test frequencies for NR operating band n86 (SUL)

Table 4.3.1.1.1.86-1: Test frequencies for NR operating band n86 and SCS 15 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1652.03 | 330406 | 504 |
|  |  |  | High | 1777.5 | 355500 | 1774.17 | 354834 | 6 |
| 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |
|  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |
| 15 | 79 | Uplink | Low | 1717.5 | 343500 | 1710.39 | 342078 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1647.17 | 329434 | 504 |
|  |  |  | High | 1772.5 | 354500 | 1764.31 | 352862 | 6 |
| 20 | 106 | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1644.74 | 328948 | 504 |
|  |  |  | High | 1770 | 354000 | 1759.38 | 351876 | 6 |
| 40 | 216 | Uplink | Low | 1730 | 346000 | 1710.56 | 342112 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1634.84 | 326968 | 504 |
|  |  |  | High | 1760 | 352000 | 1739.48 | 347896 | 6 |

Table 4.3.1.1.1.86-2: Test frequencies for NR operating band n86 and SCS 30 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |
|  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |
| 15 | 38 | Uplink | Low | 1717.5 | 343500 | 1710.66 | 342132 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1556.72 | 311344 | 504 |
|  |  |  | High | 1772.5 | 354500 | 1763.5 | 352700 | 6 |
| 20 | 51 | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1554.38 | 310876 | 504 |
|  |  |  | High | 1770 | 354000 | 1758.66 | 351732 | 6 |
| 40 | 106 | Uplink | Low | 1730 | 346000 | 1710.92 | 342184 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1544.48 | 308896 | 504 |
|  |  |  | High | 1760 | 352000 | 1738.76 | 347752 | 6 |

Table 4.3.1.1.1.86-3: Test frequencies for NR operating band n86 and SCS 60 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1378.16 | 275632 | 504 |
|  |  |  | High | 1775 | 355000 | 1766.72 | 353344 | 6 |
| 15 | 18 | Uplink | Low | 1717.5 | 343500 | 1711.02 | 342204 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1375.64 | 275128 | 504 |
|  |  |  | High | 1772.5 | 354500 | 1761.7 | 352340 | 6 |
| 20 | 24 | Uplink | Low | 1720 | 344000 | 1711.36 | 342272 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1373.48 | 274696 | 504 |
|  |  |  | High | 1770 | 354000 | 1757.04 | 351408 | 6 |
| 40 | 51 | Uplink | Low | 1730 | 346000 | 1711.64 | 342328 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1363.76 | 272752 | 504 |
|  |  |  | High | 1760 | 352000 | 1737.32 | 347464 | 0 |

4.3.1.1.1.87 to 4.3.1.1.1.94	FFS

4.3.1.1.1.95	Reference test frequencies for NR operating band n95 (SUL)

Table 4.3.1.1.1.95-1: Test frequencies for NR operating band n95 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 2012.5 | 402500 | 2010.25 | 402050 | 0 |
|  |  |  | Mid | 2017.5 | 403500 | 1924.53 | 384906 | 504 |
|  |  |  | High | 2022.5 | 404500 | 2019.17 | 403834 | 6 |
| 10 | 52 | Uplink | Low | 2015 | 403000 | 2010.32 | 402064 | 0 |
|  |  |  | Mid | 2017.5 | 403500 | 1922.1 | 384420 | 504 |
|  |  |  | High | 2020 | 404000 | 2014.24 | 402848 | 6 |
| 15 | 79 | Uplink | Low, Mid, High | 2017.5 | 403500 | 2010.39 | 402078 | 0 |

Table 4.3.1.1.1.95-2: Test frequencies for NR operating band n95 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 2015 | 403000 | 2010.68 | 402136 | 0 |
|  |  |  | Mid | 2017.5 | 403500 | 1831.74 | 366348 | 504 |
|  |  |  | High | 2020 | 404000 | 2013.52 | 402704 | 6 |
| 15 | 38 | Uplink | Low, Mid, High | 2017.5 | 403500 | 2010.66 | 402132 | 0 |

Table 4.3.1.1.1.95-3: Test frequencies for NR operating band n95 and SCS 60 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Uplink | Low | 2015 | 403000 | 2011.04 | 402208 | 0 |
|  |  |  | Mid | 2017.5 | 403500 | 1650.66 | 330132 | 504 |
|  |  |  | High | 2020 | 404000 | 2011.72 | 402344 | 6 |
| 15 | 18 | Uplink | Low, Mid, High | 2017.5 | 403500 | 2011.02 | 402204 | 0 |

4.3.1.1.1.96	FFS

4.3.1.1.1.97	Reference test frequencies for NR operating band n97 (SUL)

Table 4.3.1.1.1.97-1: Test frequencies for NR operating band n97 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 2302.5 | 460500 | 2300.25 | 460050 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2329.39 | 465878 | 102 |
|  |  |  | High | 2397.5 | 479500 | 2304.53 | 460906 | 504 |
| 10 | 52 | Uplink | Low | 2305 | 461000 | 2300.32 | 460064 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2326.96 | 465392 | 102 |
|  |  |  | High | 2395 | 479000 | 2299.6 | 459920 | 504 |
| 15 | 79 | Uplink | Low | 2307.5 | 461500 | 2300.39 | 460078 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2324.53 | 464906 | 102 |
|  |  |  | High | 2392.5 | 478500 | 2294.67 | 458934 | 504 |
| 20 | 106 | Uplink | Low | 2310 | 462000 | 2300.46 | 460092 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2322.1 | 464420 | 102 |
|  |  |  | High | 2390 | 478000 | 2289.74 | 457948 | 504 |
| 25 | 133 | Uplink | Low | 2312.5 | 462500 | 2300.53 | 460106 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2319.67 | 463934 | 102 |
|  |  |  | High | 2387.5 | 477500 | 2284.81 | 456962 | 504 |
| 30 | 160 | Uplink | Low | 2315 | 463000 | 2300.6 | 460120 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2317.24 | 463448 | 102 |
|  |  |  | High | 2385 | 477000 | 2279.88 | 455976 | 504 |
| 40 | 216 | Uplink | Low | 2320 | 464000 | 2300.56 | 460112 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2312.2 | 462440 | 102 |
|  |  |  | High | 2380 | 476000 | 2269.84 | 453968 | 504 |
| 50 | 270 | Uplink | Low | 2325 | 465000 | 2300.7 | 460140 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2307.34 | 461468 | 102 |
|  |  |  | High | 2375 | 475000 | 2259.98 | 451996 | 504 |

Table 4.3.1.1.1.97-2: Test frequencies for NR operating band n97 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 2305 | 461000 | 2300.68 | 460136 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2308.96 | 461792 | 102 |
|  |  |  | High | 2395 | 479000 | 2209.24 | 441848 | 504 |
| 15 | 38 | Uplink | Low | 2307.5 | 461500 | 2300.66 | 460132 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2306.44 | 461288 | 102 |
|  |  |  | High | 2392.5 | 478500 | 2204.22 | 440844 | 504 |
| 20 | 51 | Uplink | Low | 2310 | 462000 | 2300.82 | 460164 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2304.1 | 460820 | 102 |
|  |  |  | High | 2390 | 478000 | 2199.38 | 439876 | 504 |
| 25 | 65 | Uplink | Low | 2312.5 | 462500 | 2300.8 | 460160 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2301.58 | 460316 | 102 |
|  |  |  | High | 2387.5 | 477500 | 2194.36 | 438872 | 504 |
| 30 | 78 | Uplink | Low | 2315 | 463000 | 2300.96 | 460192 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2299.24 | 459848 | 102 |
|  |  |  | High | 2385 | 477000 | 2189.52 | 437904 | 504 |
| 40 | 106 | Uplink | Low | 2320 | 464000 | 2300.92 | 460184 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2294.2 | 458840 | 102 |
|  |  |  | High | 2380 | 476000 | 2179.48 | 435896 | 504 |
| 50 | 133 | Uplink | Low | 2325 | 465000 | 2301.06 | 460212 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2289.34 | 457868 | 102 |
|  |  |  | High | 2375 | 475000 | 2169.62 | 433924 | 504 |
| 60 | 162 | Uplink | Low | 2330 | 466000 | 2300.84 | 460168 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2284.12 | 456824 | 102 |
|  |  |  | High | 2370 | 474000 | 2159.4 | 431880 | 504 |
|  |  |  | Low | 2335 | 467000 | 2300.98 | 460196 | 0 |
| 70 | 189 | Uplink | Mid | 2350 | 470000 | 2279.26 | 455852 | 102 |
|  |  |  | High | 2365 | 473000 | 2149.54 | 429908 | 504 |
| 80 | 217 | Uplink | Low | 2340 | 468000 | 2300.94 | 460188 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2274.22 | 454844 | 102 |
|  |  |  | High | 2360 | 472000 | 2139.5 | 427900 | 504 |
|  |  |  | Low | 2345 | 469000 | 2300.9 | 460180 | 0 |
| 90 | 245 | Uplink | Mid | 2350 | 470000 | 2269.18 | 453836 | 102 |
|  |  |  | High | 2355 | 471000 | 2129.46 | 425892 | 504 |
|  |  |  | Low | 2350 | 470000 | 2300.86 | 460172 | 0 |
| 100 | 273 | Uplink | Mid | 2350 | 470000 | 2264.14 | 452828 | 102 |
|  |  |  | High | 2350 | 470000 | 2119.42 | 423884 | 504 |

Table 4.3.1.1.1.97-3: Test frequencies for NR operating band n97 and SCS 60 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Uplink | Low | 2305 | 461000 | 2301.04 | 460208 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2272.6 | 454520 | 102 |
|  |  |  | High | 2395 | 479000 | 2028.16 | 405632 | 504 |
| 15 | 18 | Uplink | Low | 2307.5 | 461500 | 2301.02 | 460204 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2270.08 | 454016 | 102 |
|  |  |  | High | 2392.5 | 478500 | 2023.14 | 404628 | 504 |
| 20 | 24 | Uplink | Low | 2310 | 462000 | 2301.36 | 460272 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2267.92 | 453584 | 102 |
|  |  |  | High | 2390 | 478000 | 2018.48 | 403696 | 504 |
| 25 | 31 | Uplink | Low | 2312.5 | 462500 | 2301.34 | 460268 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2265.4 | 453080 | 102 |
|  |  |  | High | 2387.5 | 477500 | 2013.46 | 402692 | 504 |
| 30 | 38 | Uplink | Low | 2315 | 463000 | 2301.32 | 460264 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2262.88 | 452576 | 102 |
|  |  |  | High | 2385 | 477000 | 2008.44 | 401688 | 504 |
| 40 | 51 | Uplink | Low | 2320 | 464000 | 2301.64 | 460328 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2258.2 | 451640 | 102 |
|  |  |  | High | 2380 | 476000 | 1998.76 | 399752 | 504 |
| 50 | 65 | Uplink | Low | 2325 | 465000 | 2301.6 | 460320 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2253.16 | 450632 | 102 |
|  |  |  | High | 2375 | 475000 | 1988.72 | 397744 | 504 |
| 60 | 79 | Uplink | Low | 2330 | 466000 | 2301.56 | 460312 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2248.12 | 449624 | 102 |
|  |  |  | High | 2370 | 474000 | 1978.68 | 395736 | 504 |
|  |  |  | Low | 2335 | 467000 | 2301.52 | 460304 | 0 |
| 70 | 93 | Uplink | Mid | 2350 | 470000 | 2243.08 | 448616 | 102 |
|  |  |  | High | 2365 | 473000 | 1968.64 | 393728 | 504 |
| 80 | 107 | Uplink | Low | 2340 | 468000 | 2301.48 | 460296 | 0 |
|  |  |  | Mid | 2350 | 470000 | 2238.04 | 447608 | 102 |
|  |  |  | High | 2360 | 472000 | 1958.6 | 391720 | 504 |
|  |  |  | Low | 2345 | 469000 | 2301.44 | 460288 | 0 |
| 90 | 121 | Uplink | Mid | 2350 | 470000 | 2233 | 446600 | 102 |
|  |  |  | High | 2355 | 471000 | 1948.56 | 389712 | 504 |
|  |  |  | Low | 2350 | 470000 | 2301.4 | 460280 | 0 |
| 100 | 135 | Uplink | Mid | 2350 | 470000 | 2227.96 | 445592 | 102 |
|  |  |  | High | 2350 | 470000 | 1938.52 | 387704 | 504 |

4.3.1.1.1.98	FFS

4.3.1.1.1.99	Reference test frequencies for NR operating band n99 (SUL)

Table 4.3.1.1.1.99-1: Test frequencies for NR operating band n99 and SCS 15 kHz

| UL CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 1630.0 | 326000 | 1627.75 | 325550 | 0 |
|  |  |  | Mid | 1635.0 | 327000 | 1542.03 | 308406 | 504 |
|  |  |  | High | 1654.0 | 330800 | 1650.67 | 330134 | 6 |
| 10 | 52 | Uplink | Low Mid | 1632.5 | 326500 | 1627.82 | 325564 | 0 |
|  |  |  | High | 1651.5 | 330300 | 1645.74 | 329148 | 6 |

Table 4.3.1.1.1.99-2: Test frequencies for NR operating band n99 and SCS 30 kHz

| UL CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low Mid | 1632.5 | 326500 | 1627.82 | 325564 | 0 |
|  |  |  | High | 1651.5 | 330300 | 1645.02 | 329004 | 6 |

Table 4.3.1.1.1.99-3: Test frequencies for NR operating band n99 and SCS 60 kHz

| UL CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Uplink | Low Mid | 1632.5 | 326500 | 1628.54 | 325708 | 0 |
|  |  |  | High | 1651.5 | 330300 | 1643.22 | 328644 | 6 |

4.3.1.1.2	NR inter-band CA configurations in FR1

4.3.1.1.2.1	NR inter-band CA configurations in FR1 (two bands)

Table 4.3.1.1.2.1-1: Inter-band NR CA configurations (FR1, two bands)

| NR CA configuration | Uplink NR CA configuration | NR CA downlink configuration band 1 (Note 3) | NR CA downlink configuration band 2 | NR CA uplink configuration band 1 | NR CA uplink configuration band 2 | Applicable for protocol testing (Note 2) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A | CA_n1A-n3A | n1A | n3A | n1A | n3A | Yes |
| CA_n1A-n8A | CA_n1A-n8A | n1A | n8A | n1A | n8A | Yes |
| CA_n1(2A)-n3A | - | CA_n1(2A) | n3A | n1A | n3A | No |
| CA_n1(2A)-n5A | - | CA_n1(2A) | n5A | - | - | No |
| CA_n1(2A)-n8A | - | CA_n1(2A) | n8A | - | - | No |
| CA_n1A-n77A | - | n1A | n77A | - | - | Yes |
| CA_n1A-n78A | CA_n1A-n78A | n1A | n78A | n1A | n78A | Yes |
| CA_n1A-n78(2A) | CA_n1A-n78A | n1A | CA_n78(2A) | n1A | n78A | No |
| CA_n1A-n78C | CA_n1A-n78A | n1A | CA_n78C | n1A | n78A | No |
| CA_n1(2A)-n78A | - | CA_n1(2A) | n78A | - | - | No |
| CA_n1A-n79A | CA_n1A-n79A | n1A | n79A | n1A | n79A | Yes |
| CA_n2A-n5A | CA_n2A-n5A | n2A | n5A | n2A | n5A | Yes |
| CA_n2A-n48A | CA_n2A-n48A | n2A | n48A | n2A | n48A | Yes |
| CA_n2A-n66A | CA_n2A-n66A | n2A | n66A | n2A | n66A | Yes |
| CA_n2A-n77A | CA_n2A-n77A | n2A | n77A | n2A | n77A | Yes |
| CA_n3A-n5A | CA_n3A-n5A | n3A | n5A | n3A | n5A | Yes |
| CA_n3(2A)-n5A | - | CA_n3(2A) | n5A | - | - | No |
| CA_n3(2A)-n8A | - | CA_n3(2A) | n8A | - | - | No |
| CA_n3A-n41A | CA_n3A-n41A | n3A | n41A | n3A | n41A | Yes |
| CA_n3A-n77A | - | n3A | n77A | - | - | Yes |
| CA_n3A-n78A | CA_n3A-n78A | n3A | n78A | n3A | n78A | Yes |
| CA_n3A-n78(2A) | CA_n3A-n78A | n3A | CA_n78(2A) | n3A | n78A | No |
|  | CA_n78(2A) | n3A | CA_n78(2A) | - | CA_n78(2A) | No |
| CA_n3(2A)-n78A | - | CA_n3(2A) | n78A | - | - | No |
| CA_n3A-n79A | CA_n3A-n79A | n3A | n79A | n3A | n79A | Yes |
| CA_n5A-n7A | - | n5A | n7A | - | - | Yes |
| CA_n5A-n48A | CA_n5A-n48A | n5A | n48A | n5A | n48A | Yes |
| CA_n5A-n66A | CA_n5A-n66A | n5A | n66A | n5A | n66A | Yes |
| CA_n5A-n78A | - | n5A | n78A | - | - | Yes |
| CA_n5A-n78(2A) | CA_n5A-n78A | n5A | CA_n78(2A) | n5A | n78A | No |
| CA_n7A-n78A | - | n7A | n78A | - | - | Yes |
| CA_n8A-n75A | - | n8A | n75A | - | - | Yes |
| CA_n8A-n78A | CA_n8A-n78A | n8A | n78A | n8A | n78A | Yes |
| CA_n8A-n78(2A) | CA_n8A-n78A | n8A | CA_n78(2A) | n8A | CA_n78(2A) | No |
| CA_n8A-n79A | CA_n8A-n79A | n8A | n79A | n8A | n79A | Yes |
| CA_n24A-n41A | CA_n24A-n41A | n24A | n41A | n24A | n41A | Yes |
| CA_n24A-n41(2A) | CA_n24A-n41A | n24A | CA_n41(2A) | n24A | n41A | No |
| CA_n24A-n48A | CA_n24A-n48A | n24A | n48A | n24A | n48A | Yes |
| CA_n24A-n48B | CA_n24A-n48A | n24A | CA_n48B | n24A | n48A | No |
| CA_n24A-n48(2A) | CA_n24A-n48A | n24A | CA_n48(2A) | n24A | n48A | No |
| CA_n24A-n77A | CA_n24A-n77A | n24A | n77A | n24A | n77A | Yes |
| CA_n24A-n77C | CA_n24A-n77A | n24A | CA_n77C | n24A | n77A | No |
| CA_n26A-n66A | CA_n26A-n66A | n26A | n66A | n26A | n66A | Yes |
| CA_n26A-n66(2A) | CA_n26A-n66A | n26A | CA_n66(2A) | n26A | n66A | No |
| CA_n26A-n70A | CA_n26A-n70A | n26A | n70A | n26A | n70A | Yes |
| CA_n28A-n41A | CA_n28A-n41A | n28A | n41A | n28A | n41A | Yes |
| CA_n28A-n75A | - | n28A | n75A | - | - | Yes |
| CA_n28A-n78A | CA_n28A-n78A | n28A | n78A | n28A | n78A | Yes |
| CA_n28A-n79A | CA_n28A-n79A | n28A | n79A | n28A | n79A | Yes |
| CA_n29A-n66A | - | n29A | n66A (Note 1) | - | - | Yes |
| CA_n29B-n66B | - | n29A | CA_n66B (Note 1) | - | - | No |
| CA_n29A-n66(2A) | - | n29A | CA_n66(2A) (Note 1) | - | - | No |
| CA_n29A-n70A | - | n29A | n70A (Note 1) | - | - | Yes |
| CA_n29A-n71A | - | n29A | n71A (Note 1) | - | - | Yes |
| CA_n41A-n78A | - | n41A | n78A | - | - | Yes |
| CA_n41A-n79A | CA_n41A-n79A | n41A | n79A | n41A | n79A | Yes |
| CA_n41C-n79A | CA_n41A-n79A | CA_n41C | n79A | n41A | n79A | No |
|  | CA_n41C | CA_n41C | n79A | CA_n41C | - | No |
| CA_n48A-n66A | CA_n48A-n66A | n48A | n66A | n48A | n66A | Yes |
| CA_n48A-n66(2A) | CA_n48A-n66A | n48A | CA_n66(2A) | n48A | n66A | No |
| CA_n48A-n70A | CA_n48A-n70A | n48A | n70A | n48A | n70A | Yes |
| CA_n48A-n71A | CA_n48A-n71A | n48A | n71A | n48A | n71A | Yes |
| CA_n48A-n71(2A) | CA_n48A-n71A | n48A | CA_n71(2A) | n48A | n71A | No |
| CA_n48B-n66A | CA_n48A-n66A | CA_n48B | n66 | n48A | n66A | No |
| CA_n48B-n70A | CA_n48A-n70A | CA_n48B | n70A | n48A | n70A | No |
| CA_n48B-n71A | CA_n48A-n71A | CA_n48B | n71A | n48A | n71A | No |
| CA_n48(2A)-n66A | CA_n48A-n66A | CA_n48(2A) | n66 | n48A | n66A | No |
| CA_n48(2A)-n70A | CA_n48A-n70A | CA_n48(2A) | n70A | n48A | n70A | No |
| CA_n48(2A-n71A | CA_n48A-n71A | CA_n48(2A) | n71A | n48A | n71A | No |
| CA_n66A-n70A | - | n66A | n70A | - | - | Yes |
| CA_n66B-n70A | - | CA_n66B | n70A | - | - | No |
| CA_n66(2A)-n70A | - | CA_n66(2A) | n70A | - | - | No |
| CA_n66A-n71A | CA_n66A-n71A | n66A | n71A | n66A | n71A | Yes |
| CA_n66A-n71(2A) | CA_n66A-n71A | n66A | CA_n71(2A) | n66A | n71A | No |
| CA_n66B-n71A | CA_n66A-n71A | CA_n66B | n71A | n66A | n71A | No |
| CA_n66(2A)-n71A | CA_n66A-n71A | CA_n66(2A) | n71A | n66A | n71A | No |
| CA_n66(2A)-n71(2A) | CA_n66A-n71A | CA_n66(2A) | CA_n71(2A) | n66A | n71A | No |
| CA_n70A-n71A | CA_n70A-n71A | n70A | n71A | n70A | n71A | Yes |
| CA_n70A-n71(2A) | CA_n70A-n71A | n70A | CA_n71(2A) | n70A | n71A | No |
| CA_n75A-n78A | - | n75A | n78A (Note 1) | - | - | Yes |
| CA_n76A-n78A | - | n76A | n78A (Note 1) | - | - | Yes |
| CA_n77A-n79A | - | n77A | n79A | - | - | Yes |
| CA_n78A-n79A | - | n78A | n79A | - | - | Yes |
| Note 1: This band is used as PCell. Note 2: Protocol testing is limited to NR CA configurations with 2CC. Note 3: PCell is configured on this band unless otherwise stated. |  |  |  |  |  |  |

4.3.1.1.2.2	NR inter-band CA configurations in FR1 (three bands)

Table 4.3.1.1.2.2-1: Inter-band NR CA configurations within FR1 (three bands)

| NR CA configuration (Note 3) | Uplink NR CA configuration | NR CA downlink configuration band 1 (Note 4) | NR CA downlink configuration band 2 | NR CA downlink configuration band 3 | NR CA uplink configuration band 1 | NR CA uplink configuration band 2 | NR CA uplink configuration band 3 | Applicable for protocol testing (Note 2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n26A-n66A-n70A | CA_n26A-n66A | n26A | n66A | n70A | n26A | n66A | - | No |
|  | CA_n26A-n70A | n26A | n66A | n70A | n26A | - | n70A | No |
| CA_n29A-n66A-n70A | - | n29A (Note1) | n66A | n70A | - | - | - | No |
| CA_n29A-n66B-n70A | - | n29A (Note1) | CA_n66B | n70A | - | - | - | No |
| CA_n29A-n66(2A)-n70A | - | n29A (Note1) | CA_n66(2A) | n70A | - | - | - | No |
| CA_n48A-n66A-n70A | CA_n48A-n66A | n48A | n66A | n70A | n48A | n66A | - | No |
|  | CA_n48A-n70A | n48A | n66A | n70A | n48A | - | n70A | No |
| CA_n48A-n66A-n71A | CA_n48A-n66A | n48A | n66A | n71A | n48A | n66A | - | No |
|  | CA_n48A-n71A | n48A | n66A | n71A | n48A | - | n71A | No |
|  | CA_n66A-n71A | n48A | n66A | n71A | - | n66A | n71A | No |
| CA_n48A-n70A-n71A | CA_n48A-n70A | n48A | n70A | n71A | n48A | n70A | - | No |
|  | CA_n48A-n71A | n48A | n70A | n71A | n48A | - | n71A | No |
|  | CA_n70A-n71A | n48A | n70A | n71A | - | n70A | n71A | No |
| CA_n66A-n70A-n71A | CA_n66A-n71A | n66A | n70A | n71A | n66A | - | n71A | No |
|  | CA_n70A-n71A | n66A | n70A | n71A | - | n70A | n71A | No |
| CA_n66A-n70A-n71(2A) | CA_n66A-n71A | n66A | n70A | CA_n71(2A) | n66A | - | n71A | No |
|  | CA_n70A-n71A | n66A | n70A | CA_n71(2A) | - | n70A | n71A | No |
| CA_n66B-n70A-n71A | CA_n66A-n71A | CA_n66B | n70A | n71A | n66A | - | n71A | No |
|  | CA_n70A-n71A | CA_n66B | n70A | n71A | - | n70A | n71A | No |
| CA_n66(2A)-n70A-n71A | CA_n66A-n71A | CA_n66(2A) | n70A | n71A | n66A | - | n71A | No |
|  | CA_n70A-n71A | CA_n66(2A) | n70A | n71A | - | n70A | n71A | No |
| Note 1: This band cannot be used as PCell. Note 2: Protocol testing is limited to NR CA configurations with 3CC. Note 3: The band with the lowest UL frequency is used as PCell if nothing else is specified for in the table or in the test case for the specific configuration. Note 4: PCell is configured on this band unless otherwise stated. |  |  |  |  |  |  |  |  |

4.3.1.1.3	NR intra-band contiguous CA in FR1

4.3.1.1.3.1 – 4.3.1.1.3.39	FFS

4.3.1.1.3.40	NR Intra-band contiguous configurations CA_n40

4.3.1.1.3.40.1	CA_n40B

Editor's note:	Test frequencies for CA_n40B with mixed numerology with SCS CC1=15 kHz and SCS CC2=30 kHz or 60 kHz; and SCS CC1=30 kHz and SCS CC2= 15 kHz or 60kHz is FFS.

Table 4.3.1.1.3.40.1-1: NR Intra-Band contiguous CA configuration CA_n40B (PCC=CC1 and SCC=CC2), CC1 SCS = 15kHz, CC2 SCS = 15kHz

| CBW combination [MHz] | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+50 | CC1 | 50 | 15 | 270 | Downlink | Low | 2325 | 465000 | 2300.7 | 460140 | 0 | 30 |  | 5763 | 461070 |  | 10 |  | 3 | 0 (2) | 5 |
|  |  |  |  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Uplink | High | 2325.2 | 465040 | 2210.18 | 442036 | 504 |  |  | 5762 | 461050 |  | 2 |  | 2 | 0 (2) | 508 |
|  | Channel spacing CC1-CC2=49.8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 15 | 270 | Downlink | Low | 2374.8 | 474960 | 2350.5 | 470100 | 0 | 30 |  | 5886 | 470910 |  | 6 |  | 0 | 0 (2) | 2 |
|  |  |  |  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Uplink | High | 2375 | 475000 | 2259.98 | 451996 | 504 |  |  | 5888 | 471130 |  | 6 |  | 1 | 1 (6) | 511 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.3.40.1-2: NR Intra-Band contiguous CA configuration CA_n40B (PCC=CC1 and SCC=CC2), CC1 SCS = 30kHz, CC2 SCS = 30kHz

| CBW combination [MHz] | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20+80 | CC1 | 20 | 30 | 51 | Downlink | Low | 2310 | 462000 | 2300.82 | 460164 | 0 | 30 |  | 5763 | 461070 |  | 14 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Uplink | High | 2310.2 | 462040 | 2119.58 | 423916 | 504 |  |  | 5762 | 461050 |  | 18 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=49.8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 2359.8 | 471960 | 2320.74 | 464148 | 0 | 30 |  | 5811 | 464910 |  | 14 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Uplink | High | 2360 | 472000 | 2139.5 | 427900 | 504 |  |  | 5813 | 465130 |  | 2 |  | 0 | 3 (3) | 1014 |
| 50+50 | CC1 | 50 | 30 | 133 | Downlink | Low | 2325 | 465000 | 2301.06 | 460212 | 0 | 30 |  | 5763 | 461070 |  | 22 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Uplink | High | 2325.2 | 465040 | 2119.82 | 423964 | 504 |  |  | 5762 | 461050 |  | 2 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=49.8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 2374.8 | 474960 | 2350.86 | 470172 | 0 | 30 |  | 5886 | 470910 |  | 6 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Uplink | High | 2375 | 475000 | 2169.62 | 433924 | 504 |  |  | 5888 | 471130 |  | 18 |  | 0 | 2 (2) | 1012 |
| 80+20 | CC1 | 80 | 30 | 217 | Downlink | Low | 2340 | 468000 | 2300.94 | 460188 | 0 | 30 |  | 5763 | 461070 |  | 6 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Uplink | High | 2340.2 | 468040 | 2119.7 | 423940 | 504 |  |  | 5762 | 461050 |  | 10 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=49.8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 30 | 51 | Downlink | Low | 2389.8 | 477960 | 2380.62 | 476124 | 0 | 30 |  | 5961 | 476910 |  | 22 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Uplink | High | 2390 | 478000 | 2199.38 | 439876 | 504 |  |  | 5960 | 476890 |  | 2 |  | 0 | 0 (0) | 1008 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.3.41	NR Intra-band contiguous configurations CA_n41

4.3.1.1.3.41.1	CA_n41C

Editor's note:	Test frequencies for CA_n41C with mixed numerology with SCS CC1=15 kHz and SCS CC2=30 kHz or 60 kHz; and SCS CC1=30 kHz and SCS CC2= 30 kHz or 60kHz is FFS.

Table 4.3.1.1.3.41.1-1: NR Intra-Band contiguous CA configuration CA_n41C (PCC=CC1 and SCC=CC2), CC1 SCS = 30kHz, CC2 SCS = 30 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40+80 | CC1 | 40 | 30 | 106 | Downlink | Low | 2516.01 | 503202 | 2496.93 | 499386 | 0 | 30 |  | 6252 | 500190 |  | 4 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2553 | 510600 | 2497.2 | 499440 | 102 |  |  | 6345 | 507630 |  | 18 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2590.17 | 518034 | 2389.65 | 477930 | 504 |  |  | 6438 | 515070 |  | 20 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=59.82 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 2575.83 | 515166 | 2536.77 | 507354 | 0 | 30 |  | 6351 | 508110 |  | 12 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2612.82 | 522564 | 2537.04 | 507408 | 102 |  |  | 6444 | 515550 |  | 2 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2649.99 | 529998 | 2429.49 | 485898 | 504 |  |  | 6537 | 522990 |  | 4 |  | 0 | 1 (1) | 1010 |
| 40+100 | CC1 | 40 | 30 | 106 | Downlink | Low | 2516.01 | 503202 | 2496.93 | 499386 | 0 | 30 |  | 6252 | 500190 |  | 4 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2543.01 | 508602 | 2487.21 | 497442 | 102 |  |  | 6321 | 505710 |  | 20 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2570.28 | 514056 | 2369.76 | 473952 | 504 |  |  | 6387 | 510990 |  | 10 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=69.72 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 2585.73 | 517146 | 2536.59 | 507318 | 0 | 30 |  | 6351 | 508110 |  | 0 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2612.73 | 522546 | 2526.87 | 505374 | 102 |  |  | 6420 | 513630 |  | 16 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2640 | 528000 | 2409.42 | 481884 | 504 |  |  | 6486 | 518910 |  | 6 |  | 0 | 0 (0) | 1008 |
| 50+60 | CC1 | 50 | 30 | 133 | Downlink | Low | 2521.02 | 504204 | 2497.08 | 499416 | 0 | 30 |  | 6252 | 500190 |  | 18 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2562.99 | 512598 | 2502.33 | 500466 | 102 |  |  | 6357 | 508590 |  | 20 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2605.02 | 521004 | 2399.64 | 479928 | 504 |  |  | 6462 | 516990 |  | 18 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=54.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 2575.98 | 515196 | 2546.82 | 509364 | 0 | 30 |  | 6378 | 510270 |  | 14 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 2617.95 | 523590 | 2552.07 | 510414 | 102 |  |  | 6483 | 518670 |  | 16 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2659.98 | 531996 | 2449.38 | 489876 | 504 |  |  | 6588 | 527070 |  | 14 |  | 0 | 2 (2) | 1012 |
| 50+80 | CC1 | 50 | 30 | 133 | Downlink | Low | 2521.02 | 504204 | 2497.08 | 499416 | 0 | 30 |  | 6252 | 500190 |  | 18 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2553 | 510600 | 2492.34 | 498468 | 102 |  |  | 6333 | 506670 |  | 22 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2585.13 | 517026 | 2379.75 | 475950 | 504 |  |  | 6414 | 513150 |  | 16 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=64.86 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 2585.88 | 517176 | 2546.82 | 509364 | 0 | 30 |  | 6378 | 510270 |  | 14 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 2617.86 | 523572 | 2542.08 | 508416 | 102 |  |  | 6456 | 516510 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2649.99 | 529998 | 2429.49 | 485898 | 504 |  |  | 6537 | 522990 |  | 4 |  | 0 | 1 (1) | 1010 |
| 50+100 | CC1 | 50 | 30 | 133 | Downlink | Low | 2521.02 | 504204 | 2497.08 | 499416 | 0 | 30 |  | 6252 | 500190 |  | 18 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2543.01 | 508602 | 2482.35 | 496470 | 102 |  |  | 6309 | 504750 |  | 0 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 2565.24 | 513048 | 2359.86 | 471972 | 504 |  |  | 6363 | 509070 |  | 6 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=74.76 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 2595.78 | 519156 | 2546.64 | 509328 | 0 | 30 |  | 6378 | 510270 |  | 2 |  | 0 | 3(3) | 6 |
|  |  |  |  |  | & | Mid | 2617.77 | 523554 | 2531.91 | 506382 | 102 |  |  | 6432 | 514590 |  | 0 |  | 0 | 2(2) | 208 |
|  |  |  |  |  | Uplink | High | 2640 | 528000 | 2409.42 | 481884 | 504 |  |  | 6486 | 518910 |  | 6 |  | 0 | 0(0) | 1008 |
| 60+50 | CC1 | 60 | 30 | 162 | Downlink | Low | 2526 | 505200 | 2496.84 | 499368 | 0 | 30 |  | 6252 | 500190 |  | 10 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2568 | 513600 | 2502.12 | 500424 | 102 |  |  | 6357 | 508590 |  | 10 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2610.03 | 522006 | 2399.43 | 479886 | 504 |  |  | 6462 | 516990 |  | 8 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=54.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 2580.96 | 516192 | 2557.02 | 511404 | 0 | 30 |  | 6402 | 512190 |  | 22 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2622.96 | 524592 | 2562.3 | 512460 | 102 |  |  | 6507 | 520590 |  | 22 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2664.99 | 532998 | 2459.61 | 491922 | 504 |  |  | 6612 | 528990 |  | 20 |  | 0 | 0 (0) | 1008 |
| 60+60 | CC1 | 60 | 30 | 162 | Downlink | Low | 2526 | 505200 | 2496.84 | 499368 | 0 | 30 |  | 6252 | 500190 |  | 10 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2562.99 | 512598 | 2497.11 | 499422 | 102 |  |  | 6345 | 507630 |  | 0 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2599.98 | 519996 | 2389.38 | 477876 | 504 |  |  | 6438 | 515070 |  | 14 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=60 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 2586 | 517200 | 2556.84 | 511368 | 0 | 30 |  | 6402 | 512190 |  | 10 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2622.99 | 524598 | 2557.11 | 511422 | 102 |  |  | 6495 | 519630 |  | 0 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2659.98 | 531996 | 2449.38 | 489876 | 504 |  |  | 6588 | 527070 |  | 14 |  | 0 | 2 (2) | 1012 |
| 60+80 | CC1 | 60 | 30 | 162 | Downlink | Low | 2526 | 505200 | 2496.84 | 499368 | 0 | 30 |  | 6252 | 500190 |  | 10 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2553 | 510600 | 2487.12 | 497424 | 102 |  |  | 6321 | 505710 |  | 2 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 2580.09 | 516018 | 2369.49 | 473898 | 504 |  |  | 6387 | 510990 |  | 4 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=69.9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 2595.9 | 519180 | 2556.84 | 511368 | 0 | 30 |  | 6402 | 512190 |  | 10 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2622.9 | 524580 | 2547.12 | 509424 | 102 |  |  | 6471 | 517710 |  | 2 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 2649.99 | 529998 | 2429.49 | 485898 | 504 |  |  | 6537 | 522990 |  | 4 |  | 0 | 1 (1) | 1010 |
| 60+100 | CC1 | 60 | 30 | 162 | Downlink | Low | 2526 | 505200 | 2496.84 | 499368 | 0 | 30 |  | 6252 | 500190 |  | 10 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2543.01 | 508602 | 2477.13 | 495426 | 102 |  |  | 6294 | 503550 |  | 20 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2560.2 | 512040 | 2349.6 | 469920 | 504 |  |  | 6339 | 507150 |  | 2 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=79.8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 2605.8 | 521160 | 2556.66 | 511332 | 0 | 30 |  | 6402 | 512190 |  | 22 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2622.81 | 524562 | 2536.95 | 507390 | 102 |  |  | 6444 | 515550 |  | 8 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2640 | 528000 | 2409.42 | 481884 | 504 |  |  | 6486 | 518910 |  | 6 |  | 0 | 0 (0) | 1008 |
| 80+40 | CC1 | 80 | 30 | 217 | Downlink | Low | 2536.02 | 507204 | 2496.96 | 499392 | 0 | 30 |  | 6252 | 500190 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2573.01 | 514602 | 2497.23 | 499446 | 102 |  |  | 6345 | 507630 |  | 16 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2610.18 | 522036 | 2389.68 | 477936 | 504 |  |  | 6438 | 515070 |  | 18 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=59.82 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 30 | 106 | Downlink | Low | 2595.84 | 519168 | 2576.76 | 515352 | 0 | 30 |  | 6453 | 516270 |  | 18 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 2632.83 | 526566 | 2577.03 | 515406 | 102 |  |  | 6543 | 523470 |  | 0 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2670 | 534000 | 2469.48 | 493896 | 504 |  |  | 6636 | 530910 |  | 2 |  | 0 | 0 (0) | 1008 |
| 80+50 | CC1 | 80 | 30 | 217 | Downlink | Low | 2536.02 | 507204 | 2496.96 | 499392 | 0 | 30 |  | 6252 | 500190 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2568 | 513600 | 2492.22 | 498444 | 102 |  |  | 6333 | 506670 |  | 6 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2600.13 | 520026 | 2379.63 | 475926 | 504 |  |  | 6414 | 513150 |  | 0 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=64.86 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 2600.88 | 520176 | 2576.94 | 515388 | 0 | 30 |  | 6453 | 516270 |  | 6 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 2632.86 | 526572 | 2572.2 | 514440 | 102 |  |  | 6531 | 522510 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2664.99 | 532998 | 2459.61 | 491922 | 504 |  |  | 6612 | 528990 |  | 20 |  | 0 | 0 (0) | 1008 |
| 80+60 | CC1 | 80 | 30 | 217 | Downlink | Low | 2536.02 | 507204 | 2496.96 | 499392 | 0 | 30 |  | 6252 | 500190 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2562.99 | 512598 | 2487.21 | 497442 | 102 |  |  | 6321 | 505710 |  | 20 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2590.08 | 518016 | 2369.58 | 473916 | 504 |  |  | 6387 | 510990 |  | 22 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=69.9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 2605.92 | 521184 | 2576.76 | 515352 | 0 | 30 |  | 6453 | 516270 |  | 18 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 2632.89 | 526578 | 2567.01 | 513402 | 102 |  |  | 6519 | 521550 |  | 4 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2659.98 | 531996 | 2449.38 | 489876 | 504 |  |  | 6588 | 527070 |  | 14 |  | 0 | 2 (2) | 1012 |
| 80+80 | CC1 | 80 | 30 | 217 | Downlink | Low | 2536.02 | 507204 | 2496.96 | 499392 | 0 | 30 |  | 6252 | 500190 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2553 | 510600 | 2477.22 | 495444 | 102 |  |  | 6294 | 503550 |  | 14 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2570.01 | 514002 | 2349.51 | 469902 | 504 |  |  | 6336 | 506910 |  | 0 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=79.98 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 2616 | 523200 | 2576.94 | 515388 | 0 | 30 |  | 6453 | 516270 |  | 6 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 2632.98 | 526596 | 2557.2 | 511440 | 102 |  |  | 6495 | 519630 |  | 18 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2649.99 | 529998 | 2429.49 | 485898 | 504 |  |  | 6537 | 522990 |  | 4 |  | 0 | 1 (1) | 1010 |
| 80+100 | CC1 | 80 | 30 | 217 | Downlink | Low | 2536.02 | 507204 | 2496.96 | 499392 | 0 | 30 |  | 6252 | 500190 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2543.01 | 508602 | 2467.23 | 493446 | 102 |  |  | 6270 | 501630 |  | 16 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2550.12 | 510024 | 2329.62 | 465924 | 504 |  |  | 6288 | 503070 |  | 22 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=89.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 2625.9 | 525180 | 2576.76 | 515352 | 0 | 30 |  | 6453 | 516270 |  | 18 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 2632.89 | 526578 | 2547.03 | 509406 | 102 |  |  | 6468 | 517470 |  | 0 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2640 | 528000 | 2409.42 | 481884 | 504 |  |  | 6486 | 518910 |  | 6 |  | 0 | 0 (0) | 1008 |
| 100+40 | CC1 | 100 | 30 | 273 | Downlink | Low | 2546.01 | 509202 | 2496.87 | 499374 | 0 | 30 |  | 6252 | 500190 |  | 8 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2573.01 | 514602 | 2487.15 | 497430 | 102 |  |  | 6321 | 505710 |  | 0 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 2600.28 | 520056 | 2369.7 | 473940 | 504 |  |  | 6387 | 510990 |  | 14 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=69.72 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 30 | 106 | Downlink | Low | 2615.73 | 523146 | 2596.65 | 519330 | 0 | 30 |  | 6501 | 520110 |  | 20 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2642.73 | 528546 | 2586.93 | 517386 | 102 |  |  | 6570 | 525630 |  | 12 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2670 | 534000 | 2469.48 | 493896 | 504 |  |  | 6636 | 530910 |  | 2 |  | 0 | 0 (0) | 1008 |
| 100+50 | CC1 | 100 | 30 | 273 | Downlink | Low | 2546.01 | 509202 | 2496.87 | 499374 | 0 | 30 |  | 6252 | 500190 |  | 8 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2568 | 513600 | 2482.14 | 496428 | 102 |  |  | 6306 | 504510 |  | 6 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2590.23 | 518046 | 2359.65 | 471930 | 504 |  |  | 6363 | 509070 |  | 20 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=74.76 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 2620.77 | 524154 | 2596.83 | 519366 | 0 | 30 |  | 6501 | 520110 |  | 8 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2642.76 | 528552 | 2582.1 | 516420 | 102 |  |  | 6558 | 524670 |  | 14 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 2664.99 | 532998 | 2459.61 | 491922 | 504 |  |  | 6612 | 528990 |  | 20 |  | 0 | 0 (0) | 1008 |
| 100+60 | CC1 | 100 | 30 | 273 | Downlink | Low | 2546.01 | 509202 | 2496.87 | 499374 | 0 | 30 |  | 6252 | 500190 |  | 8 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2562.99 | 512598 | 2477.13 | 495426 | 102 |  |  | 6294 | 503550 |  | 20 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2580.18 | 516036 | 2349.6 | 469920 | 504 |  |  | 6339 | 507150 |  | 2 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=79.8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 2625.81 | 525162 | 2596.65 | 519330 | 0 | 30 |  | 6501 | 520110 |  | 20 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2642.79 | 528558 | 2576.91 | 515382 | 102 |  |  | 6543 | 523470 |  | 8 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2659.98 | 531996 | 2449.38 | 489876 | 504 |  |  | 6588 | 527070 |  | 14 |  | 0 | 2 (2) | 1012 |
| 100+80 | CC1 | 100 | 30 | 273 | Downlink | Low | 2546.01 | 509202 | 2496.87 | 499374 | 0 | 30 |  | 6252 | 500190 |  | 8 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 2553 | 510600 | 2467.14 | 493428 | 102 |  |  | 6270 | 501630 |  | 22 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 2560.11 | 512022 | 2329.53 | 465906 | 504 |  |  | 6288 | 503070 |  | 4 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=89.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 2635.89 | 527178 | 2596.83 | 519366 | 0 | 30 |  | 6501 | 520110 |  | 8 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 2642.88 | 528576 | 2567.1 | 513420 | 102 |  |  | 6519 | 521550 |  | 22 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 2649.99 | 529998 | 2429.49 | 485898 | 504 |  |  | 6537 | 522990 |  | 4 |  | 0 | 1 (1) | 1010 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.3.42 – 4.3.1.1.3.47	FFS

4.3.1.1.3.48	NR Intra-band contiguous configurations CA_n48

4.3.1.1.3.48.1	CA_n48B

Editor's note:	Test frequencies for CA_n48B with mixed numerology with SCS CC1=15 kHz and SCS CC2=30 kHz or 60 kHz; and SCS CC1=30 kHz and SCS CC2= 15 kHz or 60kHz is FFS.

Table 4.3.1.1.3.48.1-1: NR Intra-Band contiguous CA configuration CA_n48B (PCC=CC1 and SCC=CC2), CC1 SCS = 15kHz, CC2 SCS = 15kHz

| CBW combination [MHz] (BCS) | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10+10 | CC1 | 10 | 15 | 52 | Downlink | Low | 3555 | 637000 | 3550.32 | 636688 | 0 | 30 |  | 7884 | 636960 |  | 8 |  | 0 | 0 (2) | 2 |
| (0,2) |  |  |  |  | & | Mid | 3619.98 | 641332 | 3596.94 | 639796 | 102 |  |  | 7930 | 641376 |  | 8 |  | 3 | 1 (6) | 111 |
| Note 6 |  |  |  |  | Uplink | High | 3685.02 | 645668 | 3589.62 | 639308 | 504 |  |  | 7975 | 645696 |  | 4 |  | 2 | 1 (6) | 512 |
|  | Channel spacing CC1-CC2=9.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 15 | 52 | Downlink | Low | 3564.96 | 637664 | 3560.28 | 637352 | 0 | 30 |  | 7891 | 637632 |  | 4 |  | 1 | 0 (2) | 3 |
|  |  |  |  |  | & | Mid | 3629.94 | 641996 | 3606.9 | 640460 | 102 |  |  | 7936 | 641952 |  | 4 |  | 0 | 0 (2) | 104 |
|  |  |  |  |  | Uplink | High | 3694.98 | 646332 | 3599.58 | 639972 | 504 |  |  | 7982 | 646368 |  | 0 |  | 3 | 1 (6) | 513 |
| 10+15 | CC1 | 10 | 15 | 52 | Downlink | Low | 3555 | 637000 | 3550.32 | 636688 | 0 | 30 |  | 7884 | 636960 |  | 8 |  | 0 | 0 (2) | 2 |
| (0,2) |  |  |  |  | & | Mid | 3617.49 | 641166 | 3594.45 | 639630 | 102 |  |  | 7928 | 641184 |  | 6 |  | 1 | 1 (6) | 109 |
| Note 6 |  |  |  |  | Uplink | High | 3680.01 | 645334 | 3584.61 | 638974 | 504 |  |  | 7971 | 645312 |  | 2 |  | 2 | 0 (2) | 508 |
|  | Channel spacing CC1-CC2=12.48 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 15 | 79 | Downlink | Low | 3567.48 | 637832 | 3560.37 | 637358 | 0 | 30 |  | 7891 | 637632 |  | 10 |  | 0 | 0 (2) | 2 |
|  |  |  |  |  | & | Mid | 3629.97 | 641998 | 3604.5 | 640300 | 102 |  |  | 7935 | 641856 |  | 8 |  | 1 | 1 (6) | 109 |
|  |  |  |  |  | Uplink | High | 3692.49 | 646166 | 3594.66 | 639644 | 504 |  |  | 7978 | 645984 |  | 4 |  | 2 | 0 (2) | 508 |
| 10+20 | CC1 | 10 | 15 | 52 | Downlink | Low | 3555 | 637000 | 3550.32 | 636688 | 0 | 30 |  | 7884 | 636960 |  | 8 |  | 0 | 0 (2) | 2 |
| (0,2) |  |  |  |  | & | Mid | 3615 | 641000 | 3591.96 | 639464 | 102 |  |  | 7926 | 640992 |  | 4 |  | 3 | 0 (2) | 107 |
| Note 6 |  |  |  |  | Uplink | High | 3675.36 | 645024 | 3579.96 | 638664 | 504 |  |  | 7968 | 645024 |  | 0 |  | 0 | 1 (6) | 510 |
|  | Channel spacing CC1-CC2=14.64 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 15 | 106 | Downlink | Low | 3569.64 | 637976 | 3560.1 | 637340 | 0 | 30 |  | 7891 | 637632 |  | 4 |  | 2 | 0 (2) | 4 |
|  |  |  |  |  | & | Mid | 3629.64 | 641976 | 3601.74 | 640116 | 102 |  |  | 7933 | 641664 |  | 0 |  | 1 | 1 (6) | 109 |
|  |  |  |  |  | Uplink | High | 3690 | 646000 | 3589.74 | 639316 | 504 |  |  | 7975 | 645696 |  | 8 |  | 1 | 1 (6) | 511 |
| 10+30 | CC1 | 10 | 15 | 52 | Downlink | Low | 3555 | 637000 | 3550.32 | 636688 | 0 | 30 |  | 7884 | 636960 |  | 8 |  | 0 | 0 (2) | 2 |
| (2) |  |  |  |  | & | Mid | 3609.99 | 640666 | 3586.95 | 639130 | 102 |  |  | 7923 | 640704 |  | 2 |  | 3 | 1 (6) | 111 |
|  |  |  |  |  | Uplink | High | 3665.31 | 644354 | 3569.91 | 637994 | 504 |  |  | 7961 | 644352 |  | 10 |  | 3 | 0 (2) | 509 |
|  | Channel spacing CC1-CC2=19.68 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 30 | 15 | 160 | Downlink | Low | 3574.68 | 638312 | 3560.28 | 637352 | 0 | 30 |  | 7891 | 637632 |  | 4 |  | 1 | 0 (2) | 3 |
|  |  |  |  |  |  | Mid | 3629.67 | 641978 | 3596.91 | 639794 | 102 |  |  | 7930 | 641376 |  | 10 |  | 3 | 1 (6) | 111 |
|  |  |  |  |  |  | High | 3684.99 | 645666 | 3579.87 | 638658 | 504 |  |  | 7968 | 645024 |  | 6 |  | 0 | 1 (6) | 510 |
| 10+40 | CC1 | 10 | 15 | 52 | Downlink | Low | 3555 | 637000 | 3550.32 | 636688 | 0 | 30 |  | 7884 | 636960 |  | 8 |  | 0 | 0 (2) | 2 |
| (2) |  |  |  |  | & | Mid | 3604.98 | 640332 | 3581.94 | 638796 | 102 |  |  | 7919 | 640320 |  | 0 |  | 3 | 0 (2) | 107 |
|  |  |  |  |  | Uplink | High | 3655.62 | 643708 | 3560.22 | 637348 | 504 |  |  | 7954 | 643680 |  | 8 |  | 1 | 0 (2) | 507 |
|  | Channel spacing CC1-CC2=24.36 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 15 | 216 | Downlink | Low | 3579.36 | 638624 | 3559.92 | 637328 | 0 | 30 |  | 7891 | 637632 |  | 4 |  | 3 | 0 (2) | 5 |
|  |  |  |  |  |  | Mid | 3629.34 | 641956 | 3591.54 | 639436 | 102 |  |  | 7926 | 640992 |  | 8 |  | 1 | 1 (6) | 109 |
|  |  |  |  |  |  | High | 3679.98 | 645332 | 3569.82 | 637988 | 504 |  |  | 7961 | 644352 |  | 4 |  | 0 | 1 (6) | 510 |
| 10+50 | CC1 | 10 | 15 | 52 | Downlink | Low | 3555 | 637000 | 3550.32 | 636688 | 0 | 30 |  | 7884 | 636960 |  | 8 |  | 0 | 0 (2) | 2 |
| (1,2) |  |  |  |  | & | Mid | 3600 | 640000 | 3576.96 | 638464 | 102 |  |  | 7916 | 640032 |  | 8 |  | 2 | 1 (6) | 110 |
|  |  |  |  |  | Uplink | High | 3645.6 | 643040 | 3550.2 | 636680 | 504 |  |  | 7947 | 643008 |  | 4 |  | 1 | 0 (2) | 507 |
|  | Channel spacing CC1-CC2=29.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 15 | 270 | Downlink | Low | 3584.4 | 638960 | 3560.1 | 637340 | 0 | 30 |  | 7891 | 637632 |  | 4 |  | 2 | 0 (2) | 4 |
|  |  |  |  |  |  | Mid | 3629.4 | 641960 | 3586.74 | 639116 | 102 |  |  | 7922 | 640608 |  | 4 |  | 0 | 0 (2) | 104 |
|  |  |  |  |  |  | High | 3675 | 645000 | 3559.98 | 637332 | 504 |  |  | 7954 | 643680 |  | 0 |  | 3 | 0 (2) | 509 |
| 15+5 | CC1 | 15 | 15 | 79 | Downlink | Low | 3557.52 | 637168 | 3550.41 | 636694 | 0 | 30 |  | 7884 | 636960 |  | 2 |  | 0 | 0 (2) | 2 |
| (0) |  |  |  |  | & | Mid | 3622.5 | 641500 | 3597.03 | 639802 | 102 |  |  | 7930 | 641376 |  | 2 |  | 3 | 1 (6) | 111 |
|  |  |  |  |  | Uplink | High | 3687.63 | 645842 | 3589.8 | 639320 | 504 |  |  | 7975 | 645696 |  | 4 |  | 1 | 1 (6) | 511 |
|  | Channel spacing CC1-CC2=9.855 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 5 | 15 | 25 | Downlink | Low | 3567.375 | 637825 | 3565.125 | 637675 | 0 | Note |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  | & | Mid | 3632.355 | 642157 | 3611.745 | 640783 | 102 | 5 |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  | Uplink | High | 3697.485 | 646499 | 3604.515 | 640301 | 504 |  |  | - | - |  | - |  | - | - | - |
| 15+10 | CC1 | 15 | 15 | 79 | Downlink | Low | 3557.52 | 637168 | 3550.41 | 636694 | 0 | 30 |  | 7884 | 636960 |  | 2 |  | 0 | 0 (2) | 2 |
| (0) |  |  |  |  | & | Mid | 3619.98 | 641332 | 3594.51 | 639634 | 102 |  |  | 7928 | 641184 |  | 2 |  | 1 | 1 (6) | 109 |
|  |  |  |  |  | Uplink | High | 3682.5 | 645500 | 3584.67 | 638978 | 504 |  |  | 7971 | 645312 |  | 10 |  | 1 | 0 (2) | 507 |
|  | Channel spacing CC1-CC2=12.48 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 15 | 52 | Downlink | Low | 3570 | 638000 | 3565.32 | 637688 | 0 | 30 |  | 7895 | 638016 |  | 4 |  | 1 | 1 (6) | 7 |
|  |  |  |  |  | & | Mid | 3632.46 | 642164 | 3609.42 | 640628 | 102 |  |  | 7938 | 642144 |  | 4 |  | 2 | 0 (2) | 106 |
|  |  |  |  |  | Uplink | High | 3694.98 | 646332 | 3599.58 | 639972 | 504 |  |  | 7982 | 646368 |  | 0 |  | 3 | 1 (6) | 513 |
| 15+15 | CC1 | 15 | 15 | 79 | Downlink | Low | 3557.52 | 637168 | 3550.41 | 636694 | 0 | 30 |  | 7884 | 636960 |  | 2 |  | 0 | 0 (2) | 2 |
| (0,2) |  |  |  |  | & | Mid | 3617.49 | 641166 | 3592.02 | 639468 | 102 |  |  | 7926 | 640992 |  | 0 |  | 3 | 0 (2) | 107 |
| Note 6 |  |  |  |  | Uplink | High | 3677.49 | 645166 | 3579.66 | 638644 | 504 |  |  | 7968 | 645024 |  | 8 |  | 1 | 1 (6) | 511 |
|  | Channel spacing CC1-CC2=15 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 15 | 79 | Downlink | Low | 3572.52 | 638168 | 3565.41 | 637694 | 0 | 30 |  | 7895 | 638016 |  | 10 |  | 0 | 1 (6) | 6 |
|  |  |  |  |  | & | Mid | 3632.49 | 642166 | 3607.02 | 640468 | 102 |  |  | 7937 | 642048 |  | 8 |  | 3 | 1 (6) | 111 |
|  |  |  |  |  | Uplink | High | 3692.49 | 646166 | 3594.66 | 639644 | 504 |  |  | 7978 | 645984 |  | 4 |  | 2 | 0 (2) | 508 |
| 15+20 | CC1 | 15 | 15 | 79 | Downlink | Low | 3557.52 | 637168 | 3550.41 | 636694 | 0 | 30 |  | 7884 | 636960 |  | 2 |  | 0 | 0 (2) | 2 |
| (0,2) |  |  |  |  | & | Mid | 3615 | 641000 | 3589.53 | 639302 | 102 |  |  | 7924 | 640800 |  | 10 |  | 0 | 0 (2) | 104 |
| Note 6 |  |  |  |  | Uplink | High | 3672.84 | 644856 | 3575.01 | 638334 | 504 |  |  | 7965 | 644736 |  | 6 |  | 3 | 1 (6) | 513 |
|  | Channel spacing CC1-CC2=17.16 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 15 | 106 | Downlink | Low | 3574.68 | 638312 | 3565.14 | 637676 | 0 | 30 |  | 7895 | 638016 |  | 4 |  | 2 | 1 (6) | 8 |
|  |  |  |  |  | & | Mid | 3632.16 | 642144 | 3604.26 | 640284 | 102 |  |  | 7935 | 641856 |  | 0 |  | 3 | 1 (6) | 111 |
|  |  |  |  |  | Uplink | High | 3690 | 646000 | 3589.74 | 639316 | 504 |  |  | 7975 | 645696 |  | 8 |  | 1 | 1 (6) | 511 |
| 15+30 | CC1 | 15 | 15 | 79 | Downlink | Low | 3557.52 | 637168 | 3550.41 | 636694 | 0 | 30 |  | 7884 | 636960 |  | 2 |  | 0 | 0 (2) | 2 |
| (2) |  |  |  |  | & | Mid | 3609.99 | 640666 | 3584.52 | 638968 | 102 |  |  | 7921 | 640512 |  | 8 |  | 0 | 1 (6) | 108 |
|  |  |  |  |  | Uplink | High | 3662.79 | 644186 | 3564.96 | 637664 | 504 |  |  | 7958 | 644064 |  | 4 |  | 3 | 1 (6) | 513 |
|  | Channel spacing CC1-CC2=22.2 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 30 | 15 | 160 | Downlink | Low | 3579.72 | 638648 | 3565.32 | 637688 | 0 | 30 |  | 7895 | 638016 |  | 4 |  | 1 | 1 (6) | 7 |
|  |  |  |  |  |  | Mid | 3632.19 | 642146 | 3599.43 | 639962 | 102 |  |  | 7931 | 641472 |  | 10 |  | 1 | 0 (2) | 105 |
|  |  |  |  |  |  | High | 3684.99 | 645666 | 3579.87 | 638658 | 504 |  |  | 7968 | 645024 |  | 6 |  | 0 | 1 (6) | 510 |
| 15+40 | CC1 | 15 | 15 | 79 | Downlink | Low | 3557.52 | 637168 | 3550.41 | 636694 | 0 | 30 |  | 7884 | 636960 |  | 2 |  | 0 | 0 (2) | 2 |
| (0,1,2) |  |  |  |  | & | Mid | 3604.98 | 640332 | 3579.51 | 638634 | 102 |  |  | 7917 | 640128 |  | 6 |  | 0 | 0 (2) | 104 |
| Note 6 |  |  |  |  | Uplink | High | 3653.1 | 643540 | 3555.27 | 637018 | 504 |  |  | 7951 | 643392 |  | 2 |  | 1 | 1 (6) | 511 |
|  | Channel spacing CC1-CC2=26.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 15 | 216 | Downlink | Low | 3584.4 | 638960 | 3564.96 | 637664 | 0 | 30 |  | 7895 | 638016 |  | 4 |  | 3 | 1 (6) | 9 |
|  |  |  |  |  |  | Mid | 3631.86 | 642124 | 3594.06 | 639604 | 102 |  |  | 7928 | 641184 |  | 8 |  | 3 | 1 (6) | 111 |
|  |  |  |  |  |  | High | 3679.98 | 645332 | 3569.82 | 637988 | 504 |  |  | 7961 | 644352 |  | 4 |  | 0 | 1 (6) | 510 |
| 15+50 | CC1 | 15 | 15 | 79 | Downlink | Low | 3557.52 | 637168 | 3550.41 | 636694 | 0 | 30 |  | 7884 | 636960 |  | 2 |  | 0 | 0 (2) | 2 |
| (0,1,2) |  |  |  |  | & | Mid | 3600 | 640000 | 3574.53 | 638302 | 102 |  |  | 7914 | 639840 |  | 2 |  | 0 | 1 (6) | 108 |
| Note 6 |  |  |  |  | Uplink | High | 3643.08 | 642872 | 3545.25 | 636350 | 504 |  |  | 7944 | 642720 |  | 10 |  | 0 | 1 (6) | 510 |
|  | Channel spacing CC1-CC2=31.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 15 | 270 | Downlink | Low | 3589.44 | 639296 | 3565.14 | 637676 | 0 | 30 |  | 7895 | 638016 |  | 4 |  | 2 | 1 (6) | 8 |
|  |  |  |  |  |  | Mid | 3631.92 | 642128 | 3589.26 | 639284 | 102 |  |  | 7924 | 640800 |  | 4 |  | 2 | 0 (2) | 106 |
|  |  |  |  |  |  | High | 3675 | 645000 | 3559.98 | 637332 | 504 |  |  | 7954 | 643680 |  | 0 |  | 3 | 0 (2) | 509 |
| 20+5 | CC1 | 20 | 15 | 106 | Downlink | Low | 3560.01 | 637334 | 3550.47 | 636698 | 0 | 30 |  | 7885 | 637056 |  | 10 |  | 3 | 1 (6) | 9 |
| (0) |  |  |  |  | & | Mid | 3622.5 | 641500 | 3594.6 | 639640 | 102 |  |  | 7928 | 641184 |  | 8 |  | 0 | 1 (6) | 108 |
|  |  |  |  |  | Uplink | High | 3685.2 | 645680 | 3584.94 | 638996 | 504 |  |  | 7971 | 645312 |  | 4 |  | 0 | 0 (2) | 506 |
|  | Channel spacing CC1-CC2=12.285 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 5 | 15 | 25 | Downlink | Low | 3572.295 | 638153 | 3570.045 | 638003 | 0 | Note |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  | & | Mid | 3634.785 | 642319 | 3614.175 | 640945 | 102 | 5 |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  | Uplink | High | 3697.485 | 646499 | 3604.515 | 640301 | 504 |  |  | - | - |  | - |  | - | - | - |
| 20+10 | CC1 | 20 | 15 | 106 | Downlink | Low | 3560.01 | 637334 | 3550.47 | 636698 | 0 | 30 |  | 7885 | 637056 |  | 10 |  | 3 | 1 (6) | 9 |
| (0,2) |  |  |  |  | & | Mid | 3619.98 | 641332 | 3592.08 | 639472 | 102 |  |  | 7926 | 640992 |  | 8 |  | 2 | 0 (2) | 106 |
| Note 6 |  |  |  |  | Uplink | High | 3680.34 | 645356 | 3580.08 | 638672 | 504 |  |  | 7968 | 645024 |  | 4 |  | 3 | 0 (2) | 509 |
|  | Channel spacing CC1-CC2=14.64 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 15 | 52 | Downlink | Low | 3574.65 | 638310 | 3569.97 | 637998 | 0 | 30 |  | 7898 | 638304 |  | 6 |  | 3 | 0 (2) | 5 |
|  |  |  |  |  | & | Mid | 3634.62 | 642308 | 3611.58 | 640772 | 102 |  |  | 7940 | 642336 |  | 4 |  | 2 | 1 (6) | 110 |
|  |  |  |  |  | Uplink | High | 3694.98 | 646332 | 3599.58 | 639972 | 504 |  |  | 7982 | 646368 |  | 0 |  | 3 | 1 (6) | 513 |
| 20+15 | CC1 | 20 | 15 | 106 | Downlink | Low | 3560.01 | 637334 | 3550.47 | 636698 | 0 | 30 |  | 7885 | 637056 |  | 10 |  | 3 | 1 (6) | 9 |
| (0,2) |  |  |  |  | & | Mid | 3617.49 | 641166 | 3589.59 | 639306 | 102 |  |  | 7924 | 640800 |  | 6 |  | 0 | 0 (2) | 104 |
| Note 6 |  |  |  |  | Uplink | High | 3675.33 | 645022 | 3575.07 | 638338 | 504 |  |  | 7965 | 644736 |  | 2 |  | 3 | 1 (6) | 513 |
|  | Channel spacing CC1-CC2=17.16 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 15 | 79 | Downlink | Low | 3577.17 | 638478 | 3570.06 | 638004 | 0 | 30 |  | 7898 | 638304 |  | 0 |  | 3 | 0 (2) | 5 |
|  |  |  |  |  | & | Mid | 3634.65 | 642310 | 3609.18 | 640612 | 102 |  |  | 7938 | 642144 |  | 8 |  | 3 | 0 (2) | 107 |
|  |  |  |  |  | Uplink | High | 3692.49 | 646166 | 3594.66 | 639644 | 504 |  |  | 7978 | 645984 |  | 4 |  | 2 | 0 (2) | 508 |
| 20+20 | CC1 | 20 | 15 | 106 | Downlink | Low | 3560.01 | 637334 | 3550.47 | 636698 | 0 | 30 |  | 7885 | 637056 |  | 10 |  | 3 | 1 (6) | 9 |
| (0,2) |  |  |  |  | & | Mid | 3615 | 641000 | 3587.1 | 639140 | 102 |  |  | 7923 | 640704 |  | 4 |  | 2 | 1 (6) | 110 |
| Note 6 |  |  |  |  | Uplink | High | 3670.02 | 644668 | 3569.76 | 637984 | 504 |  |  | 7961 | 644352 |  | 8 |  | 0 | 1 (6) | 510 |
|  | Channel spacing CC1-CC2=19.98 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 15 | 106 | Downlink | Low | 3579.99 | 638666 | 3570.45 | 638030 | 0 | 30 |  | 7898 | 638304 |  | 10 |  | 0 | 0 (2) | 2 |
|  |  |  |  |  | & | Mid | 3634.98 | 642332 | 3607.08 | 640472 | 102 |  |  | 7937 | 642048 |  | 4 |  | 3 | 1 (6) | 111 |
|  |  |  |  |  | Uplink | High | 3690 | 646000 | 3589.74 | 639316 | 504 |  |  | 7975 | 645696 |  | 8 |  | 1 | 1 (6) | 511 |
| 20+30 | CC1 | 20 | 15 | 106 | Downlink | Low | 3560.01 | 637334 | 3550.47 | 636698 | 0 | 30 |  | 7885 | 637056 |  | 10 |  | 3 | 1 (6) | 9 |
| (2) |  |  |  |  | & | Mid | 3609.99 | 640666 | 3582.09 | 638806 | 102 |  |  | 7919 | 640320 |  | 2 |  | 2 | 0 (2) | 106 |
|  |  |  |  |  | Uplink | High | 3660.03 | 644002 | 3559.77 | 637318 | 504 |  |  | 7954 | 643680 |  | 2 |  | 0 | 1 (6) | 510 |
|  | Channel spacing CC1-CC2=24.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 30 | 15 | 160 | Downlink | Low | 3584.97 | 638998 | 3570.57 | 638038 | 0 | 30 |  | 7898 | 638304 |  | 2 |  | 0 | 0 (2) | 2 |
|  |  |  |  |  |  | Mid | 3634.95 | 642330 | 3602.19 | 640146 | 102 |  |  | 7933 | 641664 |  | 6 |  | 2 | 0 (2) | 106 |
|  |  |  |  |  |  | High | 3684.99 | 645666 | 3579.87 | 638658 | 504 |  |  | 7968 | 645024 |  | 6 |  | 0 | 1 (6) | 510 |
| 30+30 | CC1 | 30 | 15 | 160 | Downlink | Low | 3565.02 | 637668 | 3550.62 | 636708 | 0 | 30 |  | 7885 | 637056 |  | 0 |  | 3 | 1 (6) | 9 |
| (2) |  |  |  |  | & | Mid | 3609.99 | 640666 | 3577.23 | 638482 | 102 |  |  | 7916 | 640032 |  | 2 |  | 1 | 1 (6) | 109 |
|  |  |  |  |  | Uplink | High | 3654.99 | 643666 | 3549.87 | 636658 | 504 |  |  | 7947 | 643008 |  | 2 |  | 3 | 0 (2) | 509 |
|  | Channel spacing CC1-CC2=30 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 30 | 15 | 160 | Downlink | Low | 3595.02 | 639668 | 3580.62 | 638708 | 0 | 30 |  | 7905 | 638976 |  | 4 |  | 0 | 0 (2) | 2 |
|  |  |  |  |  |  | Mid | 3639.99 | 642666 | 3607.23 | 640482 | 102 |  |  | 7937 | 642048 |  | 6 |  | 2 | 1 (6) | 110 |
|  |  |  |  |  |  | High | 3684.99 | 645666 | 3579.87 | 638658 | 504 |  |  | 7968 | 645024 |  | 6 |  | 0 | 1 (6) | 510 |
| 40+30 | CC1 | 40 | 15 | 216 | Downlink | Low | 3570 | 638000 | 3550.56 | 636704 | 0 | 30 |  | 7885 | 637056 |  | 4 |  | 3 | 1 (6) | 9 |
| (2) |  |  |  |  | & | Mid | 3609.99 | 640666 | 3572.19 | 638146 | 102 |  |  | 7912 | 639648 |  | 2 |  | 1 | 0 (2) | 105 |
|  |  |  |  |  | Uplink | High | 3650.31 | 643354 | 3540.15 | 636010 | 504 |  |  | 7940 | 642336 |  | 2 |  | 1 | 0 (2) | 507 |
|  | Channel spacing CC1-CC2=34.68 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 30 | 15 | 160 | Downlink | Low | 3604.68 | 640312 | 3590.28 | 639352 | 0 | 30 |  | 7912 | 639648 |  | 8 |  | 2 | 0 (2) | 4 |
|  |  |  |  |  |  | Mid | 3644.67 | 642978 | 3611.91 | 640794 | 102 |  |  | 7940 | 642336 |  | 6 |  | 0 | 1 (6) | 108 |
|  |  |  |  |  |  | High | 3684.99 | 645666 | 3579.87 | 638658 | 504 |  |  | 7968 | 645024 |  | 6 |  | 0 | 1 (6) | 510 |
| 20+40 | CC1 | 20 | 15 | 106 | Downlink | Low | 3560.01 | 637334 | 3550.47 | 636698 | 0 | 30 |  | 7885 | 637056 |  | 10 |  | 3 | 1 (6) | 9 |
| (1,2) |  |  |  |  | & | Mid | 3604.98 | 640332 | 3577.08 | 638472 | 102 |  |  | 7916 | 640032 |  | 0 |  | 2 | 1 (6) | 110 |
|  |  |  |  |  | Uplink | High | 3650.28 | 643352 | 3550.02 | 636668 | 504 |  |  | 7947 | 643008 |  | 4 |  | 2 | 0 (2) | 508 |
|  | Channel spacing CC1-CC2=29.7 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 15 | 216 | Downlink | Low | 3589.71 | 639314 | 3570.27 | 638018 | 0 | 30 |  | 7898 | 638304 |  | 10 |  | 1 | 0 (2) | 3 |
|  |  |  |  |  |  | Mid | 3634.68 | 642312 | 3596.88 | 639792 | 102 |  |  | 7929 | 641280 |  | 0 |  | 0 | 0 (2) | 104 |
|  |  |  |  |  |  | High | 3679.98 | 645332 | 3569.82 | 637988 | 504 |  |  | 7961 | 644352 |  | 4 |  | 0 | 1 (6) | 510 |
| 20+50 | CC1 | 20 | 15 | 106 | Downlink | Low | 3560.01 | 637334 | 3550.47 | 636698 | 0 | 30 |  | 7885 | 637056 |  | 10 |  | 3 | 1 (6) | 9 |
| (1,2) |  |  |  |  | & | Mid | 3600 | 640000 | 3572.1 | 638140 | 102 |  |  | 7912 | 639648 |  | 8 |  | 1 | 0 (2) | 105 |
|  |  |  |  |  | Uplink | High | 3640.26 | 642684 | 3540 | 636000 | 504 |  |  | 7940 | 642336 |  | 0 |  | 2 | 0 (2) | 508 |
|  | Channel spacing CC1-CC2=34.74 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 15 | 270 | Downlink | Low | 3594.75 | 639650 | 3570.45 | 638030 | 0 | 30 |  | 7898 | 638304 |  | 10 |  | 0 | 0 (2) | 2 |
|  |  |  |  |  |  | Mid | 3634.74 | 642316 | 3592.08 | 639472 | 102 |  |  | 7926 | 640992 |  | 8 |  | 2 | 0 (2) | 106 |
|  |  |  |  |  |  | High | 3675 | 645000 | 3559.98 | 637332 | 504 |  |  | 7954 | 643680 |  | 0 |  | 3 | 0 (2) | 509 |
| 40+40 | CC1 | 40 | 15 | 216 | Downlink | Low | 3570 | 638000 | 3550.56 | 636704 | 0 | 30 |  | 7885 | 637056 |  | 4 |  | 3 | 1 (6) | 9 |
| (1,2) |  |  |  |  | & | Mid | 3604.98 | 640332 | 3567.18 | 637812 | 102 |  |  | 7909 | 639360 |  | 0 |  | 1 | 1 (6) | 109 |
|  |  |  |  |  | Uplink | High | 3640.02 | 642668 | 3529.86 | 635324 | 504 |  |  | 7933 | 641664 |  | 4 |  | 2 | 0 (2) | 508 |
|  | Channel spacing CC1-CC2=39.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 15 | 216 | Downlink | Low | 3609.96 | 640664 | 3590.52 | 639368 | 0 | 30 |  | 7912 | 639648 |  | 4 |  | 1 | 0 (2) | 3 |
|  |  |  |  |  |  | Mid | 3644.94 | 642996 | 3607.14 | 640476 | 102 |  |  | 7937 | 642048 |  | 0 |  | 3 | 1 (6) | 111 |
|  |  |  |  |  |  | High | 3679.98 | 645332 | 3569.82 | 637988 | 504 |  |  | 7961 | 644352 |  | 4 |  | 0 | 1 (6) | 510 |
| 40+50 | CC1 | 40 | 15 | 216 | Downlink | Low | 3570 | 638000 | 3550.56 | 636704 | 0 | 30 |  | 7885 | 637056 |  | 4 |  | 3 | 1 (6) | 9 |
| (1,2) |  |  |  |  | & | Mid | 3600 | 640000 | 3562.2 | 637480 | 102 |  |  | 7905 | 638976 |  | 8 |  | 0 | 0 (2) | 104 |
|  |  |  |  |  | Uplink | High | 3630.06 | 642004 | 3519.9 | 634660 | 504 |  |  | 7926 | 640992 |  | 8 |  | 1 | 0 (2) | 507 |
|  | Channel spacing CC1-CC2=44.94 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 15 | 270 | Downlink | Low | 3614.94 | 640996 | 3590.64 | 639376 | 0 | 30 |  | 7912 | 639648 |  | 8 |  | 0 | 0 (2) | 2 |
|  |  |  |  |  |  | Mid | 3644.94 | 642996 | 3602.28 | 640152 | 102 |  |  | 7933 | 641664 |  | 0 |  | 2 | 0 (2) | 106 |
|  |  |  |  |  |  | High | 3675 | 645000 | 3559.98 | 637332 | 504 |  |  | 7954 | 643680 |  | 0 |  | 3 | 0 (2) | 509 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-3 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 5: No SS/PBCH block fits within the channel bandwidth. The channel bandwidth can only be used as SCell. Note 6: UL CA is only supported for BCS0. I.e.the test frequencies for uplink CC2 are only supported for BCS0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.3.48.1-2: NR Intra-Band contiguous CA configuration CA_n48B (PCC=CC1 and SCC=CC2), CC1 SCS = 30kHz, CC2 SCS = 30kHz

| CBW combination [MHz] (BCS) | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10+10 | CC1 | 10 | 30 | 24 | Downlink | Low | 3555 | 637000 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (0,2) |  |  |  |  | & | Mid | 3620.01 | 641334 | 3578.97 | 638598 | 102 |  | 7930 | 641376 | 18 | 0 | 3 (3) | 210 |
| Note 5 |  |  |  |  | Uplink | High | 3685.02 | 645668 | 3499.26 | 633284 | 504 |  | 7975 | 645696 | 4 | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=9.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 30 | 24 | Downlink | Low | 3564.96 | 637664 | 3560.64 | 637376 | 0 | 30 | 7891 | 637632 | 16 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3629.97 | 641998 | 3588.93 | 639262 | 102 |  | 7936 | 641952 | 2 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3694.98 | 646332 | 3509.22 | 633948 | 504 |  | 7982 | 646368 | 12 | 0 | 3 (3) | 1014 |
| 10+15 | CC1 | 10 | 30 | 24 | Downlink | Low | 3555 | 637000 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (0,2) |  |  |  |  | & | Mid | 3617.49 | 641166 | 3576.45 | 638430 | 102 |  | 7928 | 641184 | 18 | 0 | 2 (2) | 208 |
| Note 5 |  |  |  |  | Uplink | High | 3680.01 | 645334 | 3494.25 | 632950 | 504 |  | 7971 | 645312 | 2 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=12.48 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 30 | 38 | Downlink | Low | 3567.48 | 637832 | 3560.64 | 637376 | 0 | 30 | 7891 | 637632 | 16 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3629.97 | 641998 | 3586.41 | 639094 | 102 |  | 7935 | 641856 | 2 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3692.49 | 646166 | 3504.21 | 633614 | 504 |  | 7978 | 645984 | 10 | 0 | 1 (1) | 1010 |
| 10+20 | CC1 | 10 | 30 | 24 | Downlink | Low | 3555 | 637000 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (0,2) |  |  |  |  | & | Mid | 3615 | 641000 | 3573.96 | 638264 | 102 |  | 7926 | 640992 | 16 | 0 | 1 (1) | 206 |
| Note 5 |  |  |  |  | Uplink | High | 3675.36 | 645024 | 3489.6 | 632640 | 504 |  | 7968 | 645024 | 0 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=14.64 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 30 | 51 | Downlink | Low | 3569.64 | 637976 | 3560.46 | 637364 | 0 | 30 | 7891 | 637632 | 4 | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3629.64 | 641976 | 3583.74 | 638916 | 102 |  | 7933 | 641664 | 12 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3690 | 646000 | 3499.38 | 633292 | 504 |  | 7975 | 645696 | 20 | 0 | 2 (2) | 1012 |
| 10+30 | CC1 | 10 | 30 | 24 | Downlink | Low | 3555 | 637000 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (2) |  |  |  |  | & | Mid | 3609.99 | 640666 | 3568.95 | 637930 | 102 |  | 7923 | 640704 | 14 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3665.31 | 644354 | 3479.55 | 631970 | 504 |  | 7961 | 644352 | 22 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=19.68 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 30 | 30 | 78 | Downlink | Low | 3574.68 | 638312 | 3560.64 | 637376 | 0 | 30 | 7891 | 637632 | 16 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3629.67 | 641978 | 3578.91 | 638594 | 102 |  | 7930 | 641376 | 22 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3684.99 | 645666 | 3489.51 | 632634 | 504 |  | 7968 | 645024 | 6 | 0 | 2 (2) | 1012 |
| 10+40 | CC1 | 10 | 30 | 24 | Downlink | Low | 3555 | 637000 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (2) |  |  |  |  | & | Mid | 3605.01 | 640334 | 3563.97 | 637598 | 102 |  | 7919 | 640320 | 10 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3655.62 | 643708 | 3469.86 | 631324 | 504 |  | 7954 | 643680 | 20 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=24.36 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 30 | 106 | Downlink | Low | 3579.36 | 638624 | 3560.28 | 637352 | 0 | 30 | 7891 | 637632 | 16 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3629.37 | 641958 | 3573.57 | 638238 | 102 |  | 7926 | 640992 | 18 | 0 | 2 (2) | 208 |
|  |  |  |  |  |  | High | 3679.98 | 645332 | 3479.46 | 631964 | 504 |  | 7961 | 644352 | 4 | 0 | 2 (2) | 1012 |
| 10+70 | CC1 | 10 | 30 | 24 | Downlink | Low | 3555 | 637000 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (2) |  |  |  |  | & | Mid | 3590.01 | 639334 | 3548.97 | 636598 | 102 |  | 7909 | 639360 | 2 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3625.5 | 641700 | 3439.74 | 629316 | 504 |  | 7933 | 641664 | 12 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=39.48 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3594.48 | 639632 | 3560.46 | 637364 | 0 | 30 | 7891 | 637632 | 4 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3629.49 | 641966 | 3558.75 | 637250 | 102 |  | 7916 | 640032 | 22 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3664.98 | 644332 | 3449.52 | 629968 | 504 |  | 7940 | 642336 | 8 | 0 | 1 (1) | 1010 |
| 10+50 | CC1 | 10 | 30 | 24 | Downlink | Low | 3555 | 637000 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (1,2) |  |  |  |  | & | Mid | 3600 | 640000 | 3558.96 | 637264 | 102 |  | 7916 | 640032 | 8 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3645.6 | 643040 | 3459.84 | 630656 | 504 |  | 7947 | 643008 | 16 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=29.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 3584.4 | 638960 | 3560.46 | 637364 | 0 | 30 | 7891 | 637632 | 4 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3629.4 | 641960 | 3568.74 | 637916 | 102 |  | 7922 | 640608 | 4 | 0 | 0 (0) | 204 |
|  |  |  |  |  |  | High | 3675 | 645000 | 3469.62 | 631308 | 504 |  | 7954 | 643680 | 12 | 0 | 1 (1) | 1010 |
| 10+60 | CC1 | 10 | 30 | 24 | Downlink | Low | 3555 | 637000 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (1,2) |  |  |  |  | & | Mid | 3594.99 | 639666 | 3553.95 | 636930 | 102 |  | 7912 | 639648 | 6 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3635.55 | 642370 | 3449.79 | 629986 | 504 |  | 7940 | 642336 | 14 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=34.44 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3589.44 | 639296 | 3560.28 | 637352 | 0 | 30 | 7891 | 637632 | 16 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3629.43 | 641962 | 3563.55 | 637570 | 102 |  | 7919 | 640320 | 14 | 0 | 2 (2) | 208 |
|  |  |  |  |  |  | High | 3669.99 | 644666 | 3459.39 | 630626 | 504 |  | 7947 | 643008 | 22 | 0 | 1 (1) | 1010 |
| 10+80 | CC1 | 10 | 30 | 24 | Downlink | Low | 3555 | 637000 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (1,2) |  |  |  |  | & | Mid | 3585 | 639000 | 3543.96 | 636264 | 102 |  | 7905 | 638976 | 0 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3615.48 | 641032 | 3429.72 | 628648 | 504 |  | 7926 | 640992 | 8 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=44.52 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3599.52 | 639968 | 3560.46 | 637364 | 0 | 30 | 7891 | 637632 | 4 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3629.52 | 641968 | 3553.74 | 636916 | 102 |  | 7912 | 639648 | 20 | 0 | 1 (1) | 206 |
|  |  |  |  |  |  | High | 3660 | 644000 | 3439.5 | 629300 | 504 |  | 7933 | 641664 | 4 | 0 | 1 (1) | 1010 |
| 10+90 | CC1 | 10 | 30 | 24 | Downlink | Low | 3555 | 637000 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (1,2) |  |  |  |  | & | Mid | 3579.99 | 638666 | 3538.95 | 635930 | 102 |  | 7902 | 638688 | 22 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3605.43 | 640362 | 3419.67 | 627978 | 504 |  | 7919 | 640320 | 6 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=49.56 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3604.56 | 640304 | 3560.46 | 637364 | 0 | 30 | 7891 | 637632 | 4 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3629.55 | 641970 | 3548.73 | 636582 | 102 |  | 7909 | 639360 | 18 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3654.99 | 643666 | 3429.45 | 628630 | 504 |  | 7926 | 640992 | 2 | 0 | 1 (1) | 1010 |
| 15+10 | CC1 | 15 | 30 | 38 | Downlink | Low | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (0,2) |  |  |  |  | & | Mid | 3620.01 | 641334 | 3576.45 | 638430 | 102 |  | 7928 | 641184 | 18 | 0 | 2 (2) | 208 |
| Note 5 |  |  |  |  | Uplink | High | 3682.5 | 645500 | 3494.22 | 632948 | 504 |  | 7971 | 645312 | 4 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=12.48 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 30 | 24 | Downlink | Low | 3570 | 638000 | 3565.68 | 637712 | 0 | 30 | 7895 | 638016 | 16 | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3632.49 | 642166 | 3591.45 | 639430 | 102 |  | 7938 | 642144 | 2 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3694.98 | 646332 | 3509.22 | 633948 | 504 |  | 7982 | 646368 | 12 | 0 | 3 (3) | 1014 |
| 15+15 | CC1 | 15 | 30 | 38 | Downlink | Low | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (0,2) |  |  |  |  | & | Mid | 3617.49 | 641166 | 3573.93 | 638262 | 102 |  | 7926 | 640992 | 18 | 0 | 1 (1) | 206 |
| Note 5 |  |  |  |  | Uplink | High | 3677.49 | 645166 | 3489.21 | 632614 | 504 |  | 7968 | 645024 | 2 | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=15 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 30 | 38 | Downlink | Low | 3572.52 | 638168 | 3565.68 | 637712 | 0 | 30 | 7895 | 638016 | 16 | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3632.49 | 642166 | 3588.93 | 639262 | 102 |  | 7936 | 641952 | 2 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3692.49 | 646166 | 3504.21 | 633614 | 504 |  | 7978 | 645984 | 10 | 0 | 1 (1) | 1010 |
| 15+20 | CC1 | 15 | 30 | 38 | Downlink | Low | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (0,2) |  |  |  |  | & | Mid | 3615 | 641000 | 3571.44 | 638096 | 102 |  | 7924 | 640800 | 16 | 0 | 0 (0) | 204 |
| Note 5 |  |  |  |  | Uplink | High | 3672.84 | 644856 | 3484.56 | 632304 | 504 |  | 7964 | 644640 | 0 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=17.16 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 30 | 51 | Downlink | Low | 3574.68 | 638312 | 3565.5 | 637700 | 0 | 30 | 7895 | 638016 | 4 | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3632.16 | 642144 | 3586.26 | 639084 | 102 |  | 7935 | 641856 | 12 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3690 | 646000 | 3499.38 | 633292 | 504 |  | 7975 | 645696 | 20 | 0 | 2 (2) | 1012 |
| 15+30 | CC1 | 15 | 30 | 38 | Downlink | Low | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (2) |  |  |  |  | & | Mid | 3609.99 | 640666 | 3566.43 | 637762 | 102 |  | 7921 | 640512 | 14 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3662.79 | 644186 | 3474.51 | 631634 | 504 |  | 7958 | 644064 | 22 | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=22.2 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 30 | 30 | 78 | Downlink | Low | 3579.72 | 638648 | 3565.68 | 637712 | 0 | 30 | 7895 | 638016 | 16 | 0 | 2 (2) | 4 |
|  |  |  |  |  |  | Mid | 3632.19 | 642146 | 3581.43 | 638762 | 102 |  | 7931 | 641472 | 22 | 0 | 0 (0) | 204 |
|  |  |  |  |  |  | High | 3684.99 | 645666 | 3489.51 | 632634 | 504 |  | 7968 | 645024 | 6 | 0 | 2 (2) | 1012 |
| 15+70 | CC1 | 15 | 30 | 38 | Downlink | Low | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (2) |  |  |  |  | & | Mid | 3590.01 | 639334 | 3546.45 | 636430 | 102 |  | 7907 | 639168 | 2 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3622.98 | 641532 | 3434.7 | 628980 | 504 |  | 7930 | 641376 | 12 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=42 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3599.52 | 639968 | 3565.5 | 637700 | 0 | 30 | 7895 | 638016 | 4 | 0 | 3 (3) | 6 |
|  |  |  |  |  |  | Mid | 3632.01 | 642134 | 3561.27 | 637418 | 102 |  | 7917 | 640128 | 22 | 0 | 0 (0) | 204 |
|  |  |  |  |  |  | High | 3664.98 | 644332 | 3449.52 | 629968 | 504 |  | 7940 | 642336 | 8 | 0 | 1 (1) | 1010 |
| 15+40 | CC1 | 15 | 30 | 38 | Downlink | Low | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (1,2) |  |  |  |  | & | Mid | 3605.01 | 640334 | 3561.45 | 637430 | 102 |  | 7917 | 640128 | 10 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3653.1 | 643540 | 3464.82 | 630988 | 504 |  | 7951 | 643392 | 20 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=26.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 30 | 106 | Downlink | Low | 3584.4 | 638960 | 3565.32 | 637688 | 0 | 30 | 7895 | 638016 | 16 | 0 | 3 (3) | 6 |
|  |  |  |  |  |  | Mid | 3631.89 | 642126 | 3576.09 | 638406 | 102 |  | 7928 | 641184 | 18 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3679.98 | 645332 | 3479.46 | 631964 | 504 |  | 7961 | 644352 | 4 | 0 | 2 (2) | 1012 |
| 15+50 | CC1 | 15 | 30 | 38 | Downlink | Low | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (1,2) |  |  |  |  | & | Mid | 3600 | 640000 | 3556.44 | 637096 | 102 |  | 7914 | 639840 | 8 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3643.08 | 642872 | 3454.8 | 630320 | 504 |  | 7944 | 642720 | 16 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=31.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 3589.44 | 639296 | 3565.5 | 637700 | 0 | 30 | 7895 | 638016 | 4 | 0 | 3 (3) | 6 |
|  |  |  |  |  |  | Mid | 3631.92 | 642128 | 3571.26 | 638084 | 102 |  | 7924 | 640800 | 4 | 0 | 1 (1) | 206 |
|  |  |  |  |  |  | High | 3675 | 645000 | 3469.62 | 631308 | 504 |  | 7954 | 643680 | 12 | 0 | 1 (1) | 1010 |
| 15+60 | CC1 | 15 | 30 | 38 | Downlink | Low | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (1,2) |  |  |  |  | & | Mid | 3594.99 | 639666 | 3551.43 | 636762 | 102 |  | 7910 | 639456 | 6 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3633.03 | 642202 | 3444.75 | 629650 | 504 |  | 7937 | 642048 | 14 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=36.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3594.48 | 639632 | 3565.32 | 637688 | 0 | 30 | 7895 | 638016 | 16 | 0 | 3 (3) | 6 |
|  |  |  |  |  |  | Mid | 3631.95 | 642130 | 3566.07 | 637738 | 102 |  | 7921 | 640512 | 14 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3669.99 | 644666 | 3459.39 | 630626 | 504 |  | 7947 | 643008 | 22 | 0 | 1 (1) | 1010 |
| 15+80 | CC1 | 15 | 30 | 38 | Downlink | Low | 3557.52 | 637168 | 3550.68 | 636712 | 0 | 30 | 7884 | 636960 | 8 | 0 | 0 (0) | 0 |
| (1,2) |  |  |  |  | & | Mid | 3585 | 639000 | 3541.44 | 636096 | 102 |  | 7903 | 638784 | 0 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3612.96 | 640864 | 3424.68 | 628312 | 504 |  | 7923 | 640704 | 8 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=47.04 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3604.56 | 640304 | 3565.5 | 637700 | 0 | 30 | 7895 | 638016 | 4 | 0 | 3 (3) | 6 |
|  |  |  |  |  |  | Mid | 3632.04 | 642136 | 3556.26 | 637084 | 102 |  | 7914 | 639840 | 20 | 0 | 2 (2) | 208 |
|  |  |  |  |  |  | High | 3660 | 644000 | 3439.5 | 629300 | 504 |  | 7933 | 641664 | 4 | 0 | 1 (1) | 1010 |
| 20+10 | CC1 | 20 | 30 | 51 | Downlink | Low | 3560.01 | 637334 | 3550.83 | 636722 | 0 | 30 | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
| (0,2) |  |  |  |  | & | Mid | 3620.01 | 641334 | 3574.11 | 638274 | 102 |  | 7926 | 640992 | 6 | 0 | 1 (1) | 206 |
| Note 5 |  |  |  |  | Uplink | High | 3680.34 | 645356 | 3489.72 | 632648 | 504 |  | 7968 | 645024 | 16 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=14.64 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 30 | 24 | Downlink | Low | 3574.65 | 638310 | 3570.33 | 638022 | 0 | 30 | 7898 | 638304 | 18 | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3634.65 | 642310 | 3593.61 | 639574 | 102 |  | 7940 | 642336 | 2 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3694.98 | 646332 | 3509.22 | 633948 | 504 |  | 7982 | 646368 | 12 | 0 | 3 (3) | 1014 |
| 20+15 | CC1 | 20 | 30 | 51 | Downlink | Low | 3560.01 | 637334 | 3550.83 | 636722 | 0 | 30 | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
| (0,2) |  |  |  |  | & | Mid | 3617.49 | 641166 | 3571.59 | 638106 | 102 |  | 7924 | 640800 | 6 | 0 | 0 (0) | 204 |
| Note 5 |  |  |  |  | Uplink | High | 3675.33 | 645022 | 3484.71 | 632314 | 504 |  | 7965 | 644736 | 14 | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=17.16 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 30 | 38 | Downlink | Low | 3577.17 | 638478 | 3570.33 | 638022 | 0 | 30 | 7898 | 638304 | 18 | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3634.65 | 642310 | 3591.09 | 639406 | 102 |  | 7938 | 642144 | 2 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3692.49 | 646166 | 3504.21 | 633614 | 504 |  | 7978 | 645984 | 10 | 0 | 1 (1) | 1010 |
| 20+20 | CC1 | 20 | 30 | 51 | Downlink | Low | 3560.01 | 637334 | 3550.83 | 636722 | 0 | 30 | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
| (0,2) |  |  |  |  | & | Mid | 3615 | 641000 | 3569.1 | 637940 | 102 |  | 7923 | 640704 | 4 | 0 | 3 (3) | 210 |
| Note 5 |  |  |  |  | Uplink | High | 3670.02 | 644668 | 3479.4 | 631960 | 504 |  | 7961 | 644352 | 8 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=19.98 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 30 | 51 | Downlink | Low | 3579.99 | 638666 | 3570.81 | 638054 | 0 | 30 | 7898 | 638304 | 10 | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3634.98 | 642332 | 3589.08 | 639272 | 102 |  | 7937 | 642048 | 16 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3690 | 646000 | 3499.38 | 633292 | 504 |  | 7975 | 645696 | 20 | 0 | 2 (2) | 1012 |
| 20+30 | CC1 | 20 | 30 | 51 | Downlink | Low | 3560.01 | 637334 | 3550.83 | 636722 | 0 | 30 | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3609.99 | 640666 | 3564.09 | 637606 | 102 |  | 7919 | 640320 | 2 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3660.03 | 644002 | 3469.41 | 631294 | 504 |  | 7954 | 643680 | 2 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=24.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 30 | 30 | 78 | Downlink | Low | 3584.97 | 638998 | 3570.93 | 638062 | 0 | 30 | 7898 | 638304 | 2 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3634.95 | 642330 | 3584.19 | 638946 | 102 |  | 7933 | 641664 | 6 | 0 | 1 (1) | 206 |
|  |  |  |  |  |  | High | 3684.99 | 645666 | 3489.51 | 632634 | 504 |  | 7968 | 645024 | 6 | 0 | 2 (2) | 1012 |
| 20+70 | CC1 | 20 | 30 | 51 | Downlink | Low | 3560.01 | 637334 | 3550.83 | 636722 | 0 | 30 | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3590.01 | 639334 | 3544.11 | 636274 | 102 |  | 7905 | 638976 | 14 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3620.16 | 641344 | 3429.54 | 628636 | 504 |  | 7926 | 640992 | 20 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=44.82 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3604.83 | 640322 | 3570.81 | 638054 | 0 | 30 | 7898 | 638304 | 10 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3634.83 | 642322 | 3564.09 | 637606 | 102 |  | 7919 | 640320 | 2 | 0 | 1 (1) | 206 |
|  |  |  |  |  |  | High | 3664.98 | 644332 | 3449.52 | 629968 | 504 |  | 7940 | 642336 | 8 | 0 | 1 (1) | 1010 |
| 30+10 | CC1 | 30 | 30 | 78 | Downlink | Low | 3565.02 | 637668 | 3550.98 | 636732 | 0 | 30 | 7885 | 637056 | 12 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3620.01 | 641334 | 3569.25 | 637950 | 102 |  | 7923 | 640704 | 18 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3675.3 | 645020 | 3479.82 | 631988 | 504 |  | 7961 | 644352 | 4 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=19.68 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 30 | 24 | Downlink | Low | 3584.7 | 638980 | 3580.38 | 638692 | 0 | 30 | 7905 | 638976 | 20 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3639.69 | 642646 | 3598.65 | 639910 | 102 |  | 7943 | 642624 | 2 | 0 | 1 (1) | 206 |
|  |  |  |  |  |  | High | 3694.98 | 646332 | 3509.22 | 633948 | 504 |  | 7982 | 646368 | 12 | 0 | 3 (3) | 1014 |
| 30+15 | CC1 | 30 | 30 | 78 | Downlink | Low | 3565.02 | 637668 | 3550.98 | 636732 | 0 | 30 | 7885 | 637056 | 12 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3617.49 | 641166 | 3566.73 | 637782 | 102 |  | 7921 | 640512 | 18 | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3670.29 | 644686 | 3474.81 | 631654 | 504 |  | 7958 | 644064 | 2 | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=22.2 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 30 | 38 | Downlink | Low | 3587.22 | 639148 | 3580.38 | 638692 | 0 | 30 | 7905 | 638976 | 20 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3639.69 | 642646 | 3596.13 | 639742 | 102 |  | 7941 | 642432 | 2 | 0 | 0 (0) | 204 |
|  |  |  |  |  |  | High | 3692.49 | 646166 | 3504.21 | 633614 | 504 |  | 7978 | 645984 | 10 | 0 | 1 (1) | 1010 |
| 30+20 | CC1 | 30 | 30 | 78 | Downlink | Low | 3565.02 | 637668 | 3550.98 | 636732 | 0 | 30 | 7885 | 637056 | 12 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3615 | 641000 | 3564.24 | 637616 | 102 |  | 7919 | 640320 | 16 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3665.04 | 644336 | 3469.56 | 631304 | 504 |  | 7954 | 643680 | 16 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=24.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 30 | 51 | Downlink | Low | 3589.98 | 639332 | 3580.8 | 638720 | 0 | 30 | 7905 | 638976 | 16 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3639.96 | 642664 | 3594.06 | 639604 | 102 |  | 7940 | 642336 | 20 | 0 | 1 (1) | 206 |
|  |  |  |  |  |  | High | 3690 | 646000 | 3499.38 | 633292 | 504 |  | 7975 | 645696 | 20 | 0 | 2 (2) | 1012 |
| 30+30 | CC1 | 30 | 30 | 78 | Downlink | Low | 3565.02 | 637668 | 3550.98 | 636732 | 0 | 30 | 7885 | 637056 | 12 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3609.99 | 640666 | 3559.23 | 637282 | 102 |  | 7916 | 640032 | 14 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3654.99 | 643666 | 3459.51 | 630634 | 504 |  | 7947 | 643008 | 14 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=30 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 30 | 30 | 78 | Downlink | Low | 3595.02 | 639668 | 3580.98 | 638732 | 0 | 30 | 7905 | 638976 | 4 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3639.99 | 642666 | 3589.23 | 639282 | 102 |  | 7937 | 642048 | 6 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3684.99 | 645666 | 3489.51 | 632634 | 504 |  | 7968 | 645024 | 6 | 0 | 2 (2) | 1012 |
| 30+40 | CC1 | 30 | 30 | 78 | Downlink | Low | 3565.02 | 637668 | 3550.98 | 636732 | 0 | 30 | 7885 | 637056 | 12 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3605.01 | 640334 | 3554.25 | 636950 | 102 |  | 7912 | 639648 | 10 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3645.3 | 643020 | 3449.82 | 629988 | 504 |  | 7940 | 642336 | 12 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=34.68 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 30 | 106 | Downlink | Low | 3599.7 | 639980 | 3580.62 | 638708 | 0 | 30 | 7905 | 638976 | 4 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3639.69 | 642646 | 3583.89 | 638926 | 102 |  | 7933 | 641664 | 2 | 0 | 2 (2) | 208 |
|  |  |  |  |  |  | High | 3679.98 | 645332 | 3479.46 | 631964 | 504 |  | 7961 | 644352 | 4 | 0 | 2 (2) | 1012 |
| 30+50 | CC1 | 30 | 30 | 78 | Downlink | Low | 3565.02 | 637668 | 3550.98 | 636732 | 0 | 30 | 7885 | 637056 | 12 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3600 | 640000 | 3549.24 | 636616 | 102 |  | 7909 | 639360 | 8 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3635.28 | 642352 | 3439.8 | 629320 | 504 |  | 7933 | 641664 | 8 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=39.72 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 3604.74 | 640316 | 3580.8 | 638720 | 0 | 30 | 7905 | 638976 | 16 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3639.72 | 642648 | 3579.06 | 638604 | 102 |  | 7930 | 641376 | 12 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3675 | 645000 | 3469.62 | 631308 | 504 |  | 7954 | 643680 | 12 | 0 | 1 (1) | 1010 |
| 30+60 | CC1 | 30 | 30 | 78 | Downlink | Low | 3565.02 | 637668 | 3550.98 | 636732 | 0 | 30 | 7885 | 637056 | 12 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3594.99 | 639666 | 3544.23 | 636282 | 102 |  | 7905 | 638976 | 6 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3625.23 | 641682 | 3429.75 | 628650 | 504 |  | 7926 | 640992 | 6 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=44.76 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3609.78 | 640652 | 3580.62 | 638708 | 0 | 30 | 7905 | 638976 | 4 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3639.75 | 642650 | 3573.87 | 638258 | 102 |  | 7926 | 640992 | 22 | 0 | 1 (1) | 206 |
|  |  |  |  |  |  | High | 3669.99 | 644666 | 3459.39 | 630626 | 504 |  | 7947 | 643008 | 22 | 0 | 1 (1) | 1010 |
| 30+70 | CC1 | 30 | 30 | 78 | Downlink | Low | 3565.02 | 637668 | 3550.98 | 636732 | 0 | 30 | 7885 | 637056 | 12 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3590.01 | 639334 | 3539.25 | 635950 | 102 |  | 7902 | 638688 | 2 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3615.18 | 641012 | 3419.7 | 627980 | 504 |  | 7919 | 640320 | 4 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=49.8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3614.82 | 640988 | 3580.8 | 638720 | 0 | 30 | 7905 | 638976 | 16 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3639.81 | 642654 | 3569.07 | 637938 | 102 |  | 7923 | 640704 | 6 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3664.98 | 644332 | 3449.52 | 629968 | 504 |  | 7940 | 642336 | 8 | 0 | 1 (1) | 1010 |
| 40+10 | CC1 | 40 | 30 | 106 | Downlink | Low | 3570 | 638000 | 3550.92 | 636728 | 0 | 30 | 7885 | 637056 | 16 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3620.01 | 641334 | 3564.21 | 637614 | 102 |  | 7919 | 640320 | 18 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3670.62 | 644708 | 3470.1 | 631340 | 504 |  | 7954 | 643680 | 4 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=24.36 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 30 | 24 | Downlink | Low | 3594.36 | 639624 | 3590.04 | 639336 | 0 | 30 | 7912 | 639648 | 0 | 0 | 3 (3) | 6 |
|  |  |  |  |  |  | Mid | 3644.37 | 642958 | 3603.33 | 640222 | 102 |  | 7946 | 642912 | 2 | 0 | 0 (0) | 204 |
|  |  |  |  |  |  | High | 3694.98 | 646332 | 3509.22 | 633948 | 504 |  | 7982 | 646368 | 12 | 0 | 3 (3) | 1014 |
| 40+15 | CC1 | 40 | 30 | 106 | Downlink | Low | 3570 | 638000 | 3550.92 | 636728 | 0 | 30 | 7885 | 637056 | 16 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3617.49 | 641166 | 3561.69 | 637446 | 102 |  | 7918 | 640224 | 18 | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3665.61 | 644374 | 3465.09 | 631006 | 504 |  | 7951 | 643392 | 2 | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=26.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 30 | 38 | Downlink | Low | 3596.88 | 639792 | 3590.04 | 639336 | 0 | 30 | 7912 | 639648 | 0 | 0 | 3 (3) | 6 |
|  |  |  |  |  |  | Mid | 3644.37 | 642958 | 3600.81 | 640054 | 102 |  | 7945 | 642816 | 2 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3692.49 | 646166 | 3504.21 | 633614 | 504 |  | 7978 | 645984 | 10 | 0 | 1 (1) | 1010 |
| 40+20 | CC1 | 40 | 30 | 106 | Downlink | Low | 3570 | 638000 | 3550.92 | 636728 | 0 | 30 | 7885 | 637056 | 16 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3615 | 641000 | 3559.2 | 637280 | 102 |  | 7916 | 640032 | 16 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3660.3 | 644020 | 3459.78 | 630652 | 504 |  | 7947 | 643008 | 20 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=29.7 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 30 | 51 | Downlink | Low | 3599.7 | 639980 | 3590.52 | 639368 | 0 | 30 | 7912 | 639648 | 16 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3644.7 | 642980 | 3598.8 | 639920 | 102 |  | 7943 | 642624 | 16 | 0 | 0 (0) | 204 |
|  |  |  |  |  |  | High | 3690 | 646000 | 3499.38 | 633292 | 504 |  | 7975 | 645696 | 20 | 0 | 2 (2) | 1012 |
| 40+30 | CC1 | 40 | 30 | 106 | Downlink | Low | 3570 | 638000 | 3550.92 | 636728 | 0 | 30 | 7885 | 637056 | 16 | 0 | 3 (3) | 6 |
| (2) |  |  |  |  | & | Mid | 3609.99 | 640666 | 3554.19 | 636946 | 102 |  | 7912 | 639648 | 14 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3650.31 | 643354 | 3449.79 | 629986 | 504 |  | 7940 | 642336 | 14 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=34.68 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 30 | 30 | 78 | Downlink | Low | 3604.68 | 640312 | 3590.64 | 639376 | 0 | 30 | 7912 | 639648 | 8 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3644.67 | 642978 | 3593.91 | 639594 | 102 |  | 7940 | 642336 | 6 | 0 | 2 (2) | 208 |
|  |  |  |  |  |  | High | 3684.99 | 645666 | 3489.51 | 632634 | 504 |  | 7968 | 645024 | 6 | 0 | 2 (2) | 1012 |
| 20+40 | CC1 | 20 | 30 | 51 | Downlink | Low | 3560.01 | 637334 | 3550.83 | 636722 | 0 | 30 | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
| (1,2) |  |  |  |  | & | Mid | 3605.01 | 640334 | 3559.11 | 637274 | 102 |  | 7916 | 640032 | 22 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3650.28 | 643352 | 3459.66 | 630644 | 504 |  | 7947 | 643008 | 4 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=29.7 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 30 | 106 | Downlink | Low | 3589.71 | 639314 | 3570.63 | 638042 | 0 | 30 | 7898 | 638304 | 22 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3634.71 | 642314 | 3578.91 | 638594 | 102 |  | 7930 | 641376 | 22 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3679.98 | 645332 | 3479.46 | 631964 | 504 |  | 7961 | 644352 | 4 | 0 | 2 (2) | 1012 |
| 20+50 | CC1 | 20 | 30 | 51 | Downlink | Low | 3560.01 | 637334 | 3550.83 | 636722 | 0 | 30 | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
| (1,2) |  |  |  |  | & | Mid | 3600 | 640000 | 3554.1 | 636940 | 102 |  | 7912 | 639648 | 20 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3640.26 | 642684 | 3449.64 | 629976 | 504 |  | 7940 | 642336 | 0 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=34.74 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 3594.75 | 639650 | 3570.81 | 638054 | 0 | 30 | 7898 | 638304 | 10 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3634.74 | 642316 | 3574.08 | 638272 | 102 |  | 7926 | 640992 | 8 | 0 | 1 (1) | 206 |
|  |  |  |  |  |  | High | 3675 | 645000 | 3469.62 | 631308 | 504 |  | 7954 | 643680 | 12 | 0 | 1 (1) | 1010 |
| 20+60 | CC1 | 20 | 30 | 51 | Downlink | Low | 3560.01 | 637334 | 3550.83 | 636722 | 0 | 30 | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
| (1,2) |  |  |  |  | & | Mid | 3594.99 | 639666 | 3549.09 | 636606 | 102 |  | 7909 | 639360 | 18 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3630.21 | 642014 | 3439.59 | 629306 | 504 |  | 7933 | 641664 | 22 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=39.78 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3599.79 | 639986 | 3570.63 | 638042 | 0 | 30 | 7898 | 638304 | 22 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3634.77 | 642318 | 3568.89 | 637926 | 102 |  | 7923 | 640704 | 18 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3669.99 | 644666 | 3459.39 | 630626 | 504 |  | 7947 | 643008 | 22 | 0 | 1 (1) | 1010 |
| 20+80 | CC1 | 20 | 30 | 51 | Downlink | Low | 3560.01 | 637334 | 3550.83 | 636722 | 0 | 30 | 7885 | 637056 | 22 | 0 | 3 (3) | 6 |
| (1,2) |  |  |  |  | & | Mid | 3585 | 639000 | 3539.1 | 635940 | 102 |  | 7902 | 638688 | 12 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3610.14 | 640676 | 3419.52 | 627968 | 504 |  | 7919 | 640320 | 16 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=49.86 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3609.87 | 640658 | 3570.81 | 638054 | 0 | 30 | 7898 | 638304 | 10 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3634.86 | 642324 | 3559.08 | 637272 | 102 |  | 7916 | 640032 | 0 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3660 | 644000 | 3439.5 | 629300 | 504 |  | 7933 | 641664 | 4 | 0 | 1 (1) | 1010 |
| 40+40 | CC1 | 40 | 30 | 106 | Downlink | Low | 3570 | 638000 | 3550.92 | 636728 | 0 | 30 | 7885 | 637056 | 16 | 0 | 3 (3) | 6 |
| (0,1,2) |  |  |  |  | & | Mid | 3605.01 | 640334 | 3549.21 | 636614 | 102 |  | 7909 | 639360 | 10 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3640.02 | 642668 | 3439.5 | 629300 | 504 |  | 7933 | 641664 | 4 | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=39.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 30 | 106 | Downlink | Low | 3609.96 | 640664 | 3590.88 | 639392 | 0 | 30 | 7912 | 639648 | 16 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3644.97 | 642998 | 3589.17 | 639278 | 102 |  | 7937 | 642048 | 10 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3679.98 | 645332 | 3479.46 | 631964 | 504 |  | 7961 | 644352 | 4 | 0 | 2 (2) | 1012 |
| 40+50 | CC1 | 40 | 30 | 106 | Downlink | Low | 3570 | 638000 | 3550.92 | 636728 | 0 | 30 | 7885 | 637056 | 16 | 0 | 3 (3) | 6 |
| (1,2) |  |  |  |  | & | Mid | 3600 | 640000 | 3544.2 | 636280 | 102 |  | 7905 | 638976 | 8 | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3630.06 | 642004 | 3429.54 | 628636 | 504 |  | 7926 | 640992 | 20 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=44.94 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 3614.94 | 640996 | 3591 | 639400 | 0 | 30 | 7912 | 639648 | 8 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 3644.94 | 642996 | 3584.28 | 638952 | 102 |  | 7933 | 641664 | 0 | 0 | 1 (1) | 206 |
|  |  |  |  |  |  | High | 3675 | 645000 | 3469.62 | 631308 | 504 |  | 7954 | 643680 | 12 | 0 | 1 (1) | 1010 |
| 40+60 | CC1 | 40 | 30 | 106 | Downlink | Low | 3570 | 638000 | 3550.92 | 636728 | 0 | 30 | 7885 | 637056 | 16 | 0 | 3 (3) | 6 |
| (1,2) |  |  |  |  | & | Mid | 3594.99 | 639666 | 3539.19 | 635946 | 102 |  | 7902 | 638688 | 6 | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3620.07 | 641338 | 3419.55 | 627970 | 504 |  | 7919 | 640320 | 14 | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=49.92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3619.92 | 641328 | 3590.76 | 639384 | 0 | 30 | 7912 | 639648 | 0 | 0 | 1 (1) | 2 |
|  |  |  |  |  |  | Mid | 3644.91 | 642994 | 3579.03 | 638602 | 102 |  | 7930 | 641376 | 14 | 0 | 3 (3) | 210 |
|  |  |  |  |  |  | High | 3669.99 | 644666 | 3459.39 | 630626 | 504 |  | 7947 | 643008 | 22 | 0 | 1 (1) | 1010 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 5: UL CA is only supported for BCS0. I.e. the test frequencies for uplink CC2 are only supported for BCS0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.3.49 – 4.3.1.1.3.65	FFS

4.3.1.1.3.66	NR Intra-band contiguous configurations CA_n66

4.3.1.1.3.66.1	CA_n66B

Editor's note: Test frequencies for CA_n66B with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.3.66.1-1: NR Intra-Band contiguous CA configuration CA_n66B (PCC=CC1 and SCC=CC2), SCS 15 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5+20 | CC1 | 5 | 15 | 25 | Downlink | Low | 2112.5 | 422500 | 2110.25 | 422050 | 0 | 15 | 5279 | 422410 | 0 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2124.39 | 424878 | 102 |  | 5361 | 428910 | 0 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2177.5 | 435500 | 2084.53 | 416906 | 504 |  | 5443 | 435410 | 0 | 0 | 0 (0) | 504 |
|  |  |  |  |  | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1652.03 | 330406 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1777.5 | 355500 | 1774.17 | 354834 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=12 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 15 | 106 | Downlink | Low | 2124.5 | 424900 | 2114.96 | 422992 | 0 | 15 | 5291 | 423370 | 6 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 2157 | 431400 | 2129.1 | 425820 | 102 |  | 5373 | 429870 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2189.5 | 437900 | 2089.24 | 417848 | 504 |  | 5455 | 436370 | 6 | 0 | 0 (0) | 504 |
| 5+40 | CC1 | 5 | 15 | 25 | Downlink | Low | 2112.5 | 422500 | 2110.25 | 422050 | 0 | 15 | 5279 | 422410 | 0 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 2135.3 | 427060 | 2114.69 | 422938 | 102 |  | 5336 | 426970 | 0 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2158.1 | 431620 | 2065.13 | 413026 | 504 |  | 5393 | 431530 | 0 | 0 | 0 (0) | 504 |
|  |  |  |  |  | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1735.3 | 347060 | 1642.33 | 328466 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1758.1 | 351620 | 1754.77 | 350954 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=21.9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 15 | 216 | Downlink | Low | 2134.4 | 426880 | 2114.96 | 422992 | 0 | 15 | 5291 | 423370 | 6 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 2157.2 | 431440 | 2119.4 | 423880 | 102 |  | 5348 | 427930 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2180 | 436000 | 2069.84 | 413968 | 504 |  | 5405 | 432490 | 6 | 0 | 0 (0) | 504 |
| 10+15 | CC1 | 10 | 15 | 52 | Downlink | Low | 2115 | 423000 | 2110.32 | 422064 | 0 | 15 | 5280 | 422430 | 2 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2121.96 | 424392 | 102 |  | 5355 | 428430 | 2 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2175 | 435000 | 2079.6 | 415920 | 504 |  | 5430 | 434430 | 2 | 0 | 0 (0) | 504 |
|  |  |  |  |  | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=12.3 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 15 | 79 | Downlink | Low | 2127.3 | 425460 | 2120.19 | 424038 | 0 | 15 | 5307 | 424590 | 4 | 1 | 2 (4) | 5 |
|  |  |  |  |  |  | Mid | 2157.3 | 431460 | 2131.83 | 426366 | 102 |  | 5382 | 430590 | 4 | 1 | 2 (4) | 107 |
|  |  |  |  |  |  | High | 2187.3 | 437460 | 2089.47 | 417894 | 504 |  | 5457 | 436590 | 4 | 1 | 2 (4) | 509 |
| 10+20 | CC1 | 10 | 15 | 52 | Downlink | Low | 2115 | 423000 | 2110.32 | 422064 | 0 | 15 | 5280 | 422430 | 2 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2121.96 | 424392 | 102 |  | 5355 | 428430 | 2 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2175 | 435000 | 2079.6 | 415920 | 504 |  | 5430 | 434430 | 2 | 0 | 0 (0) | 504 |
|  |  |  |  |  | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=14.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 15 | 106 | Downlink | Low | 2129.4 | 425880 | 2119.86 | 423972 | 0 | 15 | 5304 | 424350 | 6 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 2159.4 | 431880 | 2131.5 | 426300 | 102 |  | 5379 | 430350 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2189.4 | 437880 | 2089.14 | 417828 | 504 |  | 5454 | 436350 | 6 | 0 | 0 (0) | 504 |
| 10+40 | CC1 | 10 | 15 | 52 | Downlink | Low | 2115 | 423000 | 2110.32 | 422064 | 0 | 15 | 5280 | 422430 | 2 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 2135.4 | 427080 | 2112.36 | 422472 | 102 |  | 5331 | 426510 | 2 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2155.7 | 431140 | 2060.3 | 412060 | 504 |  | 5381 | 430570 | 2 | 0 | 0 (0) | 504 |
|  |  |  |  |  | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1735.4 | 347080 | 1640 | 328000 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1755.7 | 351140 | 1749.94 | 349988 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=24.3 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 15 | 216 | Downlink | Low | 2139.3 | 427860 | 2119.86 | 423972 | 0 | 15 | 5304 | 424350 | 6 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 2159.7 | 431940 | 2121.9 | 424380 | 102 |  | 5355 | 428430 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2180 | 436000 | 2069.84 | 413968 | 504 |  | 5405 | 432490 | 6 | 0 | 0 (0) | 504 |
| 15+10 | CC1 | 15 | 15 | 79 | Downlink | Low | 2117.5 | 423500 | 2110.39 | 422078 | 0 | 15 | 5281 | 422450 | 4 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2119.53 | 423906 | 102 |  | 5349 | 427950 | 4 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2172.5 | 434500 | 2074.67 | 414934 | 504 |  | 5417 | 433450 | 4 | 0 | 0 (0) | 504 |
|  |  |  |  |  | Uplink | Low | 1717.5 | 343500 | 1710.39 | 342078 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1647.17 | 329434 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1772.5 | 354500 | 1764.31 | 352862 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=12.3 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 15 | 52 | Downlink | Low | 2129.8 | 425960 | 2125.12 | 425024 | 0 | 15 | 5320 | 425570 | 2 | 1 | 2 (4) | 5 |
|  |  |  |  |  |  | Mid | 2157.3 | 431460 | 2134.26 | 426852 | 102 |  | 5388 | 431070 | 2 | 1 | 2 (4) | 107 |
|  |  |  |  |  |  | High | 2184.8 | 436960 | 2089.4 | 417880 | 504 |  | 5456 | 436570 | 2 | 1 | 2 (4) | 509 |
| 15+15 | CC1 | 15 | 15 | 79 | Downlink | Low | 2117.5 | 423500 | 2110.39 | 422078 | 0 | 15 | 5281 | 422450 | 4 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2119.53 | 423906 | 102 |  | 5349 | 427950 | 4 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2172.5 | 434500 | 2074.67 | 414934 | 504 |  | 5417 | 433450 | 4 | 0 | 0 (0) | 504 |
|  |  |  |  |  | Uplink | Low | 1717.5 | 343500 | 1710.39 | 342078 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1647.17 | 329434 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1772.5 | 354500 | 1764.31 | 352862 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=15 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 15 | 79 | Downlink | Low | 2132.5 | 426500 | 2125.39 | 425078 | 0 | 15 | 5320 | 425570 | 8 | 1 | 1 (2) | 3 |
|  |  |  |  |  |  | Mid | 2160 | 432000 | 2134.53 | 426906 | 102 |  | 5388 | 431070 | 8 | 1 | 1 (2) | 105 |
|  |  |  |  |  |  | High | 2187.5 | 437500 | 2089.67 | 417934 | 504 |  | 5456 | 436570 | 8 | 1 | 1 (2) | 507 |
| 15+20 | CC1 | 15 | 15 | 79 | Downlink | Low | 2117.5 | 423500 | 2110.39 | 422078 | 0 | 15 | 5281 | 422450 | 4 | 0 | 0 (0) | 0 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2119.53 | 423906 | 102 |  | 5349 | 427950 | 4 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2172.5 | 434500 | 2074.67 | 414934 | 504 |  | 5417 | 433450 | 4 | 0 | 0 (0) | 504 |
|  |  |  |  |  | Uplink | Low | 1717.5 | 343500 | 1710.39 | 342078 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1647.17 | 329434 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1772.5 | 354500 | 1764.31 | 352862 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=17.1 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 15 | 106 | Downlink | Low | 2134.6 | 426920 | 2125.06 | 425012 | 0 | 15 | 5320 | 425570 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  |  |  | Mid | 2162.1 | 432420 | 2134.2 | 426840 | 102 |  | 5388 | 431070 | 6 | 1 | 2 (4) | 107 |
|  |  |  |  |  |  | High | 2189.6 | 437920 | 2089.34 | 417868 | 504 |  | 5456 | 436570 | 6 | 1 | 2 (4) | 509 |
| 20+5 | CC1 | 20 | 15 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2117.1 | 423420 | 102 |  | 5343 | 427470 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2170 | 434000 | 2069.74 | 413948 | 504 |  | 5407 | 432530 | 2 | 0 | 1 (2) | 506 |
|  |  |  |  |  | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1644.74 | 328948 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1770 | 354000 | 1759.38 | 351876 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=12 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 5 | 15 | 25 | Downlink | Low | 2132 | 426400 | 2129.75 | 425950 | 0 | 15 | 5330 | 426490 | 0 | 1 | 2 (4) | 5 |
|  |  |  |  |  |  | Mid | 2157 | 431400 | 2136.39 | 427278 | 102 |  | 5391 | 431310 | 0 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2182 | 436400 | 2089.03 | 417806 | 504 |  | 5455 | 436370 | 8 | 1 | 0 (0) | 505 |
| 20+10 | CC1 | 20 | 15 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2117.1 | 423420 | 102 |  | 5343 | 427470 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2170 | 434000 | 2069.74 | 413948 | 504 |  | 5407 | 432530 | 2 | 0 | 1 (2) | 506 |
|  |  |  |  |  | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1644.74 | 328948 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1770 | 354000 | 1759.38 | 351876 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=14.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 15 | 52 | Downlink | Low | 2134.4 | 426880 | 2129.72 | 425944 | 0 | 15 | 5330 | 426490 | 2 | 1 | 2 (4) | 5 |
|  |  |  |  |  |  | Mid | 2159.4 | 431880 | 2136.36 | 427272 | 102 |  | 5391 | 431310 | 2 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2184.4 | 436880 | 2089 | 417800 | 504 |  | 5455 | 436370 | 10 | 1 | 0 (0) | 505 |
| 20+15 | CC1 | 20 | 15 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2117.1 | 423420 | 102 |  | 5343 | 427470 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  |  |  | High | 2170 | 434000 | 2069.74 | 413948 | 504 |  | 5407 | 432530 | 2 | 0 | 1 (2) | 506 |
|  |  |  |  |  | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1644.74 | 328948 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1770 | 354000 | 1759.38 | 351876 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=17.1 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 15 | 79 | Downlink | Low | 2137.1 | 427420 | 2129.99 | 425998 | 0 | 15 | 5330 | 426490 | 8 | 1 | 1 (2) | 3 |
|  |  |  |  |  |  | Mid | 2162.1 | 432420 | 2136.63 | 427326 | 102 |  | 5394 | 431550 | 4 | 1 | 2 (4) | 107 |
|  |  |  |  |  |  | High | 2187.1 | 437420 | 2089.27 | 417854 | 504 |  | 5455 | 436370 | 4 | 0 | 0 (0) | 504 |
| 40+5 | CC1 | 40 | 15 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2107.2 | 421440 | 102 |  | 5319 | 425550 | 2 | 0 | 1 (2) | 104 |
|  |  |  |  |  |  | High | 2160 | 432000 | 2049.84 | 409968 | 504 |  | 5358 | 428670 | 6 | 1 | 2 (4) | 509 |
|  |  |  |  |  | Uplink | Low | 1730 | 346000 | 1710.56 | 342112 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1634.84 | 326968 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1760 | 352000 | 1739.48 | 347896 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=21.9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 5 | 15 | 25 | Downlink | Low | 2151.9 | 430380 | 2149.65 | 429930 | 0 | 15 | 5379 | 430350 | 8 | 1 | 0 (0) | 1 |
|  |  |  |  |  |  | Mid | 2166.9 | 433380 | 2146.29 | 429258 | 102 |  | 5418 | 433470 | 0 | 1 | 2 (4) | 107 |
|  |  |  |  |  |  | High | 2181.9 | 436380 | 2088.93 | 417786 | 504 |  | 5454 | 436350 | 8 | 1 | 0 (0) | 505 |
| 40+10 | CC1 | 40 | 15 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2107.2 | 421440 | 102 |  | 5319 | 425550 | 2 | 0 | 1 (2) | 104 |
|  |  |  |  |  |  | High | 2160 | 432000 | 2049.84 | 409968 | 504 |  | 5358 | 428670 | 6 | 1 | 2 (4) | 509 |
|  |  |  |  |  | Uplink | Low | 1730 | 346000 | 1710.56 | 342112 | 0 | - | - | - | - | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1634.84 | 326968 | 504 |  | - | - | - | - | - | - |
|  |  |  |  |  |  | High | 1760 | 352000 | 1739.48 | 347896 | 6 |  | - | - | - | - | - | - |
|  | Channel spacing CC1-CC2=24.3 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 15 | 52 | Downlink | Low | 2154.3 | 430860 | 2149.62 | 429924 | 0 | 15 | 5379 | 430350 | 10 | 1 | 0 (0) | 1 |
|  |  |  |  |  |  | Mid | 2169.3 | 433860 | 2146.26 | 429252 | 102 |  | 5418 | 433470 | 2 | 1 | 2 (4) | 107 |
|  |  |  |  |  |  | High | 2184.3 | 436860 | 2088.9 | 417780 | 504 |  | 5454 | 436350 | 10 | 1 | 0 (0) | 505 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.3.66.1-2: NR Intra-Band contiguous CA configuration CA_n66B (PCC=CC1 and SCC=CC2), SCS 30 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10+15 | CC1 | 10 | 30 | 24 | Downlink | Low | 2115 | 423000 | 2110.68 | 422136 | 0 | 15 |  | 5286 | 422910 |  | 18 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2103.96 | 420792 | 102 |  |  | 5361 | 428910 |  | 18 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2175 | 435000 | 1989.24 | 397848 | 504 |  |  | 5436 | 434910 |  | 18 |  | 0 | 0 (5) | 1018 |
|  |  |  |  |  | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=12.3 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 30 | 38 | Downlink | Low | 2127.3 | 425460 | 2120.46 | 424092 | 0 | 15 |  | 5310 | 424830 |  | 6 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2157.3 | 431460 | 2113.74 | 422748 | 102 |  |  | 5385 | 430830 |  | 6 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2187.3 | 437460 | 1999.02 | 399804 | 504 |  |  | 5460 | 436830 |  | 6 |  | 0 | 0 (5) | 1018 |
| 10+20 | CC1 | 10 | 30 | 24 | Downlink | Low | 2115 | 423000 | 2110.68 | 422136 | 0 | 15 |  | 5286 | 422910 |  | 18 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2103.96 | 420792 | 102 |  |  | 5361 | 428910 |  | 18 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2175 | 435000 | 1989.24 | 397848 | 504 |  |  | 5436 | 434910 |  | 18 |  | 0 | 0 (5) | 1018 |
|  |  |  |  |  | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=14.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 30 | 51 | Downlink | Low | 2129.4 | 425880 | 2120.22 | 424044 | 0 | 15 |  | 5310 | 424830 |  | 22 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2159.4 | 431880 | 2113.5 | 422700 | 102 |  |  | 5385 | 430830 |  | 22 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2189.4 | 437880 | 1998.78 | 399756 | 504 |  |  | 5460 | 436830 |  | 22 |  | 0 | 0 (5) | 1018 |
| 10+40 | CC1 | 10 | 30 | 24 | Downlink | Low | 2115 | 423000 | 2110.68 | 422136 | 0 | 15 |  | 5286 | 422910 |  | 18 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2135.4 | 427080 | 2094.36 | 418872 | 102 |  |  | 5337 | 426990 |  | 18 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2155.7 | 431140 | 1969.94 | 393988 | 504 |  |  | 5387 | 431050 |  | 18 |  | 0 | 0 (5) | 1018 |
|  |  |  |  |  | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1735.4 | 347080 | 1549.64 | 309928 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1755.7 | 351140 | 1749.22 | 349844 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=24.3 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 40 | 30 | 106 | Downlink | Low | 2139.3 | 427860 | 2120.22 | 424044 | 0 | 15 |  | 5310 | 424830 |  | 22 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2159.7 | 431940 | 2103.9 | 420780 | 102 |  |  | 5361 | 428910 |  | 22 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2180 | 436000 | 1979.48 | 395896 | 504 |  |  | 5411 | 432970 |  | 22 |  | 0 | 0 (5) | 1018 |
| 15+10 | CC1 | 15 | 30 | 38 | Downlink | Low | 2117.5 | 423500 | 2110.66 | 422132 | 0 | 15 |  | 5287 | 422930 |  | 2 |  | 0 | 1 (6) | 12 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2101.44 | 420288 | 102 |  |  | 5355 | 428430 |  | 2 |  | 0 | 1 (6) | 216 |
|  |  |  |  |  |  | High | 2172.5 | 434500 | 1984.22 | 396844 | 504 |  |  | 5423 | 433930 |  | 2 |  | 0 | 1 (6) | 1020 |
|  |  |  |  |  | Uplink | Low | 1717.5 | 343500 | 1710.66 | 342132 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1556.72 | 311344 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1772.5 | 354500 | 1763.5 | 352700 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=12.3 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 30 | 24 | Downlink | Low | 2129.8 | 425960 | 2125.48 | 425096 | 0 | 15 |  | 5326 | 426050 |  | 6 |  | 0 | 3 (8) | 16 |
|  |  |  |  |  |  | Mid | 2157.3 | 431460 | 2116.26 | 423252 | 102 |  |  | 5394 | 431550 |  | 6 |  | 0 | 3 (8) | 220 |
|  |  |  |  |  |  | High | 2184.8 | 436960 | 1999.04 | 399808 | 504 |  |  | 5462 | 437050 |  | 6 |  | 0 | 3 (8) | 1024 |
| 15+15 | CC1 | 15 | 30 | 38 | Downlink | Low | 2117.5 | 423500 | 2110.66 | 422132 | 0 | 15 |  | 5287 | 422930 |  | 2 |  | 0 | 1 (6) | 12 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2101.44 | 420288 | 102 |  |  | 5355 | 428430 |  | 2 |  | 0 | 1 (6) | 216 |
|  |  |  |  |  |  | High | 2172.5 | 434500 | 1984.22 | 396844 | 504 |  |  | 5423 | 433930 |  | 2 |  | 0 | 1 (6) | 1020 |
|  |  |  |  |  | Uplink | Low | 1717.5 | 343500 | 1710.66 | 342132 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1556.72 | 311344 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1772.5 | 354500 | 1763.5 | 352700 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=15 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 30 | 38 | Downlink | Low | 2132.5 | 426500 | 2125.66 | 425132 | 0 | 15 |  | 5326 | 426050 |  | 18 |  | 0 | 2 (7) | 14 |
|  |  |  |  |  |  | Mid | 2160 | 432000 | 2116.44 | 423288 | 102 |  |  | 5394 | 431550 |  | 18 |  | 0 | 2 (7) | 218 |
|  |  |  |  |  |  | High | 2187.5 | 437500 | 1999.22 | 399844 | 504 |  |  | 5462 | 437050 |  | 18 |  | 0 | 2 (7) | 1022 |
| 15+20 | CC1 | 15 | 30 | 38 | Downlink | Low | 2117.5 | 423500 | 2110.66 | 422132 | 0 | 15 |  | 5287 | 422930 |  | 2 |  | 0 | 1 (6) | 12 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2101.44 | 420288 | 102 |  |  | 5355 | 428430 |  | 2 |  | 0 | 1 (6) | 216 |
|  |  |  |  |  |  | High | 2172.5 | 434500 | 1984.22 | 396844 | 504 |  |  | 5423 | 433930 |  | 2 |  | 0 | 1 (6) | 1020 |
|  |  |  |  |  | Uplink | Low | 1717.5 | 343500 | 1710.66 | 342132 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1556.72 | 311344 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1772.5 | 354500 | 1763.5 | 352700 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=17.1 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 30 | 51 | Downlink | Low | 2134.6 | 426920 | 2125.42 | 425084 | 0 | 15 |  | 5323 | 425810 |  | 2 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2162.1 | 432420 | 2116.2 | 423240 | 102 |  |  | 5391 | 431310 |  | 2 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2189.6 | 437920 | 1998.98 | 399796 | 504 |  |  | 5459 | 436810 |  | 2 |  | 0 | 0 (5) | 1018 |
| 20+10 | CC1 | 20 | 30 | 51 | Downlink | Low | 2120 | 424000 | 2110.82 | 422164 | 0 | 15 |  | 5285 | 422890 |  | 2 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2099.1 | 419820 | 102 |  |  | 5349 | 427950 |  | 22 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2170 | 434000 | 1979.38 | 395876 | 504 |  |  | 5413 | 433010 |  | 18 |  | 0 | 1 (6) | 1020 |
|  |  |  |  |  | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1554.38 | 310876 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1770 | 354000 | 1758.66 | 351732 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=14.4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 30 | 24 | Downlink | Low | 2134.4 | 426880 | 2130.08 | 426016 | 0 | 15 |  | 5336 | 426970 |  | 6 |  | 0 | 3 (8) | 16 |
|  |  |  |  |  |  | Mid | 2159.4 | 431880 | 2118.36 | 423672 | 102 |  |  | 5397 | 431790 |  | 18 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2184.4 | 436880 | 1998.64 | 399728 | 504 |  |  | 5461 | 436850 |  | 14 |  | 0 | 1 (6) | 1020 |
| 20+15 | CC1 | 20 | 30 | 51 | Downlink | Low | 2120 | 424000 | 2110.82 | 422164 | 0 | 15 |  | 5285 | 422890 |  | 2 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2099.1 | 419820 | 102 |  |  | 5349 | 427950 |  | 22 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2170 | 434000 | 1979.38 | 395876 | 504 |  |  | 5413 | 433010 |  | 18 |  | 0 | 1 (6) | 1020 |
|  |  |  |  |  | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1554.38 | 310876 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1770 | 354000 | 1758.66 | 351732 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=17.1 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 15 | 30 | 38 | Downlink | Low | 2137.1 | 427420 | 2130.26 | 426052 | 0 | 15 |  | 5336 | 426970 |  | 18 |  | 0 | 2 (7) | 14 |
|  |  |  |  |  |  | Mid | 2162.1 | 432420 | 2118.54 | 423708 | 102 |  |  | 5397 | 431790 |  | 6 |  | 0 | 0 (5) | 214 |
|  |  |  |  |  |  | High | 2187.1 | 437420 | 1998.82 | 399764 | 504 |  |  | 5461 | 436850 |  | 2 |  | 0 | 1 (6) | 1020 |
| 40+10 | CC1 | 40 | 30 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 |  | 5286 | 422910 |  | 2 |  | 0 | 0 (5) | 10 |
|  |  |  |  |  |  | Mid | 2145 | 429000 | 2089.2 | 417840 | 102 |  |  | 5325 | 426030 |  | 18 |  | 0 | 1 (6) | 216 |
|  |  |  |  |  |  | High | 2160 | 432000 | 1959.48 | 391896 | 504 |  |  | 5361 | 428910 |  | 2 |  | 0 | 0 (5) | 1018 |
|  |  |  |  |  | Uplink | Low | 1730 | 346000 | 1710.92 | 342184 | 0 | - |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | Mid | 1745 | 349000 | 1544.48 | 308896 | 504 |  |  | - | - |  | - |  | - | - | - |
|  |  |  |  |  |  | High | 1760 | 352000 | 1738.76 | 347752 | 6 |  |  | - | - |  | - |  | - | - | - |
|  | Channel spacing CC1-CC2=24.3 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 10 | 30 | 24 | Downlink | Low | 2154.3 | 430860 | 2149.98 | 429996 | 0 | 15 |  | 5385 | 430830 |  | 14 |  | 0 | 1 (6) | 12 |
|  |  |  |  |  |  | Mid | 2169.3 | 433860 | 2128.26 | 425652 | 102 |  |  | 5424 | 433950 |  | 6 |  | 0 | 3 (8) | 220 |
|  |  |  |  |  |  | High | 2184.3 | 436860 | 1998.54 | 399708 | 504 |  |  | 5460 | 436830 |  | 14 |  | 0 | 1 (6) | 1020 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.3.67 – 4.3.1.1.3.76	FFS

4.3.1.1.3.77	NR Intra-band contiguous configurations CA_n77

4.3.1.1.3.77.1	CA_n77C

Editor's note: Test frequencies for CA_n78C with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.3.77.1-1: NR Intra-Band contiguous CA configuration CA_n77C (PCC=CC1 and SCC=CC2), SCS 30 kHz

| CBW combination (BCS) | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10+100 | CC1 | 10 | 30 | 24 | Downlink | Low | 3305,01 | 620334 | 3300,69 | 620046 | 0 | 30 |  | 7711 | 620352 |  | 18 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3658,95 | 643930 | 102 |  |  | 7985 | 646656 |  | 14 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 4095,39 | 673026 | 3909,63 | 660642 | 504 |  |  | 8260 | 673056 |  | 6 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=54,6 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3359,61 | 623974 | 3310,47 | 620698 | 0 | 30 |  | 7718 | 621024 |  | 14 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3754,59 | 650306 | 3668,73 | 644582 | 102 |  |  | 7992 | 647328 |  | 10 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 15+90 | CC1 | 15 | 30 | 38 | Downlink | Low | 3307,5 | 620500 | 3300,66 | 620044 | 0 | 30 |  | 7711 | 620352 |  | 20 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3661,44 | 644096 | 102 |  |  | 7987 | 646848 |  | 16 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4102,92 | 673528 | 3914,64 | 660976 | 504 |  |  | 8263 | 673344 |  | 8 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=52,08 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3359,58 | 623972 | 3315,48 | 621032 | 0 | 30 |  | 7721 | 621312 |  | 16 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3757,08 | 650472 | 3676,26 | 645084 | 102 |  |  | 7997 | 647808 |  | 12 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 15+100 | CC1 | 15 | 30 | 38 | Downlink | Low | 3307,5 | 620500 | 3300,66 | 620044 | 0 | 30 |  | 7711 | 620352 |  | 20 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3656,43 | 643762 | 102 |  |  | 7983 | 646464 |  | 14 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4092,87 | 672858 | 3904,59 | 660306 | 504 |  |  | 8256 | 672672 |  | 6 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=57,12 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3364,62 | 624308 | 3315,48 | 621032 | 0 | 30 |  | 7721 | 621312 |  | 16 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3757,11 | 650474 | 3671,25 | 644750 | 102 |  |  | 7994 | 647520 |  | 10 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 20+90 | CC1 | 20 | 30 | 51 | Downlink | Low | 3310,02 | 620668 | 3300,84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3659,1 | 643940 | 102 |  |  | 7985 | 646656 |  | 4 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 4100,1 | 673340 | 3909,48 | 660632 | 504 |  |  | 8260 | 673056 |  | 16 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=54,9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3364,92 | 624328 | 3320,82 | 621388 | 0 | 30 |  | 7725 | 621696 |  | 20 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3759,9 | 650660 | 3679,08 | 645272 | 102 |  |  | 7999 | 648000 |  | 16 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 20+100 | CC1 | 20 | 30 | 51 | Downlink | Low | 3310,02 | 620668 | 3300,84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3654,09 | 643606 | 102 |  |  | 7982 | 646368 |  | 2 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 4090,05 | 672670 | 3899,43 | 659962 | 504 |  |  | 8253 | 672384 |  | 14 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=59,94 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3369,96 | 624664 | 3320,82 | 621388 | 0 | 30 |  | 7725 | 621696 |  | 20 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3759,93 | 650662 | 3674,07 | 644938 | 102 |  |  | 7996 | 647712 |  | 14 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 25+80 | CC1 | 25 | 30 | 65 | Downlink | Low | 3312,51 | 620834 | 3300,81 | 620054 | 0 | 30 |  | 7711 | 620352 |  | 10 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3661,59 | 644106 | 102 |  |  | 7987 | 646848 |  | 6 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4107,66 | 673844 | 3914,52 | 660968 | 504 |  |  | 8263 | 673344 |  | 16 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=52,32 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3364,83 | 624322 | 3325,77 | 621718 | 0 | 30 |  | 7728 | 621984 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3762,33 | 650822 | 3686,55 | 645770 | 102 |  |  | 8004 | 648480 |  | 22 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 25+90 | CC1 | 25 | 30 | 65 | Downlink | Low | 3312,51 | 620834 | 3300,81 | 620054 | 0 | 30 |  | 7711 | 620352 |  | 10 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3656,58 | 643772 | 102 |  |  | 7983 | 646464 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4097,64 | 673176 | 3904,5 | 660300 | 504 |  |  | 8256 | 672672 |  | 12 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=57,36 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3369,87 | 624658 | 3325,77 | 621718 | 0 | 30 |  | 7728 | 621984 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3762,36 | 650824 | 3681,54 | 645436 | 102 |  |  | 8001 | 648192 |  | 20 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 25+100 | CC1 | 25 | 30 | 65 | Downlink | Low | 3312,51 | 620834 | 3300,81 | 620054 | 0 | 30 |  | 7711 | 620352 |  | 10 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3651,57 | 643438 | 102 |  |  | 7980 | 646176 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4087,59 | 672506 | 3894,45 | 659630 | 504 |  |  | 8249 | 672000 |  | 10 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=62,4 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3374,91 | 624994 | 3325,77 | 621718 | 0 | 30 |  | 7728 | 621984 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3762,39 | 650826 | 3676,53 | 645102 | 102 |  |  | 7997 | 647808 |  | 18 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 30+80 | CC1 | 30 | 30 | 78 | Downlink | Low | 3315 | 621000 | 3300,96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3659,25 | 643950 | 102 |  |  | 7985 | 646656 |  | 18 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4105,14 | 673676 | 3909,66 | 660644 | 504 |  |  | 8260 | 673056 |  | 4 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=54,84 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3369,84 | 624656 | 3330,78 | 622052 | 0 | 30 |  | 7732 | 622368 |  | 4 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3764,85 | 650990 | 3689,07 | 645938 | 102 |  |  | 8006 | 648672 |  | 22 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 30+90 | CC1 | 30 | 30 | 78 | Downlink | Low | 3315 | 621000 | 3300,96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3654,24 | 643616 | 102 |  |  | 7982 | 646368 |  | 16 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4095,12 | 673008 | 3899,64 | 659976 | 504 |  |  | 8253 | 672384 |  | 0 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=59,88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3374,88 | 624992 | 3330,78 | 622052 | 0 | 30 |  | 7732 | 622368 |  | 4 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3764,88 | 650992 | 3684,06 | 645604 | 102 |  |  | 8003 | 648384 |  | 20 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 30+100 | CC1 | 30 | 30 | 78 | Downlink | Low | 3315 | 621000 | 3300,96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3649,23 | 643282 | 102 |  |  | 7978 | 645984 |  | 14 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4085,07 | 672338 | 3889,59 | 659306 | 504 |  |  | 8246 | 671712 |  | 22 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=64,92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3379,92 | 625328 | 3330,78 | 622052 | 0 | 30 |  | 7732 | 622368 |  | 4 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3764,91 | 650994 | 3679,05 | 645270 | 102 |  |  | 7999 | 648000 |  | 18 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 40+70 | CC1 | 40 | 30 | 106 | Downlink | Low | 3320,01 | 621334 | 3300,93 | 620062 | 0 | 30 |  | 7711 | 620352 |  | 2 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3714,99 | 647666 | 3659,19 | 643946 | 102 |  |  | 7985 | 646656 |  | 22 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4110,15 | 674010 | 3909,63 | 660642 | 504 |  |  | 8260 | 673056 |  | 6 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=54,84 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3374,85 | 624990 | 3340,83 | 622722 | 0 | 30 |  | 7739 | 623040 |  | 6 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3769,83 | 651322 | 3699,09 | 646606 | 102 |  |  | 8013 | 649344 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4164,99 | 677666 | 3949,53 | 663302 | 504 |  |  | 8287 | 675648 |  | 10 |  | 0 | 0 (0) | 1008 |
| 40+80 | CC1 | 40 | 30 | 106 | Downlink | Low | 3320,01 | 621334 | 3300,93 | 620062 | 0 | 30 |  | 7711 | 620352 |  | 2 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3654,21 | 643614 | 102 |  |  | 7982 | 646368 |  | 18 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4100,16 | 673344 | 3899,64 | 659976 | 504 |  |  | 8253 | 672384 |  | 0 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=59,82 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3379,83 | 625322 | 3340,77 | 622718 | 0 | 30 |  | 7739 | 623040 |  | 10 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3769,83 | 651322 | 3694,05 | 646270 | 102 |  |  | 8009 | 648960 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 40+90 | CC1 | 40 | 30 | 106 | Downlink | Low | 3320,01 | 621334 | 3300,93 | 620062 | 0 | 30 |  | 7711 | 620352 |  | 2 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3649,2 | 643280 | 102 |  |  | 7978 | 645984 |  | 16 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4090,2 | 672680 | 3889,68 | 659312 | 504 |  |  | 8246 | 671712 |  | 16 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=64,8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3384,81 | 625654 | 3340,71 | 622714 | 0 | 30 |  | 7739 | 623040 |  | 14 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3769,8 | 651320 | 3688,98 | 645932 | 102 |  |  | 8006 | 648672 |  | 4 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 40+100 | CC1 | 40 | 30 | 106 | Downlink | Low | 3320,01 | 621334 | 3300,93 | 620062 | 0 | 30 |  | 7711 | 620352 |  | 2 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3644,19 | 642946 | 102 |  |  | 7975 | 645696 |  | 14 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4080,27 | 672018 | 3879,75 | 658650 | 504 |  |  | 8239 | 671040 |  | 6 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=69,72 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3389,73 | 625982 | 3340,59 | 622706 | 0 | 30 |  | 7739 | 623040 |  | 22 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3769,71 | 651314 | 3683,85 | 645590 | 102 |  |  | 8002 | 648288 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 50+60 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325,02 | 621668 | 3301,08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
| (0,1) |  |  |  |  | & | Mid | 3720 | 648000 | 3659,34 | 643956 | 102 |  |  | 7985 | 646656 |  | 12 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4115,04 | 674336 | 3909,66 | 660644 | 504 |  |  | 8260 | 673056 |  | 4 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=54,96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3379,98 | 625332 | 3350,82 | 623388 | 0 | 30 |  | 7746 | 623712 |  | 12 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3774,96 | 651664 | 3709,08 | 647272 | 102 |  |  | 8020 | 650016 |  | 8 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4170 | 678000 | 3959,4 | 663960 | 504 |  |  | 8294 | 676320 |  | 0 |  | 0 | 1 (1) | 1010 |
| 50+70 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325,02 | 621668 | 3301,08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3714,99 | 647666 | 3654,33 | 643622 | 102 |  |  | 7982 | 646368 |  | 10 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4105,11 | 673674 | 3899,73 | 659982 | 504 |  |  | 8253 | 672384 |  | 18 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=59,88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3384,9 | 625660 | 3350,88 | 623392 | 0 | 30 |  | 7746 | 623712 |  | 8 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3774,87 | 651658 | 3704,13 | 646942 | 102 |  |  | 8016 | 649632 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4164,99 | 677666 | 3949,53 | 663302 | 504 |  |  | 8287 | 675648 |  | 10 |  | 0 | 0 (0) | 1008 |
| 50+80 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325,02 | 621668 | 3301,08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
| (0,1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3649,35 | 643290 | 102 |  |  | 7978 | 645984 |  | 6 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4095,12 | 673008 | 3889,74 | 659316 | 504 |  |  | 8246 | 671712 |  | 12 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=64,86 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3389,88 | 625992 | 3350,82 | 623388 | 0 | 30 |  | 7746 | 623712 |  | 12 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3774,87 | 651658 | 3699,09 | 646606 | 102 |  |  | 8013 | 649344 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 50+90 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325,02 | 621668 | 3301,08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3644,34 | 642956 | 102 |  |  | 7975 | 645696 |  | 4 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4085,16 | 672344 | 3879,78 | 658652 | 504 |  |  | 8239 | 671040 |  | 4 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=69,84 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3394,86 | 626324 | 3350,76 | 623384 | 0 | 30 |  | 7746 | 623712 |  | 16 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3774,84 | 651656 | 3694,02 | 646268 | 102 |  |  | 8009 | 648960 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 50+100 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325,02 | 621668 | 3301,08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
| (0,1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3639,33 | 642622 | 102 |  |  | 7971 | 645312 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4075,23 | 671682 | 3869,85 | 657990 | 504 |  |  | 8232 | 670368 |  | 18 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=74,76 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3399,78 | 626652 | 3350,64 | 623376 | 0 | 30 |  | 7745 | 623616 |  | 0 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3774,75 | 651650 | 3688,89 | 645926 | 102 |  |  | 8006 | 648672 |  | 10 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 60+60 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300,84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3720 | 648000 | 3654,12 | 643608 | 102 |  |  | 7982 | 646368 |  | 0 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 4110 | 674000 | 3899,4 | 659960 | 504 |  |  | 8253 | 672384 |  | 16 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=60 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3390 | 626000 | 3360,84 | 624056 | 0 | 30 |  | 7753 | 624384 |  | 16 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3780 | 652000 | 3714,12 | 647608 | 102 |  |  | 8023 | 650304 |  | 8 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4170 | 678000 | 3959,4 | 663960 | 504 |  |  | 8294 | 676320 |  | 0 |  | 0 | 1 (1) | 1010 |
| 60+70 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300,84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3714,99 | 647666 | 3649,11 | 643274 | 102 |  |  | 7978 | 645984 |  | 22 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4100,07 | 673338 | 3889,47 | 659298 | 504 |  |  | 8246 | 671712 |  | 6 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=64,92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3394,92 | 626328 | 3360,9 | 624060 | 0 | 30 |  | 7753 | 624384 |  | 12 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3779,91 | 651994 | 3709,17 | 647278 | 102 |  |  | 8020 | 650016 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4164,99 | 677666 | 3949,53 | 663302 | 504 |  |  | 8287 | 675648 |  | 10 |  | 0 | 0 (0) | 1008 |
| 60+80 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300,84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3644,13 | 642942 | 102 |  |  | 7975 | 645696 |  | 18 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4090,08 | 672672 | 3879,48 | 658632 | 504 |  |  | 8239 | 671040 |  | 0 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=69,9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3399,9 | 626660 | 3360,84 | 624056 | 0 | 30 |  | 7753 | 624384 |  | 16 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3779,91 | 651994 | 3704,13 | 646942 | 102 |  |  | 8016 | 649632 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 60+90 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300,84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3639,12 | 642608 | 102 |  |  | 7971 | 645312 |  | 16 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4080,12 | 672008 | 3869,52 | 657968 | 504 |  |  | 8232 | 670368 |  | 16 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=74,88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3404,88 | 626992 | 3360,78 | 624052 | 0 | 30 |  | 7753 | 624384 |  | 20 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3779,88 | 651992 | 3699,06 | 646604 | 102 |  |  | 8013 | 649344 |  | 4 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 60+100 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300,84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3634,11 | 642274 | 102 |  |  | 7968 | 645024 |  | 14 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4070,19 | 671346 | 3859,59 | 657306 | 504 |  |  | 8225 | 669696 |  | 6 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=79,8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3409,8 | 627320 | 3360,66 | 624044 | 0 | 30 |  | 7752 | 624288 |  | 4 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3779,79 | 651986 | 3693,93 | 646262 | 102 |  |  | 8009 | 648960 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 70+60 | CC1 | 70 | 30 | 189 | Downlink | Low | 3335,01 | 622334 | 3300,99 | 620066 | 0 | 30 |  | 7711 | 620352 |  | 22 |  | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3720 | 648000 | 3649,26 | 643284 | 102 |  |  | 7978 | 645984 |  | 12 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4105,08 | 673672 | 3889,62 | 659308 | 504 |  |  | 8246 | 671712 |  | 20 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=64,92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3399,93 | 626662 | 3370,77 | 624718 | 0 | 30 |  | 7759 | 624960 |  | 2 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3784,92 | 652328 | 3719,04 | 647936 | 102 |  |  | 8027 | 650688 |  | 16 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4170 | 678000 | 3959,4 | 663960 | 504 |  |  | 8294 | 676320 |  | 0 |  | 0 | 1 (1) | 1010 |
| 70+70 | CC1 | 70 | 30 | 189 | Downlink | Low | 3335,01 | 622334 | 3300,99 | 620066 | 0 | 30 |  | 7711 | 620352 |  | 22 |  | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3714,99 | 647666 | 3644,25 | 642950 | 102 |  |  | 7975 | 645696 |  | 10 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4095,03 | 673002 | 3879,57 | 658638 | 504 |  |  | 8239 | 671040 |  | 18 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=69,96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3404,97 | 626998 | 3370,95 | 624730 | 0 | 30 |  | 7760 | 625056 |  | 14 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3784,95 | 652330 | 3714,21 | 647614 | 102 |  |  | 8023 | 650304 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4164,99 | 677666 | 3949,53 | 663302 | 504 |  |  | 8287 | 675648 |  | 10 |  | 0 | 0 (0) | 1008 |
| 70+80 | CC1 | 70 | 30 | 189 | Downlink | Low | 3335,01 | 622334 | 3300,99 | 620066 | 0 | 30 |  | 7711 | 620352 |  | 22 |  | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3639,27 | 642618 | 102 |  |  | 7971 | 645312 |  | 6 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4085,04 | 672336 | 3869,58 | 657972 | 504 |  |  | 8232 | 670368 |  | 12 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=74,94 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3409,95 | 627330 | 3370,89 | 624726 | 0 | 30 |  | 7760 | 625056 |  | 18 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3784,95 | 652330 | 3709,17 | 647278 | 102 |  |  | 8020 | 650016 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 70+90 | CC1 | 70 | 30 | 189 | Downlink | Low | 3335,01 | 622334 | 3300,99 | 620066 | 0 | 30 |  | 7711 | 620352 |  | 22 |  | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3634,26 | 642284 | 102 |  |  | 7968 | 645024 |  | 4 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4075,08 | 671672 | 3859,62 | 657308 | 504 |  |  | 8225 | 669696 |  | 4 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=79,92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3414,93 | 627662 | 3370,83 | 624722 | 0 | 30 |  | 7760 | 625056 |  | 22 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3784,92 | 652328 | 3704,1 | 646940 | 102 |  |  | 8016 | 649632 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 70+100 | CC1 | 70 | 30 | 189 | Downlink | Low | 3335,01 | 622334 | 3300,99 | 620066 | 0 | 30 |  | 7711 | 620352 |  | 22 |  | 0 | 1 (1) | 2 |
| (1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3629,25 | 641950 | 102 |  |  | 7964 | 644640 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4065,15 | 671010 | 3849,69 | 656646 | 504 |  |  | 8218 | 669024 |  | 18 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=84,84 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3419,85 | 627990 | 3370,71 | 624714 | 0 | 30 |  | 7759 | 624960 |  | 6 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3784,83 | 652322 | 3698,97 | 646598 | 102 |  |  | 8013 | 649344 |  | 10 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 80+60 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340,02 | 622668 | 3300,96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3720 | 648000 | 3644,22 | 642948 | 102 |  |  | 7975 | 645696 |  | 12 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4100,1 | 673340 | 3879,6 | 658640 | 504 |  |  | 8239 | 671040 |  | 16 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=69,9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3409,92 | 627328 | 3380,76 | 625384 | 0 | 30 |  | 7766 | 625632 |  | 8 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3789,9 | 652660 | 3724,02 | 648268 | 102 |  |  | 8030 | 650976 |  | 20 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4170 | 678000 | 3959,4 | 663960 | 504 |  |  | 8294 | 676320 |  | 0 |  | 0 | 1 (1) | 1010 |
| 80+70 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340,02 | 622668 | 3300,96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3714,99 | 647666 | 3639,21 | 642614 | 102 |  |  | 7971 | 645312 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4090,05 | 672670 | 3869,55 | 657970 | 504 |  |  | 8232 | 670368 |  | 14 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=74,94 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3414,96 | 627664 | 3380,94 | 625396 | 0 | 30 |  | 7767 | 625728 |  | 20 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3789,93 | 652662 | 3719,19 | 647946 | 102 |  |  | 8027 | 650688 |  | 6 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4164,99 | 677666 | 3949,53 | 663302 | 504 |  |  | 8287 | 675648 |  | 10 |  | 0 | 0 (0) | 1008 |
| 80+80 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340,02 | 622668 | 3300,96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3634,23 | 642282 | 102 |  |  | 7968 | 645024 |  | 6 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4080 | 672000 | 3859,5 | 657300 | 504 |  |  | 8225 | 669696 |  | 12 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=79,98 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3420 | 628000 | 3380,94 | 625396 | 0 | 30 |  | 7767 | 625728 |  | 20 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3789,99 | 652666 | 3714,21 | 647614 | 102 |  |  | 8023 | 650304 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 80+90 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340,02 | 622668 | 3300,96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3629,22 | 641948 | 102 |  |  | 7964 | 644640 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4070,04 | 671336 | 3849,54 | 656636 | 504 |  |  | 8218 | 669024 |  | 4 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=84,96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3424,98 | 628332 | 3380,88 | 625392 | 0 | 30 |  | 7766 | 625632 |  | 0 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3789,96 | 652664 | 3709,14 | 647276 | 102 |  |  | 8020 | 650016 |  | 4 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 80+100 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340,02 | 622668 | 3300,96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3624,21 | 641614 | 102 |  |  | 7961 | 644352 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4060,11 | 670674 | 3839,61 | 655974 | 504 |  |  | 8211 | 668352 |  | 18 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=89,88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3429,9 | 628660 | 3380,76 | 625384 | 0 | 30 |  | 7766 | 625632 |  | 8 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3789,87 | 652658 | 3704,01 | 646934 | 102 |  |  | 8016 | 649632 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 90+60 | CC1 | 90 | 30 | 245 | Downlink | Low | 3345 | 623000 | 3300,9 | 620060 | 0 | 30 |  | 7711 | 620352 |  | 4 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3720 | 648000 | 3639,18 | 642612 | 102 |  |  | 7971 | 645312 |  | 12 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4095,12 | 673008 | 3869,58 | 657972 | 504 |  |  | 8232 | 670368 |  | 12 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=74,88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3419,88 | 627992 | 3390,72 | 626048 | 0 | 30 |  | 7773 | 626304 |  | 16 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3794,88 | 652992 | 3729 | 648600 | 102 |  |  | 8034 | 651360 |  | 0 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 4170 | 678000 | 3959,4 | 663960 | 504 |  |  | 8294 | 676320 |  | 0 |  | 0 | 1 (1) | 1010 |
| 90+70 | CC1 | 90 | 30 | 245 | Downlink | Low | 3345 | 623000 | 3300,9 | 620060 | 0 | 30 |  | 7711 | 620352 |  | 4 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3714,99 | 647666 | 3634,17 | 642278 | 102 |  |  | 7968 | 645024 |  | 10 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4085,07 | 672338 | 3859,53 | 657302 | 504 |  |  | 8225 | 669696 |  | 10 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=79,92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3424,92 | 628328 | 3390,9 | 626060 | 0 | 30 |  | 7773 | 626304 |  | 4 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3794,91 | 652994 | 3724,17 | 648278 | 102 |  |  | 8030 | 650976 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4164,99 | 677666 | 3949,53 | 663302 | 504 |  |  | 8287 | 675648 |  | 10 |  | 0 | 0 (0) | 1008 |
| 90+80 | CC1 | 90 | 30 | 245 | Downlink | Low | 3345 | 623000 | 3300,9 | 620060 | 0 | 30 |  | 7711 | 620352 |  | 4 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3629,19 | 641946 | 102 |  |  | 7964 | 644640 |  | 6 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4075,02 | 671668 | 3849,48 | 656632 | 504 |  |  | 8218 | 669024 |  | 8 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=84,96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3429,96 | 628664 | 3390,9 | 626060 | 0 | 30 |  | 7773 | 626304 |  | 4 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3794,97 | 652998 | 3719,19 | 647946 | 102 |  |  | 8027 | 650688 |  | 6 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 90+90 | CC1 | 90 | 30 | 245 | Downlink | Low | 3345 | 623000 | 3300,9 | 620060 | 0 | 30 |  | 7711 | 620352 |  | 4 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3624,18 | 641612 | 102 |  |  | 7961 | 644352 |  | 4 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4065 | 671000 | 3839,46 | 655964 | 504 |  |  | 8211 | 668352 |  | 4 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=90 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3435 | 629000 | 3390,9 | 626060 | 0 | 30 |  | 7773 | 626304 |  | 4 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3795 | 653000 | 3714,18 | 647612 | 102 |  |  | 8023 | 650304 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 90+100 | CC1 | 90 | 30 | 245 | Downlink | Low | 3345 | 623000 | 3300,9 | 620060 | 0 | 30 |  | 7711 | 620352 |  | 4 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3619,17 | 641278 | 102 |  |  | 7957 | 643968 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4055,07 | 670338 | 3829,53 | 655302 | 504 |  |  | 8204 | 667680 |  | 18 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=94,92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3439,92 | 629328 | 3390,78 | 626052 | 0 | 30 |  | 7773 | 626304 |  | 12 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3794,91 | 652994 | 3709,05 | 647270 | 102 |  |  | 8020 | 650016 |  | 10 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| 100+60 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350,01 | 623334 | 3300,87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3720 | 648000 | 3634,14 | 642276 | 102 |  |  | 7968 | 645024 |  | 12 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4090,2 | 672680 | 3859,62 | 657308 | 504 |  |  | 8225 | 669696 |  | 4 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=79,8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3429,81 | 628654 | 3400,65 | 626710 | 0 | 30 |  | 7780 | 626976 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3799,8 | 653320 | 3733,92 | 648928 | 102 |  |  | 8037 | 651648 |  | 8 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 4170 | 678000 | 3959,4 | 663960 | 504 |  |  | 8294 | 676320 |  | 0 |  | 0 | 1 (1) | 1010 |
| 100+70 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350,01 | 623334 | 3300,87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3714,99 | 647666 | 3629,13 | 641942 | 102 |  |  | 7964 | 644640 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4080,15 | 672010 | 3849,57 | 656638 | 504 |  |  | 8218 | 669024 |  | 2 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=84,84 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 70 | 30 | 189 | Downlink | Low | 3434,85 | 628990 | 3400,83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3799,83 | 653322 | 3729,09 | 648606 | 102 |  |  | 8034 | 651360 |  | 18 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4164,99 | 677666 | 3949,53 | 663302 | 504 |  |  | 8287 | 675648 |  | 10 |  | 0 | 0 (0) | 1008 |
| 100+80 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350,01 | 623334 | 3300,87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3710,01 | 647334 | 3624,15 | 641610 | 102 |  |  | 7961 | 644352 |  | 6 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4070,1 | 671340 | 3839,52 | 655968 | 504 |  |  | 8211 | 668352 |  | 0 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=89,88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3439,89 | 629326 | 3400,83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3799,89 | 653326 | 3724,11 | 648274 | 102 |  |  | 8030 | 650976 |  | 14 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4159,98 | 677332 | 3939,48 | 662632 | 504 |  |  | 8280 | 674976 |  | 8 |  | 0 | 0 (0) | 1008 |
| 100+90 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350,01 | 623334 | 3300,87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
| (1) |  |  |  |  | & | Mid | 3705 | 647000 | 3619,14 | 641276 | 102 |  |  | 7957 | 643968 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4060,08 | 670672 | 3829,5 | 655300 | 504 |  |  | 8204 | 667680 |  | 20 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=94,92 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 90 | 30 | 245 | Downlink | Low | 3444,93 | 629662 | 3400,83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3799,92 | 653328 | 3719,1 | 647940 | 102 |  |  | 8027 | 650688 |  | 12 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4155 | 677000 | 3929,46 | 661964 | 504 |  |  | 8273 | 674304 |  | 4 |  | 0 | 0 (0) | 1008 |
| 100 +100 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350,01 | 623334 | 3300,87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
| (0,1) |  |  |  |  | & | Mid | 3699,99 | 646666 | 3614,13 | 640942 | 102 |  |  | 7954 | 643680 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 4050,03 | 670002 | 3819,45 | 654630 | 504 |  |  | 8197 | 667008 |  | 18 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=99,96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3449,97 | 629998 | 3400,83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3799,95 | 653330 | 3714,09 | 647606 | 102 |  |  | 8023 | 650304 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 4149,99 | 676666 | 3919,41 | 661294 | 504 |  |  | 8266 | 673632 |  | 2 |  | 0 | 0 (0) | 1008 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.3.78	NR Intra-band contiguous configurations CA_n78

4.3.1.1.3.78.1	CA_n78C

Editor's note: Test frequencies for CA_n78C with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.3.78.1-1: NR Intra-Band contiguous CA configuration CA_n78C (PCC=CC1 and SCC=CC2), SCS 30 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50+60 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325.02 | 621668 | 3301.08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3519.99 | 634666 | 3459.33 | 630622 | 102 |  |  | 7846 | 633312 |  | 2 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3715.02 | 647668 | 3509.64 | 633976 | 504 |  |  | 7982 | 646368 |  | 8 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=54.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3379.98 | 625332 | 3350.82 | 623388 | 0 | 30 |  | 7746 | 623712 |  | 12 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3574.95 | 638330 | 3509.07 | 633938 | 102 |  |  | 7881 | 636672 |  | 22 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3769.98 | 651332 | 3559.38 | 637292 | 504 |  |  | 8016 | 649632 |  | 4 |  | 0 | 0 (0) | 1008 |
| 50+80 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325.02 | 621668 | 3301.08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3510 | 634000 | 3449.34 | 629956 | 102 |  |  | 7840 | 632736 |  | 20 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3695.13 | 646342 | 3489.75 | 632650 | 504 |  |  | 7968 | 645024 |  | 14 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=64.86 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3389.88 | 625992 | 3350.82 | 623388 | 0 | 30 |  | 7746 | 623712 |  | 12 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3574.86 | 638324 | 3499.08 | 633272 | 102 |  |  | 7874 | 636000 |  | 16 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3759.99 | 650666 | 3539.49 | 635966 | 504 |  |  | 8003 | 648384 |  | 10 |  | 0 | 3 (3) | 1014 |
| 50+100 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325.02 | 621668 | 3301.08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3500.01 | 633334 | 3439.35 | 629290 | 102 |  |  | 7833 | 632064 |  | 14 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3675.24 | 645016 | 3469.86 | 631324 | 504 |  |  | 7954 | 643680 |  | 20 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=74.76 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3399.78 | 626652 | 3350.64 | 623376 | 0 | 30 |  | 7745 | 623616 |  | 0 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3574.77 | 638318 | 3488.91 | 632594 | 102 |  |  | 7867 | 635328 |  | 22 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 |  |  | 7989 | 647040 |  | 4 |  | 0 | 3 (3) | 1014 |
| 60+50 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300.84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3525 | 635000 | 3459.12 | 630608 | 102 |  |  | 7846 | 633312 |  | 16 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3720.03 | 648002 | 3509.43 | 633962 | 504 |  |  | 7982 | 646368 |  | 22 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=54.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 3384.96 | 625664 | 3361.02 | 624068 | 0 | 30 |  | 7753 | 624384 |  | 4 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3579.96 | 638664 | 3519.3 | 634620 | 102 |  |  | 7888 | 637344 |  | 12 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3774.99 | 651666 | 3569.61 | 637974 | 504 |  |  | 8024 | 650400 |  | 18 |  | 0 | 3 (3) | 1014 |
| 60+60 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300.84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3519.99 | 634666 | 3454.11 | 630274 | 102 |  |  | 7843 | 633024 |  | 14 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3709.98 | 647332 | 3499.38 | 633292 | 504 |  |  | 7975 | 645696 |  | 20 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=60 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3390 | 626000 | 3360.84 | 624056 | 0 | 30 |  | 7753 | 624384 |  | 16 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3579.99 | 638666 | 3514.11 | 634274 | 102 |  |  | 7885 | 637056 |  | 22 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3769.98 | 651332 | 3559.38 | 637292 | 504 |  |  | 8016 | 649632 |  | 4 |  | 0 | 0 (0) | 1008 |
| 60+80 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300.84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3510 | 634000 | 3444.12 | 629608 | 102 |  |  | 7836 | 632352 |  | 8 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3690.09 | 646006 | 3479.49 | 631966 | 504 |  |  | 7961 | 644352 |  | 2 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=69.9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3399.9 | 626660 | 3360.84 | 624056 | 0 | 30 |  | 7753 | 624384 |  | 16 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3579.9 | 638660 | 3504.12 | 633608 | 102 |  |  | 7878 | 636384 |  | 16 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3759.99 | 650666 | 3539.49 | 635966 | 504 |  |  | 8003 | 648384 |  | 10 |  | 0 | 3 (3) | 1014 |
| 60+100 | CC1 | 60 | 30 | 162 | Downlink | Low | 3330 | 622000 | 3300.84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3500.01 | 633334 | 3434.13 | 628942 | 102 |  |  | 7829 | 631680 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3670.2 | 644680 | 3459.6 | 630640 | 504 |  |  | 7947 | 643008 |  | 8 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=79.8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3409.8 | 627320 | 3360.66 | 624044 | 0 | 30 |  | 7752 | 624288 |  | 4 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3579.81 | 638654 | 3493.95 | 632930 | 102 |  |  | 7871 | 635712 |  | 22 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 |  |  | 7989 | 647040 |  | 4 |  | 0 | 3 (3) | 1014 |
| 80+50 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340.02 | 622668 | 3300.96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3525 | 635000 | 3449.22 | 629948 | 102 |  |  | 7839 | 632640 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3710.13 | 647342 | 3489.63 | 632642 | 504 |  |  | 7968 | 645024 |  | 22 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=64.86 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 3404.88 | 626992 | 3380.94 | 625396 | 0 | 30 |  | 7767 | 625728 |  | 20 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3589.86 | 639324 | 3529.2 | 635280 | 102 |  |  | 7895 | 638016 |  | 0 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3774.99 | 651666 | 3569.61 | 637974 | 504 |  |  | 8024 | 650400 |  | 18 |  | 0 | 3 (3) | 1014 |
| 80+60 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340.02 | 622668 | 3300.96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3519.99 | 634666 | 3444.21 | 629614 | 102 |  |  | 7836 | 632352 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3700.08 | 646672 | 3479.58 | 631972 | 504 |  |  | 7961 | 644352 |  | 20 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=69.9 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3409.92 | 627328 | 3380.76 | 625384 | 0 | 30 |  | 7766 | 625632 |  | 8 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3589.89 | 639326 | 3524.01 | 634934 | 102 |  |  | 7891 | 637632 |  | 10 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3769.98 | 651332 | 3559.38 | 637292 | 504 |  |  | 8016 | 649632 |  | 4 |  | 0 | 0 (0) | 1008 |
| 80+80 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340.02 | 622668 | 3300.96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3510 | 634000 | 3434.22 | 628948 | 102 |  |  | 7829 | 631680 |  | 20 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3680.01 | 645334 | 3459.51 | 630634 | 504 |  |  | 7947 | 643008 |  | 14 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=79.98 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3420 | 628000 | 3380.94 | 625396 | 0 | 30 |  | 7767 | 625728 |  | 20 |  | 0 | 3 (3) | 6 |
|  |  |  |  |  | & | Mid | 3589.98 | 639332 | 3514.2 | 634280 | 102 |  |  | 7885 | 637056 |  | 16 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3759.99 | 650666 | 3539.49 | 635966 | 504 |  |  | 8003 | 648384 |  | 10 |  | 0 | 3 (3) | 1014 |
| 80+100 | CC1 | 80 | 30 | 217 | Downlink | Low | 3340.02 | 622668 | 3300.96 | 620064 | 0 | 30 |  | 7711 | 620352 |  | 0 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3500.01 | 633334 | 3424.23 | 628282 | 102 |  |  | 7822 | 631008 |  | 14 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3660.12 | 644008 | 3439.62 | 629308 | 504 |  |  | 7933 | 641664 |  | 20 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=89.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3429.9 | 628660 | 3380.76 | 625384 | 0 | 30 |  | 7766 | 625632 |  | 8 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3589.89 | 639326 | 3504.03 | 633602 | 102 |  |  | 7878 | 636384 |  | 22 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 |  |  | 7989 | 647040 |  | 4 |  | 0 | 3 (3) | 1014 |
| 100+50 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3525 | 635000 | 3439.14 | 629276 | 102 |  |  | 7832 | 631968 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3700.23 | 646682 | 3469.65 | 631310 | 504 |  |  | 7954 | 643680 |  | 10 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=74.76 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 3424.77 | 628318 | 3400.83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3599.76 | 639984 | 3539.1 | 635940 | 102 |  |  | 7902 | 638688 |  | 12 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3774.99 | 651666 | 3569.61 | 637974 | 504 |  |  | 8024 | 650400 |  | 18 |  | 0 | 3 (3) | 1014 |
| 100+60 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3519.99 | 634666 | 3434.13 | 628942 | 102 |  |  | 7829 | 631680 |  | 2 |  | 0 | 2 (2) | 208 |
|  |  |  |  |  | Uplink | High | 3690.18 | 646012 | 3459.6 | 630640 | 504 |  |  | 7947 | 643008 |  | 8 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=79.8 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 60 | 30 | 162 | Downlink | Low | 3429.81 | 628654 | 3400.65 | 626710 | 0 | 30 |  | 7780 | 626976 |  | 2 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3599.79 | 639986 | 3533.91 | 635594 | 102 |  |  | 7898 | 638304 |  | 22 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3769.98 | 651332 | 3559.38 | 637292 | 504 |  |  | 8016 | 649632 |  | 4 |  | 0 | 0 (0) | 1008 |
| 100+80 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3510 | 634000 | 3424.14 | 628276 | 102 |  |  | 7822 | 631008 |  | 20 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3670.11 | 644674 | 3439.53 | 629302 | 504 |  |  | 7933 | 641664 |  | 2 |  | 0 | 1 (1) | 1010 |
|  | Channel spacing CC1-CC2=89.88 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 80 | 30 | 217 | Downlink | Low | 3439.89 | 629326 | 3400.83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3599.88 | 639992 | 3524.1 | 634940 | 102 |  |  | 7891 | 637632 |  | 4 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3759.99 | 650666 | 3539.49 | 635966 | 504 |  |  | 8003 | 648384 |  | 10 |  | 0 | 3 (3) | 1014 |
| 100 | CC1 | 100 | 30 | 273 | Downlink | Low | 3350.01 | 623334 | 3300.87 | 620058 | 0 | 30 |  | 7711 | 620352 |  | 6 |  | 0 | 2 (2) | 4 |
| +100 |  |  |  |  | & | Mid | 3500.01 | 633334 | 3414.15 | 627610 | 102 |  |  | 7815 | 630336 |  | 14 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3650.04 | 643336 | 3419.46 | 627964 | 504 |  |  | 7919 | 640320 |  | 20 |  | 0 | 0 (0) | 1008 |
|  | Channel spacing CC1-CC2=99.96 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 100 | 30 | 273 | Downlink | Low | 3449.97 | 629998 | 3400.83 | 626722 | 0 | 30 |  | 7780 | 626976 |  | 14 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3599.97 | 639998 | 3514.11 | 634274 | 102 |  |  | 7885 | 637056 |  | 22 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3750 | 650000 | 3519.42 | 634628 | 504 |  |  | 7989 | 647040 |  | 4 |  | 0 | 3 (3) | 1014 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.3.78.2	CA_n78B

Editor's note: Test frequencies for CA_n78B with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.3.78.2-1: NR Intra-Band contiguous CA configuration CA_n78B (PCC=CC1 and SCC=CC2), SCS 15 kHz and ΔFRaster 15 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20+50 | CC1 | 20 | 15 | 106 | Downlink | Low | 3310.005 | 620667 | 3300.465 | 620031 | 0 | 30 |  | 7711 | 620352 |  | 9 |  | 0 | 1 (6) | 6 |
|  |  |  |  |  | & | Mid | 3525 | 635000 | 3497.1 | 633140 | 102 |  |  | 7860 | 634656 |  | 4 |  | 2 | 0 (2) | 106 |
|  |  |  |  |  | Uplink | High | 3740.25 | 649350 | 3639.99 | 642666 | 504 |  |  | 8010 | 649056 |  | 6 |  | 2 | 1 (6) | 512 |
|  | Channel spacing CC1-CC2=34.74 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 15 | 270 | Downlink | Low | 3344.745 | 622983 | 3320.445 | 621363 | 0 | 30 |  | 7725 | 621696 |  | 9 |  | 1 | 1 (6) | 7 |
|  |  |  |  |  | & | Mid | 3559.74 | 637316 | 3517.08 | 634472 | 102 |  |  | 7874 | 636000 |  | 4 |  | 3 | 0 (2) | 107 |
|  |  |  |  |  | Uplink | High | 3774.99 | 651666 | 3659.97 | 643998 | 504 |  |  | 8024 | 650400 |  | 6 |  | 3 | 1 (6) | 513 |
| 50+20 | CC1 | 50 | 15 | 270 | Downlink | Low | 3325.005 | 621667 | 3300.705 | 620047 | 0 | 30 |  | 7711 | 620352 |  | 5 |  | 3 | 0 (2) | 5 |
|  |  |  |  |  | & | Mid | 3540 | 636000 | 3497.34 | 633156 | 102 |  |  | 7860 | 634656 |  | 0 |  | 1 | 0 (2) | 105 |
|  |  |  |  |  | Uplink | High | 3755.25 | 650350 | 3640.23 | 642682 | 504 |  |  | 8010 | 649056 |  | 2 |  | 1 | 1 (6) | 511 |
|  | Channel spacing CC1-CC2=34.74 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 15 | 106 | Downlink | Low | 3359.745 | 623983 | 3350.205 | 623347 | 0 | 30 |  | 7745 | 623616 |  | 5 |  | 0 | 0 (2) | 2 |
|  |  |  |  |  | & | Mid | 3574.74 | 638316 | 3546.84 | 636456 | 102 |  |  | 7895 | 638016 |  | 0 |  | 2 | 1 (6) | 110 |
|  |  |  |  |  | Uplink | High | 3789.99 | 652666 | 3689.73 | 645982 | 504 |  |  | 8044 | 652320 |  | 2 |  | 2 | 0 (2) | 508 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.3.78.2-2: NR Intra-Band contiguous CA configuration CA_n78B (PCC=CC1 and SCC=CC2), SCS 30 kHz and ΔFRaster 30 kHz

| CBW combination | CC Note 2 | CBW [MHz] | SCS [kHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] Note 2 | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] |  | SS block SCS [kHz] | GSCN |  | absoluteFrequencySSB [ARFCN] |  |  | Offset carrier CORESET#0 [RBs] Note 3 | CORESET#0 Index (Offset [RBs]) Note 4 | offsetToPointA (SIB1) [PRBs] Note 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20+50 | CC1 | 20 | 30 | 51 | Downlink | Low | 3310.02 | 620668 | 3300.84 | 620056 | 0 | 30 |  | 7711 | 620352 |  | 8 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3525 | 635000 | 3479.1 | 631940 | 102 |  |  | 7860 | 634656 |  | 4 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3740.25 | 649350 | 3549.63 | 636642 | 504 |  |  | 8010 | 649056 |  | 6 |  | 0 | 3 (3) | 1014 |
|  | Channel spacing CC1-CC2=34.74 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 50 | 30 | 133 | Downlink | Low | 3344.76 | 622984 | 3320.82 | 621388 | 0 | 30 |  | 7725 | 621696 |  | 20 |  | 0 | 2 (2) | 4 |
|  |  |  |  |  | & | Mid | 3559.74 | 637316 | 3499.08 | 633272 | 102 |  |  | 7874 | 636000 |  | 16 |  | 0 | 1 (1) | 206 |
|  |  |  |  |  | Uplink | High | 3774.99 | 651666 | 3569.61 | 637974 | 504 |  |  | 8024 | 650400 |  | 18 |  | 0 | 3 (3) | 1014 |
| 50+20 | CC1 | 50 | 30 | 133 | Downlink | Low | 3325.02 | 621668 | 3301.08 | 620072 | 0 | 30 |  | 7711 | 620352 |  | 16 |  | 0 | 1 (1) | 2 |
|  |  |  |  |  | & | Mid | 3540 | 636000 | 3479.34 | 631956 | 102 |  |  | 7860 | 634656 |  | 12 |  | 0 | 0 (0) | 204 |
|  |  |  |  |  | Uplink | High | 3755.25 | 650350 | 3549.87 | 636658 | 504 |  |  | 8010 | 649056 |  | 14 |  | 0 | 2 (2) | 1012 |
|  | Channel spacing CC1-CC2=34.74 MHz (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CC2 | 20 | 30 | 51 | Downlink | Low | 3359.76 | 623984 | 3350.58 | 623372 | 0 | 30 |  | 7745 | 623616 |  | 4 |  | 0 | 0 (0) | 0 |
|  |  |  |  |  | & | Mid | 3574.74 | 638316 | 3528.84 | 635256 | 102 |  |  | 7895 | 638016 |  | 0 |  | 0 | 3 (3) | 210 |
|  |  |  |  |  | Uplink | High | 3789.99 | 652666 | 3599.37 | 639958 | 504 |  |  | 8044 | 652320 |  | 2 |  | 0 | 1 (1) | 1010 |
| Note 1: Corresponds to nominal channel spacing in accordance with TS 38.101-1 [7], clause 5.4A.1 for the CC1 and CC2 channel bandwidth combination. Note 2: CCs are specified in increasing frequency order. CC1 is used as PCell if nothing else is specified in the test case. Note 3: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 4: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-4 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.1.4	Void

4.3.1.1.5	NR intra-band non-contiguous CA configurations in FR1

4.3.1.1.5.1 – 4.3.1.1.5.47	FFS

4.3.1.1.5.48	CA_n48(2A)

Editor’s note:	Test frequencies for CA_n48(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.5.48-1: NR Intra-Band non-contiguous CA configuration CA_n48(2A) without UL CA, SCS=15 kHz, Max Wgap

| SB CBW combination | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range (Note 1) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n48(2A); n48A (10MHz) + n48A (10-50MHz) |  |  |  |  |  |  |
| 10+10 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=10 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=10 MHz |
| 10+15 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=10 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=15 MHz |
| 10+20 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=10 MHz |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=20 MHz |
| 10+40 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=10 MHz |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=40 MHz |
| 10+50 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=10 MHz |
|  | SB2 | 50 | 270 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=50 MHz |
| CA_n48(2A); n48A (15MHz) + n48A (10-50MHz) |  |  |  |  |  |  |
| 15+10 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=15 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=10 MHz |
| 15+15 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=15 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=15 MHz |
| 15+20 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=15 MHz |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=20 MHz |
| 15+40 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=15 MHz |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=40 MHz |
| 15+50 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=15 MHz |
|  | SB2 | 50 | 270 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=50 MHz |
| CA_n48(2A); n48A (20MHz) + n48A (10-50MHz) |  |  |  |  |  |  |
| 20+10 | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=20 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=10 MHz |
| 20+15 | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=20 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=15 MHz |
| 20+20 | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=20 MHz |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=20 MHz |
| 20+40 | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=20 MHz |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=40 MHz |
| 20+50 | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=20 MHz |
|  | SB2 | 50 | 270 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=50 MHz |
| CA_n48(2A); n48A (40MHz) + n48A (10-50MHz) |  |  |  |  |  |  |
| 40+10 | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=40 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=10 MHz |
| 40+15 | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=40 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=15 MHz |
| 40+20 | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=40 MHz |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=20 MHz |
| 40+40 | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=40 MHz |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=40 MHz |
| 40+50 | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=40 MHz |
|  | SB2 | 50 | 270 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=50 MHz |
| CA_n48(2A); n48A (50MHz) + n48A (10-50MHz) |  |  |  |  |  |  |
| 50+10 | SB1 | 50 | 270 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=50 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=10 MHz |
| 50+15 | SB1 | 50 | 270 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=50 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=15 MHz |
| 50+20 | SB1 | 50 | 270 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=50 MHz |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=20 MHz |
| 50+40 | SB1 | 50 | 270 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=50 MHz |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=40 MHz |
| 50+50 | SB1 | 50 | 270 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-1: Low range for CBW=50 MHz |
|  | SB2 | 50 | 270 | Downlink & Uplink |  | Table 4.3.1.1.1.48-1: High range for CBW=50 MHz |
| Note 1: CA_n48(2A) is specified in [7] 38.101-1 without uplink CA. PCC is configured on SB1 unless otherwise stated. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |

Table 4.3.1.1.5.48-2: NR Intra-Band non-contiguous CA configuration CA_n48(2A) without UL CA, SCS=30 kHz, Max Wgap

| CBW combination | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range (Note 1) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n48(2A); n48A (10MHz) + n48A (10-100MHz) |  |  |  |  |  |  |
| 10+10 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 10+15 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 10+20 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 10+40 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 10+50 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 10+60 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| 10+80 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 80 | 217 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=80 MHz |
| 10+90 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 90 | 245 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=90 MHz |
| 10+100 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=10 MHz |
|  | SB2 | 100 | 273 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=100 MHz |
| CA_n48(2A); n48A (15MHz) + n48A (10-100MHz) |  |  |  |  |  |  |
| 15+10 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 15+15 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 15+20 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 15+40 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 15+50 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 15+60 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| 15+80 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 80 | 217 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=80 MHz |
| 15+90 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 90 | 245 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=90 MHz |
| 15+100 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=15 MHz |
|  | SB2 | 100 | 273 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=100 MHz |
| CA_n48(2A); n48A (20MHz) + n48A (10-100MHz) |  |  |  |  |  |  |
| 20+10 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 20+15 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 20+20 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 20+40 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 20+50 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 20+60 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| 20+80 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 80 | 217 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=80 MHz |
| 20+90 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 90 | 245 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=90 MHz |
| 20+100 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=20 MHz |
|  | SB2 | 100 | 273 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=100 MHz |
| CA_n48(2A); n48A (40MHz) + n48A (10-100MHz) |  |  |  |  |  |  |
| 40+10 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 40+15 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 40+20 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 40+40 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 40+50 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 40+60 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| 40+80 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 80 | 217 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=80 MHz |
| 40+90 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 90 | 245 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=90 MHz |
| 40+100 | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=40 MHz |
|  | SB2 | 100 | 273 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=100 MHz |
| CA_n48(2A); n48A (50MHz) + n48A (10-90MHz) |  |  |  |  |  |  |
| 50+10 | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 50+15 | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 50+20 | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 50+40 | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 50+50 | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 50+60 | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| 50+80 | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  | SB2 | 80 | 217 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=80 MHz |
| 50+90 | SB1 | 50 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=50 MHz |
|  | SB2 | 90 | 245 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=90 MHz |
| CA_n48(2A); n48A (60MHz) + n48A (10-80MHz) |  |  |  |  |  |  |
| 60+10 | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 60+15 | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 60+20 | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 60+40 | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 60+50 | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 60+60 | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| 60+80 | SB1 | 60 | 162 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=60 MHz |
|  | SB2 | 80 | 217 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=80 MHz |
| CA_n48(2A); n48A (80MHz) + n48A (10-60MHz) |  |  |  |  |  |  |
| 80+10 | SB1 | 80 | 217 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=80 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 80+15 | SB1 | 80 | 217 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=80 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 80+20 | SB1 | 80 | 217 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=80 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 80+40 | SB1 | 80 | 217 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=80 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 80+50 | SB1 | 80 | 217 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=80 MHz |
|  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| 80+60 | SB1 | 80 | 217 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=80 MHz |
|  | SB2 | 60 | 162 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=60 MHz |
| CA_n48(2A); n48A (90MHz) + n48A (10-50MHz) |  |  |  |  |  |  |
| 90+10 | SB1 | 90 | 245 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=90 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 90+15 | SB1 | 90 | 245 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=90 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 90+20 | SB1 | 90 | 245 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=90 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 90+40 | SB1 | 90 | 245 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=90 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| 90+50 | SB1 | 90 | 245 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=90 MHz |
|  | SB2 | 50 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=50 MHz |
| CA_n48(2A); n48A (100MHz) + n48A (10-40MHz) |  |  |  |  |  |  |
| 100+10 | SB1 | 100 | 273 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=100 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=10 MHz |
| 100+15 | SB1 | 100 | 273 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=100 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=15 MHz |
| 100+20 | SB1 | 100 | 273 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=100 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=20 MHz |
| 100+40 | SB1 | 100 | 273 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.48-2: Low range for CBW=100 MHz |
|  | SB2 | 40 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.48-2: High range for CBW=40 MHz |
| Note 1: CA_n48(2A) is specified in [7] 38.101-1 without uplink CA. PCC is configured on SB1 unless otherwise stated. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |

4.3.1.1.5.49 – 4.3.1.1.5.65	FFS

4.3.1.1.5.66	CA_n66(2A)

Editor’s note:	Test frequencies for CA_n66(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.5.66-1: NR Intra-Band non-contiguous CA configuration CA_n66(2A) without UL CA, SCS=15 kHz, Max Wgap

| CBW combination (BCS) | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range (Note 1) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n66(2A); n66A (5MHz) + n66A(5-40MHz) |  |  |  |  |  |  |
| 5+5 (0,1,2) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/5 |
| 5+10 (0,1,2) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 10/10 |
| 5+15 (0,1,2) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 15/15 |
| 5+20 (0,1,2) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 5+25 (1) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 5+30 (1) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 5+40 (0,1,2) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); n66A (10MHz) + n66A(5-40MHz) |  |  |  |  |  |  |
| 10+5 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/5 |
| 10+10 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 10/10 |
| 10+15 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 15/15 |
| 10+20 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 10+25 (1) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 10+30 (1) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 10+40 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); n66A (15MHz) + n66A(5-40MHz) |  |  |  |  |  |  |
| 15+5 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/5 |
| 15+10 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 10/10 |
| 15+15 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 15/15 |
| 15+20 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 15+25 (1) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 15+30 (1) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 15+40 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); n66A (20MHz) + n66A(5-40MHz) |  |  |  |  |  |  |
| 20+5 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 5/5 |
| 20+10 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 10/10 |
| 20+15 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 15/15 |
| 20+20 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 20+25 (1) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 20+30 (1) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 20+40 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); n66A (25MHz) + n66A(5-40MHz) |  |  |  |  |  |  |
| 25+5 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 5/5 |
| 25+10 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 10/10 |
| 25+15 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 15/15 |
| 25+20 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 25+25 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 25+30 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 25+40 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); n66A (30MHz) + n66A(5-40MHz) |  |  |  |  |  |  |
| 30+5 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 5/5 |
| 30+10 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 10/10 |
| 30+15 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 15/15 |
| 30+20 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 30+25 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 30+30 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 30+40 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); n66A (40MHz) + n66A(5-40MHz) |  |  |  |  |  |  |
| 40+5 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 5/5 |
| 40+10 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 10/10 |
| 40+15 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 15/15 |
| 40+20 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | able 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 40+25 (1) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 40+30 (1) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 40+40 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| Note 1: CA_n66(2A) is specified in [7] 38.101-1 without uplink CA. PCC is configured on SB1 unless otherwise stated. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |

Table 4.3.1.1.5.66-2: NR Intra-Band non-contiguous CA configuration CA_n66(2A) without UL CA, SCS=30 kHz, Max Wgap

| CBW combination (BCS) | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n66(2A); n66A (10MHz) + n66A(10-40MHz) |  |  |  |  |  |  |
| 10+10 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 10/10 |
| 10+15 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 15/15 |
| 10+20 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
| 10+25 (1) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 10+30 (1) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 10+40 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); n66A (15MHz) +n66A(10-40MHz) |  |  |  |  |  |  |
| 15+10 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 10/10 |
| 15+15 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 15/15 |
| 15+20 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 20/20 |
| 15+25 (1) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 15+30 (1) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 15+40 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); n66A (20MHz) + n66A(10-40MHz) |  |  |  |  |  |  |
| 20+10 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 10/10 |
| 20+15 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 15/15 |
| 20+20 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
| 20+25 (1) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 20+30 (1) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 20+40 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); n66A (25MHz) + n66A(10-40MHz) |  |  |  |  |  |  |
| 25+10 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 10/10 |
| 25+15 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 15/15 |
| 25+20 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 25+25 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 25+30 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 25+40 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); n66A (30MHz) + n66A(10-40MHz) |  |  |  |  |  |  |
| 30+10 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 10/10 |
| 30+15 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 15/15 |
| 30+20 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 30+25 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 30+30 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 30+40 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); n66A (40MHz) + n66A(10-40MHz) |  |  |  |  |  |  |
| 40+10 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 10/10 |
| 40+15 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/40 or 20/40 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 15/15 |
| 40+20 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
| 40+25 (1) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 40+30 (1) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 40+40 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| Note 1: CA_n66(2A) is specified in [7] 38.101-1 without uplink CA. PCC is configured on SB1 unless otherwise stated. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |

4.3.1.1.5.67 – 4.3.1.1.5.70	FFS

4.3.1.1.5.71	CA_n71(2A)

Editor’s note:	Test frequencies for CA_n71(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz; and SCS CC1=30kHz and SCS CC2=15 kHz is FFS.

Table 4.3.1.1.5.71-1: NR Intra-Band non-contiguous CA configuration CA_n71(2A) without UL CA, SCS=15 kHz, Max Wgap

| CBW combination | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A); n71A (5MHz) + n71A (5-20MHz) |  |  |  |  |  |  |
| 5+5 | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=5 MHz |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=5 MHz |
| 5+10 | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=5 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=10 MHz |
| 5+15 | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=5 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=15 MHz |
| 5+20 | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=5 MHz |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=20 MHz |
| CA_n71(2A); n71A (10MHz) + n71A (5-20MHz) |  |  |  |  |  |  |
| 10+5 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=10 MHz |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=5 MHz |
| 10+10 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=10 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=10 MHz |
| 10+15 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=10 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=15 MHz |
| 10+20 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=10 MHz |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=20 MHz |
| CA_n71(2A); n71A (15MHz) + n71A (5-15MHz) |  |  |  |  |  |  |
| 15+5 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=15 MHz |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=5 MHz |
| 15+10 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=15 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=10 MHz |
| 15+15 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=15 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=15 MHz |
| CA_n71(2A); n71A (20MHz) + n71A (5-10MHz) |  |  |  |  |  |  |
| 20+5 | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=20 MHz |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=5 MHz |
| 20+10 | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=20 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=10 MHz |
| Note 1: CA_n71(2A) is specified in [7] 38.101-1 without uplink CA. PCC is configured on SB1 unless otherwise stated. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |

Table 4.3.1.1.5.71-2: NR Intra-Band non-contiguous CA configuration CA_n71(2A) without UL CA, SCS=30 kHz, Max Wgap

| SB CBW combination | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A); n71A (10MHz) + n71A (10-20MHz) |  |  |  |  |  |  |
| 10+10 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=10 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=10 MHz |
| 10+15 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=10 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=15 MHz |
| 10+20 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=10 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=20 MHz |
| CA_n71(2A); n71A (15MHz) + n71A (10-15MHz) |  |  |  |  |  |  |
| 15+10 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=15 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=10 MHz |
| 15+15 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=15 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=15 MHz |
| CA_n71(2A); n71A (20MHz) + n71A (10MHz) |  |  |  |  |  |  |
| 20+10 | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=20 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=10 MHz |
| Note 1: CA_n71(2A) is specified in [7] 38.101-1 without uplink CA. PCC is configured on SB1 unless otherwise stated. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |

4.3.1.1.5.72 – 4.3.1.1.5.76	FFS

4.3.1.1.5.77	CA_n77(2A)

Editor’s note:	Test frequencies for CA_n77(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.5.77-1: NR Intra-Band non-contiguous CA configuration CA_n77(2A) without UL CA, SCS=15 kHz, Max Wgap

| CBW combination | SB | Bandwidth [MHz] | Range (Note 1) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n77(2A); n77A(20-40MHz) + n77A(20-40MHz) |  |  |  |  |  |
| CBW1+CBW2 | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.77-1: Low range for CBW = 20 OR 40 MHz |
|  |  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.77-1: High range for CBW = 20 OR 40 MHz |
| Note 1: CA_n77(2A) BCS0 is specified in [7] 38.101-1 without uplink CA. PCC is configured on SB1 unless otherwise stated. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |

Table 4.3.1.1.5.77-2: NR Intra-Band non-contiguous CA configuration CA_n77(2A) without UL CA, SCS=30 kHz, Max Wgap

| CBW combination | SB | Bandwidth [MHz] | Range (Note 1) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n77(2A); n77A(20-100MHz) + n77A(20-100MHz) |  |  |  |  |  |
| CBW1+CBW2 | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.77-2: Low range for CBW = 20, 40, 80 OR 100 MHz |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.77-2: High range for CBW = 20, 40, 80 OR 100 MHz |
| Note 1: CA_n77(2A) BCS0 is specified in [7] 38.101-1 without uplink CA. PCC is configured on SB1 unless otherwise stated. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |

4.3.1.1.5.78	CA_n78(2A)

Editor’s note:	Test frequencies for CA_n78(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.5.78-1: NR Intra-Band non-contiguous CA configuration CA_n78(2A) without UL CA, SCS=15 kHz, Max Wgap

| CBW combination | SB | Bandwidth [MHz] | Range | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n78(2A); n78A(10-50MHz) + n78A(10-50MHz) |  |  |  |  |  |
| CBW1+CBW2 | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.78-1: Low range for CBW = 10, 20, 25, 30, 40 OR 50 MHz |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.78-1: High range for CBW = 10, 20, 25, 30, 40 OR 50 MHz |
| Note 1: CA_n78(2A) is specified in [7] 38.101-1 with and without uplink CA. PCC is configured on SB1 unless otherwise stated. This table is applicable for the case of without UL CA but test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |

Table 4.3.1.1.5.78-1A: NR Intra-Band non-contiguous CA configuration CA_n78(2A) with UL CA, SCS=15 kHz, Max Wgap

FFS

Table 4.3.1.1.5.78-2: NR Intra-Band non-contiguous CA configuration CA_n78(2A) without UL CA, SCS=30 kHz, Max Wgap

| CBW combination | SB | Bandwidth [MHz] | Range | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- |
| CA_n78(2A); n78A(10-100MHz) + n78A(10-100MHz) |  |  |  |  |  |
| CBW1+CBW2 | SB1 | CBW1 | Downlink & Uplink | Max Wgap | For CBW1, use Table 4.3.1.1.1.78-2: Low range for CBW = 10, 20, 25, 30, 40, 50, 60, 70, 80, 90 OR 100 MHz |
|  | SB2 | CBW2 | Downlink & Uplink |  | For CBW2, use Table 4.3.1.1.1.78-2: High range for CBW = 10, 20, 25, 30, 40, 50, 60, 70, 80, 90 OR 100 MHz |
| Note 1: CA_n78(2A) is specified in [7] 38.101-1 with and without uplink CA. PCC is configured on SB1 unless otherwise stated. This table is applicable for the case of without UL CA but test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |

Table 4.3.1.1.5.78-2A: NR Intra-Band non-contiguous CA configuration CA_n78(2A) with UL CA, SCS=30 kHz, Max Wgap

FFS

4.3.1.1.6	NR Operating SUL band combinations in FR1

For SUL band combinations specified in TS 38.101-1 [7] Table 5.5C-1, the test frequencies and signalling parameters in clause 4.3.1.1.1 apply for each operating bands.

For SUL band combinations with intra-band non-contiguous CA specified in TS 38.101-1 [7] Table 5.5C-2, the test frequencies and signalling parameters in clause 4.3.1.1.1 apply for SUL band. The test frequencies and signalling parameters in clause 4.3.1.1.5 apply for each DL CA component carrier.

For SUL band combinations with intra-band contiguous CA specified in TS 38.101-1 [7] Table 5.5C-3, the test frequencies and signalling parameters in clause 4.3.1.1.1 apply for SUL band. The test frequencies and signalling parameters in clause 4.3.1.1.3 apply for each DL CA component carrier.

For SUL band combinations with inter-band CA specified in TS 38.101-1 [7] Table 5.5C-4, the test frequencies and signalling parameters in clause 4.3.1.1.1 apply for each operating bands.

4.3.1.1.7	NR inter-band NR-DC configurations in FR1

4.3.1.1.7.1	NR inter-band NR-DC configurations in FR1 (two bands)

Table 4.3.1.1.7.1-1: Inter-band NR-DC configurations (FR1, two bands)

| NR-DC configuration | Uplink NR-DC configuration | NR-DC downlink configuration band 1 | NR-DC downlink configuration band 2 | NR-DC uplink configuration band 1 | NR-DC uplink configuration band 2 | Applicable for protocol testing (Note 1) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_n48A-n70A | DC_n48A-n70A | n48A | n70A | n48A | n70A | Yes |
| Note 1: Protocol testing is limited to NR-DC configurations with 2CC. |  |  |  |  |  |  |

4.3.1.2	Test frequencies for NR operating bands in FR2

4.3.1.2.1	NR operating bands in FR2

4.3.1.2.1.1	Reference test frequencies for NR operating band n257

Table 4.3.1.2.1.1-1: Test frequencies for NR operating band n257, SCS 60 kHz and ΔFRaster 60 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | 66 | Downlink | Low | 26525.04 | 2054583 | 26501.28 | 2054187 | 0 | 120 | 22388 | 2054683 | 4 | 13 | 1 (8) | 21 |
|  |  | & | Mid | 27999.96 | 2079165 | 27902.76 | 2077545 | 102 |  | 22473 | 2079163 | 10 | 4 | 1 (8) | 114 |
|  |  | Uplink | High | 29475 | 2103749 | 29088.36 | 2097305 | 504 |  | 22558 | 2103643 | 2 | 4 | 0 (0) | 508 |
| 100 | 132 | Downlink | Low | 26550 | 2054999 | 26502.48 | 2054207 | 0 | 120 | 22388 | 2054683 | 8 | 11 | 1 (8) | 19 |
|  |  | & | Mid | 27999.96 | 2079165 | 27879 | 2077149 | 102 |  | 22472 | 2078875 | 10 | 13 | 1 (8) | 123 |
|  |  | Uplink | High | 29449.92 | 2103331 | 29039.52 | 2096491 | 504 |  | 22555 | 2102779 | 0 | 0 | 0 (0) | 504 |
| 200 | 264 | Downlink | Low | 26600.04 | 2055833 | 26505 | 2054249 | 0 | 120 | 22388 | 2054683 | 2 | 8 | 1 (8) | 16 |
|  |  | & | Mid | 27999.96 | 2079165 | 27831.48 | 2076357 | 102 |  | 22469 | 2078011 | 10 | 7 | 1 (8) | 117 |
|  |  | Uplink | High | 29400 | 2102499 | 28942.08 | 2094867 | 504 |  | 22550 | 2101339 | 4 | 7 | 1 (8) | 519 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-7 in TS 38.213 [22]. The value of CORESET#0 Index is signalled controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.1.1-2: Test frequencies for NR operating band n257, SCS 120kHz and ΔFRaster 120 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | 32 | Downlink | Low | 26525.04 | 2054583 | 26502 | 2054199 | 0 | 120 | 22388 | 2054683 | 2 | 6 | 1 (4) | 20 |
|  |  | & | Mid | 27999.96 | 2079165 | 27830.04 | 2076333 | 102 |  | 22473 | 2079163 | 11 | 1 | 1 (4) | 214 |
|  |  | Uplink | High | 29475 | 2103749 | 28726.2 | 2091269 | 504 |  | 22558 | 2103643 | 7 | 1 | 0 (0) | 1010 |
| 100 | 66 | Downlink | Low | 26550 | 2054999 | 26502.48 | 2054207 | 0 | 120 | 22388 | 2054683 | 10 | 5 | 1 (4) | 18 |
|  |  | & | Mid | 27999.96 | 2079165 | 27805.56 | 2075925 | 102 |  | 22472 | 2078875 | 11 | 6 | 1 (4) | 224 |
|  |  | Uplink | High | 29449.92 | 2103331 | 28676.64 | 2090443 | 504 |  | 22555 | 2102779 | 0 | 0 | 0 (0) | 1008 |
| 200 | 132 | Downlink | Low | 26600.04 | 2055833 | 26505 | 2054249 | 0 | 120 | 22388 | 2054683 | 1 | 4 | 1 (4) | 16 |
|  |  | & | Mid | 27999.96 | 2079165 | 27758.04 | 2075133 | 102 |  | 22469 | 2078011 | 11 | 3 | 1 (4) | 218 |
|  |  | Uplink | High | 29400 | 2102499 | 28579.2 | 2088819 | 504 |  | 22550 | 2101339 | 8 | 3 | 1 (4) | 1022 |
| 400 | 264 | Downlink | Low | 26700 | 2057499 | 26509.92 | 2054331 | 0 | 120 | 22388 | 2054683 | 8 | 0 | 1 (4) | 8 |
|  |  | & | Mid | 27999.96 | 2079165 | 27663 | 2073549 | 102 |  | 22463 | 2076283 | 11 | 1 | 0 (0) | 206 |
|  |  | Uplink | High | 29299.92 | 2100831 | 28384.08 | 2085567 | 504 |  | 22539 | 2098171 | 2 | 7 | 1 (4) | 1030 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.1.2	Reference test frequencies for NR operating band n258

Table 4.3.1.2.1.2-1: Test frequencies for NR operating band n258, SCS 60 kHz and ΔFRaster 60 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | 66 | Downlink | Low | 24275.04 | 2017083 | 24251.28 | 2016687 | 0 | 120 | 22257 | 2016955 | 4 | 2 | 0 (0) | 2 |
|  |  | & | Mid | 25875 | 2043749 | 25777.8 | 2042129 | 102 |  | 22350 | 2043739 | 2 | 4 | 1 (8) | 114 |
|  |  | Uplink | High | 27474.96 | 2070415 | 27088.32 | 2063971 | 504 |  | 22443 | 2070523 | 0 | 14 | 1 (8) | 526 |
| 100 | 132 | Downlink | Low | 24300 | 2017499 | 24252.48 | 2016707 | 0 | 120 | 22257 | 2016955 | 8 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 25875 | 2043749 | 25754.04 | 2041733 | 102 |  | 22349 | 2043451 | 2 | 13 | 1 (8) | 123 |
|  |  | Uplink | High | 27450 | 2069999 | 27039.6 | 2063159 | 504 |  | 22440 | 2069659 | 8 | 9 | 1 (8) | 521 |
| 200 | 264 | Downlink | Low | 24350.04 | 2018333 | 24255 | 2016749 | 0 | 120 | 22258 | 2017243 | 2 | 13 | 1 (8) | 21 |
|  |  | & | Mid | 25875 | 2043749 | 25706.52 | 2040941 | 102 |  | 22346 | 2042587 | 2 | 7 | 1 (8) | 117 |
|  |  | Uplink | High | 27399.96 | 2069165 | 26942.04 | 2061533 | 504 |  | 22434 | 2067931 | 2 | 1 | 1 (8) | 513 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-7 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.1.2: Test frequencies for NR operating band n258, SCS 120kHz and ΔFRaster 120 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | 32 | Downlink | Low | 24275.04 | 2017083 | 24252 | 2016699 | 0 | 120 | 22257 | 2016955 | 8 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 25875 | 2043749 | 25705.08 | 2040917 | 102 |  | 22350 | 2043739 | 7 | 1 | 1 (4) | 214 |
|  |  | Uplink | High | 27474.96 | 2070415 | 26726.16 | 2057935 | 504 |  | 22443 | 2070523 | 6 | 6 | 1 (4) | 1028 |
| 100 | 66 | Downlink | Low | 24300 | 2017499 | 24252.48 | 2016707 | 0 | 120 | 22257 | 2016955 | 4 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 25875 | 2043749 | 25680.6 | 2040509 | 102 |  | 22349 | 2043451 | 7 | 6 | 1 (4) | 224 |
|  |  | Uplink | High | 27450 | 2069999 | 26676.72 | 2057111 | 504 |  | 22440 | 2069659 | 10 | 4 | 1 (4) | 1024 |
| 200 | 132 | Downlink | Low | 24350.04 | 2018333 | 24255 | 2016749 | 0 | 120 | 22258 | 2017243 | 7 | 6 | 1 (4) | 20 |
|  |  | & | Mid | 25875 | 2043749 | 25633.08 | 2039717 | 102 |  | 22346 | 2042587 | 7 | 3 | 1 (4) | 218 |
|  |  | Uplink | High | 27399.96 | 2069165 | 26579.16 | 2055485 | 504 |  | 22434 | 2067931 | 7 | 0 | 1 (4) | 1016 |
| 400 | 264 | Downlink | Low | 24450 | 2019999 | 24259.92 | 2016831 | 0 | 120 | 22258 | 2017243 | 2 | 3 | 1 (4) | 14 |
|  |  | & | Mid | 25875 | 2043749 | 25538.04 | 2038133 | 102 |  | 22340 | 2040859 | 7 | 1 | 0 (0) | 206 |
|  |  | Uplink | High | 27300 | 2067499 | 26384.16 | 2052235 | 504 |  | 22423 | 2064763 | 0 | 4 | 1 (4) | 1024 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.1.3	Reference test frequencies for NR operating band n259

Table 4.3.1.2.1.3-1: Test frequencies for NR operating band n259, SCS 60 kHz and ΔFRaster 60 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | 66 | Downlink | Low | 39525 | 2271249 | 39501.24 | 2270853 | 0 | 120 | 23140 | 2271259 | 10 | 5 | 1 (8) | 13 |
|  |  | & | Mid | 41499.96 | 2304165 | 41402.76 | 2302545 | 102 |  | 23254 | 2304091 | 10 | 6 | 0 (0) | 108 |
|  |  | Uplink | High | 43474.92 | 2337081 | 43088.28 | 2330637 | 504 |  | 23369 | 2337211 | 10 | 15 | 1 (8) | 527 |
| 100 | 132 | Downlink | Low | 39550.08 | 2271667 | 39502.56 | 2270875 | 0 | 120 | 23140 | 2271259 | 0 | 4 | 1 (8) | 12 |
|  |  | & | Mid | 41499.96 | 2304165 | 41379 | 2302149 | 102 |  | 23253 | 2303803 | 10 | 7 | 1 (8) | 117 |
|  |  | Uplink | High | 43449.96 | 2336665 | 43039.56 | 2329825 | 504 |  | 23366 | 2336347 | 6 | 11 | 1 (8) | 523 |
| 200 | 264 | Downlink | Low | 39600 | 2272499 | 39504.96 | 2270915 | 0 | 120 | 23140 | 2271259 | 8 | 0 | 1 (8) | 8 |
|  |  | & | Mid | 41499.96 | 2304165 | 41331.48 | 2301357 | 102 |  | 23250 | 2302939 | 10 | 1 | 1 (8) | 111 |
|  |  | Uplink | High | 43399.92 | 2335831 | 42942 | 2328199 | 504 |  | 23360 | 2334619 | 0 | 3 | 1 (8) | 515 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-7 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.1.3-2: Test frequencies for NR operating band n259, SCS 120kHz and ΔFRaster 120 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | 32 | Downlink | Low | 39525 | 2271249 | 39501.96 | 2270865 | 0 | 120 | 23140 | 2271259 | 5 | 2 | 1 (4) | 12 |
|  |  | & | Mid | 41499.96 | 2304165 | 41330.04 | 2301333 | 102 |  | 23254 | 2304091 | 11 | 2 | 0 (0) | 208 |
|  |  | Uplink | High | 43474.92 | 2337081 | 42726.12 | 2324601 | 504 |  | 23369 | 2337211 | 5 | 7 | 1 (4) | 1030 |
| 100 | 66 | Downlink | Low | 39550.08 | 2271667 | 39502.56 | 2270875 | 0 | 120 | 23140 | 2271259 | 0 | 2 | 1 (4) | 12 |
|  |  | & | Mid | 41499.96 | 2304165 | 41305.56 | 2300925 | 102 |  | 23253 | 2303803 | 11 | 3 | 1 (4) | 218 |
|  |  | Uplink | High | 43449.96 | 2336665 | 42676.68 | 2323777 | 504 |  | 23366 | 2336347 | 9 | 5 | 1 (4) | 1026 |
| 200 | 132 | Downlink | Low | 39600 | 2272499 | 39504.96 | 2270915 | 0 | 120 | 23140 | 2271259 | 4 | 0 | 1 (4) | 8 |
|  |  | & | Mid | 41499.96 | 2304165 | 41258.04 | 2300133 | 102 |  | 23250 | 2302939 | 11 | 0 | 1 (4) | 212 |
|  |  | Uplink | High | 43399.92 | 2335831 | 42579.12 | 2322151 | 504 |  | 23360 | 2334619 | 6 | 1 | 1 (4) | 1018 |
| 400 | 264 | Downlink | Low | 39700.08 | 2274167 | 39510 | 2270999 | 0 | 120 | 23140 | 2271259 | 10 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 41499.96 | 2304165 | 41163 | 2298549 | 102 |  | 23245 | 2301499 | 11 | 6 | 1 (4) | 224 |
|  |  | Uplink | High | 43299.96 | 2334165 | 42384.12 | 2318901 | 504 |  | 23349 | 2331451 | 11 | 4 | 1 (4) | 1024 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.1.4	Reference test frequencies for NR operating band n260

Table 4.3.1.2.1.4-1: Test frequencies for NR operating band n260, SCS 60 kHz and ΔFRaster 60 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | 66 | Downlink | Low | 37025.04 | 2229583 | 37001.28 | 2229187 | 0 | 120 | 22995 | 2229499 | 0 | 6 | 0 (0) | 6 |
|  |  | & | Mid | 38499.96 | 2254165 | 38402.76 | 2252545 | 102 |  | 23081 | 2254267 | 6 | 13 | 1 (8) | 123 |
|  |  | Uplink | High | 39975 | 2278749 | 39588.36 | 2272305 | 504 |  | 23166 | 2278747 | 10 | 4 | 1 (8) | 516 |
| 100 | 132 | Downlink | Low | 37050 | 2229999 | 37002.48 | 2229207 | 0 | 120 | 22995 | 2229499 | 4 | 4 | 0 (0) | 4 |
|  |  | & | Mid | 38499.96 | 2254165 | 38379 | 2252149 | 102 |  | 23079 | 2253691 | 6 | 6 | 0 (0) | 108 |
|  |  | Uplink | High | 39949.92 | 2278331 | 39539.52 | 2271491 | 504 |  | 23163 | 2277883 | 8 | 0 | 1 (8) | 512 |
| 200 | 264 | Downlink | Low | 37100.04 | 2230833 | 37005 | 2229249 | 0 | 120 | 22995 | 2229499 | 10 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 38499.96 | 2254165 | 38331.48 | 2251357 | 102 |  | 23076 | 2252827 | 6 | 0 | 0 (0) | 102 |
|  |  | Uplink | High | 39900 | 2277499 | 39442.08 | 2269867 | 504 |  | 23157 | 2276155 | 0 | 0 | 0 (0) | 504 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-7 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.1.4-2: Test frequencies for NR operating band n260, SCS 120kHz and ΔFRaster 120 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | 32 | Downlink | Low | 37025.04 | 2229583 | 37002 | 2229199 | 0 | 120 | 22995 | 2229499 | 6 | 2 | 0 (0) | 4 |
|  |  | & | Mid | 38499.96 | 2254165 | 38330.04 | 2251333 | 102 |  | 23081 | 2254267 | 3 | 6 | 1 (4) | 224 |
|  |  | Uplink | High | 39975 | 2278749 | 39226.2 | 2266269 | 504 |  | 23166 | 2278747 | 11 | 1 | 1 (4) | 1018 |
| 100 | 66 | Downlink | Low | 37050 | 2229999 | 37002.48 | 2229207 | 0 | 120 | 22995 | 2229499 | 2 | 2 | 0 (0) | 4 |
|  |  | & | Mid | 38499.96 | 2254165 | 38305.56 | 2250925 | 102 |  | 23079 | 2253691 | 3 | 3 | 0 (0) | 210 |
|  |  | Uplink | High | 39949.92 | 2278331 | 39176.64 | 2265443 | 504 |  | 23163 | 2277883 | 4 | 0 | 1 (4) | 1016 |
| 200 | 132 | Downlink | Low | 37100.04 | 2230833 | 37005 | 2229249 | 0 | 120 | 22995 | 2229499 | 5 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 38499.96 | 2254165 | 38258.04 | 2250133 | 102 |  | 23076 | 2252827 | 3 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 39900 | 2277499 | 39079.2 | 2263819 | 504 |  | 23157 | 2276155 | 0 | 0 | 0 (0) | 1008 |
| 400 | 264 | Downlink | Low | 37200 | 2232499 | 37009.92 | 2229331 | 0 | 120 | 22996 | 2229787 | 0 | 5 | 1 (4) | 18 |
|  |  | & | Mid | 38499.96 | 2254165 | 38163 | 2248549 | 102 |  | 23071 | 2251387 | 3 | 2 | 1 (4) | 216 |
|  |  | Uplink | High | 39799.92 | 2275831 | 38884.08 | 2260567 | 504 |  | 23146 | 2272987 | 6 | 3 | 0 (0) | 1014 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

4.3.1.2.1.5	Reference test frequencies for NR operating band n261

Table 4.3.1.2.1.5-1: Test frequencies for NR operating band n261, SCS 60 kHz and ΔFRaster 60 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | 66 | Downlink | Low | 27525 | 2071249 | 27501.24 | 2070853 | 0 | 120 | 22446 | 2071387 | 6 | 16 | 1 (8) | 24 |
|  |  | & | Mid | 27924.96 | 2077915 | 27827.76 | 2076295 | 102 |  | 22469 | 2078011 | 0 | 13 | 1 (8) | 123 |
|  |  | Uplink | High | 28324.92 | 2084581 | 27938.28 | 2078137 | 504 |  | 22492 | 2084635 | 6 | 9 | 1 (8) | 521 |
| 100 | 132 | Downlink | Low | 27550.08 | 2071667 | 27502.56 | 2070875 | 0 | 120 | 22446 | 2071387 | 8 | 14 | 1 (8) | 22 |
|  |  | & | Mid | 27924.96 | 2077915 | 27804 | 2075899 | 102 |  | 22467 | 2077435 | 0 | 6 | 0 (0) | 108 |
|  |  | Uplink | High | 28299.96 | 2084165 | 27889.56 | 2077325 | 504 |  | 22489 | 2083771 | 2 | 5 | 1 (8) | 517 |
| 200 | 264 | Downlink | Low | 27600 | 2072499 | 27504.96 | 2070915 | 0 | 120 | 22446 | 2071387 | 4 | 11 | 1 (8) | 19 |
|  |  | & | Mid | 27924.96 | 2077915 | 27756.48 | 2075107 | 102 |  | 22464 | 2076571 | 0 | 0 | 0 (0) | 102 |
|  |  | Uplink | High | 28249.92 | 2083331 | 27792 | 2075699 | 504 |  | 22483 | 2082043 | 8 | 4 | 0 (0) | 508 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-7 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.2.1.5-2: Test frequencies for NR operating band n261, SCS 120kHz and ΔFRaster 120 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | 32 | Downlink | Low | 27525 | 2071249 | 27501.96 | 2070865 | 0 | 120 | 22446 | 2071387 | 9 | 7 | 1 (4) | 22 |
|  |  | & | Mid | 27924.96 | 2077915 | 27755.04 | 2075083 | 102 |  | 22469 | 2078011 | 0 | 6 | 1 (4) | 224 |
|  |  | Uplink | High | 28324.92 | 2084581 | 27576.12 | 2072101 | 504 |  | 22492 | 2084635 | 3 | 4 | 1 (4) | 1024 |
| 100 | 66 | Downlink | Low | 27550.08 | 2071667 | 27502.56 | 2070875 | 0 | 120 | 22446 | 2071387 | 4 | 7 | 1 (4) | 22 |
|  |  | & | Mid | 27924.96 | 2077915 | 27730.56 | 2074675 | 102 |  | 22467 | 2077435 | 0 | 3 | 0 (0) | 210 |
|  |  | Uplink | High | 28299.96 | 2084165 | 27526.68 | 2071277 | 504 |  | 22489 | 2083771 | 7 | 2 | 1 (4) | 1020 |
| 200 | 132 | Downlink | Low | 27600 | 2072499 | 27504.96 | 2070915 | 0 | 120 | 22446 | 2071387 | 8 | 5 | 1 (4) | 18 |
|  |  | & | Mid | 27924.96 | 2077915 | 27683.04 | 2073883 | 102 |  | 22464 | 2076571 | 0 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 28249.92 | 2083331 | 27429.12 | 2069651 | 504 |  | 22483 | 2082043 | 4 | 2 | 0 (0) | 1012 |
| 400 | 264 | Downlink | Low | 27700.08 | 2074167 | 27510 | 2070999 | 0 | 120 | 22446 | 2071387 | 2 | 2 | 1 (4) | 12 |
|  |  | & | Mid | 27924.96 | 2077915 | 27588 | 2072299 | 102 |  | 22459 | 2075131 | 0 | 2 | 1 (4) | 216 |
|  |  | Uplink | High | 28149.96 | 2081665 | 27234.12 | 2066401 | 504 |  | 22472 | 2078875 | 9 | 1 | 1 (4) | 1018 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-8 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<< End of changes >>