3GPP TSG-RAN5 Meeting #96-e	R5-224242

# Electronic Meeting, 15th – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1777 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding additional tolerance to test requirement of TxD Transmitter power test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_TxD-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Additional tolerance ΔTIB,c was missing in the test requirement of some TxD Transmitter power test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The missing additional tolerance ΔTIB,c was added to the test requirement of some TxD Transmitter power test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Good UE may fail the test without considering the additional tolerance ΔTIB,c. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2G.1.5, 6.2G.2.5, 6.2G.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

6.2G.1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2G.1.5-1 for Power Class 2 and Table 6.2G.1.5-2 for Power Class 1.5.

Table 6.2G.1.5-1: Maximum Output Power test requirement for Power Class 2

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n34 |  |  | 26 | +2+TT/-3-TT |  |  |
| n39 |  |  | 26 | +2+TT/-3-TT |  |  |
| n41 |  |  | 26 | +2+TT/-33-TT |  |  |
| n77 |  |  | 26 | +2+TT/-3-TT |  |  |
| n78 |  |  | 26 | +2+TT/-3-TT |  |  |
| n79 |  |  | 26 | +2+TT/-3-TT |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2G.1.5-3 |  |  |  |  |  |  |

Table 6.2G.1.5-2: Maximum Output Power test requirement for Power Class 1.5

| NR band | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n41 | 295 | +2+TT/-33-TT |  |  |  |  |
| n77 | 295 | +2+TT/-3-TT |  |  |  |  |
| n78 | 295 | +2+TT/-3-TT |  |  |  |  |
| n79 | 295 | +2+TT/-3-TT |  |  |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2G.1.5-3 NOTE 5: Achieved via dual Tx NOTE 6: No test required since there is no satisfying test points defined. |  |  |  |  |  |  |

Table 6.2G.1.5-3: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied.

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< Unchanged sections omitted >>

6.2G.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2G.2.5-1 to Table 6.2G.2.5-3.

Table 6.2G.2.5-1: UE Power Class test requirements (for Bands n34, n39, n41, n77, n78, n79) for Power Class 2 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 3.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.5 - TT2） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0- TT2） |
| 4 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 3.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.5- TT2） |
| 5 | 26 | 0 | 0.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 |  | 3 | 28.0 + TT | 22.5 - TT | （21.0 - TT2） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 8 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.5 - TT2） |
| 9 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 |  | 3 | 28.0 + TT | 21.5 - TT | （20.0 - TT2） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 12 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.0 - TT2） |
| 13 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 14 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0- TT2） |
| 15 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.5 - TT2） |
| 16 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 17 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 18 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 19 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.5 - TT2） |
| 20 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 21 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5- TT2） |
| 22 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 23 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.0 - TT2） |
| 24 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 26 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 27 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 28 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 29 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 30 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| 31 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| 32 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-2: UE Power Class test requirements (for Band n41, n77, n78, n79) for Power Class 1.5 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 0.5 | 0 | （1.52） | 28.5 | (27.0²) | 3.0 |  | 3 | 31.0 + TT | 25.5 - TT | (24.0 - TT²) |
| 2 | 29 | 0 | 6 | 0 | （1.52） | 23.0 | (21.5²) | 3.0 | （5.02） | 3 | 31.0 + TT | 20.0 - TT | (16.5 - TT²) |
| 3 | 29 | 0 | 6 | 0 | （1.52） | 23.0 | (21.5²) | 3.0 | （5.02） | 3 | 31.0 + TT | 20.0 - TT | (16.5 - TT²) |
| 4 | 29 | 0 | 2 | 0 | （1.52） | 27.0 | (25.5²) | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | (22.5 - TT²) |
| 5 | 29 | 0 | 0.5 | 0 | （1.52） | 28.5 | (27.0²) | 3.0 |  | 3 | 31.0 + TT | 25.5 - TT | (24.0 - TT²) |
| 6 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 7 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 8 | 29 | 0 | 2.5 | 0 | （1.52） | 26.5 | (25.0²) | 3.0 |  | 3 | 31.0 + TT | 23.5 - TT | (22.0 - TT²) |
| 9 | 29 | 0 | 1.5 | 0 | （1.52） | 27.5 | (26.0²) | 3.0 |  | 3 | 31.0 + TT | 24.5 - TT | (23.0 - TT²) |
| 10 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 11 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 12 | 29 | 0 | 3.5 | 0 | （1.52） | 25.5 | (24.0²) | 3.0 |  | 3 | 31.0 + TT | 22.5 - TT | (21.0 - TT²) |
| 13 | 29 | 0 | 3.5 | 0 | （1.52） | 25.5 | (24.0²) | 3.0 |  | 3 | 31.0 + TT | 22.5 - TT | (21.0 - TT²) |
| 14 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 15 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 16 | 29 | 0 | 4 | 0 | （1.52） | 25.0 | (23.5²) | 3.0 |  | 3 | 31.0 + TT | 22.0 - TT | (20.5 - TT²) |
| 17 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 18 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 19 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 20 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 21 | 29 | 0 | 2 | 0 | （1.52） | 27.0 | (25.5²) | 3.0 |  | 3 | 31.0 + TT | 24.0 - TT | (22.5 - TT²) |
| 22 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 23 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 24 | 29 | 0 | 4.5 | 0 | （1.52） | 24.5 | (23.0²) | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | (20.0 - TT²) |
| 25 | 29 | 0 | 2.5 | 0 | （1.52） | 26.5 | (25.0²) | 3.0 |  | 3 | 31.0 + TT | 23.5 - TT | (22.0 - TT²) |
| 26 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 27 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28 | 29 | 0 | 4.5 | 0 | （1.52） | 24.5 | (23.0²) | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | (20.0 - TT²) |
| 29 | 29 | 0 | 4.5 | 0 | （1.52） | 24.5 | (23.0²) | 3.0 |  | 3 | 31.0 + TT | 21.5 - TT | (20.0 - TT²) |
| 30 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 31 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 5.0 |  | 3 | 31.0 + TT | 17.5 - TT | (16.0 - TT²) |
| 32 | 29 | 0 | 5 | 0 | （1.52） | 24.0 | (22.5²) | 3.0 | （5.02） | 3 | 31.0 + TT | 21.0 - TT | (17.5 - TT²) |
| 33 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | (19.0²) | 6.0 | （5.02） | 3 | 31.0 + TT | 14.5 - TT | (14.0 - TT²) |
| 34 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | (19.0²) | 6.0 | （5.02） | 3 | 31.0 + TT | 14.5 - TT | (14.0 - TT²) |
| 35 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | (19.0²) | 6.0 | （5.02） | 3 | 31.0 + TT | 14.5 - TT | (14.0 - TT²) |
| 36 | 29 | 0 | 8.5 | 0 | （1.52） | 20.5 | (19.0²) | 6.0 | （5.02） | 3 | 31.0 + TT | 14.5 - TT | (14.0 - TT²) |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-3: UE Power Class test requirements (for Bands n34, n39, n41, n77, n78, n79) for Power Class 2 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 26 | 0 | 2 | 1.5 | 0 | 1.52 | 22.5 | 21.02 | 5.0 | 5.02 | 3 | 28.0 + TT | 17.5 - TT | 16 - TT2 |
| 15 | 26 | 0 | 2 | 1 | 0 | 1.52 | 23.0 | 21.52 | 3.0 | 5.02 | 3 | 28.0 + TT | 20.0 - TT | 16.5 - TT2 |
| 26 | 26 | 0 | 3.5 | 1.5 | 0 | 1.52 | 21.0 | 19.52 | 5.0 | 5.02 | 3 | 28.0 + TT | 16.0 - TT | 14.5 - TT2 |
| 27 | 26 | 0 | 3.5 | 1 | 0 | 1.52 | 21.5 | 20.02 | 5.0 | 6.02 | 3 | 28.0 + TT | 16.5 - TT | 14.0 - TT2 |
| 34 | 26 | 0 | 2.5 | 1.5 | 0 | 1.52 | 22.0 | 20.52 | 5.0 | 6.02 | 3 | 28.0 + TT | 17.0 - TT | 14.5 - TT2 |
| 35 | 26 | 0 | 2.5 | 1 | 0 | 1.52 | 22.5 | 21.02 | 5.0 | 5.02 | 3 | 28.0 + TT | 17.5 - TT | 16.0 - TT2 |
| 46 | 26 | 0 | 3.5 | 1.5 | 0 | 1.52 | 21.0 | 19.52 | 5.0 | 5.02 | 3 | 28.0 + TT | 16.0 - TT | 14.5 - TT2 |
| 47 | 26 | 0 | 3.5 | 1 | 0 | 1.52 | 21.5 | 20.02 | 5.0 | 6.02 | 3 | 28.0 + TT | 16.5 - TT | 14.0 - TT2 |
| 56 | 26 | 0 | 4.5 | 1.5 | 0 | 1.52 | 20.0 | 18.52 | 6.0 | 5.02 | 3 | 28.0 + TT | 14.0 - TT | 13.5 - TT2 |
| 57 | 26 | 0 | 4.5 | 1 | 0 | 1.52 | 20.5 | 19.02 | 6.0 | 5.02 | 3 | 28.0 + TT | 14.5 - TT | 14.0 - TT2 |
| 66 | 26 | 0 | 8 | 1.5 | 0 | 1.52 | 16.5 | 152 | 5.0 | 5.02 | 3 | 28.0 + TT | 11.5 - TT | 10.0 - TT2 |
| 67 | 26 | 0 | 8 | 1 | 0 | 1.52 | 17.0 | 15.52 | 5.0 | 5.02 | 3 | 28.0 + TT | 12.0 - TT | 10.5 - TT2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2G.2.5-4. NOTE 4: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 5: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 6: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 8: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.2.5-4: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied.

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< Unchanged sections omitted >>

6.2G.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2G.3.5-1. The allowed A-MPR values specified in table 6.2G.3.3-1 are in addition to the allowed MPR requirements specified in clause 6.2G.2. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2G.1.3-1 apply.

Table 6.2G.3.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2G.3.5-23: UE Power Class 2 test requirements for NS_47 (contiguous allocation)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 2 | 26 | 0 | 0.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 3 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 4 | 26 | 0 | 0.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 5 | 26 | 0 | 0.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 6 | 26 | 0 | 0.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 7 | 26 | 0 | 0.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 8 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 9 | 26 | 0 | 0.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 10 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 11 | 26 | 0 | 1.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 12 | 26 | 0 | 1.0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 13 | 26 | 0 | 1.0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 14 | 26 | 0 | 1.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 15 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 16 | 26 | 0 | 1.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 17 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 18 | 26 | 0 | 2.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 19 | 26 | 0 | 2.0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 20 | 26 | 0 | 2.0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 21 | 26 | 0 | 2.0 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 22 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 23 | 26 | 0 | 2.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 24 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 25 | 26 | 0 | 2.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 26 | 26 | 0 | 2.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 27 | 26 | 0 | 2.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 28 | 26 | 0 | 2.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 29 | 26 | 0 | 4.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 30 | 26 | 0 | 4.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 31 | 26 | 0 | 4.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 32 | 26 | 0 | 4.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 33 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 34 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 35 | 26 | 0 | 4.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 36 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 37 | 26 | 0 | 1.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 38 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 39 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 40 | 26 | 0 | 3 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 41 | 26 | 0 | 3 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 42 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 43 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 44 | 26 | 0 | 2.0 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 45 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 46 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 47 | 26 | 0 | 3 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 48 | 26 | 0 | 3 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 49 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 50 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 51 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 52 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 53 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 54 | 26 | 0 | 3.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 55 | 26 | 0 | 3.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 56 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 57 | 26 | 0 | 6.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 58 | 26 | 0 | 6.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 59 | 26 | 0 | 6.5 | 0 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 26 | 0 | 6.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 61 | 26 | 0 | 6.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 62 | 26 | 0 | 6.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 63 | 26 | 0 | 6.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2G.3.5-0. |  |  |  |  |  |  |  |  |  |  |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied.

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

<< End of changes >>