3GPP TSG-RAN WG5 Meeting #96-e                                                                                                                                  R5-224256

# Electronic Meeting, 15th – 26th August 2022

Agenda Item:	5.4.17

Source: 	CAICT

Title: 	Discussion on the treatment for the pending R15 NR FR1 band n50 and n86

Document for:	Discussion and Endorsement

Introduction

RAN5 has endorsed the proposal to clean up all the Bands, CBWs, configurations marked as pending in PRD21 in all RAN5 specs. This paper is for the discussion of the exception to the endorsed rule for R15 NR FR1 band n50 and n86.

Discussion

The status of the R15 NR FR1 band n50 and n86 in PRD 21 are as below:

| Release introduced in TS 38.101-x | Type of band | TS and Source Table | NR band / NR band CBW Extension | CBWs SCS[kHz]:[MHz]+...+[MHz] | PC3 Status | Completed Power Class(es) (Note 1) | RAN5 Completion Meeting (Note 1) | RAN5 Completion Reference (Note 1) | Interested operator (PC3) | Responsible Company (contact) (PC3) | Assignment [RAN5 meeting] (PC3) | Applicable RAN5 WI code(s) for CRs (Note 1) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rel-15 | TDD FR1 | 38.101-1:Table 5.3.5-1 | n50 | SCS15:5+10+15+20+40+50 SCS30:10+15+20+40+50+60+80 SCS60:10+15+20+40+50+60+80 | Pending |  |  |  |  |  |  | PC3:TEI15_Test, 5GS_NR_LTE-UEConTest |
| Rel-15 | SUL FR1 | 38.101-1:Table 5.3.5-1 | n86 | SCS15:5+10+15+20+40 SCS30:10+15+20+40 SCS60:10+15+20+40 | Pending |  |  |  |  |  |  | PC3:TEI15_Test, 5GS_NR_LTE-UEConTest |

Observation 1: The R15 NR FR1 bands n50 and n86 are still pending in PRD 21.

Most of RAN5 works for these two R15 bands have been finished, including 38.521-1(e.g., 5.3.5, 5.3.6, 5.4.2.3, 5.4.3.3, 6.2.1, 6.2.2, 6.2.3, 6.2.4, 6.5.3, 7.3.2,7.6.2, 7.6.3, 7.6.4 for n50), 38.508-1( e.g. test CBW and test frequencies) and 38.508-2.

Some of the examples in 38.521-1 v17.5.0 are as below:

Table 5.3.5-1: Channel Bandwidths for each NR band

| NR Band | SCS (kHz) |  | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 45 | 50 | 60 | 706 | 80 | 906 | 100 |
| n4711 | 15 |  | 10 |  | 20 |  | 30 | 40 |  |  |  |  |  |  |  |
|  | 30 |  | 10 |  | 20 |  | 30 | 40 |  |  |  |  |  |  |  |
|  | 60 |  | 10 |  | 20 |  | 30 | 40 |  |  |  |  |  |  |  |
| n48 | 15 | 57 | 10 | 15 | 20 |  | 30 | 40 |  | 508 |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 |  | 30 | 40 |  | 508 | 608 | 708 | 808 | 908,10 | 1008 |
|  | 60 |  | 10 | 15 | 20 |  | 30 | 40 |  | 508 | 608 | 708 | 808 | 908,10 | 1008 |
| n50 | 15 | 57 | 10 | 15 | 20 |  |  | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 |  |  | 40 |  | 50 | 60 |  | 803 |  |  |
|  | 60 |  | 10 | 15 | 20 |  |  | 40 |  | 50 | 60 |  | 803 |  |  |
| n51 | 15 | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n65 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| n66 | 15 | 5 | 10 | 15 | 20 | 256 | 306 | 40 |  |  |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 | 256 | 306 | 40 |  |  |  |  |  |  |  |
|  | 60 |  | 10 | 15 | 20 | 256 | 306 | 40 |  |  |  |  |  |  |  |
| n84 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  | 50 |  |  |  |  |  |
|  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  | 50 |  |  |  |  |  |
| n86 | 15 | 5 | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
|  | 60 |  | 10 | 15 | 20 |  |  | 40 |  |  |  |  |  |  |  |
| n95 | 15 | 5 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |
| n96 | 15 |  |  |  | 20 |  |  | 40 |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 20 |  |  | 40 |  |  | 60 |  | 80 |  |  |
|  | 60 |  |  |  | 20 |  |  | 40 |  |  | 60 |  | 80 |  |  |
| n97 | 15 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |
| n99 | 15 | 5 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: This UE channel bandwidth is applicable only to downlink. NOTE 4: For test configuration tables from the transmitter and receiver tests in Section 6 and 7 that refer to this table for test SCS, the Lowest SCS refers to lowest supported SCS per channel bandwidth, Highest SCS refers to highest supported SCS per channel bandwidth. NOTE 5: For test configuration tables from the transmitter and receiver tests in Section 6 and 7 that refer to this table and list and list the test SCS as Mid or any other value; if that value is not supported by the UE in UL and/or DL, select the closest SCS supported by the UE in both UL and DL. NOTE 6: This UE channel bandwidth is optional in R15. NOTE 7: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as an SCell part of DC or CA configuration. NOTE 8: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. NOTE 9: For the 20 MHz bandwidth, the minimum requirements are specified for NR UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz. For the 30MHz bandwidth, the minimum requirements are specified for NR UL transmission bandwidth configuration confined to either 703-733 or 718-748 MHz. NOTE 10: This UE channel bandwidth is optional in R16. NOTE 11: These UE channel bandwidths are applicable to sidelink operation. NOTE 12: This UE channel bandwidth is optional in R17. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.1.5-1: Maximum Output Power test requirement for Power Class 3

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 23 | ±2±TT |
| n2 |  |  |  |  | 23 | ±23±TT |
| n3 |  |  |  |  | 23 | ±23±TT |
| n5 |  |  |  |  | 23 | ±2±TT |
| n7 |  |  |  |  | 23 | ±23±TT |
| n8 |  |  |  |  | 23 | ±23±TT |
| n12 |  |  |  |  | 23 | ±23±TT |
| n14 |  |  |  |  | 23 | ±2±TT |
| n20 |  |  |  |  | 23 | ±23±TT |
| n24 |  |  |  |  | 23 | +2+TT/-3.03-TT |
| n25 |  |  |  |  | 23 | ±23±TT |
| n26 |  |  |  |  | 23 | ±23±TT |
| n28 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n30 |  |  |  |  | 23 | ±2±TT |
| n34 |  |  |  |  | 23 | ±2±TT |
| n38 |  |  |  |  | 23 | ±2±TT |
| n39 |  |  |  |  | 23 | ±2±TT |
| n40 |  |  |  |  | 23 | ±2±TT |
| n41 |  |  |  |  | 23 | ± 23±TT |
| n48 |  |  |  |  | 23 | +2+TT/-3-TT |
| n50 |  |  |  |  | 23 | ±2±TT |
| n51 |  |  |  |  | 23 | ±2±TT |
| n53 |  |  |  |  | 23 | ±2±TT |
| n65 |  |  |  |  | 23 | ±2±TT |
| n66 |  |  |  |  | 23 | ±2±TT |
| n70 |  |  |  |  | 23 | ±2±TT |
| n71 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n74 |  |  |  |  | 23 | ±2±TT |
| n77 |  |  |  |  | 23 | +2+TT/-3-TT |
| n78 |  |  |  |  | 23 | +2+TT/-3-TT |
| n79 |  |  |  |  | 23 | +2+TT/-3-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3 |  |  |  |  |  |  |

Table 6.2C.3.5-1: Maximum Output Power test requirement for Power Class 3

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n80 |  |  |  |  | 23 | ±2±TT |
| n81 |  |  |  |  | 23 | ±2±TT |
| n82 |  |  |  |  | 23 | ±2±TT |
| n83 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n84 |  |  |  |  | 23 | ±2±TT |
| n86 |  |  |  |  | 23 | ±2±TT |
| n97 |  |  |  |  | 23 | ±2±TT |
| n99 |  |  |  |  | 23 | 2+TT/-3.03-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2.1.5-3 |  |  |  |  |  |  |

Table 7.3.2.3-1b: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11) |  |
| n381 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11) |  |
| n39 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11) |  |
| n40 | 15 | 5, 10, 15, 20, 25, 30, 40, 50 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.5 + 10log10(NRB/11) |  |
| n411 | 15 | 10, 15, 20, 30, 40, 50 | -94.8 + 10log10(NRB/50) | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.5 + 10log10(NRB/11) |  |
| n481 | 15 | 5, 10, 15, 20, 30, 40, 505 | -99 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 30, 40, 505, 605, 705, 805, 905, 1005 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 30, 40, 505, 605, 705, 805, 905, 1005 | -96.5 + 10log10(NRB/11) |  |
| n50 | 15 | 5, 10, 15, 20, 30, 40, 50 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.5 + 10log10(NRB/11) |  |
| n51 | 15 | 5 | -100 | TDD |
| n53 | 15 | 5, 10 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10 | -97.1 |  |
|  | 60 | 10 | -97.5 |  |
| n757 | 15 | 5,10,15,20 | -100 + 10log10(NRB/25) | SDL |
|  | 30 | 10,15,20 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10,15,20 | -97.5 + 10log10(NRB/11) |  |
| n767 | 15 | 5 | -100 | SDL |
| n771,4 | 15 | 10, 15, 20, 40, 50 | -95.3 + 10log10(NRB/50) | TDD |
|  | 30 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -95.6 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -96.0 + 10log10(NRB/11) |  |
| n781 | 15 | 10, 15, 20, 40, 50 | -95.8 + 10log10(NRB/50) | TDD |
|  | 30 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11) |  |
| n791 | 15 | 40, 50 | -89.6 + 10log10(NRB/216) | TDD |
|  | 30 | 40, 50, 60, 80, 100 | -89.7 + 10log10(NRB/106) |  |
|  | 60 | 40, 50, 60, 80, 100 | -89.9 + 10log10(NRB/51) |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Void NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. |  |  |  |  |

Table 7.3C.2.4.1-1: Test Configuration Table for SUL without exceptions

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Mid range for SUL carrier. Low, Mid, High range for non-SUL carrier With following exceptions: SUL_n78-n80: High in band n78 |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Lowest, Mid, Highest for Non-SUL carrier For SUL band: n80: 30 MHz n81: 20 MHz n82: 20 MHz n83: 20 MHz n84: 20 MHz n86: 40 MHz n95: 15 MHz |  |  |
| Test SCS as specified in Table 5.5C-1 |  |  |  | 15kHz for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Downlink Configuration |  | UL Configuration |  | SUL Configuration |  |
|  | Modulation | RB allocation |  |  | Modulation | RB allocation (NOTE 2) |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) | N/A |  | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: REFSENS refers to Table 7.3C.2.4.1-1a which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: In a band where UE supports 4Rx, the test needs to be repeated with only 2Rx antennas connected and the other antennas terminated. |  |  |  |  |  |  |

Some of the examples in 38.508-1 v17.5.0 are as below:

Table 4.3.1.0A-1: Mid Test Channel bandwidths for each NR band, FR1

| NR Band | UE Mid Test Channel bandwidth [MHz]1, 1a, 1b | RedCap UE Mid Test Channel bandwidth [MHz] |
| --- | --- | --- |
| n1 | 25 | 15 |
| n2 | 20 | 15 |
| n3 | 20 | 15 |
| n5 | 15 | 15 |
| n7 | 25 | 15 |
| n8 | 15 | 15 |
| n12 | 10 | 10 |
| n14 | 10 | 10 |
| n20 | 15 | 15 |
| n24 | 10 | 10 |
| n25 | 20 | 15 |
| n26 | 10 | 15 |
| n28 | 15 | 15 |
| n29 | 102 | N/A |
| n30 | 10 | 10 |
| n34 | 10 | 10 |
| n38 | 20 | 15 |
| n39 | 20 | 15 |
| n40 | 30 | 15 |
| n41 | 60 | 15 |
| n48 | 204, 405 | 15 |
| n50 | 20 | 15 |
| n51 | 5 | 5 |
| n53 | 10 | 10 |
| n65 | 15 | 15 |
| n66 | 20 | 108 |
| n70 | 15 | 10 |
| n71 | 10 | 10 |
| n74 | 15 | 15 |
| n75 | 152 | N/A |
| n76 | 52 | N/A |
| n77 | 50 | 15 |
| n78 | 50 | 15 |
| n79 | 60 | 20 |
| n80 | 203 | [15] |
| n81 | 153 | [15] |
| n82 | 153 | [15] |
| n83 | 153 | [15] |
| n84 | 253 | [15] |
| n86 | 203 | [15] |
| n95 | 103 | [10] |
| n97 | 503 | [15] |
| n99 | 103 | [10] |
| Note 1: Mid channel BW is the closest channel BW to the arithmetic average channel BW of all the possible channel BWs per band in Table 5.3.5-1 of TS 38.521-1 [14] among all SCSs. If there are two channel bandwidths that have same distance to the average value, the higher one is selected. In case such bandwidth is not applicable for a given subcarrier spacing, the closer bandwidth to the value in this table applicable for such subcarrier spacing shall be tested. In case such bandwidth is not defined in the UE release specification according to 38.101-1 [7] Table 5.3.5-1: - If the bandwidth listed above is equal or higher than maximum bandwidth supported in the UE release according to 38.101-1 [7] Table 5.3.5-1, the closest channel BW to the arithmetic average among the values supported in the UE release specification shall be tested. - Otherwise, the closer bandwidth to the value in this table defined for that band in the UE release specification shall be tested. If there are two channel bandwidths that have same distance to the arithmetic average, the higher one is selected Note 1a: Values listed in this table assume that the (non-optional) channel bandwidths specified in Table 5.3.5-1 of TS 38.101-1 [7] lower than the maximum are supported. However, these channel bandwidths are mandatory with capability parameter as defined in [55] TS 38.306 clause 4.2.1 for channelBWs-DL/channelBWs-UL parameters. Hence the UE might indicate them as not supported. In such case, select the closest channel bandwidth in both DL and UL. Note 1b: For CA, DC, SDL and SUL, the mid-test channel bandwidth per component carrier is chosen to test the closest aggregated bandwidth to the mathematical centre between minimum and maximum aggregated bandwidth defined for and within a given bandwidth combination set. In case no set of channel bandwidths per component carrier supported by the UE can achieve such aggregated bandwidth, select one combination of bandwidths per component carrier within the bandwidth combination set that minimizes the difference to the target aggregated bandwidth. Note 2: This UE channel bandwidth is applicable only to downlink. Note 3: This UE channel bandwidth is applicable only to uplink. Note 4: Applicable when for use as single carrier, PCell in CA or PCell in DC configuration. Note 5: Applicable for use as SCell in CA or SCell in DC configuration. Note 6: Void Note 7: Void Note 8: Void Note 9: Void |  |  |

- 4.3.1.1.1.50	Reference test frequencies for NR operating band n50

Table 4.3.1.1.1.50-1: Test frequencies for NR operating band n50 and SCS 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset[RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Downlink | Low | 1434.5 | 286900 | 1432.25 | 286450 | 0 | (Note 3) | - | 287170 | - | - | - | - |
|  |  | & | Mid | 1474.5 | 294900 | 1453.89 | 290778 | 102 |  | - | 295170 | - | - | - | - |
|  |  | Uplink | High | 1514.5 | 302900 | 1421.53 | 284306 | 504 |  | - | 303170 | - | - | - | - |
| 10 | 52 | Downlink | Low | 1437 | 287400 | 1432.32 | 286464 | 0 | 30 | 3591 | 287310 | 6 | 1 | 0 (2) | 3 |
|  |  | & | Mid | 1474.5 | 294900 | 1451.46 | 290292 | 102 |  | 3687 | 294990 | 6 | 2 | 1 (6) | 110 |
|  |  | Uplink | High | 1512 | 302400 | 1416.6 | 283320 | 504 |  | 3780 | 302430 | 10 | 0 | 1 (6) | 510 |
| 15 | 79 | Downlink | Low | 1439.5 | 287900 | 1432.39 | 286478 | 0 | 30 | 3592 | 287330 | 8 | 1 | 0 (2) | 3 |
|  |  | & | Mid | 1474.5 | 294900 | 1449.03 | 289806 | 102 |  | 3678 | 294270 | 0 | 0 | 0 (2) | 104 |
|  |  | Uplink | High | 1509.5 | 301900 | 1411.67 | 282334 | 504 |  | 3767 | 301450 | 0 | 1 | 1 (6) | 511 |
| 20 | 106 | Downlink | Low | 1442 | 288400 | 1432.46 | 286492 | 0 | 30 | 3590 | 287290 | 2 | 0 | 0 (2) | 2 |
|  |  | & | Mid | 1474.5 | 294900 | 1446.6 | 289320 | 102 |  | 3672 | 293790 | 2 | 0 | 0 (2) | 104 |
|  |  | Uplink | High | 1507 | 301400 | 1406.74 | 281348 | 504 |  | 3754 | 300290 | 2 | 0 | 0 (2) | 506 |
| 40 | 216 | Downlink | Low | 1452 | 290400 | 1432.56 | 286512 | 0 | 30 | 3591 | 287310 | 2 | 0 | 0 (2) | 2 |
|  |  | & | Mid | 1474.5 | 294900 | 1436.7 | 287340 | 102 |  | 3648 | 291870 | 10 | 1 | 0 (2) | 105 |
|  |  | Uplink | High | 1497 | 299400 | 1386.84 | 277368 | 504 |  | 3705 | 296430 | 6 | 3 | 0 (2) | 509 |
| 50 | 270 | Downlink | Low | 1457 | 291400 | 1432.7 | 286540 | 0 | 30 | 3593 | 287530 | 6 | 1 | 1 (6) | 7 |
|  |  | & | Mid | 1474.5 | 294900 | 1431.84 | 286368 | 102 |  | 3636 | 290910 | 2 | 2 | 0 (2) | 106 |
|  |  | Uplink | High | 1492 | 298400 | 1376.98 | 275396 | 504 |  | 3682 | 294530 | 6 | 1 | 1 (6) | 511 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: No SS/PBCH block fits within the channel bandwidth. The channel bandwidth can only be used as SCell |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.50-2: Test frequencies for NR operating band n50 and SCS 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low | 1437 | 287400 | 1432.68 | 286536 | 0 | 30 | 3591 | 287310 | 18 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 1474.5 | 294900 | 1433.46 | 286692 | 102 |  | 3687 | 294990 | 6 | 0 | 3 (3) | 210 |
|  |  | Uplink | High | 1512 | 302400 | 1326.24 | 265248 | 504 |  | 3780 | 302430 | 10 | 0 | 2 (2) | 1012 |
| 15 | 38 | Downlink | Low | 1439.5 | 287900 | 1432.66 | 286532 | 0 | 30 | 3592 | 287330 | 2 | 0 | 1 (1) | 2 |
|  |  | & | Mid | 1474.5 | 294900 | 1430.94 | 286188 | 102 |  | 3678 | 294270 | 6 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 1509.5 | 301900 | 1321.22 | 264244 | 504 |  | 3767 | 301450 | 18 | 0 | 2 (2) | 1012 |
| 20 | 51 | Downlink | Low | 1442 | 288400 | 1432.82 | 286564 | 0 | 30 | 3590 | 287290 | 2 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 1474.5 | 294900 | 1428.6 | 285720 | 102 |  | 3672 | 293790 | 2 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 1507 | 301400 | 1316.38 | 263276 | 504 |  | 3754 | 300290 | 2 | 0 | 0 (0) | 1008 |
| 40 | 106 | Downlink | Low | 1452 | 290400 | 1432.92 | 286584 | 0 | 30 | 3591 | 287310 | 2 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 1474.5 | 294900 | 1418.7 | 283740 | 102 |  | 3648 | 291870 | 22 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 1497 | 299400 | 1296.48 | 259296 | 504 |  | 3705 | 296430 | 18 | 0 | 1 (1) | 1010 |
| 50 | 133 | Downlink | Low | 1457 | 291400 | 1433.06 | 286612 | 0 | 30 | 3593 | 287530 | 18 | 0 | 2 (2) | 4 |
|  |  | & | Mid | 1474.5 | 294900 | 1413.84 | 282768 | 102 |  | 3636 | 290910 | 2 | 0 | 1 (1) | 206 |
|  |  | Uplink | High | 1492 | 298400 | 1286.62 | 257324 | 504 |  | 3682 | 294530 | 18 | 0 | 2 (2) | 1012 |
| 60 | 162 | Downlink | Low | 1462 | 292400 | 1432.84 | 286568 | 0 | 30 | 3592 | 287330 | 14 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 1474.5 | 294900 | 1408.62 | 281724 | 102 |  | 3624 | 289950 | 6 | 0 | 2 (2) | 208 |
|  |  | Uplink | High | 1487 | 297400 | 1276.4 | 255280 | 504 |  | 3653 | 292330 | 14 | 0 | 0 (0) | 1008 |
| 80 | 217 | Downlink | Low | 1472 | 294400 | 1432.94 | 286588 | 0 | 30 | 3593 | 287530 | 2 | 0 | 3 (3) | 6 |
| (Note 3) |  |  | Mid | 1474.5 | 294900 | 1398.72 | 279744 | 102 |  | 3600 | 288030 | 2 | 0 | 3 (3) | 210 |
|  |  |  | High | 1477 | 295400 | 1256.5 | 251300 | 504 |  | 3607 | 288530 | 2 | 0 | 3 (3) | 1014 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: This UE channel bandwidth is applicable only to downlink (TS 38.101-1 table 5.3.5-1). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.50-3: Test frequencies for NR operating band n50 and SCS 60 kHz without CORESET#0

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 1437 | 287400 | 1433.04 | 286608 | 0 | 30 | - | 287328 |
|  |  | & | Mid | 1474.5 | 294900 | 1397.1 | 279420 | 102 |  | - | 294828 |
|  |  | Uplink | High | 1512 | 302400 | 1145.16 | 229032 | 504 |  | - | 302328 |
| 15 | 18 | Downlink | Low | 1439.5 | 287900 | 1433.02 | 286604 | 0 | 30 | - | 287324 |
|  |  | & | Mid | 1474.5 | 294900 | 1394.58 | 278916 | 102 |  | - | 294324 |
|  |  | Uplink | High | 1509.5 | 301900 | 1140.14 | 228028 | 504 |  | - | 301324 |
| 20 | 24 | Downlink | Low | 1442 | 288400 | 1433.36 | 286672 | 0 | 30 | - | 287392 |
|  |  | & | Mid | 1474.5 | 294900 | 1392.42 | 278484 | 102 |  | - | 293892 |
|  |  | Uplink | High | 1507 | 301400 | 1135.48 | 227096 | 504 |  | - | 300392 |
| 40 | 51 | Downlink | Low | 1452 | 290400 | 1433.64 | 286728 | 0 | 30 | - | 287448 |
|  |  | & | Mid | 1474.5 | 294900 | 1382.7 | 276540 | 102 |  | - | 291948 |
|  |  | Uplink | High | 1497 | 299400 | 1115.76 | 223152 | 504 |  | - | 296448 |
| 50 | 65 | Downlink | Low | 1457 | 291400 | 1433.6 | 286720 | 0 | 30 | - | 287440 |
|  |  | & | Mid | 1474.5 | 294900 | 1377.66 | 275532 | 102 |  | - | 290940 |
|  |  | Uplink | High | 1492 | 298400 | 1105.72 | 221144 | 504 |  | - | 294440 |
| 60 | 79 | Downlink | Low | 1462 | 292400 | 1433.56 | 286712 | 0 | 30 | - | 287432 |
|  |  | & | Mid | 1474.5 | 294900 | 1372.62 | 274524 | 102 |  | - | 289932 |
|  |  | Uplink | High | 1487 | 297400 | 1095.68 | 219136 | 504 |  | - | 292432 |
| 80 | 107 | Downlink | Low | 1472 | 294400 | 1433.48 | 286696 | 0 | 30 | - | 287416 |
| (Note 2) |  |  | Mid | 1474.5 | 294900 | 1362.54 | 272508 | 102 |  | - | 287916 |
|  |  |  | High | 1477 | 295400 | 1075.6 | 215120 | 504 |  | - | 288416 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). Note 2: This UE channel bandwidth is applicable only to downlink (TS 38.101-1 table 5.3.5-1). |  |  |  |  |  |  |  |  |  |  |  |

- 4.3.1.1.1.86	Reference test frequencies for NR operating band n86 (SUL)

Table 4.3.1.1.1.86-1: Test frequencies for NR operating band n86 and SCS 15 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1652.03 | 330406 | 504 |
|  |  |  | High | 1777.5 | 355500 | 1774.17 | 354834 | 6 |
| 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |
|  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |
| 15 | 79 | Uplink | Low | 1717.5 | 343500 | 1710.39 | 342078 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1647.17 | 329434 | 504 |
|  |  |  | High | 1772.5 | 354500 | 1764.31 | 352862 | 6 |
| 20 | 106 | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1644.74 | 328948 | 504 |
|  |  |  | High | 1770 | 354000 | 1759.38 | 351876 | 6 |
| 40 | 216 | Uplink | Low | 1730 | 346000 | 1710.56 | 342112 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1634.84 | 326968 | 504 |
|  |  |  | High | 1760 | 352000 | 1739.48 | 347896 | 6 |

Table 4.3.1.1.1.86-2: Test frequencies for NR operating band n86 and SCS 30 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |
|  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |
| 15 | 38 | Uplink | Low | 1717.5 | 343500 | 1710.66 | 342132 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1556.72 | 311344 | 504 |
|  |  |  | High | 1772.5 | 354500 | 1763.5 | 352700 | 6 |
| 20 | 51 | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1554.38 | 310876 | 504 |
|  |  |  | High | 1770 | 354000 | 1758.66 | 351732 | 6 |
| 40 | 106 | Uplink | Low | 1730 | 346000 | 1710.92 | 342184 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1544.48 | 308896 | 504 |
|  |  |  | High | 1760 | 352000 | 1738.76 | 347752 | 6 |

Table 4.3.1.1.1.86-3: Test frequencies for NR operating band n86 and SCS 60 kHz

| Bandwidth [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Uplink | Low | 1715 | 343000 | 1711.04 | 342208 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1378.16 | 275632 | 504 |
|  |  |  | High | 1775 | 355000 | 1766.72 | 353344 | 6 |
| 15 | 18 | Uplink | Low | 1717.5 | 343500 | 1711.02 | 342204 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1375.64 | 275128 | 504 |
|  |  |  | High | 1772.5 | 354500 | 1761.7 | 352340 | 6 |
| 20 | 24 | Uplink | Low | 1720 | 344000 | 1711.36 | 342272 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1373.48 | 274696 | 504 |
|  |  |  | High | 1770 | 354000 | 1757.04 | 351408 | 6 |
| 40 | 51 | Uplink | Low | 1730 | 346000 | 1711.64 | 342328 | 0 |
|  |  |  | Mid | 1745 | 349000 | 1363.76 | 272752 | 504 |
|  |  |  | High | 1760 | 352000 | 1737.32 | 347464 | 0 |

Some of the examples in 38.508-2 v17.5.0 are as below:

Table A.4.3.1-2: NR TDD FR1 RF Baseline Implementation Capabilities

| Item | NR TDD FR1 RF Baseline Implementation Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 0 | NR Frequency band: 2010-2025 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand34_Supp | NR TDD FR1 Band n34 |
| 0a to 0c | Reserved |  |  |  |  |
| 1 | NR Frequency band: 2570-2620 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand38_Supp | NR TDD FR1 Band n38 |
| 1a | NR Frequency band: 1880-1920 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand39_Supp | NR TDD FR1 Band n39 |
| 1b | NR Frequency band: 2300-2400 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand40_Supp | NR TDD FR1 Band n40 |
| 2 | NR Frequency band: 2496-2690 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand41_Supp | NR TDD FR1 Band n41 |
| 2a to 2d | Reserved |  |  |  |  |
| 2e | NR Frequency band: 5150-5925 MHz (UL / DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand46_Supp | NR TDD FR1 Band n46 |
| 2f | Reserved |  |  |  |  |
| 2g | NR Frequency band: 3550-3700 MHz (UL / DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand48_Supp | NR TDD FR1 Band n48 |
| 2h | Reserved |  |  |  |  |
| 2i | NR Frequency band: 1432-1517 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand50_Supp | NR TDD FR1 Band n50 |
| 2j | NR Frequency band: 1427-1432 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand51_Supp | NR TDD FR1 Band n51 |
| 2k | Reserved |  |  |  |  |
| 2l | NR Frequency band: 2483.5-2495 MHz (UL / DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand53_Supp | NR TDD FR1 Band n53 |
| 3 | NR Frequency band: 3300–4200 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand77_Supp | NR TDD FR1 Band n77 |
| 4 | NR Frequency band: 3300–3800 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand78_Supp | NR TDD FR1 Band n78 |
| 5 | NR Frequency band: 4400–5000 MHz (UL / DL) | 38.101-1, 5.2 | Rel-15 | pc_nrBand79_Supp | NR TDD FR1 Band n79 |
| 6 | NR Frequency band: 2496–2690 MHz (UL / DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand90_Supp | NR TDD FR1 Band n90 |
| 7-11 | Reserved |  |  |  |  |
| 12 | NR Frequency band: 5925–7125 MHz (UL / DL) | 38.101-1, 5.2 | Rel-16 | pc_nrBand96_Supp | NR TDD FR1 Band n96 |

Table A.4.3.1-5: NR SUL FR1 RF Baseline Implementation Capabilities

| Item |  | NR SUL FR1 RF Baseline Implementation Capabilities |  | Ref. |  | Release |  | Mnemonic |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | NR Frequency band: 1710-1785 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand80_Supp |  | NR SUL FR1 Band n80 |  |
| 2 |  | NR Frequency band: 880-915 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand81_Supp |  | NR SUL FR1 Band n81 |  |
| 3 |  | NR Frequency band: 832-862 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand82_Supp |  | NR SUL FR1 Band n82 |  |
| 4 |  | NR Frequency band: 703-748 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand83_Supp |  | NR SUL FR1 Band n83 |  |
| 5 |  | NR Frequency band: 1920-1980 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand84_Supp |  | NR SUL FR1 Band n84 |  |
| 6 |  | NR Frequency band: 1710-1780 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-15 |  | pc_nrBand86_Supp |  | NR SUL FR1 Band n86 |  |
| 6a to 6b |  | Reserved |  |  |  |  |  |  |  |  |  |
| 6c |  | NR Frequency band: 824-849 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-16 |  | pc_nrBand89_Supp |  | NR SUL FR1 Band n89 |  |
| 7 |  | NR Frequency band: 2010-2025 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-16 |  | pc_nrBand95_Supp |  | NR SUL FR1 Band n95 |  |
| 8 |  | NR Frequency band: 2300 MHz – 2400 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-17 |  | pc_nrBand97_Supp |  | NR SUL FR1 Band n97 |  |
| 9 |  | NR Frequency band: 1626.5-1660.5 MHz (UL) |  | 38.101-1, 5.2 |  | Rel-17 |  | pc_nrBand99_Supp |  | NR SUL FR1 Band n99 |  |

Observation 2: Most of RAN5 works for these two R15 bands have been finished, including 38.521-1, 38.508-1 and 38.508-2.

It is a waste of the previous work if we remove all the contents in RAN5 specs for these two bands.

Proposal: It is proposed to treat n50 and n86 as legacy bands before PRD21 was created, mark them as interested operator = "Legacy" so that the status can be changed to "Ongoing" with responsible company assigned to CAICT.

Conclusion

Observation 1: The R15 NR FR1 bands n50 and n86 are still pending in PRD 21.

Observation 2: Most of RAN5 works for these two R15 bands have been finished, including 38.521-1, 38.508-1 and 38.508-2.

Proposal: It is proposed to treat n50 and n86 as legacy bands before PRD21 was created, mark them as interested operator = "Legacy" so that the status can be changed to "Ongoing" with responsible company assigned to CAICT.