3GPP TSG-RAN5 Meeting #96-e	R5-224414

# Online, , 15st Aug 2022 – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5413 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 7DL CA Sustained data rate performance test case 8.7.5.1.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | DEKRA Testing and Certification |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R15-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Rel-15 7DL CA Sustained data rate performance test cases needs to be added to 36.521-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the new test case of 8.7.5.1.6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 7DL CA Sustained data rate performance test cases will be still missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.5.1.6(new), 8.1.2.3B, F.1.4, F.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Beginning of changes >>

8.1.2	Applicability of requirements

<< Skip unchanged sections >>

8.1.2.3B	Applicability and test rules for different TDD-FDD CA configurations and bandwidth combination sets

Table 8.1.2.3B-2: Applicability and test rules for CA UE demodulation tests for TDD-FDD CA with 3 or more DL CCs

| Tests | CA capability where the tests apply | CA configuration from the selected CA capability where the tests apply | CA Bandwidth combination to be tested in priority order |
| --- | --- | --- | --- |
| CA tests with 3CCs, 4CCs, 5CCs, 6CCs in Clause 8.2.3.1.1.2, 8.2.3.1.1.3, 8.2.3.1.1.4, 8.2.3.1.1.5, 8.2.3.3.1.2, 8.2.3.3.1.3, 8.2.3.3.1.4, 8.2.3.3.1.5, 8.7.5.1.2, 8.7.5.1.3, 8.7.5.1.4, 8.7.5.1.5, 8.7.5.1.6, 8.7.5.1_H.2, 8.7.5.1_H.3, 8.7.5.1_H.4, 8.7.5.1_H.5 | Any one of the supported CA capabilities with largest aggregated CA bandwidth combination | Any one of the supported TDD-FDD CA configurations with FDD PCell with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 3CCs, 4CCs, 5CCs, 6CCs in Clause 8.2.3.2.1.2, 8.2.3.2.1.3, 8.2.3.2.1.4, 8.2.3.2.1.5 | Each supported CA capability | Any one of the supported TDD-FDD CA configurations with FDD PCell in each CA capability with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 3CCs, 4CCs, 5CCs, 6CCs in Clause 8.2.3.1.2.2, 8.2.3.1.2.3, 8.2.3.1.2.4, 8.2.3.1.2.5, 8.2.3.3.2.2, 8.2.3.3.2.3, 8.2.3.3.2.4, 8.2.3.3.2.5, 8.7.5.2.2, 8.7.5.2.3, 8.7.5.2.4, 8.7.5.2.5, 8.7.5.2.6, 8.7.5.2_H.2, 8.7.5.2_H.3, 8.7.5.2_H.4, 8.7.5.2_H.5 | Any one of the supported CA capabilities with largest aggregated CA bandwidth combination | Any one of the supported TDD-FDD CA configurations with TDD PCell with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| CA tests with 3CCs, 4CCs, 5CCs, 6CCs in Clause 8.2.3.2.2.2, 8.2.3.2.2.3, 8.2.3.2.2.4, 8.2.3.2.2.5 | Each supported CA capability | Any one of the supported TDD-FDD CA configurations with TDD PCell in each CA capability with largest aggregated CA bandwidth combination | Largest aggregated CA bandwidth combination |
| NOTE 1: The applicability and test rules are specified in this table, unless otherwise stated. NOTE 2: Number of the supported bandwidth combinations to be tested from each selected CA configuration is 1. NOTE 3: A single Uplink CC is configured for all tests. |  |  |  |

<< Skip unchanged sections >>

8.7.5.1.6	TDD FDD CA Sustained data rate performance for FDD PCell (7DL CA)

8.7.5.1.6.1	Test purpose

Same test purpose as 8.7.1.1.

8.7.5.1.6.2	Test applicability

This test applies to all types of UE Release 12 and forward of UE DL category 15 that support E-UTRA FDD and TDD, 7DL TDD-FDD CA with FDD as PCell and not support 256QAM in DL.

Note: This test also applies to UE supporting 4Rx antenna ports.

8.7.5.1.6.3	Minimum conformance requirements

For UE not supporting 256QAM, the requirements for TDD FDD CA with FDD PCell are specified in Table 8.7.5.1.6.3-1 with the additional parameters specified in Table 8.7.5-1, and the downlink physical channel setup according to Annex C.3.2. The test points are applied to UE category and bandwidth combination with maximum aggregated bandwidth as specified in Table 8.7.5.1.6.3-2. The TB success rate shall be sustained during at least 300 frames.

The applicability of the requirements are specified in Clause 8.1.2.3B. The test coverage for different number of component carriers is defined in 8.1.2.4.

Table 8.7.5.1.6.3-1: test parameters for sustained downlink data rate (TDD FDD CA 64QAM)

| Test number | Bandwidth (MHz) |  |  | Number of bits of a DL-SCH transport block received within a TTI (for normal/special subframe for TDD, except for subframe #5) |  | Measurement channel |  | Reference value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC | FDD CC | TDD CC | FDD CC | TDD CC | TB success rate [%] |
| 29 | 20+6x20 | 20 | 6x20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 30 | 2x20+5x20 | 2x20 | 5x20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |

Table 8.7.5.1.6.3-2: Test points for sustained data rate (FRC 64QAM)

| CA config | Maximum supported Bandwidth/ Bandwidth combination (MHz) |  |  | Cat. 1 | Cat. 2 | Cat. 3 | Cat. 4 | Cat. 6, 7 | Cat. 9,10 | Cat 11, 12 | DL Cat. 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |  |  | DL Cat. 6, 7 | DL Cat. 9, 10 | DL Cat. 11, 12 |  |
| CA with 7 CCs | 20+6x20 | 20 | 6x20 | - | - | - | - | - | - | 29 | 29 |
|  | 2x20+5x20 | 2x20 | 5x20 | - | - | - | - | - | - | 30 | 30 |
| Note 1: Void. Note 2: Void. Note 3: If DL category is signalled by the UE under test, then select the test point according to UE DL Category. Otherwise, select the test point according to the UE category signalled. |  |  |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.101[2] clause 8.7.5.1

8.7.5.1.6.4	Test description

8.7.5.1.6.4.1	Initial conditions

Table 8.7.5.1.6.4.1-1: Further test parameters per test

| Test number |  | 29 | 30 |
| --- | --- | --- | --- |
| Bandwidth | FDD CC | 20 | 2x20 |
|  | TDD CC | 6x20 | 5x20 |
| DL Measurement channel | FDD CC | R.31-4 FDD | R.31-4 FDD |
|  | TDD CC | R.31-4 TDD | R.31-4 TDD |
| UL channel | FDD CC | R.1-4 FDD | R.1-4 FDD |
|  | TDD CC | NA | NA |
| TBsize per Codeword | FDD CC | 75376 | 75376 |
|  | TDD CC | 75376/0 | 75376/0 |
| Number of PDCP SDU per Codeword | FDD CC | 7 | 7 |
|  | TDD CC | 7 | 7 |
| PDCP SDU size [Octets] Calculation (Note 1) | FDD CC | FLOOR((TBsize – 264)/56) | FLOOR((TBsize – 264)/56) |
|  | TDD CC | FLOOR((TBsize – 264)/56) | FLOOR((TBsize – 264)/56) |
| PDCP SDU size [Octets] | FDD CC | 1341 | 1341 |
|  | TDD CC | 1341 | 1341 |
| Note 1: Transport block size under test according to applicable Fixed Reference Channel for sustained data-rate test in annex A.3.9. In case of varying TBS across SFs of the RMC, only the maximum TBS is used for PDCP SDU size calculation. In case of different resulting PDCP SDU sizes among component carriers, the smaller calculated PDCP SDU size is used across all the carriers. Note 2: Each PDCP SDU is limited to 1500 octets (to keep below maximum SDU size of ESM as specified in TS 24.301 clause 9.9.4.12). The PDCP SDU size of each PDCP SDU is: PDCP SDU size = (TBsize – N*PDCP header size - AMD PDU header size - MAC header size - Size of RLC STATUS PDU) / N, where PDCP header size is 16 bits for the RLC AM and 12-bit SN case; AMD PDU header size is CEIL[(16+(N-1)*12)/8] bytes which includes 16 bit standard AM header and (N-1) Length indicators; and MAC header size = R/R/E/LCID/F/L MAC subheader (24 bits for MAC SDU for RLC STATUS PDU with 15 bit LI) + R/R/E/LCID MAC subheader (8 bits for MAC SDU for RLC Data PDU) = 32 bits. The size of RLC STATUS PDU including one ACK_SN field and one NACK_SN field is 32 bits (if no STATUS PDU is sent or if the size of the STATUS PDU is less than 32 bits then padding will be used to fill the 32 bits). This gives: PDCP SDU size = 8*FLOOR((TBsize – N*16- 8*CEIL((16+(N-1)*12)/8) – 64)/(8*N)) bits. The calculation of PDCP SDU sizes does not consider timing advance MAC CE as timing advance is not transmitted by SS for RF test cases, and the header sizes are informative and may vary during the test. |  |  |  |

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Test Environment: Normal as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: Mid Range as defined in TS 36.508 [7] clause 4.3.1.1.

Channel Bandwidths to be tested: According to Table 8.7.5.1.6.5-1 depending on the UE category according to Table 8.7.5.1.6.5-2.

1.	Connect the SS to the UE antenna connector(s) as shown in TS 36.508 [7] Annex A, Figure group A.60 (without using faders and AWGN generators) for UE supporting only 2Rx RF bands on all CC.  Annex A, Figure A.85 for UE supporting 4Rx RF band on any of the CC.

2.	The parameter settings for the cell are set up according to Table 8.7.5-1 and Table 8.7.5.1.6.5-1 depending on the UE category according to Table 8.7.5.1.6.5-2.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and Annex C.3.2 and uplink signals according to Annex H.1 and H.3.2.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in State 4A-RF according to TS 36.508 [7] clause 4.5.4. Message contents are defined in clause 8.7.5.1.6.4.3.

8.7.5.1.6.4.2	Test procedure

1.	Configure SCCs according to Annex C.0, C.1 and Annex C.3.2 for all downlink physical channels.

2.	The SS shall configure SCCs as per TS 36.508 [7] clause 5.2A.4 with SCCs configured. PhysicalConfigDedicated-DEFAULT is defined in Table 8.7.5.1.6.4.3-3, PhysicalConfigDedicatedSCell-r10-DEFAULT is defined in Table 8.7.5.1.6.4.3-3A.

3.	The SS activates SCCs by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	Propagation conditions are set according to Annex B.1 (No external noise sources are applied).

5.	The SS looks up TBsize in Table 8.7.5.1.6.4.1-1 for the tests to be performed depending on the UE category according to Table 8.7.5.1.6.5-2.

6.	The SS sets the counters, NDL_newtx ,NDL_retx, NUL_PDCP, and NDDL_PDCP to 0.

7.	For each new DL HARQ transmission the SS generates sufficient PDCP SDUs to fill up the TB in accordance with Table 8.7.5.1.6.4.1-1 (Note 1). The SS ciphers the PDCP SDUs, concatenates the resultant PDCP PDUs to form an RLC PDU and then a MAC PDU. The SS transmits the MAC PDU. The SS increments then NDL_newtx by one and NDL_PDCP by the number of new PDCP SDUs (Note 1) included in the sent MAC PDU.

8.	If PHY requests a DL HARQ retransmission, the SS performs a HARQ retransmission and increments NDL_retx by one.

9.	Steps 7 to 8 are repeated at every TTI for at least 300 frames and the SS waits for 300ms to let any HARQ retransmissions and RLC retransmissions to finish.

10.	For each PDCP SDU received at the SS, if the content of the data matches that of the truncated version of the original PDCP SDU generated at the SS, the SS increments NUL_PDCP by one

11.	The SS calculates the TB success rate as A = 100%*NDL_newtx/ (NDL_newtx + NDL_retx). Measurements are not based on UE feedback but on SS counters instead (NDL_newtx and NDL_retx) in order to validate HARQ feedback received from the UE.

12.	The SS calculates the PDCP SDU loss as B = NDL_PDCP - NUL_PDCP

13.	The UE passes the test if A ≥ "corresponding TB success rate according to Table 8.7.5.1.6.5-1" and B = 0.

NOTE 1:	In case of RLC PDU retransmission, the number of new required PDCP SDUs is as many as to fill the rest of TB.

8.7.5.1.6.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 and 4.7A, with the following exceptions:

Table 8.7.5.1.6.4.3-1: CLOSE UE TEST LOOP (in the preamble)

| Derivation Path: 36.508 clause 4.7A table 4.7A-3 with condition UE TEST LOOP MODE A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE test loop mode A LB setup |  |  |  |
| Length of UE test loop mode A LB setup list in bytes | 0 0 0 0 0 0 1 1 | Length of one LB setup DRB (3 bytes) |  |
| LB setup DRB | 0 0 0 0 0 0 0 0, 0 0 1 0 1 0 0 0, 0 0 0 Q4 Q3 Q2 Q1 Q0 | UL PDCP SDU size = 40 bits (5 bytes) Q4..Q0 = Data Radio Bearer identity number for the default radio bearer. See 36.509 clause 6.1 |  |

Table 8.7.5.1.6.4.3-2: SecurityModeCommand (in the preamble)

| Derivation Path: TS 36.508 clause 4.6.1 table 4.6.1-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityModeCommand ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| securityModeCommand-r8 SEQUENCE { |  |  |  |
| securityConfiguration SEQUENCE { |  |  |  |
| cipheringAlgorithm | eea2 |  |  |
| nextHopChainingCount | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.7.5.1.6.4.3-3: PhysicalConfigDedicated-DEFAULT

| Derivation Path: 36.508 clause 5.5.1, Table 5.5.1.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| antennaInfo CHOICE { |  |  |  |
| explicitValue ::= SEQUENCE { |  |  |  |
| transmissionMode | tm3 |  | Transmission mode 3 |
| codebookSubsetRestriction CHOICE { |  |  |  |
| n2TxAntenna-tm3 | 10 |  |  |
| } |  |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.7.5.1.6.4.3-3A: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: 36.508 clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| nonUL-Configuration-r10 SEQUENCE { |  |  |  |
| antennaInfo-r10 CHOICE { |  |  |  |
| transmissionMode-r10 | tm3 |  | Transmission mode 3 |
| codebookSubsetRestriction-r10 | 10 |  |  |
| ue-TransmitAntennaSelection CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.7.5.1.6.4.3-4: TDD-Config-DEFAULT

| Derivation Path: 36.508 clause 4.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  |  |  |
| subframeAssignment | sa1 |  |  |
| specialSubframePatterns | ssp4 |  |  |
| } |  |  |  |

8.7.5.1.6.5	Test Requirements

The requirements are specified in Table 8.7.5.1.6.5-1 depending on the UE category according to Table 8.7.5.1.6.5-2. The PDCP SDU success rate shall be sustained during at least 300 frames.

Table 8.7.5.1.6.5-1: Test requirements per test

| Test number | Bandwidth (MHz) |  |  | Number of bits of a DL-SCH transport block received within a TTI (for normal/special subframe for TDD, except for subframe #5) |  | Measurement channel |  | Reference value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC | FDD CC | TDD CC | FDD CC | TDD CC | TB success rate [%] |
| 29 | 20+6x20 | 20 | 6x20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |
| 30 | 2x20+5x20 | 2x20 | 5x20 | 75376 | 75376/0 | R.31-4 FDD | R.31-4 TDD | 85 |

Table 8.7.5.1.6.5-2: Test applicability per UE category

| CA config | Maximum supported Bandwidth/ Bandwidth combination (MHz) |  |  | Cat. 1 | Cat. 2 | Cat. 3 | Cat. 4 | Cat. 6, 7 | Cat. 9,10 | Cat 11, 12 | DL Cat. 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total | FDD CC | TDD CC |  |  |  |  | DL Cat. 6, 7 | DL Cat. 9, 10 | DL Cat. 11, 12 |  |
| CA with 7 CCs | 20+6x20 | 20 | 6x20 | - | - | - | - | - | - | 29 | 29 |
|  | 2x20+5x20 | 2x20 | 5x20 | - | - | - | - | - | - | 30 | 30 |
| Note 1: Only one test point is tested. Select the largest aggregated CA bandwidth combination supported by the UE among the UE supported CA capabilities. Note 2: The test point is selected according to the UE DL Category, or UE Category (only if UE DL Category not available) as appropriate. |  |  |  |  |  |  |  |  |  |  |  |

<< Skip unchanged sections >>

F.1.4	Measurement of performance requirements

Table F.1.4-1: Maximum Test System Uncertainty for Performance Requirements

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 8.2.1.1.1 Multiple PRBs - Propagation Condition EVA5 - Propagation Condition ETU70 - Propagation Condition ETU300 | ± 0.8 dB | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB |
| <<Too many rows skipped>> |  |  |
| 8.7.2.1_H.7 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 Calculation applies for each CC |
| 8.7.3.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.4.1 | Same as 8.7.3.1 | Same as 8.7.3.1 |
| 8.7.5.1.1 TDD FDD CA Sustained data rate performance for FDD PCell (2DL CA) | Same as 8.7.1.1 | Same as 8.7.1.1 |

| 8.7.5.1.2 TDD FDD CA Sustained data rate performance for FDD PCell (3DL CA) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| --- | --- | --- | --- | --- |
| 8.7.5.1.3 TDD FDD CA Sustained data rate performance for FDD PCell (4DL CA) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.5.1.4 TDD FDD CA Sustained data rate performance for FDD PCell (5DL CA) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.5.1.5 TDD FDD CA Sustained data rate performance for FDD PCell (6DL CA) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.5.1.6 TDD FDD CA Sustained data rate performance for FDD PCell (7DL CA) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.5.1_H.1 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (2DL CA) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.5.1_H.2 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (3DL CA) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.5.1_H.3 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (4DL CA) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.5.1_H.4 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (5DL CA) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.5.1_H.5 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (6DL CA) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.5.2.1 TDD FDD CA Sustained data rate performance for TDD PCell (2DL CA) | Same as 8.7.2.1 |  | Same as 8.7.2.1 |  |
| 8.7.5.2.2 TDD FDD CA Sustained data rate performance for TDD PCell (3DL CA) | Same as 8.7.2.1 |  | Same as 8.7.2.1 |  |
| 8.7.5.2.3 TDD FDD CA Sustained data rate performance for TDD PCell (4DL CA) | Same as 8.7.2.1 |  | Same as 8.7.2.1 |  |
| 8.7.5.2.4 TDD FDD CA Sustained data rate performance for TDD PCell (5DL CA) | Same as 8.7.2.1 |  | Same as 8.7.2.1 |  |
| 8.7.5.2.5 TDD FDD CA Sustained data rate performance for TDD PCell (6DL CA) | Same as 8.7.2.1 |  | Same as 8.7.2.1 |  |
| 8.7.5.2.6 TDD FDD CA Sustained data rate performance for TDD PCell (7DL CA) | Same as 8.7.2.1 |  | Same as 8.7.2.1 |  |
| 8.7.5.2_H.1 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (2DL CA) | Same as 8.7.2.1 |  | Same as 8.7.2.1 |  |
| 8.7.5.2_H.2 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (3DL CA) | Same as 8.7.2.1 |  | Same as 8.7.2.1 |  |
| 8.7.5.2_H.3 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (4DL CA) | Same as 8.7.2.1 |  | Same as 8.7.2.1 |  |
| 8.7.5.2_H.4 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (5DL CA) | Same as 8.7.2.1 |  | Same as 8.7.2.1 |  |
| 8.7.5.2_H.5 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (6DL CA) | Same as 8.7.2.1 |  | Same as 8.7.2.1 |  |
| 8.7.6.1 FDD sustained data rate performance for Dual Connectivity 64QAM | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.6.2 FDD sustained data rate performance for Dual Connectivity 256QAM | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.7.1 TDD sustained data rate performance for Dual Connectivity 64QAM | Same as 8.7.2.1 |  | Same as 8.7.2.1 |  |
| 8.7.7.2 TDD sustained data rate performance for Dual Connectivity 256QAM | Same as 8.7.2.1 |  | Same as 8.7.2.1 |  |
| 8.7.9.2 FDD sustained data rate performance for 4 layer MIMO (single carrier) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.9.3 FDD sustained data rate performance for 4 layer MIMO (2DL CA) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.9.4 FDD sustained data rate performance for 4 layer MIMO (3DL CA) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.9.5 FDD sustained data rate performance for 4 layer MIMO (4DL CA) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.9.6 FDD sustained data rate performance for 4 layer MIMO (5DL CA) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.10.2 TDD sustained data rate performance for 4 layer MIMO (single carrier) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.10.3 TDD sustained data rate performance for 4 layer MIMO (2DL CA) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.10.4 TDD sustained data rate performance for 4 layer MIMO (3DL CA) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.10.5 TDD sustained data rate performance for 4 layer MIMO (4DL CA) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.10.6 TDD sustained data rate performance for 4 layer MIMO (5DL CA) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.11.2 TDD FDD CA sustained data rate performance for 4 layer MIMO (2DL CA) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.11.3 TDD FDD CA sustained data rate performance for 4 layer MIMO (3DL CA) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.11.4 TDD FDD CA sustained data rate performance for 4 layer MIMO (4DL CA) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.11.5 TDD FDD CA sustained data rate performance for 4 layer MIMO (5DL CA) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.12.1.2.1 FDD as Pcell with 2DL CA (2RX) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.12.1.2.2 FDD as Pcell with 2DL CA (4 layer MIMO) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.12.2.2.1 LAA sustained data rate performance with TDD PCell with 2DL CA (2Rx) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.7.12.2.2.2 LAA sustained data rate performance with TDD PCell for 4 layer MIMO (2DL CA) | Same as 8.7.1.1 |  | Same as 8.7.1.1 |  |
| 8.8.1.1 | ± 0.9 dB |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.2 dB |  |
| 8.8.1.2 | Same as 8.8.1.1 |  | Same as 8.8.1.1 |  |
| 8.8.2.1 | Same as 8.8.1.1 |  | Same as 8.8.1.1 |  |
| 8.8.2.2 | Same as 8.8.1.1 |  | Same as 8.8.1.1 |  |
| 8.8.3.1 | Same as 8.8.1.1 |  | Same as 8.8.1.1 |  |
| 8.8.3.2 | Same as 8.8.1.1 |  | Same as 8.8.1.1 |  |
| 8.8.4.1 | Same as 8.2.1.2.5 |  | Same as 8.2.1.2.5 |  |
| 8.8.4.2 | Same as 8.2.1.2.5 |  | Same as 8.2.1.2.5 |  |
| 8.8.5.1 | Same as 8.2.1.2.5 |  | Same as 8.2.1.2.5 |  |
| 8.8.6.1 | Same as 8.2.1.2.5 |  | Same as 8.2.1.2.5 |  |
| 8.9.1.1.1 | ± 0.8 dB |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB |  |
| 8.9.1.1.2 | Same as 8.9.1.1.1 |  | Same as 8.9.1.1.1 |  |
| 8.9.1.1.3 | Same as 8.9.1.1.1 |  | Same as 8.9.1.1.1 |  |
| 8.9.1.1.3_1 | Same as 8.9.1.1.1 |  | Same as 8.9.1.1.,1 |  |
| 8.9.1.1.1_1 FDD PDSCH Transmit Diversity 2x1 for UE category 1bis | ± 0.8 dB |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB |  |
| 8.9.1.1.2_1 FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x1 for UE Category 1bis | Same as 8.9.1.1.1_1 |  | Same as 8.9.1.1.1_1 |  |
| 8.9.1.2.1 | Same as 8.9.1.1.1 |  | Same as 8.9.1.1.1 |  |
| 8.9.1.2.1_1 TDD PDSCH Transmit Diversity 2x1 for UE category 1bis | ± 0.8 dB |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB |  |
| 8.9.1.2.2 | Same as 8.9.1.1.1 |  | Same as 8.9.1.1.1 |  |
| 8.9.1.2.3 | Same as 8.9.1.1.1 |  | Same as 8.9.1.1.1 |  |
| 8.9.1.2.3_1 | Same as 8.9.1.1.1 |  | Same as 8.9.1.1.1 |  |
| 8.9.1.2.2_1 TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x1 for UE Category 1bis | Same as 8.9.1.1.1_1 |  | Same as 8.9.1.1.1_1 |  |
| 8.9.2.1.1 | ± 0.9 dB |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.4 dB |  |
| 8.9.2.1.1_1 | Same as 8.9.2.1.1 |  | Same as 8.9.2.1.1 |  |
| 8.9.2.2.1 | ± 0.9 dB |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.4 dB |  |
| 8.9.2.2.1_1 | Same as 8.9.2.2.1 |  | Same as 8.9.2.2.1 |  |
| 8.10.1.1.1 | ± 0.9 dB |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB |  |
| 8.10.1.1.2 | Same as 8.10.1.1.1 |  | Same as 8.10.1.1.1 |  |
| 8.10.1.1.3 | Ês/Noc ± 0.8 dB for each cell |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |
| 8.10.1.1.4 | Same as 8.10.1.1.1 |  | Same as 8.10.1.1.1 |  |
| 8.10.1.1.5 | Ês/Noc ± 0.8 dB for each cell |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |
| 8.10.1.1.6 | Same as 8.3.1.2.1_D_1 |  | Same as 8.3.1.2.1_D_1 |  |
| 8.10.1.1.7 FDD Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports | Same as 8.2.1.3.1 |  | Same as 8.2.1.3.1 |  |
| 8.10.1.1.8 FDD Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas | Same as 8.2.1.3.1 |  | Same as 8.2.1.3.1 |  |
| 8.10.1.1.9 FDD 4 Layer Spatial Multiplexing (User-Specific Reference Symbols) | Same as 8.2.1.3.1 |  | Same as 8.2.1.3.1 |  |
| 8.10.1.2.1 | Same as 8.10.1.1.1 |  | Same as 8.10.1.1.1 |  |
| 8.10.1.2.2 | Same as 8.10.1.1.1 |  | Same as 8.10.1.1.1 |  |
| 8.10.1.2.3 | Ês/Noc ± 0.8 dB for each cell |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2 Tx AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |
| 8.10.1.2.4 | Same as 8.10.1.1.1 |  | Same as 8.10.1.1.1 |  |
| 8.10.1.2.5 | Ês/Noc ± 0.8 dB for each cell |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2 Tx AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |
| 8.10.1.2.6 | Same as 8.3.2.2.1_D_1 |  | Same as 8.3.2.2.1_D_1 |  |
| 8.10.1.2.7 TDD Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports | Same as 8.2.2.3.1 |  | Same as 8.2.2.3.1 |  |
| 8.10.1.2.8 TDD Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas | Same as 8.2.2.3.1 |  | Same as 8.2.2.3.1 |  |
| 8.10.1.2.9 TDD 4 Layer Spatial Multiplexing (User-Specific Reference Symbols) | Same as 8.2.2.3.1 |  | Same as 8.2.2.3.1 |  |
| 8.10.2.1.1 FDD PCFICH/PDCCH Single-antenna Port Performance 1x4 | Same as 8.4.1.1 |  |  |  |
| 8.10.2.1.2 FDD PCFICH/PDCCH Transmit Diversity 2x4 | Same as 8.4.1.2.1 |  |  |  |
| 8.10.2.1.3 FDD PCFICH/PDCCH Transmit Diversity 4x4 | Same as 8.4.1.2.1 |  |  |  |
| 8.10.2.2.1 TDD PCFICH/PDCCH Single-antenna Port Performance 1x4 | Same as 8.4.2.1 |  |  |  |
| 8.10.2.2.2 TDD PCFICH/PDCCH Transmit Diversity 2x4 | Same as 8.4.2.2.1 |  |  |  |
| 8.10.2.2.3 TDD PCFICH/PDCCH Transmit Diversity 4x4 | Same as 8.4.2.2.1 |  |  |  |
| 8.10.3.1.1 FDD PHICH Single-antenna Port Performance 1x4 | Same as 8.5.1.1 |  |  |  |
| 8.10.3.1.2 FDD PHICH Transmit Diversity 2x4 | Same as 8.5.1.2.1 |  |  |  |
| 8.10.3.1.3 FDD PHICH Transmit Diversity 4x4 | Same as 8.5.1.2.2 |  |  |  |
| 8.10.3.2.1 TDD PHICH Single-antenna Port Performance 1x4 | Same as 8.5.2.1 |  |  |  |
| 8.10.3.2.2 TDD PHICH Transmit Diversity 2x4 | Same as 8.5.2.2.1 |  |  |  |
| 8.10.3.2.3 TDD PHICH Transmit Diversity 4x4 | Same as 8.5.2.2.2 |  |  |  |
| 8.10.4.1.1 | ± 0.9 dB |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.2 dB |  |
| 8.10.4.1.2 | Same as 8.10.4.1.1 |  | Same as 8.10.4.1.1 |  |
| 8.10.4.2.1 | Same as 8.10.4.1.1 |  | Same as 8.10.4.1.1 |  |
| 8.10.4.2.2 | Same as 8.10.4.1.1 |  | Same as 8.10.4.1.1 |  |
| 8.11.1.1.1 Closed-loop spatial multiplexing performance for UE supporting CE | ± 0.9 dB |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for 2x1 spatial multiplexing AWGN flatness and signal flatness ±2.0 dB |  |
| 8.11.1.1.2 FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE supporting CE | Same as 8.11.1.1.1 |  | Same as 8.11.1.1.1 |  |
| 8.11.1.1.3.1 FDD PDSCH Transmit Diversity 2x1 for UE category M1 | Same as 8.11.1.1.1 |  | Same as 8.11.1.1.1 |  |
| 8.11.1.1.3.1_1 FDD PDSCH Transmit Diversity 2x1 for UE category M1 (CEModeB) | Same as 8.11.1.1.1 |  | Same as 8.11.1.1.1 |  |
| 8.11.1.1.3.2 FDD and half-duplex FDD PDSCH Transmit Diversity 2x1 for UE category M2 | Same as 8.11.1.1.1 |  | Same as 8.11.1.1.1 |  |
| 8.11.1.1.3.2_1 FDD and half-duplex FDD PDSCH Transmit Diversity 2x1 for UE category M2 (CEModeB) | Same as 8.11.1.1.1 |  | Same as 8.11.1.1.1 |  |
| 8.11.1.2.1 TDD Closed-loop spatial multiplexing performance in CE Mode A (Cell-Specific Reference Symbols) | Same as 8.11.1.1.1 |  | Same as 8.11.1.1.1 |  |
| 8.11.1.2.2 TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 in CE Mode | Same as 8.11.1.1.1 |  | Same as 8.11.1.1.1 |  |
| 8.11.1.2.3.1 TDD PDSCH Transmit Diversity for UE category M1 | Same as 8.11.1.1.1 |  | Same as 8.11.1.1.1 |  |
| 8.11.1.2.3.1_1 TDD PDSCH Transmit Diversity for UE category M1 (CEModeB) | Same as 8.11.1.1.1 |  | Same as 8.11.1.1.1 |  |
| 8.11.1.2.3.2 TDD PDSCH 2 Tx Antenna Port supporting wideband transmission for UE category M2 | Same as 8.11.1.1.1 |  | Same as 8.11.1.1.1 |  |
| 8.11.1.2.3.2_1 TDD PDSCH 2 Tx Antenna Port supporting wideband transmission for UE category M2 (CEModeB) | Same as 8.11.1.1.1 |  | Same as 8.11.1.1.1 |  |
| 8.11.2.1.1 FDD and HD-FDD demodulation of MPDCCH in CE Mode A | ± 0.9 dB |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.2 dB |  |
| 8.11.2.1.2 FDD and half-duplex FDD demodulation of MPDCCH in CE Mode B | Same as 8.11.2.1.1 |  | Same as 8.11.2.1.1 |  |
| 8.11.2.2.1 TDD demodulation of MPDCCH in CE Mode A | ± 0.9 dB |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.2 dB |  |
| 8.11.2.2.2 TDD demodulation of MPDCCH in CE Mode B | Same as 8.11.2.2.1 |  | Same as 8.11.2.2.1 |  |
| 8.12.1.1.1 Demodulation of NPDSCH (Cell-Specific Reference Symbols) in In-band mode for category NB1 | ± 0.9 dB |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB |  |
| 8.12.1.1.2 Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone and Guard-band mode for category NB1 | ± 0.8 dB |  | Same as 8.2.1.1.1 |  |
| 8.12.1.1.3 Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone for NB2 | ± 0.8 dB |  | Same as 8.2.1.1.1 |  |
| 8.12.2.1.1 Demodulation of NPDCCH single-antenna performance for category NB1 | ± 0.8 dB |  | Same as 8.4.1.1 |  |
| 8.12.2.1.2 Demodulation of NPDCCH Transmit diversity performance for category NB1 | ± 0.9 dB |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.2 dB |  |
| 8.13.1.1.1.2 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.1.1.1.3 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.1.1.1.4 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.1.1.1.5 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.1.1.3 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.1.1.4 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.1.2.2 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.1.2.3 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.1.2.4 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.1.2.5 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.1.3.1 | Same as 8.10.1.1.3 for each CC |  | Same as 8.10.1.1.3 Calculation applies for each CC |  |
| 8.13.1.4.1 | Same as 8.10.1.1.5 for each CC |  | Same as 8.10.1.1.5 Calculation applies for each CC |  |
| 8.13.2.1.1.2 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.2.1.1.3 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.2.1.1.4 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.2.1.1.5 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.2.1.3 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.2.1.4 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.2.2.2 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.2.2.3 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.2.2.4 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.2.2.5 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.2.3.1 | Same as 8.10.1.2.3 for each CC |  | Same as 8.10.1.2.3 Calculation applies for each CC |  |
| 8.13.2.4.1 | Same as 8.10.1.2.5 for each CC |  | Same as 8.10.1.2.5 Calculation applies for each CC |  |
| 8.13.3.1.1.2 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.3.1.1.3 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.3.1.1.4 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.3.1.1.5 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.3.1.2.2 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.3.1.2.3 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.3.1.2.4 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.3.1.2.5 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.3.2.3 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.3.2.4 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.3.2.5 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.3.2.6 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.3.2.7 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.3.2.8 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.3.2.9 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.3.2.10 | Same as 8.2.1.4.2 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.3.3.1 | Ês/Noc ± 0.8 dB for each cell |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |
| 8.13.3.3.2 | Same as 8.13.3.3.1 |  | Same as 8.13.3.3.1 |  |
| 8.13.3.4.1 | Ês/Noc ± 0.8 dB for each cell |  | Overall system uncertainty for fading conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB not expected to have any significant effect |  |
| 8.13.3.4.2 | Same as 8.13.3.4.1 |  | Same as 8.13.3.4.1 |  |
| 8.13.3.6.1 | Same as 8.2.1.3.1 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.3.6.2 | Same as 8.2.2.3.1 for each CC |  | Same as 8.2.2.3.1 Calculation applies for each CC |  |
| 8.13.3.7.1 | Same as 8.2.1.3.1 for each CC |  | Same as 8.2.1.3.1 Calculation applies for each CC |  |
| 8.13.3.7.2 | Same as 8.2.2.3.1 for each CC |  | Same as 8.2.2.3.1 Calculation applies for each CC |  |
| 8.14.1.1.1 - Propagation Condition EVA30 | ± 0.9 dB |  | Same as 8.2.1.3.1 |  |
| 8.14.1.1.2 - Propagation Condition EPA5 | Same as 8.14.1.1.1 |  |  |  |
| 8.14.1.2.1 - Propagation Condition EVA30 | Same as 8.14.1.1.1 |  |  |  |
| 8.14.1.2.2 - Propagation Condition EPA5 | Same as 8.14.1.1.1 |  |  |  |
| 8.14.2.1 - Propagation Condition EVA30 - Propagation Condition EPA5 | ± 1.0 dB |  | Overall system uncertainty for fading conditions comprises four quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness 4. Result variation due to finite test time Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2 + variation due to finite test time 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB Result variation due to finite test time ±0.4 dB |  |
| 8.14.2.2 | Same as 8.14.2.1 |  |  |  |
| 10.1 | ± 0.9 dB |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.7 dB for Tx Diversity AWGN flatness and signal flatness ±2.0 dB |  |
| 10.2 | ± 0.9 dB |  | Same as 10.1 |  |
| 14.2 | ± 0.8 dB |  | Overall system uncertainty for fading conditions comprises three quantities: 1. Signal-to-noise ratio uncertainty 2. Fading profile power uncertainty 3. Effect of AWGN flatness and signal flatness Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + Fading profile power uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB Fading profile power uncertainty ±0.5 dB for single Tx AWGN flatness and signal flatness ±2.0 dB |  |
| 14.3 | ± 0.8 dB |  | Same as 14.2 |  |
| 14.4 | ± 0.6 dB |  | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±2.0 dB |  |
| 14.6 | ± 0.8 dB |  | Same as 14.2 |  |
| 14.7 | ± 0.6 dB |  | Same as 14.4 |  |
| 14.8 | Downlink absolute power uncertainty, averaged over BWConfig ±1.0 dB Downlink EVM ≤ 3% |  | 3% EVM is equivalent to a Test system downlink SNR of 30.5dB. The noise from the Test system is then sufficiently below that required for the UE to demodulate the signal with the required % success rate. Under these conditions the UE throughput is limited by the Reference measurement channel and the UE capability, and not by the Test system EVM. |  |
| 14.9 | E-UTRA: Downlink absolute power uncertainty, averaged over BWConfig ±1.0 dB Downlink EVM ≤ 3% Sidelink: ± 0.6 dB |  | E-UTRA: Same as 8.7.1.1 Sidelink: Same as 14.4 |  |
| [Other tests FFS] |  |  |  |  |
| In addition, the following Test System uncertainties and related constraints apply: |  |  |  |  |
| AWGN Bandwidth |  |  | ≥ 1.08MHz, 2.7MHz, 4.5MHz, 9MHz, 13.5MHz, 18MHz; NRB x 180kHz according to BWConfig |  |
| AWGN absolute power uncertainty, averaged over BWConfig Note 4 |  |  | ±3 dB |  |
| AWGN flatness and signal flatness, max deviation for any Resource Block, relative to average over BWConfig |  |  | ±2 dB |  |
| AWGN peak to average ratio |  |  | ≥10 dB @0.001% |  |
| Signal-to noise ratio uncertainty, averaged over downlink transmission Bandwidth |  |  | ±0.3 dB (includes uncertainty in precoding applied by the test system, where applicable) |  |
| Signal-to noise ratio variation for any resource block, relative to average over downlink transmission Bandwidth |  |  | ±0.5 dB |  |
| Noc2 absolute power uncertainty, averaged over BWConfig Note 5 |  |  | ±3 dB |  |
| Noc1 / Noc2 ratio uncertainty, averaged over BWConfig Note 5 |  |  | ±0.3 dB |  |
| Noc3 / Noc2 ratio uncertainty, averaged over BWConfig Note 5 |  |  | ±0.3 dB |  |
| Es / Noc2 ratio (SNR) uncertainty, averaged over downlink transmission Bandwidth Note 5 |  |  | ±0.3 dB (includes uncertainty in precoding applied by the test system, where applicable) |  |
| Fading profile power uncertainty |  |  | Test-specific |  |
| Fading profile delay uncertainty, relative to frame timing |  |  | ±5 ns (excludes absolute errors related to baseband timing) |  |
| CA, DC and LAA performance requirements only: Relative frequency error between carriers |  |  | 30Hz, measured over a 1ms period, and maximum carrier spacing 80MHz |  |
| enhanced performance requirement Type B only: Frequency error on the frequency offset to cell 1 |  |  | 30Hz, measured over a 1ms period |  |
| Note 1: Only the overall stimulus error is considered here. The effect of errors in the throughput measurements due to finite test duration is not considered. Note 2: The AWGN or N ocx parameters apply to all test cases except 8.7.1 and 8.7.2. The fading parameters apply to test cases using fading Note 3: In CA, DC and LAA test cases using multiple component carriers (CCs), the uncertainties and related constraints apply for each CC. Note 4: Applies for test cases which specify Noc, a single value that remains constant with time. Note 5: Applies for test cases which specify Noc1, Noc2 and Noc3, that are symbol or subframe specific. |  |  |  |  |

F.3.4	Measurement of performance requirements

Table F.3.4-1: Derivation of Test Requirements (performance tests)

| Test |  | Minimum Requirement in TS 36.101 | Test Tolerance (TT) | Test Requirement in TS 36.521-1 |
| --- | --- | --- | --- | --- |
| 8.2.1.1.1 Multiple PRBs - Prop’n Condition EVA5 - Prop’n Condition ETU70 - Prop’n Condition ETU300 |  | SNRs as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.1.1 Multiple PRBs - Prop’n Condition HST |  | SNR as specified | 0.6dB | Formula: SNR + TT T-put limit unchanged |
| 8.2.1.1.1 Single PRB - Prop’n Condition ETU70 |  | SNRs as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| <<Too many rows skipped>> |  |  |  |  |
| 8.7.1.1 FDD sustained data rate performance |  | Downlink power -85dBm/15kHz | 0 dB | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.1.1_1 |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.1.1_2 |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.1.1_A.1 |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.1.1_A.2 |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.1.1_A.4 |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.1.1_A.5 |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.1.1_A.7 |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.1.1_H.1 |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.1.1_H.2 |  | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.1.1_H.3 |  | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.1.1_H.4 |  | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.1.1_H.5 |  | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.1.1_H.7 |  | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.2.1 TDD sustained data rate performance |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.2.1_1 |  | Same as 8.7.2.1 | Same as 8.7.2.1 | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.2.1_2 |  | Same as 8.7.2.1 | Same as 8.7.2.1 | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.2.1_A.1 |  | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.2.1_A.2 |  | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.2.1_A.3 |  | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.2.1_A.5 |  | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Formula: Downlink power + TT T-put limit unchanged |  |
| 8.7.2.1_A.7 |  | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.2.1_H.1 |  | Same as 8.7.2.1 | Same as 8.7.2.1 | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.2.1_H.2 |  | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.2.1_H.3 |  | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.2.1_H.4 |  | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.2.1_H.5 |  | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.2.1_H.7 |  | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Formula: Downlink power + TT T-put limit unchanged |
| Haule13098.7.3.1 FDD sustained data rate performance for EPDCCH scheduling |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Formula: Downlink power + TT T-put limit unchanged |
| 8.7.4.1 TDD sustained data rate performance for EPDCCH scheduling |  | Same as 8.7.2.1 | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.7.5.1.1 TDD FDD CA Sustained data rate performance for FDD PCell (2DL CA) |  | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 |
| 8.7.5.1.2 TDD FDD CA Sustained data rate performance for FDD PCell (3DL CA) |  | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 |
| 8.7.5.1.3 TDD FDD CA Sustained data rate performance for FDD PCell (4DL CA) |  | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 |
| 8.7.5.1.4 TDD FDD CA Sustained data rate performance for FDD PCell (5DL CA) |  | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 |
| 8.7.5.1.5 TDD FDD CA Sustained data rate performance for FDD PCell (6DL CA) |  | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 |
| 8.7.5.1.6 TDD FDD CA Sustained data rate performance for FDD PCell (7DL CA) |  | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 |
| 8.7.5.1_H.1 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (2DL CA) |  | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 |
| 8.7.5.1_H.2 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (3DL CA) |  | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 |
| 8.7.5.1_H.3 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (4DL CA) |  | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 |
| 8.7.5.1_H.4 TDD FDD CA Sustained data rate performance for FDD PCell for SCE-L1 (5DL CA) |  | Same as 8.7.1.1 | Same as 8.7.1.1 for each CC | Same as 8.7.1.1 |
| 8.7.5.2.1 TDD FDD CA Sustained data rate performance for TDD PCell (2DL CA) |  | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 |
| 8.7.5.2.2 TDD FDD CA Sustained data rate performance for TDD PCell (3DL CA) |  | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 |
| 8.7.5.2.3 TDD FDD CA Sustained data rate performance for TDD PCell (4DL CA) |  | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 |
| 8.7.5.2.4 TDD FDD CA Sustained data rate performance for TDD PCell (5DL CA) |  | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 |
| 8.7.5.2.5 TDD FDD CA Sustained data rate performance for TDD PCell (6DL CA) |  | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 |
| 8.7.5.2.6 TDD FDD CA Sustained data rate performance for TDD PCell (7DL CA) |  | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 |
| 8.7.5.2_H.1 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (2DL CA) |  | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 |
| 8.7.5.2_H.2 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (3DL CA) |  | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 |
| 8.7.5.2_H.3 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (4DL CA) |  | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 |
| 8.7.5.2_H.4 TDD FDD CA Sustained data rate performance for TDD PCell for SCE-L1 (5DL CA) |  | Same as 8.7.2.1 | Same as 8.7.2.1 for each CC | Same as 8.7.2.1 |
| 8.7.6.1 FDD sustained data rate performance for Dual Connectivity 64QAM |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.6.2 FDD sustained data rate performance for Dual Connectivity 256QAM |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.7.1 TDD sustained data rate performance for Dual Connectivity 64QAM |  | Same as 8.7.2.1 | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.7.7.2 TDD sustained data rate performance for Dual Connectivity 256QAM |  | Same as 8.7.2.1 | Same as 8.7.2.1 | Same as 8.7.2.1 |
| 8.7.9.2 FDD sustained data rate performance for 4 layer MIMO (single carrier) |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.9.3 FDD sustained data rate performance for 4 layer MIMO (2DL CA) |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.9.4 FDD sustained data rate performance for 4 layer MIMO (3DL CA) |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.9.5 FDD sustained data rate performance for 4 layer MIMO (4DL CA) |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.9.6 FDD sustained data rate performance for 4 layer MIMO (5DL CA) |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.10.2 TDD sustained data rate performance for 4 layer MIMO (single carrier) |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.10.3 TDD sustained data rate performance for 4 layer MIMO (2DL CA) |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.10.4 TDD sustained data rate performance for 4 layer MIMO (3DL CA) |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.10.5 TDD sustained data rate performance for 4 layer MIMO (4DL CA) |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.10.6 TDD sustained data rate performance for 4 layer MIMO (5DL CA) |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.11.2 TDD FDD CA sustained data rate performance for 4 layer MIMO (2DL CA) |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.11.3 TDD FDD CA sustained data rate performance for 4 layer MIMO (3DL CA) |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.11.4 TDD FDD CA sustained data rate performance for 4 layer MIMO (4DL CA) |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.11.5 TDD FDD CA sustained data rate performance for 4 layer MIMO (5DL CA) |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.12.1.2.1 FDD as Pcell with 2DL CA (2RX) |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.12.1.2.2 FDD as Pcell with 2DL CA (4 layer MIMO) |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.12.2.2.1 LAA sustained data rate performance with TDD PCell with 2DL CA (2Rx) |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.7.12.2.2.2 LAA sustained data rate performance with TDD PCell for 4 layer MIMO (2DL CA) |  | Same as 8.7.1.1 | Same as 8.7.1.1 | Same as 8.7.1.1 |
| 8.8.1.1 FDD distributed EPDCCH performance |  | SNR as specified | 0.9dB | Formula: SNR + TT Pm-dsg limit unchanged |
| 8.8.1.2 TDD distributed EPDCCH performance |  | SNR as specified | 0.9dB | Formula: SNR + TT Pm-dsg limit unchanged |
| 8.8.2.1 FDD localized EPDCCH performance with TM9 |  | SNR as specified | 0.9dB | Formula: SNR + TT Pm-dsg limit unchanged |
| 8.8.2.2 TDD localized EPDCCH performance with TM9 |  | SNR as specified | 0.9dB | Formula: SNR + TT Pm-dsg limit unchanged |
| 8.8.3.1 FDD localized EPDCCH transmission with TM10 Type B quasi co-location type |  | SNR as specified | 0.9dB | Formula: SNR + TT Pm-dsg limit unchanged |
| 8.8.3.2 TDD localized EPDCCH transmission with TM10 Type B quasi co-location type |  | SNR as specified | 0.9dB | Formula: SNR + TT Pm-dsg limit unchanged |
| 8.8.4.1 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +13.40 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. | 0 dB +1.85 dB +0.8 dB +0.8 dB | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +15.25 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.8.4.2 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +14.20 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. | 0 dB +1.85 dB +0.8 dB +0.8 dB | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +16.05 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.8.5.1 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +14.80 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. | 0 dB +1.85 dB +0.8 dB +0.8 dB | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +16.65 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.8.6.1 |  | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +15.90 dB Cell 2 Ês/Noc: +13.91 dB Cell 3 Ês/Noc: +3.34 dB Cell 1 Ês/Noc is defined as SNR in minimum requirement. | 0 dB +1.85 dB +0.8 dB +0.8 dB | Noc: -98dBm/15kHz Cell 1 Ês/Noc: +17.75 dB Cell 2 Ês/Noc: +14.71 dB Cell 3 Ês/Noc: +4.14 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.9.1.1.1 FDD PDSCH Transmit Diversity 2x1 for UE category 0 |  | SNR as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.9.1.1.2 FDD Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols) for UE category 0 |  | SNR as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.9.1.1.3 FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 0 |  | SNR as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.9.1.1.3_1 FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 1bis |  | SNR as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.9.1.1.1_1 FDD PDSCH Transmit Diversity 2x1 for UE category 1bis |  | SNR as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.9.1.1.2_1 FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x1 for UE Category 1bis |  | SNR as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.9.1.2.1 TDD PDSCH Transmit Diversity for UE category 0 |  | SNR as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.9.1.2.1_1 TDD PDSCH Transmit Diversity 2x1 for UE category 1bis |  | SNR as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.9.1.2.2 TDD Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols) for UE category 0 |  | SNR as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.9.1.2.2_1 TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x1 for UE Category 1bis |  | SNR as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.9.1.2.3 TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 0 |  | SNR as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.9.1.2.3_1 TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 1bis |  | SNR as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.9.2.1.1 FDD PHICH transmit diversity for UE category 0 |  | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.9.2.1.1_1 FDD PHICH transmit diversity for UE category 1bis |  | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.9.2.2.1 TDD PHICH Transmit Diversity for UE category 0 |  | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.9.2.2.1_1 TDD PHICH transmit diversity for UE category 1bis |  | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.10.1.1.1 FDD PDSCH Transmit Diversity 2x4 |  | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.10.1.1.2 FDD PDSCH Open Loop Spatial Multiplexing 2x4 |  | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.10.1.1.3 FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model – Enhanced Performance Requirement Type A |  | Noc: -98dBm/15kHz DIP 1: -1.73dB SINR: -2.3dB | 0dB +0.28dB +0.98dB | Noc: -98dBm/15kHz DIP 1: -1.45dB SINR: -1.32dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.10.1.1.4 FDD PDSCH Closed Loop Spatial Multiplexing 4x4 |  | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.10.1.1.5 FDD PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 interference model – Enhanced Performance Requirement Type A |  | Noc: -98dBm/15kHz DIP 1: -1.73dB SINR: -3.0dB | 0 dB +0.28dB +0.98dB | Noc: -98dBm/15kHz DIP1: -1.45dB SINR: -2.03dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.10.1.1.6 FDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) |  | Noc: -98dBm/15kHz Ês1 / Noc: +9.2 dB Ês2 / Noc: +7.25 dB | 0 dB +0.8 dB -0.35 dB | Noc: -98dBm/15kHz Ês1 / Noc: +10.0 dB Ês2 / Noc: +6.9 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.10.1.1.7 FDD Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports |  | SNR as specified | 0.9dB | Formula: SNR + TT T-put limit unchanged |
| 8.10.1.1.8 FDD Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas |  | SNR as specified | 0.9dB | Formula: SNR + TT T-put limit unchanged |
| 8.10.1.1.9 FDD 4 Layer Spatial Multiplexing (User-Specific Reference Symbols) |  | SNR as specified | 0.9dB | Formula: SNR + TT T-put limit unchanged |
| 8.10.1.2.1 TDD PDSCH Transmit Diversity 2x4 |  | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.10.1.2.2 TDD PDSCH Open Loop Spatial Multiplexing 2x4 |  | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.10.1.2.3 TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model – Enhanced Performance Requirement Type A |  | Noc: -98dBm/15kHz DIP 1: -1.73dB SINR: -1.9dB | 0dB +0.28dB +0.98dB | Noc: -98dBm/15kHz DIP 1: -1.45dB SINR: -0.92dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.10.1.2.4 TDD PDSCH Closed Loop Spatial Multiplexing 4x4 |  | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.10.1.2.5 FDD PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 interference model – Enhanced Performance Requirement Type A |  | Noc: -98dBm/15kHz DIP 1: -1.73dB SINR: -3.3dB | 0 dB +0.28dB +0.98dB | Noc: -98dBm/15kHz DIP1: -1.45dB SINR: -2.32dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.10.1.2.6 TDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) |  | Noc: -98dBm/15kHz Ês1 / Noc: +9.5 dB Ês2 / Noc: +7.25 dB | 0 dB +0.8 dB -0.35 dB | Noc: -98dBm/15kHz Ês1 / Noc: +10.3 dB Ês2 / Noc: +6.9 dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.10.1.2.7 TDD Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports |  | SNR as specified | 0.9dB | Formula: SNR + TT T-put limit unchanged |
| 8.10.1.2.8 TDD Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas |  | SNR as specified | 0.9dB | Formula: SNR + TT T-put limit unchanged |
| 8.10.1.2.9 TDD 4 Layer Spatial Multiplexing (User-Specific Reference Symbols) |  | SNR as specified | 0.9dB | Formula: SNR + TT T-put limit unchanged |
| 8.10.2.1.1 FDD PCFICH/PDCCH Single-antenna Port Performance 1x4 |  | SNR as specified | 0.8 dB | Formula: SNR + TT T-put limit unchanged |
| 8.10.2.1.2 FDD PCFICH/PDCCH Transmit Diversity 2x4 |  | SNR as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |
| 8.10.2.1.3 FDD PCFICH/PDCCH Transmit Diversity 4x4 |  | SNR as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |
| 8.10.2.2.1 TDD PCFICH/PDCCH Single-antenna Port Performance 1x4 |  | SNR as specified | 0.8 dB | Formula: SNR + TT T-put limit unchanged |
| 8.10.2.2.2 TDD PCFICH/PDCCH Transmit Diversity 2x4 |  | SNR as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |
| 8.10.2.2.3 TDD PCFICH/PDCCH Transmit Diversity 4x4 |  | SNR as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |
| 8.10.3.1.1 FDD PHICH Single-antenna Port Performance 1x4 |  | SNR as specified | 0.9 dB | Formula: SNR + TT Pm-an limit unchanged |
| 8.10.3.1.2 FDD PHICH Transmit Diversity 2x4 |  | SNR as specified | 1.1 dB | Formula: SNR + TT Pm-an limit unchanged |
| 8.10.3.1.3 FDD PHICH Transmit Diversity 4x4 |  | SNR as specified | 1.0 dB | Formula: SNR + TT Pm-an limit unchanged |
| 8.10.3.2.1 TDD PHICH Single-antenna Port Performance 1x4 |  | SNR as specified | 0.9 dB | Formula: SNR + TT Pm-an limit unchanged |
| 8.10.3.2.2 TDD PHICH Transmit Diversity 2x4 |  | SNR as specified | 1.1 dB | Formula: SNR + TT Pm-an limit unchanged |
| 8.10.3.2.3 TDD PHICH Transmit Diversity 4x4 |  | SNR as specified | 1.0 dB | Formula: SNR + TT Pm-an limit unchanged |
| 8.10.4.1.1 FDD distributed E2PDCCH performance 2x4 |  | SNRs as specified | 0.9dB | Formula: SNR + TT Pm-dsg limit unchanged |
| 8.10.4.1.2 TDD distributed EPDCCH performance 2x4 |  | SNR as specified | 0.9dB | Formula: SNR + TT Pm-dsg limit unchanged |
| 8.10.4.2.1 FDD localized EPDCCH performance with TM9 2x4 |  | SNRs as specified | 0.9dB | Formula: SNR + TT Pm-dsg limit unchanged |
| 8.10.4.2.2 TDD localized EPDCCH performance with TM9 2x4 |  | SNRs as specified | 0.9dB | Formula: SNR + TT Pm-dsg limit unchanged |
| 8.11.1.1.1 Closed-loop spatial multiplexing performance for UE supporting CE |  | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.11.1.1.2 FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE supporting CE |  | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.11.1.1.3.1 FDD PDSCH Transmit Diversity 2x1 for UE category M1 |  | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.11.1.1.3.1_1 FDD PDSCH Transmit Diversity 2x1 for UE category M1 (CEModeB) |  | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.11.1.1.3.2 FDD and half-duplex FDD PDSCH Transmit Diversity 2x1 for UE category M2 |  | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.11.1.1.3.2_1 FDD and half-duplex FDD PDSCH Transmit Diversity 2x1 for UE category M2 (CEModeB) |  | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.11.1.2.1 TDD Closed-loop spatial multiplexing performance in CE Mode A (Cell-Specific Reference Symbols) |  | Same as 8.11.1.1.1 | Same as 8.11.1.1.1 | Same as 8.11.1.1.1 |
| 8.11.1.2.2 TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for in CE Mode A |  | SNRs as specified | 0.9dB | Formula: SNR + TT T-put limit unchanged |
| 8.11.1.2.3.1 TDD PDSCH Transmit Diversity for UE category M1 |  | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.11.1.2.3.1_1 TDD PDSCH Transmit Diversity for UE category M1 (CEModeB) |  | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.11.1.2.3.2 TDD PDSCH 2 Tx Antenna Port supporting wideband transmission for UE category M2 |  | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.11.1.2.3.2_1 TDD PDSCH 2 Tx Antenna Port supporting wideband transmission for UE category M2 (CEModeB) |  | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.11.2.1.1 FDD and HD-FDD demodulation of MPDCCH in CE Mode A |  | SNRs as specified | 0.9dB | Formula: SNR + TT Pm-dsg limit unchanged |
| 8.11.2.1.2 FDD and HD-FDD demodulation of MPDCCH in CE Mode B |  | SNRs as specified | 0.9dB | Formula: SNR + TT Pm-dsg limit unchanged |
| 8.11.2.2.1 TDD demodulation of MPDCCH in CE Mode A |  | SNRs as specified | 0.9dB | Formula: SNR + TT Pm-dsg limit unchanged |
| 8.11.2.2.2 TDD demodulation of MPDCCH in CE Mode B |  | SNRs as specified | 0.9dB | Formula: SNR + TT Pm-dsg limit unchanged |
| 8.12.1.1.1 Demodulation of NPDSCH (Cell-Specific Reference Symbols) in In-band mode for category NB1 |  | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.12.1.1.2 Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone and Guard-band mode for category NB1 |  | SNRs as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.12.1.1.3 Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone for NB2 |  | SNRs as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 8.12.2.1.1 Demodulation of NPDCCH single-antenna performance for category NB1 |  | SNRs as specified | 0.8dB | Formula: SNR + TT Pm-dsg limit unchanged |
| 8.12.2.1.2 Demodulation of NPDCCH Transmit diversity performance for category NB1 |  | SNR as specified | 0.9 dB | Formula: SNR + TT Pm-dsg limit unchanged |
| 8.13.1.1.1.2 | SNRs as specified |  | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.1.1.1.3 | SNRs as specified |  | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.1.1.1.4 | SNRs as specified |  | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.1.1.1.5 | SNRs as specified |  | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |  |
| 8.13.1.1.3 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.1.1.4 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.1.2.2 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.1.2.3 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.1.2.4 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.1.2.5 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.1.3.1 |  | Noc: -98dBm/15kHz DIP 1: -1.73dB SINR: -2.3dB | 0dB +0.28dB +0.98dB | Noc: -98dBm/15kHz DIP 1: -1.45dB SINR: -1.32dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.13.1.4.1 |  | Noc: -98dBm/15kHz DIP 1: -1.73dB SINR: -3.0dB | 0 dB +0.28dB +0.98dB | Noc: -98dBm/15kHz DIP1: -1.45dB SINR: -2.03dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.13.2.1.1.2 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.2.1.1.3 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.2.1.1.4 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.2.1.1.5 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.2.1.3 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.2.1.4 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.2.2.2 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.2.2.3 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.2.2.4 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.2.2.5 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.2.3.1 |  | Noc: -98dBm/15kHz DIP 1: -1.73dB SINR: -1.9dB | 0dB +0.28dB +0.98dB | Noc: -98dBm/15kHz DIP 1: -1.45dB SINR: -0.92dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.13.2.4.1 |  | Noc: -98dBm/15kHz DIP 1: -1.73dB SINR: -3.3dB | 0 dB +0.28dB +0.98dB | Noc: -98dBm/15kHz DIP1: -1.45dB SINR: -2.32dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.13.3.1.1.2 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.3.1.1.3 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.3.1.1.4 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.3.1.1.5 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.3.1.2.2 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.3.1.2.3 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.3.1.2.4 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.3.1.2.5 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.3.2.3 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.3.2.4 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.3.2.5 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.3.2.6 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.3.2.7 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.3.2.8 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.3.2.9 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.3.2.10 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.3.3.1 TDD-FDD CA PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model-Enhanced Performance Requirement Type A for FDD Pcell (2DL CA) |  | - Noc: -98dBm/15kHz - DIP 1: -1.73dB - FDD Cell 2Rx SINR: 5MHz: 0.8dB 10MHz: 1.0dB 15MHz: 0.8dB 20MHz: 1.1dB - FDD Cell 4Rx SINR: 5MHz: -3.0dB 10MHz: -2.7dB 15MHz: -2.9dB 20MHz: -2.8dB - TDD Cell 2Rx SINR: 5MHz: 0.8dB 10MHz: 1.3dB 15MHz: 1.0 dB 20MHz: 1.3dB - TDD Cell 4Rx SINR: 5MHz: -2.8dB 10MHz: -2.5dB 15MHz: -2.7dB 20MHz: -2.3dB | 0dB +0.28dB +0.98dB +0.98dB +0.98dB +0.98dB | Noc: -98dBm/15kHz DIP 1: -1.45dB - FDD Cell 2Rx SINR: 5MHz: 1.78dB 10MHz: 1.98dB 15MHz: 1.78dB 20MHz: 2.08dB - FDD Cell 4Rx SINR: 5MHz: 2.02dB 10MHz: 1.72dB 15MHz: 1.92dB 20MHz: 1.82dB - TDD Cell 2Rx SINR: 5MHz: 1.78dB 10MHz: 2.28dB 15MHz: 1.98dB 20MHz: 2.28dB - TDD Cell 4Rx SINR: 5MHz: 1.82dB 10MHz: 1.52dB 15MHz: 1.72dB 20MHz: 1.32dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.13.3.3.2 TDD-FDD CA PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model-Enhanced Performance Requirement Type A for TDD Pcell (2DL CA) |  | Same as 8.13.3.3.1 | Same as 8.13.3.3.1 | Same as 8.13.3.3.1 |
| 8.13.3.4.1 TDD-FDD CA PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model-Enhanced Performance Requirement Type A for FDD Pcell (2DL CA) |  | - Noc: -98dBm/15kHz - DIP 1: -1.73dB - FDD Cell 2Rx SINR: 5MHz: -0.4dB 10MHz: 0.1dB 15MHz: -0.2dB 20MHz: -0.4dB - FDD Cell 4Rx SINR: 5MHz: -2.0dB 10MHz: -1.8dB 15MHz: -1.9dB 20MHz: -1.7dB - TDD Cell 2Rx SINR: 5MHz: -0.6dB 10MHz: -0.1dB 15MHz: -0.2dB 20MHz: -0.4dB - TDD Cell 4Rx SINR: 5MHz: -2.4dB 10MHz: -2.1dB 15MHz: -2.1dB 20MHz: -2.1dB | 0dB +0.28dB +0.98dB +0.98dB +0.98dB +0.98dB | Noc: -98dBm/15kHz DIP 1: -1.45dB - FDD Cell 2Rx SINR: 5MHz: 0.58dB 10MHz: 1.08dB 15MHz: 0.78dB 20MHz: 0.58dB - FDD Cell 4Rx SINR: 5MHz: -1.02dB 10MHz: -0.82dB 15MHz: -0.92dB 20MHz: -0.72dB - TDD Cell 2Rx SINR: 5MHz: 0.38dB 10MHz: 0.88dB 15MHz: 0.78dB 20MHz: 0.58dB - TDD Cell 4Rx SINR: 5MHz: -1.42dB 10MHz: -1.12dB 15MHz: -1.12dB 20MHz: -1.12dB T-put limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |
| 8.13.3.4.2 TDD-FDD CA PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model-Enhanced Performance Requirement Type A for TDD Pcell (2DL CA) |  | Same as 8.13.3.4.1 | Same as 8.13.3.4.1 | Same as 8.13.3.4.1 |
| 8.13.3.6.1 |  | SNR as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.3.6.2 |  | SNR as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.3.7.1 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.13.3.7.2 |  | SNRs as specified | 0.9 dB for each CC | Formula: SNR + TT T-put limit unchanged |
| 8.14.1.1.1 |  | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.14.1.1.2 |  | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.14.1.2.1 |  | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.14.1.2.2 |  | SNR as specified | 0.9 dB | Formula: SNR + TT T-put limit unchanged |
| 8.14.2.1 |  | SNR as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |
| 8.14.2.2 |  | SNR as specified | 1.0 dB | Formula: SNR + TT T-put limit unchanged |
| 10.1 |  | SNR as specified | 0.9dB | Formula: SNR + TT T-put limit unchanged |
| 10.2 |  | SNR as specified | 0.9dB | Formula: SNR + TT T-put limit unchanged |
| 14.2 |  | SNRs as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 14.3 |  | SNRs as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 14.4 |  | SNRs as specified | 0.6dB | Formula: SNR + TT T-put limit unchanged |
| 14.6 |  | SNRs as specified | 0.8dB | Formula: SNR + TT T-put limit unchanged |
| 14.7 |  | SNRs as specified | 0.6dB | Formula: SNR + TT T-put limit unchanged |
| 14.8 |  | Downlink power -85dBm/15kHz | 0 dB | Formula: Downlink power + TT T-put limit unchanged |
| 14.9 |  | SNRs as specified | E-UTRA: 0dB Sidelink: 0.6dB | E-UTRA: Same as 8.7.1.1 Sidelink: Same as 14.7 |
| [Other tests FFS] |  |  |  |  |

| Test |  | Minimum Requirement in TS 36.101 |  | Test Tolerance (TT) |  | Test Requirement in TS 36.521-1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 9.2.1.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.1.2 TDD CQI Reporting under AWGN condit.1ions – PUCCH 1-0 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.1.3_C.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for eICIC (non-MBSFN ABS) |  | Values as specified Limits as in the Test Procedure |  | No test tolerances applied |  | The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.2.1.4_C.1 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 for eICIC (non-MBSFN ABS) |  | Values as specified Limits as in the Test Procedure |  | No test tolerances applied |  | The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.2.1.5_E.1 FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for feICIC (non-MBSFN ABS) |  | Values as specified Limits as in the Test Procedure |  | No test tolerances applied |  | The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.2.1.6_E.1 TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for feICIC (non-MBSFN ABS) |  | Values as specified Limits as in the Test Procedure |  | No test tolerances applied |  | The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.2.1.7 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for 256QAM in DL |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.1.8 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 for 256QAM in DL |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.2.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-1 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.2.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-1 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.3.1_D FDD CQI Reporting under AWGN conditions – PUCCH 1-1 for eDL-MIMO |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.3.2_D TDD CQI Reporting under AWGN conditions – PUCCH 1-1 for eDL-MIMO |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.3.1A FDD CQI Reporting under AWGN conditions – PUCCH 1-1 for FD-MIMO (With channelMeasRestriction configured) |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.3.2A TDD CQI Reporting under AWGN conditions – PUCCH 1-1 for FD-MIMO (With channelMeasRestriction configured) |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.4.1A FDD CQI Reporting under AWGN conditions - Single CSI Process With interferenceMeasRestriction configured |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.4.1_F FDD CQI Reporting under AWGN conditions - Single CSI Process for CoMP |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.4.2A TDD CQI Reporting under AWGN conditions - Single CSI Process With interferenceMeasRestriction configured |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.4.2_F TDD CQI Reporting under AWGN conditions - Single CSI Process for CoMP |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.5 TDD CQI Reporting under AWGN conditions – PUCCH 1-1 (when csi-SubframeSet – r12 and EIMTA-MainConfigServCell-r12 are configured) |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.6.1 LAA CQI Reporting under AWGN Conditions with Frame Structure Type 3 with FDD as Pcell (PUSCH 3-0) |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.6.2 LAA CQI Reporting under AWGN Conditions with Frame Structure Type 3 with TDD as Pcell (PUSCH 3-0) |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.7.1 LAA CQI Reporting under AWGN Conditions with Frame Structure Type 3 with FDD as Pcell (PUSCH 3-1) |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.2.7.2 LAA CQI Reporting under AWGN Conditions with Frame Structure Type 3 with TDD as Pcell (PUSCH 3-1) |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.3.1.1.1 FDD CQI Reporting under fading conditions – PUSCH 3-0 |  | SNRs as specified 2% 55% 1.10 BLER 0.05 |  | SNR 0dB 0% 0% 0.01 BLER 0 |  | SNR unchanged  limit unchanged limit unchanged  1.09 as per Table G.5.4-1 BLER limit unchanged |  |
| 9.3.1.1.2 TDD CQI Reporting under fading conditions – PUSCH 3-0 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  |
| 9.3.1.1.5 TDD CQI Reporting under fading conditions – PUCCH 3-0 (when csi-SubframeSet –r12 is configured) |  | SNRs as specified 2% 55% 1.10 BLER 0.05~0.60 Delta CQI 3 |  | SNR 0dB 0% 0% 0.01 BLER 0 Delta CQI 0 |  | SNR unchanged  limit unchanged limit unchanged  1.09 as per Table G.5.4-1 BLER limit unchanged Delta CQI limit unchanged |  |
| 9.3.1.2.1_D FDD CQI Reporting under fading conditions – PUSCH 3-1 for eDL-MIMO |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  |
| 9.3.1.2.2_D TDD CQI Reporting under fading conditions – PUSCH 3-1 for eDL-MIMO |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  |
| 9.3.1.2.3 FDD CQI Reporting under fading conditions – PUSCH 3-1 for 256QAM in DL |  | SNRs as specified 2% 40 1.10 BLER 0.05 |  | SNR 0dB 0% 0% 0.01 BLER 0 |  | SNR unchanged  limit unchanged limit unchanged  1.09 as per Table G.5.4-1 BLER limit unchanged |  |
| 9.3.1.2.4 TDD CQI Reporting under fading conditions – PUSCH 3-1 for 256QAM in DL |  | Same as 9.3.1.2.3 |  | Same as 9.3.1.2.3 |  | Same as 9.3.1.2.3 |  |
| 9.3.1.2.6 TDD CQI Reporting under fading conditions – PUCCH 3-1 (when csi-SubframeSet –r12 is configured with one CSI process) |  | SNRs as specified 2% 55% 1.02 BLER 0.01 Delta CQI 3 |  | SNR 0dB 0% 0% 0.01 BLER 0 Delta CQI 0 |  | SNR unchanged  limit unchanged limit unchanged  1.01 as per Table G.5.4-1 BLER limit unchanged Delta CQI limit unchanged |  |
| 9.3.1.3.1_E.1 FDD CQI Reporting under fading conditions - PUSCH 3-0 for feICIC (non-MBSFN ABS) |  | SNRs as specified 2% 55% 1.10 BLER 0.01 |  | SNR 0dB 0% 0% 0.01 BLER 0 |  | SNR unchanged  limit unchanged limit unchanged  1.09 as per Table G.5.4-1 BLER limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.3.1.3.2_E.1 TDD CQI Reporting under fading conditions - PUSCH 3-0 for feICIC (non-MBSFN ABS) |  | Same as 9.3.1.3.1_E.1 |  | Same as 9.3.1.3.1_E.1 |  | Same as 9.3.1.3.1_E.1 |  |
| 9.3.2.1.1 FDD CQI Reporting under fading conditions – PUCCH 1-0 |  | SNRs as specified 20% 1.05 BLER 0.02 |  | SNR 0dB 0% 0.01 BLER 0 |  | SNR and BLER limits unchanged  limit unchanged  1.04 as per Table G.5.4-1 BLER limit unchanged |  |
| 9.3.2.1.1_1 FDD CQI Reporting under fading conditions – PUCCH 1-0 |  | Same as 9.3.2.1.1 |  | Same as 9.3.2.1.1 |  | Same as 9.3.2.1.1 |  |
| 9.3.2.1.2 TDD CQI Reporting under fading conditions – PUCCH 1-0 |  | Same as 9.3.2.1.1 |  | Same as 9.3.2.1.1 |  | Same as 9.3.2.1.1 |  |
| 9.3.2.1.2_1 TDD CQI Reporting under fading conditions – PUCCH 1-0 |  | Same as 9.3.2.1.1 |  | Same as 9.3.2.1.1 |  | Same as 9.3.2.1.1 |  |
| 9.3.2.2.1_D FDD CQI Reporting under fading conditions – PUCCH 1-1 for eDL-MIMO |  | Same as 9.3.2.1.1 |  | Same as 9.3.2.1.1 |  | Same as 9.3.2.1.1 |  |
| 9.3.2.2.2_D TDD CQI Reporting under fading conditions – PUCCH 1-1 for eDL-MIMO |  | Same as 9.3.2.1.1 |  | Same as 9.3.2.1.1 |  | Same as 9.3.2.1.1 |  |
| 9.3.3.1.1 FDD CQI Reporting under fading conditions and frequency-selective interference – PUSCH 3-0 |  | Ior and Iot as specified 60% 1.60 |  | Ior, Iot 0dB 20% 0.10 |  | Ior and Iot unchanged  40%, Formula: Min Req’t – Test Tol  1.50, Formula: Min Req’t – Test Tol The effect of AWGN flatness and signal flatness on the requirement was derived by simulation. AWGN flatness / signal flatness and the statistical effect of a finite test time both affect the T-put result. The Test Tolerance comprises two quantities: 1. Effect of AWGN flatness and signal flatness, derived by simulation 2. Statistical effect as per Table G.5.4-1 Items 1 and 2 are assumed to be uncorrelated so can be root sum squared T-put requirement Test Tol = SQRT (AWGN flatness and signal flatness effect 2 + Statistical effect 2) AWGN flatness and signal flatness effect 0.1, Statistical effect 0.01, giving overall effect 0.10 |  |
| 9.3.3.1.2 TDD CQI Reporting under fading conditions and frequency-selective interference – PUSCH 3-0 |  | Same as 9.3.3.1.1 |  | Same as 9.3.3.1.1 |  | Same as 9.3.3.1.1 |  |
| 9.3.4.1.1 FDD CQI Reporting under fading conditions – PUSCH 2-0 |  | SNRs as specified Limits as in the Test Procedure 1.2 |  | 0.01 |  | SNR unchanged  1.19 as per Table G.5.4-1 |  |
| 9.3.4.1.2 TDD CQI Reporting under fading conditions – PUSCH 2-0 |  | Same as 9.3.4.1.1 |  | Same as 9.3.4.1.1 |  | Same as 9.3.4.1.1 |  |
| 9.3.4.2.1 FDD CQI Reporting under fading conditions – PUCCH 2-0 |  | SNRs as specified Limits as in the Test Procedure 1.15 |  | 0.01 |  | SNR unchanged  1.14 as per Table G.5.4-1 |  |
| 9.3.4.2.2 TDD CQI Reporting under fading conditions – PUCCH 2-0 |  | Same as 9.3.4.2.1 |  | Same as 9.3.4.2.1 |  | Same as 9.3.4.2.1 |  |
| 9.3.5.1.1 FDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A |  | Noc: -98dBm/15kHz DIP 1: -0.41dB SINR: -2.00dB 1.8 BLER 0.02 |  | 0dB +0.03dB +0.03dB 0.01 |  | Noc: -98dBm/15kHz DIP 1: -0.38dB SINR: -1.97dB 1.79 as per Table G.5.4-1 BLER limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.3.5.1.2 TDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A |  | Same as 9.3.5.1.1 |  | Same as 9.3.5.1.1 |  | Same as 9.3.5.1.1 |  |
| 9.3.5.2.1 FDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A |  | Same as 9.3.5.1.1 Same as 9.3.5.1.1 |  | Same as 9.3.5.1.1 |  | Same as 9.3.5.1.1 |  |
| 9.3.5.2.2 TDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A |  | Same as 9.3.5.1.1 |  | Same as 9.3.5.1.1 |  |  |  |
| 9.3.6.1_F FDD CQI Reporting under fading conditions Single CSI processes for CoMP |  | 1.02 |  | 0.01 |  |  1.01 as per Table G.5.4-1 |  |
| 9.3.6.1_F.2 FDD CQI Reporting under fading conditions Three CSI processes for CoMP |  | Same as 9.3.6.1_F.1 |  | Same as 9.3.6.1_F.1 |  | Same as 9.3.6.1_F.1 |  |
| 9.3.6.1_F.3 FDD CQI Reporting under fading conditions Four CSI processes for CoMP |  | Same as 9.3.6.1_F.1 |  | Same as 9.3.6.1_F.1 |  | Same as 9.3.6.1_F.1 |  |
| 9.3.6.2_F.1 TDD CQI Reporting under fading conditions Single CSI processes for CoMP |  | 1.02 |  | 0.01 |  |  1.01 as per Table G.5.4-1 |  |
| 9.3.6.2_F.2 TDD CQI Reporting under fading conditions Three CSI processes for CoMP |  | Same as 9.3.6.2_F.1 |  | Same as 9.3.6.2_F.1 |  | Same as 9.3.6.2_F.1 |  |
| 9.3.6.2_F.3 TDD CQI Reporting under fading conditions Four CSI processes for CoMP |  | Same as 9.3.6.2_F.1 |  | Same as 9.3.6.2_F.1 |  | Same as 9.3.6.2_F.1 |  |
| 9.3.7.1 FDD CQI Reporting under fading conditions - PUSCH 3-2 for eDL MIMO Enhancement |  | SNRs as specified Limits as in the Test Procedure Noc: -98dBm/15kHz 1.05 1.15 |  | 0dB 0.01 0.01 |  | SNR unchanged Noc: -98dBm/15kHz 1.04 as per Table G.5.4-1 1.14 as per Table G.5.4-1 The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.3.7.2 TDD CQI Reporting under fading conditions - PUSCH 3-2 for eDL MIMO Enhancement |  | Same as 9.3.7.1 |  | Same as 9.3.7.1 |  | Same as 9.3.7.1 |  |
| 9.3.8.1.1 |  | Noc: -98dBm/15kHz SNR: +8.34dB Cell 2 Ês/Noc: +3.28 dB Cell 3 Ês/Noc: +0.74 dB 0.925 |  | 0 dB +0.63 dB +0.8 dB +0.8 dB 0.01 |  | Noc: -98dBm/15kHz SNR: +8.97dB Cell 2 Ês/Noc: +4.08 dB Cell 3 Ês/Noc: +1.54 dB 0.915 as per Table G.5.4-1 The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.3.8.1.2 |  | Same as 9.3.8.1.1 |  | Same as 9.3.8.1.1 |  | Same as 9.3.8.1.1 |  |
| 9.3.8.2.1 |  | Noc: -98dBm/15kHz SNR: +8.34dB Cell 2 Ês/Noc: +3.28 dB Cell 3 Ês/Noc: +0.74 dB 0.925 |  | 0 dB +0.63 dB +0.8 dB +0.8 dB 0.01 |  | Noc: -98dBm/15kHz SNR: +8.97dB Cell 2 Ês/Noc: +4.08 dB Cell 3 Ês/Noc: +1.54 dB 0.915 as per Table G.5.4-1 The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.3.8.2.2 |  | Same as 9.3.8.2.1 |  | Same as 9.3.8.2.1 |  | Same as 9.3.8.2.1 |  |
| 9.3.8.3.1 FDD CQI Reporting under fading conditions - PUCCH 1-1 (CSI Reference Symbol) TM10 with TM9 interference - Enhanced Receiver Type B |  | Noc: -98dBm/15kHz SNR: +8.34dB Cell 2 Ês/Noc: +3.28 dB Cell 3 Ês/Noc: +0.74 dB 0.925 |  | 0 dB +0.63 dB +0.8 dB +0.8 dB 0.01 |  | Noc: -98dBm/15kHz SNR: +8.97dB Cell 2 Ês/Noc: +4.08 dB Cell 3 Ês/Noc: +1.54 dB  0.915 as per Table G.5.4-1 |  |
| 9.3.8.3.2 TDD CQI Reporting under fading conditions - PUCCH 1-1 (CSI Reference Symbol) TM10 with TM9 interference - Enhanced Receiver Type B |  | Same as 9.3.8.3.1 |  | Same as 9.3.8.3.1 |  | Same as 9.3.8.3.1 |  |
| 9.4.1.1.1 FDD PMI Reporting – PUSCH 3-1 (Single PMI) |  | 1.10 |  | 0.01 |  |  1.09 as per Table G.5.4-1 |  |
| 9.4.1.1.2 TDD PMI Reporting – PUSCH 3-1 (Single PMI) |  | Same as 9.4.1.1.1 |  | Same as 9.4.1.1.1 |  | Same as 9.4.1.1.1 |  |
| 9.4.1.2.1 FDD PMI Reporting – PUCCH 2-1 (Single PMI) |  | Noc as specified in test procedure 1.2 |  | 0.01 |  | Noc unchanged Ior/ Noc=SNRrnd is a result of an approach according to G.5.2 and is reused unchanged as setting in procedure step 3.  1.19 as per Table G.5.4-1 |  |
| 9.4.1.2.2 TDD PMI Reporting – PUCCH 2-1 (Single PMI) |  | Same as 9.4.1.2.1 |  | Same as 9.4.1.2.1 |  | Same as 9.4.1.2.1 |  |
| 9.4.1.3.1_D FDD Reporting of PMI – PUSCH 3-1 (Single PMI) for eDL-MIMO |  | Same as 9.4.1.2.1 |  | Same as 9.4.1.2.1 |  | Same as 9.4.1.2.1 |  |
| 9.4.1.3.2_D TDD Reporting of PMI – PUSCH 3-1 (Single PMI) for eDL-MIMO |  | Same as 9.4.1.2.1 |  | Same as 9.4.1.2.1 |  | Same as 9.4.1.2.1 |  |
| 9.4.1.3.3 FDD PMI Reporting with 12Tx Class A codebook – PUSCH 3-1 (Single PMI) for FD-MIMO |  | Noc as specified in test procedure 2.50 |  | 0.01 |  | Noc unchanged Ior/ Noc=SNRrnd is a result of an approach according to G.5.2 and is used unchanged.  2.49 as per Table G.5.4-1 |  |
| 9.4.1.3.4 TDD PMI Reporting with 12Tx Class A codebook – PUSCH 3-1 (Single PMI) for FD-MIMO |  | Same as 9.4.1.3.3 |  | Same as 9.4.1.3.3 |  | Same as 9.4.1.3.3 |  |
| 9.4.1.4.1 FDD PMI Reporting with 4Tx enhanced codebook – PUCCH 1-1 (Single PMI) for eDL MIMO Enhancement 9.6.1.3.2, TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (3DL CA) |  | Noc: -98dBm/15kHz 1.8 |  | 0.01 |  | Noc: -98dBm/15kHz 1.79 as per Table G.5.4-1 The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.4.1.4.2 TDD PMI Reporting with 4Tx enhanced codebook – PUCCH 1-1 (Single PMI) for eDL MIMO Enhancement 9.6.1.4.2, TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (3DL CA) |  | Same as 9.4.1.4.1 |  | Same as 9.4.1.4.1 |  | Same as 9.4.1.4.1 |  |
| 9.4.1.4.3 FDD PMI Reporting with Class B alternative codebook – PUCCH 1-1 for FD-MIMO |  | Same as 9.4.1.3.3 |  | Same as 9.4.1.3.3 |  | Same as 9.4.1.3.3 |  |
| 9.4.1.4.4 TDD PMI Reporting with Class B alternative codebook – PUCCH 1-1 for FD-MIMO |  | Same as 9.4.1.3.3 |  | Same as 9.4.1.3.3 |  | Same as 9.4.1.3.3 |  |
| 9.4.2.1.1 FDD PMI Reporting – PUSCH 1-2 (Multiple PMI) |  | Noc as specified in test procedure 1.20 |  | 0.01 |  | Noc unchanged Ior/ Noc=SNRrnd is a result of an approach according to G.5.2 and is reused unchanged as setting in procedure step 3.  1.19 as per Table G.5.4-1 |  |
| 9.4.2.1.1_1 FDD PMI Reporting – PUSCH 1-2 (Multiple PMI) |  | Same as 9.4.2.1.1 |  | Same as 9.4.2.1.1 |  | Same as 9.4.2.1.1 |  |
| 9.4.2.1.2 TDD PMI Reporting – PUSCH 1-2 (Multiple PMI) |  | Same as 9.4.2.1.1 |  | Same as 9.4.2.1.1 |  | Same as 9.4.2.1.1 |  |
| 9.4.2.1.2_1 TDD PMI Reporting – PUSCH 1-2 (Multiple PMI) |  | Same as in 9.4.2.1.1 |  | Same as in 9.4.2.1.1 |  | Same as in 9.4.2.1.1 |  |
| 9.4.2.2.1 FDD PMI Reporting – PUSCH 2-2 (Multiple PMI) |  | Same as in 9.4.2.1.1 |  | Same as in 9.4.2.1.1 |  | Same as in 9.4.2.1.1 |  |
| 9.4.2.2.2 TDD PMI Reporting – PUSCH 2-2 (Multiple PMI) |  | Noc as specified in test procedure 1.15 |  | 0.01 |  | Noc unchanged Ior/ Noc=SNRrnd is a result of an approach according to G.5.2 and is reused unchanged as setting in procedure step 3.  1.14 as per Table G.5.4-1 |  |
| 9.4.2.3.1_D FDD PMI Reporting – PUSCH 1-2 (Multiple PMI) for eDL-MIMO |  | Noc as specified in test procedure 1.30 |  | 0.01 |  | Noc unchanged Ior/ Noc=SNRrnd is a result of an approach according to G.5.2 and is reused unchanged as setting in procedure step 3.  1.29 as per Table G.5.4-1 |  |
| 9.4.2.3.2_D TDD PMI Reporting – PUSCH 1-2 (Multiple PMI) for eDL-MIMO |  | Noc as specified in test procedure 3.50 |  | 0.01 |  | Noc unchanged Ior/ Noc=SNRrnd is a result of an approach according to G.5.2 and is reused unchanged as setting in procedure step 3.  3.49 as per Table G.5.4-1 |  |
| 9.4.2.3.3 FDD PMI Reporting with 4Tx enhanced codebook - PUSCH 1-2 (Multiple PMI) for eDL-MIMO Enhancement |  | Noc: -98dBm/15kHz 1.2 |  | 0.01 |  | Noc: -98dBm/15kHz 1.19 as per Table G.5.4-1 The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.4.2.3.4 TDD PMI Reporting with 4Tx enhanced codebook - PUSCH 1-2 (Multiple PMI) for eDL-MIMO Enhancement |  | Same as 9.4.2.3.3 |  | Same as 9.4.2.3.3 |  | Same as 9.4.2.3.3 |  |
| 9.4.2.3.5 FDD PMI Reporting with Class A 16Tx codebook - PUSCH 1-2 (Multiple PMI) for FD-MIMO |  | Same as 9.4.1.3.3 |  | Same as 9.4.1.3.3 |  | Same as 9.4.1.3.3 |  |
| 9.4.2.3.6 TDD PMI Reporting with Class A 16Tx codebook - PUSCH 1-2 (Multiple PMI) for FD-MIMO |  | Same as 9.4.1.3.3 |  | Same as 9.4.1.3.3 |  | Same as 9.4.1.3.3 |  |
| 9.5.1.1 FDD RI Reporting– PUCCH 1-1 |  | SNRs as specified Test 1: 21.00 Test 2: 11.05 Test 3: 21.10 |  | SNR 0dB 20.01 10.01 20.01 |  | SNR unchanged 2 0.99 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 2 1.09 as per Table G.5.4-1 |  |
| 9.5.1.1_1 FDD RI Reporting– PUCCH 1-1 (Release 10) |  | SNRs as specified Test 1: 21.00 Test 2: 11.05 Test 3: 10.90, 21.10 |  | SNR 0dB 20.01 10.01 20.01 |  | SNR unchanged 2 0.99 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 1 0.89, 2 1.09 as per Table G.5.4-1 |  |
| 9.5.1.1_2 FDD RI Reporting– PUCCH 1-1 (Release 11) |  | SNRs as specified Test 1: 21.00 Test 2: 11.05 Test 3: 10.90 |  | SNR 0dB 20.01 10.01 10.01 |  | SNR unchanged 2 0.99 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 1 0.89 as per Table G.5.4-1 |  |
| 9.5.1.2 TDD RI Reporting– PUSCH 3-1 |  | Same as 9.5.1.1 |  | Same as 9.5.1.1 |  | Same as 9.5.1.1 |  |
| 9.5.1.2_1 TDD RI Reporting– PUSCH 3-1 (Release 10) |  | Same as 9.5.1.1_1 |  | Same as 9.5.1.1_1 |  | Same as 9.5.1.1_1 |  |
| 9.5.1.2_2 TDD RI Reporting– PUSCH 3-1 (Release 11) |  | Same as 9.5.1.1_2 |  | Same as 9.5.1.1_2 |  | Same as 9.5.1.1_2 |  |
| 9.5.2.1_D FDD RI Reporting – PUCCH 1-1 for eDL-MIMO |  | SNR as specified Test 1: 21.00 Test 2: 11.05 Test 3: 10.9 |  | SNR 0dB 20.01 10.01 20.01 |  | SNR unchanged 2 0.99 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 10.89 as per Table G.5.4-1 |  |
| 9.5.2.2_D TDD RI Reporting – PUCCH 1-1 for eDL-MIMO |  | SNR as specified Test 1: 21.00 Test 2: 11.05 Test 3: 10.9 |  | SNR 0dB 20.01 10.01 20.01 |  | SNR unchanged 2 0.99 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 10.89 as per Table G.5.4-1 |  |
| 9.5.3.1_C.1 FDD RI Reporting – PUCCH 1-0 for eICIC (non-MBSFN ABS) |  | SNRs as specified Test 1: 10.90 Test 2: 11.05 |  | SNR 0dB 10.01 10.01 |  | SNR unchanged 1 0.89 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.5.3.2_C.1 TDD RI Reporting – PUCCH 1-0 for eICIC (non-MBSFN ABS) |  | SNRs as specified Test 1: 10.90 Test 2: 11.05 |  | SNR 0dB 10.01 10.01 |  | SNR unchanged 1 0.89 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.5.4.1_E.1 FDD RI Reporting - PUCCH 1-0 for feICIC (non-MBSFN ABS) |  | SNR as specified Test 1: 21.05 Test 2: 11.05 Test 3: 10.9 |  | SNR 0dB 20.01 10.01 10.01 |  | SNR unchanged 2 1.04 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 10.89 as per Table G.5.4-1 The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.5.4.2_E.1 TDD RI Reporting - PUCCH 1-0 for feICIC (non-MBSFN ABS) |  | Same as 9.5.4.1_E.1 |  | Same as 9.5.4.1_E.1 |  | Same as 9.5.4.1_E.1 |  |
| 9.5.5.1_F.1 FDD RI Reporting with Single CSI process for CoMP |  | SNRs as specified Test 1: 21.00 Test 2: 11.00 |  | SNR 0dB 20.01 10.01 |  | SNR unchanged 2 0.99 as per Table G.5.4-1 1 0.99 as per Table G.5.4-1 |  |
| 9.5.5.1_F.2 FDD RI Reporting with Multiple CSI processes for CoMP |  | SNRs as specified Test 1: 21.00 Test 2: 11.00 |  | SNR 20dB 20.01 10.01 |  | SNR unchanged 2 0.99 as per Table G.5.4-1 1 0.99 as per Table G.5.4-1 |  |
| 9.5.5.2_F.1 TDD RI Reporting with Single CSI process for CoMP |  | Same as 9.5.5.1_F.1 |  | Same as 9.5.5.1_F.1 |  | Same as 9.5.5.1_F.1 |  |
| 9.5.5.2_F.2 TDD RI Reporting with Multiple CSI processes for CoMP |  | Same as 9.5.5.2_F.1 |  | Same as 9.5.5.2_F.1 |  | Same as 9.5.5.2_F.1 |  |
| 9.6.1.1_A.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for CA (2DL CA) |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR ratios each have an uncertainty of ±0.3 dB. For uncorrelated uncertainties, the overall uncertainty in the SNR difference between any two CCs would be SQRT (0.32 + 0.32) = 0.42dB. This could widen the Median CQI difference, but the uncertainty is significantly less than one reported CQI step. Relaxing the allowed Median CQI difference would relax the test excessively, so in this case the Test requirement should be left unchanged. |  |
| 9.6.1.1_A.2 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 (3DL CA) |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | Same as 9.6.1.1_A.1 |  |
| 9.6.1.1_A.4 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 (5DL CA) |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | Same as 9.6.1.1_A.1 |  |
| 9.6.1.1_A.5 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 (6DL CA) |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | Same as 9.6.1.1_A.1 |  |
| 9.6.1.1_A.6 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 (7DL CA) |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | Same as 9.6.1.1_A.1 |  |
| 9.6.1.2_A.1 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 for 2DL CA |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 |  |
| 9.6.1.2_A.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (3DL CA) |  | Same as 9.6.1.1_A.2 |  | Same as 9.6.1.1_A.2 |  | Same as 9.6.1.1_A.2 |  |
| 9.6.1.2_A.3 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (4DL CA) |  | Same as 9.6.1.1_A.3 |  | Same as 9.6.1.1_A.3 |  | Same as 9.6.1.1_A.3 |  |
| 9.6.1.2_A.4 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (5DL CA) |  | Same as 9.6.1.1_A.4 |  | Same as 9.6.1.1_A.4 |  | Same as 9.6.1.1_A.4 |  |
| 9.6.1.2_A.5 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (6DL CA) |  | Same as 9.6.1.1_A.5 |  | Same as 9.6.1.1_A.5 |  | Same as 9.6.1.1_A.5 |  |
| 9.6.1.2_A.6 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 (7DL CA) |  | Same as 9.6.1.1_A.6 |  | Same as 9.6.1.1_A.6 |  | Same as 9.6.1.1_A.6 |  |
| 9.6.1.3.1 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (2DL CA) |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.6.1.3.2 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (3DL CA) |  | Same as 9.6.1.1_A.2 |  | Same as 9.6.1.1_A.2 |  | Same as 9.6.1.1_A.2 The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.6.1.3.3 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (4DL CA) |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 |  |
| 9.6.1.3.4 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (5DL CA) |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 |  |
| 9.6.1.4.1 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (2DL CA) |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.6.1.4.2 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (3DL CA) |  | Same as 9.6.1.1_A.2 |  | Same as 9.6.1.1_A.2 |  | Same as 9.6.1.1_A.2 The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.6.1.4.3 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (4DL CA) |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 |  |
| 9.6.1.4.4 TDD FDD CA CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (5DL CA) |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 |  | Same as 9.6.1.1_A.1 |  |
| 9.7.1.1 FDD and Half duplex FDD CQI reporting definition under AWGN conditions for UE category 0 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.7.1.2 TDD CQI reporting definition under AWGN conditions for UE category 0 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.7.2.1 FDD and Half duplex FDD CQI reporting definition under fading conditions for UE category 0 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  |
| 9.7.2.2 TDD CQI reporting definition under fading conditions for UE category 0 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  |
| 9.7_1.1.1 FDD CQI reporting under AWGN conditions - PUCCH 1-0 for UE category 1bis |  | Same as 9.2.1.1 |  | Same as 9.2.1.1 |  | Same as 9.2.1.1 |  |
| 9.7_1.1.2 TDD CQI reporting under AWGN conditions - PUCCH 1-0 for UE category 1bis |  | Same as 9.2.1.2 |  | Same as 9.2.1.2 |  | Same as 9.2.1.2 |  |
| 9.7_1.1.3 FDD CQI reporting under fading conditions - PUSCH 3-0 for UE category 1bis |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  |
| 9.7_1.1.4 TDD CQI reporting under fading conditions - PUSCH 3-0 for UE category 1bis |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  |
| 9.8.1.1 FDD and Half duplex FDD CQI reporting definition under AWGN conditions for UE category M1 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.8.1.2 TDD CQI reporting definition under AWGN conditions for UE category M1 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.8.2.1 FDD and Half duplex FDD CQI reporting definition under fading conditions for UE category M1 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  |
| 9.8.2.2 TDD CQI reporting definition under fading conditions for UE category M1 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  | Same as 9.3.1.1.1 |  |
| 9.9.1.1.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-0 with Rank 1 1x4 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.9.1.1.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-0 with Rank 1 1x4 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.9.1.2.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 2 4x4 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.9.1.2.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 2 8x4 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.9.1.3.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 4 4x4 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.9.1.3.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 4 4x4 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.9.1.4.1 FDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 3 4x4 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.9.1.4.2 TDD CQI Reporting under AWGN conditions – PUCCH 1-1 with rank 3 8x4 |  | SNRs as specified Limits as in the Test Procedure |  | No test tolerances applied |  | SNR and limits unchanged |  |
| 9.9.2.1.1 FDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A 1x4 |  | Noc: -98dBm/15kHz DIP 1: -0.41dB SINR: -4.00dB 1.8 BLER 0.02 |  | 0dB +0.03dB +0.03dB 0.01 |  | Noc: -98dBm/15kHz DIP 1: -0.38dB SINR: -3.97dB 1.79 as per Table G.5.4-1 BLER limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.9.2.1.2 TDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A 1x4 |  | Same as 9.9.2.1.1 |  | Same as 9.9.2.1.1 |  | Same as 9.9.2.1.1 |  |
| 9.9.2.2.1 FDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A 2x4 |  | Noc: -98dBm/15kHz DIP 1: -0.41dB SINR: -4.00dB 1.8 BLER 0.02 |  | 0dB +0.03dB +0.03dB 0.01 |  | Noc: -98dBm/15kHz DIP 1: -0.38dB SINR: -3.97dB 1.79 as per Table G.5.4-1 BLER limit unchanged The analysis is recorded in 3GPP TR 36.904 [17] |  |
| 9.9.2.2.2 TDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A 2x4 |  | Same as 9.9.2.2.1 |  | Same as 9.9.2.2.1 |  | Same as 9.9.2.2.1 |  |
| 9.9.3.1.1 TDD PMI Reporting – PUSCH 3-1 (Single PMI) 8x4 |  | Noc as specified in test procedure 2.50 |  | 0.01 |  | Noc unchanged Ior/ Noc=SNRrnd is a result of test procedure step 2 and is used as setting in test procedure step 3. 2.49 as per Table G.5.4-1 |  |
| 9.9.4.1.1 FDD RI Reporting– PUCCH 1-1 4x4 |  | SNRs as specified Test 1: 21.00 Test 2: 11.05 Test 3: 10.9 Test 4: 2 1.10 |  | SNR 0dB 20.01 10.01 10.01 20.01 |  | SNR unchanged 2 0.99 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 1 0.89 as per Table G.5.4-1 2 1.09 as per Table G.5.4-1 |  |
| 9.9.4.1.2 TDD RI Reporting– PUSCH 3-1 4x4 |  | SNRs as specified Test 1: 21.00 Test 2: 11.05 Test 3: 10.9 Test 4: 2 1.10 |  | SNR 0dB 20.01 10.01 10.01 20.01 |  | SNR unchanged 2 0.99 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 1 0.89 as per Table G.5.4-1 2 1.09 as per Table G.5.4-1 |  |
| 9.9.4.2.1 FDD RI Reporting- PUCCH 1-1 for eDL-MIMO 1x4 |  | SNRs as specified Test 1: 21.00 Test 2: 11.05 Test 3: 10.9 Test 4: 2 1.10 |  | SNR 0dB 20.01 10.01 10.01 20.01 |  | SNR unchanged 2 0.99 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 1 0.89 as per Table G.5.4-1 2 1.09 as per Table G.5.4-1 |  |
| 9.9.4.2.2 TDD RI Reporting- PUCCH 1-1 for eDL-MIMO 1x4 |  | SNRs as specified Test 1: 21.00 Test 2: 11.05 Test 3: 10.9 Test 4: 2 1.10 |  | SNR 0dB 20.01 10.01 10.01 20.01 |  | SNR unchanged 2 0.99 as per Table G.5.4-1 1 1.04 as per Table G.5.4-1 1 0.89 as per Table G.5.4-1 2 1.09 as per Table G.5.4-1 |  |
| 9.10.1.1 FDD CSI-RS Resource Indicator Reporting – PUSCH 3-1 |  | SNRs as specified Test 1-1: 1.2 Test 1-2: 1.2 Test 1-3: 1.3 Test 1-4:  1.35 |  | SNR 0dB 0.01 |  | SNR unchanged  1.19 as per Table G.5.4-1  1.19 as per Table G.5.4-1  1.29 as per Table G.5.4-1  1.34 as per Table G.5.4-1 |  |
| 9.10.1.2 TDD CSI-RS Resource Indicator Reporting – PUSCH 3-1 |  | SNRs as specified Test 1-1: 1.2 Test 1-2: 1.2 Test 1-3: 1.3 Test 1-4:  1.35 |  | SNR 0dB 0.01 |  | SNR unchanged  1.19 as per Table G.5.4-1  1.19 as per Table G.5.4-1  1.29 as per Table G.5.4-1  1.34 as per Table G.5.4-1 |  |
| 9.12.1.1 FDD CQI Reporting under fading conditions (with cell-specific reference symbols configured for UE supporting short TTI) |  | SNRs as specified 20% 1.05 BLER 0.02 |  | SNR 0dB 0% 0.01 BLER 0 |  | SNR and BLER limits unchanged  limit unchanged  1.04 as per Table G.5.4-1 BLER limit unchanged |  |
| 9.12.1.1 TDD CQI Reporting under fading conditions (with cell-specific reference symbols configured for UE supporting short TTI) |  | SNRs as specified 20% 1.05 BLER 0.02 |  | SNR 0dB 0% 0.01 BLER 0 |  | SNR and BLER limits unchanged  limit unchanged  1.04 as per Table G.5.4-1 BLER limit unchanged |  |
| 9.12.2.1 FDD CQI Reporting under fading conditions (with CSI reference symbols configured for UE supporting short TTI) |  | SNRs as specified 20% 1.05 BLER [0.02] |  | SNR 0dB 0% 0.01 BLER 0 |  | SNR and BLER limits unchanged  limit unchanged  1.04 as per Table G.5.4-1 BLER limit unchanged |  |
| 9.12.2.2 FDD CQI Reporting under fading conditions (with CSI reference symbols configured for UE supporting short TTI) |  | SNRs as specified 20% 1.05 BLER [0.02] |  | SNR 0dB 0% 0.01 BLER 0 |  | SNR and BLER limits unchanged  limit unchanged  1.04 as per Table G.5.4-1 BLER limit unchanged |  |

# << End of changes >>