3GPP TSG-RAN5 Meeting #96-e	R5-224416

# Online, , 15st Aug 2022 – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-2 | CR | 0987 | rev | - | Current version: | 16.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of applicability 7DL CA Sustained data rate performance test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | DEKRA Testing and Certification |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R15-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The TDD FDD 7DL CA Sustained data rate performance test cases are not defined in TS36.521-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of the applicability following test cases: 1) TC 8.7.5.1.6 2) TC 8.7.5.2.6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 7DL CA Sustained data rate performance test cases will be still missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1 and A.4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Beginning of changes >>

4.1	RF conformance test cases

NOTE:	To determine applicability of a test case, FGI support in combined or fdd-Add-UE-EUTRA-Capabilities or tdd-Add-UE-EUTRA-Capabilities is taken into account.

Table 4.1-1: Applicability of RF conformance test cases, ref. TS 36.521-1 [1]

| Clause | Title | Release |  | Applicability |  | Tested Bands / CA-Configurations Selection |  | Branch |  | Additional Information |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Condition | Comments |  |  |  |  |  |
| Transmitter Characteristics |  |  |  |  |  |  |  |  |  |  |
| 6.2.2 | UE Maximum Output Power | Rel-8 |  | C186 | UE supporting E-UTRA Power Class 3 | D01 |  | FDD, TDD |  | Not required to be tested in Band 41 for Power Class 2 UE |
| 6.2.2_1 | UE Maximum Output Power for HPUE | Rel-10 |  | C39a | UE supporting E-UTRA Power Class 1 | D16 |  |  |  |  |
|  |  |  |  | C39b | UE supporting E-UTRA Power Class 2 | D17 |  |  |  |  |
|  | <<Too many rows skipped>> |  |  |  |  |  |  |  |  |  |
| 8.7.1.1 | FDD sustained data rate performance (Rel-9 and forward) | Rel-9 |  | C76 | UE supporting E-UTRA FDD and not supporting 256QAM in DL (UE categories from1 to 4) | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  | It is not necessary for CA UEs and EPDCCH UEs to be tested in this test if 8.7.1.1_A.1 or 8.7.3.1 is executed. |
| 8.7.1.1_1 | FDD sustained data rate performance (Rel-10 and forward) | Rel-10 |  | C42 | UE supporting E-UTRA FDD and not supporting 256QAM in DL (UE categories 6, 7) | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  | It is not necessary for CA UEs and EPDCCH UEs to be tested in this test if 8.7.1.1_A.1or 8.7.3.1 is executed. |
| 8.7.1.1_2 | FDD sustained data rate performance for UE category 1bis | Rel-13 |  | C145d | UE supporting E-UTRA FDD (UE category 1bis) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.7.1.1_A.1 | FDD Sustained data rate performance for CA (2 DL CA ) | Rel-10 |  | C107 | UE supporting E-UTRA FDD and intra-band contiguous DL CA or inter-band DL CA and not supporting 256QAM in DL (UE category 3, 4, 6, 7, 9 and 10) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.7.1.1_A.2 is executed. |
| 8.7.1.1_2 | FDD sustained data rate performance for UE category 1bis | Rel-11 |  | C93 | UE supporting E-UTRA FDD and intra-band non-contiguous DL CA and not supporting 256QAM in DL (UE category 3, 4, 6, 7, 9 and 10) |  |  |  |  | Note 7 |
| 8.7.1.1_A.2 | FDD Sustained data rate performance for CA (3DL CA) | Rel-10 |  | C126 | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 and not supporting 256QAM in DL (UE category 9, 10, 11 and 12) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.7.1.1_A.4 is executed |
|  |  | Rel-10 |  | C126a | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 and supporting at most 40MHz aggregated bandwidth and not supporting 256QAM in DL (UE category 6 and 7) |  | Refer to 36.521-1 8.1.2.3 |  |  | Note 7 |
|  |  | Rel-11 |  | C127 | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 and not supporting 256QAM in DL (UE category 9, 10, 11 and 12) |  | TBD |  |  |  |
|  |  | Rel-11 |  | C127a | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 and supporting at most 40MHz aggregated bandwidth and not supporting 256QAM in DL (UE category 6 and 7) |  | TBD |  |  |  |
| 8.7.1.1_A.4 | FDD Sustained data rate performance for CA (4DL CA) | Rel-11 |  | C189 | UE supporting E-UTRA FDD and 4DL with CA configurations in Table 4.1-4 and not supporting 256QAM in DL (UE category 11 and 12) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.1.1_A.5 is executed. Note 7 |
|  |  | Rel-11 |  | C189a | UE supporting E-UTRA FDD and 4DL with CA configurations in Table 4.1-4 and supporting at most 60MHz aggregated bandwidth and not supporting 256QAM in DL (UE category 9 and 10) |  |  |  |  |  |
| 8.7.1.1_A.5 | FDD Sustained data rate performance for CA (5DL CA) | Rel-11 |  | C266 | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 and not supporting 256QAM in DL (UE DL category 11,12) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.1.1_A.6 is executed. Note 7 |
|  |  | Rel-12 |  | C267 | UE supporting E-UTRA FDD and 5DL CA configurations in Table 4.1-5 and not supporting 256QAM in DL (UE DL category 11,12,15) |  |  |  |  |  |
| 8.7.1.1_A.6 | FDD Sustained data rate performance for CA (6DL CA) | Rel-11 |  | C330 | UE supporting E-UTRA FDD and 6DL with CA configurations in Table 4.1-6 and not supporting 256QAM in DL (UE DL category 11,12) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.1.1_A.7 is executed. Note 7 |
|  |  | Rel-12 |  | C331 | UE supporting E-UTRA FDD and 6DL CA configurations in Table 4.1-6 and not supporting 256QAM in DL (UE DL category 11,12,15) |  |  |  |  |  |
| 8.7.1.1_A.7 -8.7.1.1_H.1 | FDD Sustained data rate performance for CA (7DL CA) FDD sustained data rate performance (Single Carrier) for 256QAM in DL | Rel-15 |  | C343 | UE supporting E-UTRA FDD and 7DL CA and not supporting 256QAM in DL (UE DL category 11,12,15) |  |  | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-12 |  | C42h | UE supporting E-UTRA FDD and 256QAM and UE DL category 13 |  |  |  |  | Test execution not necessary if 8.7.1.1_H.2 is executed |
| 8.7.1.1_H.2 | FDD Sustained data rate performance for CA (2DL CA) for 256QAM in DL | Rel-12 |  | C107h | UE supporting E-UTRA FDD and 2DL CA and 256QAM in DL (UE DL category 11, 12 and 13) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.1.1_H.3 is executed Note 7 |
| 8.7.1.1_H.3 | FDD Sustained data rate performance for CA (3DL CA) for 256QAM in DL | Rel-12 |  | C126h | UE supporting E-UTRA FDD and 3DL CA ,and supporting 256QAM in DL (UE DL category 11, 12 and 15) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.1.1_H.4 is executed Note 7 |
|  |  | Rel-12 |  | C126ha | UE supporting E-UTRA FDD and 3DL CA ,and supporting 256QAM in DL and supporting at most 40MHz aggregated bandwidth (UE DL category 13) |  |  |  |  | Test execution not necessary if 8.7.1.1_H.4 is executed |
| 8.7.1.1_H.4 | FDD Sustained data rate performance for CA (4DL CA) for 256QAM in DL | Rel-13 |  | C189h | UE supporting E-UTRA FDD and 4DL with CA configurations in Table 4.1-4 and supporting 256QAM in DL |  |  |  |  | Test execution not necessary if 8.7.1.1_H.5 is executed. |
| 8.7.1.1_H.5 | FDD Sustained data rate performance for CA (5DL CA) for 256QAM in DL | Rel-13 |  | C266h | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 and supporting 256QAM in DL |  |  |  |  |  |
| 8.7.1.1_H.6 | FDD Sustained data rate performance for CA (6DL CA) for 256QAM in DL | Rel-13 |  | C331h | UE supporting E-UTRA FDD and 6DL with CA configurations in Table 4.1-6 and supporting 256QAM in DL |  |  |  |  |  |
| 8.7.1.1_H.7 | FDD Sustained data rate performance for CA (7DL CA) for 256QAM in DL | Rel-15 |  | C344 | UE supporting E-UTRA FDD and 7DL and supporting 256QAM in DL |  |  |  |  | Test execution not necessary if 8.7.1.1_H.8 is executed |
| 8.7.2.1 | TDD sustained data rate performance (Rel-9 and forward) | Rel-9 |  | C111 | UE supporting E-UTRA TDD and not supporting 256QAM in DL (UE categories from 1 to 4) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | It is not necessary for CA UEs and EPDCCH UEs to be tested in this test if 8.7.2.1_A.1 or 8.7.4.1 is executed. |
| 8.7.2.1_1 | TDD sustained data rate performance (Rel-10 and forward) | Rel-10 |  | C73 | UE supporting E-UTRA TDD and not supporting 256QAM in DL (UE category 6 and 7) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  | It is not necessary for CA UEs and EPDCCH UEs to be tested in this test if 8.7.2.1_A.1or 8.7.4.1 is executed. |
| 8.7.2.1_2 | TDD sustained data rate performance for UE category 1bis | Rel-13 |  | C156f | UE supporting E-UTRA TDD (UE category 1bis) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.7.2.1_A.1 | TDD sustained data rate performance for CA (2DL CA) | Rel-10 |  | C74 | UE supporting E-UTRA TDD and intra-band contiguous DL CA or inter-band DL CA and not supporting 256QAM in DL (UE category 6, 7, 9 and 10) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_A.2 is executed. |
|  |  | Rel-11 |  | C75 | UE supporting E-UTRA TDD and intra-band non-contiguous DL CA and not supporting 256QAM in DL (UE category 6, 7, 9 and 10) |  |  |  |  | Note 7 |
| 8.7.2.1_A.2 | TDD Sustained data rate performance for CA (3DL CA) | Rel-10 |  | C130 | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 and not supporting 256QAM in DL (UE category 9, 10, 11 and 12) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_A.3 is executed. |
|  |  | Rel-11 |  | C131 | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 and not supporting 256QAM in DL (UE category 9, 10, 11 and 12) |  |  |  |  | Note 7 |
| 8.7.2.1_A.3 | TDD Sustained data rate performance for CA (4DL CA) | Rel-11 |  | C213 | UE supporting E-UTRA TDD and 4DL CA configurations in Table 4.1-3 (UE DL category 11, 12 and 15) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_A.4 is executed. Note 7 |
| 8.7.2.1_A.5 | TDD Sustained data rate performance for CA (5DL CA) | Rel-11 |  | C360 | UE supporting E-UTRA TDD and 5DL CA configurations in Table 4.1-3 (UE DL category 11, 12 and 15) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_A.6 is executed. Note 7 |
| 8.7.2.1_A.6 | TDD Sustained data rate performance for CA (6DL CA) | Rel-15 |  | C331a | UE supporting E-UTRA TDD and 6DL CA configurations in Table 4.1-3 (UE DL category 11, 12 and 15) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_A.7 is executed. Note 7 |
| 8.7.2.1_A.7 | TDD Sustained data rate performance for CA (7DL CA) | Rel-15 |  | C345 | UE supporting E-UTRA TDD and 7DL CA (UE DL category 11, 12 and 15) |  | Refer to 36.521-1 8.1.2.3 | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_A.8 is executed. Note 7 |
| 8.7.2.1_H.1 | TDD sustained data rate performance (Single Carrier) for 256QAM in DL | Rel-12 |  | C73h | UE supporting E-UTRA TDD and 256QAM in DL and UE DL category 13 |  |  |  |  | Test execution not necessary if 8.7.2.1_H.2 is executed. |
| 8.7.2.1_H.2 | TDD sustained data rate performance for CA (2DL CA) for 256QAM in DL | Rel-12 |  | C74h | UE supporting E-UTRA TDD and 2DL CA, and supporting 256QAM in DL (UE DL category 11, 12 and 13) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_H.3 is executed. Note 7 |
| 8.7.2.1_H.3 | TDD Sustained data rate performance for CA (3DL CA) for 256QAM in DL | Rel-12 |  | C130h | UE supporting E-UTRA TDD and 3DL CA and supporting 256QAM in DL (UE DL Category 11, 12 and 15) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_H.4 is executed. Note 7 |
| 8.7.2.1_H.4 | TDD Sustained data rate performance for CA (4DL CA) for 256QAM in DL | Rel-13 |  | C213h | UE supporting E-UTRA TDD and 4DL CA and supporting 256QAM in DL (UE DL Category 11, 12,15 and 16) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_H.5 is executed. Note 7 |
| 8.7.2.1_H.5 | TDD Sustained data rate performance for CA (5DL CA) for 256QAM in DL | Rel-14 |  | C332 | UE supporting E-UTRA TDD and 5DL CA and supporting 256QAM in DL (UE DL Category 16) |  |  | 2Rx, 4Rx |  | Note 7 |
| 8.7.2.1_H.6 | TDD Sustained data rate performance for CA (6DL CA) for 256QAM in DL | Rel-15 |  | C331ha | UE supporting E-UTRA TDD and 6DL CA and supporting 256QAM in DL (UE DL Category 16) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_H.7 is executed. Note 7 |
| 8.7.2.1_H.7 | TDD Sustained data rate performance for CA (7DL CA) for 256QAM in DL | Rel-15 |  | C346 | UE supporting E-UTRA TDD and 7DL CA and supporting 256QAM in DL (UE DL Category 16) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.2.1_H.8 is executed. Note 7 |
| 8.7.3.1 | FDD sustained data rate performance for EPDCCH scheduling | Rel-11 |  | C55 | UE supporting E-UTRA FDD and EPDCCH |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.7.4.1 | TDD sustained data rate performance for EPDCCH scheduling | Rel-11 |  | C56 | UE supporting E-UTRA TDD and EPDCCH |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.7.5.1.1 | TDD FDD CA Sustained data rate performance for FDD PCell (2DL CA) | Rel-12 |  | C138 | UE supporting E-UTRA FDD and TDD and 2DL CA with FDD as PCell and not supporting 256QAM in DL (UE category 3, 4, 6, 7, 9 and 10) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.1.2 is executed. Note 7 |
| 8.7.5.1.2 | TDD FDD CA Sustained data rate performance for FDD PCell (3DL CA) | Rel-12 |  | C139 | UE supporting E-UTRA FDD and TDD and 3DL CA with FDD as PCell and not supporting 256QAM in DL (UE category 9, 10, 11 and 12) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.1.3 is executed. Note 7 |
| 8.7.5.1.3 | TDD FDD CA Sustained data rate performance for FDD PCell (4DL CA) | Rel-12 |  | C139a | UE supporting E-UTRA FDD and TDD and 4DL CA with FDD as PCell and not supporting 256QAM in DL (UE category 11 and 12) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.1.4 is executed. Note 7 |
| 8.7.5.1.4 | TDD FDD CA Sustained data rate performance for FDD PCell (5DL CA) | Rel-12 |  | C139b | UE supporting E-UTRA FDD and TDD and 5DL CA with FDD as PCell and not supporting 256QAM in DL (UE DL category 15) |  | TBD | 2Rx, 4Rx |  | Note 7 |
| 8.7.5.1.5 | TDD FDD CA Sustained data rate performance for FDD PCell (6DL CA) | Rel-12 |  | C139c | UE supporting E-UTRA FDD and TDD and 6DL CA with FDD as PCell and not supporting 256QAM in DL (UE DL category 15) |  | TBD | 2Rx, 4Rx |  | Note 7 |
| 8.7.5.1.6 | TDD FDD CA Sustained data rate performance for FDD PCell (7DL CA) | Rel-12 |  | Cxxx1 | UE supporting E-UTRA FDD and TDD and 7DL CA with FDD as PCell and not supporting 256QAM in DL (UE DL category 15) |  | TBD | 2Rx, 4Rx |  | Note 7 |
| 8.7.5.1_H.1 | TDD FDD CA Sustained data rate performance for FDD PCell (2DL CA) for 256QAM in DL | Rel-12 |  | C138h | UE supporting E-UTRA FDD and TDD and 2DL TDD-FDD CA with FDD as PCell and supporting 256QAM in DL (UE DL category 11, 12 and 13) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.1_H.2 is executed. Note 7 |
| 8.7.5.1_H.2 | TDD FDD CA Sustained data rate performance for FDD PCell (3DL CA) for 256QAM in DL | Rel-12 |  | C139h | UE supporting E-UTRA FDD and TDD and 3DL TDD-FDD CA with FDD as PCell and supporting 256QAM in DL (UE DL Category 11, 12 and 15) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.1_H.3 is executed. Note 7 |
| 8.7.5.1_H.3 | TDD FDD CA Sustained data rate performance for FDD PCell (4DL CA) for 256QAM in DL | Rel-12 |  | C139ha | UE supporting E-UTRA FDD and TDD and 4DL TDD-FDD CA with FDD as PCell and supporting 256QAM in DL |  |  |  |  | Test execution not necessary if 8.7.5.1_H.4 is executed. |
| 8.7.5.1_H.4 | TDD FDD CA Sustained data rate performance for FDD PCell (5DL CA) for 256QAM in DL | Rel-12 |  | C139hb | UE supporting E-UTRA FDD and TDD and 5DL TDD-FDD CA with FDD as PCell and supporting 256QAM in DL |  |  |  |  |  |
| 8.7.5.1_H.5 | TDD FDD CA Sustained data rate performance for FDD PCell (6DL CA) for 256QAM in DL | Rel-12 |  | C139hc | UE supporting E-UTRA FDD and TDD and 6DL TDD-FDD CA with FDD as PCell and supporting 256QAM in DL |  |  |  |  |  |
| 8.7.5.2.1 | TDD FDD CA Sustained data rate performance for TDD PCell (2DL CA) | Rel-12 |  | C140 | UE supporting E-UTRA FDD and TDD and 2DL CA with TDD as PCell and not supporting 256QAM in DL (UE category 3, 4, 6, 7, 9 and 10) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.2.2 is executed. Note 7 |
| 8.7.5.2.2 | TDD FDD CA Sustained data rate performance for TDD PCell (3DL CA) | Rel-12 |  | C141 | UE supporting E-UTRA FDD and TDD and 3DL CA with TDD as PCell and not supporting 256QAM in DL (UE category 9, 10, 11 and 12) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.2.3 is executed. Note 7 |
| 8.7.5.2.3 | TDD FDD CA Sustained data rate performance for TDD PCell (4DL CA) | Rel-12 |  | C141a | UE supporting E-UTRA FDD and TDD and 4DL CA with TDD as PCell and not supporting 256QAM in DL (UE category 11 and 12) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.2.4 is executed. Note 7 |
| 8.7.5.2.4 | TDD FDD CA Sustained data rate performance for TDD PCell (5DL CA) | Rel-12 |  | C141b | UE supporting E-UTRA FDD and TDD and 5DL CA with TDD as PCell and not supporting 256QAM in DL (UE category 15) |  | TBD | 2Rx, 4Rx |  | Note 7 |
| 8.7.5.2.5 | TDD FDD CA Sustained data rate performance for TDD PCell (6DL CA) | Rel-12 |  | C141c | UE supporting E-UTRA FDD and TDD and 6DL CA with TDD as PCell and not supporting 256QAM in DL (UE category 15) |  | TBD | 2Rx, 4Rx |  | Note 7 |
| 8.7.5.2.6 | TDD FDD CA Sustained data rate performance for TDD PCell (7DL CA) | Rel-12 |  | Cxxx2 | UE supporting E-UTRA FDD and TDD and 7DL CA with TDD as PCell and not supporting 256QAM in DL (UE category 15) |  | TBD | 2Rx, 4Rx |  | Note 7 |
| 8.7.5.2_H.1 | TDD FDD CA Sustained data rate performance for TDD PCell (2DL CA) for 256QAM in DL | Rel-12 |  | C140h | UE supporting E-UTRA FDD and TDD and 2DL TDD-FDD CA with TDD as PCell and supporting 256QAM in DL (UE DL Category 11, 12 and 13) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.2_H.2 is executed. Note 7 |
| 8.7.5.2_H.2 | TDD FDD CA Sustained data rate performance for TDD PCell (3DL CA) for 256QAM in DL | Rel-12 |  | C141h | UE supporting E-UTRA FDD and TDD and 3DL TDD-FDD CA with TDD as PCell and supporting 256QAM in DL (UE DL Category 11, 12 and 15) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 8.7.5.2_H.3 is executed. Note 7 |
| 8.7.5.2_H.4 | TDD FDD CA Sustained data rate performance for TDD PCell (5DL CA) for 256QAM in DL | Rel-12 |  | C141hb | UE supporting E-UTRA FDD and TDD and 5DL TDD-FDD CA with TDD as PCell and supporting 256QAM in DL (UE DL Category 16) |  |  | 2Rx, 4Rx |  | Note 7 |
| 8.7.5.2_H.5 | TDD FDD CA Sustained data rate performance for TDD PCell (6DL CA) for 256QAM in DL | Rel-12 |  | C142h | UE supporting E-UTRA FDD and TDD and 6DL TDD-FDD CA with TDD as PCell and supporting 256QAM in DL (UE DL Category 16) |  |  | 2Rx, 4Rx |  | Note 7 |
| 8.7.6.1 | FDD sustained data rate performance for Dual Connectivity 64QAM | Rel-12 |  | C171 | UE supporting E-UTRA FDD and Dual Connectivity and not supporting 256QAM in DL (UE Category 3, 4, 6, 7, 9, and 10) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 8.7.6.2 | FDD sustained data rate performance for Dual Connectivity 256QAM | Rel-12 |  | C173 | UE supporting E-UTRA FDD and Dual Connectivity and supporting 256QAM in DL |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 8.7.7.1 | TDD sustained data rate performance for Dual Connectivity 64QAM | Rel-12 |  | C172 | UE supporting E-UTRA TDD and Dual Connectivity and not supporting 256QAM in DL (UE Category 6, 7, 9, and 10) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 8.7.7.2 | TDD sustained data rate performance for Dual Connectivity 256QAM | Rel-12 |  | C174 | UE supporting E-UTRA TDD and Dual Connectivity and supporting 256QAM in DL |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 8.7.9.2 | FDD sustained data rate performance for 4 layer MIMO (single carrier) | Rel-10 |  | C226 | UE supporting E-UTRA FDD with 4Rx antenna ports and 4-layer spatial multiplexing (UE Category 6 and 7 and UE DL category 13) |  | One "Test Number" to be performed. The selected band shall lead to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.9 is executed, where larger equivalent aggregated bandwidth can be achieved. |
| 8.7.9.3 | FDD sustained data rate performance for 4 layer MIMO (2DL CA) | Rel-10 |  | C227 | UE supporting 2DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and intra-band contiguous DL CA or inter-band DL CA (UE Category 9, 10, 11 and 12 and UE DL category 9, 10, 11, 12 and 15) |  | One "Test Number" to be performed. The selected CA configuration shall lead to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.9 is executed, where larger equivalent aggregated bandwidth can be achieved. |
|  |  | Rel-11 |  | C228 | UE supporting 2DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and intra-band non-contiguous DL CA (UE Category 9, 10, 11 and 12 and UE DL category 9, 10, 11, 12 and 15) |  |  |  |  |  |
| 8.7.9.4 | FDD sustained data rate performance for 4 layer MIMO (3DL CA) | Rel-10 |  | C229 | UE supporting 3DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 3DL with CA configurations in Table 4.1-3 (UE Category 11 and 12 and UE DL category 11, 12, 15, 16 and 18) |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.9 is executed, where larger equivalent aggregated bandwidth can be achieved. |
|  |  | Rel-11 |  | C230 | UE supporting 3DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 3DL with CA configurations in Table 4.1-3 and 4Rx antenna ports and 4-layer spatial multiplexing (UE Category 11 and 12 and UE DL category 11, 12, 15, 16 and 18) |  |  |  |  |  |
| 8.7.9.5 | FDD sustained data rate performance for 4 layer MIMO (4DL CA) | Rel-11 |  | C236 | UE supporting 4DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 4DL with CA configurations in Table 4.1-4 (UE DL category 15, 16, 18 and 19) |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.9 is executed, where larger equivalent aggregated bandwidth can be achieved. |
| 8.7.9.6 | FDD sustained data rate performance for 4 layer MIMO (5DL CA) | Rel-11 |  | C237 | UE supporting 5DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 5DL with CA configurations in Table 4.1-5 (UE DL category 15, 16, 18 and 19) |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.9 is executed, where larger equivalent aggregated bandwidth can be achieved. |
|  |  | Rel-12 |  | C238 | UE supporting 5DL FDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 5DL with CA configurations in Table 4.1-5 (UE DL category 15, 16, 18 and 19) |  |  |  |  |  |
| 8.7.10.2 | TDD sustained data rate performance for 4 layer MIMO (single carrier) | Rel-10 |  | C239 | UE supporting E-UTRA TDD with 4Rx antenna ports and 4-layer spatial multiplexing (UE Category 6 and 7 and UE DL category 13) |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.10 is executed, where larger equivalent aggregated bandwidth can be achieved. |
| 8.7.10.3 | TDD sustained data rate performance for 4 layer MIMO (2DL CA) | Rel-10 |  | C240 | UE supporting 2DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and intra-band contiguous DL CA or inter-band DL CA (UE Category 9, 10, 11 and 12 and UE DL category 9, 10, 11, 12 and 15) |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.10 is executed, where larger equivalent aggregated bandwidth can be achieved. |
|  |  | Rel-11 |  | C241 | UE supporting 2DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and intra-band non-contiguous DL CA (UE Category 9, 10, 11 and 12 and UE DL category 9, 10, 11, 12 and 15) |  |  |  |  |  |
| 8.7.10.4 | TDD sustained data rate performance for 4 layer MIMO (3DL CA) | Rel-10 |  | C242 | UE supporting 3DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 3DL with CA configurations in Table 4.1-3 (UE Category 11 and 12 and UE DL category 11, 12, 15, 16 and 18) |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.10 is executed, where larger equivalent aggregated bandwidth can be achieved. |
|  |  | Rel-11 |  | C243 | UE supporting 3DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 3DL with CA configurations in Table 4.1-3 (UE Category 11 and 12 and UE DL category 11, 12, 15, 16 and 18) |  |  |  |  |  |
| 8.7.10.5 | TDD sustained data rate performance for 4 layer MIMO (4DL CA) | Rel-11 |  | C244 | UE supporting 4DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 4DL with CA configurations in Table 4.1-4 (UE DL category 16, 18 and 19) |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.10 is executed, where larger equivalent aggregated bandwidth can be achieved. |
| 8.7.10.6 | TDD sustained data rate performance for 4 layer MIMO (5DL CA) | Rel-11 |  | C245 | UE supporting 5DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 5DL with CA configurations in Table 4.1-5 (UE DL category 18 and 19) |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.10 is executed, where larger equivalent aggregated bandwidth can be achieved. |
|  |  | Rel-12 |  | C246 | UE supporting 5DL TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing and 5DL with CA configurations in Table 4.1-5 (UE DL category 18 and 19) |  |  |  |  |  |
| 8.7.11.2 | TDD FDD CA sustained data rate performance for 4 layer MIMO (2DL CA) | Rel-12 |  | C247 | UE supporting 2DL FDD-TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing (UE Category 9, 10, 11 and 12 and UE DL category 9, 10, 11, 12 and 15) |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.11 is executed, where larger equivalent aggregated bandwidth can be achieved. |
| 8.7.11.3 | TDD FDD CA sustained data rate performance for 4 layer MIMO (3DL CA) | Rel-12 |  | C248 | UE supporting 3DL FDD-TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing (UE Category 11 and 12 and UE DL category 11, 12, 15, 16 and 18) |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.11 is executed, where larger equivalent aggregated bandwidth can be achieved. |
| 8.7.11.4 | TDD FDD CA sustained data rate performance for 4 layer MIMO (4DL CA) | Rel-12 |  | C249 | UE supporting 4DL FDD-TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing (UE DL category 15, 16, 18 and 19) |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.11 is executed, where larger equivalent aggregated bandwidth can be achieved. |
| 8.7.11.5 | TDD FDD CA sustained data rate performance for 4 layer MIMO (5DL CA) | Rel-12 |  | C250 | UE supporting 5DL FDD-TDD CA with 4Rx antenna ports and 4-layer spatial multiplexing (UE DL category 16, 18 and 19) |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest equivalent aggregated bandwidth Note 3 supported by UE. |  |  | Test execution not necessary if another test case in clause 8.7.11 is executed, where larger equivalent aggregated bandwidth can be achieved. |
| 8.7.12.1.2.1 | LAA sustained data rate performance with FDD PCell with 2DL CA (2Rx) | Rel-13 |  | C209 | UE supporting E-UTRA FDD and downlink LAA with FDD as Pcell |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest |  |  |  |
| 8.7.12.1.2.2 | LAA sustained data rate performance with FDD PCell for 4 Layer MIMO (2DL CA) | Rel-13 |  | C328 | UE supporting E-UTRA FDD and downlink LAA with FDD as Pcell abd 4-layer spatial multiplexing |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest |  |  |  |
| 8.7.12.2.2.1 | LAA sustained data rate performance with TDD PCell with 2DL CA (2Rx) | Rel-13 |  | C210 | UE supporting E-UTRA TDD and downlink LAA |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest |  |  |  |
| 8.7.12.2.2.2 | LAA sustained data rate performance with TDD PCell for 4 Layer MIMO (2DL CA) | Rel-13 |  | C329 | UE supporting E-UTRA TDD and downlink LAA with FDD as Pcell abd 4-layer spatial multiplexing |  | One "Test Number" to be performed, in a chosen CA configuration, which leads to the largest |  |  |  |
| 8.8.1.1 | FDD distributed EPDCCH performance | Rel-11 |  | C55 | UE supporting E-UTRA FDD and EPDCCH |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.8.1.2 | TDD distributed EPDCCH performance | Rel-11 |  | C56 | UE supporting E-UTRA TDD and EPDCCH |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.8.2.1 | FDD localized EPDCCH performance with TM9 | Rel-11 to Rel-14 |  | C91 | UE supporting E-UTRA FDD and EPDCCH and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C91m | UE supporting E-UTRA FDD and EPDCCH and Feature Group Indicator 103 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and EPDCCH and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.8.2.2 | TDD localized EPDCCH performance with TM9 | Rel-11 to Rel-14 |  | C92 | UE supporting E-UTRA TDD and EPDCCH and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C92m | UE supporting E-UTRA TDD and EPDCCH and Feature Group Indicator 103 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and EPDCCH and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.8.3.1 | FDD localized EPDCCH transmission with TM10 Type B quasi co-location type | Rel-11 |  | C57 | UE supporting E-UTRA FDD and EPDCCH and Multiple CSI processes on a component carrier within a band with PDSCH transmission mode 10 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.8.3.2 | TDD localized EPDCCH transmission with TM10 Type B quasi co-location type | Rel-11 |  | C58 | UE supporting E-UTRA TDD and EPDCCH and Multiple CSI processes on a component carrier within a band with PDSCH transmission mode 10 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.8.4.1 | FDD Enhanced Downlink Control Channel Performance Requirements Type A for EDPCCH - Localized Transmission with CRS Interference Model | Rel-13 |  | C289 | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |
| 8.8.4.2 | TDD Enhanced Downlink Control Channel Performance Requirements Type A for EDPCCH - Localized Transmission with CRS Interference Model | Rel-13 |  | C290 | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |
| 8.8.5.1 | TDD Enhanced Downlink Control Channel Performance Requirements Type A for EDPCCH - Distributed Transmission with TM9 Interference Model | Rel-13 |  | C290 | E-UTRA TDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |
| 8.8.6.1 | FDD Enhanced Downlink Control Channel Performance Type A for EDPCCH- Distributed Transmission with TM3 Interference Model | Rel-13 |  | C289 | E-UTRA FDD UEs supporting Enhanced downlink control channel performance requirements type A |  |  |  |  |  |
| 8.9.1.1.1 | Transmit diversity performance for UE category 0 (Cell-Specific Reference Symbols) | Rel-12 |  | C145 | UE supporting E-UTRA FDD (UE category 0) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.1.1.1_1 | FDD PDSCH Transmit Diversity 4x1 for UE category 1bis | Rel-13 |  | C145d | UE supporting E-UTRA FDD (UE category 1bis) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.1.1.2 | FDD closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols) | Rel-12 |  | C145 | UE supporting E-UTRA FDD (UE category 0) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.1.1.2_1 | FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 4x1 for UE Category 1bis | Rel-13 |  | C145d | UE supporting E-UTRA FDD (UE category 1bis) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.1.1.3 | FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 0 | Rel-12 |  | C157 | UE supporting E-UTRA FDD (UE category 0) and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.1.1.3_1 | FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 1bis | Rel-13 |  | C157a | UE supporting E-UTRA FDD (UE category 1bis) and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.1.2.1 | TDD PDSCH Transmit Diversity for UE category 0 | Rel-12 |  | C156 | UE supporting E-UTRA TDD (UE category 0) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.1.2.1_1 | TDD PDSCH Transmit Diversity for UE category 1bis | Rel-13 |  | C156f | UE supporting E-UTRA TDD (UE category 1bis) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.1.2.2 | TDD closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols) | Rel-12 |  | C145 | UE supporting E-UTRA FDD (UE category 0) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.1.2.2_1 | TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x1 for UE Category 1bis | Rel-13 |  | C156f | UE supporting E-UTRA TDD (UE category 1bis) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.1.2.3 | TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 0 | Rel-12 |  | C158 | UE supporting E-UTRA TDD (UE category 0) and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.1.2.3_1 | TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE category 1bis | Rel-13 |  | C158a | UE supporting E-UTRA TDD (UE category 1bis) and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.2.1.1 | FDD PHICH Transmit Diversity for UE category 0 | Rel-12 |  | C145 | UE supporting E-UTRA FDD (UE category 0) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.2.1.1_1 | FDD PHICH Transmit Diversity for UE category 1bis | Rel-13 |  | C145d | UE supporting E-UTRA FDD (UE category 1bis) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.2.2.1 | TDD PHICH Transmit Diversity for UE category 0 | Rel-12 |  | C156 | UE supporting E-UTRA TDD (UE category 0) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.9.2.2.1_1 | TDD PHICH Transmit Diversity for UE category 1bis | Rel-13 |  | C156f | UE supporting E-UTRA TDD (UE category 1bis) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.10.1.1.1 | FDD PDSCH Transmit Diversity 2x4 | Rel-10 |  | C113b | UE supporting E-UTRA FDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.1.2 | FDD PDSCH Open Loop Spatial Multiplexing 2x4 | Rel-10 |  | C113b | UE supporting E-UTRA FDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.1.3 | FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model – Enhanced Performance Requirement Type A | Rel-11 |  | C113d | UE supporting E-UTRA FDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.1.4 | FDD PDSCH Closed Loop Spatial Multiplexing 4x4 | Rel-10 |  | C113b | UE supporting E-UTRA FDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.1.5 | FDD PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 interference model – Enhanced Performance Requirement Type A | Rel-11 to Rel-14 |  | C113e | UE supporting E-UTRA FDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
|  |  | Rel-15 |  | C113em | UE supporting E-UTRA FDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE and Feature Group Indicator 103 and (UE Category < 8 and 8 < UE Category < 11 and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.10.1.1.6 | FDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) | Rel-10 to Rel-14 |  | C113c | UE supporting E-UTRA FDD and Feature Group Indicator 103 and 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
|  |  | Rel-15 |  | C113cm | UE supporting E-UTRA FDD and Feature Group Indicator 103 and 4Rx antenna ports and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and 4Rx antenna ports (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.10.1.1.7 | FDD Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports | Rel-10 |  | C220 | UE supporting E-UTRA FDD with 4Rx antenna ports and 3-layer spatial multiplexing |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.1.8 | FDD Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas | Rel-10 |  | C220 | UE supporting E-UTRA FDD with 4Rx antenna ports and 4-layer spatial multiplexing |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.1.9 | FDD 4 Layer Spatial Multiplexing (User-Specific Reference Symbols) | Rel-10 to Rel-14 |  | C113c | UE supporting E-UTRA FDD and eDL-MIMO and Feature Group Indicator 103 and 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
|  |  | Rel-15 |  | C113cm | UE supporting E-UTRA FDD and eDL-MIMO and Feature Group Indicator 103 and 4Rx antenna ports and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and eDL-MIMO and 4Rx antenna ports (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.10.1.2.1 | TDD PDSCH Transmit Diversity 2x4 | Rel-10 |  | C198 | UE supporting E-UTRA TDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.2.2 | TDD PDSCH Open Loop Spatial Multiplexing 2x4 | Rel-10 |  | C198 | UE supporting E-UTRA TDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.2.3 | TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model – Enhanced Performance Requirement Type A | Rel-11 |  | C198 a | UE supporting E-UTRA TDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.2.4 | TDD PDSCH Closed Loop Spatial Multiplexing 4x4 | Rel-10 |  | C198 | UE supporting E-UTRA TDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.2.5 | TDD PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 interference model – Enhanced Performance Requirement Type A | Rel-11 |  | C198c | UE supporting E-UTRA TDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.2.6 | TDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) | Rel-10 |  | C198b | UE supporting E-UTRA TDD with 4Rx antenna ports and UE Category 2 or higher and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.2.7 | TDD Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports | Rel-10 |  | C235 | UE supporting E-UTRA TDD with 4Rx antenna ports and 3-layer spatial multiplexing |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.2.8 | TDD Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas | Rel-10 |  | C235 | UE supporting E-UTRA TDD with 4Rx antenna ports and 4-layer spatial multiplexing |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.1.2.9 | TDD 4 Layer Spatial Multiplexing (User-Specific Reference Symbols) | Rel-10 to Rel-14 |  | C183 | UE supporting E-UTRA TDD and UE Category ≥ 5 and Feature Group Indicator 103 and 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
|  |  | Rel-15 |  | C183m | UE supporting E-UTRA TDD and Feature Group Indicator 103 with 4Rx antenna ports and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13)), or UE supporting E-UTRA TDD with 4Rx antenna ports and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.10.2.1.1 | FDD PCFICH/PDCCH Single-antenna Port Performance 1x4 | Rel-10 |  | C113b | UE supporting E-UTRA FDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.2.1.2 | FDD PCFICH/PDCCH Transmit Diversity Performance 2x4 | Rel-10 |  | C113b | UE supporting E-UTRA FDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.2.1.3 | FDD PCFICH/PDCCH Transmit Diversity Performance 4x4 | Rel-10 |  | C113b | UE supporting E-UTRA FDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.2.2.1 | TDD PCFICH/PDCCH Single-antenna Port Performance 1x4 | Rel-10 |  | C184 | UE supporting E-UTRA TDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.2.2.2 | TDD PCFICH/PDCCH Transmit Diversity Performance 2x4 | Rel-10 |  | C184 | UE supporting E-UTRA TDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.2.2.3 | TDD PCFICH/PDCCH Transmit Diversity Performance 4x4 | Rel-10 |  | C184 | UE supporting E-UTRA TDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.3.1.1 | FDD PHICH Single-antenna Port Performance 1x4 | Rel-10 |  | C113b | UE supporting E-UTRA FDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.3.1.2 | FDD PHICH Transmit Diversity Performance 2x4 | Rel-10 |  | C113b | UE supporting E-UTRA FDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.3.1.3 | FDD PHICH Transmit Diversity Performance 4x4 | Rel-10 |  | C113b | UE supporting E-UTRA FDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.3.2.1 | TDD PHICH Single-antenna Port Performance 1x4 | Rel-10 |  | C184 | UE supporting E-UTRA TDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.3.2.2 | TDD PHICH Transmit Diversity Performance 2x4 | Rel-10 |  | C184 | UE supporting E-UTRA TDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.3.2.3 | TDD PHICH Transmit Diversity Performance 4x4 | Rel-10 |  | C184 | UE supporting E-UTRA TDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.4.1.1 | FDD distributed EPDCCH performance 2x4 | Rel-10 |  | C164 | UE supporting E-UTRA FDD and EPDCCH with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.4.1.2 | TDD distributed EPDCCH performance 2x4 | Rel-10 |  | C165 | UE supporting E-UTRA TDD and EPDCCH with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 8.10.4.2.1 | FDD localized EPDCCH performance with TM9 2x4 | Rel-10 to Rel-14 |  | C166 | UE supporting E-UTRA FDD and EPDCCH and Feature Group Indicator 103 with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
|  |  | Rel-15 |  | C166m | UE supporting E-UTRA FDD and EPDCCH and Feature Group Indicator 103 with 4Rx antenna ports and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and EPDCCH with 4Rx antenna ports and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.10.4.2.2 | TDD localized EPDCCH performance with TM9 2x4 | Rel-10 to Rel-14 |  | C167 | UE supporting E-UTRA TDD and EPDCCH and Feature Group Indicator 103 with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
|  |  | Rel-15 |  | C167m | UE supporting E-UTRA TDD and EPDCCH and Feature Group Indicator 103 with 4Rx antenna ports and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and EPDCCH with 4Rx antenna ports (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.11.1.1.1 | FDD and half-duplex FDD Closed-loop spatial multiplexing performance for UE supporting CE | Rel-13 |  | C145e | UE supporting E-UTRA FDD and UE category M1 and TM6 in CEModeA |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-14 |  | C145f | UE supporting E-UTRA FDD and UE category M2 and TM6 in CEModeA |  |  |  |  | Note 6 |
| 8.11.1.1.2 | FDD and half-duplex FDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for UE supporting CE | Rel-13 |  | C145g | UE supporting E-UTRA FDD and UE category M1 and TM9 in CEModeA |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-14 |  | C145h | UE supporting E-UTRA FDD and UE category M2 and TM9 in CEModeA |  |  |  |  |  |
| 8.11.1.1.3.1 8.11.1.1.3.1_1 | FDD and half-duplex FDD PDSCH Transmit Diversity 2x1 for UE category M1 FDD and half-duplex FDD PDSCH Transmit Diversity 2x1 for UE category M1 (CEmodeB) | Rel-13 |  | C145a | UE supporting E-UTRA FDD and UE category M1 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-13 |  | C156c | UE supporting E-UTRA FDD and (UE category M1 and CEModeB) |  |  |  |  |  |
| 8.11.1.2.1 | TDD Closed-loop spatial multiplexing performance for CE UE in CE Mode A (Cell-Specific Reference Symbols) | Rel-13 |  | C156q | UE supporting E-UTRA TDD and UE category M1 and TM6 in CEModeA |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.11.1.1.3.1_1 | FDD and half-duplex FDD PDSCH Transmit Diversity 2x1 for UE category M1 (CEmodeB) | Rel-14 |  | C156h | UE supporting E-UTRA TDD and UE category M2 and TM6 in CEModeA |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.11.1.2.2 | TDD PDSCH Single-layer Spatial Multiplexing on antenna ports 7 or 8 for CE UE in CE Mode A | Rel-13 |  | C156e | UE supporting E-UTRA TDD and UE category M1 and TM9 in CEModeA |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-14 |  | C156i | UE supporting E-UTRA TDD and UE category M2 and TM9 in CEModeA |  |  |  |  |  |
| 8.11.1.2.3.1 8.11.1.2.3.1_1 | TDD PDSCH Transmit Diversity for UE category M1 TDD PDSCH Transmit Diversity for UE category M1 (CEModeB) | Rel-13 |  | C156b | UE supporting E-UTRA TDD and UE category M1 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-13 |  | C156d | UE supporting E-UTRA TDD and UE category M1 and CEModeB |  |  |  |  |  |
| 8.11.1.2.3.2 | TDD PDSCH 2 Tx Antenna Port supporting wideband transmission for UE category M2 | Rel-14 |  | C316a | UE supporting E-UTRA TDD and UE category M2 and CEModeA |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.11.1.2.3.2_1, | TDD PDSCH 2 Tx Antenna Port supporting wideband transmission for UE category M2 (CEModeB) | Rel-14 |  | C316b | UE supporting E-UTRA TDD and UE category M2 and CEModeB |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.11.2.1.1 | FDD demodulation of MPDCCH in CE Mode A | Rel-13 |  | C145b | UE supporting E-UTRA FDD and (UE category M1 or CEModeA) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-14 |  | C314 | UE supporting E-UTRA FDD and (UE category M2 or CEModeA) |  |  |  |  |  |
| 8.11.2.1.2 | FDD and half-duplex FDD demodulation of MPDCCH in CE Mode B | Rel-13 |  | C156c | UE supporting E-UTRA FDD and (UE category M1 and CEModeB) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-14 |  | C315 | UE supporting E-UTRA FDD and (UE category M1 and CEModeB) |  |  |  |  |  |
| 8.11.2.2.1 | TDD demodulation of MPDCCH in CE Mode A | Rel-13 |  | C156b | UE supporting E-UTRA TDD and (UE category M1 or UE category M2 or CEModeA) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-14 |  | C316 | UE supporting E-UTRA TDD and (UE category M2 or CEModeA) |  |  |  |  |  |
| 8.11.2.2.2 | TDD demodulation of MPDCCH in CE Mode B | Rel-13 |  | C156d | UE supporting E-UTRA TDD and (UE category M1 or UE category M2 and CEModeB) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-14 |  | C317 | UE supporting E-UTRA TDD and (UE category M1 and CEModeB) |  |  |  |  |  |
| 8.12.1.1.1 | Demodulation of NPDSCH (Cell-Specific Reference Symbols) in In-band mode for Category NB1 and NB2 | Rel-13 |  | C347 | UE supporting NB-IoT FDD |  | Each "Test Number" to be performed once, in a chosen band |  |  |  |
| 8.12.1.1.2 | Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone and Guard-band mode for category NB1 and NB2 | Rel-13 |  | C347 | UE supporting NB-IoT FDD |  | Each "Test Number" to be performed once, in a chosen band |  |  |  |
| 8.12.1.1.3 | Demodulation of NPDSCH (Cell-Specific Reference Symbols) in standalone for NB2 | Rel-14 |  | C298 | UE supporting E-UTRA FDD and category NB2 |  | Each “Test Number” to be performed once, in a chosen band |  |  |  |
| 8.12.2.1.1 | Demodulation of NPDCCH single-antenna performance for category NB1 and NB2 | Rel-13 |  | C347 | UE supporting NB-IoT FDD |  | Each "Test Number" to be performed once, in a chosen band |  |  |  |
| 8.12.2.1.2 | Demodulation of NPDCCH in In-band mode Transmit Diversity performance for Category NB1 and NB2 | Rel-13 |  | C347 | UE supporting NB-IoT FDD |  | Each "Test Number" to be performed once, in a chosen band |  |  |  |
| 8.13.1.1.3 | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 with 256QAM (2DL CA) | Rel-10 |  | C278 | UE supporting E-UTRA FDD and intra-band contiguous DL CA or inter-band DL CA and 4 Rx antenna ports and 256QAM in DL (UE Category >= 5) |  |  |  |  |  |
| 8.12.2.1.2 | Demodulation of NPDCCH in In-band mode Transmit Diversity performance for Category NB1 and NB2 | Rel-11 |  | C279 | UE supporting E-UTRA FDD and intra-band non-contiguous DL CA and 4 Rx antenna ports and 256QAM in DL (UE Category >= 5) |  | Each "Test Number" to be performed once, in a chosen band |  |  |  |
| 8.13.1.1.4 8.13.1.1.1.2 | FDD PDSCH Closed Loop Four-Layer Spatial Multiplexing for CA (2DL CA) FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (2DL CA) | Rel-11 |  | C280 | UE supporting E-UTRA FDD and 2DL CA with 4Rx antenna ports and 4-layer spatial multiplexing (UE Category >= 5) |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.1.1.1.3 or 8.13.1.1.1.4 or 8.13.1.1.1.5 is executed. |
|  |  | Rel-10 |  | C253 | UE supporting E-UTRA FDD and intra-band contiguous DL CA or inter-band DL CA (UE Category >= 5) and 4Rx antenna ports |  |  |  |  |  |
| 8.13.1.1.4 | FDD PDSCH Closed Loop Four-Layer Spatial Multiplexing for CA (2DL CA) | Rel-11 |  | C254 | UE supporting E-UTRA FDD and intra-band non-contiguous DL CA (UE Category >= 5) and 4Rx antenna ports |  |  |  |  |  |
| 8.13.1.1.1.3 | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (3DL CA) | Rel-10 |  | C255 | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  | Test execution not necessary if 8.13.1.1.1.4 or 8.13.1.1.1.5 is executed. |
|  |  | Rel-11 |  | C256 | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) and 4Rx antenna ports |  |  |  |  |  |
| 8.13.1.1.1.4 8.13.1.1.1.5 | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (4DL CA) FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (5DL CA) | Rel-11 |  | C257 | UE supporting E-UTRA FDD and 4DL with CA configurations in Table 4.1-4 (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  | Test execution not necessary if 8.13.1.1.1.5 is executed. |
|  |  | Rel-11 |  | C258 | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  |  |
| 8.13.1.1.1.4 | FDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (4DL CA) | Rel-12 |  | C259 | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) and 4Rx antenna ports |  |  |  |  |  |
| 8.13.1.2.2 | FDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (2DL CA) | Rel-10 |  | C253 | UE supporting E-UTRA FDD and intra-band contiguous DL CA or inter-band DL CA (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  | Test execution not necessary if 8.13.1.2.3 or 8.13.1.2.4 or 8.13.1.2.5 is executed. |
|  |  | Rel-11 |  | C254 | UE supporting E-UTRA FDD and intra-band non-contiguous DL CA (UE Category >= 5) and 4Rx antenna ports |  |  |  |  |  |
| 8.13.1.2.3 | FDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (3DL CA) | Rel-10 |  | C255 | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  | Test execution not necessary if 8.13.1.2.4 or 8.13.1.2.5 is executed. |
|  |  | Rel-11 |  | C256 | UE supporting E-UTRA FDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) and 4Rx antenna ports |  |  |  |  |  |
| 8.13.1.2.4 | FDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (4DL CA) | Rel-11 |  | C257 | UE supporting E-UTRA FDD and 4DL with CA configurations in Table 4.1-4 (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  | Test execution not necessary if 8.13.1.2.5 is executed. |
| 8.13.1.2.5 | FDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (5DL CA) | Rel-11 |  | C258 | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  |  |
|  |  | Rel-12 |  | C259 | UE supporting E-UTRA FDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) and 4Rx antenna ports |  |  |  |  |  |
| 8.13.1.3.1 | FDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model – Enhanced Performance Requirement Type A (2DL CA) | Rel-11 |  | C281 | UE supporting E-UTRA FDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A (UE Category >= 5) |  |  |  |  |  |
| 8.13.1.4.1 | FDD PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type A (2DL CA) | Rel-11to Rel-14 |  | C282 | UE supporting E-UTRA FDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A and Feature Group Indictor 103 (UE Category >= 5) |  |  |  |  |  |
|  |  | Rel-15 |  | C282m | UE supporting E-UTRA FDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A and Feature Group Indictor 103 and ((5 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.13.2.1.1.2 | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (2DL CA) | Rel-10 |  | C291 | UE supporting E-UTRA TDD and intra-band contiguous DL CA or inter-band DL CA (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.2.1.1.3 or 8.13.2.1.1.4 or 8.13.2.1.1.5 is executed. |
|  |  | Rel-11 |  | C292 | UE supporting E-UTRA TDD and intra-band non-contiguous DL CA (UE Category >= 5) and 4Rx antenna ports |  |  |  |  |  |
| 8.13.2.1.1.3 | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (3DL CA) | Rel-10 |  | C293 | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  | Test execution not necessary if 8.13.2.1.1.4 or 8.13.2.1.1.5 is executed. |
|  |  | Rel-11 |  | C294 | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) and 4Rx antenna ports |  |  |  |  |  |
| 8.13.2.1.1.4 | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (4DL CA) | Rel-11 |  | C295 | UE supporting E-UTRA TDD and 4DL with CA configurations in Table 4.1-4 (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  | Test execution not necessary if 8.13.2.1.1.5 is executed. |
| 8.13.2.1.1.5 | TDD PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 (5DL CA) | Rel-11 |  | C296 | UE supporting E-UTRA TDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  |  |
|  |  | Rel-12 |  | C297 | UE supporting E-UTRA TDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) and 4Rx antenna ports |  |  |  |  |  |
| 8.13.2.2.2 | TDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (2DL CA) | Rel-10 |  | C291 | UE supporting E-UTRA TDD and intra-band contiguous DL CA or inter-band DL CA (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  | Test execution not necessary if 8.13.2.2.3 or 8.13.2.2.4 or 8.13.2.2.5 is executed. |
|  |  | Rel-11 |  | C292 | UE supporting E-UTRA TDD and intra-band non-contiguous DL CA (UE Category >= 5) and 4Rx antenna ports |  |  |  |  |  |
| 8.13.2.2.3 | TDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (3DL CA) | Rel-10 |  | C293 | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  | Test execution not necessary if 8.13.2.2.4 or 8.13.2.2.5 is executed. |
|  |  | Rel-11 |  | C294 | UE supporting E-UTRA TDD and 3DL with CA configurations in Table 4.1-3 (UE Category >= 5) and 4Rx antenna ports |  |  |  |  |  |
| 8.13.2.2.4 | TDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (4DL CA) | Rel-11 |  | C295 | UE supporting E-UTRA TDD and 4DL with CA configurations in Table 4.1-4 (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  | Test execution not necessary if 8.13.2.2.5 is executed. |
| 8.13.2.2.5 | TDD Dual-Layer Spatial Multiplexing 2x4 (User-Specific Reference Symbols) (5DL CA) | Rel-11 |  | C296 | UE supporting E-UTRA TDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.6.5 |  |  |  |
|  |  | Rel-12 |  | C297 | UE supporting E-UTRA TDD and 5DL with CA configurations in Table 4.1-5 (UE Category 8, >= 11) and 4Rx antenna ports |  |  |  |  |  |
| 8.13.2.3.1 | TDD PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model – Enhanced Performance Requirement Type A (2DL CA) | Rel-11 |  | C283 | UE supporting E-UTRA TDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A (UE Category >= 5) |  |  |  |  |  |
| 8.13.2.4.1 | TDD PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type A (2DL CA) | Rel-11 to Rel-14 |  | C284 | UE supporting E-UTRA TDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A and Feature Group Indictor 103 (UE Category >= 5) |  |  |  |  |  |
|  |  | Rel-15 |  | C284m | UE supporting E-UTRA TDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A and Feature Group Indictor 103 and ((5 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.13.3.1.1.2 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for FDD PCell (2DL CA) | Rel-12 to Rel-14 |  | C306 | UE supporting E-UTRA FDD and TDD and 2DL CA with FDD as PCell (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.3.1.1.3 or 8.13.3.1.1.4 or 8.13.3.1.1.5 is executed. |
|  |  | Rel-15 |  | C234m | UE supporting E-UTRA FDD and TDD and 2DL CA with 4Rx antenna ports and Feature Group Indicator 103 and the enhanced performance requirements type A for LTE and TDD as PCell and ((5 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and TDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A for LTE and TDD as PCell and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 8.13.3.1.1.3 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for FDD PCell (3DL CA) | Rel-12 |  | C307 | UE supporting E-UTRA FDD and TDD and 3DL CA with FDD as PCell (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.3.1.1.4 or 8.13.3.1.1.5 is executed. |
| 8.13.3.1.1.4 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for FDD PCell (4DL CA) | Rel-12 |  | C308 | UE supporting E-UTRA FDD and TDD and 4DL CA with FDD as PCell (UE Category >= 8) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.3.1.1.5 is executed. |
| 8.13.3.1.1.5 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for FDD PCell (5DL CA) | Rel-12 |  | C309 | UE supporting E-UTRA FDD and TDD and 5DL CA with FDD as PCell (UE Category >= 8) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.3 |  |  |  |
| 8.13.3.1.2.2 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for TDD PCell (2DL CA) | Rel-12 |  | C310 | UE supporting E-UTRA FDD and TDD and 2DL CA with TDD as PCell (UE Category >=5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.3.1.2.3 or 8.13.3.1.2.4 or 8.13.3.1.2.5 is executed. |
| 8.13.3.1.2.3 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for TDD PCell (3DL CA) | Rel-12 |  | C311 | UE supporting E-UTRA FDD and TDD and 3DL CA with TDD as PCell (UE Category >= 5) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.3.1.2.4 or 8.13.3.1.2.5 is executed. |
| 8.13.3.1.2.4 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for TDD PCell (4DL CA) | Rel-12 |  | C312 | UE supporting E-UTRA FDD and TDD and 4DL CA with TDD as PCell (UE Category >= 8) |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.3.1.2.5 is executed. |
| 8.13.3.1.2.5 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 for TDD PCell (5DL CA) | Rel-12 |  | C322 | UE supporting E-UTRA FDD and TDD and 5DL CA with TDD as PCell (UE Category 8, and Category11 and onwards) and 4Rx antenna ports |  | Refer to 36.521-1 8.1.2.3 |  |  |  |
| 8.13.3.2.3 | TDD-FDD CA Dual-Layer Spatial Multiplexing 2x4 for FDD PCell (User-Specific Reference Symbols) (2DL CA) | Rel-12 |  | C154 | UE supporting E-UTRA FDD and TDD and 2DL CA with FDD as PCell (UE Category >= 5) |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.3.2.4 or 8.13.3.2.5 or 8.13.3.2.6 is executed. |
| 8.13.3.2.4 | TDD-FDD CA Dual-Layer Spatial Multiplexing 2x4 for FDD PCell (User-Specific Reference Symbols) (3DL CA) | Rel-12 |  | C133 | UE supporting E-UTRA FDD and TDD and 3DL CA with FDD as PCell (UE Category >= 5) |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.3.2.5 or 8.13.3.2.6 is executed. |
| 8.13.3.2.5 | TDD-FDD CA Dual-Layer Spatial Multiplexing 2x4 for FDD PCell (User-Specific Reference Symbols) (4DL CA) | Rel-12 |  | C133a | UE supporting E-UTRA FDD and TDD and 4DL CA with FDD as PCell (UE Category >= 8) |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.3.2.6 is executed. |
| 8.13.3.2.6 | TDD-FDD CA Dual-Layer Spatial Multiplexing 2x4 for FDD PCell (User-Specific Reference Symbols) (5DL CA) | Rel-12 |  | C133b | UE supporting E-UTRA FDD and TDD and 5DL CA with FDD as PCell (UE Category >= 8) |  | Refer to 36.521-1 8.1.2.3 |  |  |  |
| 8.13.3.2.7 | TDD-FDD CA Dual-Layer Spatial Multiplexing 2x4 for TDD PCell (User-Specific Reference Symbols) (2DL CA) |  |  | C155 | UE supporting E-UTRA FDD and TDD and 2DL CA with TDD as PCell (UE Category >=5) |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.3.2.8 or 8.13.3.2.9 or 8.13.3.2.10 is executed. |
| 8.13.3.2.8 | TDD-FDD CA Dual-Layer Spatial Multiplexing 2x4 for TDD PCell (User-Specific Reference Symbols) (3DL CA) |  |  | C135 | UE supporting E-UTRA FDD and TDD and 3DL CA with TDD as PCell (UE Category >= 5) |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.3.2.9 or 8.13.3.2.10 is executed. |
| 8.13.3.2.9 | TDD-FDD CA Dual-Layer Spatial Multiplexing 2x4 for TDD PCell (User-Specific Reference Symbols) (4DL CA) |  |  | C135a | UE supporting E-UTRA FDD and TDD and 4DL CA with TDD as PCell (UE Category >= 8) |  | Refer to 36.521-1 8.1.2.3 |  |  | Test execution not necessary if 8.13.3.2.10 is executed. |
| 8.13.3.2.10 | TDD-FDD CA Dual-Layer Spatial Multiplexing 2x4 for TDD PCell (User-Specific Reference Symbols) (5DL CA) |  |  | C135b | UE supporting E-UTRA FDD and TDD and 5DL CA with TDD as PCell (UE Category 8, and Category11 and onwards) |  | Refer to 36.521-1 8.1.2.3 |  |  |  |
| 8.13.3.3.1 | TDD-FDD CA PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model-Enhanced Performance Requirement Type A for FDD Pcell (2DL CA) | Rel-12 |  | C231 | UE supporting E-UTRA FDD and TDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A for LTE and FDD as PCell (UE Category >=5) |  |  |  |  |  |
| 8.13.3.3.2 | TDD-FDD CA PDSCH Closed Loop Single Layer Spatial Multiplexing 2x4 with TM4 Interference Model-Enhanced Performance Requirement Type A for TDD Pcell (2DL CA) | Rel-12 |  | C232 | UE supporting E-UTRA FDD and TDD and 2DL CA with 4Rx antenna ports and the enhanced performance requirements type A for LTE and TDD as PCell (UE Category >=5) |  |  |  |  |  |
| 8.13.3.4.1 | TDD-FDD CA PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type A for FDD PCell (2DL CA) | Rel-12 |  | C233 | UE supporting E-UTRA FDD and TDD and 2DL CA with 4Rx antenna ports and Feature Group Indictor 103 and the enhanced performance requirements type A for LTE and FDD as PCell (UE Category >=5) |  |  |  |  |  |
| 8.13.3.4.2 | TDD-FDD CA PDSCH Single-layer Spatial Multiplexing 2x4 on antenna ports 7 or 8 with TM9 Interference Model - Enhanced Performance Requirement Type A for TDD PCell (2DL CA) | Rel-12 |  | C234 | UE supporting E-UTRA FDD and TDD and 2DL CA with 4Rx antenna ports and Feature Group Indictor 103 and the enhanced performance requirements type A for LTE and TDD as PCell (UE Category >=5) |  |  |  |  |  |
| 8.13.3.6.1 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 with 256QAM for FDD PCell (2DL CA) | Rel-12 |  | C251 | UE supporting E-UTRA FDD and TDD and 2DL CA with 4Rx antenna ports and 256QAM in DL and FDD as Pcell(UE Category >=5) |  |  |  |  |  |
| 8.13.3.6.2 | TDD-FDD CA PDSCH Closed Loop Multi Layer Spatial Multiplexing 4x4 with 256QAM for TDD PCell (2DL CA) | Rel-12 |  | C252 | UE supporting E-UTRA FDD and TDD and 2DL CA with 4Rx antenna ports and 256QAM in DL and TDD as Pcell(UE Category >=5) |  |  |  |  |  |
| 8.14.1.1.1 | FDD slot/subslot-PDSCH Open Loop Spatial Multiplexing Performance (Cell-Specific Reference Symbols) | Rel-15 |  | C354 | UE supporting E-UTRA FDD and slot/subslot TTI (UE categories >=2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.14.1.1.2 | FDD slot/subslot-PDSCH Closed Loop Spatial Multiplexing Performance (User-Specific Reference Signals) | Rel-15 |  | C355 | UE supporting E-UTRA FDD and Feature Group Indicator 103 and slot/subslot TTI |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.14.1.2.1 | TDD slot-PDSCH Open Loop Spatial Multiplexing Performance (Cell-Specific Reference Symbols) | Rel-15 |  | C356 | UE supporting E-UTRA TDD and slot TTI |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.14.1.2.2 | TDD slot-PDSCH Closed Loop Spatial Multiplexing Performance (User-Specific Reference Signals) | Rel-15 |  | C355a | UE supporting E-UTRA TDD and Feature Group Indicator 103 and slot TTI |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.14.2.1 | FDD SPDCCH Transmit diversity performance | Rel-15 |  | C355 | UE supporting E-UTRA FDD and Feature Group Indicator 103 and slot/subslot TTI |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 8.14.2.2 | TDD SPDCCH Transmit diversity performance | Rel-15 |  | C355a | UE supporting E-UTRA TDD and Feature Group Indicator 103 and slot TTI |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| Reporting of Channel State Information |  |  |  |  |  |  |  |  |  |  |
| 9.2.1.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 | Rel-8 |  | C01 | UE supporting E-UTRA FDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.2.1.2 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 | Rel-8 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.2.1.3_C.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for eICIC (non-MBSFN ABS) | Rel-10 |  | C29 | UE supporting E-UTRA FDD and Feature Group Indicator 115 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.2.1.4_C.1 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for eICIC (non-MBSFN ABS) | Rel-10 |  | C30 | UEs supporting E-UTRA TDD and Feature Group Indictor 115 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.2.1.5_E.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for feICIC (non-MBSFN ABS) | Rel-11 |  | C77 | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.2.1.6_E.1 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for feICIC (non-MBSFN ABS) | Rel-11 |  | C78 | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115(UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.2.1.7 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for 256QAM in DL | Rel-12 |  | C01h | UE supporting E-UTRA FDD and 256QAM in DL(UE category 11-12 and UE DL category >=11) |  |  | 2Rx, 4Rx |  | Note 7 |
| 9.2.1.8 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for 256QAM in DL | Rel-12 |  | C02h | UE supporting E-UTRA TDD and 256QAM in DL(UE category 11-12 and UE DL category >=11) |  |  | 2Rx, 4Rx |  | Note 7 |
| 9.2.2.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-1 | Rel-8 |  | C13 b | UE supporting E-UTRA FDD (UE categories >=2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.2.2.2 | TDD CQI Reporting under AWGN conditions - PUCCH 1-1 | Rel-8 to Rel-14 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  |  |
|  |  | Rel-15 |  | C25m | UE supporting E-UTRA FDD and Feature Group Indicator 103 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 Note 7 |
| 9.2.3.1A | FDD CQI Reporting under AWGN conditions – PUCCH 1-1 for FD-MIMO (With channelMeasRestriction configured) | Rel-13 |  | C337 | UE supporting E-UTRA FDD and Feature Group Indicator 103 and channeleMeasRestriction-r13 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.2.3.1_D | FDD CQI Reporting under AWGN conditions - PUCCH 1-1 for eDL-MIMO | Rel-10 to Rel-14 |  | C25a | UE supporting E-UTRA FDD and Feature Group Indicators 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C25m | UE supporting E-UTRA FDD and Feature Group Indicator 103 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.2.3.2A | TDD CQI Reporting under AWGN conditions – PUCCH 1-1 for FD-MIMO (With channelMeasRestriction configured) | Rel-13 |  | C338 | UE supporting E-UTRA TDD and Feature Group Indicator 103 and channeleMeasRestriction-r13 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.2.3.2_D | TDD CQI Reporting under AWGN conditions - PUCCH 1-1 for eDL-MIMO | Rel-10 to Rel-14 |  | C26a | UE supporting E-UTRA TDD and Feature Group Indicators 104 and 110 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C26am | UE supporting E-UTRA TDD and Feature Group Indicators 104 and 110 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and Feature Group Indicator 110 and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.2.4.1A | FDD CQI Reporting under AWGN conditions - Single CSI Process With interferenceMeasRestriction configured | Rel-13 |  | C339 | UE supporting E-UTRA FDD and Maximum CSI processes of One, Three or Four on a component carrier within a band with PDSCH transmission mode 10 and interferenceMeasRestriction-13 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.2.4.1_F | FDD CQI Reporting under AWGN conditions - Single CSI Process for CoMP | Rel-11 |  | C117 | UE supporting E-UTRA FDD and Maximum CSI processes of One, Three or Four on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.2.4.2A | TDD CQI Reporting under AWGN conditions - Single CSI Process With interferenceMeasRestriction configured | Rel-13 |  | C340 | UE supporting E-UTRA TDD and Maximum CSI processes of One, Three or Four on a component carrier within a band with PDSCH transmission mode 10 and interferenceMeasRestriction-13 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.2.4.2_F | TDD CQI Reporting under AWGN conditions - Single CSI Process for CoMP | Rel-11 |  | C118 | UE supporting E-UTRA TDD and Maximum CSI processes of One, Three or Four on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.2.5 | TDD CQI Reporting under AWGN conditions – PUCCH 1-1 (when csi-SubframeSet –r12 and EIMTA-MainConfigServCell-r12 are configured) | Rel-12 |  | C275 | UE supporting E-UTRA TDD and eIMTA TDD UL-DL reconfiguration and Rel-12 CSI subframe sets (UE Category >= 2) |  |  |  |  |  |
| 9.2.6.1 | LAA CQI Reporting under AWGN Conditions with Frame Structure Type 3 with FDD as Pcell (PUSCH 3-0) | Rel-13 |  | C209 | UE supporting E-UTRA FDD and downlink LAA with FDD as Pcell |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.2.6.2 | LAA CQI Reporting under AWGN Conditions with Frame Structure Type 3 with TDD as Pcell (PUSCH 3-0) | Rel-13 |  | C217 | UE supporting E-UTRA TDD and downlink LAA with TDD as Pcell |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.2.7.1 | LAA CQI Reporting under AWGN Conditions with Frame Structure Type 3 with FDD as Pcell (PUSCH 3-1) | Rel-13 |  | C218 | UE supporting E-UTRA FDD and downlink LAA with FDD as Pcell and TM9 on LAA cells |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.2.7.2 | LAA CQI Reporting under AWGN Conditions with Frame Structure Type 3 with TDD as Pcell (PUSCH 3-1) | Rel-13 |  | C219 | UE supporting E-UTRA TDD and downlink LAA and TM9 on LAA cells |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.1.1.1 | FDD CQI Reporting under fading conditions - PUSCH 3-0 | Rel-8 |  | C01 | UE supporting E-UTRA FDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.1.1.2 | TDD CQI Reporting under fading conditions - PUSCH 3-0 | Rel-8 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.1.1.5 | TDD CQI Reporting under fading conditions – PUCCH 3-0 (when csi-SubframeSet –r12 is configured) | Rel-12 |  | C276 | UE supporting E-UTRA TDD and Rel-12 CSI subframe sets (UE category >= 1) |  |  |  |  |  |
| 9.3.1.2.1_D | FDD CQI Reporting under fading conditions - PUSCH 3-1 for eDL-MIMO | Rel-10 to Rel-14 |  | C25 | UE supporting E-UTRA FDD and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  |  |
|  |  | Rel-15 |  | C25m | UE supporting E-UTRA FDD and Feature Group Indicator 103 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 Note 7 |
| 9.3.1.2.2_D | TDD CQI Reporting under fading conditions - PUSCH 3-1 for eDL-MIMO | Rel-10 to Rel-14 |  | C25a | UE supporting E-UTRA TDD and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  |  |
|  |  | Rel-15 |  | C25am | UE supporting E-UTRA TDD and Feature Group Indicator 103 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 Note 7 |
| 9.3.1.2.3 | FDD CQI Reporting under fading conditions - PUSCH 3-1 for 256QAM in DL | Rel-12 to Rel-14 |  | C260 | UE supporting E-UTRA FDD and 256QAM in DL(UE category 11-12 and UE DL category >=11) and Feature Group Indicator 103 |  |  | 2Rx, 4Rx |  |  |
|  |  | Rel-15 |  | C260m | UE supporting E-UTRA FDD and 256QAM in DL and Feature Group Indicator 103 and UE DL Category = 13, or UE supporting E-UTRA FDD and 256QAM in DL and (UE category 11-12 and UE DL category 11, 12, 14 or higher) |  |  |  |  | Note 6 Note 7 |
| 9.3.1.2.4 | TDD CQI Reporting under fading conditions - PUSCH 3-1 for 256QAM in DL | Rel-12 to Rel-14 |  | C261 | UE supporting E-UTRA TDD and 256QAM in DL(UE category 11-12 and UE DL category >=11) and Feature Group Indicator 103 |  |  | 2Rx, 4Rx |  |  |
|  |  | Rel-15 |  | C261m | UE supporting E-UTRA TDD and 256QAM in DL and Feature Group Indicator 103 and UE DL Category = 13, or UE supporting E-UTRA TDD and 256QAM in DL and (UE category 11-12 and UE DL category 11, 12, 14 or higher) |  |  |  |  | Note 6 Note 7 |
| 9.3.1.2.6 | TDD CQI Reporting under fading conditions – PUCCH 3-1 (when csi-SubframeSet –r12 is configured with one CSI process) | Rel-12 |  | C277 | UE supporting E-UTRA TDD and Rel-12 CSI subframe sets and TM10 ( UE Category >= 1). |  |  | 2Rx |  |  |
| 9.3.1.3.1_E.1 | FDD CQI Reporting under fading conditions - PUSCH 3-0 for feICIC (non-MBSFN ABS) | Rel-11 |  | C79 | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.1.3.2_E.1 | TDD CQI Reporting under fading conditions - PUSCH 3-0 for feICIC (non-MBSFN ABS) | Rel-11 |  | C80 | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.2.1.1 | FDD CQI Reporting under fading conditions - PUCCH 1-0 | Rel-8 |  | C13 b | UE supporting E-UTRA FDD (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.2.1.1_1 | FDD CQI Reporting under fading conditions - PUCCH 1-0 (Release 9 and forward) | Rel-9 |  | C15 | UE supporting E-UTRA FDD (UE category 1) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.2.1.2 | TDD CQI Reporting under fading conditions - PUCCH 1-0 | Rel-8 |  | C14 | UE supporting E-UTRA TDD (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.2.1.2_1 | TDD CQI Reporting under fading conditions - PUCCH 1-0 (Release 9 and forward) | Rel-9 |  | C16 | UE supporting E-UTRA TDD (UE category 1) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.2.2.1_D | FDD CQI Reporting under fading conditions - PUCCH 1-1 for eDL-MIMO | Rel-10 to Rel-14 |  | C25x | UE supporting E-UTRA FDD and Feature Group Indicator 103 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-15 |  | C25xm | UTRA FDD and Feature Group Indicator 103 and ((2 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.3.2.2.2_D | TDD CQI Reporting under fading conditions - PUCCH 1-1 for eDL-MIMO | Rel-10 to Rel-14 |  | C28y | UE supporting E-UTRA TDD and Feature Group Indicators 104 and 110 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-15 |  | C28ym | UE supporting E-UTRA TDD and Feature Group Indicators 104 and 110 and ((2 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and Feature Group Indicator 110 and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.3.3.1.1 | FDD CQI Reporting under fading conditions and frequency-selective interference - PUSCH 3-0 | Rel-8 |  | C01 | UE supporting E-UTRA FDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.3.1.2 | TDD CQI Reporting under fading conditions and frequency-selective interference - PUSCH 3-0 | Rel-8 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.4.1.1 | FDD CQI Reporting under fading conditions - PUSCH 2-0 | Rel-9 |  | C32 | UE supporting E-UTRA FDD and Feature Group Indicator 1 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.4.1.2 | TDD CQI Reporting under fading conditions - PUSCH 2-0 | Rel-9 |  | C37 | UE supporting E-UTRA TDD and Feature Group Indicator 1 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.4.2.1 | FDD CQI Reporting under fading conditions - PUCCH 2-0 | Rel-9 |  | C36 | UE supporting E-UTRA FDD and Feature Group Indicator 2 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.4.2.2 | TDD CQI Reporting under fading conditions - PUCCH 2-0 | Rel-9 |  | C38 | UE supporting E-UTRA TDD and Feature Group Indicator 2 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.5.1.1 | FDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A | Rel-11 |  | C44 | UE supporting E-UTRA FDD and the enhanced performance requirements type A for LTE |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.5.1.2 | TDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A | Rel-11 |  | C45 | UE supporting E-UTRA TDD and the enhanced performance requirements type A for LTE |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.5.2.1 | FDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A | Rel-11 |  | C44z | UE supporting E-UTRA FDD and the enhanced performance requirements type A for LTE (UE Category >= 2) and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.5.2.2 | TDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A | Rel-11 |  | C45i | UE supporting E-UTRA TDD and the enhanced performance requirements type A for LTE (UE Category >= 2) and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.6.1_F.1 | FDD CQI Reporting under fading conditions with Single CSI process for CoMP | Rel-11 |  | C50a | UE supporting E-UTRA FDD and Maximum CSI processes of One on a component carrier within a band with PDSCH transmission mode 10 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.6.1_F.2 | FDD CQI Reporting under fading conditions with Three CSI processes for CoMP | Rel-11 |  | C96 | UE supporting E-UTRA FDD and Maximum CSI processes of Three on a component carrier within a band with PDSCH transmission mode 10 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.6.1_F.3 | FDD CQI Reporting under fading conditions with Four CSI processes for CoMP | Rel-11 |  | C97 | UE supporting E-UTRA FDD and Maximum CSI processes of Four on a component carrier within a band with PDSCH transmission mode 10 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.6.2_F.1 | TDD CQI Reporting under fading conditions with Single CSI process for CoMP | Rel-11 |  | C51a | UE supporting E-UTRA TDD and Maximum CSI processes of One on a component carrier within a band with PDSCH transmission mode 10 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.6.2_F.2 | TDD CQI Reporting under fading conditions with Three CSI processes for CoMP | Rel-11 |  | C98 | UE supporting E-UTRA TDD and Maximum CSI processes of Three on a component carrier within a band with PDSCH transmission mode 10 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.6.2_F.3 | TDD CQI Reporting under fading conditions with Four CSI processes for CoMP | Rel-11 |  | C99 | UE supporting E-UTRA TDD and Maximum CSI processes of Four on a component carrier within a band with PDSCH transmission mode 10 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.3.7.1 | FDD CQI Reporting under fading conditions - PUSCH 3-2 for eDL MIMO Enhancement | Rel-12 to Rel-14 |  | C25b | UE supporting E-UTRA FDD and Feature Group Indicator 103 and PUSCH feedback mode 3-2 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | 9.3.7.1 Note 7 |
|  |  | Rel-15 |  | C25bm | UE supporting E-UTRA FDD and Feature Group Indicator 103 and PUSCH feedback mode 3-2 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and PUSCH feedback mode 3-2 and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.3.7.2 | TDD CQI Reporting under fading conditions - PUSCH 3-2 for eDL MIMO Enhancement | Rel-12 to Rel-14 |  | C25c | UE supporting E-UTRA TDD and Feature Group Indicator 103 and PUSCH feedback mode 3-2 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-15 |  | C25cm | UE supporting E-UTRA TDD and Feature Group Indicator 103 and PUSCH feedback mode 3-2 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and PUSCH feedback mode 3-2 and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.3.8.1.1 | FDD CQI Reporting under fading conditions - PUCCH 1-1 (Cell-Specific Reference Symbols) TM4 - Enhanced Receiver Type B | Rel-12 |  | C152 | UE supporting E-UTRA FDD and the enhanced performance requirements type B for LTE (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.8.1.2 | TDD CQI Reporting under fading conditions - PUCCH 1-1 (Cell-Specific Reference Symbols) TM4 - Enhanced Receiver Type B | Rel-12 |  | C153 | UE supporting E-UTRA TDD and the enhanced performance requirements type B for LTE (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.8.2.1 | FDD CQI Reporting under fading conditions - PUCCH 1-1 (CSI Reference Symbol) TM9 - Enhanced Receiver Type B | Rel-12 |  | C152 | UE supporting E-UTRA FDD and the enhanced performance requirements type B for LTE (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.8.2.2 | TDD CQI Reporting under fading conditions - PUCCH 1-1 (CSI Reference Symbol) TM9 - Enhanced Receiver Type B | Rel-12 |  | C153 | UE supporting E-UTRA TDD and the enhanced performance requirements type B for LTE (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.8.3.1 | FDD CQI Reporting under fading conditions - PUCCH 1-1 (CSI Reference Symbol) TM10 with TM9 interference - Enhanced Receiver Type B | Rel-12 |  | C152 | UE supporting E-UTRA FDD and the enhanced performance requirements type B for LTE (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.3.8.3.2 | TDD CQI Reporting under fading conditions - PUCCH 1-1 (CSI Reference Symbol) TM10 with TM9 interference - Enhanced Receiver Type B | Rel-12 |  | C153 | UE supporting E-UTRA TDD and the enhanced performance requirements type B for LTE (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.4.1.1.1 | FDD PMI Reporting - PUSCH 3-1 (Single PMI) | Rel-8 |  | C01 | UE supporting E-UTRA FDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.1.1.2 | TDD PMI Reporting - PUSCH 3-1 (Single PMI) | Rel-8 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.1.2.1 | FDD PMI Reporting - PUCCH 2-1 (Single PMI) | Rel-9 |  | C36 | UE supporting E-UTRA FDD and Feature Group Indicator 2 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.1.2.2 | TDD PMI Reporting - PUCCH 2-1 (Single PMI) | Rel-9 |  | C38 | UE supporting E-UTRA TDD and Feature Group Indicator 2 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.1.3.1_D | FDD PMI Reporting - PUSCH 3-1 (Single PMI) for eDL-MIMO | Rel-10 to Rel-14 |  | C25 | UE supporting E-UTRA FDD and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-15 |  | C25m | UE supporting E-UTRA FDD and Feature Group Indicator 103 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.4.1.3.2_D | TDD PMI Reporting - PUSCH 3-1 (Single PMI) for eDL-MIMO | Rel-10 to Rel-14 |  | C26 | UE supporting E-UTRA TDD and Feature Group Indicator 104 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-15 |  | C26m | UE supporting E-UTRA TDD and Feature Group Indicator 104 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.4.1.3.3 | FDD PMI Reporting with 12Tx Class A codebook – PUSCH 3-1 (Single PMI) for FD-MIMO | Rel-13 |  | C13b | UE supporting E-UTRA FDD (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.1.3.4 | TDD PMI Reporting with 12Tx Class A codebook – PUSCH 3-1 (Single PMI) for FD-MIMO | Rel-13 |  | C14b | UE supporting E-UTRA TDD (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.1.4.1 | FDD PMI Reporting with 4Tx enhanced codebook - PUCCH 1-1 (Single PMI) for eDL MIMO Enhancement | Rel-12 to Rel-14 |  | C25d | UE supporting E-UTRA FDD and Feature Group Indicator 103 and enhanced 4Tx codebook |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-15 |  | C25m | UE supporting E-UTRA FDD and Feature Group Indicator 103 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.4.1.4.2 | TDD PMI Reporting with 4Tx enhanced codebook - PUCCH 1-1 (Single PMI) for eDL MIMO Enhancement | Rel-12 to Rel-14 |  | C25e | UE supporting E-UTRA TDD and Feature Group Indicator 103 and enhanced 4Tx codebook |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-15 |  | C25em | UE supporting E-UTRA TDD and Feature Group Indicator 103 and enhanced 4Tx codebook and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and enhanced 4Tx codebook and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.4.1.4.3 | FDD PMI Reporting with Class B alternative codebook – PUCCH 1-1 for FD-MIMO | Rel-13 |  | C13b | UE supporting E-UTRA FDD (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.1.4.4 | TDD PMI Reporting with Class B alternative codebook – PUCCH 1-1 for FD-MIMO | Rel-13 |  | C14b | UE supporting E-UTRA TDD (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.2.1.1 | FDD PMI Reporting - PUSCH 1-2 (Multiple PMI) | Rel-8 only |  | C11 | UE supporting E-UTRA FDD and operating bands supporting 20 MHz Bandwidth (UE categories 2, 3, 4, 5) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.2.1.1_1 | FDD PMI Reporting - PUSCH 1-2 (Multiple PMI) (Release 9 and forward) | Rel-9 |  | C01 | UE supporting E-UTRA FDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.2.1.2 | TDD PMI Reporting - PUSCH 1-2 (Multiple PMI) | Rel-8 only |  | C12 | UE supporting E-UTRA TDD and operating bands supporting 20 MHz Bandwidth (UE categories 2, 3, 4, 5) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.2.1.2_1 | TDD PMI Reporting - PUSCH 1-2 (Multiple PMI) (Release 9 and forward) | Rel-9 |  | C02 | UE supporting E-UTRA TDD |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.2.2.1 | FDD PMI Reporting - PUSCH 2-2 (Multiple PMI) | Rel-9 |  | C32 | UE supporting E-UTRA FDD and Feature Group Indicators 1 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.2.2.2 | TDD PMI Reporting - PUSCH 2-2 (Multiple PMI) | Rel-9 |  | C33 | UE supporting E-UTRA TDD and Feature Group Indicators 1 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.2.3.1_D | FDD PMI Reporting - PUSCH 1-2 (Multiple PMI) for eDL-MIMO | Rel-10 to Rel-14 |  | C25 | UE supporting E-UTRA FDD and Feature Group Indicator 103 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-15 |  | C25m | UE supporting E-UTRA FDD and Feature Group Indicator 103 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.4.2.3.2_D | TDD PMI Reporting - PUSCH 1-2 (Multiple PMI) for eDL-MIMO | Rel-10 to Rel-14 |  | C26 | UE supporting E-UTRA TDD and Feature Group Indicator 104 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C26m | UE supporting E-UTRA TDD and Feature Group Indicator 104 and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.4.2.3.3 | FDD PMI Reporting with 4Tx enhanced codebook - PUSCH 1-2 (Multiple PMI) for eDL-MIMO Enhancement | Rel-12 to Rel-14 |  | C25d | UE supporting E-UTRA FDD and Feature Group Indicator 103 and enhanced 4Tx codebook |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-15 |  | C25dm | UE supporting E-UTRA FDD and Feature Group Indicator 103 and enhanced 4Tx codebook and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and enhanced 4Tx codebook and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.4.2.3.4 | TDD PMI Reporting with 4Tx enhanced codebook - PUSCH 1-2 (Multiple PMI) for eDL-MIMO Enhancement | Rel-12 to Rel-14 |  | C25e | UE supporting E-UTRA TDD and eDL-MIMO Enhancement and Feature Group Indicator 103 and enhanced 4Tx codebook |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  |  |
|  |  | Rel-15 |  | C25em | UE supporting E-UTRA TDD and Feature Group Indicator 103 and enhanced 4Tx codebook and ((UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and enhanced 4Tx codebook and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.4.2.3.5 | FDD PMI Reporting with Class A 16Tx codebook - PUSCH 1-2 (Multiple PMI) for FD-MIMO | Rel-13 |  | C13b | UE supporting E-UTRA FDD (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.4.2.3.6 | TDD PMI Reporting with Class A 16Tx codebook - PUSCH 1-2 (Multiple PMI) for FD-MIMO | Rel-13 |  | C14b | UE supporting E-UTRA TDD (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.5.1.1 | FDD RI Reporting - PUCCH 1-1 | Rel-8 and Rel-9 only |  | C13a | UE supporting E-UTRA FDD (UE Category 2-5) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.5.1.1_1 | FDD RI Reporting - PUCCH 1-1 (Release 10) | Rel-10 only |  | C13 | UE supporting E-UTRA FDD (UE Category 2-8) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.5.1.1_2 | FDD RI Reporting- PUCCH 1-1 (Release 11) | Rel-11 |  | C13b | UE supporting E-UTRA FDD (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.5.1.2 | TDD RI Reporting - PUSCH 3-1 | Rel-8 and Rel-9 only |  | C14a | UE supporting E-UTRA TDD (UE Category 2-5) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.5.1.2_1 | TDD RI Reporting - PUSCH 3-1 (Release 10) | Rel-10 only |  | C14 | UE supporting E-UTRA TDD (UE Category 2-8) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.5.1.2_2 | TDD RI Reporting- PUSCH 3-1 (Release 11) | Rel-11 |  | C14b | UE supporting E-UTRA TDD (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.5.2.1_D | FDD RI Reporting - PUCCH 1-1 for eDL-MIMO | Rel-10 to Rel-14 |  | C25x | UE supporting E-UTRA FDD and Feature Group Indicators 103 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C25xm | UE supporting E-UTRA FDD and Feature Group Indicator 103 and ((2 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.5.2.2_D | TDD RI Reporting - PUCCH 1-1 for eDL-MIMO | Rel-10 to Rel-14 |  | C25y | UE supporting E-UTRA TDD and Feature Group Indicator 103 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C25ym | UE supporting E-UTRA TDD and Feature Group Indicator 103 and ((2 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.5.3.1_C.1 | FDD RI Reporting - PUCCH 1-0 for eICIC (non-MBSFN ABS) | Rel-10 |  | C29 | UE supporting E-UTRA FDD and Feature Group Indicator 115 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.5.3.2_C.1 | TDD RI Reporting - PUCCH 1-0 for eICIC (non-MBSFN ABS) | Rel-10 |  | C30 | UE supporting E-UTRA TDD and Feature Group Indicator 115 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.5.4.1_E.1 | FDD RI Reporting - PUCCH 1-0 for feICIC (non-MBSFN ABS) | Rel-11 |  | C77 | UE supporting E-UTRA FDD and CRS interference handling and Feature Group Indicator 115 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.5.4.2_E.1 | TDD RI Reporting - PUCCH 1-0 for feICIC (non-MBSFN ABS) | Rel-11 |  | C78 | UE supporting E-UTRA TDD and CRS interference handling and ss-CCH interference handling and Feature Group Indicator 115(UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.5.5.1_F.1 | FDD RI Reporting with Single CSI processes for CoMP | Rel-11 |  | C50 | UE supporting E-UTRA FDD and Maximum CSI processes of One on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.5.5.1_F.2 | FDD RI Reporting with Multiple CSI processes for CoMP | Rel-11 |  | C52 | UE supporting E-UTRA FDD and Maximum CSI processes of Three or Four on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.5.5.2_F.1 | TDD RI Reporting with Single CSI process for CoMP | Rel-11 |  | C51 | UE supporting E-UTRA TDD and Maximum CSI processes of One on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.5.5.2_F.2 | TDD RI Reporting with Multiple CSI processes for CoMP | Rel-11 |  | C53 | UE supporting E-UTRA TDD and Maximum CSI processes of Three or Four on a component carrier within a band with PDSCH transmission mode 10 (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.6.1.1_A.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (2 DL CA) | Rel-10 |  | C108 | UE supporting E-UTRA FDD and intra-band contiguous DL CA or inter-band DL CA (UE Category >= 3) |  | Refer to 36.521-1 9.1.1.2 | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.1_A.2 is executed. Note 7 |
|  |  | Rel-11 |  | C103 | UE supporting E-UTRA FDD and intra-band non-contiguous DL CA(UE Category >= 3) |  |  |  |  |  |
| 9.6.1.1_A.2 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (3 DL CA) | Rel-10 |  | C124 | UE supporting E-UTRA FDD and 3DL with intra-band contiguous CA, or 3DL with inter-band CA, or 3DL with intra-band contiguous and inter-band CA (UE Category >= 5) |  | Refer to 36.521-1 9.1.1.2 | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.1_A.3 is executed. Note 7 |
|  |  | Rel-11 |  | C125 | UE supporting E-UTRA FDD and 3DL with intra-band non-contiguous and inter-band CA, or 3DL with intra-band non-contiguous and intra-band contiguous CA (UE Category >= 5) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.1_A.3 is executed. Note 7 |
| 9.6.1.1_A.3 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (4DL CA) | Rel-11 |  | C192 | UE supporting E-UTRA FDD and 4DL with intra-band contiguous CA, or 4DL with inter-band CA, or 4DL with intra-band contiguous and inter-band CA (UE Category >= 8) |  |  | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.1_A.4 is executed. Note 7 |
|  |  | Rel-11 |  | C193 | UE supporting E-UTRA FDD and 4DL with intra-band non-contiguous and inter-band CA, or 4DL with intra-band non-contiguous and intra-band contiguous CA (UE Category >= 8) |  |  |  |  | Test execution not necessary if 9.6.1.1_A.4 is executed. |
| 9.6.1.1_A.4 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (5DL CA) | Rel-11 |  | C192a | UE supporting E-UTRA FDD and 5DL with Intra-band contiguous and Inter-band CA or 5DL with Intra-band non-contiguous and Inter-band CA or 5DL with Intra-band non-contiguous and Intra-band contiguous CA (UE Category 8 and >= 11) |  |  | 2Rx, 4Rx |  | Note 7 |
|  |  | Rel-12 |  | C193a | UE supporting E-UTRA FDD and 5DL Inter-band CA (UE Category 8 and>= 11) |  |  |  |  |  |
| 9.6.1.1_A.5 | FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for CA (6DL CA) | Rel-14 |  | C363 | UE supporting E-UTRA FDD and 6DL with Intra-band contiguous and Inter-band CA or 6DL with Intra-band non-contiguous and Inter-band CA or 6DL with Inter-band CA (UE Category 8 and >= 11) |  |  | 2Rx, 4Rx |  | Note 7 |
| 9.6.1.1_A.6 | FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for CA (7DL CA) | Rel-14 |  | C364 | UE supporting E-UTRA FDD and 7DL with Intra-band contiguous and Inter-band CA or 7DL with Intra-band non-contiguous and Inter-band CA (UE Category 8 and >= 11) |  |  | 2Rx, 4Rx |  | Note 7 |
| 9.6.1.2_A.1 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (2DL CA) | Rel-10 |  | C114 | UE supporting E-UTRA TDD and intra-band contiguous DL CA (UE Category >= 3) |  | Refer to 36.521-1 9.1.1.2 | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.2_A.2 is executed. Note 7 |
| 9.6.1.2_A .2 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (3 DL CA) | Rel-10 |  | C128 | UE supporting E-UTRA TDD and 3DL with intra-band contiguous CA, or 3DL with inter-band CA, or 3DL with intra-band contiguous and inter-band CA (UE Category >= 5) |  | Refer to 36.521-1 9.1.1.2 | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.2_A.3 is executed. Note 7 |
|  |  | Rel-11 |  | C129 | UE supporting E-UTRA TDD and 3DL with intra-band non-contiguous and inter-band CA, or 3DL with intra-band non-contiguous and intra-band contiguous CA (UE Category >= 5) |  |  |  |  | Test execution not necessary if 9.6.1.2_A.3 is executed. |
| 9.6.1.2_A.3 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (4 DL CA) | Rel-11 |  | C270 | UE supporting E-UTRA TDD and 4DL with intra-band contiguous CA, or 4DL with inter-band CA, or 4DL with intra-band contiguous and inter-band CA (UE Category >= 8) |  |  |  |  | Test execution not necessary if 9.6.1.2_A.4 is executed. |
|  |  | Rel-11 |  | C271 | UE supporting E-UTRA TDD and 4DL with intra-band non-contiguous and inter-band CA, or 4DL with intra-band non-contiguous and intra-band contiguous CA (UE Category >= 8) |  |  |  |  | Test execution not necessary if 9.6.1.2_A.4 is executed. |
| 9.6.1.2_A.4 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (5 DL CA) | Rel-11 |  | C272 | UE supporting E-UTRA TDD and 5DL with intra-band contiguous CA, or 5DL with inter-band CA, or 5DL with intra-band contiguous and inter-band CA (UE Category 8 and >=11) |  |  |  |  |  |
|  |  | Rel-11 |  | C273 | UE supporting E-UTRA TDD and 5DL with intra-band non-contiguous and inter-band CA, or 5DL with intra-band non-contiguous and intra-band contiguous CA (UE Category 8 and >=11) |  |  |  |  |  |
| 9.6.1.2_A.5 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (6 DL CA) | Rel-14 |  | C365 | UE supporting E-UTRA TDD and 6DL with Intra-band contiguous and Inter-band CA or 6DL with Intra-band non-contiguous and Inter-band CA or 6DL with Inter-band CA (UE Category 8 and >= 11) |  |  |  | 2Rx, 4Rx | Note 7 |
| 9.6.1.2_A.6 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 for CA (7 DL CA) | Rel-14 |  | C366 | UE supporting E-UTRA TDD and 7DL with Intra-band contiguous and Inter-band CA or 7DL with Intra-band non-contiguous and Inter-band CA (UE Category 8 and >= 11) |  |  |  | 2Rx, 4Rx | Note 7 |
| 9.6.1.3.1 | TDD FDD CA CQI Reporting under AWGN conditions - PUCCH 1-0 for FDD PCell (2DL CA) | Rel-12 |  | C132 | UE supporting E-UTRA FDD and TDD and 2DL CA with FDD as PCell (UE Category >= 3) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.3.2 is executed. Note 7 |
| 9.6.1.3.2 | TDD FDD CA CQI Reporting under AWGN conditions - PUCCH 1-0 for FDD PCell (3DL CA) | Rel-12 |  | C133 | UE supporting E-UTRA FDD and TDD and 3DL CA with FDD as PCell (UE Category >= 5) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.3.3 is executed. Note 7 |
| 9.6.1.3.3 | TDD FDD CA CQI Reporting under AWGN conditions - PUCCH 1-0 for FDD PCell (4DL CA) | Rel-12 |  | C133a | UE supporting E-UTRA FDD and TDD and 4DL CA with FDD as PCell (UE Category >= 8) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.3.4 is executed. Note 7 |
| 9.6.1.3.4 | TDD FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for FDD PCell (5DL CA) | Rel-12 |  | C133b | UE supporting E-UTRA FDD and TDD and 5DL CA with FDD as PCell (UE Category 8, and Category 11 and onwards) |  | TBD | 2Rx, 4Rx |  | Note 7 |
| 9.6.1.4.1 | TDD FDD CA CQI Reporting under AWGN conditions - PUCCH 1-0 for TDD PCell (2DL CA) | Rel-12 |  | C134 | UE supporting E-UTRA FDD and TDD and 2DL CA with TDD as PCell (UE Category >= 3) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.4.2 is executed. Note 7 |
| 9.6.1.4.2 | TDD FDD CA CQI Reporting under AWGN conditions - PUCCH 1-0 for TDD PCell (3DL CA) | Rel-12 |  | C135 | UE supporting E-UTRA FDD and TDD and 3DL CA with TDD as PCell (UE Category >= 5) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.4.3 is executed. Note 7 |
| 9.6.1.4.3 | TDD FDD CA CQI Reporting under AWGN conditions - PUCCH 1-0 for TDD PCell (4DL CA) | Rel-12 |  | C135a | UE supporting E-UTRA FDD and TDD and 4DL CA with TDD as PCell (UE Category >= 8) |  | TBD | 2Rx, 4Rx |  | Test execution not necessary if 9.6.1.4.4 is executed. Note 7 |
| 9.6.1.4.4 | TDD FDD CQI Reporting under AWGN conditions – PUCCH 1-0 for TDD PCell (5DL CA) | Rel-12 |  | C133b | UE supporting E-UTRA FDD and TDD and 5DL CA with FDD as PCell (UE Category 8, and Category 11 and onwards) |  | TBD | 2Rx, 4Rx |  | Note 7 |
| 9.7.1.1 | FDD and Half duplex FDD CQI reporting definition under AWGN conditions for UE category 0 | Rel-12 |  | C145 | UE supporting E-UTRA FDD (UE category 0) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.7_1.1.1 | FDD CQI reporting under AWGN conditions – PUCCH 1-0 for UE category 1bis | Rel-13 |  | C145d | UE supporting E-UTRA FDD (UE category 1bis) |  | Each "Test Number" to be performed once, in a chosen band supporting |  |  |  |
| 9.7_1.1.2 | TDD CQI reporting under AWGN conditions - PUCCH 1-0 for UE category 1bis | Rel-13 |  | C156f | UE supporting E-UTRA TDD (UE category 1bis) |  | Each "Test Number" to be performed once, in a chosen band supporting |  |  |  |
| 9.7_1.1.3 | FDD CQI reporting under fading conditions - PUSCH 3-0 for UE category 1bis | Rel-13 |  | C145d | UE supporting E-UTRA FDD (UE category 1bis) |  | Each "Test Number" to be performed once, in a chosen band supporting |  |  |  |
| 9.7_1.1.4 | TDD CQI reporting under fading conditions - PUSCH 3-0 for UE category 1bis | Rel-13 |  | C156f | UE supporting E-UTRA TDD (UE category 1bis) |  | Each "Test Number" to be performed once, in a chosen band supporting |  |  |  |
| 9.7.1.2 | TDD CQI reporting definition under AWGN conditions for UE category 0 | Rel-12 |  | C119 | UE supporting E-UTRA TDD (UE category 0) |  |  |  |  |  |
| 9.7.2.1 | FDD and Half duplex FDD CQI reporting definition under fading conditions for UE category 0 | Rel-12 |  | C145 | UE supporting E-UTRA FDD (UE category 0) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.7.2.2 | TDD CQI reporting definition under fading conditions for UE category 0 | Rel-12 |  | C156 | UE supporting E-UTRA TDD (UE category 0) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.8.1.1 | FDD and Half duplex FDD CQI reporting definition under AWGN conditions for UE supporting coverage enhancement | Rel-13 |  | C145a | UE supporting E-UTRA FDD and UE category M1 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.8.1.2 | TDD CQI reporting definition under AWGN conditions for UE supporting coverage enhancement | Rel-13 |  | C156a | UE supporting E-UTRA TDD and UE category M1 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.8.2.1 | FDD and Half-duplex FDD UE-selected subband CQI for UE supporting coverage enhancement | Rel-13 |  | C145a | UE supporting E-UTRA FDD and UE category M1 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.8.2.2 | TDD UE-selected subband CQI for UE supporting coverage enhancement | Rel-13 |  | C156a | UE supporting E-UTRA TDD and UE category M1 |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.8.3.1 | FDD and half duplex FDD CQI reporting under AWGN conditions for UE supporting coverage enhancement and 64QAM | Rel-15 |  | C145i | UE supporting E-UTRA FDD and CEModeA and 64QAM |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.8.3.2 | TDD CQI reporting under AWGN conditions for UE supporting coverage enhancement and 64QAM | Rel-15 |  | C156k | UE supporting E-UTRA TDD and CEModeA and 64QAM |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.8.4.1 | FDD and half duplex FDD CQI reporting under AWGN conditions for UE supporting coverage enhancement alternative table | Rel-15 |  | C145j | UE supporting E-UTRA FDD and CEModeA and alternative table |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.8.4.2 | TDD CQI reporting under AWGN conditions for UE supporting coverage enhancement alternative table | Rel-15 |  | C156l | UE supporting E-UTRA TDD and CEModeA and alternative table |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.9.1.1.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-0 with Rank 1 1x4 | Rel-10 |  | C113b | UE supporting E-UTRA FDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.9.1.1.2 | TDD CQI Reporting under AWGN conditions - PUCCH 1-0 with Rank 1 1x4 | Rel-10 |  | C177 | UE supporting E-UTRA TDD with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.9.1.2.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-1 with rank 2 4x4 | Rel-10 to Rel-14 |  | C178 | UE supporting E-UTRA FDD and eDL-MIMO and Feature Group Indicator 103 with 4Rx antenna ports (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C178m | UE supporting E-UTRA FDD and eDL-MIMO and Feature Group Indicator 103 with 4Rx antenna ports and ((2 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD with 4Rx antenna ports and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.9.1.2.2 | TDD CQI Reporting under AWGN conditions - PUCCH 1-1 with rank 2 8x4 | Rel-10 to Rel-14 |  | C179 | UE supporting E-UTRA TDD and eDL-MIMO and Feature Group Indicator 104 with 4Rx antenna ports (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C179m | UE supporting E-UTRA TDD and eDL-MIMO and Feature Group Indicator 104 with 4Rx antenna ports and ((2 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD with 4Rx antenna ports and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.9.1.3.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-1 with rank 4 4x4 | Rel-10 |  | C180 | UE supporting E-UTRA FDD with 4Rx antenna ports and 4-layer spatial multiplexing (UE Category >= 5) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.9.1.3.2 | TDD CQI Reporting under AWGN conditions - PUCCH 1-1 with rank 4 4x4 | Rel-10 |  | C181 | UE supporting E-UTRA TDD with 4Rx antenna ports and 4-layer spatial multiplexing (UE Category >= 5) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 9.9.1.4.1 | FDD CQI Reporting under AWGN conditions - PUCCH 1-1 with rank 3 4x4 | Rel-10 to Rel-14 |  | C182 | UE supporting E-UTRA FDD and eDL-MIMO and Feature Group Indicator 103 with 4Rx antenna ports (UE Category >= 5) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C182m | UE supporting E-UTRA FDD and eDL-MIMO and Feature Group Indicator 103 with 4Rx antenna ports and ((5 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD with 4Rx antenna ports and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.9.1.4.2 | TDD CQI Reporting under AWGN conditions - PUCCH 1-1 with rank 3 4x4 | Rel-10 to Rel-14 |  | C183 | UE supporting E-UTRA TDD andUE Category ≥ 5 and Feature Group Indicator 103 with 4Rx antenna ports |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C183m | UE supporting E-UTRA TDD and Feature Group Indicator 103 with 4Rx antenna ports and ((5 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD with 4Rx antenna ports and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.9.2.1.1 | FDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A 1x4 | Rel-11 |  | C197 | UE supporting E-UTRA FDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE |  | Each" Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 9.9.2.1.2 | TDD CQI Reporting under fading conditions - PUCCH 1-0 - Enhanced Performance Requirement Type A 1x4 | Rel-11 |  | C198 | UE supporting E-UTRA TDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE |  | Each" Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 9.9.2.2.1 | FDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A2 x4 | Rel-11 to Rel-14 |  | C199 | UE supporting E-UTRA FDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE (UE Category >= 2) and Feature Group Indicator 103 |  | Each" Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
|  |  | Rel-15 |  | C199m | UE supporting E-UTRA FDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE and Feature Group Indicator 103 and ((2 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.9.2.2.2 | TDD CQI Reporting under fading conditions - PUCCH 1-1 - Enhanced Performance Requirement Type A2 x4 | Rel-11 to Rel-14 |  | C200 | UE supporting E-UTRA TDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE (UE Category >= 2) and Feature Group Indicator 103 |  | Each" Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
|  |  | Rel-15 |  | C200m | UE supporting E-UTRA TDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE and Feature Group Indicator 103 and ((2 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD with 4Rx antenna ports and the enhanced performance requirements type A for LTE and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.9.3.1.1 | TDD PMI Reporting - PUSCH 3-1 (Single PMI) 8x4 | Rel-10 to Rel-14 |  | C179 | UE supporting E-UTRA TDD and eDL-MIMO and Feature Group Indicator 104 with 4Rx antenna ports (UE Category >= 2) |  | Each" Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
|  |  | Rel-15 |  | C179m | UE supporting E-UTRA TDD and eDL-MIMO and Feature Group Indicator 104 with 4Rx antenna ports and ((2 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD with 4Rx antenna ports and (UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.9.4.1.1 | FDD RI Reporting- PUCCH 1-1 4x4 | Rel-10 |  | C203 | UE supporting E-UTRA FDD with 4Rx antenna ports (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 9.9.4.1.2 | TDD RI Reporting- PUSCH 3-1 4x4 | Rel-10 |  | C204 | UE supporting E-UTRA TDD with 4Rx antenna ports (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
| 9.9.4.2.1 | FDD RI Reporting- PUCCH 1-1 for eDL-MIMO 4x4 | Rel-10 to Rel-14 |  | C205 | UE supporting E-UTRA FDD and eDL-MIMO and Feature Group Indicator 103 with 4Rx antenna ports (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
|  |  | Rel-15 |  | C205m | UE supporting E-UTRA FDD and eDL-MIMO and Feature Group Indicator 103 with 4Rx antenna ports and ((2 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA FDD with 4Rx antenna ports and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.9.4.2.2 | TDD RI Reporting- PUCCH 1-1 for eDL-MIMO 4x4 | Rel-10 to Rel-14 |  | C206 | UE supporting E-UTRA TDD and eDL-MIMO and Feature Group Indicator 103 with 4Rx antenna ports (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW and 4Rx antenna ports |  |  |  |
|  |  | Rel-15 |  | C206m | UE supporting E-UTRA TDD and eDL-MIMO and Feature Group Indicator 103 with 4Rx antenna ports and ((2 <= UE Category < 8 or 8 < UE Category < 11) and (UE DL Category < 11 or UE DL Category = 13 )), or UE supporting E-UTRA TDD with 4Rx antenna ports and (UE Category = 8 or UE Category >= 11 or UE DL Category = 11 or UE DL Category = 12 or UE DL Category >=14) |  |  |  |  | Note 6 |
| 9.10.1.1 | FDD CSI-RS Resource Indicator Reporting – PUSCH 3-1 | Rel-13 |  | C13b | UE supporting E-UTRA FDD (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.10.1.2 | TDD CSI-RS Resource Indicator Reporting – PUSCH 3-1 | Rel-13 |  | C341 | UE supporting E-UTRA TDD (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.12.1.1 | FDD CQI reporting under fading conditions for slot/subslot TTI (Cell-Specific Reference Symbol) | Rel-15 |  | C354 | UE supporting E-UTRA FDD and slot/subslot TTI (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.12.1.2 | TDD CQI reporting under fading conditions for slot/subslot TTI (Cell-Specific Reference Symbol) | Rel-15 |  | C354a | UE supporting E-UTRA TDD and slot TTI (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.12.2.1 | FDD CQI reporting under fading conditions for slot/subslot TTI (CSI Reference Symbol) | Rel-15 |  | C357 | UE supporting E-UTRA FDD and Feature Group Indicators 103 and slot/subslot TTI (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| 9.12.2.2 | TDD CQI reporting under fading conditions for slot/subslot TTI (CSI Reference Symbol) | Rel-15 |  | C357a | UE supporting E-UTRA TDD and Feature Group Indicators 103 slot TTI (UE Category >= 2) |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW | 2Rx, 4Rx |  | Note 7 |
| MBMS Performance Testing |  |  |  |  |  |  |  |  |  |  |
| 10.1 | FDD MBMS performance (Fixed Reference Channel) | Rel-9 | C03 |  | UE supporting E-UTRA FDD and MBMS |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 10.1_1 | FDD MBMS performance (Fixed Reference Channel) (Release 13 and forward) | Rel-13 | C03 |  | UE supporting E-UTRA FDD and MBMS |  | Performed once |  |  |  |
| 10.2 | TDD MBMS performance (Fixed Reference Channel) | Rel-9 | C04 |  | UE supporting E-UTRA TDD and MBMS |  | Each "Test Number" to be performed once, in a chosen band supporting tested BW |  |  |  |
| 10.2_1 | TDD MBMS performance (Fixed Reference Channel) (Release 13 and forward) | Rel-13 | C04 |  | UE supporting E-UTRA TDD and MBMS |  | Performed once |  |  |  |
| V2X Sidelink Performance Testing |  |  |  |  |  |  |  |  |  |  |
| 14.2 | Demodulation of PSSCH / Non-concurrent with E-UTRA uplink transmissions | Rel-14 | C313 |  | UE supporting V2X Sidelink communication |  |  |  |  |  |
| 14.3 | Demodulation of PSCCH / Non-concurrent with E-UTRA uplink transmissions | Rel-14 | C313 |  | UE supporting V2X Sidelink communication |  |  |  |  |  |
| 14.4 | Power imbalance performance with two links / Non-concurrent with E-UTRA uplink transmissions | Rel-14 | C313 |  | UE supporting V2X Sidelink communication |  |  |  |  |  |
| 14.6 | Demodulation of PSSCH with eNB based synchronization | Rel-14 | C320 |  | UE supporting E-UTRA and V2X Sidelink communication |  | E16 | FDD,TDD |  |  |
| 14.7 | Soft buffer test | Rel-14 | C313 |  | UE supporting V2X Sidelink communication |  | D14 | TDD |  |  |
| 14.8 | PSCCH/PSSCH decoding capability test / Non-concurrent with E-UTRA uplink transmissions | Rel-14 | C321 |  | UE supporting V2X Sidelink communication |  |  |  |  |  |
| 14.9 | Sustained downlink data rate with active sidelink | Rel-14 | C320 |  | UE supporting E-UTRA and V2X Sidelink communication |  | E16 | FDD,TDD |  |  |
| Note 1: Due to UE capability signalling for UL 64QAM is introduced from Rel-12, this test case can optionally be executed with a Rel-12 UE. Note 2: For a transition period until RAN5#72, this condition in version 13.0.0 of 36.521-2 shall be used. This is to ensure no test coverage is lost before the UL 64QAM test case becomes available. Note 3: Equivalent aggregated bandwidth is defined as: . Where is number of CCs, and is MIMO layer and bandwidth of CC . The number of MIMO layer for CC in each CA configuration is according to Table A.4.6.1-3, Table A.4.6.2-3, Table A.4.6.3-3, Table A.4.6.3-4 or Table A.4.6.3-5. Note 4: For a transition period until RAN5#83, this condition in version 15.3.1 of 36.521-2 shall be used. This is to ensure no test coverage is lost before the UL 256QAM test cases become available. Note 5: Void. Note 6: The categories for which FGI 103/104 are mandated are (UE category >=11 OR UE DL Category = 11 OR UE DL Category = 12 OR UE DL Category >= 14) according to TS 36.331. The UE DL Categories higher than 14 are not explicitly listed in the corresponding expression in Table 4.1-1a because in this version of core specification UE DL Category > 14 requires supporting of UE Category 11 or 12. Note 7: Skipping 2RX testing if UE is verified with four RX antenna ports in operating bands where it is equipped with four RX antenna port. |  |  |  |  |  |  |  |  |  |  |

Table 4.1-1a: Applicability of RF conformance test cases Conditions

C01 IF NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/1 THEN R ELSE N/A

C01h IF (A.4.1-1/1 AND A.4.5-1/18) THEN R ELSE N/A

C02 IF NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/2 THEN R ELSE N/A

C02h IF (A.4.1-1/2 AND A.4.5-1/18) THEN R ELSE N/A

C03 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/1 AND A.4.2-1/1) THEN R ELSE N/A

C04 IF (NOT(A.4.3-4a/1 OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.2-1/1) THEN R ELSE N/A

<<Too many rows skipped>>

C355 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1) AND A.4.4-3a/103 AND A.4.5-1/x01 AND (A.4.5-1/x02a OR A.4.5-1/x02b OR A.4.5-1/x02c)) THEN R ELSE N/A

C355a IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/2) AND A.4.4-3a/103 AND A.4.5-1/x01 AND A.4.5-1/x02a) THEN R ELSE N/A

C356 IF NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND A.4.1-1/2 AND A.4.5-1/x01 AND A.4.5-1/x02a THEN R ELSE N/A

C357 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1 AND A.4.4-3a/103) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12 ) AND A.4.5-1/x01 AND (A.4.5-1/x02a OR A.4.5-1/x02b OR A.4.5-1/x02c)) THEN R ELSE N/A

C357a IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/2 AND A.4.4-3a/103) AND (A.4.3-4/2 OR A.4.3-4/3 OR A.4.3-4/4 OR A.4.3-4/5 OR A.4.3-4/6 OR A.4.3-4/7 OR A.4.3-4/8 OR A.4.3-4/9 OR A.4.3-4/10 OR A.4.3-4/11 OR A.4.3-4/12 ) AND A.4.5-1/x01 AND A.4.5-1/x02a) THEN R ELSE N/A

C358 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/6 AND A.4.5-1/15 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C359 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/6 AND A.4.5-1/14 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C360 IF A.4.1-1/2 AND A.4.2-1/2 AND (NOT A.4.5-1/18) AND (A.4.3-4/11 OR A.4.3-4/12 OR A.4.3-4a/6 OR A.4.3-4a/7 OR A.4.3-4a/10) AND (A.4.6.3-1/13 OR A.4.6.3-1/14 OR A.4.6.3-1/15 OR A.4.6.3-1/16 OR A.4.6.3-1/17 OR A.4.6.3-1/18) THEN R ELSE N/A

C361 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/1) AND A.4.4-3a/103) AND A.4.3-7/9 THEN R ELSE N/A

C362 IF (NOT(A.4.3-4a/1 OR A.4.3-4a/1a OR A.4.3-4aa/1) AND (A.4.1-1/2 AND A.4.4-3b/104) AND A.4.3-7/9) THEN R ELSE N/A

C363 IF (A.4.1-1/1 AND A.4.2-1/2 AND A.4.6-1/5 AND (NOT A.4.5-1/18) AND (A.4.6.3-1/20 OR A.4.6.3-1/21 OR A.4.6.3-1/22 OR A.4.6.3-1/23 OR A.4.6.3-1/24) AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C364 IF (A.4.1-1/1 AND A.4.2-1/2 AND A.4.6-1/6 AND (NOT A.4.5-1/18) AND A.4.6.3-1/20 AND (A.4.3-4/8 OR A.4.3-4/11 OR A.4.3-4/12)) THEN R ELSE N/A

C365 IF (A.4.1-1/2 AND A.4.6-1/5 AND ((A.4.6.3-1/19) OR (A.4.6.3-1/20)) AND (A.4.3-4/8 AND (A.4.3-4/11 OR A.4.3-4/12))) THEN R ELSE N/A

C366 IF (A.4.1-1/2 AND A.4.6-1/6 AND (A.4.6.3-1/19) AND (A.4.3-4/8 AND (A.4.3-4/11 OR A.4.3-4/12))) THEN R ELSE N/A

Cxxx1 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/6 AND A.4.5-1/15 AND (NOT A.4.5-1/18) AND A.4.3-4a/10 AND (A.4.6.3-1/26 OR A.4.6.3-1/27)) THEN R ELSE N/A

Cxxx2 IF ((A.4.1-1/1 AND A.4.1-1/2) AND A.4.6-1/6 AND A.4.5-1/14 AND (NOT A.4.5-1/18) AND A.4.3-4a/10 AND (A.4.6.3-1/26 OR A.4.6.3-1/27)) THEN R ELSE N/A

Note 1: Cxxxh applicability is defined for small cell enhancements for physical layer related test.

<< Skip unchanged sections >>

A.4.6.3	Inter-band CA Physical Layer Baseline Implementation Capabilities

Table A.4.6.3-1: Downlink Inter-band CA Bandwidth Class Combination capabilities (for one or more of the supported CA configurations in Table A.4.6.3-3, A.4.6.3-4, A.4.6.3-5)

| Item | Bandwidth Class | Ref. | Comments |
| --- | --- | --- | --- |
| 1 | DL Inter-band CA BW Class Combination A-A | 36.101, 5.6A 36.331, 6.3.6 |  |
| 2 | DL Inter-band CA BW Class Combination A-A-A (two bands) | 36.101, 5.6A 36.331, 6.3.6 |  |
| 3 | DL Inter-band CA BW Class Combination A-A-A (three bands) | 36.101, 5.6A 36.331, 6.3.6 |  |
| 4 | DL Inter-band CA BW Class Combination A-C or A-B (two bands) | 36.101, 5.6A 36.331, 6.3.6 |  |
| 5 | DL Inter-band CA BW Class Combination A-A where one of the bands is DL-only | 36.101, 5.5 |  |
| 6 | DL Inter-band CA BW Class Combination A-A-A-A (four bands) | 36.101, 5.6A 36.331, 6.3.6 |  |
| 7 | DL Inter-band CA BW Class Combination A-A-C or A-A-B (three bands) | 36.101, 5.6A 36.331, 6.3.6 |  |
| 8 | DL Inter-band CA BW Class Combination A-A-A-C (four bands) | 36.101, 5.6A 36.331, 6.3.6 |  |
| 9 | DL Inter-band CA BW Class Combination A-D or C-C or C-B (two bands) | 36.101, 5.6A 36.331, 6.3.6 |  |
| 10 | DL Inter-band CA BW Class Combination A-A-C or A-A-B (two bands) | 36.101, 5.6A 36.331, 6.3.6 |  |
| 11 | DL Inter-band CA BW Class Combination A-A-A-A (two bands) | 36.101, 5.6A 36.331, 6.3.6 |  |
| 12 | DL Inter-band CA BW Class Combination A-A-A-A (three bands) | 36.101, 5.6A 36.331, 6.3.6 |  |
| 13 | DL Inter-band CA BW Class Combination A-A-A-B or A-A-A-C (three bands) | 36.101, 5.6A 36.331, 6.3.6 |  |
| 14 | DL Inter-band CA BW Class Combination A-A-A-A-A (five bands) | 36.101, 5.6A 36.331, 6.3.6 |  |
| 15 | DL Inter-band CA BW Class Combination A-A-D (three bands) | 36.101, 5.6A 36.331, 6.3.6 |  |
| 16 | DL Inter-band CA BW Class Combination A-A-A-B or A-A-A-C (four bands) | 36.101, 5.6A 36.331, 6.3.6 |  |
| 17 | DL Inter-band CA BW Class Combination A-A-A-A-A (four bands) | 36.101, 5.6A 36.331, 6.3.6 |  |
| 18 | DL Inter-band CA BW Class Combination A-A-A-C or A-A-A-B (two bands) | 36.101, 5.6A 36.331, 6.3.6 |  |
| 19 | DL Inter-band CA BW Class Combination C-E (two bands) | 36.101, 5.6A 36.331, 6.3.6 |  |
| 20 | DL Inter-band CA BW Class Combination A-A-E (three bands) | 36.101, 5.6A 36.331, 6.3.6 |  |
| 21 | DL Inter-band CA BW Class Combination A-C-D (three bands) | 36.101, 5.6A 36.331, 6.3.6 |  |
| 22 | DL Inter-band CA BW Class Combination A-A-A-D (four bands) | 36.101, 5.6A 36.331, 6.3.6 |  |
| 23 | DL Inter-band CA BW Class Combination A-A-C-C (four bands) | 36.101, 5.6A 36.331, 6.3.6 |  |
| 24 | DL Inter-band CA BW Class Combination C-D (two bands) | 36.101, 5.6A 36.331, 6.3.6 |  |
| 25 | DL Inter-band CA BW Class Combination A-A-A-A-A-A (six bands) | 36.101, 5.6A 36.331, 6.3.6 |  |
| 26 | DL Inter-band CA BW Class Combination A-C-E (three bands) | 36.101, 5.6A 36.331, 6.3.6 |  |
| 27 | DL Inter-band CA BW Class Combination A-C-D-A (four bands) | 36.101, 5.6A 36.331, 6.3.6 |  |

# << End of changes >>