3GPP TSG-RAN5 Meeting #96-e	R5-224418

# Electronic Meeting, 15th Aug– 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2464 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add UplinkDataCompression into PDCP-Config for NR UDC test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UDC-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is no NR UDC related parameters in PDCP-Config. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add UplinkDataCompression into PDCP-Config for NR UDC test. 2. Set condition “UDC” in PDCP-Config. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR UDC related parameters in PDCP-Config is not configured |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

4.6.3	Radio resource control information elements

…

PDCP-Config

Table 4.6.3-99: PDCP-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| drb SEQUENCE { |  |  |  |
| discardTimer | infinity |  |  |
| pdcp-SN-Size-UL | len18bits |  |  |
| pdcp-SN-Size-DL | len18bits |  |  |
| headerCompression CHOICE { |  |  |  |
| notUsed | NULL |  |  |
| } |  |  |  |
| integrityProtection | Not present |  |  |
| statusReportRequired | true | AM is default |  |
|  | Not present |  | UM |
| outOfOrderDelivery | Not present |  |  |
| } |  |  |  |
| drb | Not present |  | SRB, Split_SRB |
| moreThanOneRLC | Not present |  |  |
| moreThanOneRLC SEQUENCE { |  |  | Split, Split_SRB, NR_split |
| primaryPath SEQUENCE { |  |  |  |
| cellGroup | CellGroupId |  |  |
|  | 0 | MCG path | Split_SRB, NR_split |
| logicalChannel | LogicalChannelIdentity |  |  |
| } |  |  |  |
| ul-DataSplitThreshold | infinity |  |  |
|  | Not present |  | Split_SRB |
| pdcp-Duplication | false |  |  |
|  | Not present | one UL path | Split_SRB |
| } |  |  |  |
| t-Reordering | Not present |  |  |
|  | 80ms |  | UM |
| uplinkDataCompression-r17 | Not present |  |  |
| uplinkDataCompression-r17 CHOICE { |  |  | UDC |
| setup CHOICE { |  |  |  |
| bufferSize-r17 | kbyte2 |  |  |
| dictionary-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Split | More than one RLC |
| SRB | SRB |
| UM | RLC UM DRB |
| Split_SRB | SRB with more than one RLC |
| NR_split | MCG and split for NR-DC. |
| UDC | RLC AM DRB with uplinkDataCompression |

<End of modification>