3GPP TSG-RAN WG5 Meeting #96-e	R5-224454

# Online, , 15th August 2022 - 26th August 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1504 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to A.7.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As specified in TS 24.011, clause 7.3.3, RP-User Data is an optional field in RP-ACK message. Hence RP-User Data including Delivery Report may not be included in RP-ACK message. TS 24.011 ### 7.3.3 RP-ACK This message is sent between the MSC and the mobile station in both directions and used to relay the acknowledgement of a RP-DATA or RP-SMMA message reception. The information elements are in line with 3GPP TS 23.040. See table 7.7/3GPP TS 24.011. Table 7.7/3GPP TS 24.011: RP-ACK message content IEI; Information element; Reference; Presence; Format; Length. RP-Message Type; Subclause 8.2.2; M; V; 3 bits. RP-Message Reference; Subclause 8.2.3; M; V; 1 octet. 41; RP-User Data; Subclause 8.2.5.3; O; TLV; 234 octets. |
| Summary of change: |  | Updated the Message-Body to check for only RP-ACK message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of modified section>

## A.7.2	MESSAGE for delivery report for MT SMS

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| Method |  | MESSAGE |  | RFC 3428 [91] TS 24.341 [90] |
| Request-URI |  | same P-Asserted-Identity URI as received in A.7.1 |  |  |
| SIP-Version |  | SIP/2.0 |  |  |
| Via |  |  |  | RFC 3261 [15] |
| sent-protocol |  | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) |  |  |
| sent-by |  | not checked |  |  |
| via-branch |  | value starting with ‘z9hG4bK’ |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | SIP URI of the UE |  |  |
| tag |  | any value |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | same as P-Asserted-Identity URI received in A.7.1 |  | TS 24.341 [90] |
| tag |  | not present |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | any value (but different from the Call-ID values used in preceding requests of this test case) |  |  |
| In-Reply-to |  |  |  | RFC 3261 [15] |
| callid |  | The value of the Call-Id received in the original MT SMS |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value |  | any value |  |  |
| method |  | MESSAGE |  |  |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | non-zero value |  |  |
| Content-Type |  |  |  | RFC 3261 [15] |
| media-type |  | application/vnd.3gpp.sms |  |  |
| Content-Length |  | header shall be present if UE uses TCP to send this request |  | RFC 3261 [15] |
| value |  | length of message-body |  |  |
| Message-body |  | RP-ACK message- |  | TS 24.011 [92] |

NOTE:	X is not checked

## <End of modifed section>