3GPP TSG-RAN5 Meeting #96-e	R5-224480

# Online, , 15th Aug 2022 - 26th Aug 2022

Title: 	OTA environment limitations and test case status

Source: 	Rohde & Schwarz

Agenda Item:	6.4.7

Document for:	Discussion and Agreement

- 1) Introduction

OTA test methods are defined in 38.508-1 section 6.1.3 for signalling test cases. For NR FR1, E-UTRA and UTRA the OTA link is uncalibrated link and therefore has certain restriction to cover all test cases that can be run in an OTA environment. Currently several test cases are not ready for RAN5 verification due to this limitation.

In RAN5#94-e meeting R5-220856 was discussed and it was concluded to provide more time to see If proposal 1 can be explored before concluding on this topic. Since no proposal were brought forward for RAN5#95-e meeting there was an action point created AP#95.01 to identify and update the test cases accordingly. The scope was to cover ALL FR2 test cases where the limitation are also present in FR1.

The aim of the discussion document is to present the current status and take necessary steps to conclude on some of the open FR2 test cases status.

- 2) Discussion and Proposals

The current limitation is mainly on FR1 OTA (NR/E-UTRA/UTRA) being an uncalibrated link and therefore limits any test cases that require

- Cell Reselection measurements and

- Connected Mode measurements and

- Cell Selection between two active cells and

- Multiple Signal Levels

The attached spreadsheet contains all the Rel-15 test cases that remains open in the prose /TTCN for FR2 verification. Certain test case have already been identified as not possible with the current OTA test setup and therefore will only be applicable for FR1 verification. The list of TCs can be seen in Col C marked as “No”

Proposal 1

To update the test cases selected as not possible for OTA test setups  CRs have been submitted for this RAN5 meeting

The spreadsheet also contains all the test cases that are open in Rel-15 workplan for FR2 verification. RAN5 needs to conclude on a way forward for these test cases.

Proposal 2-1

To keep it open for interested companies to pick up and update the test cases, the work plan will show that the test cases are not 100% complete for FR2

Proposal 2-2

To update the test cases as not possible for OTA test setups and interested companies can analyze the requirements and come up with CRs in future meetings to make them applicable for OTA test setups.

- 3) Conclusion

We would like RAN5 to endorse Proposal 1.

We would like RAN5 to discuss and conclude on Proposal 2-1 or Proposal 2-2