3GPP TSG-RAN5 Meeting #96-e	R5-224487

Electronic Meeting, 15th August– 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 2647 | rev | - | Current version: | 16.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Default Configuration Parameters for Test 1 in Test Case 7.1.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | STMicroelectronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test |  |  |  |  |  | Date: |  |  | 2022-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 has changed the configurations for Test case 7.1.11 Test 1 in 36.133 due to R4-2210985 CR 7148. Thus the same changes need to be done for this test case in 36.521-3 to keep the specifications consistent. In Test case 7.1.11 Test 1, the UE is configured with SRS transmissions with periodicity of 2ms or every other subframe. Since the UE is operating in a half-duplex mode, the UE requires an unoccupied subframe between receive and transmit. A continuous uplink activity (i.e., the SRS transmissions) every other subframe will cause a deadlock at the UE side. That is, the UE has no opportunity to receive any downlink activity. Hence, the UE will not be able to measure the cell, detect any downlink timing changes, or even maintain its timing and frequency loops locked Indeed, due to this tight SRS periodicity configuration (i.e., 2 ms), a HD-FDD UE will be unable to receive any DCI 6-0A. Hence, the UE cannot transmit the RRCReconfigurationComplete message in step 4 as the new configurations (including configuring SRS with periodicity of 2 ms) are applied before RRC submits the RRCReconfigurationComplete to lower layers for transmission as per 36.331, section 5.3.5.3. The UE is expected to keep sending SchedulingRequest (SR) as it has data (RRCReconfigurationComplete) to transmit. Even if DCIs are being sent to the UE, it will fail to receive them. Thus SR_COUNTER will reach dsr-TransMax and the RRC will be notified to release the PUCCH and SRS as per 36.321, section 5.4.4. In this case, step 8 that measures the transmit offset based on SRS transmission cannot be verified as SRS transmission would have stopped. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The default configuration parameters for test 1 have been updated such that the SRS periodicity becomes 10msec. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specifications 36.133 and 36.521-3 will not be consistent and a conformant HD-FDD UE will fail Test 1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.11.4.3 , 7.1.11.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

### 7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA

#### 7.1.11.1	Test purpose

To verify that the Cat-M1 UE in CEModeA have the capability to follow the frame timing change of the connected System Simulator. The method used is that the UE initial transmit timing accuracy, the maximum amount of timing change in one adjustment, and the minimum and maximum adjustment rate are within the specified limits based on the requirements.

#### 7.1.11.2	Test applicability

This test applies to all types of E-UTRA HD-FDD UE release 13 and forward of UE category M1. Applicability requires support for FGI bit 5.

#### 7.1.11.3	Minimum conformance requirements

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is defined in TS 36.133 [4] clause 7.24.2 and shown in table 7.1.11.3-1. This requirement applies when it is the first transmission in a DRX, eDRX_CONN cycle, or the first transmission in a repetition period (R>1) for PUCCH, PUSCH and SRS or it is the PRACH transmission. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus . The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the reference cell. NTA_Ref for PRACH is defined as 0.  (in Ts units) for other channels is the difference between UE transmission timing and the Downlink timing immediately after when the last timing advance in clause 7.3 was applied. NTA_Ref for other channels is not changed until next timing advance is received.

Table 7.1.11.3-1: Te Timing Error Limit

| CE Mode | Te_ |
| --- | --- |
| A | 24*TS |
| B | 48*Ts |
| NOTE 1: TS is the basic timing unit defined in TS 36.211. NOTE 2: This requirement applies regardless of the downlink carrier bandwidth. |  |

When it is not the first transmission in a DRX or eDRX_CONN cycle or there is no DRX or no eDRX_CONN cycle, and when it is the transmission for PUCCH, PUSCH and SRS transmission, the UE shall, when no repetitions are configured on the uplink or the repetition period is R=1, be capable of changing the transmission timing according to the received downlink frame of the reference cell except when the timing advance in clause 7.3 is applied such that the UE transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.1.11.3-1.

When no repetition period is configured, or the configured repetition period is R=1, all adjustments made to the UE uplink timing shall follow these rules:1)	The maximum amount of the magnitude of the timing change in one adjustment shall be Tq seconds.

2)	The minimum aggregate adjustment rate shall be 7 * TS per second.

3)	The maximum aggregate adjustment rate shall be Tq per 200 ms.

Where the maximum autonomous time adjustment step Tq is defined in TS 36.133 [4] clause 7.24.2 and shown in table 7.1.11.3-2.

Table 7.1.11.3-2: Tq Maximum Autonomous Time Adjustment Step

| CE Mode | Tq_ |
| --- | --- |
| A | 17.5*TS |
| B | 17.5*Ts |
| NOTE 1: TS is the basic timing unit defined in TS 36.211. NOTE 2: This requirement applies regardless of the downlink carrier bandwidth. |  |

When a repetition period is configured on the uplink, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period for which R>1.

The normative reference for this requirement is TS 36.133 [4] clause 7.24 and A.7.1.11.

Note 1:	Both the allowed UE frequency error and the UE time adjustment quantisation can affect the result of the test, and it is not possible to test the UE Transmit Timing Accuracy in isolation from these core requirements. They are therefore taken into account when setting the Test Tolerances and the Test Requirements

#### 7.1.11.4	Test description

##### 7.1.11.4.1	Initial conditions

Test Environment: Normal, as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-1 and TS 36.508 [7] clauses 4.4.2 and 4.3.1.

Channel Bandwidth to be tested: 10 MHz as defined in TS 36.508 [7] clause 4.3.1.

1.	Connect the SS (node B emulator) and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure A.18 (using only main UE Tx/Rx antenna).

2.	Propagation conditions are set according to Annex B clause B.0.

3.	There is one E-UTRA HD-FDD carrier and one cell specified in the test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.0 and C.1 for this test.

##### 7.1.11.4.2	Test procedure

The test consists of a single cell. The transmit timing accuracy is verified related to the downlink frame timing of Cell 1. The downlink timing of Cell 1 is changed and the changes in UE transmit timing are observed. The transmit timing is verified by the UE transmitting SRS used as a measurement reference facilitating the SS timing estimation.

1.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 7.2A.3AA.

2.	Set the parameters according to Test 1, Test 2, or Test 3 in Tables 7.1.11.5-1, 7.1.11.5-2 and 7.1.11.5-3 as appropriate. Propagation conditions are set according to Annex B clause B.1.1.

3.	SS shall transmit an RRCConnectionReconfiguration message.

4.	The UE shall transmit RRCConnectionReconfigurationComplete message.

5.	The SS shall check that the UE transmit timing offset is within the limits specified in Table 7.1.11.5-4  with respect to the first detected path (in time) of the corresponding downlink frame of Cell 1.

6.	The SS adjusts the downlink timing for Cell 1 to a delay of +64 × TS (approximately +2 µs) for Test 1 and Test2 and to and a delay of +32× TS for Test 3 compared to that in step 5. For Test 2 and Test 3, the downlink timing adjustment shall be done at (40 ± 15) ms before the next DRX On duration period.

7.   Void.

8.	In case for Test 1, the SS shall check that the UE transmit timing offset stays within the limits specified in Table 7.1.11.5-4  with respect to the first detected path (in time) of the corresponding downlink frame of Cell 1. In case for Test 2 and Test 3, the SS shall check that the UE transmit timing offset stays within the limits specified in Table 7.1.11.5-4. For that purpose, the SRS transmission that takes place in the first DRX ON duration after downlink timing of Cell 1 is changed is used.

9.	The SS shall transmit RRCConnectionRelease message to release the RRC connection, which includes the release of the established radio bearers as well as all radio resources.

10.	After the RRC connection release, the SS switches off and on the UE and ensures the UE is in State 3A-RF –CE according to TS 36.508 [7] clause 7.2A.3AA.

11.	Repeat step 1-10 for each sub-test in Tables 7.1.11.5-1, 7.1.11.5-2 and 7.1.11.5-3 as appropriate.

##### 7.1.11.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 4.6 with the condition CEModeA with the following exceptions:

Table 7.1.11.4.3-1: Common Exception messages for UE transmit timing accuracy for E-UTRAN HD-FDD Cat-M1 UE in CEModeA test requirement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.8-1 |
| Default RRC messages and information elements contents exceptions | Table H.3.4-2 Table H.7.1-1 |

Table 7.1.11.4.3-2: RRCConnectionReconfiguration: Additional UE transmit timing accuracy for E-UTRAN HD-FDD Cat-M1 UE in CEModeA test requirement

| Derivation Path: TS 36.331 [5] clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |
| radioResourceConfigDedicated SEQUENCE { |  |  |  |  |
| mac-MainConfig CHOICE { |  |  |  |  |
| explicitValue |  | MAC-MainConfig-RBC |  |  |
| } |  |  |  |  |
| physicalConfigDedicated |  | PhysicalConfigDedicated- DEFAULT |  | RBC |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.11.4.3-3: SoundingRS-RL-ConfigCommon-DEFAULT: Additional UE transmit timing accuracy for E-UTRAN HD-FDD Cat-M1 UE in CEModeA test requirement

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-21 SoundingRS-UL-ConfigCommon-DEFAULT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SoundingRS-UL-ConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| srs-BandwidthConfig |  | bw5 |  |  |
| srs-SubframeConfig |  |  | Set according to specific test; sc1 for Test 1 and sc3 for Test 2 and Test 3 | FDD |
| ackNackSRS-SimultaneousTransmission |  | FALSE |  |  |
| srsMaxUpPts |  | Not present |  | FDD |
| } |  |  |  |  |

Table 7.1.11.4.3-4: SoudingRS-UL-ConfigDedicated-DEFAULT: Additional UE transmit timing accuracy for E-UTRAN HD-FDD test requirement

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-22 SoundingRS-UL-ConfigDedicated-DEFAULT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SoundingRS-UL-ConfigDedicated-DEFAULT ::= CHOICE { |  |  |  |  |
| setup SEQUENCE { |  |  |  |  |
| srs-Bandwidth |  | bw1 |  |  |
| srs-HoppingBandwidth |  | hbw0 |  |  |
| freqDomainPosition |  | 0 |  |  |
| duration |  | TRUE | Indefinite duration |  |
| srs-ConfigIndex |  | 7 | Set according to specific test; 7 for test 1 and 77 for Test 2 and 317 for Test 3 |  |
| transmissionComb |  | 0 |  |  |
| cyclicShift |  | cs0 | No cyclic shift |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.11.4.3-5: MAC-MainConfig-RBC: Additional UE transmit timing accuracy for E-UTRAN HD-FDD Cat-M1 UE in CEModeA test requirement for Test 2 and Test 3

| Derivation Path: TS 36.508 [7] clause 4.8.2.1.5, Table 4.8.2.1.5-1 MAC-MainConfig-RBC |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MAC-MainConfig-RBC ::= SEQUENCE { |  |  |  |  |
| ul-SCH-Config SEQUENCE { |  |  |  |  |
| maxHARQ-Tx |  | n5 |  |  |
| periodicBSR-Timer |  | sf20 |  |  |
| retxBSR-Timer |  | sf1280 |  |  |
| ttiBundling |  | FALSE |  |  |
| } |  |  |  |  |
| drx-Config CHOICE { |  |  |  | DRX_S |
| setup SEQUENCE { |  |  |  |  |
| onDurationTimer |  | psf1 |  |  |
| drx-InactivityTimer |  | psf1 |  |  |
| drx-RetransmissionTimer |  | psf1 |  |  |
| longDRX-CycleStartOffset CHOICE { |  |  | Set according to specific test; sf80 for Test 2 and sf640 for Test 3 |  |
| sf80 |  | 0 |  |  |
| } |  |  |  |  |
| shortDRX |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.11.4.3-6: CQI-ReportConfig-DEFAULT: Additional UE transmit timing accuracy for E-UTRAN HD-FDD Cat-M1 UE in CEModeA test requirement

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-2 CQI-ReportConfig-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportModeAperiodic | rm20 | Allowed value for CEModeA configured UE |  |
| nomPDSCH-RS-EPRE-Offset | 0 |  |  |
| cqi-ReportPeriodic CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |

Table 7.1.11.4.3-7: MAC-MainConfig-RBC: Additional UE transmit timing accuracy for E-UTRAN HD-FDD test requirement

| Derivation Path: TS 36.508 [7] clause 4.8.2, Table 4.8.2.1.5-1 MAC-MainConfig-RBC |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MAC-MainConfig-RBC ::= SEQUENCE { |  |  |  |  |
| timeAlignmentTimerDedicated |  | Infinity |  |  |
| } |  |  |  |  |

#### 7.1.11.5	Test requirement

Tables 7.1.11.5-1, 7.1.11.5-2 and 7.1.11.5-3 define the primary settings including test tolerances for UE transmit timing accuracy for E-UTRAN HD-FDD test.

Tables 7.1.11.5-4, 7.1.11.5-5 and 7.1.11.5-6 define the rules for adjustments made to the UE uplink timing include Test Tolerances.

Table 7.1.11.5-1: Cell Specific Test requirement Parameters for UE transmit timing accuracy for E-UTRAN HD-FDD for Cat-M1 UE in CEModeA test case

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 | Test 3 |
| E-UTRA RF Channel Number |  | 1 | 1 | 1 |
| Channel Bandwidth (BWchannel) | MHz | 10 | 10 | 10 |
| DRX cycle | ms | N/A | 80Note5 | 640Note5 |
| PRACH Configuration |  | PRACH_2CE As specified in A.9 |  |  |
| MPCCH Reference measurement channelNote1 |  | R.16 HD-FDD | R.16 HD-FDD | R.16 HD-FDD |
| OCNG PatternNote2 |  | OP.21 FDD | OP.21 FDD | OP.21 FDD |
| PBCH_RA | dB | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |
| PSS_RA |  |  |  |  |
| SSS_RA |  |  |  |  |
| PHICH_RA |  |  |  |  |
| PHICH_RB |  |  |  |  |
| MPDCCH_RA |  |  |  |  |
| MPDCCH_RB |  |  |  |  |
| OCNG_RANote3 |  |  |  |  |
| OCNG_RBNote3 |  |  |  |  |
|  | dBm/15 kHz | -98 | -98 | -98 |
|  | dB | 3.30 | 3.30 | 3.30 |
|  | dB | 3.30 | 3.30 | 3.30 |
| IoNote4 | dBm/9 MHz | -65.25 | -65.25 | -65.25 |
|  | dBm/1.08 MHz | N/A | N/A | N/A |
| Propagation condition | - | AWGN | AWGN | AWGN |
| Note 1: For the reference measurement channels, see clause A.7.2. Note 2: For the OCNG pattern, see clause D.1.21. Note 3: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 4: Io level has been derived from other parameters for information purpose. It is not a settable parameter. Note 5: DRX related parameters are defined in Table 7.1.11.5-3. |  |  |  |  |

Table 7.1.11.5-2: Sounding Reference Signal Configuration to be used in UE transmit timing accuracy for E-UTRAN HD-FDD for Cat-M1 UE in CEModeA test case

| Field | Value |  |  | Comment |
| --- | --- | --- | --- | --- |
|  | Test 1 | Test 2 | Test 3 |  |
| srsBandwidthConfiguration | bw5 | bw5 | bw5 |  |
| srsSubframeConfiguration | sc1 | sc3 | sc3 |  |
| ackNackSrsSimultaneousTransmission | FALSE | FALSE | FALSE |  |
| srsMaxUpPTS | N/A | N/A | N/A | Not applicable for FDD |
| srsBandwidth | 0 | 0 | 0 | No hopping |
| srsHoppingBandwidth | hbw0 | hbw0 | hbw0 |  |
| frequencyDomainPosition | 0 | 0 | 0 |  |
| duration | TRUE | TRUE | TRUE | Indefinite duration |
| Srs-ConfigurationIndex | 7 | 77 | 317 | SRS periodicity of 10ms, 80ms and 320ms for Test 1, 2 and 4, respectively. |
| transmissionComb | 0 | 0 | 0 |  |
| cyclicShift | cs0 | cs0 | cs0 | No cyclic shift |
| SRS-AntennaPort | an1 |  |  | Number of antenna ports used for SRS transmission |
| Note 1: For further information see clause 6.3.2 in TS 36.331[5]. |  |  |  |  |

Table 7.1.11.5-3: DRX Configuration to be used in UE transmit timing accuracy for E-UTRAN FDD for Cat-M1 UE in CEModeA test case

| Field |  | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 2 | Test 3 |  |
| onDurationTimer |  | psf1 | psf1 |  |
| drx-InactivityTimer |  | psf1 | psf1 |  |
| drx-RetransmissionTimer |  | psf1 | psf1 |  |
| longDRX-CycleStartOffset |  | sf80 | sf640 |  |
| shortDRX |  | disable | disable |  |
| Note 1: For further information see clause 6.3.2 in TS 36.331[5]. |  |  |  |  |

For parameters specified in Tables 7.1.11.5-1 and 7.1.11.5-2, the initial transmit timing accuracy, the maximum amount of timing change in one adjustment, the minimum and the maximum adjustment rate shall be within the limits defined in clause 7.24.2.

The following sequence of events shall be used to verify that the requirements are met.

For the 10MHz channel bandwidth, the test sequence shall be carried out in RRC_CONNECTED for both non-DRX (for Test1) and DRX with a cycle length of 80 ms or a cycle length of 640 ms (Tests 2 and , respectively):

a) 	After a connection is set up with the cell, the test system shall verify that the UE transmit timing offset is within NTA×TS   ± 27×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

b) 	The test system adjusts the downlink transmit timing for the cell by +64TS (for Test 1 and Test 2) or +32TS (for Test 3) compared to that in (a).

c) 	The test system shall verify that for Test 1 the adjustment step size and the adjustment rate shall be according to the requirements in clause 7.24.2 until the UE transmit timing offset is within NTA×TS  ± 27×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1. Skip this step for Test 2 and Test 3.

d) 	The test system shall verify that the UE transmit timing offset stays within NTA×TS  ± 27×TS with respect to the first detected path  (in time) of the corresponding downlink frame of cell 1. For test 2 and test 3 the UE transmit timing offset shall be verified for the first transmission in the DRX cycle immediately after DL timing adjustment.

The UE transmit timing offset shall be within the requirements in Table 7.1.11.5-4.

The reference point for the UE initial transmit timing control test requirement shall be the downlink timing minus .

Table 7.1.11.5-4: Test requirement for Te Timing Error Limit

| CE Mode | Te_ |
| --- | --- |
| A | 27*TS |
| NOTE 1: TS is the basic timing unit defined in TS 36.211. NOTE 2: This requirement applies regardless of the downlink carrier bandwidth. |  |

The UE shall be capable of changing the transmission timing according to the received downlink frame. When the transmission timing error between the UE and the reference cell exceeds Te the UE is required to adjust its timing to within Te.

All adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum adjustment step size Tq shall be within the requirements in Table 7.1.11.5-5

2)	The minimum aggregate adjustment rate shall be 3.4*TS per second (To be adjusted after TT analysis).

3)	The maximum aggregate adjustment rate shall be within the requirements in Table 7.1.11.5-6.

Table 7.1.11.5-5: Test requirement for Tq Maximum Autonomous Time Adjustment Step

| CE Mode | Tq_ |
| --- | --- |
| A | 18*TS |
| NOTE 1: TS is the basic timing unit defined in TS 36.211 [9]. NOTE 2: This requirement applies regardless of the downlink carrier bandwidth. |  |

Table 7.1.11.5-6: Test requirement for maximum aggregate adjustment rate per 198 ms

| CE Mode | Maximum adjustment per 198ms |
| --- | --- |
| A | 18.6*TS |
| NOTE 1: TS is the basic timing unit defined in TS 36.211 [9]. NOTE 2: This requirement applies regardless of the downlink carrier bandwidth. |  |

< End of changes >