3GPP TSG-RAN5 Meeting #96-e	R5-224552

# Electronic Meeting, 15th – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1909 | rev | - | Current version: | 17.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR SA FR2 RRM TC 7.1.1.2 - inter-freq reselection with TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To finish NR SA FR2 RRM TC 7.1.1.2. - To update test parameters in NR SA FR2 RRM TC 7.1.1.2 according to TT analysis. - To align with 38.133 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - NR SA FR2 RRM TC 7.1.1.2 is finished - Test parameters in NR SA FR2 RRM TC 7.1.1.2 are updated according to TT analysis. - Aligned with 38.133 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.903 CR 0358 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.1.1.2	NR SA FR2-FR2 cell re-selection

Editor’s Note: This test case is complete for the following configurations:

- Test frequency f ≤ 40.8 GHz.

- UE PC3

This test case is incomplete for Test frequency f > 40.8 GHz

This test case is incomplete for UE power class other than PC3.

7.1.1.2.1	Test purpose

The purpose of this test is to verify the requirement for the inter frequency NR cell reselection requirements specified in TS 38.133 clause 4.2.2.4.

7.1.1.2.2	Test applicability

This test applies to all types of NR UE from Release 15 onwards.

7.1.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.1.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.1.1.2.

7.1.1.2.4	Test description

7.1.1.2.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 7.1.1.2.4.1-1.

Table 7.1.1.2.4.1-1: Supported test configurations for NR SA FR2-FR2 cell re-selection

| Configuration | Description for serving cell | Description for target cell |
| --- | --- | --- |
| 7.1.1.2-1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 7.1.1.2-2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 7.1.1.2.4.1-2.

Table 7.1.1.2.4.1-2: Initial conditions for NR SA FR2-FR2 cell re-selection

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.1.1.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.1.1.2.4.1-3.

2. Message contents are defined in clause 7.1.1.2.4.3.

3. There is two NR carrier and 2 NR Cells specified in the test. Cell 2 is the PCell and Cell 1 is the neighbour cell in a different carrier than cell 2. Cell 1 and Cell 2 are configured according to Annex C.1.2.

Table 7.1.1.2.4.1-3: General test parameters for NR SA FR2-FR2 cell re-selection

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell2 | The UE camps on cell 2 in the initial phase and during T1 period the UE reselects to cell 1 |
|  | Neighbour cell |  | 1, 2 | Cell1 |  |
| T1 end condition | Active cell |  | 1, 2 | Cell1 | The UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cells |  | 1, 2 | Cell2 |  |
| T3 end condition | Active cell |  | 1, 2 | Cell2 | The UE shall perform reselection to cell 2 with higher priority during T3 |
|  | Neighbour cell |  | 1, 2 | Cell1 |  |
| RF Channel Number |  |  | 1, 2 | 1, 2 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2 |  |
|  |  |  | 2 | SSB.2 FR2 |  |
| SMTC configuration |  |  | 1, 2 | SMTC.1 |  |
| DRX cycle length |  | s | 1, 2 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | 35 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2 | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 1, 2 | 95 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

7.1.1.2.4.2	Test procedure

The test consists of one active cell and one neighbour cell. The UE is requested to monitor the neighbouring cell on one of the NR carriers. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of higher priority than cell 1. Furthermore, UE has not registered with network for the tracking area containing cell 1.

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRC SETUP REQUEST message to perform a Registration procedure for mobility.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR, connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 2.

2.	Set the parameters according to T1 in Table 7.1.1.2.5-1. Propagation conditions are set according to Annex B clause B.1.1. T1 starts.

3.	Void.

4.	The SS waits for random access requests information from the UE to perform cell re-selection on the lower priority cell, Cell 1.

5.	If the UE responds on lower priority cell, Cell 1 during time duration T1 within 27 seconds from the beginning of time period T1, then count a success for the event “Re-select lower priority Cell 1”. Otherwise count a fail for the event “Re-select lower priority Cell 1”.

6.	If the UE has re-selected Cell 1 within T1, after the re-selection or when T1 expires, continue with step 6a.
Otherwise, if T1 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE and skip to step 12.

6a.	The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 1.

7.	The SS shall switch the power setting from T1 to T2 as specified in Table 7.1.1.2.5-1. During time duration T2, Cell 2 shall be powered OFF and the physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) shall be changed to ensure Cell 2 is not detected by the UE.

8.	When T2 expires, the SS shall switch the power setting from T2 to T3 as specified in Table 7.1.1.2.5-1.

9.	The SS waits for random access requests information from the UE to perform cell re-selection on the higher priority cell, Cell 2.

10.	If the UE responds on higher priority cell, Cell 2 during time duration T3 within 87 seconds from the beginning of time period T3, then count a success for the event “Re-select higher priority Cell 2”. Otherwise count a fail for the event “Re-select higher priority Cell 2”.

11.	If the UE has re-selected Cell 2 within T3, after the re-selection or when T3 expires, continues with step 11a.
Otherwise, if T3 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and continue with step 12.

11a.The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 2.

12.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR, connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 2.

13.	Repeat step 3-12 until a test verdict has been achieved.
Each of the events “Re-select lower priority Cell 1” and “Re-select higher priority Cell 2” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict.
If both events pass, the test passes. If one event fails, the test fails.

7.1.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.1.1.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.2-1 for NR Cell 1 |
| Default RRC messages and information elements contents exceptions |  |

Table 7.1.1.2.4.3-2: RACH-ConfigGeneric: NR cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 190 |  |  |
| } |  |  |  |

Table 7.1.1.2.4.3-3: SIB1 (Cell 1 and Cell 2)

| Derivation Path: Table H.2.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |  |
| q-RxLevMin |  | -70 | Actual value is -62*2 = -140dBm | 7.1.1.2-1 and Cell 1 |
|  |  | -68 | Actual value is -62*2 = -136dBm | 7.1.1.2-2 and Cell 1 |
|  |  | -62 | Actual value is -62*2 = -124dBm | 7.1.1.2-1 and Cell 2 |
|  |  | -60 | Actual value is -62*2 = -120dBm | 7.1.1.2-2 and Cell 2 |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.2.4.3-4: SIB2(Cell2)

| Derivation Path: Table H.2.2-1 with condition NR Cell 2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | 31 | Actual value is 31*2 = 62dBm |  |
| threshServingLowP | 31 | Actual value is 31*2 = 62dBm |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.2.4.3-5: SIB4 (Cell 1)

| Derivation Path: Table H.2.2-2 with condition NR Cell 1, SMTC.1 and Synchronous cells |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo { | 1 Entry |  |  |
| InterFreqCarrierFreqInfo[1] SEQUENCE { |  | Entry 1 |  |
| q-RxLevMin | -70 | Actual value = -70*2 = -140dBm | 7.1.1.2-1 |
|  | -68 | Actual value = -68*2 = -136dBm | 7.1.1.2-2 |
| threshX-HighP | 16 | Actual value is 16*2 = 32dBm |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.2.4.3-6: SIB4 (Cell 2)

| Derivation Path: Table H.2.2-2 with condition NR Cell 2, SMTC.1 and Synchronous cells |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo { | 1 Entry |  |  |
| InterFreqCarrierFreqInfo[1] SEQUENCE { |  | Entry 1 |  |
| q-RxLevMin | -62 | Actual value = -62*2 = -124dBm | 7.1.1.2-1 |
|  | -60 | Actual value = -60*2 = -120dBm | 7.1.1.2-2 |
| threshX-LowP | 9 | Actual value = 9*2 = 18dBm |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

7.1.1.2.5	Test requirement

Tables 7.1.1.2.4.1-3 and 7.1.1.2.5-1 define the primary level settings including test tolerances for inter frequency NR cell re-selection test case.

Table 7.1.1.2.5-1: Cell specific test parameters for NR SA FR2-FR2 cell re-selection

| Parameter |  |  |  |  | Unit |  |  |  |  | Test configuration |  |  |  | Cell 1 |  |  |  |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | T1 | T2 | T3 |  |  | T1 |  | T2 | T3 |
| TDD configuration |  |  |  |  |  |  |  |  | 1, 2 |  |  |  | TDDConf.3.1 |  |  |  |  |  | TDDConf.3.1 |  |  |  |  |
| PDSCH RMC configuration |  |  |  |  |  |  |  |  | 1, 2 |  |  |  | SR.3.1 TDD |  |  |  |  |  | SR.3.1 TDD |  |  |  |  |
| RMSI CORESET parameters |  |  |  |  |  |  |  |  | 1, 2 |  |  |  | CR.3.1 TDD |  |  |  |  |  | CR.3.1 TDD |  |  |  |  |
| RMSI CORESET RMC configuration |  |  |  |  |  |  |  |  | 1, 2 |  |  |  | CCR.3.1 TDD |  |  |  |  |  | CCR.3.1 TDD |  |  |  |  |
| OCNG Pattern |  |  |  |  |  |  |  |  | 1, 2 |  |  |  | OP.1 |  |  |  |  |  | OP.1 |  |  |  |  |
| BWchannel |  |  |  | MHz |  |  |  |  | 1, 2 |  |  |  | 100: NRB,c = 66 |  |  |  |  |  | 100: NRB,c = 66 |  |  |  |  |
| Data RBs allocated |  |  |  |  |  |  |  |  | 1, 2 |  |  |  | 66 |  |  |  |  |  | 66 |  |  |  |  |
| Initial DL BWP configuration |  |  |  |  |  |  |  |  | 1, 2 |  |  |  | DLBWP.0.1 |  |  |  |  |  | DLBWP.0.1 |  |  |  |  |
| Initial UL BWP configuration |  |  |  |  |  |  |  |  | 1, 2 |  |  |  | ULBWP.0.1 |  |  |  |  |  | ULBWP.0.1 |  |  |  |  |
| RLM-RS |  |  |  |  |  |  |  |  | 1, 2 |  |  |  | SSB |  |  |  |  |  | SSB |  |  |  |  |
| Qrxlevmin |  |  |  | dBm/SCS |  |  |  |  | 1 |  |  |  | -140 |  |  |  |  |  | -124 Note 6 |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 2 |  |  |  | -137 |  |  |  |  |  | -121 Note 6 |  |  |  |  |
| Pcompensation |  |  |  | dB |  |  |  |  | 1, 2 |  |  |  | 0 |  |  |  |  |  | 0 |  |  |  |  |
| Qhysts |  |  |  | dB |  |  |  |  | 1, 2 |  |  |  | 0 |  |  |  |  |  | 0 |  |  |  |  |
| Qoffsets, n |  |  |  | dB |  |  |  |  | 1, 2 |  |  |  | 0 |  |  |  |  |  | 0 |  |  |  |  |
| Cell_selection_and_ reselection_quality_measurement |  |  |  |  |  |  |  |  | 1, 2 |  |  |  | SS-RSRP |  |  |  |  |  | SS-RSRP |  |  |  |  |
| AoA setup |  |  |  |  |  |  |  |  | 1, 2 |  |  |  | Setup 1 defined in A. 9.1 |  |  |  |  |  | Setup 1 defined in A. 9.1 |  |  |  |  |
| Beam assumptionNote 4 |  |  |  |  |  |  |  |  | 1,2 |  |  |  | Rough |  |  |  |  |  | Rough |  |  |  |  |
| Es/Iot BB Note 5 |  | dB |  |  |  |  |  |  |  |  | 1 |  |  |  | 9.95 | 9.95 | 7.45 |  |  | -3.25 |  | -infinity | 7.95 |
|  |  |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  |  |  |  | dBm/SCS |  |  |  |  | 1 |  |  |  | -93 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | 2 |  |  |  | -90 |  |  |  |  |  |  |  |  |
| Note2 |  |  |  |  |  | dBm/15 kHz |  |  |  |  | 1 |  |  |  | -102 |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | dB |  |  |  |  |  | 1 |  |  |  |  |  |  |  | 10.5 | 10.5 | 8 |  |  | -2.7 Note 6 |  | -infinity | 8.5 |
|  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS |  |  |  |  |  | 1 |  |  |  |  |  |  |  | -82.5 | -82.5 | -85 |  |  | -95.7 |  | -infinity | -84.5 |
|  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  | -79.5 | -79.5 | -82 |  |  | -92.7 |  | -infinity | -91.5 |
| Io | dBm/95.04 MHz |  |  |  |  |  | 1 |  |  |  |  |  |  |  | -53.11 | -53.11 | -55.34 |  |  | -62.12 |  | -63.98 | -54.91 |
|  |  |  |  |  |  |  |  |  |  |  | 2 |  |  |  | -53.11 | -53.11 | -55.34 |  |  | -62.12 |  | -63.98 | -54.91 |
| Treselection | s |  |  |  |  |  | 1, 2 |  |  |  |  |  |  |  | 0 | 0 | 0 |  |  | 0 |  | 0 | 0 |
| SnonintrasearchP |  |  | dB |  |  |  |  | 1, 2 |  |  |  | 50 |  |  |  |  |  | 62 Note 6 |  |  |  |  |  |
| Threshx, highP |  |  | dB |  |  |  |  | 1, 2 |  |  |  | 32 Note 6 |  |  |  |  |  | 48 |  |  |  |  |  |
| Threshserving, lowP |  |  | dB |  |  |  |  | 1, 2 |  |  |  | 44 |  |  |  |  |  | 62 Note 6 |  |  |  |  |  |
| Threshx, lowP |  |  | dB |  |  |  |  | 1, 2 |  |  |  | 50 |  |  |  |  |  | 18 Note 6 |  |  |  |  |  |
| Propagation Condition |  |  |  |  |  |  |  |  |  |  | 1, 2 |  |  |  | AWGN |  |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. Note 6: Including the test tolerance given in Annex F. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The cell reselection delay to a higher priority cell is defined as the time from the beginning of time period T3, to the moment when the UE camps again on cell 2, and starts to send preambles on the PRACH for sending the RRC SETUP REQUEST message to perform a Registration procedure for mobility on cell 2.

The cell re-selection delay to a higher priority cell test requirement in this case is expressed as:

Cell re-selection delay to a higher priority cell = Thigher_priority_search + Tevaluate, NR_ inter + TSI-NR

Thigher_priority_search = 60 s, as specified in TS 38.133 [6] clause 4.2.2.7;

Tevaluate, NR_ inter = 25.6 s, as specified in TS 38.133 [6] clause 4.2.2.4;

TSI-NR = 1280 ms; maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell is assumed in this test.

The cell re-selection delay to a higher priority cell shall be less than a total of 86.88 seconds in this test case (note: this gives a total of 86.88 seconds but this test allows 87 seconds)

The cell reselection delay to a lower priority cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRC SETUP REQUEST message to perform a Registration procedure for mobility on cell 1.

The cell re-selection delay to a lower priority cell test requirement in this case is expressed as:

Cell re-selection delay to a lower priority cell = Tevaluate, NR_ inter + TSI-NR

Tevaluate, NR_ inter = 25.6 s, as specified in TS 38.133 [6] clause 4.2.2.4;

TSI-NR = 1280 ms; maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell is assumed in this test.

The cell re-selection delay to a lower priority cell shall be less than a total of 26.88 seconds in this test case (note: this gives a total of 26.88 seconds but this test allows 27 seconds)

For the test to pass, both events above shall pass.

The statistical pass/fail decisions are done separated for each event. For an event to pass, the total number of successful loops shall be more than 90% of the cases with a confidence level of 95%.

<End of modified section 1>