3GPP TSG-RAN5 Meeting #96-e	R5-224660

# Online, , 15th August – 26th August 2022

Agenda Item:	6.6.6.6

Source:	MCC TF160

Title:	On the implementation complexity of some IMS emergency over 5G test cases

Document for:	Endorsement

## 1.	Introduction

The majority of test cases written in TS 34.229-1[2] and TS 34.229-5[1] require a simple implementation of a single cell with a UE already/to be configured in RRC_IDLE state.  This enables the TTCN implementation to consist of a relatively simple IPCAN test model (IPCAN = IP-Connectivity Access Network) consisting of ASPs which are easily reproducible over a number of different RATs.

There are however a small number of, emergency call related, test cases which each have their own more complex lower layer (below IP) test requirements.

## 2.	Discussion

The equivalent test cases defined in TS 34.229-1[2] using LTE include a number of tests which require the UE to move to UTRAN and/or GERAN.  These could not be implemented in the 'normal' ATS "IMS_EUTRA", they had to be implemented in the TTCN in a separate ATS “IMS_IRAT” specially created for that purpose, which uses the same existing multi-RAT test model as that used for the test cases defined in TS 36.523-1[3].

As TS 34.229-5[1] is defined explicitly to use 5GS, the test cases which require the UE to move to UTRAN and/or GERAN have already been included in TS 38.523-1[4], not TS 34.229-5[1].

Affected Test cases

| 34.229-5[1] TC | Equivalent 34.229-1[2] TC | Title | Reason for TC complexity | Comment |
| --- | --- | --- | --- | --- |
| 10.6 | 19.3.4 | Non-UE detectable emergency call / IM CN sends 380 with an Alternative Service / Previous emergency IMS registration not expired / 5GS | This sets up, then releases an emergency call. It then triggers the UE to start a voice call, but then initiates an emergency call after receiving a 380 Alternative Service. | Due to the complexity of the NAS layer, which is currently not completely specified in the prose – does the SS keep/release the emergency PDU session and voice QoS media between the 2 calls – it is proposed to move this to 38.523-1 |
| 10.7 | 19.4.1 | Emergency call without emergency registration / UE does not contain an ISIM or USIM / 5GS | This is a limited-service TC therefore the UE will not be configured in RRC_IDLE state. | This test is the same as steps 1 - 4 of 38.523-1 11.4.3 -the only difference is that this uses MT call release and 11.4.3 uses MO; this has no impact on the TP. |
| 10.8 | 19.4.2 | Emergency call without emergency registration / UE contains an ISIM or USIM / UE is in state 5GMM-REGISTERED.LIMITED-SERVICE / 5GS | This is a limited-service TC whose preamble requires the UE to change cells and receive reject cause #15 - No suitable cells in tracking area. | This test is the same as steps 1 - 10C of 38.523-1 11.4.5 - the only difference is that this uses MT call release and 11.4.3 uses MO; this has no impact on the TP. The only TP of 10.8 is already the same as TP1 of 11.4.5. |
| 10.11 | 19.5.1 | New initial emergency registration after obtaining new IP address different than the IP address / 5GS | This requires the SS to send a NAS Deregister message with cause value "re-attach required" during the test body | Due to its complexity, the LTE equivalent TC is included in IMS_IRAT |
| 10.15 | 19.5.10 | Deregistration upon emergency registration expiration / 5GS | This checks that the UE doesn't attempt to re-register the UE after an emergency call has finished. The final TP is to check that the IMS Register message isn't sent, but in practice, this will be (and has been implemented in 19.5.10) to check the UE doesn't send an RRC Setup Request. | Due to the requirement to check the message is not received on the RRC layer, the LTE equivalent TC is included in IMS_IRAT |

Proposal#1

To move the TS 34.229-5 test cases listed above to the multi-layer emergency service section in TS 38.523-1 clause 11.4.*.

As mentioned above, TCs 10.7 and 10.8 are already covered in existing TCs defined in this section.

Proposal#2

Keep the test cases listed above as currently defined in TS 34.229-5 but implement these in TTCN in a separate ATS – a 5GS equivalent of “IMS_IRAT”.

## 3.	Proposal/Way Forward

MCC TF160 support proposal#1 – to move the affected test cases to TS 38.523-1 as this will reduce test case redundancy and reduce the total number of ATS to manage (i.e. reduce TF160 workload).

## 4.	References

[1]	TS 34.229-5: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 5: Protocol conformance specification using 5G System (5GS)

[2]	TS 34.229-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification

[3]	 TS 36.523-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; EUTRA; User Equipment (UE) conformance specification; Part 1: Protocol

[4]	TS 38.523-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; 5GS; User Equipment (UE) conformance specification; Part 1: Protocol