3GPP TSG-RAN5 Meeting #96-e	R5-224695

# Electronic Meeting, 15th – 26th Aug. 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3151 | rev | - | Current version: | 16.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to NR TC 7.1.2.3.10 to test RedCap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - RedCap UE use 12bit SN for AM RLC, so the RLC header is 2 bytes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modify steps 19/20/23/24 for RedCap UE. - Modify Note 1/2/3/5/6 for RedCap UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the TC or a non-conformant UE may pass the TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.1.2.3.10	AM RLC / Re-transmission of RLC PDU with and without re-segmentation

7.1.2.3.10.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives a STATUS PDU including a NACK_SN for missing AMD PDUs and missing AMD PDUs can be transmitted as indicated by lower layer at the particular transmission opportunity }

then { UE successfully retransmits missing AMD PDUs without re-segmentation }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { NACK received for missing AMD PDUs and RETX_COUNT < maxRetxThreshold }

then { UE retransmits AMD PDUs }

}

(3)

with { UE in RRC_CONNECTED state }

ensure that {

when { AMD PDU to be retransmitted does not fit in new allocated TBS }

then { UE segments AMD PDU }

}

(4)

with { UE in RRC_CONNECTED state }

ensure that {

when { AMD PDU segment to be retransmitted does not fit in new allocated TBS }

then { UE re-segments AMD PDU segment to fit TBS }

}

7.1.2.3.10.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.322, clauses 5.3.2, 6.2.2.5, 6.2.3.3, 6.2.3.4, 6.2.3.5, 6.2.3.7, 6.2.3.10, 6.2.3.12, 6.2.3.14 and 6.2.3.15. Unless otherwise stated these are Rel-15 requirements.

[TS 38.322, clause 5.3.2]

The transmitting side of an AM RLC entity can receive a negative acknowledgement (notification of reception failure by its peer AM RLC entity) for an RLC SDU or an RLC SDU segment by the following:

-	STATUS PDU from its peer AM RLC entity.

When receiving a negative acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall:

-	if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer:

-	consider the RLC SDU or the RLC SDU segment for which a negative acknowledgement was received for retransmission.

When an RLC SDU or an RLC SDU segment is considered for retransmission, the transmitting side of the AM RLC entity shall:

-	if the RLC SDU or RLC SDU segment is considered for retransmission for the first time:

-	set the RETX_COUNT associated with the RLC SDU to zero.

-	else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU:

-	increment the RETX_COUNT.

-	if RETX_COUNT = maxRetxThreshold:

-	indicate to upper layers that max retransmission has been reached.

When retransmitting an RLC SDU or an RLC SDU segment, the transmitting side of an AM RLC entity shall:

-	if needed, segment the RLC SDU or the RLC SDU segment;

-	form a new AMD PDU which will fit within the total size of AMD PDU(s) indicated by lower layer at the particular transmission opportunity;

-	submit the new AMD PDU to lower layer.

When forming a new AMD PDU, the transmitting side of an AM RLC entity shall:

-	only map the original RLC SDU or RLC SDU segment to the Data field of the new AMD PDU;

-	modify the header of the new AMD PDU in accordance with the description in sub clause 6.2.2.4;

-	set the P field according to sub clause 5.3.3.

[TS 38.322, clause 6.2.2.4]

AMD PDU consists of a Data field and an AMD PDU header. The AMD PDU header is byte aligned.

An AM RLC entity is configured by RRC to use either a 12 bit SN or a 18 bit SN. The length of the AMD PDU header is two and three bytes respectively.

An AMD PDU header contains a D/C, a P, a SI, and a SN. An AMD PDU header contains the SO field only when the Data field consists of an RLC SDU segment which is not the first segment, in which case a 16 bit SO is present.

Figure 6.2.2.4-1: AMD PDU with 12 bit SN (No SO)

Figure 6.2.2.4-2: AMD PDU with 18 bit SN (No SO)

Figure 6.2.2.4-3: AMD PDU with 12 bit SN with SO

Figure 6.2.2.4-4: AMD PDU with 18 bit SN with SO

[TS 38.322, clause 6.2.2.5]

STATUS PDU consists of a STATUS PDU payload and an RLC control PDU header.

RLC control PDU header consists of a D/C and a CPT field.

The STATUS PDU payload starts from the first bit following the RLC control PDU header, and it consists of one ACK_SN and one E1, zero or more sets of a NACK_SN, an E1, an E2 and an E3, and possibly a pair of a SOstart and a SOend or a NACK range field for each NACK_SN.

Figure 6.2.2.5-1: STATUS PDU with 12 bit SN

Figure 6.2.2.5-2: STATUS PDU with 18 bit SN

[TS 38.322, clause 6.2.3.3]

Length: 12 bits or 18 bits (configurable) for AMD PDU. 6 bits or 12 bits (configurable) for UMD PDU.

The SN field indicates the sequence number of the corresponding RLC SDU. For RLC AM, the sequence number is incremented by one for every RLC SDU. For RLC UM, the sequence number is incremented by one for every segmented RLC SDU.

[TS 38.322, clause 6.2.3.4]

Length: 2 bits.

The SI field indicates whether an RLC PDU contains a complete RLC SDU or the first, middle, last segment of an RLC SDU.

Table 6.2.3.4-1: SI field interpretation

| Value | Description |
| --- | --- |
| 00 | Data field contains all bytes of an RLC SDU |
| 01 | Data field contains the first segment of an RLC SDU |
| 10 | Data field contains the last segment of an RLC SDU |
| 11 | Data field contains neither the first nor last segment of an RLC SDU |

[TS 38.322, clause 6.2.3.5]

Length: 16 bits

The SO field indicates the position of the RLC SDU segment in bytes within the original RLC SDU. Specifically, the SO field indicates the position within the original RLC SDU to which the first byte of the RLC SDU segment in the Data field corresponds. The first byte of the original RLC SDU is referred by the SO field value "0000000000000000", i.e., numbering starts at zero.

[TS 38.322, clause 6.2.3.7]

Length: 1 bit.

The P field indicates whether or not the transmitting side of an AM RLC entity requests a STATUS report from its peer AM RLC entity. The interpretation of the P field is provided in Table 6.2.3.7-1.

Table 6.2.3.7-1: P field interpretation

| Value | Description |
| --- | --- |
| 0 | Status report not requested |
| 1 | Status report is requested |

[TS 38.322, clause 6.2.3.10]

Length: 12 bits or 18 bits (configurable).

The ACK_SN field indicates the SN of the next not received RLC SDU which is not reported as missing in the STATUS PDU. When the transmitting side of an AM RLC entity receives a STATUS PDU, it interprets that all RLC SDUs up to but not including the RLC SDU with SN = ACK_SN have been received by its peer AM RLC entity, excluding those RLC SDUs indicated in the STATUS PDU with NACK_SN, portions of RLC SDUs indicated in the STATUS PDU with NACK_SN, SOstart and SOend, RLC SDUs indicated in the STATUS PDU with NACK_SN and NACK_range, and portions of RLC SDUs indicated in the STATUS PDU with NACK_SN, NACK range, SOstart and SOend.

[TS 38.322, clause 6.2.3.12]

Length: 12 bits or 18 bits (configurable).

The NACK_SN field indicates the SN of the RLC SDU (or RLC SDU segment) that has been detected as lost at the receiving side of the AM RLC entity.

[TS 38.322, clause 6.2.3.14]

Length: 16 bits.

The SOstart field (together with the SOend field) indicates the portion of the RLC SDU with SN = NACK_SN (the NACK_SN for which the SOstart is related to) that has been detected as lost at the receiving side of the AM RLC entity. Specifically, the SOstart field indicates the position of the first byte of the portion of the RLC SDU in bytes within the original RLC SDU. The first byte of the original RLC SDU is referred by the SOstart field value "0000000000000000", i.e., numbering starts at zero.

[TS 38.322, clause 6.2.3.15]

Length: 16 bits.

When E3 is 0, the SOend field (together with the SOstart field) indicates the portion of the RLC SDU with SN = NACK_SN (the NACK_SN for which the SOend is related to) that has been detected as lost at the receiving side of the AM RLC entity. Specifically, the SOend field indicates the position of the last byte of the portion of the RLC SDU in bytes within the original RLC SDU. The first byte of the original RLC SDU is referred by the SOend field value "0000000000000000", i.e., numbering starts at zero. The special SOend value "1111111111111111" is used to indicate that the missing portion of the RLC SDU includes all bytes to the last byte of the RLC SDU.

When E3 is 1, the SOend field indicates the portion of the RLC SDU with SN = NACK_SN + NACK range - 1 that has been detected as lost at the receiving side of the AM RLC entity. Specifically, the SOend field indicates the position of the last byte of the portion of the RLC SDU in bytes within the original RLC SDU. The first byte of the original RLC SDU is referred by the SOend field value "0000000000000000", i.e., numbering starts at zero. The special SOend value "1111111111111111" is used to indicate that the missing portion of the RLC SDU includes all bytes to the last byte of the RLC SDU.

7.1.2.3.10.3	Test description

7.1.2.3.10.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.1 with the exception for the AM DRB is configured according to Tables 7.1.2.3.10.3.1-1.

Table 7.1.2.3.10.3.1-1: RLC settings

| Parameter | Value |
| --- | --- |
| t-PollRetransmit | ms150 |

7.1.2.3.10.3.2	Test procedure sequence

Table 7.1.2.3.10.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits one AMD PDU containing SDU#1 (91 bytes) in its data field. | <-- | AMD PDU#1 | - | - |
| 2 | The UE transmits one AMD PDU containing SDU#1 in its data field. | --> | AMD PDU#1 (SN=0) | - | - |
| 3 | The SS transmits one AMD PDU containing SDU#2 (91 bytes) in its data field. | <-- | AMD PDU#2 | - | - |
| 4 | The UE transmits one AMD PDU containing SDU#2 in its data field. | --> | AMD PDU#2 (SN=1) | - | - |
| 5 | The SS transmits a RLC STATUS PDU. ACK_SN=2, NACK_SN=0. | <-- | STATUS PDU | - | - |
| 6 | Check: Does the UE transmit the AMD PDU not yet acknowledged? | --> | AMD PDU#1 (SN=0) | 1 | P |
| 7 | The SS transmits a RLC STATUS PDU. ACK_SN=2. | <-- | STATUS PDU | - | - |
| 8 | The SS transmits one AMD PDU containing SDU#3 (91 bytes) in its data field. | <-- | AMD PDU#3 | - | - |
| 9 | The UE transmits an AMD PDU containing SDU#3 in its data field. | --> | AMD PDU#3 (SN=2) | - | - |
| - | EXCEPTION: Steps 10 to 11 shall be repeated until RETX_COUNT= maxRetxThreshold-1. | - | - | - | - |
| 10 | The SS transmits a RLC STATUS PDU. ACK_SN=3 and NACK_SN=2. | <-- | STATUS PDU | - | - |
| 11 | Check: Does the UE retransmit the AMD PDU not yet acknowledged? | --> | AMD PDU#3 (SN=2) | 2 | P |
| 12 | The SS transmits a RLC STATUS PDU. ACK_SN=3. | <-- | STATUS PDU | - | - |
| 13 | The SS stops the UL grant transmission. | - | - | - | - |
| 14 | The SS transmits one AMD PDU containing SDU#4 (91 bytes) in its data field. | <-- | AMD PDU#4 (SN=3) | - | - |
| 15 | The SS waits for 60 ms and allocates one UL grant of size 808 bits. (Note 1) | <-- | (UL grant, 808 bits) | - | - |
| 16 | The UE transmits an AMD PDU with the same data contents as received in the corresponding part of SDU#4? | --> | AMD PDU#4 (SN=3) | - | - |
| 17 | The SS transmits a STATUS PDU. This PDU nacks the AMD PDU with SN=3. ACK_SN=4 and NACK_SN=3. | <-- | STATUS PDU | - | - |
| 18 | The SS waits for 100 ms and then allocates 1 UL grant of size 408 bits (Note 2) | <-- | (UL grant, 408 bits) | - | - |
| 18A | The SS waits for 20 ms and then allocates 1 UL grant of size 456 bits (Note 5) | <-- | (UL grant, 456 bits) | - | - |
| 19 | Check: Does the UE transmit an SDU segment with SI=01 and SOEnd=43 for pc_RedCap_r17=false or SOEnd=44 for pc_RedCap_r17=true and the same data contents at the received positions as in the original SDU#4? | --> | SDU#4 segment 1 (SN=3) | 3 | P |
| 20 | Check: Does the UE transmit an SDU segment with SI=10 and SOStart=44 for pc_RedCap_r17=false or SOStart=45 for pc_RedCap_r17=true and the same data contents at the received positions as in the original SDU#4? | --> | SDU#4 segment 2 (SN=3) | 3 | P |
| 21 | After 100 ms SS transmits a STATUS PDU. This PDU nacks the SDU with SN=3. NACK_SN=3, SOStart=0, SOEnd=43 for pc_RedCap_r17=false or SOEnd=44 for pc_RedCap_r17=true and ACK_SN=4. | <-- | STATUS PDU | - | - |
| 22 | The SS waits for 100 ms and then allocates 2 UL grants (UL grant allocation type 2) at an interval of 20 ms of size 240 bits (Note 3, Note 6) | <-- | (UL grant, 240 bits) | - | - |
| 22A | Void |  |  | - | - |
| 23 | Check: Does the UE transmit an AMD PDU segment with SI=01 and SOEnd=22 for pc_RedCap_r17=false or SOEnd=23 for pc_RedCap_r17=true and the same data contents at the received positions as in the original SDU#4? | --> | SDU#4 segment 1, first part (SN=3) | 4 | P |
| 24 | Check: Does the UE transmit an AMD PDU segment with SI=11, SOStart=23 for pc_RedCap_r17=false or SOStart=24 for pc_RedCap_r17=true and the same data contents at the received positions as in the original SDU#4? | --> | SDU#4 segment 1, second part (SN=3) | 4 | P |
| 25 | The SS transmits a STATUS PDU. This PDU acks the AMD PDUs with SN=3. ACK_SN=4. | <-- | STATUS PDU | - | - |
| Note 1: UL grant of 808 bits=101 bytes (LRBs & IMCS as per 38.523-3[3] annex B) is chosen to allow the UE to transmit one PDU (91 bytes RLC SDU and 3 bytes RLC Header and 2 bytes MAC Sub PDU header and 2 bytes short BSR + 3 bytes padding or 5 bytes padding) for pc_RedCap_r17=false, or one PDU (91 bytes RLC SDU and 2 bytes RLC Header and 2 bytes MAC Sub PDU header and 2 bytes short BSR + 4 bytes padding or 6 bytes padding) for pc_RedCap_r17=true at a time. Note 2: UL grant of 408 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen such that UE will segment into 2 AMD PDUs. MAC PDU of 408 bits=51 bytes fit an AMD PDU payload of 44 bytes + 3 bytes for the first segment of the AMD PDU header + 2 bytes for MAC header + 2 bytes of MAC BSR CE for pc_RedCap_r17=false, or an AMD PDU payload of 45 bytes + 2 bytes for the first segment of the AMD PDU header + 2 bytes for MAC header + 2 bytes of MAC BSR CE for pc_RedCap_r17=true. Note 3: UL grant of 240 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen such that UE will segment into 2 AMD PDUs. MAC PDU of 240 bits=30 bytes fit an AMD PDU payload of = 23 bytes + 3 bytes for the first segment of the AMD PDU header + 2 bytes for MAC header + 2 bytes for MAC BSR CE for pc_RedCap_r17=false, or an AMD PDU payload of = 24 bytes + 2 bytes for the first segment of the AMD PDU header + 2 bytes for MAC header + 2 bytes for MAC BSR CE for pc_RedCap_r17=true. Note 4: Void. Note 5: UL grant of 456 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen such that UE will segment into 2 AMD PDUs. MAC PDU of 456 bits=57 bytes fit an AMD PDU payload of 47 bytes + 5 bytes for the second segment of the AMD PDU header + 2 bytes for MAC header +2 bytes for possible short BSR and 1 byte padding or only 3 bytes padding for pc_RedCap_r17=false, or an AMD PDU payload of 46 bytes + 4 bytes for the second segment of the AMD PDU header + 2 bytes for MAC header +2 bytes for possible short BSR and 3 byte padding or only 5 bytes padding for pc_RedCap_r17=true. Note 6: UL grant of 240 bits (LRBs & IMCS as per 38.523-3[3] annex B) is chosen such that UE will segment into 2 AMD PDUs. MAC PDU of 240 bits=28 bytes fit an AMD PDU payload of 21 bytes + 3 bytes for the second segment of the AMD PDU header + 2 bytes for MAC header +2 bytes for possible short BSR or padding for pc_RedCap_r17=false,or an AMD PDU payload of 20 bytes + 2 bytes for the second segment of the AMD PDU header + 2 bytes for MAC header +2 bytes for possible short BSR and 2 bytes padding or only 4 bytes padding for pc_RedCap_r17=true. |  |  |  |  |  |

7.1.2.3.10.3.3	Specific message contents

None.

<End of modified section 1>