3GPP TSG-RAN5 Meeting #96-e	R5-224814

# Online, , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1829 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add PC2 test configuration and requirement table for n1 A-MPR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_PC2_UE_FDD-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | High Power UE for NR FDD n1 was introduced by RAN4 , the relevant test description need to be specificed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added A-MPR test configuation and requirement for n1 PC2 case |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage is not fully specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

<Unchanged Text Skipped>

6.2.3.3.4	A-MPR for NS_05 and NS_05U

Table 6.2.3.3.4-1: A-MPR regions for NS_05 and NS_05U (Power Class 3)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 | 1922.5 ≤ FC < 1927.5 | <1.62MHz/12/SCS | >2.52MHz/12/SCS | A3 |  |  |  |  |  |  |
| 10 | 1925 ≤ FC < 1935 | <1.62MHz/12/SCS | >0 | A1 | >1.62MHz/12/SCS ≤3.60MHz/12/SCS | >5.4MHz/12/SCS | A7 | ≥7.2MHz/12/SCS | ≤1.08MHz/12/SCS | A2 |
| 10 | 1935 ≤ FC < 1945 |  | >4.5MHz/12/SCS | A4 |  |  |  |  |  |  |
| 15 | 1927.5 ≤ FC < 1932.5 | <3.24MHz/12/SCS | >0 | A1 | >3.24MHz/12/SCS ≤5.40MHz/12/SCS | >8.1MHz/12/SCS | A7 | ≥10.08MHz/12/SCS | ≤1.08MHz/12/SCS | A2 |
| 15 | 1932.5 ≤ FC < 1942.5 | <1.62MHz/12/SCS | >0 | A1 |  |  |  | ≥12.24 MHz/12/SCS | ≤1.08MHz/12/SCS | A2 |
| 15 | 1942.5 ≤ FC < 1947.5 |  | >7.2MHz/12/SCS | A5 |  |  |  |  |  |  |
| 20 | 1930 ≤ FC < 1950 | <4.86MHz/12/SCS | >0 | A1 | >4.86MHz/12/SCS ≤7.20MHz/12/SCS | >9.0MHz/12/SCS | A7 | ≥13.68 MHz/12/SCS | ≤1.08MHz/12/SCS | A2 |
| 20 | 1950 ≤ FC <1960 |  | >9.0MHz/12/SCS | A6 |  |  |  |  |  |  |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.3.4-2 and 6.2.3.3.4-3. NOTE 2: Void. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.3.4-2: A-MPR for NS_05 and NS_05U (Power Class 3)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer | Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤ 10 | ≤ 5 | ≤ 4 |  |
|  | QPSK | ≤ 10 | ≤ 5 | ≤ 4.5 |  |
|  | 16 QAM | ≤ 10 | ≤ 5 | ≤ 6 |  |
|  | 64 QAM | ≤ 11 | ≤ 5 | ≤ 6 |  |
|  | 256 QAM | ≤ 13 | ≤ 5 | ≤ 7 |  |
| CP-OFDM | QPSK | ≤ 10 | ≤ 5 | ≤ 7.5 | ≤ 2 |
|  | 16 QAM | ≤ 10 | ≤ 5 | ≤ 7.5 |  |
|  | 64 QAM | ≤ 11 | ≤ 5 | ≤ 8 |  |
|  | 256 QAM | ≤ 13 |  | ≤ 10 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |

Table 6.2.3.3.4-3: A-MPR for NS_05 (Power Class 3)

| Modulation/Waveform |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |  | A7 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1 | N/A | ≤ 1 |  | ≤ 1 | N/A | ≤ 6 |
|  | QPSK |  |  | ≤ 1.5 |  | ≤ 1.5 |  | ≤ 6 |
|  | 16 QAM |  |  |  |  |  |  | ≤ 6 |
|  | 64 QAM |  |  |  |  |  |  | ≤ 6 |
|  | 256 QAM |  |  |  |  |  |  | ≤ 6 |
| CP-OFDM | QPSK | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 6 |
|  | 16 QAM | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 3.5 |  | ≤ 6 |
|  | 64 QAM |  |  |  | ≤ 4 |  |  | ≤ 6 |
|  | 256 QAM |  |  |  |  |  |  | ≤ 6 |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |  |

Table 6.2.3.3.4-4: A-MPR for modulation and waveform type for NS_05U (Power Class 3)

| Modulation/Waveform |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |  | A7 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 | N/A | ≤ 2 |  | ≤ 2 | N/A | ≤ 6 |
|  | QPSK | ≤ 2 |  | ≤ 2 |  | ≤ 2 |  | ≤ 6 |
|  | 16 QAM | ≤ 2.5 |  | ≤ 2.5 |  | ≤ 2.5 |  | ≤ 6 |
|  | 64 QAM | ≤ 3 |  | ≤ 3 |  | ≤ 3 |  | ≤ 6 |
|  | 256 QAM | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 6 |
| CP-OFDM | QPSK | ≤ 4 |  | ≤ 4 |  | ≤ 4 |  | ≤ 6 |
|  | 16 QAM | ≤ 4 |  | ≤ 4 |  | ≤ 4 |  | ≤ 6 |
|  | 64 QAM | ≤ 4 |  | ≤ 4 | ≤ 4 | ≤ 4 |  | ≤ 6 |
|  | 256 QAM | ≤ 6.5 |  | ≤ 6.5 |  | ≤ 6.5 |  | ≤ 6.5 |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |  |

Table 6.2.3.4-11: A-MPR regions for NS_05 and NS_05U (Power Class 2)

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 | 1922.5 ≤ FC < 1927.5 | < 1.98 MHz/12/SCS | > 1.44 MHz/12/SCS | A3 | < 0.72 MHz/12/SCS | ≤ 1.44 MHz/12/SCS | A4 |  |  |  |
| 10 | 1925 ≤ FC < 1935 | < 1.98 MHz/12/SCS | > 0 | A1 | ≥ 1.98 MHz/12/SCS | > max(0, RBstart-1.08 MHz/12/SCS) | A7 | ≥ 7.2 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A1 |
|  |  |  |  |  | ≥ 1.98, ≤2.7 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A8 |  |  |  |
| 10 | 1935 ≤ FC < 1945 |  | > 3.96 MHz/12/SCS | A4 |  |  |  |  |  |  |
| 15 | 1927.5 ≤ FC < 1932.5 | < 3.6 MHz/12/SCS | > 0 | A1 | ≥ 3.6 MHz/12/SCS | > max(0, RBstart-1.8 MHz/12/SCS) | A7 | ≥ 10.08 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A1 |
|  |  |  |  |  | ≥ 3.6, ≤4.68 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A8 |  |  |  |
| 15 | 1932.5 ≤ FC < 1942.5 | < 1.98 MHz/12/SCS | > 0 | A1 | ≥ 1.98 MHz/12/SCS | > max(0, RBstart+1.08 MHz/12/SCS) | A7 | ≥ 12.24 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A1 |
| 15 | 1942.5 ≤ FC < 1947.5 |  | > 5.04 MHz/12/SCS | A5 |  |  |  |  |  |  |
| 20 | 1930 ≤ FC < 1950 | < 5.04 MHz/12/SCS | > 0 | A1 | ≥ 5.04 MHz/12/SCS | > max(0, RBstart-3.6 MHz/12/SCS) | A7 | ≥ 13.68 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A1 |
|  |  |  |  |  | ≥ 5.04, ≤6.66 MHz/12/SCS | ≤ 1.08 MHz/12/SCS | A8 |  |  |  |
| 20 | 1950 ≤ FC < 1960 |  | > 9.0 MHz/12/SCS | A6 |  |  |  |  |  |  |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.4-12 and 6.2.3.4-13. NOTE 2: Void |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4-12: A-MPR for NS_05 and NS_05U (Power Class 2)

| Modulation/Waveform |  | A1 (dB) | A2 (dB) | A3 (dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [13] | ≤ 6 | ≤ 6.5 | ≤ 2 |
|  | QPSK | ≤ [13] | ≤ 6 | ≤ 7 | ≤ 2 |
|  | 16 QAM | ≤ [13] | ≤ 6 | ≤ 8.5 | ≤ 2 |
|  | 64 QAM | ≤ [14] | ≤ 6 | ≤ 9 | ≤ 2 |
|  | 256 QAM | ≤ [15] | ≤ 6 | ≤ 9.5 |  |
| CP-OFDM | QPSK | ≤ [13] | ≤ 6 | ≤ 10 | ≤ 4 |
|  | 16 QAM | ≤ [13] | ≤ 6 | ≤ 10 | ≤ 4 |
|  | 64 QAM | ≤ [14] | ≤ 6 | ≤ 10 | ≤ 4 |
|  | 256 QAM | ≤ [16] |  | ≤ 10 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |

Table 6.2.3.4-13: A-MPR for NS_05 and NS_05U (Power Class 2)

| Modulation/Waveform |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |  | A7 (dB) | A8 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3 | N/A | ≤ 2 |  | ≤ 2 | N/A | ≤ 8 | ≤ 3.5 |
|  | QPSK | ≤ 3 |  | ≤ 2 |  | ≤ 2 |  | ≤ 8 | ≤ 3.5 |
|  | 16 QAM | ≤ 3.5 |  | ≤ 2.5 |  | ≤ 2 |  | ≤ 8 | ≤ 3.5 |
|  | 64 QAM | ≤ 3.5 |  | ≤ 2.5 |  |  |  | ≤ 8 | ≤ 3.5 |
|  | 256 QAM |  |  |  |  |  |  | ≤ 8 |  |
| CP-OFDM | QPSK | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 4 |  | ≤ 8.5 | ≤ 3.5 |
|  | 16 QAM | ≤ 4.5 |  | ≤ 4.5 |  | ≤ 4 |  | ≤ 8.5 | ≤ 3.5 |
|  | 64 QAM | ≤ 5 |  | ≤ 5 | ≤ 5 | ≤ 4 |  | ≤ 8.5 |  |
|  | 256 QAM |  |  |  |  |  |  | ≤ 8.5 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.2.3.3.26	A-MPR for NS_48

Table 6.2.3.26-3: A-MPR regions for NS_48 (Power Class 2)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  |  | A-MPR |
| --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz |  | LCRB*12*SCS MHz |  |
| 10 MHz | 1925≤ FC ≤ 1975 |  | ≥0 | ≥8.1 | A6 |
|  |  |  | <1.8 | ≥0 | A6 |
| 15 MHz | 1927.5≤ FC ≤ 1972.5 |  | ≥0 | ≥9 | A6 |
|  |  |  | ≥0 | ≥max (0,12*SCS* RBend - 2.88) <9 | A6 |
| 20 MHz | 1930≤ FC ≤ 1970 |  | ≥0 | ≥9.72 | A4 |
|  |  |  | ≥0 | ≥max (0,12*SCS* RBend - 3.6) <9.72 | A6 |
| 25 MHz | 1932.5≤ FC ≤ 1967.5 | ≥0 |  | ≥9.72 | A4 |
|  |  | ≥18.72 |  | <1.08 | A3 |
|  |  | ≥0 |  | ≥max (0, 12*SCS*RBend - 1.08) < 9.72 | A4 |
| 30 MHz | 1935 ≤ FC ≤ 1965 | ≥0 |  | ≥12.96 | A4 |
|  |  | ≥21.6 |  | <1.08 | A5 |
|  |  | ≥0 |  | ≥max (0, 12*SCS*RBend - 3.6) < 12.96 | A4 |
| 40 MHz | 1940 ≤ FC ≤ 1960 | ≥0, <2.88 |  | ≥0 | A1 |
|  |  | ≥2.88, <17.1 |  | ≥max (0, 12*SCS*RBend - 4.68) | A3 |
|  |  | ≥17.1, <27.36 |  | ≥12.96 | A4 |
|  |  | ≥27.36, <34.56 |  | ≥12.96 | A2 |
|  |  | ≥27.36, <34.56 |  | <1.08 | A3 |
|  |  | ≥34.56 |  | ≥0 | A1 |
| 45 MHz | 1942.5 ≤ FC ≤ 1957.5 | ≥0, <5.22 |  | >0 | A1 |
|  |  | ≥5.22, <19 |  | ≥max (0, 12*SCS*RBend - 5.4) | A4 |
|  |  | ≥19, <37.44 |  | ≥14.04 | A2 |
|  |  | ≥30.96, <37.44 |  | <1.08 | A5 |
|  |  | ≥37.44 |  | >0 | A1 |
| 50 MHz | 1945 ≤ FC ≤ 1955 | ≥0, <7.2 |  | >0 | A1 |
|  |  | ≥7.2, <20.7 |  | ≥max (0, 12*SCS*RBend – 5.4) | A4 |
|  |  | ≥20.7, <41.04 |  | ≥15.12 | A2 |
|  |  | ≥33.84, <41.04 |  | <1.08 | A5 |
|  |  | ≥41.04 |  | >0 | A1 |

Table 6.2.3.26-4: A-MPR for NS_48 (Power Class 2)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤[12] | ≤8.0 | ≤4.0 | ≤4.5 | ≤6.0 | ≤3.0 |
|  | QPSK | ≤[12] | ≤8.0 | ≤4.0 | ≤4.5 | ≤6.0 | ≤3.0 |
|  | 16 QAM | ≤[12] | ≤8.0 | ≤4.0 | ≤4.5 | ≤6.0 | ≤3.0 |
|  | 64 QAM | ≤[12] | ≤8.0 | ≤4.0 | ≤4.5 | ≤6.0 | ≤3.0 |
|  | 256 QAM | ≤[12] | ≤8.0 | ≤4.0 | ≤4.5 | ≤6.0 | ≤3.0 |
| CP-OFDM | QPSK | ≤[12.5] | ≤9.0 | ≤5.5 | ≤6.0 | ≤6.0 | ≤4.5 |
|  | 16 QAM | ≤[12.5] | ≤9.0 | ≤5.5 | ≤6.0 | ≤6.0 | ≤4.5 |
|  | 64 QAM | ≤[12.5] | ≤9.0 | ≤5.5 | ≤6.0 | ≤6.0 | ≤4.5 |
|  | 256 QAM | ≤[12.5] | ≤9.0 | ≤5.5 | ≤6.0 | ≤6.0 | ≤4.5 |

<Unchanged Text Skipped>

6.2.3.27	A-MPR for NS_49

<Unchanged Text Skipped>

Table 6.2.3.27-3: A-MPR regions for NS_49 (Power Class 2)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 1925≤ FC ≤ 1975 | ≥0 | ≥8.1 | A3 |
|  |  | <1.8 | ≥0 | A3 |
| 15 MHz | 1927.5≤ FC ≤ 1972.5 | ≥0 | ≥9 | A3 |
|  |  | ≥0 | ≥max (0,12*SCS* RBend - 2.88) <9 | A3 |
| 20 MHz | 1930≤ FC ≤ 1970 | ≥0 | ≥9.72 | A4 |
|  |  | ≥0 | ≥max (0,12*SCS* RBend - 3.6) <9.72 | A3 |
| 25 MHz | 1932.5≤ FC ≤ 1967.5 | ≥0 | ≥7.92 | A4 |
|  |  | ≥18.72 | <1.08 | A3 |
|  |  | ≤6.48 | <3.6 | A3 |
| 30 MHz | 1935 ≤ FC ≤ 1965 | ≥0, <3.6 | ≥0 | A1 |
|  |  | ≥3.6, <7.92 | ≥0 | A5 |
|  |  | ≥7.92, <14.4 | ≥max (0,12*SCS* RBend - 4.32) | A3 |
|  |  | ≥14.4, <21.6 | ≥10.44 | A4 |
|  |  | ≥21.6 | ≥10.44 | A2 |
|  |  | ≥21.6 | <1.8 | A1 |
| 40 MHz | 1940 ≤ FC ≤ 1960 | ≥0, <9 | ≥0 | A1 |
|  |  | ≥9, <11.52 | <3.06 | A5 |
|  |  | ≥9, <18 | ≥max (3.06, 12*SCS*RBend – 6.48) | A4 |
|  |  | ≥18, <34.56 | ≥11.16, <27 | A2 |
|  |  | ≥27.36, <34.56 | <1.08 | A5 |
|  |  | <34.56 | ≥27 | A1 |
|  |  | ≥34.56 | ≥0 | A1 |
|  |  | ≥7.92, <12.42 | < min [1.08, max(0,12*SCS* RBend-7.92)] | A5 |
|  |  | ≥30.76, <36.72 | <1.08 | A5 |
| 45 MHz | 1942.5 ≤ FC ≤ 1957.5 | <36.72 | ≥12.24, <max (0, 12*SCS*RBend – 7.92) | A2 |
|  |  | <36.72 | ≥max (0, 12*SCS*RBend – 7.92) | A1 |
|  |  | ≥36.72 | >0 | A1 |
| 50 MHz | 1945 ≤ FC ≤ 1955 | ≥10.08, <14.4 | < min [1.08, max(0,12*SCS* RBend-10.08)] | A5 |
|  |  | ≥36, <39.6 | <1.08 | A5 |
|  |  | <39.6 | ≥13.68, <max (0, 12*SCS*RBend – 10.08) | A2 |
|  |  | <39.6 | ≥max (0, 12*SCS*RBend – 10.08) | A1 |
|  |  | ≥39.6 | >0 | A1 |

Table 6.2.3.27-4: A-MPR for NS_49 (Power Class 2)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤[12.5] | ≤7 | ≤4 | ≤5.5 | ≤6 |
|  | QPSK | ≤[12.5] | ≤7 | ≤4 | ≤5.5 | ≤6 |
|  | 16 QAM | ≤[12.5] | ≤7 | ≤4 | ≤5.5 | ≤6 |
|  | 64 QAM | ≤[12.5] | ≤7 | ≤4 | ≤5.5 | ≤6 |
|  | 256 QAM | ≤[12.5] | ≤7 | ≤4 | ≤5.5 | ≤6 |
| CP-OFDM | QPSK | ≤[13.5] | ≤8 | ≤5.5 | ≤7.5 | ≤6 |
|  | 16 QAM | ≤[13.5] | ≤8 | ≤5.5 | ≤7.5 | ≤6 |
|  | 64 QAM | ≤[13.5] | ≤8 | ≤5.5 | ≤7.5 | ≤6 |
|  | 256 QAM | ≤[13.5] | ≤8 | ≤5.5 | ≤7.5 | ≤6 |

<Unchanged Text Skipped>

6.2.3.4	Test description

6.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in tables 6.2.3.4.1-1 to 6.2.3.4.1-30. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Network signalling value NS_10 is not tested. The relaxation specified under NS_10 is for the purpose to control / mitigate self-desensitization of the UE’s own receiver, but no corresponding REFSENS requirements are specified. There is no need to test A-MPR requirements for its own.

<Unchanged Text Skipped>

Table 6.2.3.4.1-4: Test Configuration table for NS_05 power class 3

<Unchanged Text Skipped>

Table 6.2.3.4.1-5: Test Configuration table for NS_05U power class 3

Table 6.2.3.4.1-4a: Test Configuration table for NS_05 and NS_05U power class 2

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier centre frequency (Fc) as specified in test parameters |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest unless otherwise specified in test parameters. |  |  |  |
| A-MPR test parameters for NS_05 and NS_05U |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | 1922.5 | 5 | 15 | N/A for A-MPR testing | A3 | DFT-s-OFDM | PI/2 BPSK |  | Outer_Full |  |  |
| 2 | 1922.5 | 5 | Default |  | A4 |  | PI/2 BPSK |  | 8@1 | 4@1 |  |
| 3 | 1925 | 10 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |
| 4 | 1925 | 10 | Default |  | A7 |  | PI/2 BPSK |  | 30@11 | 15@6 | 6@3 |
| 5 | 1925 | 10 | Default |  | A2 |  | PI/2 BPSK |  | 6@40 | 3@20 | 1@10 |
| 6 | 1925 | 10 | Default |  | A8 |  | PI/2 BPSK |  | 6@11 | 3@6 | 1@3 |
| 7 | 1935 | 10 | Default |  | A4 |  | PI/2 BPSK |  | Outer_Full |  |  |
| 8 | 1927.5 | 15 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |
| 9 | 1927.5 | 15 | Default |  | A7 |  | PI/2 BPSK |  | 52@20 | 26@10 | 12@5 |
| 10 | 1927.5 | 15 | Default |  | A2 |  | PI/2 BPSK |  | 6@56 | 3@28 | 1@14 |
| 11 | 1927.5 | 15 | Default |  | A8 |  | PI/2 BPSK |  | 6@20 | 3@10 | 1@5 |
| 12 | 1932.5 | 15 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |
| 13 | 1932.5 | 15 | Default |  | A7 |  | PI/2 BPSK |  | 62@11 | 30@6 | 14@3 |
| 14 | 1932.5 | 15 | Default |  | A2 |  | PI/2 BPSK |  | 6@68 | 3@34 | 1@17 |
| 15 | 1942.5 | 15 | Default |  | A5 |  | PI/2 BPSK |  | Outer_Full |  |  |
| 16 | 1930 | 20 | Default |  | A1 |  | PI/2 BPSK |  | Outer_Full |  |  |
| 17 | 1930 | 20 | Default |  | A7 |  | PI/2 BPSK |  | 72@28 | 36@14 | 16@7 |
| 18 | 1930 | 20 | Default |  | A2 |  | PI/2 BPSK |  | 6@76 | 3@38 | 1@19 |
| 19 | 1930 | 20 | Default |  | A8 |  | PI/2 BPSK |  | 6@28 | 3@14 | 1@7 |
| 20 | 1950 | 20 | Default |  | A6 |  | PI/2 BPSK |  | Outer_Full |  |  |
| 21 | 1922.5 | 5 | 15 |  | A3 |  | QPSK |  | Outer_Full |  |  |
| 22 | 1922.5 | 5 | Default |  | A4 |  | QPSK |  | 8@1 | 4@1 |  |
| 23 | 1925 | 10 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |
| 24 | 1925 | 10 | Default |  | A7 |  | QPSK |  | 30@11 | 15@6 | 6@3 |
| 25 | 1925 | 10 | Default |  | A2 |  | QPSK |  | 6@40 | 3@20 | 1@10 |
| 26 | 1925 | 10 | Default |  | A8 |  | QPSK |  | 6@11 | 3@6 | 1@3 |
| 27 | 1935 | 10 | Default |  | A4 |  | QPSK |  | Outer_Full |  |  |
| 28 | 1927.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |
| 29 | 1927.5 | 15 | Default |  | A7 |  | QPSK |  | 52@20 | 26@10 | 12@5 |
| 30 | 1927.5 | 15 | Default |  | A2 |  | QPSK |  | 6@56 | 3@28 | 1@14 |
| 31 | 1927.5 | 15 | Default |  | A8 |  | QPSK |  | 6@20 | 3@10 | 1@5 |
| 32 | 1932.5 | 15 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |
| 33 | 1932.5 | 15 | Default |  | A7 |  | QPSK |  | 62@11 | 30@6 | 14@3 |
| 34 | 1932.5 | 15 | Default |  | A2 |  | QPSK |  | 6@68 | 3@34 | 1@17 |
| 35 | 1942.5 | 15 | Default |  | A5 |  | QPSK |  | Outer_Full |  |  |
| 36 | 1930 | 20 | Default |  | A1 |  | QPSK |  | Outer_Full |  |  |
| 37 | 1930 | 20 | Default |  | A7 |  | QPSK |  | 72@28 | 36@14 | 16@7 |
| 38 | 1930 | 20 | Default |  | A2 |  | QPSK |  | 6@76 | 3@38 | 1@19 |
| 39 | 1930 | 20 | Default |  | A8 |  | QPSK |  | 6@28 | 3@14 | 1@7 |
| 40 | 1950 | 20 | Default |  | A6 |  | QPSK |  | Outer_Full |  |  |
| 41 | 1922.5 | 5 | 15 |  | A3 |  | 16 QAM |  | Outer_Full |  |  |
| 42 | 1922.5 | 5 | Default |  | A4 |  | 16 QAM |  | 8@1 | 4@1 |  |
| 43 | 1925 | 10 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |
| 44 | 1925 | 10 | Default |  | A7 |  | 16 QAM |  | 30@11 | 15@6 | 6@3 |
| 45 | 1925 | 10 | Default |  | A2 |  | 16 QAM |  | 6@40 | 3@20 | 1@10 |
| 46 | 1925 | 10 | Default |  | A8 |  | 16 QAM |  | 6@11 | 3@6 | 1@3 |
| 47 | 1935 | 10 | Default |  | A4 |  | 16 QAM |  | Outer_Full |  |  |
| 48 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM |  | Outer_Full |  |  |
| 49 | 1927.5 | 15 | Default |  | A7 |  | 16 QAM |  | 52@20 | 26@10 | 12@5 |
| 50 | 1927.5 | 15 | Default |  | A2 |  | 16 QAM |  | 6@56 | 3@28 | 1@14 |

| A-MPR test parameters for NS_05 and NS_05U |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz |  | SCS 60 kHz |
| 51 | 1927.5 | 15 | Default |  | A8 |  | 16 QAM | 6@20 |  | 3@10 | 1@5 |
| 52 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 53 | 1932.5 | 15 | Default |  | A7 |  | 16 QAM | 62@11 |  | 30@6 | 14@3 |
| 54 | 1932.5 | 15 | Default |  | A2 |  | 16 QAM | 6@68 |  | 3@34 | 1@17 |
| 55 | 1942.5 | 15 | Default |  | A5 |  | 16 QAM | Outer_Full |  |  |  |
| 56 | 1930 | 20 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |
| 57 | 1930 | 20 | Default |  | A7 |  | 16 QAM | 72@28 |  | 36@14 | 16@7 |
| 58 | 1930 | 20 | Default |  | A2 |  | 16 QAM | 6@76 |  | 3@38 | 1@19 |
| 59 | 1930 | 20 | Default |  | A8 |  | 16 QAM | 6@28 |  | 3@14 | 1@7 |
| 60 | 1950 | 20 | Default |  | A6 |  | 16 QAM | Outer_Full |  |  |  |
| 61 | 1922.5 | 5 | 15 |  | A3 |  | 64 QAM | Outer_Full |  |  |  |
| 62 | 1922.5 | 5 | Default |  | A4 |  | 64 QAM | 8@1 |  | 4@1 |  |
| 63 | 1925 | 10 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 64 | 1925 | 10 | Default |  | A7 |  | 64 QAM | 30@11 |  | 15@6 | 6@3 |
| 65 | 1925 | 10 | Default |  | A2 |  | 64 QAM | 6@40 |  | 3@20 | 1@10 |
| 66 | 1925 | 10 | Default |  | A8 |  | 64 QAM | 6@11 |  | 3@6 | 1@3 |
| 67 | 1935 | 10 | Default |  | A4 |  | 64 QAM | Outer_Full |  |  |  |
| 68 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 69 | 1927.5 | 15 | Default |  | A7 |  | 64 QAM | 52@20 |  | 26@10 | 12@5 |
| 70 | 1927.5 | 15 | Default |  | A2 |  | 64 QAM | 6@56 |  | 3@28 | 1@14 |
| 71 | 1927.5 | 15 | Default |  | A8 |  | 64 QAM | 6@20 |  | 3@10 | 1@5 |
| 72 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 73 | 1932.5 | 15 | Default |  | A7 |  | 64 QAM | 62@11 |  | 30@6 | 14@3 |
| 74 | 1932.5 | 15 | Default |  | A2 |  | 64 QAM | 6@68 |  | 3@34 | 1@17 |
| 75 | 1942.5 | 15 | Default |  | A5 |  | 64 QAM | Outer_Full |  |  |  |
| 76 | 1930 | 20 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |
| 77 | 1930 | 20 | Default |  | A7 |  | 64 QAM | 72@28 |  | 36@14 | 16@7 |
| 78 | 1930 | 20 | Default |  | A2 |  | 64 QAM | 6@76 |  | 3@38 | 1@19 |
| 79 | 1930 | 20 | Default |  | A8 |  | 64 QAM | 6@28 |  | 3@14 | 1@7 |
| 80 | 1922.5 | 5 | 15 |  | A3 |  | 256 QAM | Outer_Full |  |  |  |
| 81 | 1925 | 10 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 82 | 1925 | 10 | Default |  | A7 |  | 256 QAM | 30@11 |  | 15@6 | 6@3 |
| 83 | 1925 | 10 | Default |  | A2 |  | 256 QAM | 6@40 |  | 3@20 | 1@10 |
| 84 | 1927.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 85 | 1927.5 | 15 | Default |  | A7 |  | 256 QAM | 52@20 |  | 26@10 | 12@5 |
| 86 | 1927.5 | 15 | Default |  | A2 |  | 256 QAM | 6@56 |  | 3@28 | 1@14 |
| 87 | 1932.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 88 | 1932.5 | 15 | Default |  | A7 |  | 256 QAM | 62@11 |  | 30@6 | 14@3 |
| 89 | 1932.5 | 15 | Default |  | A2 |  | 256 QAM | 6@68 |  | 3@34 | 1@17 |
| 90 | 1930 | 20 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |
| 91 | 1930 | 20 | Default |  | A7 |  | 256 QAM | 72@28 |  | 36@14 | 16@7 |
| 92 | 1930 | 20 | Default |  | A2 |  | 256 QAM | 6@76 |  | 3@38 | 1@19 |
| 93 | 1922.5 | 5 | 15 |  | A3 |  | QPSK | Outer_Full |  |  |  |
| 94 | 1922.5 | 5 | Default |  | A4 |  | QPSK | 8@1 |  | 4@1 |  |
| 95 | 1925 | 10 | Default |  | A1 |  | QPSK | Outer_Full |  |  |  |
| 96 | 1925 | 10 | Default |  | A7 |  | QPSK | 30@11 |  | 15@6 | 6@3 |
| 97 | 1925 | 10 | Default |  | A2 |  | QPSK | 6@40 |  | 3@20 | 1@10 |
| 98 | 1925 | 10 | Default |  | A8 |  | QPSK | 6@11 |  | 3@6 | 1@3 |
| 99 | 1935 | 10 | Default |  | A4 |  | QPSK | Outer_Full |  |  |  |
| 100 | 1927.5 | 15 | Default |  | A1 |  | QPSK | Outer_Full |  |  |  |
| 101 | 1927.5 | 15 | Default |  | A7 |  | QPSK | 52@20 |  | 26@10 | 12@5 |
| 102 | 1927.5 | 15 | Default |  | A2 |  | QPSK | 6@56 |  | 3@28 | 1@14 |
| 103 | 1927.5 | 15 | Default |  | A8 |  | QPSK | 6@20 |  | 3@10 | 1@5 |
| 104 | 1932.5 | 15 | Default |  | A1 |  | QPSK | Outer_Full |  |  |  |
| 105 | 1932.5 | 15 | Default |  | A7 |  | QPSK | 62@11 |  | 30@6 | 14@3 |
| 106 | 1932.5 | 15 | Default |  | A2 |  | QPSK | 6@68 |  | 3@34 | 1@17 |
| 107 | 1942.5 | 15 | Default |  | A5 |  | QPSK | Outer_Full |  |  |  |

| A-MPR test parameters for NS_05 and NS_05U |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |  |  |  |  |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz |  | SCS 60 kHz |  |  |
| 108 | 1930 | 20 | Default |  | A1 | CP-OFDM | QPSK | Outer_Full |  |  |  |  |  |
| 109 | 1930 | 20 | Default |  | A7 |  | QPSK | 72@28 |  | 36@14 |  |  | 16@7 |
| 110 | 1930 | 20 | Default |  | A2 |  | QPSK | 6@76 |  | 3@38 |  | 1@19 |  |
| 111 | 1930 | 20 | Default |  | A8 |  | QPSK | 6@28 |  | 3@14 |  |  | 1@7 |
| 112 | 1950 | 20 | Default |  | A6 |  | QPSK | Outer_Full |  |  |  |  |  |
| 113 | 1922.5 | 5 | 15 |  | A3 |  | 16 QAM | Outer_Full |  |  |  |  |  |
| 114 | 1922.5 | 5 | Default |  | A4 |  | 16 QAM | 8@1 |  | 4@1 |  |  |  |
| 115 | 1925 | 10 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |  |  |
| 116 | 1925 | 10 | Default |  | A7 |  | 16 QAM | 30@11 |  | 15@6 |  | 6@3 |  |
| 117 | 1925 | 10 | Default |  | A2 |  | 16 QAM | 6@40 |  | 3@20 |  | 1@10 |  |
| 118 | 1925 | 10 | Default |  | A8 |  | 16 QAM | 6@11 |  | 3@6 |  | 1@3 |  |
| 119 | 1935 | 10 | Default |  | A4 |  | 16 QAM | Outer_Full |  |  |  |  |  |
| 120 | 1927.5 | 15 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |  |  |
| 121 | 1927.5 | 15 | Default |  | A7 |  | 16 QAM | 52@20 |  | 26@10 |  | 12@5 |  |
| 122 | 1927.5 | 15 | Default |  | A2 |  | 16 QAM | 6@56 |  | 3@28 |  | 1@14 |  |
| 123 | 1927.5 | 15 | Default |  | A8 |  | 16 QAM | 6@20 |  | 3@10 |  | 1@5 |  |
| 124 | 1932.5 | 15 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |  |  |
| 125 | 1932.5 | 15 | Default |  | A7 |  | 16 QAM | 62@11 |  | 30@6 |  | 14@3 |  |
| 126 | 1932.5 | 15 | Default |  | A2 |  | 16 QAM | 6@68 |  | 3@34 |  | 1@17 |  |
| 127 | 1942.5 | 15 | Default |  | A5 |  | 16 QAM | Outer_Full |  |  |  |  |  |
| 128 | 1930 | 20 | Default |  | A1 |  | 16 QAM | Outer_Full |  |  |  |  |  |
| 129 | 1930 | 20 | Default |  | A7 |  | 16 QAM | 72@28 |  | 36@14 |  | 16@7 |  |
| 130 | 1930 | 20 | Default |  | A2 |  | 16 QAM | 6@76 |  | 3@38 |  | 1@19 |  |
| 131 | 1930 | 20 | Default |  | A8 |  | 16 QAM | 6@28 |  | 3@14 |  | 1@7 |  |
| 132 | 1950 | 20 | Default |  | A6 |  | 16 QAM | Outer_Full |  |  |  |  |  |
| 133 | 1922.5 | 5 | 15 |  | A3 |  | 64 QAM | Outer_Full |  |  |  |  |  |
| 134 | 1922.5 | 5 | Default |  | A4 |  | 64 QAM | 8@1 |  | 4@1 |  |  |  |
| 135 | 1925 | 10 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |  |  |
| 136 | 1925 | 10 | Default |  | A7 |  | 64 QAM | 30@11 |  | 15@6 |  | 6@3 |  |
| 137 | 1925 | 10 | Default |  | A2 |  | 64 QAM | 6@40 |  | 3@20 |  | 1@10 |  |
| 138 | 1935 | 10 | Default |  | A4 |  | 64 QAM | Outer_Full |  |  |  |  |  |
| 139 | 1927.5 | 15 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |  |  |
| 140 | 1927.5 | 15 | Default |  | A7 |  | 64 QAM | 52@20 |  | 26@10 |  | 12@5 |  |
| 141 | 1927.5 | 15 | Default |  | A2 |  | 64 QAM | 6@56 |  | 3@28 |  | 1@14 |  |
| 142 | 1932.5 | 15 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |  |  |
| 143 | 1932.5 | 15 | Default |  | A7 |  | 64 QAM | 62@11 |  | 30@6 |  | 14@3 |  |
| 144 | 1932.5 | 15 | Default |  | A2 |  | 64 QAM | 6@68 |  | 3@34 |  | 1@17 |  |
| 145 | 1942.5 | 15 | Default |  | A5 |  | 64 QAM | Outer_Full |  |  |  |  |  |
| 146 | 1930 | 20 | Default |  | A1 |  | 64 QAM | Outer_Full |  |  |  |  |  |
| 147 | 1930 | 20 | Default |  | A7 |  | 64 QAM | 72@28 |  | 36@14 |  | 16@7 |  |
| 148 | 1930 | 20 | Default |  | A2 |  | 64 QAM | 6@76 |  | 3@38 |  | 1@19 |  |
| 149 | 1950 | 20 | Default |  | A6 |  | 64 QAM | Outer_Full |  |  |  |  |  |
| 150 | 1922.5 | 5 | 15 |  | A3 |  | 256 QAM | Outer_Full |  |  |  |  |  |
| 151 | 1925 | 10 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |  |  |
| 152 | 1925 | 10 | Default |  | A7 |  | 256 QAM | 30@11 |  | 15@6 |  | 6@3 |  |
| 153 | 1927.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |  |  |
| 154 | 1927.5 | 15 | Default |  | A7 |  | 256 QAM | 52@20 |  | 26@10 |  | 12@5 |  |
| 155 | 1932.5 | 15 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |  |  |
| 156 | 1932.5 | 15 | Default |  | A7 |  | 256 QAM | 62@11 |  | 30@6 |  | 14@3 |  |
| 157 | 1930 | 20 | Default |  | A1 |  | 256 QAM | Outer_Full |  |  |  |  |  |
| 158 | 1930 | 20 | Default |  | A7 |  | 256 QAM | 72@28 |  | 36@14 |  | 16@7 |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

Table 6.2.3.4.1-20: Test Configuration table for NS_48 (Power class 2)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies |  |  |  |  |  |  |  | lowest and highest carrier centre frequency for each CBW in table 6.2.3.3.26-1 |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 25 MHz, 30MHz, 40MHz, 45MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_48 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 10 | Default | N/A for A-MPR testing. | DFT-s-OFDM | QPSK | Outer_Full (A6) |  |  |  |
| 2 | Default | 10 | Default |  |  | QPSK | 8@0 (A6) |  | 4@0 (A6) | 2@0 (A6) |
| 3 | Default | 15 | Default |  |  | QPSK | Outer_Full (A6) |  |  |  |
| 4 | Default | 15 | Default |  |  | QPSK | 48@0 (A6) |  | 24@0 (A6) | 12@0 (A6) |
| 5 | Default | 20 | Default |  |  | QPSK | Outer_Full (A4) |  |  |  |
| 6 | Default | 20 | Default |  |  | QPSK | 52@0 (A6) |  | 26@0 (A6) | 13@0 (A6) |
| 7 | Default | 25 | Default |  |  | QPSK | Outer_Full (A4) |  |  |  |
| 8 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 9 | Default | 25 | Default |  |  | QPSK | 52@0 (A4) |  | 26@0 (A4) | 13@0 (A4) |
| 10 | Default | 30 | Default |  |  | QPSK | Outer_Full (A4) |  |  |  |
| 11 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |  |
| 12 | Default | 30 | Default |  |  | QPSK | 70@0 (A4) |  | 34@0 (A4) | 17@0 (A4) |
| 13 | Default | 40 | Default |  |  | QPSK | 16@0 (A1) |  | 8@0 (A1) | 4@0 (A1) |
| 14 | Default | 40 | Default |  |  | QPSK | 90@0 (A3) |  | 48@0 (A3) | 24@0 (A3) |
| 15 | Default | 40 | Default |  |  | QPSK | 150@0 (A4) |  | 75@0 (A4) | 36@0 (A4) |
| 16 | Default | 40 | Default |  |  | QPSK | 192@0 (A2) |  | 96@0 (A2) | 48@0 (A2) |
| 17 | Default | 40 | Default |  |  | QPSK | 5@187 (A3) |  | 2@94 (A3) | 1@47 (A3) |
| 18 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 19 | Default | 45 | Default |  |  | QPSK | 28@0 (A1) |  | 14@0 (A1) | 7@0 (A1) |
| 20 | Default | 45 | Default |  |  | QPSK | 100@0 (A4) |  | 50@0 (A4) | 27@0 (A4) |
| 21 | Default | 45 | Default |  |  | QPSK | 200@0 (A2) |  | 100@0 (A2) | 50@0 (A2) |
| 22 | Default | 45 | Default |  |  | QPSK | 5@203 (A5) |  | 2@102 (A5) | 1@51 (A5) |
| 23 | Default | 45 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 24 | Default | 50 | Default |  |  | QPSK | 36@0 (A1) |  | 18@0 (A1) | 9@0 (A1) |
| 25 | Default | 50 | Default |  |  | QPSK | 108@0 (A4) |  | 54@0 (A4) | 27@0 (A4) |
| 26 | Default | 50 | Default |  |  | QPSK | 225@0 (A2) |  | 108@0 (A2) | 54@0 (A2) |
| 27 | Default | 50 | Default |  |  | QPSK | 5@223 (A5) |  | 2@112 (A5) | 1@56 (A5) |
| 28 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 29 | Default | 10 | Default |  |  | 256QAM | Outer_Full (A6) |  |  |  |
| 30 | Default | 10 | Default |  |  | 256QAM | 8@0 (A6) |  | 8@0 (A6) | 8@0 (A6) |
| 31 | Default | 15 | Default |  |  | 256QAM | Outer_Full (A6) |  |  |  |
| 32 | Default | 15 | Default |  |  | 256QAM | 48@0 (A6) |  | 48@0 (A6) | 48@0 (A6) |
| 33 | Default | 20 | Default |  |  | 256QAM | Outer_Full (A4) |  |  |  |
| 34 | Default | 20 | Default |  |  | 256QAM | 52@0 (A6) |  | 52@0 (A6) | 52@0 (A6) |
| 35 | Default | 25 | Default |  |  | 256QAM | Outer_Full (A4) |  |  |  |
| 36 | Default | 25 | Default |  |  | 256QAM | Edge_1RB_Right (A3) |  |  |  |
| 37 | Default | 25 | Default |  |  | 256QAM | 52@0 (A4) |  | 52@0 (A4) | 52@0 (A4) |
| 38 | Default | 30 | Default |  |  | 256QAM | Outer_Full (A4) |  |  |  |
| 39 | Default | 30 | Default |  |  | 256QAM | Edge_1RB_Right (A5) |  |  |  |
| 40 | Default | 30 | Default |  |  | 256QAM | 70@0 (A4) |  | 70@0 (A4) | 70@0 (A4) |
| 41 | Default | 40 | Default |  |  | 256QAM | 16@0 (A1) |  | 16@0 (A1) | 16@0 (A1) |
| 42 | Default | 40 | Default |  |  | 256QAM | 90@0 (A3) |  | 90@0 (A3) | 90@0 (A3) |
| 43 | Default | 40 | Default |  |  | 256QAM | 150@0 (A4) |  | 150@0 (A4) | 150@0 (A4) |
| 44 | Default | 40 | Default |  |  | 256QAM | 192@0 (A2) |  | 192@0 (A2) | 192@0 (A2) |
| 45 | Default | 40 | Default |  |  | 256QAM | 5@187 (A3) |  | 5@187 (A3) | 5@187 (A3) |
| 46 | Default | 40 | Default |  |  | 256QAM | Outer_Full (A1) |  |  |  |
| 47 | Default | 45 | Default |  |  | 256QAM | 28@0 (A1) |  | 28@0 (A1) | 28@0 (A1) |
| 48 | Default | 45 | Default |  |  | 256QAM | 100@0 (A4) |  | 100@0 (A4) | 100@0 (A4) |
| 49 | Default | 45 | Default |  |  | 256QAM | 200@0 (A2) |  | 200@0 (A2) | 200@0 (A2) |
| 50 | Default | 45 | Default |  |  | 256QAM | 5@203 (A5) |  | 5@203 (A5) | 5@203 (A5) |
| 51 | Default | 45 | Default |  |  | 256QAM | Outer_Full (A1) |  |  |  |
| 52 | Default | 50 | Default |  |  | 256QAM | 36@0 (A1) |  | 36@0 (A1) | 36@0 (A1) |
| 53 | Default | 50 | Default |  |  | 256QAM | 108@0 (A4) |  | 108@0 (A4) | 108@0 (A4) |
| 54 | Default | 50 | Default |  |  | 256QAM | 225@0 (A2) |  | 225@0 (A2) | 225@0 (A2) |
| 55 | Default | 50 | Default |  |  | 256QAM | 5@223 (A5) |  | 5@223 (A5) | 5@223 (A5) |
| 56 | Default | 50 | Default |  |  | 256QAM | Outer_Full (A1) |  |  |  |
| 57 | Default | 10 | Default |  | CP-OFDM | QPSK | Outer_Full (A6) |  |  |  |
| 58 | Default | 10 | Default |  |  | QPSK | 8@0 (A6) |  | 8@0 (A6) | 8@0 (A6) |
| 59 | Default | 15 | Default |  |  | QPSK | Outer_Full (A6) |  |  |  |
| 60 | Default | 15 | Default |  |  | QPSK | 48@0 (A6) |  | 48@0 (A6) | 48@0 (A6) |
| 61 | Default | 20 | Default |  |  | QPSK | Outer_Full (A4) |  |  |  |
| 62 | Default | 20 | Default |  |  | QPSK | 52@0 (A6) |  | 52@0 (A6) | 52@0 (A6) |
| 63 | Default | 25 | Default |  |  | QPSK | Outer_Full (A4) |  |  |  |
| 64 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 65 | Default | 25 | Default |  |  | QPSK | 52@0 (A4) |  | 52@0 (A4) | 52@0 (A4) |
| 66 | Default | 30 | Default |  |  | QPSK | Outer_Full (A4) |  |  |  |
| 67 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A5) |  |  |  |
| 68 | Default | 30 | Default |  |  | QPSK | 70@0 (A4) |  | 70@0 (A4) | 70@0 (A4) |
| 69 | Default | 40 | Default |  |  | QPSK | 16@0 (A1) |  | 16@0 (A1) | 16@0 (A1) |
| 70 | Default | 40 | Default |  |  | QPSK | 90@0 (A3) |  | 90@0 (A3) | 90@0 (A3) |
| 71 | Default | 40 | Default |  |  | QPSK | 150@0 (A4) |  | 150@0 (A4) | 150@0 (A4) |
| 72 | Default | 40 | Default |  |  | QPSK | 192@0 (A2) |  | 192@0 (A2) | 192@0 (A2) |
| 73 | Default | 40 | Default |  |  | QPSK | 5@187 (A3) |  | 5@187 (A3) | 5@187 (A3) |
| 74 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 75 | Default | 45 | Default |  |  | QPSK | 28@0 (A1) |  | 28@0 (A1) | 28@0 (A1) |
| 76 | Default | 45 | Default |  |  | QPSK | 100@0 (A4) |  | 100@0 (A4) | 100@0 (A4) |
| 77 | Default | 45 | Default |  |  | QPSK | 200@0 (A2) |  | 200@0 (A2) | 200@0 (A2) |
| 78 | Default | 45 | Default |  |  | QPSK | 5@203 (A5) |  | 5@203 (A5) | 5@203 (A5) |
| 79 | Default | 45 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 80 | Default | 50 | Default |  |  | QPSK | 36@0 (A1) |  | 36@0 (A1) | 36@0 (A1) |
| 81 | Default | 50 | Default |  |  | QPSK | 108@0 (A4) |  | 108@0 (A4) | 108@0 (A4) |
| 82 | Default | 50 | Default |  |  | QPSK | 225@0 (A2) |  | 225@0 (A2) | 225@0 (A2) |
| 83 | Default | 50 | Default |  |  | QPSK | 5@223 (A5) |  | 5@223 (A5) | 5@223 (A5) |
| 84 | Default | 50 | Default |  |  | QPSK | Outer_Full (A6) |  |  |  |
| 85 | Default | 10 | Default |  |  | 256QAM | Outer_Full (A6) |  |  |  |
| 86 | Default | 10 | Default |  |  | 256QAM | 8@0 (A6) |  | 8@0 (A6) | 8@0 (A6) |
| 87 | Default | 15 | Default |  |  | 256QAM | Outer_Full (A6) |  |  |  |
| 88 | Default | 15 | Default |  |  | 256QAM | 48@0 (A6) |  | 48@0 (A6) | 48@0 (A6) |
| 89 | Default | 20 | Default |  |  | 256QAM | Outer_Full (A4) |  |  |  |
| 90 | Default | 20 | Default |  |  | 256QAM | 52@0 (A6) |  | 52@0 (A6) | 52@0 (A6) |
| 91 | Default | 25 | Default |  |  | 256QAM | Outer_Full (A4) |  |  |  |
| 92 | Default | 25 | Default |  |  | 256QAM | Edge_1RB_Right (A3) |  |  |  |
| 93 | Default | 25 | Default |  |  | 256QAM | 52@0 (A4) |  | 52@0 (A4) | 52@0 (A4) |
| 94 | Default | 30 | Default |  |  | 256QAM | Outer_Full (A4) |  |  |  |
| 95 | Default | 30 | Default |  |  | 256QAM | Edge_1RB_Right (A5) |  |  |  |
| 96 | Default | 30 | Default |  |  | 256QAM | 70@0 (A4) |  | 70@0 (A4) | 70@0 (A4) |
| 97 | Default | 40 | Default |  |  | 256QAM | 16@0 (A1) |  | 16@0 (A1) | 16@0 (A1) |
| 98 | Default | 40 | Default |  |  | 256QAM | 90@0 (A3) |  | 90@0 (A3) | 90@0 (A3) |
| 99 | Default | 40 | Default |  |  | 256QAM | 150@0 (A4) |  | 150@0 (A4) | 150@0 (A4) |
| 100 | Default | 40 | Default |  |  | 256QAM | 192@0 (A2) |  | 192@0 (A2) | 192@0 (A2) |
| 101 | Default | 40 | Default |  |  | 256QAM | 5@187 (A3) |  | 5@187 (A3) | 5@187 (A3) |
| 102 | Default | 40 | Default |  |  | 256QAM | Outer_Full (A1) |  |  |  |
| 103 | Default | 45 | Default |  |  | 256QAM | 28@0 (A1) |  | 28@0 (A1) | 28@0 (A1) |
| 104 | Default | 45 | Default |  |  | 256QAM | 100@0 (A4) |  | 100@0 (A4) | 100@0 (A4) |
| 105 | Default | 45 | Default |  |  | 256QAM | 200@0 (A2) |  | 200@0 (A2) | 200@0 (A2) |
| 106 | Default | 45 | Default |  |  | 256QAM | 5@203 (A5) |  | 5@203 (A5) | 5@203 (A5) |
| 107 | Default | 45 | Default |  |  | 256QAM | Outer_Full (A1) |  |  |  |
| 108 | Default | 50 | Default |  |  | 256QAM | 36@0 (A1) |  | 36@0 (A1) | 36@0 (A1) |
| 109 | Default | 50 | Default |  |  | 256QAM | 108@0 (A4) |  | 108@0 (A4) | 108@0 (A4) |
| 110 | Default | 50 | Default |  |  | 256QAM | 225@0 (A2) |  | 225@0 (A2) | 225@0 (A2) |
| 111 | Default | 50 | Default |  |  | 256QAM | 5@223 (A5) |  | 5@223 (A5) | 5@223 (A5) |
| 112 | Default | 50 | Default |  |  | 256QAM | Outer_Full (A6) |  |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

Table 6.2.3.4.1-29: Test Configuration table for NS_49(power class 3)

<Unchanged Text Skipped>

Table 6.2.3.4.1-29a: Test Configuration table for NS_49(power class 2)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Low range, High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 25 MHz, 30MHz, 40MHz, 50MHz |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz | SCS 60 kHz |
| 1 | Default | 10 | Default | N/A for A-MPR testing. | DFT-s-OFDM | QPSK | Outer_Full (A3) |  |  |  |
| 2 | Default | 10 | Default |  |  | QPSK | Edge_1RB_Left (A3) |  |  |  |
| 3 | Default | 15 | Default |  |  | QPSK | Outer_Full (A3) |  |  |  |
| 4 | Default | 15 | Default |  |  | QPSK | 48@0 (A3) |  | 24@0 (A3) | 12@0 (A3) |
| 5 | Default | 20 | Default |  |  | QPSK | Outer_Full (A4) |  |  |  |
| 6 | Default | 20 | Default |  |  | QPSK | 52@0 (A3) |  | 26@0 (A3) | 13@0 (A3) |
| 7 | Default | 25 | Default |  |  | QPSK | Outer_Full (A4) |  |  |  |
| 8 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |  |
| 9 | Default | 25 | Default |  |  | QPSK | 18@18 (A3) |  | 9@9 (A3) | 4@5 (A3) |
| 10 | Default | 30 | Default |  |  | QPSK | 20@0 (A1) |  | 10@0 (A1) | 5@0 (A1) |
| 11 | Default | 30 | Default |  |  | QPSK | 44@0 (A5) |  | 22@0 (A5) | 11@0 (A5) |
| 12 | Default | 30 | Default |  |  | QPSK | 80@0 (A3) |  | 40@0 (A3) | 20@0 (A3) |
| 13 | Default | 30 | Default |  |  | QPSK | 120@0 (A4) |  | 60@0 (A4) | 30@0 (A4) |
| 14 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |  |
| 15 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A1) |  |  |  |
| 16 | Default | 40 | Default |  |  | QPSK | 50@0 (A1) |  | 25@0 (A1) | 12@0 (A1) |
| 17 | Default | 40 | Default |  |  | QPSK | 16@48 (A5) |  | 8@24 (A5) | 4@12 (A5) |
| 18 | Default | 40 | Default |  |  | QPSK | 100@0 (A4) |  | 50@0 (A4) | 25@0 (A4) |
| 19 | Default | 40 | Default |  |  | QPSK | 144@52 (A2) |  | 72@26(A2) | 36@12 (A2) |
| 20 | Default | 40 | Default |  |  | QPSK | 5@187 (A5) |  | 2@94 (A5) | 1@47 (A5) |
| 21 | Default | 40 | Default |  |  | QPSK | 192@0 (A1) |  | 96@0 (A1) | 48@0 (A1) |
| 22 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 23 | Default | 50 | Default |  |  | QPSK | 5@75 (A5) |  | 2@38 (A5) | 1@19 (A5) |
| 24 | Default | 50 | Default |  |  | QPSK | 5@215 (A5) |  | 2@108 (A5) | 1@54 (A5) |
| 25 | Default | 50 | Default |  |  | QPSK | 150@45 (A2) |  | 81@23 (A2) | 40@12 (A2) |
| 26 | Default | 50 | Default |  |  | QPSK | 216@0 (A1) |  | 108@0 (A1) | 54@0 (A1) |
| 27 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |  |
| 28 | Default | 10 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 29 | Default | 10 | Default |  |  | 256 QAM | Edge_1RB_Left (A3) |  |  |  |
| 30 | Default | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |  |
| 31 | Default | 15 | Default |  |  | 256 QAM | 48@0 (A3) |  | 24@0 (A3) | 12@0 (A3) |
| 32 | Default | 20 | Default |  |  | 256 QAM | Outer_Full (A4) |  |  |  |
| 33 | Default | 20 | Default |  |  | 256 QAM | 52@0 (A3) |  | 26@0 (A3) | 13@0 (A3) |
| 34 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A4) |  |  |  |
| 35 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |  |
| 36 | Default | 25 | Default |  |  | 256 QAM | 18@18 (A3) |  | 9@9 (A3) | 4@5 (A3) |
| 37 | Default | 30 | Default |  |  | 256 QAM | 20@0 (A1) |  | 10@0 (A1) | 5@0 (A1) |
| 38 | Default | 30 | Default |  |  | 256 QAM | 44@0 (A5) |  | 22@0 (A5) | 11@0 (A5) |
| 39 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A3) |  | 40@0 (A3) | 20@0 (A3) |
| 40 | Default | 30 | Default |  |  | 256 QAM | 120@0 (A4) |  | 60@0 (A4) | 30@0 (A4) |
| 41 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |  |
| 42 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A1) |  |  |  |
| 43 | Default | 40 | Default |  |  | 256 QAM | 50@0 (A1) |  | 25@0 (A1) | 12@0 (A1) |
| 44 | Default | 40 | Default |  |  | 256 QAM | 16@48 (A5) |  | 8@24 (A5) | 4@12 (A5) |
| 45 | Default | 40 | Default |  |  | 256 QAM | 100@0 (A4) |  | 50@0 (A4) | 25@0 (A4) |
| 46 | Default | 40 | Default |  |  | 256 QAM | 144@52 (A2) |  | 72@26(A2) | 36@12 (A2) |
| 47 | Default | 40 | Default |  |  | 256 QAM | 5@187 (A5) |  | 2@94 (A5) | 1@47 (A5) |
| 48 | Default | 40 | Default |  |  | 256 QAM | 192@0 (A1) |  | 96@0 (A1) | 48@0 (A1) |
| 49 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |  |
| 50 | Default | 50 | Default |  |  | 256 QAM | 5@75 (A5) |  | 2@38 (A5) | 1@19 (A5) |
| 51 | Default | 50 | Default |  |  | 256 QAM | 5@215 (A5) |  | 2@108 (A5) | 1@54 (A5) |
| 52 | Default | 50 | Default |  |  | 256 QAM | 150@45 (A2) |  | 81@23 (A2) | 40@12 (A2) |
| 53 | Default | 50 | Default |  |  | 256 QAM | 216@0 (A1) |  | 108@0 (A1) | 54@0 (A1) |

| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 54 | Default | 50 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 55 | Default | 10 | Default |  | CP-OFDM | QPSK | Outer_Full (A3) |  |  |
| 56 | Default | 10 | Default |  |  | QPSK | Edge_1RB_Left (A3) |  |  |
| 57 | Default | 15 | Default |  |  | QPSK | Outer_Full (A3) |  |  |
| 58 | Default | 15 | Default |  |  | QPSK | 48@0 (A3) | 24@0 (A3) | 12@0 (A3) |
| 59 | Default | 20 | Default |  |  | QPSK | Outer_Full (A4) |  |  |
| 60 | Default | 20 | Default |  |  | QPSK | 52@0 (A3) | 26@0 (A3) | 13@0 (A3) |
| 61 | Default | 25 | Default |  |  | QPSK | Outer_Full (A4) |  |  |
| 62 | Default | 25 | Default |  |  | QPSK | Edge_1RB_Right (A3) |  |  |
| 63 | Default | 25 | Default |  |  | QPSK | 18@18 (A3) | 9@9 (A3) | 4@5 (A3) |
| 64 | Default | 30 | Default |  |  | QPSK | 20@0 (A1) | 10@0 (A1) | 5@0 (A1) |
| 65 | Default | 30 | Default |  |  | QPSK | 44@0 (A5) | 22@0 (A5) | 11@0 (A5) |
| 66 | Default | 30 | Default |  |  | QPSK | 80@0 (A3) | 40@0 (A3) | 20@0 (A3) |
| 67 | Default | 30 | Default |  |  | QPSK | 120@0 (A4) | 60@0 (A4) | 30@0 (A4) |
| 68 | Default | 30 | Default |  |  | QPSK | Outer_Full (A2) |  |  |
| 69 | Default | 30 | Default |  |  | QPSK | Edge_1RB_Right (A1) |  |  |
| 70 | Default | 40 | Default |  |  | QPSK | 50@0 (A1) | 25@0 (A1) | 12@0 (A1) |
| 71 | Default | 40 | Default |  |  | QPSK | 16@48 (A5) | 8@24 (A5) | 4@12 (A5) |
| 72 | Default | 40 | Default |  |  | QPSK | 100@0 (A4) | 50@0 (A4) | 25@0 (A4) |
| 73 | Default | 40 | Default |  |  | QPSK | 144@52 (A2) | 72@26(A2) | 36@12 (A2) |
| 74 | Default | 40 | Default |  |  | QPSK | 5@187 (A5) | 2@94 (A5) | 1@47 (A5) |
| 75 | Default | 40 | Default |  |  | QPSK | 192@0 (A1) | 96@0 (A1) | 48@0 (A1) |
| 76 | Default | 40 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 77 | Default | 50 | Default |  |  | QPSK | 5@75 (A5) | 2@38 (A5) | 1@19 (A5) |
| 78 | Default | 50 | Default |  |  | QPSK | 5@215 (A5) | 2@108 (A5) | 1@54 (A5) |
| 79 | Default | 50 | Default |  |  | QPSK | 150@45 (A2) | 81@23 (A2) | 40@12 (A2) |
| 80 | Default | 50 | Default |  |  | QPSK | 216@0 (A1) | 108@0 (A1) | 54@0 (A1) |
| 81 | Default | 50 | Default |  |  | QPSK | Outer_Full (A1) |  |  |
| 82 | Default | 10 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 83 | Default | 10 | Default |  |  | 256 QAM | Edge_1RB_Left (A3) |  |  |
| 84 | Default | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 85 | Default | 15 | Default |  |  | 256 QAM | 48@0 (A3) | 24@0 (A3) | 12@0 (A3) |
| 86 | Default | 20 | Default |  |  | 256 QAM | Outer_Full (A4) |  |  |
| 87 | Default | 20 | Default |  |  | 256 QAM | 52@0 (A3) | 26@0 (A3) | 13@0 (A3) |
| 88 | Default | 25 | Default |  |  | 256 QAM | Outer_Full (A4) |  |  |
| 89 | Default | 25 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |
| 90 | Default | 25 | Default |  |  | 256 QAM | 18@18 (A3) | 9@9 (A3) | 4@5 (A3) |
| 91 | Default | 30 | Default |  |  | 256 QAM | 20@0 (A1) | 10@0 (A1) | 5@0 (A1) |
| 92 | Default | 30 | Default |  |  | 256 QAM | 44@0 (A5) | 22@0 (A5) | 11@0 (A5) |
| 93 | Default | 30 | Default |  |  | 256 QAM | 80@0 (A3) | 40@0 (A3) | 20@0 (A3) |
| 94 | Default | 30 | Default |  |  | 256 QAM | 120@0 (A4) | 60@0 (A4) | 30@0 (A4) |
| 95 | Default | 30 | Default |  |  | 256 QAM | Outer_Full (A2) |  |  |
| 96 | Default | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A1) |  |  |
| 97 | Default | 40 | Default |  |  | 256 QAM | 50@0 (A1) | 25@0 (A1) | 12@0 (A1) |
| 98 | Default | 40 | Default |  |  | 256 QAM | 16@48 (A5) | 8@24 (A5) | 4@12 (A5) |
| 99 | Default | 40 | Default |  |  | 256 QAM | 100@0 (A4) | 50@0 (A4) | 25@0 (A4) |
| 100 | Default | 40 | Default |  |  | 256 QAM | 144@52 (A2) | 72@26(A2) | 36@12 (A2) |
| 101 | Default | 40 | Default |  |  | 256 QAM | 5@187 (A5) | 2@94 (A5) | 1@47 (A5) |
| 102 | Default | 40 | Default |  |  | 256 QAM | 192@0 (A1) | 96@0 (A1) | 48@0 (A1) |
| 103 | Default | 40 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 104 | Default | 50 | Default |  |  | 256 QAM | 5@75 (A5) | 2@38 (A5) | 1@19 (A5) |
| 105 | Default | 50 | Default |  |  | 256 QAM | 5@215 (A5) | 2@108 (A5) | 1@54 (A5) |
| 106 | Default | 50 | Default |  |  | 256 QAM | 150@45 (A2) | 81@23 (A2) | 40@12 (A2) |
| 107 | Default | 50 | Default |  |  | 256 QAM | 216@0 (A1) | 108@0 (A1) | 54@0 (A1) |
| 108 | Default | 50 | Default |  |  | 256 QAM | Outer_Full (A1) |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.2.3.5	Test requirement

The maximum output power derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2.3.5-1. The allowed A-MPR values specified in table 6.2.3.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.2. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2.1.3-1 apply.

Table 6.2.3.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

<Unchanged Text Skipped>

Table 6.2.3.5-6a: UE Power Class 2 test requirements (NS_05 and NS_05U) for bands n1, n65

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 0.5 | 6.5 | 0 | 19.5 | 3.5 | 3 | 28+TT | 16-TT |
| 2 | 26 | 0 | 0 | 3 | 0 | 23 | 2 | 3 | 28+TT | 20-TT |
| 3 | 26 | 0 | 0.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 4 | 26 | 0 | 0.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 5 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 6 | 26 | 0 | 0 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 7 | 26 | 0 | 0.5 | 3 | 0 | 23 | 2 | 3 | 28+TT | 20-TT |
| 8 | 26 | 0 | 0.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 9 | 26 | 0 | 0.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 10 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 11 | 26 | 0 | 0 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 12 | 26 | 0 | 0.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 13 | 26 | 0 | 0.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 14 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 15 | 26 | 0 | 0.5 | 2 | 0 | 24 | 2 | 3 | 28+TT | 21-TT |
| 16 | 26 | 0 | 0.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 17 | 26 | 0 | 0.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 18 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 19 | 26 | 0 | 0 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 20 | 26 | 0 | 0.5 | 2 | 0 | 24 | 2 | 3 | 28+TT | 21-TT |
| 21 | 26 | 0 | 1 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 22 | 26 | 0 | 0 | 3 | 0 | 23 | 2 | 3 | 28+TT | 20-TT |
| 23 | 26 | 0 | 1 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 24 | 26 | 0 | 1 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 25 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 26 | 26 | 0 | 0 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 27 | 26 | 0 | 1 | 3 | 0 | 23 | 2 | 3 | 28+TT | 20-TT |
| 28 | 26 | 0 | 1 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 29 | 26 | 0 | 1 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 30 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 31 | 26 | 0 | 0 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 32 | 26 | 0 | 1 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 33 | 26 | 0 | 1 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 34 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 35 | 26 | 0 | 1 | 2 | 0 | 24 | 2 | 3 | 28+TT | 21-TT |
| 36 | 26 | 0 | 1 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 37 | 26 | 0 | 1 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 38 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 39 | 26 | 0 | 0 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 40 | 26 | 0 | 1 | 2 | 0 | 24 | 2 | 3 | 28+TT | 21-TT |
| 41 | 26 | 0 | 2 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 42 | 26 | 0 | 1 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 43 | 26 | 0 | 2 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 44 | 26 | 0 | 2 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 45 | 26 | 0 | 1 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 46 | 26 | 0 | 1 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 47 | 26 | 0 | 2 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 48 | 26 | 0 | 2 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 49 | 26 | 0 | 2 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 50 | 26 | 0 | 1 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 51 | 26 | 0 | 1 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 52 | 26 | 0 | 2 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 53 | 26 | 0 | 2 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 54 | 26 | 0 | 1 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 55 | 26 | 0 | 2 | 2.5 | 0 | 23.5 | 2 | 3 | 28+TT | 20.5-TT |
| 56 | 26 | 0 | 2 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 57 | 26 | 0 | 2 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 58 | 26 | 0 | 1 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 59 | 26 | 0 | 1 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 26 | 0 | 2 | 2 | 0 | 24 | 2 | 3 | 28+TT | 21-TT |
| 61 | 26 | 0 | 2.5 | 9 | 0 | 17 | 5 | 3 | 28+TT | 12-TT |
| 62 | 26 | 0 | 2.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 63 | 26 | 0 | 2.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 64 | 26 | 0 | 2.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 65 | 26 | 0 | 2.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 66 | 26 | 0 | 2.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 67 | 26 | 0 | 2.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 68 | 26 | 0 | 2.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 69 | 26 | 0 | 2.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 70 | 26 | 0 | 2.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 71 | 26 | 0 | 2.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 72 | 26 | 0 | 2.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 73 | 26 | 0 | 2.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 74 | 26 | 0 | 2.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 75 | 26 | 0 | 2.5 | 2.5 | 0 | 23.5 | 2 | 3 | 28+TT | 20.5-TT |
| 76 | 26 | 0 | 2.5 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 77 | 26 | 0 | 2.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 78 | 26 | 0 | 2.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 79 | 26 | 0 | 2.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 80 | 26 | 0 | 4.5 | 9.5 | 0 | 16.5 | 5 | 3 | 28+TT | 11.5-TT |
| 81 | 26 | 0 | 4.5 | 15 | 0 | 11 | 6 | 3 | 28+TT | 5-TT |
| 82 | 26 | 0 | 4.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 83 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 84 | 26 | 0 | 4.5 | 15 | 0 | 11 | 6 | 3 | 28+TT | 5-TT |
| 85 | 26 | 0 | 4.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 86 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 87 | 26 | 0 | 4.5 | 15 | 0 | 11 | 6 | 3 | 28+TT | 5-TT |
| 88 | 26 | 0 | 4.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 89 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 90 | 26 | 0 | 4.5 | 15 | 0 | 11 | 6 | 3 | 28+TT | 5-TT |
| 91 | 26 | 0 | 4.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 92 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 93 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 94 | 26 | 0 | 1.5 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 95 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 96 | 26 | 0 | 3 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 97 | 26 | 0 | 1.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 98 | 26 | 0 | 1.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 99 | 26 | 0 | 3 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 100 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 101 | 26 | 0 | 3 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 102 | 26 | 0 | 1.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 103 | 26 | 0 | 1.5 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 104 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 105 | 26 | 0 | 3 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 106 | 26 | 0 | 1.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 107 | 26 | 0 | 3 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 108 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 109 | 26 | 0 | 3 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 110 | 26 | 0 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 111 | 26 | 0 | 3 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 112 | 26 | 0 | 3 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 113 | 26 | 0 | 3 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 114 | 26 | 0 | 2 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 115 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 116 | 26 | 0 | 3 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 117 | 26 | 0 | 2 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 118 | 26 | 0 | 2 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 119 | 26 | 0 | 3 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 120 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 121 | 26 | 0 | 3 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 122 | 26 | 0 | 2 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| Test ID |  | PPowerClass (dBm) |  | PPowerClass (dB) |  | MPR (dB) |  | A-MPR (dB) |  | ΔTC,c (dB) |  | PCMAX_L,c (dBm) |  | T(PCMAX_L,c) (dB) |  | TL,c (dB) |  | Upper limit (dBm) |  | Lower limit (dBm) |  |
| 123 | 26 | 0 | 2 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 124 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 125 | 26 | 0 | 3 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 126 | 26 | 0 | 2 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 127 | 26 | 0 | 3 | 4.5 | 0 | 21.5 | 2 | 3 | 28+TT | 18.5-TT |
| 128 | 26 | 0 | 3 | 13 | 0 | 13 | 5 | 3 | 28+TT | 8-TT |
| 129 | 26 | 0 | 3 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 130 | 26 | 0 | 2 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 131 | 26 | 0 | 2 | 3.5 | 0 | 22.5 | 2 | 3 | 28+TT | 19.5-TT |
| 132 | 26 | 0 | 3 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 133 | 26 | 0 | 3.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 134 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 135 | 26 | 0 | 3.5 | 14 | 0 | 12 | 6 | 3 | 28+TT | 6-TT |
| 136 | 26 | 0 | 3.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 137 | 26 | 0 | 3.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 138 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 139 | 26 | 0 | 3.5 | 14 | 0 | 12 | 6 | 3 | 28+TT | 6-TT |
| 140 | 26 | 0 | 3.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 141 | 26 | 0 | 3.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 142 | 26 | 0 | 3.5 | 14 | 0 | 12 | 6 | 3 | 28+TT | 6-TT |
| 143 | 26 | 0 | 3.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 144 | 26 | 0 | 3.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 145 | 26 | 0 | 3.5 | 5 | 0 | 21 | 2 | 3 | 28+TT | 18-TT |
| 146 | 26 | 0 | 3.5 | 14 | 0 | 12 | 6 | 3 | 28+TT | 6-TT |
| 147 | 26 | 0 | 3.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 148 | 26 | 0 | 3.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 149 | 26 | 0 | 3.5 | 4 | 0 | 22 | 2 | 3 | 28+TT | 19-TT |
| 150 | 26 | 0 | 6.5 | 10 | 0 | 16 | 5 | 3 | 28+TT | 11-TT |
| 151 | 26 | 0 | 6.5 | 16 | 0 | 10 | 6 | 3 | 28+TT | 4-TT |
| 152 | 26 | 0 | 6.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 153 | 26 | 0 | 6.5 | 16 | 0 | 10 | 6 | 3 | 28+TT | 4-TT |
| 154 | 26 | 0 | 6.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 155 | 26 | 0 | 6.5 | 16 | 0 | 10 | 6 | 3 | 28+TT | 4-TT |
| 156 | 26 | 0 | 6.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| 157 | 26 | 0 | 6.5 | 16 | 0 | 10 | 6 | 3 | 28+TT | 4-TT |
| 158 | 26 | 0 | 6.5 | 8.5 | 0 | 17.5 | 5 | 3 | 28+TT | 12.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

Table 6.2.3.5-24a: UE Power Class 2 test requirements for NS_48 (PC2 contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 26 |  | 0 |  | 1 |  | 3 |  | 0 |  | 23 |  | 2 |  | 3 |  | 28+TT |  | 20-TT |  |
| 2 |  | 26 |  | 0 |  | 1 |  | 3 |  | 0 |  | 23 |  | 2 |  | 3 |  | 28+TT |  | 20-TT |  |
| 3 |  | 26 |  | 0 |  | 1 |  | 3 |  | 0 |  | 23 |  | 2 |  | 3 |  | 28+TT |  | 20-TT |  |
| 4 |  | 26 |  | 0 |  | 1 |  | 3 |  | 0 |  | 23 |  | 2 |  | 3 |  | 28+TT |  | 20-TT |  |
| 5 |  | 26 |  | 0 |  | 1 |  | 4.5 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 6 |  | 26 |  | 0 |  | 1 |  | 3 |  | 0 |  | 23 |  | 2 |  | 3 |  | 28+TT |  | 20-TT |  |
| 7 |  | 26 |  | 0 |  | 1 |  | 4.5 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 8 |  | 26 |  | 0 |  | 3.5 |  | 4 |  | 0 |  | 22 |  | 2 |  | 3 |  | 28+TT |  | 19-TT |  |
| 9 |  | 26 |  | 0 |  | 1 |  | 4.5 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 10 |  | 26 |  | 0 |  | 1 |  | 4.5 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 11 |  | 26 |  | 0 |  | 3.5 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 12 |  | 26 |  | 0 |  | 1 |  | 4.5 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 13 |  | 26 |  | 0 |  | 1 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 14 |  | 26 |  | 0 |  | 1 |  | 4 |  | 0 |  | 22 |  | 2 |  | 3 |  | 28+TT |  | 19-TT |  |
| 15 |  | 26 |  | 0 |  | 1 |  | 4.5 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 16 |  | 26 |  | 0 |  | 1 |  | 8 |  | 0 |  | 18 |  | 4 |  | 3 |  | 28+TT |  | 14-TT |  |
| 17 |  | 26 |  | 0 |  | 0 |  | 4 |  | 0 |  | 22 |  | 2 |  | 3 |  | 28+TT |  | 19-TT |  |
| 18 |  | 26 |  | 0 |  | 1 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 19 |  | 26 |  | 0 |  | 1 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 20 |  | 26 |  | 0 |  | 1 |  | 4.5 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 21 |  | 26 |  | 0 |  | 1 |  | 8 |  | 0 |  | 18 |  | 4 |  | 3 |  | 28+TT |  | 14-TT |  |
| 22 |  | 26 |  | 0 |  | 0 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 23 |  | 26 |  | 0 |  | 1 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 24 |  | 26 |  | 0 |  | 1 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 25 |  | 26 |  | 0 |  | 1 |  | 4.5 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 26 |  | 26 |  | 0 |  | 1 |  | 8 |  | 0 |  | 18 |  | 4 |  | 3 |  | 28+TT |  | 14-TT |  |
| 27 |  | 26 |  | 0 |  | 0 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 28 |  | 26 |  | 0 |  | 1 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 29 |  | 26 |  | 0 |  | 4.5 |  | 3 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 30 |  | 26 |  | 0 |  | 4.5 |  | 3 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 31 |  | 26 |  | 0 |  | 4.5 |  | 3 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 32 |  | 26 |  | 0 |  | 4.5 |  | 3 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 33 |  | 26 |  | 0 |  | 4.5 |  | 4.5 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 34 |  | 26 |  | 0 |  | 4.5 |  | 3 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 35 |  | 26 |  | 0 |  | 4.5 |  | 4.5 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 36 |  | 26 |  | 0 |  | 4.5 |  | 4 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 37 |  | 26 |  | 0 |  | 4.5 |  | 4.5 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 38 |  | 26 |  | 0 |  | 4.5 |  | 4.5 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 39 |  | 26 |  | 0 |  | 4.5 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 40 |  | 26 |  | 0 |  | 4.5 |  | 4.5 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 41 |  | 26 |  | 0 |  | 4.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 42 |  | 26 |  | 0 |  | 4.5 |  | 4 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 43 |  | 26 |  | 0 |  | 4.5 |  | 4.5 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 44 |  | 26 |  | 0 |  | 4.5 |  | 8 |  | 0 |  | 18 |  | 4 |  | 3 |  | 28+TT |  | 14-TT |  |
| 45 |  | 26 |  | 0 |  | 4.5 |  | 4 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 46 |  | 26 |  | 0 |  | 4.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 47 |  | 26 |  | 0 |  | 4.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 48 |  | 26 |  | 0 |  | 4.5 |  | 4.5 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 49 |  | 26 |  | 0 |  | 4.5 |  | 8 |  | 0 |  | 18 |  | 4 |  | 3 |  | 28+TT |  | 14-TT |  |
| 50 |  | 26 |  | 0 |  | 4.5 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 51 |  | 26 |  | 0 |  | 4.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 52 |  | 26 |  | 0 |  | 4.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 53 |  | 26 |  | 0 |  | 4.5 |  | 4.5 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 54 |  | 26 |  | 0 |  | 4.5 |  | 8 |  | 0 |  | 18 |  | 4 |  | 3 |  | 28+TT |  | 14-TT |  |
| 55 |  | 26 |  | 0 |  | 4.5 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 56 |  | 26 |  | 0 |  | 4.5 |  | 12 |  | 0 |  | 14 |  | 5 |  | 3 |  | 28+TT |  | 9-TT |  |
| 57 |  | 26 |  | 0 |  | 3 |  | 4.5 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 58 |  | 26 |  | 0 |  | 3 |  | 4.5 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 59 |  | 26 |  | 0 |  | 3 |  | 4.5 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 60 |  | 26 |  | 0 |  | 3 |  | 4.5 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 61 |  | 26 |  | 0 |  | 3 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62 |  | 26 |  | 0 |  | 3 |  | 4.5 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 63 |  | 26 |  | 0 |  | 3 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 64 | 26 | 0 | 3.5 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 65 |  | 26 |  | 0 |  | 3 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 66 |  | 26 |  | 0 |  | 3 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 67 |  | 26 |  | 0 |  | 3.5 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 68 |  | 26 |  | 0 |  | 3 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 69 |  | 26 |  | 0 |  | 3 |  | 12.5 |  | 0 |  | 13.5 |  | 5 |  | 3 |  | 28+TT |  | 8.5-TT |  |
| 70 |  | 26 |  | 0 |  | 3 |  | 5.5 |  | 0 |  | 20.5 |  | 2.5 |  | 3 |  | 28+TT |  | 17.5-TT |  |
| 71 |  | 26 |  | 0 |  | 3 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 72 |  | 26 |  | 0 |  | 3 |  | 9 |  | 0 |  | 17 |  | 5 |  | 3 |  | 28+TT |  | 12-TT |  |
| 73 |  | 26 |  | 0 |  | 3 |  | 5.5 |  | 0 |  | 20.5 |  | 2.5 |  | 3 |  | 28+TT |  | 17.5-TT |  |
| 74 |  | 26 |  | 0 |  | 3 |  | 12.5 |  | 0 |  | 13.5 |  | 5 |  | 3 |  | 28+TT |  | 8.5-TT |  |
| 75 |  | 26 |  | 0 |  | 3 |  | 12.5 |  | 0 |  | 13.5 |  | 5 |  | 3 |  | 28+TT |  | 8.5-TT |  |
| 76 |  | 26 |  | 0 |  | 3 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 77 |  | 26 |  | 0 |  | 3 |  | 9 |  | 0 |  | 17 |  | 5 |  | 3 |  | 28+TT |  | 12-TT |  |
| 78 |  | 26 |  | 0 |  | 1.5 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 79 |  | 26 |  | 0 |  | 3 |  | 12.5 |  | 0 |  | 13.5 |  | 5 |  | 3 |  | 28+TT |  | 8.5-TT |  |
| 80 |  | 26 |  | 0 |  | 3 |  | 12.5 |  | 0 |  | 13.5 |  | 5 |  | 3 |  | 28+TT |  | 8.5-TT |  |
| 81 |  | 26 |  | 0 |  | 3 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 82 |  | 26 |  | 0 |  | 3 |  | 9 |  | 0 |  | 17 |  | 5 |  | 3 |  | 28+TT |  | 12-TT |  |
| 83 |  | 26 |  | 0 |  | 1.5 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 84 |  | 26 |  | 0 |  | 3 |  | 12.5 |  | 0 |  | 13.5 |  | 5 |  | 3 |  | 28+TT |  | 8.5-TT |  |
| 85 |  | 26 |  | 0 |  | 6.5 |  | 4.5 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 86 |  | 26 |  | 0 |  | 6.5 |  | 4.5 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 87 |  | 26 |  | 0 |  | 6.5 |  | 4.5 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 88 |  | 26 |  | 0 |  | 6.5 |  | 4.5 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 89 |  | 26 |  | 0 |  | 6.5 |  | 6 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 90 |  | 26 |  | 0 |  | 6.5 |  | 4.5 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 91 |  | 26 |  | 0 |  | 6.5 |  | 6 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 92 |  | 26 |  | 0 |  | 6.5 |  | 5.5 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 93 |  | 26 |  | 0 |  | 6.5 |  | 6 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 94 |  | 26 |  | 0 |  | 6.5 |  | 6 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 95 |  | 26 |  | 0 |  | 6.5 |  | 6 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 96 |  | 26 |  | 0 |  | 6.5 |  | 6 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 97 |  | 26 |  | 0 |  | 6.5 |  | 12.5 |  | 0 |  | 13.5 |  | 5 |  | 3 |  | 28+TT |  | 8.5-TT |  |
| 98 |  | 26 |  | 0 |  | 6.5 |  | 5.5 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 99 |  | 26 |  | 0 |  | 6.5 |  | 6 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 100 |  | 26 |  | 0 |  | 6.5 |  | 9 |  | 0 |  | 17 |  | 5 |  | 3 |  | 28+TT |  | 12-TT |  |
| 101 |  | 26 |  | 0 |  | 6.5 |  | 5.5 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 102 |  | 26 |  | 0 |  | 6.5 |  | 12.5 |  | 0 |  | 13.5 |  | 5 |  | 3 |  | 28+TT |  | 8.5-TT |  |
| 103 |  | 26 |  | 0 |  | 6.5 |  | 12.5 |  | 0 |  | 13.5 |  | 5 |  | 3 |  | 28+TT |  | 8.5-TT |  |
| 104 |  | 26 |  | 0 |  | 6.5 |  | 6 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 105 |  | 26 |  | 0 |  | 6.5 |  | 9 |  | 0 |  | 17 |  | 5 |  | 3 |  | 28+TT |  | 12-TT |  |
| 106 |  | 26 |  | 0 |  | 6.5 |  | 6 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 107 |  | 26 |  | 0 |  | 6.5 |  | 12.5 |  | 0 |  | 13.5 |  | 5 |  | 3 |  | 28+TT |  | 8.5-TT |  |
| 108 |  | 26 |  | 0 |  | 6.5 |  | 12.5 |  | 0 |  | 13.5 |  | 5 |  | 3 |  | 28+TT |  | 8.5-TT |  |
| 109 |  | 26 |  | 0 |  | 6.5 |  | 6 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 110 |  | 26 |  | 0 |  | 6.5 |  | 9 |  | 0 |  | 17 |  | 5 |  | 3 |  | 28+TT |  | 12-TT |  |
| 111 |  | 26 |  | 0 |  | 6.5 |  | 6 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 112 |  | 26 |  | 0 |  | 6.5 |  | 12.5 |  | 0 |  | 13.5 |  | 5 |  | 3 |  | 28+TT |  | 8.5-TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

Table 6.2.3.5-33a: UE Power Class 2 test requirements (NS_49)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 26 |  | 0 |  | 1 |  | 4 |  | 0 |  | 22 |  | 2 |  | 3 |  | 28+TT |  | 19-TT |  |
| 2 |  | 26 |  | 0 |  | 3.5 |  | 4 |  | 0 |  | 22 |  | 2 |  | 3 |  | 28+TT |  | 19-TT |  |
| 3 |  | 26 |  | 0 |  | 1 |  | 4 |  | 0 |  | 22 |  | 2 |  | 3 |  | 28+TT |  | 19-TT |  |
| 4 |  | 26 |  | 0 |  | 1 |  | 4 |  | 0 |  | 22 |  | 2 |  | 3 |  | 28+TT |  | 19-TT |  |
| 5 |  | 26 |  | 0 |  | 1 |  | 5.5 |  | 0 |  | 20.5 |  | 2.5 |  | 3 |  | 28+TT |  | 17.5-TT |  |
| 6 |  | 26 |  | 0 |  | 1 |  | 4 |  | 0 |  | 22 |  | 2 |  | 3 |  | 28+TT |  | 19-TT |  |
| 7 |  | 26 |  | 0 |  | 1 |  | 5.5 |  | 0 |  | 20.5 |  | 2.5 |  | 3 |  | 28+TT |  | 17.5-TT |  |
| 8 |  | 26 |  | 0 |  | 3.5 |  | 4 |  | 0 |  | 22 |  | 2 |  | 3 |  | 28+TT |  | 19-TT |  |
| 9 |  | 26 |  | 0 |  | 0 |  | 4 |  | 0 |  | 22 |  | 2 |  | 3 |  | 28+TT |  | 19-TT |  |
| 10 |  | 26 |  | 0 |  | 1 |  | 12.5 |  | 0 |  | 13.5 |  | 5 |  | 3 |  | 28+TT |  | 8.5-TT |  |
| 11 |  | 26 |  | 0 |  | 1 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 12 |  | 26 |  | 0 |  | 1 |  | 4 |  | 0 |  | 22 |  | 2 |  | 3 |  | 28+TT |  | 19-TT |  |
| 13 |  | 26 |  | 0 |  | 1 |  | 5.5 |  | 0 |  | 20.5 |  | 2.5 |  | 3 |  | 28+TT |  | 17.5-TT |  |
| 14 |  | 26 |  | 0 |  | 1 |  | 7 |  | 0 |  | 19 |  | 3.5 |  | 3 |  | 28+TT |  | 15.5-TT |  |
| 15 |  | 26 |  | 0 |  | 3.5 |  | 12.5 |  | 0 |  | 13.5 |  | 5 |  | 3 |  | 28+TT |  | 8.5-TT |  |
| 16 |  | 26 |  | 0 |  | 1 |  | 12.5 |  | 0 |  | 13.5 |  | 5 |  | 3 |  | 28+TT |  | 8.5-TT |  |
| 17 |  | 26 |  | 0 |  | 0 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 18 |  | 26 |  | 0 |  | 1 |  | 5.5 |  | 0 |  | 20.5 |  | 2.5 |  | 3 |  | 28+TT |  | 17.5-TT |  |
| 19 |  | 26 |  | 0 |  | 0 |  | 7 |  | 0 |  | 19 |  | 3.5 |  | 3 |  | 28+TT |  | 15.5-TT |  |
| 20 |  | 26 |  | 0 |  | 0 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 21 | 26 | 0 | 1 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 22 | 26 | 0 | 1 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 23 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 24 | 26 | 0 | 0 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 25 | 26 | 0 | 0 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 26 |  | 26 |  | 0 |  | 1 |  | 12.5 |  | 0 |  | 13.5 |  | 5 |  | 3 |  | 28+TT |  | 8.5-TT |  |
| 27 |  | 26 |  | 0 |  | 1 |  | 12.5 |  | 0 |  | 13.5 |  | 5 |  | 3 |  | 28+TT |  | 8.5-TT |  |
| 28 |  | 26 |  | 0 |  | 4.5 |  | 4 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 29 |  | 26 |  | 0 |  | 4.5 |  | 4 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 30 |  | 26 |  | 0 |  | 4.5 |  | 4 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 31 |  | 26 |  | 0 |  | 4.5 |  | 4 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 32 |  | 26 |  | 0 |  | 4.5 |  | 5.5 |  | 0 |  | 20.5 |  | 2.5 |  | 3 |  | 28+TT |  | 17.5-TT |  |
| 33 |  | 26 |  | 0 |  | 4.5 |  | 4 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 34 |  | 26 |  | 0 |  | 4.5 |  | 5.5 |  | 0 |  | 20.5 |  | 2.5 |  | 3 |  | 28+TT |  | 17.5-TT |  |
| 35 |  | 26 |  | 0 |  | 4.5 |  | 4 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 36 |  | 26 |  | 0 |  | 4.5 |  | 4 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 37 |  | 26 |  | 0 |  | 4.5 |  | 12.5 |  | 0 |  | 13.5 |  | 5 |  | 3 |  | 28+TT |  | 8.5-TT |  |
| 38 |  | 26 |  | 0 |  | 4.5 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 39 |  | 26 |  | 0 |  | 4.5 |  | 4 |  | 0 |  | 21.5 |  | 2 |  | 3 |  | 28+TT |  | 18.5-TT |  |
| 40 |  | 26 |  | 0 |  | 4.5 |  | 5.5 |  | 0 |  | 20.5 |  | 2.5 |  | 3 |  | 28+TT |  | 17.5-TT |  |
| 41 |  | 26 |  | 0 |  | 4.5 |  | 7 |  | 0 |  | 19 |  | 3.5 |  | 3 |  | 28+TT |  | 15.5-TT |  |
| 42 |  | 26 |  | 0 |  | 4.5 |  | 12.5 |  | 0 |  | 13.5 |  | 5 |  | 3 |  | 28+TT |  | 8.5-TT |  |
| 43 |  | 26 |  | 0 |  | 4.5 |  | 12.5 |  | 0 |  | 13.5 |  | 5 |  | 3 |  | 28+TT |  | 8.5-TT |  |
| 44 |  | 26 |  | 0 |  | 4.5 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 45 |  | 26 |  | 0 |  | 4.5 |  | 5.5 |  | 0 |  | 20.5 |  | 2.5 |  | 3 |  | 28+TT |  | 17.5-TT |  |
| 46 | 26 | 0 | 4.5 | 7 | 0 | 19 | 3.5 | 3 | 28+TT | 15.5-TT |
| 47 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 48 | 26 | 0 | 4.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 49 | 26 | 0 | 4.5 | 12.5 | 0 | 13.5 | 5 | 3 | 28+TT | 8.5-TT |
| 50 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 51 | 26 | 0 | 4.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 52 |  | 26 |  | 0 |  | 4.5 |  | 7 |  | 0 |  | 19 |  | 3.5 |  | 3 |  | 28+TT |  | 15.5-TT |  |
| 53 |  | 26 |  | 0 |  | 4.5 |  | 12.5 |  | 0 |  | 13.5 |  | 5 |  | 3 |  | 28+TT |  | 8.5-TT |  |
| 54 |  | 26 |  | 0 |  | 4.5 |  | 12.5 |  | 0 |  | 13.5 |  | 5 |  | 3 |  | 28+TT |  | 8.5-TT |  |
| 55 |  | 26 |  | 0 |  | 3 |  | 5.5 |  | 0 |  | 20.5 |  | 2.5 |  | 3 |  | 28+TT |  | 17.5-TT |  |
| 56 |  | 26 |  | 0 |  | 3.5 |  | 5.5 |  | 0 |  | 20.5 |  | 2.5 |  | 3 |  | 28+TT |  | 17.5-TT |  |
| 57 |  | 26 |  | 0 |  | 3 |  | 5.5 |  | 0 |  | 20.5 |  | 2.5 |  | 3 |  | 28+TT |  | 17.5-TT |  |
| 58 |  | 26 |  | 0 |  | 3 |  | 5.5 |  | 0 |  | 20.5 |  | 2.5 |  | 3 |  | 28+TT |  | 17.5-TT |  |
| 59 |  | 26 |  | 0 |  | 3 |  | 7.5 |  | 0 |  | 18.5 |  | 4 |  | 3 |  | 28+TT |  | 14.5-TT |  |

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 |  | 26 |  | 0 |  | 3 |  | 5.5 |  | 0 |  | 20.5 |  | 2.5 |  | 3 |  | 28+TT |  | 17.5-TT |  |
| 61 |  | 26 |  | 0 |  | 3 |  | 7.5 |  | 0 |  | 18.5 |  | 4 |  | 3 |  | 28+TT |  | 14.5-TT |  |
| 62 |  | 26 |  | 0 |  | 3.5 |  | 5.5 |  | 0 |  | 20.5 |  | 2.5 |  | 3 |  | 28+TT |  | 17.5-TT |  |
| 63 |  | 26 |  | 0 |  | 1.5 |  | 5.5 |  | 0 |  | 20.5 |  | 2.5 |  | 3 |  | 28+TT |  | 17.5-TT |  |
| 64 |  | 26 |  | 0 |  | 3 |  | 13.5 |  | 0 |  | 12.5 |  | 6 |  | 3 |  | 28+TT |  | 6.5-TT |  |
| 65 | 26 | 0 | 3 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 66 | 26 | 0 | 3 | 5.5 | 0 | 20.5 | 2.5 | 3 | 28+TT | 17.5-TT |
| 67 | 26 | 0 | 3 | 7.5 | 0 | 18.5 | 4 | 3 | 28+TT | 14.5-TT |
| 68 | 26 | 0 | 3 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 69 | 26 | 0 | 3.5 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 70 | 26 | 0 | 3 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 71 |  | 26 |  | 0 |  | 1.5 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 72 |  | 26 |  | 0 |  | 3 |  | 7.5 |  | 0 |  | 18.5 |  | 4 |  | 3 |  | 28+TT |  | 14.5-TT |  |
| 73 |  | 26 |  | 0 |  | 3 |  | 8 |  | 0 |  | 18 |  | 4 |  | 3 |  | 28+TT |  | 14-TT |  |
| 74 |  | 26 |  | 0 |  | 1.5 |  | 6 |  | 0 |  | 20 |  | 2.5 |  | 3 |  | 28+TT |  | 17-TT |  |
| 75 |  | 26 |  | 0 |  | 3 |  | 13.5 |  | 0 |  | 12.5 |  | 6 |  | 3 |  | 28+TT |  | 6.5-TT |  |
| 76 | 26 | 0 | 3 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 77 | 26 | 0 | 1.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 78 | 26 | 0 | 1.5 | 6 | 0 | 20 | 2.5 | 3 | 28+TT | 17-TT |
| 79 | 26 | 0 | 1.5 | 8 | 0 | 18 | 4 | 3 | 28+TT | 14-TT |
| 80 | 26 | 0 | 3 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 81 | 26 | 0 | 3 | 13.5 | 0 | 12.5 | 6 | 3 | 28+TT | 6.5-TT |
| 82 |  | 26 |  | 0 |  | 6.5 |  | 5.5 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 83 |  | 26 |  | 0 |  | 6.5 |  | 5.5 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 84 |  | 26 |  | 0 |  | 6.5 |  | 5.5 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 85 |  | 26 |  | 0 |  | 6.5 |  | 5.5 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 86 |  | 26 |  | 0 |  | 6.5 |  | 7.5 |  | 0 |  | 18.5 |  | 4 |  | 3 |  | 28+TT |  | 14.5-TT |  |
| 87 |  | 26 |  | 0 |  | 6.5 |  | 5.5 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 88 |  | 26 |  | 0 |  | 6.5 |  | 7.5 |  | 0 |  | 18.5 |  | 4 |  | 3 |  | 28+TT |  | 14.5-TT |  |
| 89 |  | 26 |  | 0 |  | 6.5 |  | 5.5 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 90 |  | 26 |  | 0 |  | 6.5 |  | 5.5 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 91 |  | 26 |  | 0 |  | 6.5 |  | 13.5 |  | 0 |  | 12.5 |  | 6 |  | 3 |  | 28+TT |  | 6.5-TT |  |
| 92 |  | 26 |  | 0 |  | 6.5 |  | 6 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 93 |  | 26 |  | 0 |  | 6.5 |  | 5.5 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 94 |  | 26 |  | 0 |  | 6.5 |  | 7.5 |  | 0 |  | 18.5 |  | 4 |  | 3 |  | 28+TT |  | 14.5-TT |  |
| 95 |  | 26 |  | 0 |  | 6.5 |  | 8 |  | 0 |  | 18 |  | 4 |  | 3 |  | 28+TT |  | 14-TT |  |
| 96 |  | 26 |  | 0 |  | 6.5 |  | 13.5 |  | 0 |  | 12.5 |  | 6 |  | 3 |  | 28+TT |  | 6.5-TT |  |
| 97 |  | 26 |  | 0 |  | 6.5 |  | 13.5 |  | 0 |  | 12.5 |  | 6 |  | 3 |  | 28+TT |  | 6.5-TT |  |
| 98 |  | 26 |  | 0 |  | 6.5 |  | 6 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 99 |  | 26 |  | 0 |  | 6.5 |  | 7.5 |  | 0 |  | 18.5 |  | 4 |  | 3 |  | 28+TT |  | 14.5-TT |  |
| 100 |  | 26 |  | 0 |  | 6.5 |  | 8 |  | 0 |  | 18 |  | 4 |  | 3 |  | 28+TT |  | 14-TT |  |
| 101 |  | 26 |  | 0 |  | 6.5 |  | 6 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 102 |  | 26 |  | 0 |  | 6.5 |  | 13.5 |  | 0 |  | 12.5 |  | 6 |  | 3 |  | 28+TT |  | 6.5-TT |  |
| 103 |  | 26 |  | 0 |  | 6.5 |  | 13.5 |  | 0 |  | 12.5 |  | 6 |  | 3 |  | 28+TT |  | 6.5-TT |  |
| 104 |  | 26 |  | 0 |  | 6.5 |  | 6 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 105 |  | 26 |  | 0 |  | 6.5 |  | 6 |  | 0 |  | 19.5 |  | 3.5 |  | 3 |  | 28+TT |  | 16-TT |  |
| 106 |  | 26 |  | 0 |  | 6.5 |  | 8 |  | 0 |  | 18 |  | 4 |  | 3 |  | 28+TT |  | 14-TT |  |
| 107 |  | 26 |  | 0 |  | 6.5 |  | 13.5 |  | 0 |  | 12.5 |  | 6 |  | 3 |  | 28+TT |  | 6.5-TT |  |
| 108 |  | 26 |  | 0 |  | 6.5 |  | 13.5 |  | 0 |  | 12.5 |  | 6 |  | 3 |  | 28+TT |  | 6.5-TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

[End of change]