3GPP TSG-RAN5 Meeting #96-e	R5-225856

# Electronic Meeting, 15th – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1895 | rev | 1 | Current version: | 17.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR PS RRM TC 7.1.1.4 - intra-freq reselection not-at-cell-edge with TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To finish NR PS RRM TC 7.1.1.4. - To update test parameters in NR PS RRM TC 7.1.1.4 according to TT analysis. - To align with 38.133 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - NR PS RRM TC 7.1.1.4 is finished - Test parameters in NR PS RRM TC 7.1.1.4 are updated according to TT analysis. - Aligned with 38.133 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 CR 1898 TS 38.903 CR 0354 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-224534r1 |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.1.1.4	NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion

Editor’s Note: This test case is complete for the following configurations:

- Test frequency f ≤ 40.8 GHz.

- UE PC3

This test case is incomplete for Test frequency f > 40.8 GHz

This test case is incomplete for UE power class other than PC3.

7.1.1.4.1	Test purpose

The purpose of this test is to verify the requirement for the intra frequency NR cell reselection requirements when UE fulfils the not-at-cell edge relaxed measurement criterion specified in TS 38.133 clause 4.2.2.9.

7.1.1.4.2	Test applicability

This test applies to all types of NR UE from Release 16 onwards supporting relaxed RRM measurement.

7.1.1.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 7.1.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.7.1.1.4.

7.1.1.4.4	Test description

7.1.1.4.4.1	Initial conditions

This test shall be run in one of the configurations defined in Table 7.1.1.4.4.1-1.

Table 7.1.1.4.4.1-1: Supported test configurations for NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion

| Configuration | Description |
| --- | --- |
| 7.1.1.4-1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 7.1.1.4-2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 7.1.1.4.4.1-2.

Table 7.1.1.4.4.1-2: Initial conditions for NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.1.1.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.6.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1. The general test parameter settings are set up according to Table 7.1.1.4.4.1-3.

2. Message contents are defined in clause 7.1.1.4.4.3.

3. There is one NR carrier and 2 NR Cells specified in the test. Cell 1 is the PCell and Cell 2 is the neighbour cell. Cell 1 and Cell 2 are configured according to Annex C.1.1 and C.1.2.

Table 7.1.1.4.4.1-3: General test parameters for NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell1 | The UE camps on cell 1 in the initial phase |
|  | Neighbour cells |  | 1, 2 | Cell2 |  |
| T1 end condition | Active cell |  | 1, 2 | Cell2 | The UE reselects to cell 2 during T1 period |
|  | Neighbour cells |  | 1, 2 | Cell1 |  |
| Final condition | Active cell |  | 1, 2 | Cell1 |  |
|  | Neighbour cells |  | 1,2 | Cell2 |  |
| RF Channel Number |  |  | 1, 2 | 1 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SMTC configuration |  |  | 1, 2 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | 100 |  |
| T2 |  | s | 1, 2 | 100 |  |

7.1.1.4.4.2	Test procedure

Two cells are deployed in the test, which are one FR2 NR PCell (Cell 1) and an NR neighbour cell (Cell 2) on the same frequency. The test consists of 2 successive time periods, with time duration of T1, T2 respectively. Both cell 1 and cell2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas.

The UE is configured only not-at-cell edge criterion defined in clause 5.2.4.9.1 in TS 38.304 [30]. So, Cell 1 configures the UE as follows:

-	lowMobilityEvaluation [2] is not configured;

-	cellEdgeEvaluation [2] is configured according to the parameters listed in Table 7.1.1.4.5-1;

-	combineRelaxedMeasCondition [2] is not configured

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRC SETUP REQUEST message to perform a Registration procedure for mobility.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Set the parameters according to T1 in Table 7.1.1.4.5-1. T1 starts.

3.	The SS waits for random access requests information from the UE to perform cell re-selection to Cell 2.

4.	If the UE responds on Cell 2 during time duration T1 within 100 seconds from the beginning of time period T1, then count a success for cell reselection. Otherwise count a fail for cell reselection.

5.	If the UE has re-selected Cell 2 within T1, after the re-selection or when T1 expires, continue with step 6. Otherwise, if T1 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and skip to step 10.

5a	The SS shall send an RRCConnectionRelease to ensure that the UE is in state RRC_IDLE.

6.	The SS shall switch the power setting from T1 to T2 as specified in Table 7.1.1.4.5-1. T2 starts.

7.	The SS waits for random access requests information from the UE to perform cell re-selection to an already detected cell, Cell 1.

8.	If the UE responds to Cell 1 during time duration T2 within 100 seconds from the beginning of time period T2, then count a success for cell reselection. Otherwise count a fail for cell reselection.

9.	If the UE has re-selected Cell 1 within T2, after the re-selection or when T2 expires, continue with step 10. Otherwise, if T2 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE and continue with step 10.

10.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

11.	Repeat step 2-10 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

7.1.1.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 7.1.1.4.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.1-2 |
| Default RRC messages and information elements contents exceptions |  |

Table 7.1.1.4.4.3-2: RACH-ConfigGeneric: NR cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 190 |  |  |
| } |  |  |  |

Table 7.1.1.4.4.3-2A: SIB1 (Cell 1 and Cell 2)

| Derivation Path: Table H.2.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |  |
| q-RxLevMin |  | -60 | Actual value is -60*2 = -120dBm | 7.1.1.4-1 |
|  |  | -58 | Actual value is -58*2 = -116dBm | 7.1.1.4-2 |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.4.4.3-3: SIB2 (Cell 1 and Cell 2)

| Derivation Path: Table H.2.1-1 with condition SMTC.1 and Synchronous cells |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -60 | Actual value is -60*2 = -120dBm | 7.1.1.4-1 |
|  | -58 | Actual value is -58*2 = -116dBm | 7.1.1.4-2 |
| s-IntraSearchP | 25 | Actual value is 25*2 = 50dB |  |
| relaxedMeasurement-r16 SEQUENCE { |  |  |  |
| lowMobilityEvaluation-r16 | Not present |  |  |
| cellEdgeEvaluation-r16 SEQUENCE { |  |  |  |
| s-SearchThresholdP-r16 | 4 | Actual value is 4*2 = 8dBm |  |
| } |  |  |  |
| combineRelaxedMeasCondition-r16 | Not present |  |  |
| highPriorityMeasRelax-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

7.1.1.4.5	Test requirement

Tables 7.1.1.4.4.1-3 and 7.1.1.4.5-1 define the primary level settings including test tolerances for intra frequency NR cell re-selection test case for UE fulfilling not-at-cell edge relaxed measurement criterion.

Table 7.1.1.4.5-1: Cell specific test parameters for NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| PDSCH RMC |  | 1 | SR.3.1 TDD |  | SR.3.1 TDD |  |
| configuration |  | 2 | SR.3.1 TDD |  | SR.3.1 TDD |  |
| RMSI CORESET |  | 1 | CR.3.1 TDD |  | CR.3.1 TDD |  |
| RMC configuration |  | 2 | CR.3.1 TDD |  | CR.3.1 TDD |  |
| Dedicated CORESET |  | 1 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |
| RMC configuration |  | 2 | CCR.3.1 TDD |  | CCR.3.1 TDD |  |
| SSB configuration |  | 1 | SSB.3 FR2 |  | SSB.7 FR2 |  |
|  |  | 2 | SSB.4 FR2 |  | SSB.8 FR2 |  |
| OCNG Pattern |  | 1, 2 | OP.4 |  | OP.4 |  |
| BWchannel | MHz | 1, 2 | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 1, 2 | 66 |  | 66 |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | 1 | -120 Note 6 |  | -120 Note 6 |  |
|  |  | 2 | -117 Note 6 |  | -117 Note 6 |  |
| Pcompensation | dB | 1, 2 | 0 |  | 0 |  |
| Qhysts | dB | 1, 2 | 0 |  | 0 |  |
| Qoffsets, n | dB | 1, 2 | 0 |  | 0 |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP |  | SS-RSRP |  |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.15.1 |  | Setup 1 defined in A.3.15.1 |  |
| Beam assumptionNote 4 |  | 1,2 | Rough |  | Rough |  |
| Es/Iot BB Note 5 | dB | 1 | -3.34 | 1.61 | 1.61 | -3.34 |
|  |  | 2 |  |  |  |  |
| Note2 | dBm/SCS | 1 | -93 |  |  |  |
|  |  | 2 | -90 |  |  |  |
| Note2 | dBm/15 kHz | 1 | -102 |  |  |  |
|  |  | 2 |  |  |  |  |
|  | dB | 1 | -2.90 Note 6 | 2.05 Note 6 | 2.05 Note 6 | -2.90 Note 6 |
|  |  | 2 |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -95.9 | -90.9 | -90.9 | -95.9 |
|  |  | 2 | -92.9 | -87.9 | -87.9 | -92.9 |
| Io on SSB symbols of each cell | dBm/95.04 MHz | 1 | -67.37 | -65.01 | -65.01 | -67.37 |
|  |  | 2 | -64.36 | -62.00 | -62.00 | -64.36 |
| Treselection | s | 1, 2 | 0 | 0 | 0 | 0 |
| SSearchThresholdP |  | 1, 2 | 8 Note 6 | 8 Note 6 | 8 Note 6 | 8 Note 6 |
| SintrasearchP | dB | 1, 2 | 50 |  | 50 |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. Note 6: Including the test tolerance given in Annex F. |  |  |  |  |  |  |

The cell reselection delay to an already detected cell for UE fulfilling not-at-cell edge relaxed criterion is defined as the time from the beginning of time period T1, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell reselection delay to an already detected cell for UE fulfilling not-at-cell edge relaxed criterion is also defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 1.

The cell re-selection delay to an already detected cell shall be less than 79 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

The cell re-selection delay to an already detected cell test requirement in this case is expressed as:

Cell re-selection to an already detected cell delay = Tevaluate,NR_Intra + TSI-NR

Tevaluate,NR_Intra = 76.8 s; as specified in TS 38.133 [6] clause 4.2.2.9.

TSI-NR = 1280 ms; maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell is assumed in this test.

This gives a total of 78.08 s, allow 79s for the cell re-selection delay to an already detected cell for UE fulfilling not-at-cell edge criterion in the test case.

<End of modified section 1>