Source: 	Keysight Technologies, Huawei

Title: 	Discussion on test point selection for NR ACS in FR1

Introduction

At RAN5#78, it was stated that each test case would require specific analysis to conform its test configuration table based on the study for Frequency, Channel BW, SCS, RB allocation (including modulations). This analysis will be captured in TR 38.905

No test proposal has been defined yet in [2] for NR Adjacent Channel Selectivity so all aspects related to this test case keep opened to be studied.

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for NR Adjacent Channel Selectivity test case in FR1. Firstly single transmission case is done, then UL-MIMO case is provided with exception test points analysis based on the outcome from comparing the minimum requirements and waveforms allowed.

Discussion

Adjacent Channel Selectivity for FR1 will be defined in [2] where it is explained the scope of the Adjacent Channel Selectivity as follows:

Adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

Based on the Minimum conformance requirements for NR Adjacent Channel Selectivity for non-CA in [1]:

7.5.1	Adjacent Channel Selectivity for non-CA

The UE shall fulfil the minimum requirements specified in Table 7.5.1-1 for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz and the minimum requirements specified in Table 7.5.1-2 for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz. These requirements apply for all values of an adjacent channel interferer up to -25 dBm and for any SCS specified for the channel bandwidth of the wanted signal [under test]. However, it is not possible to directly measure the ACS; instead the lower and upper range of test parameters are chosen as in Table 7.5.1-3 and Table 7.5.1-4 for verification of the requirements specified in Table 7.5.1-1 and as in Table 7.5.1-5, and Table 7.5.1-6 for verification of the requirements specified in Table 7.5.1-2. For these test parameters, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels.

Table 7.5.1-1: ACS for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| ACS | dB | [33] | [33] | [30] | [27] | [26] |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 40 MHz | 50 MHz | 60 MHz | 100 MHz |  |
| ACS | dB | [24] | [23] | [22.5] | [N/A] |  |

Table 7.5.1-2: ACS for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 40 MHz | 60 MHz | 80 MHz |
| ACS | dB | [33] | [33] | [33] | [33] | [33] |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 100 MHz |  |  |  |  |
| ACS | dB | [33] |  |  |  |  |

Table 7.5.1-3: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 1

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS + [45.5] dB | REFSENS + [45.5] dB | REFSENS + [42.5] dB | REFSENS + [39.5] dB | REFSENS + [38.5] dB |
| BWinterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| Finterferer (offset) | MHz | 5 / -5 | 7.5 / -7.5 | 10 / -10 | 12.5 / -12.5 | 15 / -15 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 40 MHz | 50 MHz | 60 MHz |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB | REFSENS + 14 dB | REFSENS + 14 dB |  |  |
| Pinterferer | dBm | REFSENS + [36.5] dB | REFSENS + [35.5] dB | REFSENS + [35] dB |  |  |
| BWinterferer | MHz | 5 | 5 | 5 |  |  |
| Finterferer (offset) | MHz | 22.5 / -22.5 | 27.5 / -27.5 | 32.5 / -32.5 |  |  |
| NOTE 1: The transmitter shall be set to 4dB below […]. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the NR interferer RMC specified in […] |  |  |  |  |  |  |

Table 7.5.1-4: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 2

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Power in transmission bandwidth configuration | dBm | [-56.5] | [-56.5] | [-53.5] | [-50.5] | [-49.5] |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| Finterferer (offset) | MHz | 5 / -5 | 7.5 / -7.5 | 10 / -10 | 12.5 / -12.5 | 15 / -15 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 40 MHz | 50 MHz | 60 MHz |  |  |
| Power in transmission bandwidth configuration | dBm | [-47.5] | [-46.5] | [-46] |  |  |
| Pinterferer | dBm | -25 | -25 | -25 |  |  |
| BWinterferer | MHz | 5 | 5 | 5 |  |  |
| Finterferer (offset) | MHz | 22.5 / -22.5 | 27.5 / -27.5 | 32.5 / -32.5 |  |  |
| NOTE 1: The transmitter shall be set to 4dB below […]. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in […] |  |  |  |  |  |  |

Table 7.5.1-5: Test parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 1

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 40 MHz | 60 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS + [45.5] dB |  |  |  |  |
| BWinterferer | MHz | 10 | 20 | 40 | 60 | 80 |
| Finterferer (offset) | MHz | 10 / -10 | 20 / -20 | 40 / -40 | 60 / -60 | 80 / -80 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 100 MHz |  |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |  |
| Pinterferer | dBm | REFSENS + [45.5] dB |  |  |  |  |
| BWinterferer | MHz | 100 |  |  |  |  |
| Finterferer (offset) | MHz | 100 / -100 |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below […]. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in […] |  |  |  |  |  |  |

Table 7.5.1-6: Test parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 40 MHz | 60 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | [-56.5] |  |  |  |  |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 10 | 20 | 40 | 60 | 80 |
| Finterferer (offset) | MHz | 10 / -10 | 20 / -20 | 40 / -40 | 60 / -60 | 80 / -80 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 100 MHz |  |  |  |  |
| Power in transmission bandwidth configuration | dBm | [-56.5] |  |  |  |  |
| Pinterferer | dBm | -25 |  |  |  |  |
| BWinterferer | MHz | 100 |  |  |  |  |
| Finterferer (offset) | MHz | 100 / -100 |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below […]. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in […] |  |  |  |  |  |  |

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing, uplink and downlink configurations and throughput criteria aspects.

Test Environment

In LTE, environment condition for Adjacent Channel Selectivity (ACS) test case is specified for Normal Conditions (NC). Due to NR Adjacent Channel Selectivity test is defined in similar way than LTE, it is reasonable to use the same criteria than LTE.

Proposal 1: Define Test Environment as Normal Conditions (NC) for Adjacent Channel Selectivity measurement in FR1.

Test Subcarrier Spacing

The goal of this section is to expose technical reasons to select the SCS to be tested in NR Adjacent Channel Selectivity FR1. This study is focused to detect the most critical use cases, based on SCS selection, when the nominal operation mode of the UE is compromised in this test.

This test case measure receiver's ability to receive an NR signal at its assigned channel frequency in the presence of an interfering signal.

Since in [1] is specified that both downlink wanted and interferer signals will have to use the same SCS configuration and both wanted an interferer will be allocated close each other, then it seems to be the most critical situation when the limit for the transmission bandwidth is close to limit defined by the bandwidth configuration.

The maximum transmission bandwidth configuration NRB for each channel bandwidth and subcarrier spacing is specified in Table 2.2-1 (obtained from Table 5.3.2-1 in [1]).

Table 2.2-1 Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 217 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 107 | 135 |

The maximum transmission bandwidth (in MHz) per each channel bandwidth and SCS is calculated using the values of the maximum number of Resource block and subcarrier spacing information (assuming 12 subcarriers per resource block), these values are shown in Table 2.2-2.

Table 2.2-2 Maximum transmission bandwidth configuration in MHz

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 4.5 | 9.36 | 14.22 | 19.08 | 23.94 | 28.8 | 38.88 | 48.6 | N/A | N/A | N/A |
| 30 | 3.96 | 8.64 | 13.68 | 18.36 | 23.4 | 28.08 | 38.16 | 47.88 | 58.32 | 78.12 | 98.28 |
| 60 | N/A | 7.92 | 12.96 | 17.28 | 22.32 | 27.36 | 36.72 | 46.8 | 56.88 | 77.04 | 97.2 |

Maximum transmission bandwidth for each channel bandwidth is achieved with lowest SCS values. This same assumption can be applied for both wanted and interferer signals since they share the same SCS configuration.

Observation 1: Maximum transmission bandwidth for each channel bandwidth is achieved with lowest SCS values.

Besides Table 2.2-3 (taken from Table 5.3.3-1 in [1]) depicts the minimum guardband for a UE channel bandwidth in each subcarrier spacing. This guardband is the frequency separation between the channel edge and the transmission bandwidth edge.

Table 2.2-3 Minimum Guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | [592.5] | 552.5 | 692.5 | N/A | N/A | N/A |
| 30 | 505 | 665 | 645 | 805 | 785 | [945] | 905 | 1045 | 825 | 925 | 845 |
| 60 | N/A | 1010 | 990 | 1330 | 1310 | [1290] | 1610 | 1570 | 1530 | 1450 | 1370 |

Observation 2: Lower guardbands are achieved with lower SCS for the same channel bandwidth. This same assumption can be applied for both wanted and interferer signals since they share the same SCS configuration.

Proposal 2: Select the Lowest SCS supported for UE for Adjacent Channel Selectivity measurement in FR1.

Observation 3: Selected SCS for this test case is attending exclusively technical criteria. However, this test case would not test the new SCS defined by NR, so it would be interesting to state to execute the NR Sensitivity test case with all SCS to verify the UE behaviour related to SCS configuration.

  - Test Frequencies

In LTE, Adjacent Channel Selectivity measurement is tested for Mid frequency. This criteria is reasonable to verify behaviour of the UE related to the frequency selectivity eliminating other unwanted reception behaviours which can change with the frequency. For this reason, it is reasonable to keep the same criteria for NR Adjacent Channel Selectivity.

Proposal 3: Test Mid Frequency for Adjacent Channel Selectivity measurement in FR1.

Test Channel Bandwidth

In LTE, Adjacent Channel Selectivity is tested for Lowest, 5 MHz and Highest channel bandwidths. This assumption seems to be reasonable to test different behaviours of the UE receiver related to different frequency selectivities with different channel bandwidth.

Considering Table 2.2-3 of this document, for a specific SCS, the channel bandwidth with narrowest guardband corresponds with the Lowest channel bandwidth. This situation makes the UE to conform its filter with narrow band-pass and with a very high slope, which it is difficult to obtain without adding losses in the bandpass of the filter. This difficulty would compromise the receptor filter design affecting to frequency selectivity. Based on this, it would make reasonable to test the Lowest channel bandwidths.

However, channel bandwidths with wider band pass are especially sensible to obtain ramp-down with high slope with filters whose size is small. So, the highest channel bandwidths are really sensitive to frequency selectivity problems. For this it is interesting to test the Highest bandwidths as well.

Observation 4: Test the Lowest and Highest seems to cover the most critical situations.

Although the most critical scenarios are covered by Lowest and Highest channel bandwidths, due to this test case will not increase dramatically the number of test points if this document is approved, then most conservative proposal might be applied by following the LTE guideline. Based on it, NR Adjacent Channel Selectivity for FR1 might be tested for Lower, Mid and Highest.

Proposal 4: Test the Lowest, Mid and Highest Channel Bandwidth for Adjacent Channel Selectivity measurement in FR1.

Downlink Configuration

In LTE, Adjacent Channel Selectivity and Reference Sensitivity Level test cases use the same values for the downlink configuration table related to modulations and resource allocations. In LTE these test cases use the QPSK modulation and full allocation for downlink.

It is reasonable to follow LTE philosophy for NR Adjacent Channel Selectivity. Furthermore, the minimum conformance requirements for this test case in [1] specifies the downlink power for the wanted signal as an offset from sensitivity value (for some steps). So, it makes sense to use the same downlink configuration used for sensitivity test, in section 7.3 in [2], for NR Adjacent Channel Selectivity test.

Due to this test case will be configured to work close sensitivity power levels and the goal is that UE eliminates the unwanted signal received, it seems to be logical to eliminate any influence from modulations issues by selecting the modulation more robust, QPSK, due to it has lower PAPR than the others (although the CP-OFDM waveform does not present strong variations with the modulation).

Proposal 5: Select CP-OFDM QPSK modulation for downlink configuration for Adjacent Channel Selectivity measurement in FR1.

Several criteria can be evaluated to decide the downlink allocation. The first one, was presented in section 2.2 of this document, is related to the maximum spectrum utilization per channel bandwidth. The worst case related to the frequency selectivity is when both signals wanted and interferer are close. This situation can be achieved when the RB allocation for downlink is outer position (full or partial). Considering the second aspect which is the power levels then power per subcarrier is lower when downlink configuration is full allocation.

Observation 5: Considering both the maximum spectrum utilization and the lower density power in sensibility levels criteria, it is decided that the most convenient downlink configuration is full allocation.

Proposal 6: Select maximum transmission bandwidth configuration, in terms of NRB, for downlink configuration for Adjacent Channel Selectivity measurement in FR1.

Observation 6: Both NR Adjacent Channel Selectivity and NR Reference Sensitivity should use the same modulations and RB allocations configurations.

Uplink Configuration

This section pretends to evaluate the uplink configuration to test in test case NR Adjacent Channel Selectivity for FR1 in terms of waveform, modulation and number of allocated resource blocks.

For uplink data transmissions two different waveforms are specified, DFT-s-OFDM and CP-OFDM. The modulations for these types of waveforms are presented in Table 2.6-1.

Table 2.6-1 Uplink Modulations in [3]

UL Modulations

DFT-s-OFDM PI/2 BPSK

DFT-s-OFDM QPSK

DFT-s-OFDM 16 QAM

DFT-s-OFDM 64 QAM

DFT-s-OFDM 256 QAM

CP-OFDM QPSK

CP-OFDM 16 QAM

CP-OFDM 64 QAM

CP-OFDM 256 QAM

Considering NR Adjacent Channel Selectivity evaluates the receptor characteristics does not seem so critical the selection of the uplink modulations. However, this receiver characteristic is measured by measuring downlink Throughput, due to the HARQ feedback is sent in PUSCH configuration is important to select the right modulation to avoid any issue with uplink (which is near the maximum power).

Observation 7: It is reasonable to use DFT-s-OFDM waveform which presents lower PAPR compared to CP-OFDM to avoid any uplink issues. Besides this kind of waveform is recommended in case of high power level due to it is more power efficient.

Proposal 7: Select DFT-s-OFDM waveform for uplink configuration for Adjacent Channel Selectivity measurement in FR1.

As commented in downlink section, LTE Adjacent Channel Selectivity and Reference Sensitivity Level test cases use the same values for the uplink configuration table related to modulations and resource allocations. In LTE these test cases use the QPSK modulation and outer allocations (full and partial) in all cases except band 20 where the inner configuration is specified.

Observation 8: To select the uplink modulation it is logical to apply the same criteria used for the waveform selection. These criteria are based on the lowest PAPR (linearity). For these reasons the most appropriated modulation to use is QPSK.

Proposal 8: Select QPSK modulation for uplink configuration for Adjacent Channel Selectivity measurement in FR1.

Applying for the resource allocation selection the same criteria than previously, the uplink configuration for NR Adjacent Channel Selectivity should use the same values as specified in the uplink configuration table defined in minimum conformance requirements for NR Sensitivity Reference test case in [2]. Using this configuration, uplink allocations will use outer allocations (full or partial) in all NR bands cases except for n20 band where the inner configuration is specified.

Proposal 9: NR Adjacent Channel Selectivity measurement in FR1 will use the same uplink resource allocation as NR Sensitivity Level test in FR1, where all allocations are outers (full or partial) for all NR bands except for n20 band where the inner configuration is specified.

Proposal 10: Uplink allocation in test configuration table for Adjacent Channel Selectivity measurement in FR1 will include a reference to uplink configuration table for NR Reference Sensitivity in FR1.

Exceptions for REFSENSE UL-MIMO test

Extracted from Clause 7.5D [1]

7.5D	Adjacent channel selectivity for UL-MIMO

For UE(s) with two transmitter antenna connectors in closed-loop spatial multiplexing scheme, the minimum requirements specified in sub-clause 7.5 shall be met with the UL-MIMO configurations described in sub-clause 6.2D.1. For UL-MIMO, the parameter PCMAX_L is defined as the total transmitter power over the two transmit antenna connectors.

Observation 9: Same adjacent channel selectivity requirements as specified for single transmission apply with UL-MIMO configuration.

According to [4], CP-OFDM is mandatory to UL-MIMO transmission with two layer data, therefore DFT-s-OFDM is precluded from the test points for Rx ACS UL-MIMO measurement. Generally receiver test are not sensitive to the UL waveform and modulation configuration, straightforwardly test CO-OFDM instead of DFT-s-OFDM.

Proposal 11: Test CP-OFDM QPSK for Rx ACS UL-MIMO measurement in FR1, while the other test parameters are the same as single transmission case.

Exceptions for Asymmetric CH BWs

Similar to any 1UL/xDL intra-band contiguous CA, asymmetric CH BWs should not have UL impact in Tx requirements. But similarly, it should be tested in Rx requirements.

Proposal 12: From the supported asymmetric CH BWs to a given band defined in clause 5.3.6 of TS 38.521-1, the test CH BW UL/DL combination shall be selected the following way (DL channel bandwidth shall be selected first):

- Lowest UL CH BW / Lowest DL CH BW

- Lowest UL CH BW / Highest DL CH BW

Conclusion

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 Adjacent Channel Selectivity test case definition:

Proposal 1: Define Test Environment as Normal Conditions (NC) for Adjacent Channel Selectivity measurement in FR1.

Proposal 2: Select the Lowest SCS supported for UE for Adjacent Channel Selectivity measurement in FR1.

Proposal 3: Test Mid Frequency for Adjacent Channel Selectivity measurement in FR1.

Proposal 4: Test the Lowest, Mid and Highest Channel Bandwidth for Adjacent Channel Selectivity measurement in FR1.

Proposal 5: Select CP-OFDM QPSK modulation for downlink configuration for Adjacent Channel Selectivity measurement in FR1.

Proposal 6: Select maximum transmission bandwidth configuration, in terms of NRB, for downlink configuration for Adjacent Channel Selectivity measurement in FR1.

Proposal 7: Select DFT-s-OFDM waveform for uplink configuration for Adjacent Channel Selectivity measurement in FR1.

Proposal 8: Select QPSK modulation for uplink configuration for Adjacent Channel Selectivity measurement in FR1.

Proposal 9: NR Adjacent Channel Selectivity measurement in FR1 will use the same uplink resource allocation as NR Sensitivity Level test in FR1, where all allocations are outers (full or partial) for all NR bands except for n20 band where the inner configuration is specified.

Proposal 10: Uplink allocation in test configuration table for Adjacent Channel Selectivity measurement in FR1 will include a reference to uplink configuration table for NR Reference Sensitivity in FR1.

Proposal 11: Test CP-OFDM QPSK for Rx ACS UL-MIMO measurement in FR1, while the other test parameters are the same as single transmission case

Proposal 12: From the supported asymmetric CH BWs to a given band defined in clause 5.3.6 of TS 38.521-1, the test CH BW UL/DL combination shall be selected the following way (DL channel bandwidth shall be selected first):

- Lowest UL CH BW / Lowest DL CH BW

- Lowest UL CH BW / Highest DL CH BW

- References

[1] 3GPP TS 38.101-1 v15.0.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 15)

[2] 3GPP TS 38.521-1 v0.3.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone (Release 15)

[3] 3GPP TS 38.211 v15.0.0 Physical channels and modulation (Release 15)

[4] R5-192024,” Discussion on the waveform selection for UL MIMO tests”, Huawei, RAN5#82, Athens, Feb.2019