Source: 	Keysight Technologies, CAICT

Title: 	Discussion on test point selection for NR In-Band Blocking in FR1

Introduction

At RAN5#78, it was stated that each test case would require specific analysis to conform its test configuration table based on the study for Frequency, Channel BW, SCS, RB allocation (including modulations). This analysis will be captured in TR 38.905.

Test proposal was added in [2] for NR Inband Blocking leaving open, for further study, test procedure, test configuration table and test points selection for Frequency, Channel BW, SCS, Resource Allocations and modulations.

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for NR Inband Blocking test case in FR1.

Firstly single transmission case is done, then UL-MIMO case is provided with exception test points analysis based on the outcome from comparing the minimum requirements and waveforms allowed.

Discussion

Inband Blocking for FR1 is defined in Blocking Characteristics section in [2] where it is explained the scope of the blocking characteristic measurement as follows:

The blocking characteristic is a measure of the receiver’s ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occurs.

Based on the Minimum conformance requirements for NR Inband Blocking in [1]:

7.6.2	Inband Blocking

For NR bands with FDL_high < 2700 MHz and FUL_high < 2700 in-band blocking (IBB) is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band.  The throughput of the wanted signal shall be ≥ 95% of the maximum throughput of the reference measurement channels with parameters specified in Table 7.6.2-1 and Table 7.6.2-2. The said relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal.

Table 7.6.2-1: In-band blocking parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below |  |  |  |  |
|  | dB | 6 | 6 | 7 | 9 | 10 |
| BWinterferer | MHz | 5 |  |  |  |  |
| FIoffset, case 1 | MHz | 7.5 |  |  |  |  |
| FIoffset, case 2 | MHz | 12.5 |  |  |  |  |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 40 MHz | 50 MHz | 60 MHz |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below | REFSENS + channel specific value below | REFSENS + channel specific value below |  |  |
|  | dB | 12 | 13 | 14 |  |  |
| BWinterferer | MHz | 5 | 5 | 5 |  |  |
| FIoffset, case 1 | MHz | 7.5 | 7.5 | 7.5 |  |  |
| FIoffset, case 2 | MHz | 12.5 | 12.5 | 12.5 |  |  |
| NOTE 1: The transmitter shall be set to 4dB below […]. NOTE 2: The interferer consists of the RMC specified in […] |  |  |  |  |  |  |

Table 7.6.2-2: In-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 | Case 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 | -15 |
| n1, n3, n7, n8, n20, n28, n41, n66, n70, n71 | Finterferer (offset) | MHz | -CBW/2 – FIoffset, case 1 and CBW/2 + FIoffset, case 1 | ≤ -CBW/2 – FIoffset, case 2 and ≥ CBW/2 + FIoffset, case 2 |  |
|  | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  |
| n71 | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 | FDL_low – 12 |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -CBW/2 – FIoffset, case 1; b: CBW/2 + FIoffset, case 1 |  |  |  |  |  |

For NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz in-band blocking (IBB) is defined for an unwanted interfering signal falling into the UE receive band or into an immediately adjacent frequency range up 3CBW below or above the UE receive band with CBW is the bandwidth of the wanted signal. The throughput of the wanted signal shall be ≥ 95% of the maximum throughput of the reference measurement channels with parameters specified in Table 7.6.2-3 and Table 7.6.2-4. The said relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal.

Table 7.6.2-3: In-band blocking parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 40 MHz | 60 MHz | 80 MHz |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below |  |  |  |  |
|  | dB | 6 |  |  |  |  |
| BWinterferer | MHz | 10 | 20 | 40 | 60 | 80 |
| FIoffset, case 1 | MHz | 15 | 30 | 60 | 90 | 120 |
| FIoffset, case 2 | MHz | 25 | 50 | 100 | 150 | 200 |
| RX parameter | Units | Channel bandwidth |  |  |  |  |
|  |  | 100 MHz |  |  |  |  |
| Power in transmission bandwidth configuration | dBm | REFSENS + channel specific value below |  |  |  |  |
|  | dB | 6 |  |  |  |  |
| BWinterferer | MHz | 100 |  |  |  |  |
| FIoffset, case 1 | MHz | 150 |  |  |  |  |
| FIoffset, case 2 | MHz | 250 |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below […]. NOTE 2: The interferer consists of the RMC specified in […] |  |  |  |  |  |  |

Table 7.6.2-4: In-band blocking for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 |
| n77, n78, n79 | Finterferer (offset) | MHz | -CBW/2 – FIoffset, case 1 and BW/2 + FIoffset, case 1 | ≤ -CBW/2 – FIoffset, case 2 and ≥ CBW/2 + FIoffset, case 2 |
|  | Finterferer |  | NOTE 2 | FDL_low – 3CBW to FDL_high + 3CBWl |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -CBW/2 – FIoffset, case 1; b: CBW/2 + FIoffset, case 1 NOTE 3: CBW denotes the channel bandwidth of the wanted signal |  |  |  |  |

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing, uplink and downlink configurations and throughput criteria aspects.

Test Environment

In LTE, environment condition for InBand Blocking test case is specified for Normal Conditions (NC). Due to NR InBand Blocking test is defined in similar way than LTE, it is reasonable to use the same criteria than LTE.

Proposal 1: Define Test Environment as Normal Conditions (NC) for InBand Blocking for Non-CA measurement in FR1.

Test Subcarrier Spacing

The goal of this section is to expose technical reasons to select the SCS to be tested in NR InBand Blocking for Non-CA FR1. This study is focused to detect the most critical use cases, based on SCS selection, when the nominal operation mode of the UE is compromised in this test.

This test case measure receiver’s ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band.

Since in [1] is specified that both downlink wanted and interferer signals will have to use the same SCS configuration and both wanted an interferer will be allocated close each other, then it seems to be the most critical situation when the limit for the transmission bandwidth is close to limit defined by the bandwidth configuration. Going deeper on this idea, one interesting criteria can be focused on Spectrum utilization and Minimum Guardband for each channel (wanted and interferer).

The maximum transmission bandwidth configuration NRB for each channel bandwidth and subcarrier spacing is specified in Table 2.2-1 (obtained from Table 5.3.2-1 in [1]).

Table 2.2-1 Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 217 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 107 | 135 |

The maximum transmission bandwidth (in MHz) per each channel bandwidth and SCS is calculated using the values of the maximum number of Resource block and subcarrier spacing information (assuming 12 subcarriers per resource block), these values are shown in Table 2.2-2.

Table 2.2-2 Maximum transmission bandwidth configuration in MHz

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 4.5 | 9.36 | 14.22 | 19.08 | 23.94 | 28.8 | 38.88 | 48.6 | N/A | N/A | N/A |
| 30 | 3.96 | 8.64 | 13.68 | 18.36 | 23.4 | 28.08 | 38.16 | 47.88 | 58.32 | 78.12 | 98.28 |
| 60 | N/A | 7.92 | 12.96 | 17.28 | 22.32 | 27.36 | 36.72 | 46.8 | 56.88 | 77.04 | 97.2 |

Maximum transmission bandwidth for each channel bandwidth is achieved with lowest SCS values. This same assumption can be applied for both wanted and interferer signals since they share the same SCS configuration.

Observation 1: Maximum transmission bandwidth for each channel bandwidth is achieved with lowest SCS values.

Besides Table 2.2-3 (taken from Table 5.3.3-1 in [1]) depicts the minimum guardband for a UE channel bandwidth in each subcarrier spacing. This guardband is the frequency separation between the channel edge and the transmission bandwidth edge.

Table 2.2-3 Minimum Guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | [592.5] | 552.5 | 692.5 | N/A | N/A | N/A |
| 30 | 505 | 665 | 645 | 805 | 785 | [945] | 905 | 1045 | 825 | 925 | 845 |
| 60 | N/A | 1010 | 990 | 1330 | 1310 | [1290] | 1610 | 1570 | 1530 | 1450 | 1370 |

Observation 2: Lower guardbands are achieved with lower SCS for the same channel bandwidth. This same assumption can be applied for both wanted and interferer signals since they share the same SCS configuration.

Proposal 2: Select the Lowest SCS supported for UE for InBand Blocking for Non-CA measurement in FR1.

Observation 3: Selected SCS for this test case is attending exclusively technical criteria. However, this test case would not test the new SCS defined by NR, so it would be interesting to state to execute the NR Sensitivity test case with all SCS to verify the UE behaviour related to SCS configuration.

  - Test Frequencies

In LTE, InBand Blocking measurement is tested for Mid frequency. This criteria is reasonable to verify behaviour of the UE related to the frequency selectivity eliminating other unwanted reception behaviours which can change with the frequency. For this reason, it is reasonable to keep the same criteria for NR InBand Blocking.

Proposal 3: Test Mid Frequency for InBand Blocking for Non-CA measurement in FR1.

Test Channel Bandwidth

In LTE, InBand Blocking is tested for Lowest, 5 MHz and Highest channel bandwidths. This assumption seems to be reasonable to test different behaviours of the UE receiver related to different frequency selectivities with different channel bandwidth.

Considering Table 2.2-3 of this document, for a specific SCS, the channel bandwidth with narrowest guardband corresponds with the Lowest channel bandwidth. This situation makes the UE to conform its filter with narrow band-pass and with a very high slope, which it is difficult to obtain without adding losses in the bandpass of the filter. This difficulty would compromise the receptor filter design affecting to frequency selectivity. Based on this, it would make reasonable to test the Lowest channel bandwidths.

However, channel bandwidths with wider band pass are especially sensible to obtain ramp-down with high slope with filters whose size is small. So, the highest channel bandwidths are really sensitive to frequency selectivity problems. For this it is interesting to test the Highest bandwidths as well.

Observation 4: Test the Lowest and Highest seems to cover the most critical situations.

Although the most critical scenarios are covered by Lowest and Highest channel bandwidths, due to this test case will not increase dramatically the number of test points if this document is approved, then most conservative proposal might be applied by following the LTE guideline. Based on it, NR InBand Blocking for FR1 might be tested for Lower, Mid and Highest.

Proposal 4: Test the Lowest, Mid and Highest Channel Bandwidth for InBand Blocking for Non-CA measurement in FR1.

Downlink Configuration

In LTE, InBand Blocking and Reference Sensitivity Level test cases use the same values for the downlink configuration table related to modulations and resource allocations. In LTE these test cases use the QPSK modulation and full allocation for downlink.

It is reasonable to follow LTE philosophy for NR InBand Blocking. Furthermore, the minimum conformance requirements for this test case in [1] specifies the downlink power for the wanted signal as an offset from sensitivity value (for some steps). So, it makes sense to use the same downlink configuration used for sensitivity test, in section 7.3 in [2], for NR InBand Blocking test.

Due to this test case will be configured to work close sensitivity power levels and the goal is that UE eliminates the unwanted signal received, it seems to be logical to eliminate any influence from modulations issues by selecting the modulation more robust, QPSK, due to it has lower PAPR than the others (although the CP-OFDM waveform does not present strong variations with the modulation).

Proposal 5: Select CP-OFDM QPSK modulation for downlink configuration for InBand Blocking for Non-CA measurement in FR1.

Several criteria can be evaluated to decide the downlink allocation. The first one, was presented in section 2.2 of this document, is related to the maximum spectrum utilization per channel bandwidth. The worst case related to the frequency selectivity is when both signals wanted and interferer are close. This situation can be achieved when the RB allocation for downlink is outer position (full or partial). Considering the second aspect which is the power levels then power per subcarrier is lower when downlink configuration is full allocation.

Observation 5: Considering both the maximum spectrum utilization and the lower density power in sensitivity levels criteria, it is decided that the most convenient downlink configuration is full allocation.

Proposal 6: Select maximum transmission bandwidth configuration, in terms of NRB, for downlink configuration for InBand Blocking for Non-CA measurement in FR1.

Observation 6: Both NR InBand Blocking and NR Reference Sensitivity should use the same modulations and RB allocations configurations.

Uplink Configuration

This section pretends to evaluate the uplink configuration to test in test case NR InBand Blocking for FR1 in terms of waveform, modulation and number of allocated resource blocks.

For uplink data transmissions two different waveforms are specified, DFT-s-OFDM and CP-OFDM. The modulations for these types of waveforms are presented in Table 2.6-1.

Table 2.6-1 Uplink Modulations in [3]

UL Modulations

DFT-s-OFDM PI/2 BPSK

DFT-s-OFDM QPSK

DFT-s-OFDM 16 QAM

DFT-s-OFDM 64 QAM

DFT-s-OFDM 256 QAM

CP-OFDM QPSK

CP-OFDM 16 QAM

CP-OFDM 64 QAM

CP-OFDM 256 QAM

Considering NR InBand Blocking evaluates the receptor characteristics does not seem so critical the selection of the uplink modulations. However, this receiver characteristic is measured by measuring downlink Throughput, due to the HARQ feedback is sent in PUSCH configuration is important to select the right modulation to avoid any issue with uplink (which is near the maximum power).

Observation 7: It is reasonable to use DFT-s-OFDM waveform which presents lower PAPR compared to CP-OFDM to avoid any uplink issues. Besides this kind of waveform is recommended in case of high power level due to it is more power efficient.

Proposal 7: Select DFT-s-OFDM waveform for uplink configuration for InBand Blocking for Non-CA measurement in FR1.

As commented in downlink section, LTE InBand Blocking and Reference Sensitivity Level test cases use the same values for the uplink configuration table related to modulations and resource allocations. In LTE these test cases use the QPSK modulation and outer allocations (full and partial) in all cases except band 20 where the inner configuration is specified.

Observation 8: To select the uplink modulation it is logical to apply the same criteria used for the waveform selection. These criteria are based on the lowest PAPR (linearity). For these reasons the most appropriated modulation to use is QPSK.

Proposal 8: Select QPSK modulation for uplink configuration for InBand Blocking for Non-CA measurement in FR1.

Applying for the resource allocation selection the same criteria than previously, the uplink configuration for NR InBand Blocking should use the same values as specified in the uplink configuration table defined in minimum conformance requirements for NR Sensitivity Reference test case in [2]. Using this configuration, uplink allocations will use outer allocations (full or partial) in all NR bands cases except for n20 band where the inner configuration is specified.

Proposal 9: InBand Blocking for Non-CA measurement in FR1 will use the same uplink resource allocation as NR Sensitivity Level test in FR1, where all allocations are outers (full or partial) for all NR bands except for n20 band where the inner configuration is specified.

Proposal 10: Uplink allocation in test configuration table for InBand Blocking for Non-CA measurement in FR1 will include a reference to uplink configuration table for NR Reference Sensitivity in FR1.

Exceptions for Inband blocking UL-MIMO test

Extracted from Clause 7.6D of TS 38.101-1:

For UE with two transmitter antenna connectors in closed-loop spatial multiplexing scheme, the minimum requirements specified in subclause 7.6 shall be met with the UL-MIMO configurations described in sub-clause 6.2D.1. For UL-MIMO, the parameter PCMAX_L is defined as the total transmitter power over the two transmit antenna connectors.

Observation 9: Same Inband blocking requirements as specified for single-antenna port apply to UE supporting UL-MIMO.

According to [4], CP-OFDM is mandatory in case UL-MIMO transmission with two layer data, therefore DFT-s-OFDM is precluded from the proposal 7 for Inband blocking UL-MIMO measurement. Generally receiver test are not sensitive to the UL waveform and modulation configuration, straightforwardly test CP-OFDM instead of DFT-s-OFDM.

Proposal 11: Test CP-OFDM for Inband blocking UL-MIMO measurement in FR1. The other test parameters are the same as test for single-antenna port.

Exceptions for Asymmetric CH BWs

Similar to any 1UL/xDL intra-band contiguous CA, asymmetric CH BWs should not have UL impact in Tx requirements. But similarly, it should be tested in Rx requirements.

Proposal 12: From the supported asymmetric CH BWs to a given band defined in clause 5.3.6 of TS 38.521-1, the test CH BW UL/DL combination shall be selected the following way (DL channel bandwidth shall be selected first):

- Lowest UL CH BW / Lowest DL CH BW

- Lowest UL CH BW / Highest DL CH BW

Conclusion

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 InBand Blocking for Non-CA test case definition:

Proposal 1: Define Test Environment as Normal Conditions (NC) for InBand Blocking for Non-CA measurement in FR1.

Proposal 2: Select the Lowest SCS supported for UE for InBand Blocking for Non-CA measurement in FR1.

Proposal 3: Test Mid Frequency for InBand Blocking for Non-CA measurement in FR1.

Proposal 4: Test the Lowest, Mid and Highest Channel Bandwidth for InBand Blocking for Non-CA measurement in FR1.

Proposal 5: Select CP-OFDM QPSK modulation for downlink configuration for InBand Blocking for Non-CA measurement in FR1.

Proposal 6: Select maximum transmission bandwidth configuration, in terms of NRB, for downlink configuration for InBand Blocking for Non-CA measurement in FR1.

Proposal 7: Select DFT-s-OFDM waveform for uplink configuration for InBand Blocking for Non-CA measurement in FR1.

Proposal 8: Select QPSK modulation for uplink configuration for InBand Blocking for Non-CA measurement in FR1.

Proposal 9: InBand Blocking for Non-CA measurement in FR1 will use the same uplink resource allocation as NR Sensitivity Level test in FR1, where all allocations are outers (full or partial) for all NR bands except for n20 band where the inner configuration is specified.

Proposal 10: Uplink allocation in test configuration table for InBand Blocking for Non-CA measurement in FR1 will include a reference to uplink configuration table for NR Reference Sensitivity in FR1.

Proposal 11: Test CP-OFDM for Inband blocking UL-MIMO measurement in FR1. The other test parameters are the same as test for single-antenna port.

Proposal 12: From the supported asymmetric CH BWs to a given band defined in clause 5.3.6 of TS 38.521-1, the test CH BW UL/DL combination shall be selected the following way (DL channel bandwidth shall be selected first):

- Lowest UL CH BW / Lowest DL CH BW

- Lowest UL CH BW / Highest DL CH BW

References

[1] 3GPP TS 38.101-1 v15.0.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 15)

[2] 3GPP TS 38.521-1 v0.3.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone (Release 15)

[3] 3GPP TS 38.211 v15.0.0 Physical channels and modulation (Release 15)

[4] R5-192024, “Discussion on the waveform selection for UL MIMO tests”, Huawei, RAN5#82, Athens, Feb.2019.