Source: 	CAICT

Title: 	Discussion on test point selection for NR Narrow band Blocking in FR1

Background

The general agreement in RAN5 is that FR1 testing should follow what is being tested for LTE. This is also valid for test point selection for NR Narrow band Blocking in FR1.

The purpose of this paper is to provide an analysis for the parameters in test configuration table and a selection of test points for test environment, test frequencies, channel bandwidths, SCS, RB allocation and modulation.

Firstly single transmission case is done, then UL-MIMO case is provided with exception test points analysis based on the outcome from comparing the minimum requirements and waveforms allowed.

Discussion

Narrow band Blocking for FR1 is defined in Blocking Characteristics section in [2] where it is explained the scope of the Blocking measurement as follows:

The blocking characteristic is a measure of the receiver’s ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occurs.

Based on the Minimum conformance requirements for NR Narrow band Blocking in [1]:

7.6.4	Narrow band blocking

This requirement is measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an unwanted narrow band CW interferer at a frequency, which is less than the nominal channel spacing.

The relative throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6.2-1. For operating bands with an unpaired DL part (as noted in Table 5.5-1), the requirements only apply for carriers assigned in the paired part.

Table 7.6.4-1: Narrow Band Blocking

| NR band | Parameter | Unit | Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 MHz |  | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100MHz |
| n1,n2, n3, n5, n7, n8, n12, n20, n25 n28, n34, n38, n39, n40, n41, n50, n51, n66, n70, n71, n74, n75, n76 | Pw | dBm |  | PREFSENS + channel-bandwidth specific value below |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 16 |  | 13 | 14 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
|  | Puw (CW) | dBm | -55 |  | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 | -55 |
|  | Fuw (offset SCS= 15 kHz) | MHz | 2.7075 |  | 5.2125 | 7.7025 | 10.2075 | 13.0275 | 20.5575 | NA | NA | NA | NA | NA |
|  | Fuw (offset SCS= 30 kHz) | MHz | NA |  | NA | NA | NA |  |  | TBD | TBD |  |  |  |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The PREFSENS power level is specified in Table 7.3.1-1 and Table 7.3.1-1a for two and four antenna ports, respectively. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing, uplink and downlink configurations and throughput criteria aspects.

Test Environment

In LTE, environment condition for Narrow band Blocking test case is specified for Normal Conditions (NC). Due to NR Narrow band Blocking test is defined in similar way than LTE, it is reasonable to use the same criteria than LTE.

Proposal 1: Define Test Environment as Normal Conditions (NC) for Narrow band Blocking measurement in FR1.

Test Subcarrier Spacing

The goal of this section is to expose technical reasons to select the SCS to be tested in NR Narrow band Blocking FR1. This study is focused to detect the most critical use cases, based on SCS selection, when the nominal operation mode of the UE is compromised in this test.

This test case measure receiver’s ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer signal falling outside a frequency range 15 MHz below or above the UE receive band.

Since in [1] is specified that both downlink wanted and interferer signals will have to use the same SCS configuration and both wanted an interferer will be allocated close each other, then it seems to be the most critical situation when the limit for the transmission bandwidth is close to limit defined by the bandwidth configuration. Going deeper on this idea, one interesting criteria can be focused on Spectrum utilization and Minimum Guardband for each channel (wanted and interferer).

The maximum transmission bandwidth configuration NRB for each channel bandwidth and subcarrier spacing is specified in Table 2.2-1 (obtained from Table 5.3.2-1 in [1]).

Table 2.2-1 Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 217 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 107 | 135 |

The maximum transmission bandwidth (in MHz) per each channel bandwidth and SCS is calculated using the values of the maximum number of Resource block and subcarrier spacing information (assuming 12 subcarriers per resource block), these values are shown in Table 2.2-2.

Table 2.2-2 Maximum transmission bandwidth configuration in MHz

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 4.5 | 9.36 | 14.22 | 19.08 | 23.94 | 28.8 | 38.88 | 48.6 | N/A | N/A | N/A |
| 30 | 3.96 | 8.64 | 13.68 | 18.36 | 23.4 | 28.08 | 38.16 | 47.88 | 58.32 | 78.12 | 98.28 |
| 60 | N/A | 7.92 | 12.96 | 17.28 | 22.32 | 27.36 | 36.72 | 46.8 | 56.88 | 77.04 | 97.2 |

Maximum transmission bandwidth for each channel bandwidth is achieved with lowest SCS values. This same assumption can be applied for both wanted and interferer signals since they share the same SCS configuration.

Observation 1: Maximum transmission bandwidth for each channel bandwidth is achieved with lowest SCS values.

Besides Table 2.2-3 (taken from Table 5.3.3-1 in [1]) depicts the minimum guardband for a UE channel bandwidth in each subcarrier spacing. This guardband is the frequency separation between the channel edge and the transmission bandwidth edge.

Table 2.2-3 Minimum Guardband for each UE channel bandwidth and SCS (kHz)

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | [592.5] | 552.5 | 692.5 | N/A | N/A | N/A |
| 30 | 505 | 665 | 645 | 805 | 785 | [945] | 905 | 1045 | 825 | 925 | 845 |
| 60 | N/A | 1010 | 990 | 1330 | 1310 | [1290] | 1610 | 1570 | 1530 | 1450 | 1370 |

Observation 2: Lower guardbands are achieved with lower SCS for the same channel bandwidth. This same assumption can be applied for both wanted and interferer signals since they share the same SCS configuration.

Proposal 2: Select the Lowest SCS supported for UE for Narrow band Blocking measurement in FR1.

Observation 3: Selected SCS for this test case is attending exclusively technical criteria. However, this test case would not test the new SCS defined by NR, so it would be interesting to state to execute the NR Sensitivity test case with all SCS to verify the UE behaviour related to SCS configuration.

  - Test Frequencies

In LTE, Narrow band Blocking measurement is tested for Mid frequency. This criteria is reasonable to verify behaviour of the UE related to the frequency selectivity eliminating other unwanted reception behaviours which can change with the frequency. For this reason, it is reasonable to keep the same criteria for NR Narrow band Blocking.

Proposal 3: Mid frequency for Narrow band Blocking measurement in FR1.

Test Channel Bandwidth

In LTE, Narrow band Blocking is tested for Lowest, 5 MHz and Highest channel bandwidths. This assumption seems to be reasonable to test different behaviours of the UE receiver related to different frequency selectivities with different channel bandwidth.

Considering Table 2.2-3 of this document, for a specific SCS, the channel bandwidth with narrowest guardband corresponds with the Lowest channel bandwidth. This situation makes the UE to conform its filter with narrow band-pass and with a very high slope, which it is difficult to obtain without adding losses in the bandpass of the filter. This difficulty would compromise the receptor filter design affecting to frequency selectivity. Based on this, it would make reasonable to test the Lowest channel bandwidths.

However, channel bandwidths with wider band pass are especially sensible to obtain ramp-down with high slope with filters whose size is small. So, the highest channel bandwidths are really sensitive to frequency selectivity problems. For this it is interesting to test the Highest bandwidths as well.

Observation 4: Test the Lowest and Highest seems to cover the most critical situations.

Although the most critical scenarios are covered by Lowest and Highest channel bandwidths, due to this test case will not increase dramatically the number of test points if this document is approved, then most conservative proposal might be applied by following the LTE guideline. Based on it, NR Narrow band Blocking for FR1 might be tested for Lower, Mid and Highest.

Proposal 4: Test the Lowest, Mid and Highest Channel Bandwidth for Narrow band Blocking measurement in FR1.

Downlink Configuration

In LTE, Narrow band Blocking and Reference Sensitivity Level test cases use the same values for the downlink configuration table related to modulations and resource allocations. In LTE these test cases use the QPSK modulation and full allocation for downlink.

It is reasonable to follow LTE philosophy for NR Narrow band Blocking. Furthermore, the minimum conformance requirements for this test case in [1] specifies the downlink power for the wanted signal as an offset from sensitivity value (for some steps). So, it makes sense to use the same downlink configuration used for sensitivity test, in section 7.3 in [2], for NR Narrow band Blocking test.

Due to this test case will be configured to work close sensitivity power levels and the goal is that UE eliminates the unwanted signal received, it seems to be logical to eliminate any influence from modulations issues by selecting the modulation more robust, QPSK, due to it has lower PAPR than the others (although the CP-OFDM waveform does not present strong variations with the modulation).

Proposal 5: Select CP-OFDM QPSK modulation for downlink configuration for Narrow band Blocking measurement in FR1.

Several criteria can be evaluated to decide the downlink allocation. The first one, was presented in section 2.2 of this document, is related to the maximum spectrum utilization per channel bandwidth. The worst case related to the frequency selectivity is when both signals wanted and interferer are close. This situation can be achieved when the RB allocation for downlink is outer position (full or partial). Considering the second aspect which is the power levels then power per subcarrier is lower when downlink configuration is full allocation.

Observation 5: Considering both the maximum spectrum utilization and the lower density power in sensitivity levels criteria, it is decided that the most convenient downlink configuration is full allocation.

Proposal 6: Select maximum transmission bandwidth configuration, in terms of NRB, for downlink configuration for Narrow band Blocking measurement in FR1.

Observation 6: Both NR Narrow band Blocking and NR Reference Sensitivity should use the same modulations and RB allocations configurations.

Uplink Configuration

This section pretends to evaluate the uplink configuration to test in test case NR Narrow band Blocking for FR1 in terms of waveform, modulation and number of allocated resource blocks.

For uplink data transmissions two different waveforms are specified, DFT-s-OFDM and CP-OFDM. The modulations for these types of waveforms are presented in Table 2.6-1.

Table 2.6-1 Uplink Modulations in [3]

UL Modulations

DFT-s-OFDM PI/2 BPSK

DFT-s-OFDM QPSK

DFT-s-OFDM 16 QAM

DFT-s-OFDM 64 QAM

DFT-s-OFDM 256 QAM

CP-OFDM QPSK

CP-OFDM 16 QAM

CP-OFDM 64 QAM

CP-OFDM 256 QAM

Considering NR Narrow band Blocking evaluates the receptor characteristics does not seem so critical the selection of the uplink modulations. However, this receiver characteristic is measured by measuring downlink Throughput, due to the HARQ feedback is sent in PUSCH configuration is important to select the right modulation to avoid any issue with uplink (which is near the maximum power).

Observation 7: It is reasonable to use DFT-s-OFDM waveform which presents lower PAPR compared to CP-OFDM to avoid any uplink issues. Besides this kind of waveform is recommended in case of high power level due to it is more power efficient.

Proposal 7: Select DFT-s-OFDM waveform for uplink configuration for Narrow band Blocking measurement in FR1.

As commented in downlink section, LTE Narrow band Blocking and Reference Sensitivity Level test cases use the same values for the uplink configuration table related to modulations and resource allocations. In LTE these test cases use the QPSK modulation and outer allocations (full and partial) in all cases except band 20 where the inner configuration is specified.

Observation 8: To select the uplink modulation it is logical to apply the same criteria used for the waveform selection. These criteria are based on the lowest PAPR (linearity). For these reasons the most appropriated modulation to use is QPSK.

Proposal 8: Select QPSK modulation for uplink configuration for Narrow band Blocking for measurement in FR1.

Applying for the resource allocation selection the same criteria than previously, the uplink configuration for NR Narrow band Blocking should use the same values as specified in the uplink configuration table defined in minimum conformance requirements for NR Sensitivity Reference test case in [2]. Using this configuration, uplink allocations will use outer allocations (full or partial) in all NR bands cases except for n20 band where the inner configuration is specified.

Proposal 9: Narrow band Blocking for measurement in FR1 will use the same uplink resource allocation as NR Sensitivity Level test in FR1, where all allocations are outers (full or partial) for all NR bands except for n20 band where the inner configuration is specified.

Proposal 10: Uplink allocation in test configuration table for Narrow band Blocking measurement in FR1 will include a reference to uplink configuration table for NR Reference Sensitivity in FR1.

Exceptions for Narrow band blocking UL-MIMO test

Extracted from Clause 7.6D of TS 38.101-1:

For UE with two transmitter antenna connectors in closed-loop spatial multiplexing scheme, the minimum requirements specified in subclause 7.6 shall be met with the UL-MIMO configurations described in sub-clause 6.2D.1. For UL-MIMO, the parameter PCMAX_L is defined as the total transmitter power over the two transmit antenna connectors.

Observation 9: Same Narrow band blocking requirements as specified for single-antenna port apply to UE supporting UL-MIMO.

According to [4], CP-OFDM is mandatory in case UL-MIMO transmission with two layer data, therefore DFT-s-OFDM is precluded from the proposal 7 for Narrow band blocking UL-MIMO measurement. Generally receiver test are not sensitive to the UL waveform and modulation configuration, straightforwardly test CP-OFDM instead of DFT-s-OFDM.

Proposal 11: Test CP-OFDM for Narrow band blocking UL-MIMO measurement in FR1. The other test parameters are the same as test for single-antenna port.

Exceptions for Asymmetric CH BWs

Similar to any 1UL/xDL intra-band contiguous CA, asymmetric CH BWs should not have UL impact in Tx requirements. But similarly, it should be tested in Rx requirements.

Proposal 12: From the supported asymmetric CH BWs to a given band defined in clause 5.3.6 of TS 38.521-1, the test CH BW UL/DL combination shall be selected the following way (DL channel bandwidth shall be selected first):

- Lowest UL CH BW / Lowest DL CH BW

- Lowest UL CH BW / Highest DL CH BW

Conclusion

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 Narrow band Blocking test case definition:

Proposal 1: Define Test Environment as Normal Conditions (NC) for Narrow band Blocking measurement in FR1.

Proposal 2: Select the Lowest SCS supported for UE for Narrow band Blocking measurement in FR1.

Proposal 3: Test Mid Frequency for Narrow band Blocking measurement in FR1.

Proposal 4: Test the Lowest, Mid and Highest Channel Bandwidth for Narrow band Blocking measurement in FR1.

Proposal 5: Select CP-OFDM QPSK modulation for downlink configuration for Narrow band Blocking measurement in FR1.

Proposal 6: Select maximum transmission bandwidth configuration, in terms of NRB, for downlink configuration for Narrow band Blocking measurement in FR1.

Proposal 7: Select DFT-s-OFDM waveform for uplink configuration for Narrow band Blocking measurement in FR1.

Proposal 8: Select QPSK modulation for uplink configuration for Narrow band Blocking for measurement in FR1.

Proposal 9: Narrow band Blocking for measurement in FR1 will use the same uplink resource allocation as NR Sensitivity Level test in FR1.

Proposal 10: Uplink allocation in test configuration table for Narrow band Blocking measurement in FR1 will include a reference to uplink configuration table for NR Reference Sensitivity in FR1.

Proposal 11: Test CP-OFDM for Narrow band blocking UL-MIMO measurement in FR1. The other test parameters are the same as test for single-antenna port.

Proposal 12: From the supported asymmetric CH BWs to a given band defined in clause 5.3.6 of TS 38.521-1, the test CH BW UL/DL combination shall be selected the following way (DL channel bandwidth shall be selected first):

- Lowest UL CH BW / Lowest DL CH BW

- Lowest UL CH BW / Highest DL CH BW

References

[1] 3GPP TS 38.101-1 v15.2.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 15)

[2] 3GPP TS 38.521-1 v0.5.1 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone (Release 15)

[3] 3GPP TS 38.211 v15.2.0 Physical channels and modulation (Release 15)

[4] R5-192024, “Discussion on the waveform selection for UL MIMO tests”, Huawei, RAN5#82, Athens, Feb.2019.