Source: 	Anritsu, CAICT

Title: 	Test point analysis for Wide band Intermodulation for FR1

Introduction

As agreed in RAN5#78, each test case would require specific analysis to conform its test configuration table based on the study for Frequency, Channel BW, SCS, RB allocation (including modulations).

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for Wide band Intermodulation in FR1.

Firstly single transmission case is done, then UL-MIMO case is provided with exception test points analysis based on the outcome from comparing the minimum requirements and waveforms allowed.

Discussion

The purpose of the Wide band Intermodulation is as described in TS 38.101-1.

## 7.8	Intermodulation characteristics

### 7.8.1	General

Intermodulation response rejection is a measure of the capability of the receiver to receiver a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal

### 7.8.2	Wide band Intermodulation

The wide band intermodulation requirement is defined using a CW carrier and modulated NR signal as interferer 1 and interferer 2 respectively.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.8.2-1 for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz and Table 7.8.2-2 for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz. The said relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal. For operating bands with an unpaired DL part (as noted in Table 5.5-1), the requirements only apply for carriers assigned in the paired part.

Table 7.8.1-1: Wide band intermodulation parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 6 | 6 | 7 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 15 | 16 |
| PInterferer 1 (CW) |  | dBm |  | -46 |  |  |  |  |  |  |  |  |  |  |
| PInterferer 2 (Modulated) |  | dBm |  | -46 |  |  |  |  |  |  |  |  |  |  |
| BWInterferer 2 |  | MHz |  | 5 |  |  |  |  |  |  |  |  |  |  |
| FInterferer 1 (Offset) |  | MHz |  | -BW/2 – 7.5 / +BW/2 + 7.5 |  |  |  |  |  |  |  |  |  |  |
| FInterferer 2 (Offset) |  | MHz |  | 2*FInterferer 1 |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in TBD. NOTE 2: Reference measurement channel is TBD. NOTE 3: The modulated interferer consists of the Reference measurement channel specified in TBD. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.8.2-2: Wide band intermodulation parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Pw in Transmission Bandwidth Configuration, per CC | dBm |  | REFSENS + 6 |  |  |  |  |  |  |
| PInterferer 1 (CW) | dBm |  | -46 |  |  |  |  |  |  |
| PInterferer 2 (Modulated) | dBm |  | -46 |  |  |  |  |  |  |
| BWInterferer 2 | MHz |  | BW |  |  |  |  |  |  |
| FInterferer 1 (Offset) | MHz |  | -2BW / +2BW |  |  |  |  |  |  |
| FInterferer 2 (Offset) | MHz |  | 2*FInterferer 1 |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in TBD. NOTE 2: Reference measurement channel is TBD. NOTE 3: The modulated interferer consists of the Reference measurement channel specified in TBD. NOTE 4: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |  |  |  |  |  |  |  |

Figure 1 shows the positions of carrier, interferer1(CW), interfere2(modulated) and 3rd order intermodulation product of interfere1 and 2.

Figure 1 Positions of carrier, interfere1, intererer2 and 3rd order intermodulation product
(when Finterferer > Fcarrier)

In LTE, these requirements are tested with below condition.

Table 7.8.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  | NC |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 |  |  | Mid range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, Highest |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |  |
| Ch BW | Mod'n | RB allocation |  | Mod'n | RB allocation |  |
|  |  | FDD | TDD |  | FDD | TDD |
| 1.4MHz | QPSK | 6 | 6 | QPSK | 6 | 6 |
| 3MHz | QPSK | 15 | 15 | QPSK | 15 | 15 |
| 5MHz | QPSK | 25 | 25 | QPSK | 25 | 25 |
| 5MHz | QPSK | 25 | N/A | QPSK | 20 | N/A |
| 5MHz | QPSK | 25 | N/A | QPSK | 15 | N/A |
| 5MHz | QPSK | 25 | N/A | QPSK | 54 | N/A |
| 10MHz | QPSK | 50 | 50 | QPSK | 50 | 50 |
| 10MHz | QPSK | 50 | N/A | QPSK | 25 | N/A |
| 10MHz | QPSK | 50 | N/A | QPSK | 20 | N/A |
| 10MHz | QPSK | 50 | N/A | QPSK | 15 | N/A |
| 15MHz | QPSK | 75 | 75 | QPSK | 75 | 75 |
| 15MHz | QPSK | 75 | N/A | QPSK | 50 | N/A |
| 15MHz | QPSK | 75 | N/A | QPSK | 25 | N/A |
| 15MHz | QPSK | 75 | N/A | QPSK | 20 | N/A |
| 20MHz | QPSK | 100 | 100 | QPSK | 100 | 100 |
| 20MHz | QPSK | 100 | N/A | QPSK | 75 | N/A |
| 20MHz | QPSK | 100 | N/A | QPSK | 50 | N/A |
| 20MHz | QPSK | 100 | N/A | QPSK | 25 | N/A |
| 20MHz | QPSK | 100 | N/A | QPSK | 20 | N/A |
| Note 1: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 2: Depending on E-UTRA band, only the appropriate Uplink RB allocation value according to table 7.3.3-2 is tested per Test Channel Bandwidth. The allocation shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.4.2-1). Note 3: For the DL signal one sided dynamic OCNG Pattern OP.1 FDD/TDD is used. Note 4: Applicable only to E-UTRA FDD Bands 31 and 72. The UL resource blocks shall be located at Restart 10 (according to Table 7.3.3-2). |  |  |  |  |  |  |

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

Test Environment

Based on the baseline agreed in [1], test environment for Wide band Intermodulation test for FR1 can be defined as in LTE.

Proposal 1: Define Test Environment as ‘NC’

Test Subcarrier Spacing

In case the intermodulation product power is the same, the less power of carrier case would be stringent case in terms of signal to intermodulation product power ratio. According to the reference sensitivity requirement TS 38.101-1, slightly small(better) REFSENS value for highest SCS for the same BW. Hence, highest SCS could be chosen.

Proposal 2: Select highest SCS supported

  - Test Frequencies

In LTE, this test is tested only with Mid range. Considering the test purpose is how the intermodulation product due to any non-lineality in receiver circuit, the situation that no filtering applied for interferer signal will be the stringent scenario. Hence, Mid range will be reasonable to select.

Proposal 3: Test Mid frequency

Test Channel Bandwidth

In LTE, this test case is tested for lowest, 5MHz and highest channel bandwidth. As seen in Figure 1, the lowest channel BW may be stringent in terms of signal to intermodulation product power ratio if full DL RB allocation is assumed. Mid and Highest BW is thought as less stringent however for having the same coverage as LTE it is desired to test them as well.

Proposal 4: Test Lowest, Mid and Highest Channel Bandwidth

Downlink Configuration

In LTE, full RB allocation with QPSK is used. For modulation, as the power level is based on the REFSENS for QPSK, QPSK should be used for DL. For RB allocation, seeing the reference measurement channel A.3.2 specified in minimum requirement, full RB allocation needs to be used.

Proposal 5: For DL, full RB allocation with QPSK is applied

Uplink Configuration

In LTE, the all the possible RB allocation listed for UL allocation for reference sensitivity requirements are listed(R5-094914). As long as the DL level is based on REFSENS, this principle can be used as well for NR. As the uplink configuration for REFSENS is specified with QPSK DFT-s-OFDM according to 7.3.2 of 38.101-1, then DFT-s-OFDM QPSK should be used for this test.

Proposal 6: For UL, use the same parameter as used for uplink configuration for REFSES; band dependent UL RB allocation, QPSK DFT-s-OFDM

Exceptions for MaxIL UL-MIMO test

Extracted from Clause 7.8D of TS 38.101-1:

For UE(s) with two transmitter antenna connectors in closed-loop spatial multiplexing scheme, the minimum requirements in subclause 7.8 shall be met with the UL-MIMO configurations described in sub-clause 6.2D.1. For UL-MIMO, the parameter PCMAX_L is defined as the total transmitter power over the two transmit antenna connectors.

Observation 1: Same Wide band Intermodulation requirements as specified for single-antenna port apply to UE supporting UL-MIMO.

According to [3], CP-OFDM is mandatory in case UL-MIMO transmission with two layer data, therefore DFT-s-OFDM is precluded from the proposal 6 for Wide band Intermodulation UL-MIMO measurement. Generally receiver test are not sensitive to the UL waveform and modulation configuration, straightforwardly test CP-OFDM instead of DFT-s-OFDM.

Proposal 7: Test CP-OFDM QPSK for Wide band Intermodulation UL-MIMO measurement in FR1. The other test parameters are the same as test for single-antenna port.

Exceptions for Asymmetric CH BWs

Similar to any 1UL/xDL intra-band contiguous CA, asymmetric CH BWs should not have UL impact in Tx requirements. But similarly, it should be tested in Rx requirements.

Proposal 8: From the supported asymmetric CH BWs to a given band defined in clause 5.3.6 of TS 38.521-1, the test CH BW UL/DL combination shall be selected the following way (DL channel bandwidth shall be selected first):

- Lowest UL CH BW / Lowest DL CH BW

- Lowest UL CH BW / Highest DL CH BW

Conclusion

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 Wideband intermodulation test case definition:

Proposal 1: Define Test Environment as ‘NC’

Proposal 2: Select highest SCS supported

Proposal 3: Test Mid frequency

Proposal 4: Test Lowest, Mid and Highest Channel Bandwidth

Proposal 5: For DL, full RB allocation with QPSK is applied

Proposal 6: For UL, use the same parameter as used for uplink configuration for REFSES; band dependent UL RB allocation, QPSK DFT-s-OFDM

Proposal 7: Test CP-OFDM QPSK for Wide band Intermodulation UL-MIMO measurement in FR1. The other test parameters are the same as test for single-antenna port.

Proposal 8: From the supported asymmetric CH BWs to a given band defined in clause 5.3.6 of TS 38.521-1, the test CH BW UL/DL combination shall be selected the following way (DL channel bandwidth shall be selected first):

- Lowest UL CH BW / Lowest DL CH BW

- Lowest UL CH BW / Highest DL CH BW

References

[1] R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[2] R5-180111, “Discussion of test channel BW and sub-carrier spacing (SCS) approach for NR TRx test cases”, Ericsson, RAN5 Ad hoc, Munich, Jan. 2018.

[3] R5-192024, “Discussion on the waveform selection for UL MIMO tests”, Huawei, RAN5#82, Athens, Feb.2019.