3GPP TSG-RAN WG5 Meeting #96-e                                                       R5-224322

# Electronic Meeting, 15 – 26 August 2022

Source:	Qualcomm Incorporated

Title:	Way Forward for Rel-15 NR Tests Applicability on SNPN Only UE

# Document for:	Agreement

Agenda Item:	6.3.26.6

## 1.	Introduction and Background

Standalone Non-Public networks (SNPN) were introduced in Rel-16 and work is currently commencing in RAN5 for conformance specifications, which are tracking core specifications. There may be a possibility that a UE can be SNPN ONLY UE, i.e. designed to access Standalone Network only [4]. For such UE, we may have to provide coverage for applicable legacy NR (Rel-15) test cases. As per agreed w/f [5], initial scoping was captured in an xls attached with discussion document [6], Chapter 6 [1] test cases were scoped initially. After further discussion [7], below way forward was considered –

- Scoping of “SNPN Only UE” applicable R-15 test to continue for rest of the areas in SIG (RRC, NAS etc).

- General agreement on Proposal 2 i.e create a separate chapter\cl in TS38.523-1, 38.523-2 for “verticals” and add applicable test cases to that clause. However, need to address applicability to ensure legacy R-15 tests are “not” applied.

- As LS [8] was sent to GCF\PTCRB to alert them on RAN5 progress as well seek updates, if any.

- Finally, as per discussion concluded in [11], there were 2 proposals which were considered feasible. In this discussion paper, below proposal#2 is chosen for final implementation as well as final scoping data is provided in attached spreadsheet.

## 2.	Discussion

Proposal#2 – Updated Summary and Implementation

This proposal provides overall solution for reusing the existing test cases, which are applicable to SNPN only UE, with method defined as below –

2.2.1 - In 38.508-1 (e.g. cl.4.5 or subclause within): define the test environment for SNPN only UE as below. CR R5-224336 handles introduction of SNPN ONLY UE test environment

## 4.5B	Legacy common test environment for vertical UEs

### 4.5B.1	System configurations for SNPN

This clause defines the test environment which apply to all legacy test cases listed in subclause TS 38.523-2 [19] 4.3 and is applicable to a SNPN-only UEs (TS 38.508-2 A.4.1-5/3), unless otherwise specified.

- Each configured NR Cell is an SNPN cell, i.e., condition ‘SNPN’ (Standalone NPN cell) applies in the default message contents.

- In addition to the NID of the corresponding NR Cell, the SIB1 of each NR Cell shall also broadcast the NID of every other NR Cell used in the respective test case. The NID is coded as a BITSTRING based on TS 38.508-1 Table 4.4.2-4 and is part of the nid-List-r16 IE.

- UE is provisioned with a “list of subscriber data” to allow access to a SNPN Cell identified by corresponding NR Cell, in each test case.

    - In 38.523-1: Handle the applicable test cases, after scoping, as follows

  - For a legacy TC that is applicable to SNPN-only UE fully:

    - no change

  - For a legacy TC that is applicable to SNPN-only UE partially:

    - add a condition ‘NOT SNPN-only’ around the test steps not applicable to SNPN-only UE. One example is captured in CR R5-224337, TC 6.1.2.1

| 25-31 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] clause 4.9.5 indicate that the UE is camped on NR Cell 11? NOTE 1: The UE performs registration and the RRC connection is released. | - | - | 3 | - |
| --- | --- | --- | --- | --- | --- |
| - | EXCEPTION: Steps 32-34 are not applicable when pc_SNPN_withoutPLMN is set to TRUE. | - | - | - | - |
| 32 | SS change NR Cell 1 SIB1. | - | - | - | - |
| 32A | Wait for 2.1* modification period second for the SS to transmit modified system information (Note 1) |  |  |  |  |
| 33 | The SS changes SS/PBCH EPRE level of NR Cell 1 and 11 according to the row "T4" in table 6.1.2.1.3.2-1/2. | - | - | - | - |
| 34 | Check: Does the UE send an RRCSetupRequest on NR Cell 1 within the next 60 s? | - | - | 4 | F |
| Note 1: The modification period, expressed in number of radio frames = modificationPeriodCoeff * defaultPagingCycle. |  |  |  |  |  |

  - For a legacy TC that is not applicable to SNPN-only UE:

    - no change

    - in 38.523-2, create a new clause and add all applicable test cases which needs to be executed for SNPN-only UE (ex below). This is implemented via CR R5-224338

## 4.3	Protocol conformance legacy test cases applicability for Vertical UEs

Legacy test cases applicable to R-16 SNPN-only UEs (A.4.1-5/3) are listed in Table 4.3-1. The Applicability - Condition of each individual test is as identified in subclause 4.1, with the exception that A.4.1-5/1 is replaced by A.4.1-5/3. Similarly, A.4.3.7-1/24 is replaced by A.4.1-5/3.

Table 4.3-1: Protocol conformance legacy test cases applicable to R-16 SNPN-only UE, ref. TS 38.523-1 [2]

| Clause | Specific ICS | Comment |
| --- | --- | --- |
| 6.1.2.1 |  |  |
| 6.1.2.2 |  |  |

Above is enabled in TTCN implementation by adding a new wrapper function   f_38523_SelectionExpr to call the existing selection expression function. E.g.

//VB: new function called from the control part

function f_38523_SelectionExpr(NR_38523_Condition_Type p_Condition,

boolean  p_TC_ApplicableSNPN_OnlyUE := false    //VB: This parameter will be set to true to TC listed in the table in cl 4.3

) return boolean

{

var boolean v_ApplCond;

if ( not(p_TC_ApplicableSNPN_OnlyUE) and pc_SNPN_OnlyUE) {    //VB: New PICS pc_SNPN_OnlyUE to be added: SNPN-only UEs (A.4.1-5/3)

v_ApplCond := false;

} else {

v_ApplCond := f_SelectionExpr(p_Condition);

}

return v_ApplCond;

}

function f_SelectionExpr(NR_38523_Condition_Type p_Condition  //VB: no change in the existing function

) return boolean

{

var boolean v_ApplCond;

select (p_Condition) {

case (R) {

//R: UEs supporting NR

v_ApplCond := true;

}

case (NR_C01) {

//UEs supporting EN-DC

v_ApplCond := pc_EN_DC;

}

….

}

It is expected Further optionality can be applied when SNPN Only PICS is selected

    - Ensure legacy tests are not picked and SNPN applicable (R-16) tests are picked. To ensue that a new PICS is defined as below in CR R5- 224335

- Table A.4.1-5: 5GS UE Core Technologies

| Item | 5GS UE Core Technologies | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | UE Supports 5G Core Network | 24.501 | Rel-15 | pc_5GCN |  |
| 2 | UE Supports 5G Core Network over non-3GPP Access Network | 24.501, 4.7 | Rel-15 | pc_5GCN_N3AN |  |
| xx | UE Supports Stand-alone Non-Public Network | 23.501, 5.30.2.3 | Rel-16 | pc_SNPN_only | If this PICS is set to true then both pc_5GCN and pc_SNPN shall be set to false. |

## 3.	Proposal/Way Forward

- Implement Proposal#2.2.1-4 as per the examples above and agree on the CR’s mentioned

## 4.	References

[1]	TS 38.523-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; 5GS; User Equipment (UE) conformance specification; Part 1: Protocol

[2]	 TS 36.523-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; EUTRA; User Equipment (UE) conformance specification; Part 1: Protocol

[3]	TS 38.508-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio transmission and reception User Equipment (UE) conformance specification;

[4]	TS 38.300: NR; NR and NG-RAN Overall description; Stage-2; Rel-16 (cl 16.6.1)

[5]	R5-213408: Discussion paper for Rel-15 NR Tests Applicability on SNPN Only UE, RAN5#91e

[6]	R5-214736: Discussion paper for Rel-15 NR Tests Applicability on SNPN Only UE, RAN5#92e

[7]	R5-220837: Discussion paper for Rel-15 NR Tests Applicability on SNPN Only UE, RAN5#94e

[8] 	R5-222032: LS to GCF, PTCRB - Conformance Test Case Development for 3GPP Rel-16 Standalone Non-Public Network (SNPN) and other Verticals UE, RAN5#94e

[9] 	R5-222258	Addition of SNPN only applicability	Qualcomm CDMA Technologies

[10] 	R5-222259	Addition of applicable R15 tests for SNPN-only UE in a new clause	Qualcomm CDMA Technologies

[11] 	R5-223351	Addition of applicable R15 tests for SNPN-only UE in a new clause	Qualcomm CDMA Technologies