3GPP TSG-RAN5 Meeting #96-e	R5-224725

# Electronic Meeting, 15th – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2491 | rev |  | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the SN-FieldLength of PDCP-Config and RLC-Config for RedCap test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1, RedCap UE mandatory supported 12 bits PDCP SN length while 18 bits being optional and mandatory supported 12 bits RLC AM SN length while 18 bits being optional |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Set default sn for PDCH and RLC AM to 12 bits |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RedCap UE which does not support 18 bits PDCP SN or 18 bits RLC SN will fail during the registration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

–	RLC-Config

Table 4.6.3-149: RLC-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-Config ::= CHOICE { |  |  |  |
| am SEQUENCE { |  |  | AM |
| ul-AM-RLC SEQUENCE { |  |  |  |
| sn-FieldLength | size18 |  |  |
|  | size12 |  | pc_RedCap_r17 |
| t-PollRetransmit | ms80 |  | FR1 |
|  | ms30 |  | FR2 |
| pollPDU | p32768 |  |  |
| pollByte | kB750 |  |  |
| maxRetxThreshold | t8 |  |  |
| } |  |  |  |
| dl-AM-RLC SEQUENCE { |  |  |  |
| sn-FieldLength | size18 |  |  |
| t-Reassembly | ms80 |  | FR1 |
|  | ms30 |  | FR2 |
| t-StatusProhibit | ms30 |  |  |
| } |  |  |  |
| } |  |  |  |
| um-Bi-Directional SEQUENCE { |  |  | UM |
| ul-UM-RLC SEQUENCE { |  |  |  |
| sn-FieldLength | size12 |  | pc_um_WIthLongSN |
|  | size6 |  | NOT pc_um_WIthLongSN AND pc_um_WithShortSN |
| } |  |  |  |
| dl-UM-RLC SEQUENCE { |  |  |  |
| sn-FieldLength | size12 |  | pc_um_WIthLongSN |
|  | size6 |  | NOT pc_um_WIthLongSN AND pc_um_WithShortSN |
| t-Reassembly | ms80 |  | FR1 |
|  | ms30 |  | FR2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| AM | RLC AM |
| UM | RLC UM |

<End of modified section 1>

<Start of modified section 2>

–	PDCP-Config

Table 4.6.3-99: PDCP-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config ::= SEQUENCE { |  |  |  |
| drb SEQUENCE { |  |  |  |
| discardTimer | infinity |  |  |
| pdcp-SN-Size-UL | len18bits |  |  |
|  | len12bits |  | pc_RedCap_r17 |
| pdcp-SN-Size-DL | len18bits |  |  |
|  | len12bits |  | pc_RedCap_r17 |
| headerCompression CHOICE { |  |  |  |
| notUsed | NULL |  |  |
| } |  |  |  |
| integrityProtection | Not present |  |  |
| statusReportRequired | true | AM is default |  |
|  | Not present |  | UM |
| outOfOrderDelivery | Not present |  |  |
| } |  |  |  |
| drb | Not present |  | SRB, Split_SRB |
| moreThanOneRLC | Not present |  |  |
| moreThanOneRLC SEQUENCE { |  |  | Split, Split_SRB, NR_split |
| primaryPath SEQUENCE { |  |  |  |
| cellGroup | CellGroupId |  |  |
|  | 0 | MCG path | Split_SRB, NR_split |
| logicalChannel | LogicalChannelIdentity |  |  |
| } |  |  |  |
| ul-DataSplitThreshold | infinity |  |  |
|  | Not present |  | Split_SRB |
| pdcp-Duplication | false |  |  |
|  | Not present | one UL path | Split_SRB |
| } |  |  |  |
| t-Reordering | Not present |  |  |
|  | 80ms |  | UM |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Split | More than one RLC |
| SRB | SRB |
| UM | RLC UM DRB |
| Split_SRB | SRB with more than one RLC |
| NR_split | MCG and split for NR-DC. |

<End of modified section 2>