3GPP TSG-RAN5 Meeting #96-e	R5-225419

# Online, , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5106 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE testcase 8.3.1.13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2022-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current Prose the NOTE1 after Table 8.3.1.13.3.2-1 mentions following: NOTE 1: In the first report UE may not include measResultNeighCells for cell 2. This could be interpreted such that the UE must report measResultNeighCells for cell 2 in second measurement report. This will cause some UEs (like Cat-M) to fal the testcase unfairly because these may take longer to perform measurements and report Cell 2. Besides, the TP does not mandate that the UE should be reporting the cell 2 in first measurement report, or second measurement report. Hence it is proposed to remove NOTE 1 after Table 8.3.1.13.3.2-3. Similarly, it is also proposed o remove the NOTE 2 after the Table 8.3.1.13.3.2-4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the NOTE 1 and NOTE 2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase based on current Prose |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is revision of R5-224163r1 |  |  |  |  |  |  |  |  |

#### 8.3.1.13	Measurement configuration control and reporting / Intra E-UTRAN measurements / Periodic reporting (intra-frequency and inter-band measurements)

8.3.1.13.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state and measurement configured for periodic reporting of intra-frequency cells and inter-band cells }

ensure that {

when { The UE receives reference signal power for cells on the serving frequency and cells on the frequency where measurements are configured }

then { UE sends MeasurementReport message at regular intervals for these cells }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state and a MeasurementReport message for a configured periodic measurement reporting of intra-frequency and inter-band cells was sent }

ensure that {

when { A previously reported cell become unavailable or the UE receives reference signal power on a reported frequency for a cell which was previously not reported }

then { UE sends MeasurementReport message at regular intervals for the available intra-frequency and inter-band cells }

}

(3)

with { UE in E-UTRA RRC_CONNECTED state and periodic measurement reporting ongoing}

ensure that {

when { The UE receives a RRCConnectionReconfiguration message removing measIds for periodic reporting }

then { UE stops sending MeasurementReport messages for these measIds }

}

8.3.1.13.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.331, clauses 5.3.5.3, 5.5.2.2, 5.5.4.1 and 5.5.5.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

...

1>	If the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the Measurement configuration procedure as specified in 5.5.2;

...

[TS 36.331, clause 5.5.2.2]

The UE shall:

1>	for each measId value included in the received measIdToRemoveList that is part of the current UE configuration in varMeasConfig:

2>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	remove the measurement reporting entry for this measId from the VarMeasReportList, if included;

2>	stop the periodical reporting timer or timer T321, whichever one is running, and reset the associated information (e.g. timeToTrigger) for this measId;

[TS 36.331, clause 5.5.4.1]

The UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

…

2>	else:

3>	if the corresponding measObject concerns E-UTRA:

4>	consider any neighbouring cell detected on the associated frequency to be applicable when the concerned cell is not included in the blackCellsToAddModList defined within the VarMeasConfig for this measId;

…

2>	if the purpose is included and set to ‘reportStrongestCells’ or to ‘reportStrongestCellsForSON’ and if a (first) measurement result is available for one or more applicable cells:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

NOTE 1:	If the purpose is set to ‘reportStrongestCells’, the UE initiates a first measurement report immediately after the quantity to be reported becomes available for at least either serving cell or one of the applicable cells. If the purpose is set to ‘reportStrongestCellsForSON’, the UE initiates a first measurement report when it has determined the strongest cells on the associated frequency.

2>	upon expiry of the periodical reporting timer for this measId:

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

…

NOTE 2:	The UE does not stop the periodical reporting with triggerType set to ‘event’ or to ‘periodical’ while the corresponding measurement is not performed due to the serving cell RSRP being equal to or better than s-Measure or due to the measurement gap not being setup.

...

[TS 36.331, clause 5.5.5]

…

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measuredResults within the MeasurementReport message as follows:

1>	set the measId to the measurement identity that triggered the measurement reporting;

1>	set the measResultServCell to include the quantities of serving cell;

1>	if there is at least one applicable neighbouring cell to report:

2>	set the measResultsNeighCells to include the best neighbouring cells up to maxReportCells in accordance with the following:

3>	if the triggerType is set to ‘event’:

4>	include the cells included in the cellsTriggeredList as defined within the VarMeasReportList for this measId;

3>	else:

4>	include the applicable cells for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

NOTE 3:	The reliability of the report (i.e. the certainty it contains the strongest cells on the concerned frequency) depends on the measurement configuration i.e. the reportInterval. The related performance requirements are specified in TS 36.133 [16].

3>	for each cell that is included in the measResultsNeighCells, include the physCellId;

3>	if the triggerType is set to ‘event’; or the purpose is set to ‘reportStrongestCells’ or to ‘reportStrongestCellsForSON’:

4>	for each included cell, include the layer 3 filtered measured results in accordance with the reportConfig for this measId, ordered as follows:

5>	if the measObject associated with this measId concerns E-UTRA:

6>	set the measResult to include the quantity(ies) indicated in the reportQuantity within the concerned reportConfig in order of decreasing triggerQuantity, i.e. the best cell is included first;

…

1>	increment the numberOfReportsSent as defined within the VarMeasReportList for this measId by 1;

1>	stop the periodical reporting timer, if running;

1>	if the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than to reportAmount as defined corresponding reportConfig for this measId:

2>	start the periodical reporting timer with the value of reportInterval as defined within the corresponding reportConfig for this measId;

...

1>	submit the MEASUREMENT REPORT message to lower layers for transmission, upon which the procedure ends.

8.3.1.13.3	Test description

8.3.1.13.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 2, and Cell 10

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) according to [18] on Cell 1.

8.3.1.13.3.2	Test procedure sequence

Table 8.3.1.13.3.2-1 illustrates the downlink power levels to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while rows marked "T1" and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.3.1.13.3.2-1: Power levels

|  | Parameter | Unit | Cell 1 | Cell 2 (DL only) | Cell 10 (DL only) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE (FDD) | dBm/15kHz | -85 | -91 | Off |  |
|  | Cell-specific RS EPRE (TDD) | dBm/15kHz | -85 | -89 | Off |  |
| T1 | Cell-specific RS EPRE (FDD) | dBm/15kHz | -85 | -91 | -85 |  |
|  | Cell-specific RS EPRE (TDD) | dBm/15kHz | -85 | -89 | -85 |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -85 | Off | -85 |  |

Table 8.3.1.13.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS transmits an RRCConnectionReconfiguration message including measConfig to setup intra LTE measurements and periodical reporting for intra and inter-band cells. | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | Void | - | - | - | - |
| - | EXCEPTION: In parallel to events described in step 4 the steps specified in table 8.3.1.13.3.2-3, and the table 8.3.1.13.3.2-4A shall take place |  |  |  |  |
| 4 | Wait for 30 s to ensure that the UE performs a periodical intra-frequency reporting and a periodical inter-band reporting. | - | - | 1 | - |
| 5 | SS sets the cell-specific reference signal levels and switches Cell 10 “On” according to row "T1" in table 8.3.1.13.3.2-1. | - | - | - | - |
| 6 | Wait and ignore MeasurementReport messages for 8s to allow for the switching of cells and the UE measurement. | - | - | - | - |
| - | EXCEPTION: In parallel to events described in step 7 the steps specified in table 8.3.1.13.3.2-3, and table 8.3.1.13.3.2-4 shall take place. |  |  |  |  |
| 7 | Wait for 30 s to ensure that the UE performs a periodical intra-frequency reporting and a periodical inter-band reporting. | - | - | 1, 2 | - |
| 8 | SS sets the cell-specific reference signal levels and switches Cell 2 “Off” according to row "T2" in table 8.3.1.13.3.2-1. | - | - | - | - |
| 9 | Wait and ignore MeasurementReport messages for 20 s to allow for the switching of cells and UE measurement. | - | - | - | - |
| - | EXCEPTION: In parallel to events described in steps 9 to 11 the steps specified in table 8.3.1.13.3.2-3A and the table 8.3.1.13.3.2-4 shall take place | - | - | - | - |
| 10 | Wait for 30 s to ensure that the UE performs a periodical intra-frequency reporting and a periodical inter-band reporting. | - | - | 1, 2 | - |
| 11 | SS transmits an RRCConnectionReconfiguration message including measConfig to remove measIds for periodical reporting. | <-- | RRCConnectionReconfiguration | - | - |
| 12 | The UE transmits an RCConnectionReconfigurationComplete message | --> | RRCConnectionReconfigurationComplete | - | - |
| 13 | Check: Does the UE attempt to transmit an uplink message for the next 10s? | - | - | 3 | F |

Table 8.3.1.13.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: After the 1st message is received, step 1 below shall be repeated every time the duration indicated in the IE reportInterval has elapsed | - | - | - | - |
| 1 | Check: Does the UE transmit a MeasurementReport message to perform periodical intra-frequency reporting for Cell 2? | --> | MeasurementReport | 1 | P |

Table 8.3.1.13.3.2-3A: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: After the 1st message is received, step 1 below shall be repeated every time the duration indicated in the IE reportInterval has elapsed | - | - | - | - |
| 1 | Check: Does the UE transmit a MeasurementReport message to perform periodical intra-frequency reporting configured for cell 2 and without measResultNeighCells for the cell 2? | --> | MeasurementReport | 1 | P |

Table 8.3.1.13.3.2-4: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: After the 1st message is received, step 1 below shall be repeated every time the duration indicated in the IE reportInterval has elapsed. | - | - | - | - |
| 1 | Check: Does the UE transmit a MeasurementReport message to perform periodical inter-band reporting for Cell 10? | --> | MeasurementReport | 1 | P |

Table 8.3.1.13.3.2-4A: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: After the 1st message is received, step 1 below shall be repeated every time the duration indicated in the IE reportInterval has elapsed. | - | - | - | - |
| 1 | Check: Does the UE transmit a MeasurementReport message to perform periodical inter-band reporting configured for cell 10 and without measResultNeighCells for the cell 10? | --> | MeasurementReport | 1 | P |

8.3.1.13.3.3	Specific message contents

Table 8.3.1.13.3.3-0: Conditions for specific message contents
in Tables 8.3.1.13.3.3-2

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.3.1.13.3.3-1: RRCConnectionReconfiguration (step 1 and step 11, Table 8.3.1.13.3.2-2)

Derivation Path: 36.508 clause 4.6.1 table 4.6.1-8 with condition MEAS

Table 8.3.1.13.3.3-2: MeasConfig (step 1, Table 8.3.1.13.3.2-2)

| Derivation Path: 36.508, Table 4.6.6-1, condition INTER-FREQ |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| measObjectId[2] | IdMeasObject-f5 |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f5) |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |
| } |  |  |  |
| reportConfigToAddModiList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | IdReportConfig-PERIODICAL |  |  |
| reportConfig[1] | ReportConfigEUTRA-PERIODICAL |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 2 entries |  |  |
| measId[1] | 1 |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |
| reportConfigId[1] | IdReportConfig-PERIODICAL |  |  |
| measId[2] | 2 |  |  |
| measObjectId[2] | IdMeasObject-f5 |  |  |
| reportConfigId[2] | IdReportConfig-PERIODICAL |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig-GP1 |  | INTER-FREQ |
|  | MeasGapConfig-CE |  | intraFreq-CE-NeedForGaps |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  | Band > 64 |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |
| } |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f5 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.3.1.13.3.3-4: MeasConfig (step 11, Table 8.3.1.13.3.2-2)

| Derivation Path: 36.508, Table 4.6.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measIdToRemoveList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 2 entries |  |  |
| MeasId[1] | 1 |  |  |
| MeasId[2] | 2 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.3.1.13.3.3-5: MeasurementReport (step 1, Table 8.3.1.13.3.2-3)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults ::= SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell ::= SEQUENCE { |  | Report Cell 1 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA ::= SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  | Report Cell 2 |  |
| physCellId [1] | physicalCellIdentity-Cell2 |  |  |
| measResult [1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.3.1.13.3.3-5A: MeasurementReport (step 1, Table 8.3.1.13.3.2-3A)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults ::= SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell ::= SEQUENCE { |  | Report Cell 1 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE {} | Not present |  |  |
| measResultForECID-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.3.1.13.3.3-6: MeasurementReport (step 1, Table 8.3.1.13.3.2-4)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults ::= SEQUENCE { |  |  |  |
| measId | 2 |  |  |
| measResultServCell ::= SEQUENCE { |  | Report Cell 1 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA ::= SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  | Report Cell 10 |  |
| physCellId [1] | physicalCellIdentity-Cell10 |  |  |
| measResult [1] SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.3.1.13.3.3-6A: MeasurementReport (step 1, Table 8.3.1.13.3.2-4A)

| Derivation Path: 36.508 clause 4.6.1 table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults ::= SEQUENCE { |  |  |  |
| measId | 2 |  |  |
| measResultServCell ::= SEQUENCE { |  | Report Cell 1 |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE {} | Not present |  |  |
| measResultForECID-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |