# Title:	Test Tolerances for SSB based CMR and dedicated IMR L1-SINR measurement accuracy tests in NR SA FR2

# Source:	Sporton

# 1. Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for SSB based CMR and dedicated IMR L1-SINR measurement accuracy in NR SA FR2.

The test cases in question, from TS 38.533, are:

- 7.7.6.2, NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurements

The applicability of the analysis is:

- UE Power class 3, f <= 40.8 GHz

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Test case in Annex A of TS 38.133

The Test Requirements are defined in the following extracts from TS 38.133 v16.12.0 Annex A.

<< Extract from TS 38.133 >>

#### A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR

##### A.7.7.6.2.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits. This test will verify the requirements in clauses 9.8.4.2 and clause 10.1.28.2 for L1-SINR measurements with SSB based CMR and CSI-IM based IMR, with the testing configurations for NR cells in Table A.7.7.6.2.1-1.

The AoA setup for this test is Setup 1 as defined in clause A.3.15.

Table A.7.7.6.2.1-1: Applicable NR configurations for FR2 L1-SINR measurement test with SSB based CMR and CSI-IM based IMR

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE FDD, NR 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

##### A.7.7.6.2.2	Test parameters

In this set of test cases there are two cells in the test, PCell (Cell 1). The test parameters for the Cell 1 are given in Table A.7.7.6.2.2-1 and Table A.7.7.6.2.2-2 below. The absolute and relative accuracy of L1-SINR measurements are tested by using the parameters in Table A.7.7.6.2.2-1 and Table A.7.7.6.2.2-2.

Here is no measurement gap configured in the test. Before the test, UE is configured one SSB resource set with two SSB resources and one CSI-IM resource set with two CSI-IM resource. UE is configured to perform RLM and BFD measurement based on the SSB resources 0 and 1. UE is configured to perform L1-SINR measurement based on the SSBs as CMR and the CSI-IM resources as IMR.

Table A.7.7.6.2.2-1: FR2 L1-SINR general test parameters with SSB based CMR and CSI-IM based

| Parameter | Config | Unit | Test 1 |
| --- | --- | --- | --- |
| SSB GSCN | 1~2 |  | freq1 |
| Duplex mode | 1~2 |  | TDD |
| TDD Configuration | 1~2 |  | TDDConf.3.1 |
| BWchannel | 1~2 | MHz | 100: NRB,c = 66 |
| PDSCH Reference measurement channel | 1~2 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1~2 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1~2 |  | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.1 FR2 |
|  | 2 |  | SSB.2 FR2 |
| CSI-IM configuration | 1~2 |  | CSI-IM 3.1 TDD |
| OCNG Patterns | 1~2 |  | OP.1 |
| Initial BWP Configuration | 1~2 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~2 |  | DLBWP.1.3 ULBWP.1.3 |
| TRS Configuration | 1~2 |  | TRS.2.1 TDD |
| PDCCH/PDSCH TCI Configuration | 1~2 |  | TCI.State.2 |
| SMTC configuration | 1~2 |  | SMTC.1 |
| reportConfigType | 1~2 |  | periodic |
| reportQuantity-r16 | 1~2 |  | ssb-Index-SINR-r16 |
| Number of reported RS | 1~2 |  | 2 |
| L1-SINR reporting period | 1~2 |  | slot640 |
| Propagation condition | 1~2 |  | AWGN |
| Antenna configuration | 1~2 |  | 1x2 |
| EPRE ratio of PSS to SSS | 1~2 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. |  |  |  |

Table A.7.7.6.2.2-2: FR2 L1-SINR SSB specific test parameters

| Parameter | Config | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | SSB0 | SSB1 |
| Angle of arrival configuration |  |  | Setup 1 according to A.3.15.1 |  |
| Assumption for UE beamsNote 4 |  |  | Rough |  |
|  | 1~2 | dBm/15kHz | -100 |  |
|  | 1 | dBm/SSB SCS | -91 |  |
|  | 2 |  | -88 |  |
|  | 1~2 | dB | 10 | -2 |
| SS-RSRPNote1 | 1 | dBm/SCS | -81 | -93 |
|  | 2 |  | -78 | -90 |
| IoNote1 | 1~2 | dBm/95.04 MHz | -51.57 | -59.86 |
|  | 1~2 | dB | 10 | -2 |
| Note 1: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 2: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 3: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |

##### A.7.7.6.2.3	Test Requirements

After 640ms from the beginning of the test, the L1-SINR measurement accuracy for SSB#0+CSI-IM#0 and SSB#1+CSI-IM#1 of Cell 1 shall fulfil the requirements in clauses 10.1.28.2. The following requirements are to be verified:

For Test 1:

Absolute accuracy of SSB#0+CSI-IM#0 and absolute accuracy of SSB#1+CSI-IM#1. The UE is deemed to meet the requirement if the reported L1-SINR is in the range shown in Table A.7.7.6.2.3-1.

Relative accuracy of SSB#0+CSI-IM#0 compared with SSB#1+CSI-IM#1. The UE is deemed to meet the requirement if the difference in reported L1-SINR meets the requirements in Table 10.1.28.2.2-2.

Table A.7.7.6.2.3-1: L1-SINR absolute accuracy test requirement

|  | Test requirement Notes1,2 |
| --- | --- |
| SSB#0+CSI-IM#0 | L1_SINR0 -δ ≤ Reported SINR(dB) ≤ L1_SINR0 +δ |
| SSB#1+CSI-IM#1 | L1_SINR1 -δ ≤ Reported SINR(dB) ≤ L1_SINR1 +δ |
| Note 1: L1_SINRn is the equivalent SINR received by an antenna with 0dBi gain at the centre of the quiet zone configured in the test for the SSB#n+CSI-IM#n under consideration Note 2: δ is the SINR absolute accuracy requirement from Table 10.1.28.2.1-2, selected according to the Io used in the test |  |

# 3. Discussion

The test consists of two subtests, each with one cell with two SSBs configured. Subtest 1 has SSB#0 at high power and SSB#1 at lower power, and gets close to the maximum Io and using Noc and Es to give controlled Es/Iot.

After 640ms from the beginning of the test, the L1-SINR measurement accuracy for SSB#0+CSI-IM#0 and SSB#1+CSI-IM#1 of Cell 1 shall fulfil the requirements under these conditions.

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet. Values given are for Configuration 1, and other configurations are covered in section 5.

a) Original specified key parameters

The key parameters are copied from Tables A.7.7.6.2.1-1, A7.7.6.2.2-1 and A7.7.6.2.2-2 in TS 38.133. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test. The angle of arrival of the radiated signals is also recorded.

A number of UE core requirement values and their associated parameters are also entered in this section, taking into account the angle of arrival of the radiated signals. The requirements are entered because they affect the radiated signal received by the UE, or affect how the UE sees the received signals at baseband.

The key parameters appear in section a) of the accompanying spreadsheet.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

Some UE core requirements in section a) are converted into UE-related derived parameters, in a form that allows them to be used in later sections of the spreadsheet. The rationale and method for UE gain G midpoint is given in TR 38.903 clause A.2.6, and for UE Spherical coverage gain midpoint in TR 38.903 clause A.2.1.

c) Uncertainties

The Test system provides 1 cell with AWGN for subtest 1 and without AWGN for subtest 2. We propose to control the following parameters:

Test 1:

- AWGN absolute power on carrier frequency 1, Noc1 averaged over BWConfig uncertainty: ±5.65* dB

- AWGN absolute power on carrier frequency 1, Noc1 for PRBs #RBJ-RBJ+19 uncertainty: ±5.65* dB

- Ratio of signal / AWGN for SSB#0, Ês0 / Noc ±0.3 dB averaged over BWConfig

- Ratio of signal / AWGN, for SSB#0, Ês0 / Noc ±0.3 dB for PRBs #RBJ-RBJ+19

- Ratio of signal / AWGN for SSB#1, Ês1 / Noc ±0.3 dB averaged over BWConfig

- Ratio of signal / AWGN, for SSB#1, Ês1 / Noc ±0.3 dB for PRBs #RBJ-RBJ+19

*These values are applicable for frequencies <= 40.8 GHz.

In this test the UE measures the SS-RSRP over specific Physical Resource Block (PRB) numbers #RBJ to #RBJ+19 for 120kHz SCSSSB, or #RBJ to #RBJ+39 for 240kHz SCSSSB. In addition, this test has separate constraints on the overall power in the configured bandwidth Io.

UE-related uncertainties are considered, together with selected UE core requirements that have an allowed range, and are therefore handled as a UE uncertainty:

- UE L1-SINR absolute accuracy: Value taken from TS 38.133

- UE L1-SINR relative accuracy: Value taken from TS 38.133

- Spherical coverage gain variation: Value derived, using method defined in TR 38.903 A.2.1

- UE gain G variation: Value derived, using method defined in TR 38.903 A.2.6

<< Extracts from TS 38.133 >>

#### 10.1.28.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured

##### 10.1.28.2.1	Absolute Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SSB based L1-SINR in this clause apply to all SSBs configured as CMR and dedicated resources configured as IMR of the serving cell configured for L1-SINR measurement.

The accuracy requirements are defined in Table 10.1.28.2.1-1 for SSB based CMR and NZP-IMR and in Table 10.1.28.2.1-2 for SSB based CMR and ZP-IMR.

The accuracy requirements in Tables 10.1.28.2.1-1 and 10.1.28.2.1-2 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.8.2 for a corresponding Band for each relevant SSB based CMR and IMR.

-	The bandwidth of NZP-IMR and ZP-IMR is 48 PRBs and the density is 3.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.28.2.1-2: L1-SINR absolute accuracy for SSB based CMR and ZP-IMR in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB CMR Ês/Iot Note 3 | Io Note 1 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| ±4.5 | ±5.5 | ≥-3 | Same value as SSB_RP in Table in B.2.8.2, according to UE Power class, operating band and angle of arrival |  | N/A | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB CMR Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

##### 10.1.28.2.2	Relative Accuracy

The relative accuracy of SSB based L1-SINR is defined as the L1-SINR measured from one SSB configured as CMR and one IMR configured as IMR compared to the largest measured value of L1-SINR among all SSB based CMRs and IMRs of the serving cell.

The accuracy requirements are defined in Table 10.1.28.2.2-1 for SSB based CMR and NZP-IMR and in Table 10.1.28.2.2-2 for SSB based CMR and ZP-IMR.

The accuracy requirements in Tables 10.1.28.2.2-1 and 10.1.28.2.2-2 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-2 [19] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.8.2 for a corresponding Band for each relevant SSB based CMR and IMR.

-	The bandwidth of NZP-IMR and ZP-IMR is 48 PRBs and the density is 3.

-	The measured signals are in the directions covered by the percentile EIS spherical coverage of the UE, defined in clause 7.3.4 of TS 38.101-2 [19].

Table 10.1.28.2.2-2: L1-SINR relative accuracy for SSB based CMR and ZP-IMR in FR2

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB CMR Ês/Iot Note 2, Note 4 | Io Note 1 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 3 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| ±[3.5] | ±[4.5] | ≥-3 | Same value as SSB_RP in Table in B.2.8.2, according to UE Power class, operating band and angle of arrival |  | N/A | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB CMR Ês/Iot is the minimum SSB CMR Ês/Iot of the pair of SSBs to which the requirement applies. NOTE 3: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 4: In the test cases, the SSB CMR Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

Table B.2.8.2.1-2: Conditions for L1-SINR measurements with SSB based CMR and ZP-IMR in FR2

| Parameter | Angle of arrival | NR operating bands | Minimum SSB_RP Note 2, Note 3 |  |  |  |  |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | dBm / SCSSSB |  |  |  |  |  | dB |
|  |  |  | SCSSSB = 120 kHz |  |  |  |  | SCSSSB = 240 kHz |  |
|  |  |  | UE Power class |  |  |  |  | UE Power class |  |
|  |  |  | 1 | 2 | 3 | 4 | 5 | 1, 2, 3, 4, 5 |  |
| Conditions | Rx Beam Peak | n257 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 | -120.4+Y5 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 | -120.6+Y5 |  |  |
|  |  | n259 |  |  | -105.5 |  | -117.5+Y5 |  |  |
|  |  | n260 | -122.3+Y1 |  | -106.5 | -122.8+Y4 |  |  |  |
|  |  | n261 | -125.3+Y1 | -110.8 | -109.1 | -124.8+Y4 |  |  |  |
|  | Spherical coverage Note 1 | n257 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 | -112.4+Z5 | (Value for SCSSSB = 120 kHz) +3dB | ≥-3 |
|  |  | n258 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 | -112.6+Z5 |  |  |
|  |  | n259 |  |  | -92.7 |  | -109.5+Z5 |  |  |
|  |  | n260 | -114.3+Z1 |  | -93.9 | -110.8+Z4 |  |  |  |
|  |  | n261 | -117.3+Z1 | -99.8 | -98.2 | -115.8+Z4 |  |  |  |
| NOTE 1: Values based on EIS spherical coverage as defined in clause 7.3.4 of TS 38.101-2 [19]. Side condition applies for directions in which EIS spherical coverage requirement is met. NOTE 2: Values specified at the Reference point to give minimum SSB Ês/Iot, with no applied noise. NOTE 3: For UEs that support multiple FR2 bands, Rx Beam Peak values are increased by ∆MBP,n and Spherical coverage values are increased by ∆MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1 of TS 38.101-2 [19]. |  |  |  |  |  |  |  |  |  |

Normal conditions are selected.

The SSB Ês/Iot side condition is ≥ -3dB in all tables. Band n259 gives the highest SSB_RP side condition of -105.5 dBm/SCSSSB for configuration 1 and -102.5 dBm/SCSSSB for configuration 2. For SSB_RP side condition 1dB multi band relaxation is considered.

d) Controlled parameters critical to verdict

In many RRM test cases there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict. In this test case there are two aspects to consider:

- The uncertainties in the stimulus set by the Test system (Downlink signal)

- The uncertainties in the UE response (L1-SINR reports)

The stimulus set by the Test system should be within the constraints (side conditions) for the UE measurement. For each SSB they are the Es/Iot range, the SSB_RP power range, and the Io power range, over which the UE meets the specified SSB based L1-SINR reporting accuracy.

The 8 controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given in the “Requirement” column of section g) in the spreadsheet.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB. In this RRM test case there is usually a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict. The sensitivity factors are therefore derived by inspection as one or zero.

For example, an error of 1dB in the absolute AWGN level Noc would cause 1dB error in the SSB#0 SSB_RP, so the sensitivity factor is 1.000. However, the same error of 1dB in the absolute AWGN level Noc would cause no change to SSB#0 Es/Iot, because it is specified relative to Noc, so the sensitivity factor is zero.

The overall power Io is an exception, as the effect of Es/Noc uncertainty is diluted depending on the linear fraction of SSB power making up the total Io. The sensitivity factor is made equal to the linear fraction of power which was calculated considering the RB allocation.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, considering the uncertainties and sensitivity factors for each parameter that can be set by the test equipment and the UE L1-SINR relative measurement accuracy where relevant. This process follows the superposition principle. More details and explanation can be found in section 4 of TS 38.903.

The test system uncertainties are combined root-sum-square as they are uncorrelated. UE-related values have been added arithmetically, as each could be systematic not random. More detailed explanation is given in the accompanying spreadsheet, and in TR 38.903 A.2.7.

e) Determine parameters to offset

During Test 1, the variation due to uncertainties would make Io greater than the -50dBm side condition, and the UE measurements may not be valid. The AWGN is therefore reduced by 4.1dB to meet the side condition. The variation due to uncertainties would make the SSB_RP for SSB#1 lesser than the -3.00dB SSB Es/Iot side condition. The SSB_RP for SSB#1 has been increased by 0.2dB to meet the side condition.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, modified by the Test Tolerances (applied offsets).

Re-derived parameters are calculated using the same methods as were used in step b).

g) Check controlled parameters Min/Max

Using a format similar to that in step d), the nominal value of each controlled parameter is recalculated, as some will have changed from the original due to the application of the Test Tolerances in step f).

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements (Es/IotBB, Applied SSB_RP and Applied Io). The critical requirement for each parameter is given in the “Requirement” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all the requirements are met, then the chosen offsets are acceptable. It can be seen that with the uncertainty values and Test Tolerances proposed, the requirements are met.

For this test, the verdict is based on the SS-RSRP values reported by the UE. As stated earlier, the reported values are affected by:

- The uncertainties in the stimulus set by the Test system (Downlink signal)

- The uncertainties in the UE response (SS-RSRP reports)

Having ensured that the stimulus set by the Test system remains within the side conditions, and knowing its uncertainty, we now need to calculate the range of SS-RSRP values that a conformant UE could report. The mapping is defined in TS 38.133:

### 10.1.16.1 SS-SINR and CSI-SINR measurement report mapping

The reporting range of SS-SINR and CSI-SINR for L3 reporting and L1 reporing is defined from -23 dB to 40 dB with 0.5 dB resolution. The mapping of measured quantity is defined in Table 10.1.16.1-1. The range in the signalling may be larger than the guaranteed accuracy range.

The reporting range of differential SS-SINR and CSI-SINR for L1 reporting and L3 reporting is defined from -15 dB to 0 dB with 1 dB resolution.

The mapping of measured quantity is defined in Table 10.1.16.1-2. The range in the signalling may be larger than the guaranteed accuracy range.

Table 10.1.16.1-1: SS-SINR and CSI-SINR measurement report mapping

| Reported value | Measured quantity value (L3 SS-SINR and L3 CSI-SINR) | Measured quantity value (L1 SS-SINR and L1 CSI-SINR) | Unit |
| --- | --- | --- | --- |
| SINR_0 | SINR<-23 | SINR<-23 | dB |
| SINR_1 | -23≤ SINR<-22.5 | -23≤SINR<-22.5 | dB |
| SINR_2 | -22.5≤ SINR<-22 | -22.5≤SINR<-22 | dB |
| SINR_3 | -22≤ SINR<-21.5 | -22≤SINR<-21.5 | dB |
| SINR_4 | -21.5≤ SINR<-21 | -21.5≤SINR<-21 | dB |
| .. | .. | .. | … |
| SINR_123 | 38≤ SINR<38.5 | 38≤SINR<38.5 | dB |
| SINR_124 | 38.5≤ SINR<39 | 38.5≤SINR<39 | dB |
| SINR_125 | 39≤ SINR<39.5 | 39≤SINR<39.5 | dB |
| SINR_126 | 39.5≤ SINR<40 | 39.5≤SINR<40 | dB |
| SINR_127 | 40≤ SINR | 40≤SINR | dB |

Table 10.1.16.1-2: Differential SS-SINR and CSI-SINR measurement (for L1 reporting and L3 reporting) report mapping

| Reported value | Measured quantity value (difference in measured SINR from largest SINR) | Unit |
| --- | --- | --- |
| DIFFSINR_0 | 0≥ΔSINR>-1 | dB |
| DIFFSINR_1 | -1≥ΔSINR>-2 | dB |
| DIFFSINR_2 | -2≥ΔSINR>-3 | dB |
| DIFFSINR_3 | -3≥ΔSINR>-4 | dB |
| DIFFSINR_4 | -4≥ΔSINR>-5 | dB |
| DIFFSINR_5 | -5≥ΔSINR>-6 | dB |
| DIFFSINR_6 | -6≥ΔSINR>-7 | dB |
| DIFFSINR_7 | -7≥ΔSINR>-8 | dB |
| DIFFSINR_8 | -8≥ΔSINR>-9 | dB |
| DIFFSINR_9 | -9≥ΔSINR>-10 | dB |
| DIFFSINR_10 | -10≥ΔSINR>-11 | dB |
| DIFFSINR_11 | -11≥ΔSINR>-12 | dB |
| DIFFSINR_12 | -12≥ΔSINR>-13 | dB |
| DIFFSINR_13 | -13≥ΔSINR>-14 | dB |
| DIFFSINR_14 | -14≥ΔSINR>-15 | dB |
| DIFFSINR_15 | -15≥ΔSINR | dB |

The acceptable range of UE reported SS-SINR values is calculated taking into account the uncertainties in the stimulus set by the Test system, the variations and uncertainties in the UE response, and the UE measurement report mapping function. The calculation is done in section g) of the accompanying spreadsheet.

SS-SINR values calculated are for normal conditions. For extreme conditions, add 3dB extra at each end.

Absolute L1-RSRP reported values

The rationale and method is given in TR 38.903 clause A.2.6.2.

Relative L1-RSRP reported values between two cells

The rationale and method is given in TR 38.903 clause A.2.6.6.

# 5. Treatment of other configurations within the same test case

TS 38.533 test cases 7.7.6.2 contains configurations with different SSB subcarrier spacings. As the calculations use different values, a separate spreadsheet tab is provided. The following parameters are changed, shown as shaded cells on both tabs of the spreadsheet:

- SSB Subcarrier spacing, section a)

- Uncertainties over specific PRBs #RBJ-RBJ+39, section c)

- Cell 1 and Cell 2 SSB_RP minimum requirement, section g) and associated Source/Reference