3GPP TSG-RAN5 Meeting #97	R5-226391

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5134 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE measurement testcase 8.3.1.3a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test, LTE_MTCe2_L1-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At Step 3A, systemInfoValueTag IE in SIB1 is increased to indicate modification of SIB3, but systemInfoValueTagList_r13 IE is not updated to indicate the same for CAT-M1 UEs. The value systemInfoValueTagList_r13 IE needs to be increased, to indicate modification of SIB3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The value of systemInfoValueTagList_r13[k] is increased. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant CAT-M1 UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.1.3a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated with R5s221154 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

#### 8.3.1.3a	Measurement configuration control and reporting / Intra E-UTRAN measurements / Two simultaneous events A3 (intra and inter-frequency measurements) / RSRQ based measurements

8.3.1.3a.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_CONNECTED state, measurements configured for two event A3 at the same time and triggerQuantity set to rsrq }

ensure that {

when { Entry condition for event A3 is not met }

then { UE does not send MeasurementReport }

}

(2)

with { UE in E-UTRA RRC_CONNECTED state, measurements configured for two event A3 at the same time and triggerQuantity set to rsrq }

ensure that {

when { Neighbour becomes offset better than serving }

then { UE sends MeasurementReport with correct measId for event A3 }

}

8.3.1.3a.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.3.5.3, 5.5.4.1, 5.5.4.4 and 5.5.5.

[TS 36.331, clause 5.3.5.3]

If the RRCConnectionReconfiguration message does not include the mobilityControlInfo and the UE is able to comply with the configuration included in this message, the UE shall:

...

1>	if the RRCConnectionReconfiguration message includes the measConfig:

2>	perform the measurement configuration procedure as specified in 5.5.2;

[TS 36.331, clause 5.5.4.1]

The UE shall:

1>	for each measId included in the measIdList within VarMeasConfig:

2>	if the corresponding reportConfig includes a purpose set to 'reportStrongestCellsForSON':

3>	consider any neighbouring cell detected on the associated frequency to be applicable;

2>	else if the corresponding reportConfig includes a purpose set to 'reportCGI':

3>	consider any neighbouring cell detected on the associated frequency/ set of frequencies (GERAN) which has a physical cell identity matching the value of the cellForWhichToReportCGI included in the corresponding measObject within the VarMeasConfig to be applicable;

2>	else:

3>	if the corresponding measObject concerns E-UTRA:

4>	if the ue-RxTxTimeDiffPeriodical, eventA1 or eventA2 is configured in the corresponding reportConfig:

5>	consider only the serving cell to be applicable;

4>	else:

5>	consider any neighbouring cell detected on the associated frequency to be applicable when the concerned cell is not included in the blackCellsToAddModList defined within the VarMeasConfig for this measId;

3>	else if the corresponding measObject concerns UTRA or CDMA2000:

4>	consider a neighbouring cell on the associated frequency to be applicable when the concerned cell is included in the cellsToAddModList defined within the VarMeasConfig for this measId (i.e. the cell is included in the white-list);

NOTE 0:	The UE may also consider a neighbouring cell on the associated UTRA frequency to be applicable when the concerned cell is included in the csg-allowedReportingCells within the VarMeasConfig for this measId, if configured in the corresponding measObjectUTRA (i.e. the cell is included in the range of physical cell identities for which reporting is allowed).

3>	else if the corresponding measObject concerns GERAN:

4>	consider a neighbouring cell on the associated set of frequencies to be applicable when the concerned cell matches the ncc-Permitted defined within the VarMeasConfig for this measId;

2>	if the triggerType is set to 'event' and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include an measurement reporting entry for this measId (a first cell triggers the event):

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the triggerType is set to 'event' and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable cells not included in the cellsTriggeredList for all measurements after layer 3 filtering taken during timeToTrigger defined for this event within the VarMeasConfig (a subsequent cell triggers the event):

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	include the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>		if the triggerType is set to 'event' and if the leaving condition applicable for this event is fulfilled for one or more of the cells included in the cellsTriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event:

3>	remove the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if reportOnLeave is set to TRUE for the corresponding reporting configuration:

4>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	if the cellsTriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running;

2>	if the purpose is included and set to 'reportStrongestCells' or to 'reportStrongestCellsForSON' and if a (first) measurement result is available for one or more applicable cells:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

NOTE 1:	If the purpose is set to 'reportStrongestCells', the UE initiates a first measurement report immediately after the quantity to be reported becomes available for at least either serving cell or one of the applicable cells. If the purpose is set to 'reportStrongestCellsForSON', the UE initiates a first measurement report when it has determined the strongest cells on the associated frequency.

2>	upon expiry of the periodical reporting timer for this measId:

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	if the purpose is included and set to 'reportCGI' and if the UE acquired the information needed to set all fields of cgi-Info for the requested cell:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	stop timer T321;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

2>	upon expiry of the T321 for this measId:

3>	include a measurement reporting entry within the VarMeasReportList for this measId;

3>	set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0;

3>	initiate the measurement reporting procedure, as specified in 5.5.5;

NOTE 2:	The UE does not stop the periodical reporting with triggerType set to 'event' or to 'periodical' while the corresponding measurement is not performed due to the serving cell RSRP being equal to or better than s-Measure or due to the measurement gap not being setup.

NOTE 3:	If the UE is configured with DRX, the UE may delay the measurement reporting for event triggered and periodical triggered measurements until the Active Time, which is defined in TS 36.321 [6].

[TS 36.331, clause 5.5.4.4]

The UE shall:

1>	consider the entering condition for this event to be satisfied when condition A3-1, as specified below, is fulfilled;

1>	consider the leaving condition for this event to be satisfied when condition A3-2, as specified below, is fulfilled;

Inequality A3-1 (Entering condition):

Inequality A3-2 (Leaving condition):

The variables in the formula are defined as follows:

Mn is the measurement result of the neighbouring cell, not taking into account any offsets.

Ofn is the frequency specific offset of the frequency of the neighbour cell (i.e. offsetFreq as defined within measObjectEUTRA corresponding to the frequency of the neighbour cell).

Ocn is the cell specific offset of the neighbour cell (i.e. cellIndividualOffset as defined within measObjectEUTRA corresponding to the frequency of the neighbour cell), and set to zero if not configured for the neighbour cell.

Ms is the measurement result of the serving cell, not taking into account any offsets.

Ofs is the frequency specific offset of the serving frequency (i.e. offsetFreq as defined within measObjectEUTRA corresponding to the serving frequency).

Ocs is the cell specific offset of the serving cell (i.e. cellIndividualOffset as defined within measObjectEUTRA corresponding to the serving frequency), and is set to zero if not configured for the serving cell.

Hys is the hysteresis parameter for this event (i.e. hysteresis as defined within reportConfigEUTRA for this event).

Off is the offset parameter for this event (i.e. a3-Offset as defined within reportConfigEUTRA for this event).

Mn, Ms are expressed in dBm in case of RSRP, or in dB in case of RSRQ.

Ofn, Ocn, Ofs, Ocs, Hys, Off are expressed in dB.

[TS 36.331, clause 5.5.5]

Figure 5.5.5-1: Measurement reporting

The purpose of this procedure is to transfer measurement results from the UE to E-UTRAN.

For the measId for which the measurement reporting procedure was triggered, the UE shall set the measResults within the MeasurementReport message as follows:

1>	set the measId to the measurement identity that triggered the measurement reporting;

1>	set the measResultServCell to include the quantities of serving cell;

1>	if there is at least one applicable neighbouring cell to report:

2>	set the measResultNeighCells to include the best neighbouring cells up to maxReportCells in accordance with the following:

3>	if the triggerType is set to 'event':

4>	include the cells included in the cellsTriggeredList as defined within the VarMeasReportList for this measId;

3>	else:

4>	include the applicable cells for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset;

NOTE:	The reliability of the report (i.e. the certainty it contains the strongest cells on the concerned frequency) depends on the measurement configuration i.e. the reportInterval. The related performance requirements are specified in TS 36.133 [16].

3>	for each cell that is included in the measResultNeighCells, include the physCellId;

3>	if the triggerType is set to 'event'; or the purpose is set to 'reportStrongestCells' or to 'reportStrongestCellsForSON':

4>	for each included cell, include the layer 3 filtered measured results in accordance with the reportConfig for this measId, ordered as follows:

5>	if the measObject associated with this measId concerns E-UTRA:

6>	set the measResult to include the quantity(ies) indicated in the reportQuantity within the concerned reportConfig in order of decreasing triggerQuantity, i.e. the best cell is included first;

5>	else:

6>	set the measResult to the quantity as configured for the concerned RAT within the quantityConfig in order of either decreasing quantity for UTRA and GERAN or increasing quantity for CDMA2000 pilotStrength, i.e. the best cell is included first;

3>	else if the purpose is set to 'reportCGI':

4>	if the mandatory present fields of the cgi-Info for the cell indicated by the cellForWhichToReportCGI in the associated measObject have been obtained:

5>	if the cell broadcasts a CSG identity:

6>	include the csg-Identity;

6>	include the csg-MemberStatus and set it to 'member' if the cell is a CSG member cell;

5>	if the 'si-RequestForHO' is configured within the reportConfig associated with this measId:

6>	include the cgi-Info containing all the fields that have been successfully acquired, except for the plmn-IdentityList;

5>	else:

6>	include the cgi-Info containing all the fields that have been successfully acquired;

1>	if the ue-RxTxTimeDiffPeriodical is configured within the corresponding reportConfig for this measId;

2>	set the ue-RxTxTimeDiffResult to the measurement result provided by lower layers;

2>	set the currentSFN;

1>	increment the numberOfReportsSent as defined within the VarMeasReportList for this measId by 1;

1>	stop the periodical reporting timer, if running;

1>	if the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than the reportAmount as defined within the corresponding reportConfig for this measId:

2>	start the periodical reporting timer with the value of reportInterval as defined within the corresponding reportConfig for this measId;

1>	else:

2>	if the triggerType is set to 'periodical':

3> remove the entry within the VarMeasReportList for this measId;

3> remove this measId from the measIdList within VarMeasConfig;

1>	if the measured results are for CDMA2000 HRPD:

2>	set the preRegistrationStatusHRPD to the UE's CDMA2000 upper layer's HRPD preRegistrationStatus;

1>	if the measured results are for CDMA2000 1xRTT:

2>	set the preRegistrationStatusHRPD to 'FALSE';

1>	submit the MeasurementReport message to lower layers for transmission, upon which the procedure ends;

8.3.1.3a.3	Test description

8.3.1.3a.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 2 and Cell 3

-	System information combination 3 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None.

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.3.1.3a.3.2	Test procedure sequence

Table 8.3.1.3a.3.2-1 illustrates the downlink power levels to be applied for Cell 1, Cell 2 and Cell 3 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while rows marked "T1"and "T2" are to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.3.1.3a.3.2-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Cell 1 | Cell 2 (DL only) | Cell 3 (DL only) | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific RS EPRE | dBm/15kHz | -85 | -91 | "Off" | Power levels are such that entry condition for event A3 (measId 1 & 2) is not satisfied: |
|  | RSRQ | dB | -8 | -14 | - |  |
|  | Noc | dBm/15kHz | -90 | -90 | -100 |  |
| T1 | Cell-specific RS EPRE | dBm/15kHz | -91 | -85 | "Off" | Power levels are such that entry condition for event A3 (measId 1) is satisfied: |
|  | RSRQ | dB | -14 | -8 | - |  |
| T2 | Cell-specific RS EPRE | dBm/15kHz | -91 | "Off" | -85 | Power levels are such that entry condition for event A3 (measId 2) is satisfied: |
|  | RSRQ | dB | -12.33 | - | -3.76 |  |
| NOTE 1: Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1. |  |  |  |  |  |  |

Table 8.3.1.3a.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCConnectionReconfiguration message on Cell 1 including MeasConfig to setup intra E-UTRAN measurement and reporting for two event A3 (measId 1 and measId 2) (intra and inter frequency measurement). | <-- | RRCConnectionReconfiguration | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message on Cell 1. | --> | RRCConnectionReconfigurationComplete | - | - |
| 3 | Check: Does the UE transmit a MeasurementReport message on Cell 1 within the next 10s? | --> | MeasurementReport | 1 | F |
| 3A | The SS notifies the UE of change of System Information and changes the system information 3. The systemInfoValueTag in the SystemInformationBlockType1 is increased. | <-- | Paging | - | - |
| 4 | The SS re-adjusts the cell-specific reference signal level according to row "T1" in table 8.3.1.3a.3.2-1. | - | - | - | - |
| 5 | Check: Does the UE transmit a MeasurementReport message on Cell 1 to report event A3 (measId 1) with the measured RSRP and RSRQ values for Cell 2? | --> | MeasurementReport | 2 | P |
| - | EXCEPTION: Steps 6a1 and 6a2 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. | - | - | - | - |
| 6a1 | IF pc_FeatrGrp_25 THEN the SS re-adjusts the cell-specific reference signal level according to row "T2" in table 8.3.1.3a.3.2-1. | - | - | - | - |
| 6a2 | Check: Does the UE transmit a MeasurementReport message on Cell 1 to report event A3 (measId 2) with the measured RSRP and RSRQ values for Cell 3? | --> | MeasurementReport | 2 | P |
| 7 | Void | - | - | - | - |

8.3.1.3a.3.3	Specific message contents

Table 8.3.1.3a.3.3-0: Conditions for specific message contents
in Table 8.3.1.3a.3.3-2

| Condition | Explanation |
| --- | --- |
| Band > 64 | If band > 64 is selected |

Table 8.3.1.3a.3.3-1: RRCConnectionReconfiguration (step 1, Table 8.3.1.3a.3.2-2)

Derivation Path: 36.508, Table 4.6.1-8, condition MEAS

Table 8.3.1.3a.3.3-2: MeasConfig (Table 8.3.1.3a.3.3-1)

| Derivation Path: 36.508, Table 4.6.6-1, condition INTER-FREQ |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| MeasConfig ::= SEQUENCE { |  |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(f1) |  |  |  |
| measObject[1] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |  |
| measObjectId[2] | IdMeasObject-f2 |  |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(f2) |  |  |  |
| measObject[2] | MeasObjectEUTRA-GENERIC(maxEARFCN) |  | Band > 64 |  |
| } |  |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |  |
| reportConfig[1] | ReportConfigEUTRA-A3-RSRQ |  |  |  |
| } |  |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxMeasId)) OF SEQUENCE { | 2 entries |  |  |  |
| measId[1] | 1 |  |  |  |
| measObjectId[1] | IdMeasObject-f1 |  |  |  |
| reportConfigId[1] | IdReportConfig-A3 |  |  |  |
| measId[2] | 2 |  |  |  |
| measObjectId[2] | IdMeasObject-f2 |  |  |  |
| reportConfigId[2] | IdReportConfig-A3 |  |  |  |
| } |  |  |  |  |
| measGapConfig | MeasGapConfig-GP1 |  |  | INTER-FREQ |
|  | MeasGapConfig-CE |  |  | intraFreq-CE-NeedForGaps |
| measObjectToAddModList-v9e0 ::= SEQUENCE (SIZE (1..maxObjectId)) OF SEQUENCE { | 2 entries |  | Band > 64 |  |
| measObjectEUTRA-v9e0[1] SEQUENCE { |  |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f1 |  |  |  |
| } |  |  |  |  |
| measObjectEUTRA-v9e0[2] SEQUENCE { |  |  |  |  |
| carrierFreq-v9e0 | Same downlink EARFCN as used for f2 |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.3.1.3a.3.3-3: ReportConfigEUTRA-A3-RSRQ (Table 8.3.1.3a.3.3-2)

| Derivation Path: 36.508, Table 4.6.6-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigEUTRA-A3 ::= SEQUENCE { |  |  |  |
| triggerQuantity | rsrq |  |  |
| si-RequestForHO-r9 | Not present |  |  |
| ue-RxTxTimeDiffPeriodical-r9 | Not present |  |  |
| } |  |  |  |

Table 8.3.1.3a.3.3-3A: SystemInformationBlockType3 for Cell 1 (Step 3A, Table8.3.1.3a.3.2-2)

| Derivation path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| lateNonCriticalExtension { |  |  |  |
| q-QualMin-r9 | -30 dB |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.3.1.3a.3.3-3B: SystemInformationBlockType1 for Cell 1 (Step 3A, Table 8.3.1.3a.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 | Previous value is 0 |  |
| } |  |  |  |

Table 8.3.1.3a.3.3-3C: SystemInformationBlockType1-BR-r13 for Cell 1 (Step 3A when UE under test is CAT M1, Table 8.3.1.3a.3.2-2)

| Derivation Path: 36.508 table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| systemInfoValueTag | 1 | Previous value is 0 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| bandwidthReducedAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| systemInfoValueTagList-r13 SEQUENCE (SIZE (1..maxSI-Message)) OF | 3 entries | The same number of entries, and listed in the same order, as in SchedulingInfoList (without suffix) |  |
| SystemInfoValueTagSI-r13[1] | SystemInfoValueTagSI-r13[k]=1, where k is the entry corresponding to the system info including SystemInformationBlockType3. For all other entries the value is set to 0 (same as in the default SystemInformationBlockType1-BR-r13 message). |  |  |
| SystemInfoValueTagSI-r13[2] |  |  |  |
| SystemInfoValueTagSI-r13[3] |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.3.1.3a.3.3-4: MeasurementReport (step 5, Table 8.3.1.3a.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId | PhysicalCellIdentity of Cell 2 |  |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.3.1.3a.3.3-5: MeasurementReport (step 6a2, Table 8.3.1.3a.3.2-2)

| Derivation Path: 36.508, Table 4.6.1-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| measurementReport-r8 SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 2 |  |  |
| measResultServCell SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { | 1 entry |  |  |
| physCellId | PhysicalCellIdentity of Cell 3 |  |  |
| cgi-Info | Not present |  |  |
| measResult SEQUENCE { |  |  |  |
| rsrpResult | (0..97) |  |  |
| rsrqResult | (0..34) |  |  |
| additionalSI-Info-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForECID-r9 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.3.1.3a.3.3-6: Paging (step 3A, Table 8.3.1.3a.3.2-2)

| Derivation path: 36.508 Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| systemInfoModification | true |  |  |
| } |  |  |  |

<End of modified section>