3GPP TSG-RAN WG5 Meeting #97	R5-226581

# Toulouse, France, 14th – 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1457 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 7.3B.2 cleaning up_R15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Korea |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | DC combos marked as “Pending” in PRD21 shall be removed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Deleted: DC_28A_n51A DC_3A_n51A DC_46A_n78A DC_20A_n77A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not be aligned with the core specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3B.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

<< Unchanged sections skipped>>

### 7.3B.2	Reference sensitivity for EN-DC

#### 7.3B.2.0	Minimum Conformance Requirements of Reference sensitivity for EN-DC

##### 7.3B.2.0.1	Intra-band contiguous EN-DC

For intra-band contiguous EN-DC configurations, the reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports at which the throughput for the carrier(s) of the E-UTRA and NR CGs shall meet or exceed the requirements for the specified E-UTRA and NR reference measurement channels. The reference sensitivity requirements apply with all uplink carriers and all downlink carriers active for EN-DC configuration and Uplink EN-DC configuration listed in Table 5.5B.2-1 and Table 5.5B.3-1, as supported by the UE. For EN-DC configurations where uplink is not available in either the MCG or the SCG or for EN-DC configurations where the UE only supports single uplink operation, reference sensitivity requirements apply with single uplink transmission. The downlink carrier(s) from the cell group with uplink shall be configured closer to the uplink operating band than any of the downlink carriers from the cell group without uplink.

Sensitivity degradation is allowed for Intra-band contiguous EN-DC configurations listed in Table 7.3B.2.0.1-1, the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3B.2.0.1-1 and E-UTRA and NR single carrier requirements do not apply.

Table 7.3B.2.0.1-1: Reference sensitivity (MSD) for intra-band contiguous EN-DC

| EN-DC configuration/channel allocations/MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)71AA | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | 0 |  |
|  | n71 | 675.5 | 15 | 15 (RBstart = 0) | 629.5 | 1.8 |  |
| DC_(n)71AA | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | 0 |  |
|  | n71 | 680.5 | 5 | 5 (RBstart = 0) | 634.5 | 1.6 | FDD |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBend = 49) | 622 | 0 |  |
|  | n71 | 678 | 10 | 10 (RBstart = 0) | 632 | 1.7 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 |  |
|  | n71 | 678 | 10 | 10 (RBend = 51) | 632 | 29.4 |  |
| DC_(n)71AA | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | 0 |  |
|  | n71 | 675.5 | 151 | 15 (RBstart = 0) | 6321 | 2.5 |  |
| DC_(n)71AA | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | 0 |  |
|  | n71 | 680.5 | 51 | 5 (RBstart = 0) | 6371 | 2.2 | FDD |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBend = 49) | 622 | 0 |  |
|  | n71 | 678 | 101 | 10 (RBstart = 0) | 634.51 | 2.5 |  |
| DC_(n)71AA | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 |  |
|  | n71 | 678 | 101 | 10 (RBend = 51) | 634.51 | 29.1 |  |
| NOTE 1: In accordance to BCS1, the NR uplink bandwidth is specified as in this table, but the corresponding NR downlink bandwidth is 5 MHz larger. NOTE 2: The transmitters powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [5], with additional limits on configured maximum output power for the uplink according to subclause 6.2B.4. |  |  |  |  |  |  |  |

##### 7.3B.2.0.2	Intra-band non-contiguous EN-DC

For intra-band non-contiguous EN-DC configurations, the reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports at which the throughput for the carrier(s) of the E-UTRA and NR CGs shall meet or exceed the requirements for the specified E-UTRA and NR reference measurement channels.

For DC_3A_n3A intra-band non-contiguous EN-DC combination, only single switched UL is supported in rel.15, therefore, no MSD is specified and E-UTRA and NR single carrier requirements apply.

##### 7.3B.2.0.3	Inter-band EN-DC within FR1

Reference sensitivity exceptions are specified for the condition when there is uplink transmission only in the aggressor band.

7.3B.2.0.3.1	Reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by UL harmonic interference from another band part of the same EN-DC configuration. Reference sensitivity exceptions for the victim band (high) are specified in Table 7.3B.2.0.3.1-1 with uplink configuration of the aggressor band (low) specified in Table 7.3B.2.0.3.1-2.

Table 7.3B.2.0.3.1-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 1, 3 | n772, 13 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| 2 | n772, 13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.5 | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 2 | n782, 13 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| 3 | n782, 13 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| 5 | n786,7 |  | 10.5 | 8.9 | 7.8 |  |  | 5.4 | 4.2 | 3.5 |  | 2.3 | 2.1 | 1.4 |
| 5 | n776, 7, 17 |  | 10.5 | 8.9 | 7.8 | 7.2 | 6.5 | 5.1 | 4.2 | 3.5 | 2.8 | 2.3 | 2.1 | 1.4 |
|  | n774, 5, 17 |  | 10.4 | 8.9 | 7.8 | 6.7 | 6 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| 8 | n418,9 |  | 13 | 11.3 | 10.1 |  |  | 7.0 | 6.1 | 5.5 |  | 4.3 | 3.9 | 3.5 |
| 8 | n776,7 n786,7 |  | 10.8 | 9.1 | 8 |  |  | 5.1 | 4.2 | 3.5 |  | 2.3 | 2.1 | 1.4 |
| 8 | n794,5 |  |  |  |  |  |  | 6.8 | 6.2 | 5.6 |  | 4.9 |  | 4.4 |
| 12 | n668,9,10 | 10 | 7.5 | 6.2 | 5.5 |  |  | 2.4 |  |  |  |  |  |  |
| 12 | n784,5 |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 |  | 1.7 | 1.2 | 0.7 |
| 13 | n774, 5 |  | 10.4 | 8.9 | 7.8 | 6.7 | 5.7 | 4.7 | 3.7 | 3 | 2.3 | 1.7 | 1.2 | 0.7 |
| 18, 19 | n774,5 n784,5 |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 |  | 1.7 | 1.2 | 0.7 |
| 28 | n774,5 n784,5 |  | 10.4 | 8.9 | 7.8 |  |  | 4.7 | 3.7 | 3 |  | 1.7 | 1.2 | 0.7 |
| 20 | n786,7 |  | 10.8 | 9.1 | 8 |  |  | 6 | 4.0 | 3.2 |  | 2.0 | 1.5 | 1.0 |
| 26 | n418,9 |  | 10.3 | 8.4 | 7.4 |  |  | 5 | 4.3 | 3.9 |  | 3.1 | 2.7 |  |
| 26 | n776,7 n786,7 |  | 10.8 | 9.1 | 8 |  |  | 6 | 4.0 | 3.2 |  | 2.0 | 1.5 | 1.0 |
| n28 | 18,9,10 | 10.2 | 7.6 | 6.2 | 5.3 |  |  |  |  |  |  |  |  |  |
| n71 | 211 | 4.6 | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |  |  |
|  | 212 | 1.7 | 1.0 | 0.7 | 0.6 |  |  |  |  |  |  |  |  |  |
| n71 | 76,7 | 14.6 | 11.7 | 10.1 | 9 |  |  |  |  |  |  |  |  |  |
| 66 | n772, 13 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.3 | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 66 | n782, 13 |  | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.8 | 16.0 |  | 14.8 | 14.3 | 13.8 |
|  | n783 |  | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| n66 | 482,13 | 27.3 | 24.4 | 22.4 | 21.2 |  |  |  |  |  |  |  |  |  |
|  | 483 | 1.9 | 1.4 | 0.9 | 0.4 |  |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , where andare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 9 The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 10: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. NOTE 11: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 12: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 13: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the EN-DC band combination: ∆FHD = 10 MHz for DC_1_n77, DC_2_n48, DC_2_n77, DC_48_n66, DC_66_n48, DC_66_n77, DC_3_n77, DC_3_n78, DC_66_n78, and DC_25_n77. NOTE 14: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 from TS 36.101-1 apply unless otherwise specified). NOTE 15: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE. NOTE 16: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL. This band is subject to 2nd harmonic fall in B21 also which MSD is not specified. NOTE 17: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.1-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 1 | n77 | 15 |  | 25 | 36 | 50 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| 2 | n77 | 15 |  | 25 | 36 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| 2 | n78 | 15 |  | 25 | 36 | 50 |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |
| 3 | n77 | 15 |  | 25 | 36 | 50 |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |
| 3 | n78 | 15 |  | 25 | 36 | 50 |  |  | 50 | 50 | 50 |  | 50 | 50 | 50 |
| 5 | n77 | 15 |  | 16 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| 5 | n78 | 15 | 8 | 16 | 25 | 25 |  |  | 25 |  |  |  |  |  |  |
| 8 | n41 | 15 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| 8 | n77 n78 | 15 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| 8 | n79 | 15 |  |  |  |  |  |  | 25 | 25 | 25 |  | 25 |  | 25 |
| 12 | n66 | 15 | 8 | 16 | 20 | 20 |  |  | 20 |  |  |  |  |  |  |
| 12 | n78 | 15 |  | 10 | 15 | 20 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| 13 | n77 | 15 |  | 15 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| 18 | n77, n78 | 15 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| 19 | n77, n78 | 15 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 |  | 25 |
| 20 | n77 n78 | 15 |  | 16 | 25 | 251, 252 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| 26 | n41 | 15 |  | 16 | 25 | 25 |  |  | 25 | 25 |  |  |  |  |  |
| 26 | n77 n78 | 15 |  | 16 | 25 | 25 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| n28 | 1 | 15 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |  |
| 28 | n77 n78 | 15 |  | 10 | -15 | 20 |  |  | 25 | 25 | 25 |  | 25 | 25 | 25 |
| 66 | n77 | 15 |  | 25 | 36 | 50 | 64 | 80 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| 66 | n78 | 15 |  | 25 | 36 | 50 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n66 | 48 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |  |  |
| n71 | 2 | 15 | 254 85 | 254 85 | 204 85 | 204 85 |  |  |  |  |  |  |  |  |  |
| n71 | 7 | 15 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |  |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band unless the UL resource blocks exceed that specified in Table 7.3.1-2 in TS 36.101 [5] or Table 7.3.2-3 in TS 38.101-1 [2] for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 in TS 36.101 [5] or Table 7.3.2-3 in TS 38.101-1 [2] applies. NOTE 2: Void. NOTE 3: Unless stated otherwise, UL resource blocks shall be centred within the transmission bandwidth configuration for the channel bandwidth. NOTE 4: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 5: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 6: If the aggressor band is NR band, the test SCS and UL RB can be adjusted according to supported BW and lowest SCS supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.3B.2.0.3.2	Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by receiver harmonic mixing due to another band part of the same EN-DC configuration. Reference sensitivity exceptions for the victim band (low) are specified in Table 7.3B.2.0.3.2-1 with uplink configuration of the aggressor band (high) specified in Table 7.3B.2.0.3.2-2.

Table 7.3B.2.0.3.2-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 2 | n714 | 26.8 | 23.6 | 21.2 | 15.6 |  |  |  |  |  |  |  |
| n41 | 264 | 24.3 | 24.3 | 22.5 | N/A |  |  |  |  |  |  |  |
| n77 | 2 | 6.1 | 5.0 | 4.0 | 3.7 |  |  |  |  |  |  |  |
| n77 | 3 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |
| n77 | 132 | 31 | 28 |  |  |  |  |  |  |  |  |  |
| n78 | 3 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |
| n77 | 418 | 10.4 | 10.4 | 10.4 | 10.4 |  |  |  |  |  |  |  |
| n77 | 282 | 28 | 25 | 23.2 | 22 |  |  |  |  |  |  |  |
| n78 | 418 | 10.4 | 10.4 | 10.4 | 10.4 |  |  |  |  |  |  |  |
| n79 | 114 | 39.3 | 36.3 | 34.5 |  |  |  |  |  |  |  |  |
| n79 | 192 | 29.5 | 26.5 | 24.7 |  |  |  |  |  |  |  |  |
| n79 | 214 | 39.3 | 36.3 | 34.5 |  |  |  |  |  |  |  |  |
| n79 | 262 | 27 | 24 | 22.2 |  |  |  |  |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHBfULHB the UL carrier frequency in the higher band, both in MHz. NOTE 3: Void. NOTE 4: The requirements should be verified for DL EARFCN or NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and in MHz and fULHBfULHB the UL carrier frequency in the higher band, both in MHz. NOTE 5: Void. NOTE 6: Void. NOTE 7: Void. NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band andfULHBfULHB the UL carrier frequency in the higher band, both in MHz. NOTE 9: No requirements apply for the case that there is at least one individual RE within the uplink transmission bandwidth of the relative higher band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the 3 times of the frequency range of the relative lower band’s downlink channel bandwidth. The reference sensitivity is only verified when this is not the case NOTE 10: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE. NOTE 11: The MSD test points cannot be verified for the band combination in US due to the Band n77 frequency range restriction. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.2-2: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 2 | n71 | 15 | 25 | 50 | 50 | 50 |  |  |  |  |  |  |  |
| n41 | 26 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n77 | 2 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 | 3 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 | 13 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |
| n78 | 3 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 | 28 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |
| n77 | 41 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |
| n78 | 41 | 15 | 12 | 25 | 36 | 50 |  |  |  |  |  |  |  |
| n79 | 11 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 19 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 21 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| n79 | 26 | 15 | 25 | 50 | 75 |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: The UL configuration applies regardless of the channel bandwidth of the UL band. UL resource blocks allocation in the table shall be further limited to that specified in Table 7.3.1-2 in TS 36.101 [5] or Table 7.3.2-3 in TS 38.101-1[2]. NOTE 4: Unless otherwise stated, the UL resource blocks allocation is applied at the centre of the channel bandwidth. The note applies to the entire table. NOTE 5: If the aggressor band is NR band, the test SCS and UL RB can be adjusted according to supported BW and lowest SCS supported by the UE. |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.3B.2.0.3.3	Void

7.3B.2.0.3.4	Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by UL of another band part of the same EN-DC configuration due to cross band isolation issues. Reference sensitivity exceptions for the victim band are specified in Table 7.3B.2.0.3.4-1, Table 7.3B.2.0.3.4-1a with uplink configuration of the aggressor band specified in Table 7.3B.2.0.3.4-2.

Table 7.3B.2.0.3.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for PC3 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band /MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dBm) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n15 | 3 | 3 | 2.3 | 2 | 1.8 |  |  |  |  |  |  |  |  |
| 15 | n3 | 3 | 2.2 | 1.9 | 1.7 | 1.6 | 1.5 | [1.4] |  |  |  |  |  |
| 3 | n41 |  | 0.7 | 0.7 | 0.7 |  | [0.7] | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 |
| n5 | 28 | 4.5 | 3 | 2.2 | 0.3 |  |  |  |  |  |  |  |  |
| n40 | 1 | 8.3 | 8.3 | 8.3 | 8.3 |  |  |  |  |  |  |  |  |
| n41 | 2 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |
| n41 | 3 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |
| n41 | 25 | 0.6 | 0.6 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |
| n77 | 411 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |
| n41 | 661 | 3.5 | 3.5 | 3.5 | 3.5 |  |  |  |  |  |  |  |  |
| 41 | n77 |  | 8.3 | 8.3 | 8.3 |  |  | 6.3 | 5.3 | 4.5 | 4.0 | 3.9 | 3.8 |
| 30 | n66 | 8.3 | 8.3 | 8.3 | 8.3 |  |  | 8.3 |  |  |  |  |  |
| n78 | 71 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |
| n78 | 38 | 3.3 | 3.3 | 3.3 | 3.3 |  |  |  |  |  |  |  |  |
| n78 | 411 | 4.5 | 4.5 | 4.5 | 4.5 |  |  |  |  |  |  |  |  |
| 41 | n78 |  | 8.3 | 8.3 | 8.3 |  |  | 6.3 | 5.3 | 4.5 | 4.0 | 3.9 | 3.8 |
| n79 | 424 | 2.8 | 2.8 | 2.8 | 2.8 |  |  |  |  |  |  |  |  |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2515 – 2690 MHz. NOTE 3: These requirements apply when the uplink is active in Band n1, n84 and the separation between the lower edge of the uplink channel in Band n1, n84 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band n1, n84. NOTE 4: The DL victim band should be configured using the lowest SCS that is compatible with the highest CBW for which an MSD is specified. NOTE 5: MSD test point can be chosen according to supported BW and lowest SCS supported by the UE. NOTE 6: The requirements only apply for UEs supporting inter-band DC_42_n79 ENDC with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band 42 with a n77 implementation only. These restrictions are applicable to related higher order configurations. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.4-1a: Reference sensitivity exceptions (MSD) due to cross band isolation for PC2 EN-DC in NR FR1

|  | E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 3 | n41 |  | 0.7 | 0.7 | 0.7 |  |  | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 |
| n41 | 3 | 2.3 | 2.3 | 2.3 | 2.3 |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.4-2: Uplink configuration for reference sensitivity exceptions due to cross band isolation for PC3 EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n1 | 3 | 15 | 25 | 25 | 25 | 25 |  |  |  |  |  |  |  |  |
| 1 | n3 | 15 | 25 | 25 | 25 | 25 | 25 | 25 | [25] |  |  |  |  |  |
| 3 | n41 | 15 |  | 50 | 50 | 50 |  | [50] | 50 | 50 | 50 | 50 | 50 | 50 |
| n5 | 28 | 15 | 25 | 25 | 20 | 20 |  |  |  |  |  |  |  |  |
| n40 | 1 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |  |
| n41 | 2 | 30 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |  |
| n41 | 3 | 30 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |  |
| n41 | 25 | 30 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |  |  |
| n77 | 41 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |
| n41 | 66 | 30 | 128 | 128 | 128 | 128 |  |  |  |  |  |  |  |  |
| 41 | n77 | 15 |  | 100 | 100 | 100 |  |  | 100 | 100 | 100 | 100 | 100 | 100 |
| 30 | n66 | 15 | 25 | 25 | 25 | 25 |  |  | 25 |  |  |  |  |  |
| n78 | 7 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |
| n78 | 38 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |
| n78 | 41 | 30 | 270 | 270 | 270 | 270 |  |  |  |  |  |  |  |  |
| 41 | n78 | 15 |  | 100 | 100 | 100 |  |  | 100 | 100 | 100 | 100 | 100 | 100 |
| n79 | 42 | 30 | 2704 | 2704 | 2704 | 2704 |  |  |  |  |  |  |  |  |
| NOTE 1: The UL configuration applies regardless of the channel bandwidth of the UL band. UL resource blocks allocation in the table shall be further limited to that specified in Table 7.3.1-2 in TS 36.101 [5] or Table 7.3.2-3 in TS 38.101-1 [2]. NOTE 2: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 3: When the maximum UL RB allocation "LCRB" value is less than the maximum transmission bandwidth configuration “NRB” defined in Table 5.3.2-1 in 38.101-1 [2] for the specified UL band SCS, the UL band should be configured using the lowest CBW that is compatible with the maximum specified LCRB value. NOTE 4: If the aggressor band is NR band, the test SCS and UL RB can be adjusted according to supported BW and lowest SCS supported by the UE. NOTE 5: The requirements only apply for UEs supporting inter-band ENDC with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band 42 with a n77 implementation only. These restrictions are applicable to related higher order configurations. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.3B.2.0.3.5	MSD for intermodulation interference due to dual uplink operation for EN-DC in NR FR1

7.3B.2.0.3.5.0	General

For EN-DC configurations in NR FR1 the UE may indicate capability of not supporting simultaneous dual uplink operation due to possible intermodulation interference overlapping in frequency to its own primary downlink channel bandwidth if

-	the intermodulation order is 2;

-	the intermodulation order is 3 when both operating bands are between 450 MHz – 960 MHz or between 1427 MHz – 2690 MHz

In the case for EN-DC configurations in NR FR1 for which the intermodulation products caused by dual uplink operation do not interfere with its own primary downlink channel bandwidth as defined in Annex M  the UE is mandated to operate in dual and triple uplink mode.

For EN-DC configurations in NR FR1 with uplink and downlink assigned to E-UTRA and NR FR1 bands given in Table 7.3B.2.0.3.5.1-1, Table 7.3B.2.0.3.5.1-1a, Table 7.3B.2.0.3.5.2-0 and Table 7.3B.2.0.3.5.2-1 the reference sensitivity is defined only for the specific uplink and downlink test points specified in Table 7.3B.2.0.3.5.1-1, Table 7.3B.2.0.3.5.1-1a, Table 7.3B.2.0.3.5.2-0 and Table 7.3B.2.0.3.5.2-1. For these test points the reference sensitivity levels specified in clause 7.3.1 in TS 36.101 [5] and 7.3.2 of TS 38.101-1 [2] for the corresponding channel bandwidths or in clause 7.3.1 of TS 36.101 [5] are relaxed by the amount of the parameter MSD given in Table 7.3B.2.0.3.5.1-1, Table 7.3B.2.0.3.5.1-1a, Table 7.3B.2.0.3.5.2-0 and Table 7.3B.2.0.3.5.2-1.

The throughput on each of the CGs shall be ≥ 95% of the maximum throughput of the respective reference measurement channels as specified in Annex A of TS 38.101-1 [2] and Annex A of TS 36.101 [5], with parameters specified in Table 7.3B.2.0.3.5.1-1, Table 7.3B.2.0.3.5.1-1a, Table 7.3B.2.0.3.5.2-0 and Table 7.3B.2.0.3.5.2-1 with dual UL transmissions overlapping in time unless otherwise stated.

7.3B.2.0.3.5.1	MSD test points for intermodulation interference due to dual uplink operation for PC3 EN-DC in NR FR1 involving two bands

Table 7.3B.2.0.3.5.1-1: MSD test points for PCell due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration |  | EUTRA or NR band |  | UL Fc (MHz) |  | UL/DL BW (MHz) |  | UL LCRB |  | DL Fc (MHz) |  | MSD (dB) |  | IMD order |
| DC_1_n3 |  | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 23 |  | IMD3 |
|  |  | n3 |  | 1760 |  | 5 |  | 25 |  | 1855 |  | N/A |  | N/A |
| DC_1A_n8A |  | 1 |  | 1965 |  | 5 |  | 25 |  | 2155 |  | 6.0 |  | IMD4 |
|  |  | n8 |  | 887.5 |  | 5 |  | 25 |  | 932.5 |  | N/A |  | N/A |
| DC_1A_n77A |  | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 29.8 |  | IMD23 |
| DC_1A_SUL_n77A- |  |  |  |  |  |  |  |  |  |  |  | 32.54 |  |  |
| n84A |  | n77, n78 |  | 4090 |  | 10 |  | 50 |  | 4090 |  | N/A |  | N/A |
| DC_1A_n77A |  | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 8.0 |  | IMD43 |
| DC_1A-SUL_n77A-n84A |  |  |  |  |  |  |  |  |  |  |  | 10.74 |  |  |
| DC_1A_n78A, DC_1A_SUL_n78A-n84A |  | n77, n78 |  | 3710 |  | 10 |  | 50 |  | 3710 |  | N/A |  | N/A |
| DC_1A_n78A, |  | 1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 8.0 |  | IMD43 |
| DC_1A_SUL_n78A- |  |  |  |  |  |  |  |  |  |  |  | 10.74 |  |  |
| n84A |  | n78 |  | 3710 |  | 10 |  | 25 |  | 3710 |  | N/A |  |  |
| DC_2A_n66A |  | 2 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 20 |  | IMD3 |
|  |  | n66 |  | 1775 |  | 5 |  | 25 |  | 2175 |  | N/A |  | N/A |
| DC_2A_n66A |  | 2 |  | 1883.3 |  | 5 |  | 25 |  | 1963.3 |  | N/A |  | N/A |
|  |  | n66 |  | 1750 |  | 5 |  | 25 |  | 2150 |  | 4 |  | IMD5 |
| DC_2A_n77A DC_2A-2A_n77A |  | 2 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 26 |  | IMD2 |
|  |  |  |  |  |  |  |  |  |  |  |  | 28.74 |  |  |
|  |  | n77 |  | 3790 |  | 10 |  | 50 |  | 3790 |  | N/A |  | N/A |
|  |  | 2 |  | 1900 |  | 5 |  | 25 |  | 1980 |  | 8.0 |  | IMD4 |
|  |  |  |  |  |  |  |  |  |  |  |  | 10.74 |  |  |
|  |  | n77 |  | 3720 |  | 10 |  | 50 |  | 3720 |  | N/A |  | N/A |
|  |  | 2 |  | 1885 |  | 5 |  | 25 |  | 1965 |  | 5 |  | IMD5 |
|  |  |  |  |  |  |  |  |  |  |  |  | 7.74 |  |  |
|  |  | n77 |  | 3810 |  | 10 |  | 50 |  | 3810 |  | N/A |  | N/A |
| DC_2A_n78A |  | 2 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 26 |  | IMD23 |
|  |  |  |  |  |  |  |  |  |  |  |  | 28.74 |  |  |
|  |  | n78 |  | 3790 |  | 10 |  | 50 |  | 3790 |  | N/A |  | N/A |
|  |  | 2 |  | 1885 |  | 5 |  | 25 |  | 1965 |  | 8.0 |  | IMD43 |
|  |  |  |  |  |  |  |  |  |  |  |  | 10.74 |  |  |
|  |  | n78 |  | 3690 |  | 10 |  | 50 |  | 3690 |  | N/A |  | N/A |
| DC_3_n1 |  | 3 |  | 1760 |  | 5 |  | 25 |  | 1855 |  | N/A |  | N/A |
|  |  | n1 |  | 1950 |  | 5 |  | 25 |  | 2140 |  | 23 |  | IMD3 |
|  |  | 3 |  | 1771 |  | 10 |  | 50 |  | 1866 |  | 4 |  | IMD4 |
| DC_3_n5 |  | n5 |  | 838 |  | 5 |  | 25 |  | 883 |  | N/A |  | N/A |
|  |  | 3 |  | 1721 |  | 10 |  | 50 |  | 1816 |  | N/A |  | N/A |
|  |  | n5 |  | 838 |  | 5 |  | 25 |  | 883 |  | 24 |  | IMD23 |
| DC_3A_n7A |  | 3 |  | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |
| DC_3C_n7A |  | n7 |  | 2535 |  | 10 |  | 50 |  | 2655 |  | 10.25 |  | IMD4 |
| DC_3A_n8A |  | n8 |  | 900 |  | 5 |  | 25 |  | 945 |  | 8 |  | IMD43 |
|  |  | 3 |  | 1755 |  | 10 |  | 50 |  | 1850 |  | N/A |  | N/A |
|  |  | n8 |  | 897.5 |  | 5 |  | 25 |  | 942.5 |  | N/A |  | N/A |
|  |  | 3 |  | 1747.5 |  | 10 |  | 50 |  | 1842.5 |  | 6.4 |  | IMD5 |
|  |  | 3 |  | 1775 |  | 5 |  | 25 |  | 1870 |  | 4 |  | IMD4 |
| DC_3A_n20A |  | n20 |  | 840 |  | 5 |  | 25 |  | 799 |  | N/A |  | N/A |
|  |  | 3 |  | 1735 |  | 5 |  | 25 |  | 1830 |  | N/A |  | N/A |
|  |  | n20 |  | 847 |  | 5 |  | 25 |  | 806 |  | 9 |  | IMD4 |
| DC_3A_n41A |  | 3 |  | 1740 |  | 5 |  | 25 |  | 1835 |  | 8.2 |  | IMD4 |
| DC_3C_n41A |  | n41 |  | 2657.5 |  | 10 |  | 50 |  | 2657.5 |  | N/A |  | IMD4 |
| DC_3A_SUL_n41A-n80A, |  | 3 |  | 1740 |  | 5 |  | 25 |  | 1835 |  | 8.2 |  | IMD4 |
| DC_3C_SUL_n41A-n80A |  | n41 |  | 2657.5 |  | 10 |  | 52 |  | 2657.5 |  | N/A |  | N/A |
| DC_3A_n77A |  | 3 |  | 1740 |  | 5 |  | 25 |  | 1835 |  | 26 |  | IMD23 |
| DC_3A_SUL_n77A-n80A DC_3A_n78A |  |  |  |  |  |  |  |  |  |  |  | 28.74 |  |  |
| DC_3A-SUL_n78A-n80A, DC_3C_n78A |  | n77, n78 |  | 3575 |  | 10 |  | 50 |  | 3575 |  | N/A |  | N/A |
| DC_3A_n77A |  | 3 |  | 1765 |  | 5 |  | 25 |  | 1860 |  | 8.0 |  | IMD43 |
| DC_3A_SUL_n77A-n80A, DC_3A_n78A |  |  |  |  |  |  |  |  |  |  |  | 10.74 |  |  |
| DC_3A-SUL_n78A-n80A, DC_3C_n78A |  | n77, n78 |  | 3435 |  | 10 |  | 50 |  | 3435 |  | N/A |  | N/A |
| DC_5A_n66A |  | 5 |  | 838 |  | 5 |  | 25 |  | 883 |  | 30 |  | IMD23 |
|  |  | n66 |  | 1721 |  | 5 |  | 25 |  | 2121 |  | N/A |  | N/A |
| DC_5A_n78A |  | 5 |  | 844 |  | 5 |  | 25 |  | 889 |  | 8.3 |  | IMD4 |
|  |  | n78 |  | 3421 |  | 10 |  | 50 |  | 3421 |  | N/A |  | N/A |
| DC_5A_n77A8 |  | 5 |  | 844 |  | 5 |  | 25 |  | 889 |  | 8.3 |  | IMD4 |
|  |  | n77 |  | 3421 |  | 10 |  | 50 |  | 3421 |  | N/A |  | N/A |
|  |  | 5 |  | 826.5 |  | 5 |  | 25 |  | 871.5 |  | 5.5 |  | IMD5 |
|  |  | n77 |  | 4177.5 |  | 10 |  | 50 |  | 4177.5 |  | N/A |  | N/A |
| DC_7_n3 |  | 7 |  | 2535 |  | 10 |  | 50 |  | 2655 |  | 13 |  | IMD4 |
|  |  | n3 |  | 1730 |  | 5 |  | 25 |  | 1825 |  | N/A |  | N/A |
| DC_7_n5 |  | 7 |  | 2547 |  | 10 |  | 50 |  | 2667 |  | N/A |  | N/A |
|  |  | n5 |  | 834 |  | 5 |  | 25 |  | 879 |  | 12 |  | IMD33 |
| DC_7A_n77A |  | 7 |  | 2540 |  | 5 |  | 25 |  | 2660 |  | 7.1 |  | IMD4 |
|  |  | n77 |  | 3870 |  | 10 |  | 50 |  | 3870 |  | N/A |  | N/A |
| DC_7A_n66A |  | 7 |  | 2535 |  | 10 |  | 50 |  | 2655 |  | 15 |  | 4th IMD |
| DC_7A-7A_n66A DC_7C_n66A |  | n66 |  | 1730 |  | 5 |  | 25 |  | 2130 |  | N/A |  | N/A |
| DC_8A_n1A |  | 8 |  | 887.5 |  | 5 |  | 25 |  | 932.5 |  | N/A |  | N/A |
|  |  | n1 |  | 1965 |  | 5 |  | 25 |  | 2155 |  | 6 |  | IMD4 |
|  |  | 8 |  | 900 |  | 5 |  | 25 |  | 945 |  | 8 |  | IMD43 |
| DC_8A_n3A |  | n3 |  | 1755 |  | 10 |  | 50 |  | 1850 |  | N/A |  | N/A |
|  |  | 8 |  | 897.5 |  | 5 |  | 25 |  | 942.5 |  | N/A |  | N/A |
|  |  | n3 |  | 1747.5 |  | 10 |  | 50 |  | 1842.5 |  | 6.4 |  | IMD5 |
| DC_8A_n20A | n20 |  | 849.5 |  | 5 |  | 25 |  | 808.5 |  | 25 |  | IMD33 |  |
|  | 8 |  | 890.5 |  | 5 |  | 25 |  | 935.5 |  | N/A |  | N/A |  |
|  | n20 |  | 847.5 |  | 5 |  | 25 |  | 806.5 |  | N/A |  | N/A |  |
|  | 8 |  | 892.5 |  | 5 |  | 25 |  | 937.5 |  | 25 |  | IMD33 |  |
| DC_8A_n41A |  | 8 |  | 882.5 |  | 5 |  | 25 |  | 927.5 |  | 12.1 |  | IMD33 |
|  |  | n41 |  | 2685 |  | 10 |  | 50 |  | 2685 |  | N/A |  | N/A |
| DC_8A_SUL_n41A- |  | 8 |  | 882.5 |  | 5 |  | 25 |  | 927.5 |  | 12.1 |  | IMD33 |
| n81A |  | n41 |  | 2685 |  | 10 |  | 52 |  | 2685 |  | N/A |  | N/A |
| DC_8A_n77A DC_8A_n78A |  | 8 |  | 897.5 |  | 5 |  | 25 |  | 942.5 |  | 8.3 |  | IMD4 |
| DC_8A-n79C, DC_8A-SUL_n78A-n81A |  | n77, n78 |  | 3635 |  | 10 |  | 50 |  | 3635 |  | N/A |  | H4 |
| DC_12_n78 |  | 12 |  | 710 |  | 5 |  | 25 |  | 740 |  | 5.5 |  | IMD5 |
|  |  | n78 |  | 3580 |  | 10 |  | 50 |  | 3580 |  | N/A |  | N/A |
| DC_13A_n77A |  | 13 |  | 784.5 |  | 5 |  | 20 |  | 753.5 |  | 5.5 |  | IMD5 |
|  |  | n77 |  | 3891.5 |  | 10 |  | 50 |  | 3891.5 |  | N/A |  | N/A |
|  |  | 20 |  | 840 |  | 5 |  | 25 |  | 799 |  | N/A |  | N/A |
| DC_20A_n3A |  | n3 |  | 1775 |  | 5 |  | 25 |  | 1870 |  | 4 |  | IMD4 |
|  |  | 20 |  | 847 |  | 5 |  | 25 |  | 806 |  | 9 |  | IMD4 |
|  |  | n3 |  | 1735 |  | 5 |  | 25 |  | 1830 |  | N/A |  | N/A |
| DC_8A_n79A |  | 8 |  | 897.5 |  | 5 |  | 25 |  | 942.5 |  | 4.8 |  | IMD5 |
| DC_8A-SUL_n79A-n81A |  | n79 |  | 4532.5 |  | 40 |  | 216 |  | 4532.5 |  | N/A |  | N/A |
| DC_18A_n77A DC_18A_n78A |  | 18 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD4 |
|  |  | n77, n78 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |
| DC_12A_n77A |  | 12 |  | 702 |  | 5 |  | 20 |  | 732 |  | 5.5 |  | IMD5 |
|  |  | n77 |  | 3540 |  | 10 |  | 50 |  | 3540 |  | N/A |  | N/A |
| DC_19A_n78A |  | 19 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | IMD4 |
|  |  | n78 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |
| DC_20_n7 | 20 |  | 851 |  | 5 |  | 25 |  | 810 |  | 12 |  | IMD33 |  |
|  | n7 |  | 2512 |  | 10 |  | 50 |  | 2632 |  | N/A |  | N/A |  |
| DC_20A_n8A |  | 20 |  | 849.5 |  | 5 |  | 25 |  | 808.5 |  | 21 |  | IMD3 |
|  |  | n8 |  | 892.5 |  | 5 |  | 25 |  | 937.5 |  | 21 |  | IMD3 |
| DC_20A_n77A DC_20A_n78A, |  | 20 |  | 850 |  | 5 |  | 25 |  | 809 |  | 11 |  | IMD4 |
| DC_20A-SUL_n78A-n82A |  | n77, n78 |  | 3359 |  | 10 |  | 50 |  | 3359 |  | N/A |  | N/A |
| DC_20A_n77A |  | 20 |  | 840 |  | 5 |  | 25 |  | 799 |  | 6.5 |  | IMD54 |
|  |  | n77 |  | 4159 |  | 10 |  | 50 |  | 4159 |  | N/A |  | N/A |
| DC_21A_n79A |  | 21 |  | 1457.5 |  | 5 |  | 25 |  | 1505.5 |  | 18.4 |  | IMD3 |
|  |  | n79 |  | 4420.5 |  | 40 |  | 216 |  | 4420.5 |  | N/A |  | N/A |
| DC_26A_n41A |  | 26 |  | 839 |  | 5 |  | 25 |  | 884 |  | 15.6 |  | IMD3 |
|  |  | n41 |  | 2562 |  | 10 |  | 50 |  | 2562 |  | N/A |  | N/A |
|  |  | 28 |  | 730 |  | 10 |  | 50 |  | 775 |  | 15.3 |  | IMD 2 |
|  |  | n50 |  | 1500 |  | 10 |  | 50 |  | 1500 |  | N/A |  | N/A |
| DC_28_n50 |  | 28 |  | 740 |  | 10 |  | 50 |  | 785 |  | 6 |  | IMD 4 |
|  |  | n50 |  | 1500 |  | 10 |  | 50 |  | 1500 |  | N/A |  | N/A |
|  |  | 28 |  | 740 |  | 10 |  | 50 |  | 785 |  | 0.5 |  | IMD 5 |
|  |  | n50 |  | 1500 |  | 10 |  | 50 |  | 1500 |  | N/A |  | N/A |
| DC_28A_n51A |  | 28 |  | 742.3 |  | 5 |  | 25 |  | 797.3 |  | 5 |  | IMD 4 |
|  |  | n51 |  | 1429.5 |  | 5 |  | 25 |  | 1429.5 |  | N/A |  | N/A |
| DC_25A_n77A DC_25A-25A_n77A |  | 25 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 26 |  | IMD2 |
|  |  | n77 |  | 3790 |  | 10 |  | 50 |  | 3790 |  | N/A |  | N/A |
|  |  | 25 |  | 1900 |  | 5 |  | 25 |  | 1980 |  | 8 |  | IMD4 |
|  |  | n77 |  | 3720 |  | 10 |  | 50 |  | 3720 |  | N/A |  | N/A |
|  |  | 25 |  | 1885 |  | 5 |  | 25 |  | 1965 |  | 5 |  | IMD5 |
|  |  | n77 |  | 3810 |  | 10 |  | 50 |  | 3810 |  | N/A |  | N/A |
| DC_26A_n77A, |  | 26 |  | 836.5 |  | 5 |  | 25 |  | 881.5 |  | 11.1 |  | IMD4 |
| DC_26A_n78A |  | n77, n78 |  | 3391 |  | 10 |  | 50 |  | 3391 |  | N/A |  | N/A |
| DC_28A_n77A, |  | 28 |  | 705.5 |  | 5 |  | 25 |  | 760.5 |  | 5.5 |  | IMD5 |
| DC_28A_n78A, DC_28A-SUL_n78A-n83A |  | n77, n78 |  | 3582.5 |  | 10 |  | 50 |  | 3582.5 |  | N/A |  | N/A |
| DC_48A_n66A |  | 48 |  | 3630 |  | 20 |  | 100 |  | 3630 |  | N/A |  | N/A |
|  |  | n66 |  | 1715 |  | 5 |  | 25 |  | 2115 |  | 4 |  | IMD5 |
|  |  | 66 |  | 1775 |  | 5 |  | 25 |  | 2175 |  | N/A |  | N/A |
| DC_66A_n2A |  | n2 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 20 |  | IMD3 |
|  |  | 66 |  | 1750 |  | 5 |  | 25 |  | 2150 |  | 4 |  | IMD5 |
|  |  | n2 |  | 1883.3 |  | 5 |  | 25 |  | 1963.3 |  | N/A |  | N/A |
| DC_66A_n5A |  | n5 |  | 838 |  | 5 |  | 25 |  | 883 |  | 30 |  | IMD23 |
|  |  | 66 |  | 1721 |  | 5 |  | 25 |  | 2121 |  | N/A |  | N/A |
|  |  | 66 |  | 1775 |  | 5 |  | 25 |  | 2175 |  | N/A |  | N/A |
|  |  | n25 |  | 1855 |  | 5 |  | 25 |  | 1935 |  | 20 |  | IMD3 |
| DC_66A_n25A |  | 66 |  | 1712.5 |  | 5 |  | 25 |  | 2112.5 |  | 23 |  | IMD3 |
|  |  | n25 |  | 1912.5 |  | 5 |  | 25 |  | 1992.5 |  | N/A |  | N/A |
|  |  | 66 |  | 1750 |  | 5 |  | 25 |  | 2150 |  | 4 |  | IMD5 |
|  |  | n25 |  | 1883.3 |  | 5 |  | 25 |  | 1963.3 |  | N/A |  | N/A |
| DC_66A_n71A |  | 66 |  | 1750 |  | 5 |  | 25 |  | 2150 |  | 5 |  | IMD4 |
|  |  | n71 |  | 675 |  | 5 |  | 25 |  | 629 |  | N/A |  | N/A |
| DC_66A_n77A DC_66-66_n77A DC_66-66-66_n77A |  | 66 |  | 1775 |  | 5 |  | 25 |  | 2175 |  | 31.0 |  | IMD2 |
|  |  | n77 |  | 3950 |  | 10 |  | 50 |  | 3950 |  | N/A |  | N/A |
|  |  | 66 |  | 1760 |  | 5 |  | 25 |  | 2160 |  | 5.0 |  | IMD5 |
|  |  | n77 |  | 3720 |  | 10 |  | 50 |  | 3720 |  | N/A |  | N/A |
| DC_66A_n78A |  | 66 |  | 1730 |  | 5 |  | 25 |  | 2130 |  | 5.0 |  | IMD5 |
|  |  | n78 |  | 3660 |  | 5 |  | 50 |  | 3660 |  | N/A |  | N/A |
| NOTE 1: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 5: Void. NOTE 6: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 7: The frequency range in band n28 is restricted for this band combination to 728 - 738 MHz for the UL and 783 - 793 MHz for the DL. This band is subject to IMD2, IMD4 and IMD5 fall in n28 also which MSD is not specified. In addition, this band is subject to IMD4 fall in B21 also which MSD is not specified. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.5.1-1a: MSD test points for PCell due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n78A | 1 | 1950 | 5 | 25 | 2140 | 17.8 | IMD4 |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |
| DC_3A_n41A | 3 | 1740 | 5 | 25 | 1835 | 18.4 | IMD4 |
|  | n41 | 2657.5 | 10 | 50 | 2657.5 | N/A | N/A |
| DC_3A_n78A | 3 | 1740 | 5 | 25 | 1835 | 31.9 | IMD2 |
|  | n78 | 3575 | 10 | 50 | 3575 | N/A | N/A |
| DC_3A_n78A | 3 | 1765 | 5 | 25 | 1860 | 18.5 | IMD4 |
|  | n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| DC_2A_n77A DC_2A-2A_n77A DC_2A_n77C DC_2A-2A_n77C | 2 | 1855 | 5 | 25 | 1935 | 32.10 | IMD2 |
|  |  |  |  |  |  | 34.852 |  |
|  | n77 | 3790 | 10 | 50 | 3790 | N/A | N/A |
|  | 2 | 1900 | 5 | 25 | 1980 | 19.10 | IMD41 |
|  |  |  |  |  |  | 21.852 |  |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| DC_5A_n77A3 DC_5A_n77C3 | 5 | 844 | 5 | 25 | 889 | 18.60 | IMD41 |
|  | n77 | 3421 | 10 | 50 | 3421 | N/A | N/A |
| DC_13A_n77A DC_13A_n77C | 13 | 782 | 5 | 20 | 751 | 15.37 | IMD5 |
|  | n77 | 3879 | 10 | 50 | 3879 | N/A | N/A |
| DC_66A_n77A DC_66A-66A_n77A DC_66A-66A-66A_n77A DC_66A_n77C DC_66A-66A_n77C DC_66A-66A-66A_n77C | 66 | 1775 | 5 | 25 | 2175 | 34.33 | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | N/A |
|  | 66 | 1760 | 5 | 25 | 2160 | 11.27 | IMD5 |
|  | n77 | 3720 | 10 | 50 | 3720 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 3: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 4: E-UTRA carrier shall be set to min(+23 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+23 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. |  |  |  |  |  |  |  |

7.3B.2.0.3.5.2	MSD test points for intermodulation interference due to dual uplink operation for EN-DC in NR FR1 involving three bands

Table 7.3B.2.0.3.5.2-0: MSD test points for PCell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) |  | IMD order |
| DC_66A_(n)71AA | 66 | 1750 | 5 | 25 | 2150 | 5 |  | IMD4 |
|  | n71 | 678 | 10 | 10 (RBstart =0) | 632 | N/A |  | N/A |
| NOTE 1: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 2: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. |  |  |  |  |  |  |  |  |

Table 7.3B.2.0.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A-3A_n28A DC_1A-3C_n28A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 3 | 1723.5 | 5 | 25 | 1818.5 | 4.0 | IMD5 |
| DC_1A-3A_n28A DC_1A-3C_n28A | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 1 | 1949 | 5 | 25 | 2139 | 11.0 | IMD4 |
| DC_1A-7A_n28A DC_1A-7C_n28A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |
|  | 7 | 2533 | 10 | 50 | 2653 | 30.0 | IMD2 |
| DC_1A-3A_n77A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.5 | IMD2 |
|  | n77 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | 8.5 | IMD4 |
|  | n77 | 3980 | 10 | 50 | 3980 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 31.0 | IMD2 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n77 | 3915 | 10 | 50 | 3915 | N/A | N/A |
| DC_1A-3A_n78A DC_1A-3C_n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1712.5 | 5 | 25 | 1807.5 | 31.2 | IMD2 |
|  | n78 | 3757.5 | 10 | 50 | 3757.5 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 2.8 | IMD5 |
|  | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | n78 | 3725 | 10 | 50 | 3725 | N/A | N/A |
| DC_1A-5A_n78A | 1 | 1932 | 5 | 25 | 2122 | 18.1 | IMD3 |
|  | 5 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | n78 | 3780 | 10 | 50 | 3780 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 5 | 840 | 5 | 25 | 885 | 3.1 | IMD5 |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | N/A |
| DC_1A-7A_n78A DC_1A-7C_n78A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.7 | IMD4 |
|  | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |
| DC_1A_n7A-n78A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n7 | 2507.5 | 5 | 25 | 2627.5 | 9.1 | IMD4 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n7 | 2520 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 10.1 | IMD4 |
| DC_1A-3A_n79A | 1 | 1950 | 5 | 25 | 2140 | 3.6 | IMD5 |
|  | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n79 | 4860 | 40 | 216 | 4860 | N/A | N/A |
| DC_1A-8A_n78A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 8 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_1A-8A_n78A | 1 | 1945 | 5 | 25 | 2135 | N/A | N/A |
|  | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n78 | 3745 | 10 | 52 | 3745 | 14.9 | IMD3 |
|  | 1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 3.3 | IMD5 |
|  | n78 | 3380 | 10 | 52 | 3330 | N/A | N/A |
| DC_1A-18A_n77A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 18 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |
|  | 18 | 825 | 5 | 25 | 870 | N/A | N/A |
|  | n77 | 3770 | 10 | 50 | 3770 | N/A | N/A |
| DC_1A-18A_n78A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 18 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | 16.4 | IMD3 |
|  | 18 | 819 | 5 | 25 | 864 | N/A | N/A |
|  | n78 | 3758 | 10 | 50 | 3758 | N/A | N/A |
| DC_1A-18A_n79A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | 18 | 822.5 | 5 | 25 | 867.5 | 18.3 | IMD3 |
|  | n79 | 4737.5 | 40 | 216 | 4737.5 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 18 | 820 | 5 | 25 | 865 | 8.9 | IMD4 |
|  | n79 | 4925 | 40 | 216 | 4925 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 8.1 | IMD4 |
|  | 18 | 822.5 | 5 | 25 | 867.5 | N/A | N/A |
|  | n79 | 4592.5 | 40 | 216 | 4592.5 | N/A | N/A |
| DC_1A-19A_n77A DC_1A-19A_n78A | 1 | 1940 | 5 | 25 | 2130 | 17.8 | IMD3 |
|  | 19 | 832.5 | 5 | 25 | 877.5 | N/A | N/A |
|  | n77, n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
|  | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 19 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_1A-19A_n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 19 | 837.5 | 5 | 25 | 882.5 | 18.3 | IMD3 |
|  | n79 | 4782.5 | 40 | 216 | 4782.5 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 8.1 | IMD4 |
|  | 19 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |
|  | n79 | 4652.5 | 40 | 216 | 4652.5 | N/A | N/A |
| DC_1A-20A_n8A | 1 | 1925 | 5 | 25 | 2115 | N/A | N/A |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 846 | 5 | 25 | 805 | 11.5 | IMD4 |
| DC_1A-20A_n78A | 1 | 1930 | 5 | 25 | 2120 | 20.3 | IMD3 |
|  | 20 | 835 | 5 | 25 | 794 | N/A | N/A |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
| DC_1A-20A_n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 3.0 | IMD5 |
|  | n78 | 3330 | 10 | 50 | 3330 | N/A | N/A |
| DC_1A-21A_n77A DC_1A-21A_n78A | 1 | 1964.6 | 5 | 25 | 2154.6 | 30.6 | IMD2 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77, n78 | 3605 | 10 | 50 | 3605 | N/A | N/A |
|  | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD2 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | 2.9 | IMD5 |
|  | n77, n78 | 3675 | 10 | 50 | 3675 | N/A | N/A |
| DC_1A-21A_n79A | 1 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_1A-28A_n3A | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | n3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | 1 | 1949 | 5 | 25 | 2139 | 11.0 | IMD4 |
| DC_1A-28A_n77A | 1 | 1960 | 5 | 25 | 2150 | 15.8 | IMD3 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n77 | 3630 | 10 | 50 | 3630 | N/A | N/A |
| DC_1A-28A_n77A | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
|  | 28 | 725 | 5 | 25 | 780 | 4.3 | IMD5 |
|  | n77 | 3330 | 10 | 50 | 3330 | N/A | N/A |
| DC_1A-28A_n78A | 1 | 1960 | 5 | 25 | 2150 | 15.7 | IMD3 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n78 | 3630 | 10 | 50 | 3630 | N/A | N/A |
| DC_1A-28A_n78A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 28 | 739 | 5 | 25 | 794 | 4.2 | IMD5 |
|  | n78 | 3352 | 10 | 50 | 3352 | N/A | N/A |
| DC_1A_n28A-n78A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n28 | 733 | 5 | 25 | 788 | N/A | N/A |
|  | n78 | 3416 | 10 | 50 | 3416 | 15.7 | IMD3 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | N/A |
|  | n28 | 735 | 5 | 25 | 790 | 3.3 | IMD5 |
| DC_1A-28A_n79A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | 28 | 733 | 5 | 25 | 788 | 15.2 | IMD3 |
|  | n79 | 4648 | 40 | 216 | 4648 | N/A | N/A |
|  | 1 | 1925 | 5 | 25 | 2115 | N/A | N/A |
|  | 28 | 740 | 5 | 25 | 795 | 10.0 | IMD4 |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |
|  | 1 | 1977.5 | 5 | 25 | 2167.5 | 1.2 | IMD4 |
|  | 28 | 745.5 | 5 | 25 | 800.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
|  | 1 | 1935 | 5 | 25 | 2125 | 4.5 | IMD5 |
|  | 28 | 718 | 5 | 25 | 773 | N/A | N/A |
|  | n79 | 4807 | 40 | 216 | 4807 | N/A | N/A |
| DC_1A_n40A-n78A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n40 | 2340 | 5 | 25 | 2340 | N/A | N/A |
|  | n78 | 3450 | 10 | 50 | 3450 | 9.8 | IMD4 ¦3*fB1-fn40¦ |
|  | 1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
|  | n40 | 2360 | 5 | 25 | 2360 | 10.6 | IMD4 ¦3*fB1 -fn78¦ |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
| DC_1A-41A_n28A | 1 | 1935 | 5 | 25 | 2125 | N/A | N/A |
|  | n28 | 718 | 5 | 25 | 773 | N/A | N/A |
|  | 41 | 2653 | 10 | 50 | 2653 | 30 | IMD2 |
| DC_1A-41A_n77A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A |  |
|  | 41 | 2510 | 5 | 25 | 2510 | 11.0 | IMD4 |
|  | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n77 | 4150 | 10 | 50 | 4150 | N/A |  |
|  | 41 | 2510 | 5 | 25 | 2510 | 3.6 | IMD5 |
| DC_1A-41A_n78A | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 41 | 2515 | 5 | 25 | 2515 | 12 | IMD4 |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |
| DC_1A-41A_n78A | 1 | 1955 | 5 | 25 | 2145 | 8.7 | IMD4 |
|  | 41 | 2507.5 | 10 | 50 | 2507.5 | N/A | N/A |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |
| DC_1A-41A_n79A | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n79 | 4500 | 40 | 216 | 4500 | N/A |  |
|  | 41 | 2530 | 5 | 25 | 2530 | 29.4 | IMD2 |
| DC_1A-42A_n79A | 1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
|  | 42 | 3490 | 5 | 25 | 3490 | 4.8 | IMD5 |
|  | 42 | 3402.5 | 5 | 25 | 3402.5 | N/A | N/A |
|  | n79 | 4640 | 40 | 216 | 4640 | N/A | N/A |
|  | 1 | 1975 | 5 | 25 | 2165 | 15.5 | IMD3 |
|  | 42 | 3450 | 5 | 25 | 3450 | N/A | N/A |
|  | n79 | 4520 | 40 | 216 | 4520 | N/A | N/A |
|  | 1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |
| DC_1A-SUL_n77A-n80A | 1 | 1950 | 5 | 25 | 2140 | 23 | IMD3 |
|  | n80 | 1760 | 5 | 25 |  | N/A | N/A |
| DC_1A-SUL_n77A-n80A | 1 | 1922.5 | 5 | 25 | 2112.5 | N/A | N/A |
|  | n80 | 1782.5 | 5 | 25 |  | N/A | N/A |
|  | n78 | 3425 | 10 | 50 | 3425 | 13.0 | IMD4 |
| DC_1A_n78A-n79A | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | n79 | 4870 | 40 | 216 | 4870 | 15.9 | IMD3 |
|  | 1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n79 | 4670 | 40 | 216 | 4670 | N/A | N/A |
|  | n78 | 3490 | 10 | 50 | 3490 | 4.6 | IMD5 |
| DC_2A-12A_n66A | 2 | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | 12 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n66 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_2A-14A_n66A | 2 | 1874 | 5 | 25 | 1954 | 7.2 | IMD4 |
|  | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 66 | 1770 | 5 | 25 | 2170 | N/A | N/A |
| DC_2A-66A_n41A | 2 | 1860 | 5 | 25 | 1940 | 22.6 | IMD4 |
|  | 66 | 1715 | 5 | 25 | 2115 | N/A | N/A |
|  | n41 | 2685 | 5 | 25 | 2685 | N/A | N/A |
| DC_3A-5A_n78A | 3 | N/A | N/A | N/A | N/A | N/A | IMD3 |
|  | 5 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_3A-7A_n5A | 3 | 1780 | 10 | 50 | 1875 | N/A | N/A |
|  | 7 | 2505 | 10 | 50 | 2625 | 30.0 | IMD2X |
|  | n5 | 845 | 5 | 25 | 890 | N/A | N/A |
| DC_3A-7A_n8A | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | 7 | 2550 | 10 | 50 | 2670 | 29.0 | IMD2 IMD34 |
| DC_3A-7A_n28A DC_3A-7C_n28A DC_3C-7A_n28A DC_3C-7C_n28A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | 7 | 2562 | 10 | 50 | 2682 | 16.9 | IMD3 |
|  | 7 | 2543 | 10 | 50 | 2663 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 3 | 1737.5 | 5 | 25 | 1832.5 | 26.0 | IMD2 |
| DC_3A-7A_n78A DC_3C-7A_n78A DC_3C-7C_n78A DC_3A-3A-7A_n78A DC_3A-3A-7A-7A_n78A DC_3A-7A_SUL_n78A-n80A DC_3C-7A_SUL_n78A-n80A | 3 | 1725 | 5 | 25 | 1820 | 17.6 | IMD3 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 8.6 | IMD4 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3475 | 10 | 50 | 3475 | N/A | N/A |
| DC_3A-8A_n77A | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  | n77 | 4190 | 10 | 50 | 4190 | N/A | N/A |
|  | 8 | 910 | 5 | 25 | 955 | 9.7 | IMD4 |
| DC_3A-8A_n77A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n77 | 3640 | 10 | 50 | 3640 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 16.5 | IMD3 |
| DC_3A-8A_n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n78 | 3640 | 10 | 50 | 3640 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 16.5 | IMD3 |
| DC_3A-8A_n79A | 3 | 1755 | 5 | 25 | 1850 | N/A | N/A |
|  | n79 | 4465 | 40 | 216 | 4465 | N/A | N/A |
|  | 8 | 910 | 5 | 25 | 955 | 15.3 | IMD3 |
| DC_3A-8A_n79A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n79 | 4580 | 40 | 216 | 4580 | N/A | N/A |
|  | 3 | 1755 | 5 | 25 | 1850 | 8.8 | IMD4 |
| DC_3A_n7A-n78A DC_3C_n7A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 16.1 | IMD3 ¦2*fn7-fB3¦ |
| DC_3A-8A_n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | n78 | 3640 | 10 | 50 | 3640 | N/A | N/A |
|  | 3 | 1725 | 5 | 25 | 1820 | 16.5 | IMD3 |
| DC_3A-18A_n77A DC_3A-18A_n78A | 3 | N/A | N/A | N/A | N/A | N/A | IMD3 |
|  | 18 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n77, n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_3A-19A_n78A | 3 | N/A | N/A | N/A | N/A | N/A | IMD3 |
|  | 19 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_3A-19A_n79A | 3 | 1775 | 5 | 25 | 1870 | N/A | N/A |
|  | 19 | 840 | 5 | 25 | 885 | [18.5] | IMD3 |
|  | n79 | 4435 | 40 | 216 | 4435 | N/A | N/A |
|  | 3 | 1782.5 | 5 | 25 | 1877.5 | 0.2 | IMD4 |
|  | 19 | 842.5 | 5 | 25 | 887.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| DC_3A-20A_n8A | 3 | 1765 | 5 | 25 | 1860 | 14.5 | IMD4 |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
| DC_3A-20A_n28A DC_3C-20A_n28A | 20 | 852 | 5 | 25 | 811 | N/A | N/A |
|  | n28 | 728 | 5 | 25 | 783 | N/A | N/A |
|  | 3 | 1733 | 5 | 25 | 1828 | 9.4 | IMD4 |
| DC_3A_n20A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | 16.1 | IMD3 |
| DC_3A-20A_n78A DC_3C-20A_n78A | 3 | 1725 | 5 | 25 | 1820 | 17.3 | IMD3 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n78 | 3510 | 10 | 50 | 3510 | N/A | N/A |
| DC_3A-21A_n77A DC_3A-21A_n78A | 3 | 1767.5 | 5 | 25 | 1862.5 | N/A | N/A |
|  | 21 | 1459.5 | 5 | 25 | 1507.5 | 8.8 | IMD4 |
|  | n77, n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |
|  | 3 | N/A | N/A | N/A | N/A | N/A | IMD2 |
|  | 21 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_3A-21A_n77A | 3 | 1771.6 | 5 | 25 | 1866.6 | 3.4 | IMD5 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77 | 3935 | 10 | 50 | 3935 | N/A | N/A |
| DC_3A-21A_n79A | 3 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 21 | N/A | N/A | N/A | N/A | N/A | IMD3 |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 3 | 1774.2 | 5 | 25 | 1869.2 | 17.8 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |
| DC_3A-28A_n77A | 3 | 1712.5 | 5 | 25 | 1807.5 | N/A | N/A |
|  | 28 | 715 | 5 | 25 | 770 | 15.3 | IMD3 |
|  | n77 | 4195 | 10 | 50 | 4195 | N/A | N/A |
|  | 3 | 1755 | 5 | 25 | 1850 | 17.0 | IMD3 |
|  | 28 | 735 | 5 | 25 | 790 | N/A | N/A |
|  | n77 | 3320 | 10 | 50 | 3320 | N/A | N/A |
| DC_3A-28A_n78A | 3 | 1775 | 5 | 25 | 1870 | 17.3 | IMD3 |
|  | 28 | 740 | 5 | 25 | 760 | N/A | N/A |
|  | n78 | 3350 | 10 | 25 | 3350 | N/A | N/A |
| DC_3A-28A_n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | 28 | 725 | 5 | 25 | 780 | 10.3 | IMD4 |
|  | n79 | 4530 | 40 | 216 | 4530 | N/A | N/A |
|  | 3 | 1775 | 5 | 25 | 1870 | 5.7 | IMD5 |
|  | 28 | 725 | 5 | 25 | 780 | N/A | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |
| DC_3A_n28A-n78A DC_3C_n28A-n78A | 3 | 1750 | 5 | 25 | 1845 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n78 | 3764 | 10 | 50 | 3764 | 4.5 | IMD5 |
| DC_3A-40A_n1A | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | 3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | 40 | 2380 | 5 | 25 | 2380 | 8.0 | IMD5 |
| DC_3A-41A_n28A DC_3A-41C_n28A | 41 | 2543 | 10 | 50 | 2543 | N/A | N/A |
|  | n28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | 3 | 1737.5 | 5 | 25 | 1832.5 | 26 | IMD2 |
|  | 3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | n28 | 738 | 5 | 25 | 793 | N/A | N/A |
|  | 41 | 2518 | 5 | 25 | 2518 | 27.4 | IMD2 |
|  | 3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | 41 | 2687 | 5 | 25 | 2687 | 15.9 | IMD3 |
| DC_3A-41A_n77A DC_3A-41C_n77A DC_3A-41A_n77(2A) | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
|  | n77 | 3900 | 10 | 50 | 3900 | N/A | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | 5.3 | IMD5 |
|  | 41 | 2620 | 5 | 25 | 2620 | N/A | N/A |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 3 | 1745 | 5 | 25 | 1840 | 16.4 | IMD3 |
| DC_3A-41A_n78A | 41 | 2620 | 5 | 25 | 2620 | N/A | N/A |
|  | n78 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | 3 | 1745 | 5 | 25 | 1840 | 16.4 | IMD3 |
| DC_3A_n78A-n79A | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | N/A | N/A |
|  | n79 | 4910 | 40 | 216 | 4910 | 16.3 | IMD3 |
|  | 3 | 1770 | 5 | 25 | 1865 | N/A | N/A |
|  | n79 | 4510 | 40 | 216 | 4510 | N/A | N/A |
|  | n78 | 3710 | 10 | 50 | 3710 | 4.2 | IMD5 |
| DC_3A-SUL_n78A-n82A | 3 | 1775 | 5 | 25 | 1870 | 4 | IMD4 |
|  | n82 | 840 | 5 | 25 |  | N/A | N/A |
| DC_3A-21A_n79A | 3 | 1774.2 | 5 | 25 | 1869.2 | 17.8 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n79 | 4770 | 40 | 216 | 4770 | N/A | N/A |
| DC_5A-7A_n78A | 5 | 844 | 5 | 25 | 889 | N/A | N/A |
|  | 7 | 2525 | 5 | 25 | 2645 | 30.1 | IMD2 |
|  | n78 | 3489 | 10 | 50 | 3489 | N/A | N/A |
|  | 5 | 834 | 5 | 25 | 879 | 30.2 | IMD2 |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | N/A |
|  | 5 | 830 | 5 | 25 | 875 | 3.3 | IMD5 |
|  | 7 | 2525 | 5 | 25 | 2645 | N/A | N/A |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
| DC_5A_41A_n78A | 5 | 860 | 5 | 25 | 885 | 30.2 | IMD2 |
|  | 41 | 2615 | 5 | 25 | 2615 | N/A | N/A |
|  | n78 | 3500 | 10 | 50 | 3500 | N/A | N/A |
|  | 5 | 856.5 | 5 | 25 | 881.5 | 3.1 | IMD5 |
|  | 41 | 2620.5 | 5 | 25 | 2620.5 | N/A | N/A |
|  | n78 | 3490 | 10 | 50 | 3490 | N/A | N/A |
| DC_5A-41A_n79A | 5 | 835 | 5 | 25 | 880 | 23.9 | IMD3 |
|  | 41 | 2665 | 5 | 25 | 2665 | N/A | N/A |
|  | n79 | 4450 | 40 | 216 | 4450 | N/A | N/A |
|  | 5 | 826.5 | 5 | 25 | 871.5 | N/A | N/A |
|  | 41 | 2517.5 | 5 | 25 | 2517.5 | 1.8 | IMD4 |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | N/A |
| DC_7A-8A_n3A | n3 | 1735 | 5 | 25 | 1830 | N/A | N/A |
|  | 7 | 2530 | 10 | 50 | 2650 | N/A | N/A |
|  | 8 | 895 | 5 | 25 | 940 | 18.0 | IMD3 |
| DC_7A-8A_n3A | n3 | 1780 | 5 | 25 | 1875 | N/A | N/A |
|  | 8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | 7 | 2550 | 10 | 50 | 2670 | 29.0 | IMD2+IMD3Y |
| DC_7A-20A_n1A | 7 | 2510 | 10 | 50 | 2630 | N/A | N/A |
|  | 20 | 841 | 10 | 50 | 800 | 4.5 | IMD5 |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
| DC_7A-20A_n3A | 7 | 2543 | 10 | 50 | 2663 | N/A | N/A |
|  | 20 | 847 | 10 | 20 | 806 | 10.5 | IMD2 |
|  | n3 | 1737 | 5 | 25 | 1832 | N/A | N/A |
|  | 7 | 2510 | 10 | 50 | 2630 | 26.0 | IMD21 |
|  | 20 | 855 | 5 | 25 | 814 | N/A | N/A |
|  | n3 | 1775 | 10 | 50 | 1870 | N/A | N/A |
| DC_7A-20A_n8A | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n8 | 885 | 5 | 25 | 930 | N/A | N/A |
|  | 20 | 836 | 5 | 25 | 795 | 17.4 | IMD3 |
| DC_7A-20A_n8A | 7 | 2520 | 5 | 25 | 2640 | 21.1 | IMD3 |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | 20 | 840 | 5 | 25 | 799 | N/A | N/A |
| DC_7A-20A_n8A | 7 | 2504 | 5 | 25 | 2624 | 18.8 | IMD3 |
|  | n8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 20 | 857 | 5 | 25 | 816 | N/A | N/A |
| DC_7A-20A_n28A | 20 | 842 | 5 | 25 | 801 | N/A | N/A |
|  | n28 | 728 | 5 | 25 | 783 | N/A | N/A |
|  | 7 | 2520 | 10 | 50 | 2640 | 5.9 | IMD5 |
| DC_7A-20A_n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 30.5 | IMD2 |
|  | n78 | 3370 | 10 | 50 | 3370 | N/A | N/A |
| DC_7A-20A_n78A | 7 | 2560 | 5 | 25 | 2680 | N/A | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 3.0 | IMD5 |
|  | n78 | 3435 | 10 | 50 | 3435 | N/A | N/A |
| DC_7A-20A_n78A | 7 | 2555 | 5 | 25 | 2675 | 30.8 | IMD2 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n78 | 3520 | 10 | 50 | 3520 | N/A | N/A |
| DC_7A-28A_n3A | 7 | 2543 | 5 | 25 | 2663 | N/A | N/A |
|  | 28 | 741 | 5 | 25 | 796.0 | 20.0 | IMD2 |
|  | n3 | 1747 | 5 | 25 | 1842 | N/A | N/A |
|  | 7 | 2540 | 5 | 25 | 2685 | 18 | IMD3 |
|  | 28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n3 | 1715 | 5 | 25 | 1810 | N/A | N/A |
| DC_7A-28A_n5A | 7 | 2540 | 5 | 25 | 2660 | N/A | N/A |
|  | 28 | 721 | 5 | 25 | 776 | 4.4 | IMD5 |
|  | n5 | 829 | 5 | 25 | 874 | N/A | N/A |
|  | 7 | 2510 | 5 | 25 | 2630 | 5.9 | IMD5 |
|  | 28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | n5 | 840 | 5 | 25 | 874 | N/A | N/A |
| DC_7A-28A_n5A DC_7C-28A_n5A | 7 | 2540 | 5 | 25 | 2725 | N/A | N/A |
|  | 28 | 721 | 5 | 25 | 776 | 4.4 | IMD5 |
|  | n5 | 829 | 5 | 25 | 854 | N/A | N/A |
|  | 7 | 2510 | 5 | 25 | 2630 | 5.9 | IMD5 |
|  | 28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | n5 | 840 | 5 | 25 | 874 | N/A | N/A |
| DC_7A-28A_n78A | 7 | 2570 | 5 | 25 | 2670 | N/A | N/A |
|  | 28 | 720 | 5 | 25 | 780 | 8.3 | IMD2 |
|  | n78 | 3350 | 10 | 50 | 3421 | N/A | N/A |
|  | 7 | 2570 | 5 | 25 | 2670 | N/A | N/A |
|  | 28 | 720 | 5 | 25 | 790 | 3.0 | IMD5 |
|  | n78 | 3460 | 10 | 50 | 3421 | N/A | N/A |
|  | 7 | 2570 | 5 | 25 | 2650 | 30.5 | IMD2 |
|  | 28 | 740 | 5 | 25 | 768 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3421 | N/A | N/A |
| DC_7A_n28A-n78A DC_7C_n28A-n78A | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | IMD2 |
|  | 7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3365 | 10 | 50 | 3365 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 28.8 | IMD2 |
| DC_7A-46A_n78A6 | 7 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 46 | N/A | N/A | N/A | N/A | N/A | IMD2, IMD5 |
|  | n78 | N/A | N/A | N/A | N/A | N/A | N/A |
| DC_14A-66A_n2A DC_14A-66A-66A_n2A | 14 | 793 | 5 | 25 | 763 | N/A | N/A |
|  | 66 | 1762 | 5 | 25 | 2162 | 7.6 | IMD4 |
|  | n2 | 1874 | 5 | 25 | 1954 | N/A | N/A |
| DC_18A-28A_n77A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | 28 | 723 | 5 | 25 | 778 | 4.4 | IMD5 |
|  | n77 | 4058 | 10 | 50 | 4058 | N/A | N/A |
| DC_18A-28A_n77A | 18 | 820 | 5 | 25 | 865 | 3.9 | IMD5 |
|  | 28 | 723 | 5 | 25 | 778 | N/A | N/A |
|  | n77 | 3757 | 10 | 50 | 3757 | N/A | N/A |
| DC_18A-28A_n78A | 18 | 819 | 5 | 25 | 864 | 3.8 | IMD5 |
|  | 28 | 723 | 5 | 25 | 778 | N/A | N/A |
|  | n78 | 3756 | 10 | 50 | 3756 | N/A | N/A |
| DC_18A-41A_n3A DC_18A-41C_n3A | 18 | 820 | 5 | 25 | 865 | N/A | N/A |
|  | n3 | 1725 | 5 | 25 | 1820 | N/A | N/A |
|  | 41 | 2630 | 5 | 25 | 2630 | 16.0 | IMD3 |
|  | 18 | 820 | 5 | 25 | 865 | 28.9 | IMD2 |
|  | n3 | 1765 | 5 | 25 | 1820 | N/A | N/A |
|  | 41 | 2630 | 5 | 25 | 2630 | N/A | N/A |
| DC_18A-41A_n77A DC_18A-41C_n77A | 18 | 820 | 5 | 25 | 865 | 3.4 | IMD5 |
|  | n77 | 3527.5 | 10 | 50 | 3527.5 | N/A | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
| DC_18A-41A_n78A DC_18A-41C_n78A | 18 | 820 | 5 | 25 | 865 | 3.4 | IMD5 |
|  | n78 | 3527.5 | 10 | 50 | 3527.5 | N/A | N/A |
|  | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
| DC_19A-21A_n77A DC_19A-21A_n78A | 19 | 837.5 | 5 | 25 | 882.5 | 18.7 | IMD3 |
|  | 21 | 1450.4 | 5 | 25 | 1498.4 | N/A | N/A |
|  | n77, n78 | 3783.3 | 10 | 50 | 3783.3 | N/A | N/A |
| DC_19A-21A_n77A | 19 | 837.5 | 5 | 25 | 882.5 | N/A | N/A |
|  | 21 | 1454.5 | 5 | 25 | 1502.5 | 9.0 | IMD4 |
|  | n77 | 4015 | 10 | 50 | 4015 | N/A | N/A |
| DC_19A-21A_n79A | 19 | N/A | N/A | N/A | N/A | N/A | IMD5 |
|  | 21 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n79 | N/A | N/A | N/A | N/A | N/A | N/A |
|  | 19 | 837.5 | 5 | 25 | 882.2 | N/A | N/A |
|  | 21 | 1452 | 5 | 25 | 1500 | 3.8 | IMD5 |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | N/A |
| DC_20A_n1A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
|  | n78 | 3630 | 10 | 50 | 3630 | 16.0 | IMD3 |
|  | 20 | 835 | 5 | 25 | 794 | N/A | N/A |
|  | n1 | 1930 | 5 | 25 | 2120 | 15.3 | IMD3 |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | N/A |
| DC_20A_n3A-n78A | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | 16.1 | IMD3 |
|  | 20 | 845 | 5 | 25 | 804 | N/A | N/A |
|  | n3 | 1765 | 5 | 25 | 1860 | 15.7 | IMD3 |
|  | n78 | 3550 | 10 | 50 | 3550 | N/A | N/A |
| DC_20A_SUL_n78A-n80A | 20 | 847 | 5 | 25 | 806 | 9 | IMD4 |
|  | n80 | 1735 | 5 | 25 |  | N/A | N/A |
| DC_21A-28A_n77A | 21 | 1452 | 5 | 25 | 1500 | N/A | N/A |
|  | 28 | 730.5 | 5 | 25 | 785.5 | 16.9 | IMD3 |
|  | n77 | 3689.5 | 10 | 50 | 3689.5 | N/A | N/A |
|  | 21 | 1450.5 | 5 | 25 | 1498.5 | 9.9 | IMD4 |
|  | 28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |
|  | n77 | 3690 | 10 | 50 | 3690 | N/A | N/A |
| DC_21A-28A_n79A | 21 | 1450 | 5 | 25 | 1498 | 5.2 | IMD5 |
|  | 28 | 730.5 | 5 | 25 | 785.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| DC_28A_n7A-n78A | 28 | 745 | 5 | 25 | 800 | N/A | N/A |
|  | n7 | 2565 | 5 | 25 | 2685 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | 29.7 | IMD2 |
|  | 28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n7 | 2530 | 5 | 25 | 2650 | 30.5 | IMD2 |
|  | n78 | 3390 | 10 | 50 | 3390 | N/A | N/A |
| DC_28A-42A_79A | 28 | 730 | 5 | 25 | 785 | N/A | N/A |
|  | 42 | 3420 | 5 | 25 | 3420 | 15.3 | IMD3 |
|  | n79 | 4880 | 40 | 216 | 4880 | N/A | N/A |
|  | 28 | 745 | 5 | 25 | 800 | 16.2 | IMD2 |
|  | 42 | 3597.5 | 5 | 25 | 3597.5 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| DC_19A_n78A-n79A | 19 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n78 | 3680 | 10 | 50 | 3680 | N/A | N/A |
|  | n79 | 4515 | 40 | 216 | 4515 | 29.3 | IMD2 |
|  | 19 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | N/A |
|  | n78 | 3715 | 10 | 50 | 3715 | 28.8 | IMD2 |
| DC_20A_n28A-n78A DC_20A_SUL_n78A-n83A | 20 | 857 | 5 | 25 | 816 | N/A | N/A |
|  | n28, n83 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n78 | 3314 | 10 | 50 | 3314 | 8.7 | IMD4 |
|  | 20 | 837 | 5 | 25 | 796 | N/A | N/A |
|  | n78 | 3310 | 10 | 50 | 3310 | N/A | N/A |
|  | n28 | 744 | 5 | 25 | 799 | 9.4 | IMD4 |
| DC_21A_n78A-n79A | 21 | 1453 | 5 | 25 | 1501 | N/A | N/A |
|  | n78 | 3420 | 10 | 50 | 3420 | N/A | N/A |
|  | n79 | 4873 | 40 | 216 | 4873 | 30.1 | IMD2 |
|  | 21 | 1453 | 5 | 25 | 1501 | N/A | N/A |
|  | n79 | 4940 | 40 | 216 | 4940 | N/A | N/A |
|  | n78 | 3487 | 10 | 50 | 3487 | 29.8 | IMD2 |
| NOTE 1: For NR band, UL/DL BW and UL LCRB can be adjusted according to the supported BW and lowest SCS supported by the UE. NOTE 2: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 3: This band is subject to IMD3 also which MSD is not specified. NOTE 4: This MSD requirement apply with both IMD2 and IMD3 products should be generated. |  |  |  |  |  |  |  |

7.3B.2.0.3.5.3	Void

##### 7.3B.2.0.3a	Inter-band NE-DC within FR1

Reference sensitivity exceptions are specified for the condition when there is uplink transmission only in the aggressor band. This subclause addresses directly only NE-DC configurations that don't have a corresponding specified EN-DC configuration or specific NE-DC exceptions.

7.3B.2.0.3a.1	Reference sensitivity exceptions due to UL harmonic interference for NE-DC in NR FR1

Sensitivity degradation is allowed for a band if it is impacted by UL harmonic interference from another band part of the same NE-DC configuration. For the NE-DC configurations that have an EN-DC defined configuration, the reference sensitivity exceptions for the victim band (high) are specified in Table 7.3B.2.3.1-1 with uplink configuration of the aggressor band (low) specified in Table 7.3B.2.3.1-2 are applicable.

##### 7.3B.2.0.4	Inter-band EN-DC including FR2

7.3B.2.0.4.1	Void

##### 7.3B.2.0.5	Inter-band EN-DC including both FR1 and FR2

7.3B.2.0.5.1	Reference sensitivity exceptions due to UL harmonic interference for EN-DC including both FR1 and FR2

For inter-band EN-DC of E-UTRA and NR in both FR1 and FR2, the UE is allowed to apply each sensitivity degradation for EN-DC in FR1 specified in clause 7.3B.2.3 TS 38.101-3 and for EN-DC including FR2 specified in clause 7.3B.2.3 of TS 38.101-3 independently.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.3B.2.

#### 7.3B.2.1	Reference sensitivity for Intra-band Contiguous EN-DC (2 CCs)

Editor's note:	MSD test point selection is based on core spec requirement and shall be added to TP analysis to TS 38.905 [7].

7.3B.2.1.1	Test purpose

To verify the ability of UE that support intra-band contiguous EN-DC configurations to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise. A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area.

7.3B.2.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting intra-band contiguous EN-DC in FR1 with 2 DL CCs.

7.3B.2.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0.

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

7.3B.2.1.4	Test description

7.3B.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.5B, and the channel bandwidth combination for E-UTRA and NR component carriers shall follow the value specified in Table 5.3B.1.2-1. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.2, and are shown in Table 7.3B.2.1.4.1-1 for DC_(n)71AA and Table 7.3B.2.1.4.1-2 for intra-band contiguous EN-DC other than DC_(n)71AA.

The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annex A.2 and A.3 for E-UTRA RMC, and in TS 38.521-1 [8] Annex A.2 and A.3 for NR RMC respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 7.3B.2.1.4.1-1: Test configurations table for EN-DC configurations affected by Reference sensitivity exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  | Specified below |  |  |  |  |  |  |  |  |
| E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  | Specified below |  |  |  |  |  |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 |  |  |  |  | Specified below |  |  |  |  |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  | Lowest |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3 in TS 36.521-1 [10], dependent on PCC Band |  |  |  |  |  |  |  |  |
| NR/E-UTRA Test Parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Downlink Configuration |  |  |  |  | Uplink Configuration |  |  |  |  |  |  |  |  |
| NR Modulation | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation |  |  | NR Modulation |  | NR RB allocation |  | E-UTRA Modulation |  | E-UTRA RB allocation |
| CP-OFDM QPSK | Full RB (NOTE 1) | QPSK | Full RB |  |  | DFT-s-OFDM QPSK |  |  |  | QPSK |  |  |
| Test Point configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for DC_(n)71AA for dual UL UE |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | E-UTRA/NR band | FC (UL) (MHz) NUL |  | UL Channel BW (MHz) |  |  | UL allocation (LCRB) |  | FC (DL) (MHz) NDL |  | Duplex mode |  |  |  |
| 1 | 71 | 665.5MHz, EUL= 133147 |  | 5 |  |  | 5@19 |  | 619.5 MHz EDL = 68611 |  | FDD |  |  |  |
|  | n71 | 675.5 NUL= 135100 |  | 15 |  |  | 15@0 |  | 629.5 NDL = 125900 |  |  |  |  |  |
| 2 | 71 | 670.5 EUL= 133197 |  | 15 |  |  | 15@59 |  | 624.5 EDL = 68661 |  |  |  |  |  |
|  | n71 | 680.5 NUL= 136100 |  | 5 |  |  | 5@0 |  | 634.5 NDL = 126900 |  |  |  |  |  |
| 3 | 71 | 668 EUL= 133172 |  | 10 |  |  | 10@39 |  | 622 EDL = 68636 |  |  |  |  |  |
|  | n71 | 678 NUL= 135600 |  | 10 |  |  | 10@0 |  | 632 NDL = 126400 |  |  |  |  |  |
| 4 | 71 | 668 EUL= 133172 |  | 10 |  |  | 10@0 |  | 622 EDL = 68636 |  |  |  |  |  |
|  | n71 | 678 NUL= 135600 |  | 10 |  |  | 10@41 |  | 632 NDL = 126400 |  |  |  |  |  |
| 5 | 71 | 665.5 EUL= 133147 |  | 5 |  |  | 5@19 |  | 619.5 EDL = 68611 |  | FDD |  |  |  |
|  | n71 | 675.5 NUL= 135100 |  | 153 |  |  | 15@0 |  | 6323 NUL= 126400 |  |  |  |  |  |
| 6 | 71 | 670.5 EUL= 133197 |  | 15 |  |  | 15@59 |  | 624.5 EDL = 68661 |  |  |  |  |  |
|  | n71 | 680.5 NUL= 136100 |  | 53 |  |  | 5@0 |  | 6373 NUL= 127400 |  |  |  |  |  |
| 7 | 71 | 668 EUL= 133172 |  | 10 |  |  | 10@39 |  | 622 EDL = 68636 |  |  |  |  |  |
|  | n71 | 678 NUL= 135600 |  | 103 |  |  | 10@0 |  | 634.53 NUL= 126900 |  |  |  |  |  |
| 8 | 71 | 668 EUL= 133172 |  | 10 |  |  | 10@0 |  | 622 EDL = 68636 |  |  |  |  |  |
|  | n71 | 678 NUL= 135600 |  | 103 |  |  | 10@41 |  | 634.53 NUL= 126900 |  |  |  |  |  |
| Test Settings for DC_(n)71AA for single UL UE |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 71 | default |  | Highest |  |  | 0 |  | default |  | FDD |  |  |  |
|  | n71 | default |  | Highest |  |  | REFSENS_NR |  | default |  |  |  |  |  |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. NOTE 2: Test Channel Bandwidths are checked separately for each E-UTRA band, which applicable channel bandwidths are specified in Table 5.3B.1.2-1. NOTE 3: In accordance to BCS1, the NR uplink bandwidth is specified as in this table, but the corresponding NR downlink bandwidth is 5 MHz larger. NOTE 4: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE 5: REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.1.4.1-2: Test configurations table for EN-DC configurations not affected by Reference sensitivity exceptions, EN-DC configuration other than DC_(n)71AA

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  | Low range, High range |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1across bandwidth combination sets supported by the UE. |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Lowest supported SCS |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3 in TS 36.521-1 [10], dependent on PCC Band |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  | SCG -NR |  |  |  |  |
|  | Band | Range | NRB |  | Band |  | Range | NRB |  |
|  | UL MOD | DL MOD | CH BW | DLalloc / UL alloc | UL MOD |  | DL MOD | UL/DL Ch BW | DLalloc / UL alloc |
| Test Settings for a DC_(n)XAA Configuration (Intra-band contiguous EN-DC) – Note 2 |  |  |  |  |  |  |  |  |  |
| 1 (Note 4) | X | default |  |  | nX |  | default |  |  |
|  | QPSK | QPSK | Highest NRB_agg(Note 6) | All RBs / REFSENS_LTE | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | Highest NRB_agg(Note 6) | All RBs / REFSENS_NR |
| 2 (Note 5) | X | default |  |  | nX |  | default |  |  |
|  | N/A | QPSK | Highest NRB | All RBs / 0 | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | Highest NRB | All RBs / REFSENS_NR |
| Note 1: REFSENS_NR and REFSENS_LTE refer to the single carrier Uplink RB allocation for reference sensitivity according to Table 7.3.2.4.1-3 of TS 38.521-1 [8] and Table 7.3.4.1-1 of TS 36.521 [10] for NR and E-UTRA CC respectively. Note 2: Not LTE anchor agnostic configuration due to exception requirement for intra-band contiguous CA in clause 7.4B, 7.5B, 7.6B.2, 7.6B.3, 7.6B.4, 7.7B, 7.8B, 7.9B test cases Note 3: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. Note 4: Test point for UE supporting dual UL. Note 5: Test point for UE supporting single UL. Note 6: If the UE supports multiple CC combinations in the EN-DC configuration with the same NRB_agg, select the combination to test as follows: - Lowest ENBW: NR component with lowest NRB is tested. - Highest ENBW: NR component with highest NRB is tested. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2.1 for UE diagram.

2.	The parameter settings for NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	The parameter settings for E-URA cell are set up according to TS 36.508 [11] clause 4.4.3.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, C.2, C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1 of TS 38.521-1 [8].

5.	E-UTRA downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1 [10].

6.	The DL and UL Reference Measurement channels for NR are set according to Table 7.3B.2.1.4.1-1 and Table 7.3B.2.1.4.1-2.

7.	The DL and UL Reference Measurement channels for E-UTRA are set according to Table 7.3B.2.1.4.1-1 and Table 7.3B.2.1.4.1-2.

8.	NR propagation conditions are set according to Annex B.0 of TS 38.521-1 [8].

9.	E-UTRA propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

10.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 7.3B.2.1.4.3.

7.3B.2.1.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3B.2.1.4.1-1 and Table 7.3B.2.1.4.1-2 on the E-UTRA CC and NR CC. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3B.2.3.4.1-1 on both EN-DC component carriers. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	For test points in Table 7.3B.2.1.4.1-1, set the Downlink signal level to the appropriate REFSENS value defined in Table 7.3B.2.1.5-1. For test points in Table 7.3B.2.1.4.1-2, set the Downlink signal level to the appropriate REFSENS value defined in TS 38.521-1 [8], Table 7.3.3-1 for NR band and TS 36.521-1 [10] Table 7.3.3-1 for E-UTRA band. Send continuously uplink power control "up" commands in the uplink scheduling information to both carriers to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

4.	Measure the average throughput of both NR and E-UTRA for a duration sufficient to achieve statistical significance according to Annex H.2 of TS 38.521-1 [8] for NR band, and Annex G.2 of TS 36.521-1 [10] for EUTRA band.

7.3B.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED.

Message contents exceptions are according to TS 36.521-1 [10] clause 7.3.4.3 for each network signalling value.

7.3B.2.1.5	Test requirement

For intra-band contiguous EN-DC configurations DC_(n)71AA with dual UL, the throughput of each CG shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 with reference receive power level specified in Table 7.3B.2.1.5-1.

Reference sensitivity test requirements for EN-DC configurations other than DC_(n)71AA with dual UL UE, are specified in Table 7.3.5-1 in TS 36.521-1 [10] for the LTE CC, and Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC.

Table 7.3B.2.1.5-1: Reference sensitivity for intra-band DC_(n)71AA with dual UL

| EN-DC | E-UTRA/ NR band | SCS (kHz) | Channel BW (MHz) | Ref sensitivity (dBm) | Note | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| DC_(n)71AA | 71 | N/A | 5 | -96.5 | Note 1 | FDD |
|  | n71 | 15 | 15 | -89.8 +TT |  |  |
|  | 71 | N/A | 15 | -91.3 | Note 2 |  |
|  | n71 | 15 | 5 | -95.6 +TT |  |  |
|  | 71 | N/A | 10 | -93.5 | Note 3 |  |
|  | n71 | 15 | 10 | -92.3 +TT |  |  |
|  | 71 | N/A | 10 | -76.3 | Note 4 |  |
|  | n71 | 15 | 10 | -64.6 +TT |  |  |
|  | 71 | N/A | 5 | -96.5 | Note 5 |  |
|  | n71 | 15 | 159 | -89.1 +TT |  |  |
|  | 71 | N/A | 15 | -91.3 | Note 6 |  |
|  | n71 | 15 | 59 | -95.0 +TT |  |  |
|  | 71 | N/A | 10 | -93.5 | Note 7 |  |
|  | n71 | 15 | 109 | -91.5 +TT |  |  |
|  | 71 | N/A | 10 | -76.3 | Note 8 |  |
|  | n71 | 15 | 109 | -64.9 +TT |  |  |
| NOTE 1: For test configuration specified by Table 7.3B.2.1.4.1-1 Test ID 1. NOTE 2: For test configuration specified by Table 7.3B.2.1.4.1-1 Test ID 2. NOTE 3: For test configuration specified by Table 7.3B.2.1.4.1-1 Test ID 3. NOTE 4: For test configuration specified by Table 7.3B.2.1.4.1-1 Test ID 4. NOTE 5: For test configuration specified by Table 7.3B.2.1.4.1-1 Test ID 5. NOTE 6: For test configuration specified by Table 7.3B.2.1.4.1-1 Test ID 6. NOTE 7: For test configuration specified by Table 7.3B.2.1.4.1-1 Test ID 7. NOTE 8: For test configuration specified by Table 7.3B.2.1.4.1-1 Test ID 8. NOTE 9: In accordance to BCS1, the NR uplink bandwidth is specified as in this table, but the corresponding NR downlink bandwidth is 5 MHz larger. |  |  |  |  |  |  |

Table 7.3B.2.1.5-2: Test Tolerance (TT) for NR RX sensitivity level

| f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0 GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

#### 7.3B.2.2	Reference sensitivity for Intra-band non-contiguous EN-DC (2 CCs)

7.3B.2.2.1	Test purpose

Same as in clause 7.3B.2.1.1.

7.3B.2.2.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward, supporting intra-band non-contiguous EN-DC in FR1 with 2 DL CCs.

7.3B.2.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0.

For DC_3A_n3A in Rel-16, exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

For DC_3A_n3A in Rel-15 and EN-DC configurations other than DC_3A_n3A, no exception requirements for NR or E-UTRA are defined. LTE anchor agnostic approach is applied.

7.3B.2.2.4	Test Description

7.3B.2.2.4.1	Initial Condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.5B, and the channel bandwidth combination for E-UTRA and NR component carriers shall follow the value specified in Table 5.3B.1.3-1. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration specified in clause 5.3B.1.3, and are shown in Table 7.3B.2.2.4.1-1 for EN-DC configurations affected by exceptions and Table 7.3B.2.2.4.1-2 for EN-DC configurations not affected by exceptions.

The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annex A.2 and A.3 for E-UTRA RMC, and in TS 38.521-1 [8] Annex A.2 and A.3 for NR RMC respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG respectively.

Table 7.3B.2.2.4.1-1: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  | Low, mid and high range |  |  |  |  |  |  |
| E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  | Specified below |  |  |  |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 |  |  |  |  |  | Specified below |  |  |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Lowest |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3 in TS 36.521-1 [10], dependent on PCC Band |  |  |  |  |  |  |
| NR/E-UTRA Test Parameters |  |  |  |  |  |  |  |  |  |  |  |  |
| Downlink Configuration |  |  |  |  |  | Uplink Configuration |  |  |  |  |  |  |
| NR Modulation | NR RB allocation | E-UTRA Modulation |  | E-UTRA RB allocation |  | NR Modulation |  | NR RB allocation |  | E-UTRA Modulation | E-UTRA RB allocation |  |
| CP-OFDM QPSK | Full RB (NOTE 1) | QPSK |  | Full RB |  | DFT-s-OFDM QPSK |  |  |  | QPSK |  |  |
| Test Point configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for DC_3A_n3A dual UL UE |  |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | E-UTRA/NR band |  | FC (UL) (MHz) NUL |  | UL Channel BW (MHz) |  | UL allocation (LCRB) |  | FC (DL) (MHz) NDL |  | Duplex mode |  |
| 1 | 3 |  | 1782.5, NUL= TBD |  | 5 |  | 12@0 |  | 1877.5 NDL = TBD |  | FDD |  |
|  | n3 |  | 1772.5 NUL= TBD |  | 5 |  | 12@13 |  | 1867.5 NDL = TBD |  |  |  |
| 2 | 3 |  | 1782.5, NUL= TBD |  | 5 |  | 12@9 |  | 1877.5 NDL = TBD |  |  |  |
|  | n3 |  | 1752.5 NUL= TBD |  | 5 |  | 12@0 |  | 1847.5 NDL = TBD |  |  |  |
| 3 | 3 |  | 1782.5 NUL= TBD |  | 5 |  | 12@12 |  | 1877.5 NDL = TBD |  |  |  |
|  | n3 |  | 1737.5 NUL= TBD |  | 5 |  | 12@0 |  | 1832.5 NDL = TBD |  |  |  |
| 4 | 3 |  | 1737.5 NUL= TBD |  | 5 |  | 12@0 |  | 1832.5 NDL = TBD |  |  |  |
|  | n3 |  | 1782.5 NUL= TBD |  | 5 |  | 12@12 |  | 1877.5 NDL = TBD |  |  |  |
| Test Settings for DC_3A_n3A for single UL UE |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | default |  | Highest |  | 0 |  | default |  |  | FDD |
|  | n3 |  | default |  | Highest |  | REFSENS_NR |  | default |  |  |  |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. NOTE 2: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.2.4.1-2: Test configurations table for EN-DC configurations not affected by Reference sensitivity exceptions, EN-DC configuration other than DC_3A_n3A

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  | TBD |  |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1. |  |  |  |  | Refer to "NR NRB "and "E-UTRA NRB " columns |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  | Lowest supported SCS |  |  |  |  |
| Network signalling value |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3 in TS 36.521-1 [10], dependent on PCC Band |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  |  | SCG -NR |  |  |  |
|  | Band | Range | NRB |  |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | CH BW | DLalloc / UL alloc |  | UL MOD | DL MOD | UL/DL Ch BW | DLalloc / UL alloc |
| Test Settings for a DC_XA_nXA Configuration (Intra-band non-contiguous EN-DC) – Note 2 |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  |  | nX | default |  |  |
|  | N/A | N/A | Note 4 | N/A |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | Highest NRB | All RBs / REFSENS_NR |
| Note 1: REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. Note 2: LTE anchor agnostic configuration Note 3: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. Note 4: The E-UTRA channel bandwidth is the lowest supported value in Table 5.3B.1.3-1 for the EN-DC non-contiguous configuration. Note 5: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3. 	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3. E-UTRA downlink signal level, and uplink signal level are set according to Table 4.6-1.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, and C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 of TS 38.521-1 [8]. E-UTRA downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1 [10].

5.	The NR UL Reference Measurement channels are set according to Table 7.3B.2.2.4.1-1 and Table 7.3B.2.2.4.1-2 for E-UTRA CG and NR CG.

6.	Set up the NR and E-UTRA test frequencies so that NR carrier is located at the lower frequency side as specified in Table 5.3B.1.3-1. Repeat each testing with E-UTRA carrier frequency is located at the lower side as specified in Table 5.3B.1.3-1.

7.	NR propagation conditions are set according to Annex B.0 of TS 38.521-1 [8]. E-UTRA propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

8.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause7.3B.2.2.4.3.

9.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6. Step 9 only applicable to the test configuration in Table 7.3B.2.2.4.1-2.

7.3B.2.2.4.2	Test Procedure

For test points in Table 7.3B.2.2.4.1-1,

FFS

For test points in Table 7.3B.2.2.4.1-2,

1.	NR SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3B.2.2.4.1-2 on the NR CC. The NR SS sends downlink MAC padding bits on the DL RMC.

2.	NR SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3B.2.2.4.1-2. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the appropriate REFSENS value defined in TS 38.521-1 [8], Table 7.3.2.5-1 for NR band. Send continuously uplink power control "up" commands in the uplink scheduling information to NR carrier to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

4.	Measure the average throughput of the NR carrier for a duration sufficient to achieve statistical significance according to Annex H.2 of TS 38.521-1 [8] for NR band.

7.3B.2.2.4.3	Message Contents

Message contents are according to TS 38.508-1 [6] clause 4.6. Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band.

Message contents exceptions are according to TS 36.521-1 [10] clause 7.3.4.3 for each network signalling value.

7.3B.2.2.5	Test Requirement

For intra-band non-contiguous EN-DC configuration DC_3A_n3A, the measured throughput on the NR carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A with reference receive power level specified in Table 7.3B.2.2.5-1.

Reference sensitivity test requirements for EN-DC configurations other than DC_3A_n3A, are specified in Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC.

Table 7.3B.2.2.5-1: Reference sensitivity for intra-band Non-contiguous EN-DC

| EN-DC | Test ID | E-UTRA/ NR band | SCS (kHz) | Channel BW (MHz) | Ref sensitivity (dBm) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n3A | 1 | 3 | N/A | 5 | TBD | FDD |
|  |  | n3 | 15 | 5 |  |  |
|  | 2 | 3 | N/A | 5 |  |  |
|  |  | n3 | 15 | 10 |  |  |
|  | 3 | 3 | N/A | 5 |  |  |
|  |  | n3 | 15 | 15 |  |  |
|  | 4 | 3 | N/A | 5 |  |  |
|  |  | n3 | 15 | 20 |  |  |
| Note: NR band Test tolerance (TT) is specified in Table 7.3B.2.2.5-2 |  |  |  |  |  |  |

Table 7.3B.2.2.5-2: Test Tolerance (TT) for NR RX sensitivity level

| f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0 GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

#### 7.3B.2.3	Reference sensitivity for Inter-band EN-DC within FR1 (2 CCs)

7.3B.2.3.1	Test purpose

Same as in clause 7.3B.2.1.1.

7.3B.2.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting inter-band EN-DC within FR1 with 2 DL CCs.

7.3B.2.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0.

LTE anchor agnostic approach is not applied.

7.3B.2.3.4	Test description

7.3B.2.3.4.1	Void

7.3B.2.3.4.2	Test description

7.3B.2.3.4.2.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths based on EN-DC operating bands specified in clause 5.5B, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-1 [8] clause 5.3 and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2. All of these configurations shall be tested with applicable test parameters for each EN-DC configuration, and are shown in Table 7.3B.2.3.4.2.1-0 to Table 7.3B.2.3.4.2.1-6.

The details of the uplink and downlink reference measurement channels (RMCs) are specified in TS 36.521-1 [10] Annex A.2 and A.3 for E-UTRA RMC, and in TS 38.521-1 [8] Annex A.2 and A.3 for NR RMC respectively. The details of the OCNG patterns used are specified in TS 36.521-1 [10] Annex A.5 and in TS 38.521-1 [8] Annex A.5 for E-UTRA CG and NR CG, respectively. Configurations of PDSCH and PDCCH before measurement are specified in TS 36.521-1 [10] Annex C.2 and in TS 38.521-1 [8] Annex C.2 for E-UTRA CG and NR CG, respectively.

For configurations without any reference sensitivity exception, in a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. For configurations with reference sensitivity exception, in an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected.

Table 7.3B.2.3.4.2.1-0: Test configurations table for all EN-DC configurations for FR1 non-exception requirements

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  | Mid range for E-UTRA and Mid range for NR |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1 |  |  |  |  |  | Refer to "NR NRB "and "E-UTRA NRB " columns |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Lowest supported SCS |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  | SCG -NR |  |  |  |  |
|  | Band | Range | NRB |  | Band |  | Range | NRB |  |
|  | UL MOD | DL MOD | CH BW | DLalloc / UL alloc | UL MOD |  | DL MOD | UL/DL Ch BW | DLalloc / UL alloc |
| Test Settings for a DC_XA_nYA Configuration (Inter-band EN-DC) – Note 2 |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | nY |  | default |  |  |
|  | N/A | N/A | 5 MHz | N/A | DFT-s-OFDM QPSK |  | CP-OFDM QPSK | Highest NRB | All RBs / REFSENS_NR |
| NOTE 1: REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. NOTE 2: LTE anchor agnostic configuration NOTE 3: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE 4: The test configuration table only applies to EN-DC only capable devices and devices not supporting NR/5GC mode in the tested band. NOTE 5: Only one EN-DC combination per FR1 band is tested for each EN-DC configuration as defined in clause 5.5B. |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-1: Initial test conditions for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1 E-UTRA Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1 |  |  |  | Mid range for E-UTRA and NR, unless otherwise specified in Table 7.3B.2.3.4.2.1-2_1 to Table 7.3B.2.3.4.2.1-2_28 |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1 |  |  |  | Highest NRB_agg , unless otherwise specified in Table 7.3B.2.3.4.2.1-2_1 to Table 7.3B.2.3.4.2.1-2_28 |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest otherwise specified |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |
| Downlink Configuration |  |  |  | Uplink Configuration |  |  |  |
| E-UTRA Cell |  | NR Cell |  | E-UTRA Cell |  | NR Cell |  |
| Modulation | RB allocation | Modulation | RB allocation | Modulation | RB allocation | Modulation | RB allocation |
| QPSK | Full RB | CP-OFDM QPSK | Full RB (NOTE 1) | QPSK | Full RB | DFT-s-OFDM QPSK | Full RB (NOTE 1) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8] unless otherwise specified in Table 7.3B.2.3.4.2.1-2_1 to Table 7.3B.2.3.4.2.1-2_28. |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_1: Void

Table 7.3B.2.3.4.2.1-2_2: Void

Table 7.3B.2.3.4.2.1-2_3: Void

Table 7.3B.2.3.4.2.1-2_4: Void

Table 7.3B.2.3.4.2.1-2_5: Void

Table 7.3B.2.3.4.2.1-2_6: Void

Table 7.3B.2.3.4.2.1-2_7: Void

Table 7.3B.2.3.4.2.1-2_8: Void

Table 7.3B.2.3.4.2.1-2_9: Void

Table 7.3B.2.3.4.2.1-2_10: Test configurations table for exceptions due to UL harmonic interference for EN-DC 8_n41 (Test frequencies are selected to fulfil Requirement of Note 8 and 9 in Table 7.3B.2.0.3.1-1)

| E-UTRA Band 8 |  |  |  | NR Band n41 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 885 MHz/ 21500 | 16@17 | 2655 MHz/ 531000 | Lowest | REFSENS (NOTE 2) |
| 2 | 10 | 885 MHz/ 21500 | 25@12 | 2655 MHz/ 531000 | Mid | REFSENS (NOTE 2) |
| 3 |  | 890 MHz/ 21550 | 16@17 | 2670 MHz/ 534000 | Lowest | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 8 and 9 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_11: Void

Table 7.3B.2.3.4.2.1-2_12: Void

Table 7.3B.2.3.4.2.1-2_13: Test configurations table for exceptions due to UL harmonic interference for EN-DC 12_n66 (Requirement of Note 8 and 9)

| E-UTRA Band 12 |  |  |  | NR Band n66 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 704.1 MHz/ 23061 | 8@21 | 2112.5 MHz/ 422500 | Lowest | REFSENS (NOTE 2) |
| 2 | 10 | 707.5 MHz/ 23095 | 8@21 | 2122.5 MHz/ 424500 | Lowest | REFSENS (NOTE 2) |
|  |  | 707.5 MHz/ 23095 | 20@15 | 2122.5 MHz/ 424500 | Mid | REFSENS (NOTE 2) |
| 3 |  | 711 MHz/ 23130 | 8@21 | 2133 MHz/ 426600 | Lowest | REFSENS (NOTE 2) |
|  |  | 711 MHz/ 23130 | 20@15 | 2133 MHz/ 426600 | Mid Highest | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 8 and 9 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_14: Test configurations table for exceptions due to UL harmonic interference for EN-DC 18_n77 (Requirement of Note 4 and 5)

| E-UTRA Band 18 |  |  |  | NR Band n77 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 820 MHz/ 23900 | 16@17 | 4100.01 MHz/ 673334 | Lowest | REFSENS (NOTE 2) |
| 2 |  | 820 MHz/ 23900 | 25@12 | 4100.01 MHz/ 673334 | Mid Highest | REFSENS (NOTE 2) |
| 3 | 10 | 822.5 MHz/ 23925 | 16@17 | 4112.52 MHz/ 674168 | Lowest | REFSENS (NOTE 2) |
| 4 |  | 822.5 MHz/ 23925 | 25@12 | 4112.52 MHz/ 674168 | Mid Highest | REFSENS (NOTE 2) |
| 5 |  | 825 MHz/ 23950 | 16@17 | 4125 MHz/ 675000 | Lowest | REFSENS (NOTE 2) |
| 6 |  | 825 MHz/ 23950 | 25@12 | 4125 MHz/ 675000 | Mid Highest | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 4 and 5 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_15: Test configurations table for exceptions due to UL harmonic interference for EN-DC 19_n77 (Requirement of Note 4 and 5)

| E-UTRA Band 19 |  |  |  | NR Band n77 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | NR FC (DL) (MHz) NDL | NR CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 835 MHz/ 24050 | 16@17 | 4175.01 MHz/ 678334 | Lowest | REFSENS (NOTE 2) |
| 2 | 10 | 835 MHz/ 24050 | 25@12 | 4175.01 MHz/ 678334 | 40 | REFSENS (NOTE 2) |
| 3 |  | 838.9 MHz/ 24089 | 16@17 | 4194.99 MHz/ 679666 | Lowest | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 4 and 5 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: For UEs with limited UE NR channel bandwidth capability, if the channel BW is not supported by the UE, skip the test point. This shall apply only for Rel 15 UEs. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_16: Void

Table 7.3B.2.3.4.2.1-2_17: Void

Table 7.3B.2.3.4.2.1-2_18: Void

Table 7.3B.2.3.4.2.1-2_19: Void

Table 7.3B.2.3.4.2.1-2_20: Void

Table 7.3B.2.3.4.2.1-2_21: Void

Table 7.3B.2.3.4.2.1-2_22: Test configurations table for exceptions due to UL harmonic interference for EN-DC 1_n28

| NR Band n28 |  |  |  | E-UTRA Band 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | E-UTRA FC (DL) (MHz) NDL | E-UTRA CBW (MHz) | UL allocation (LCRB) |
| 1 | 10 | 715 MHz/ 143000 | 8@21 | 2145 MHz/ 350 | 5 | REFSENS (NOTE 2) |
| 2 | 10 | 715 MHz/ 143000 | 16@17 | 2145 MHz/ 350 | 10 | REFSENS (NOTE 2) |
| 3 | 10 | 715 MHz/ 143000 | 25@12 | 2145 MHz/ 350 | 15, 20 | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 8, 9 and 10 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.4.1-1 in TS 36.521-1 [10] which defines uplink RB configuration and start RB location for each channel BW. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_23: Test configurations table for exceptions due to UL harmonic interference for EN-DC 7_n71 (Requirement of Note 6 and 7)

| NR Band n71 |  |  |  | E-UTRA Band 7 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | FC (DL) (MHz) NDL | CBW (MHz) | UL allocation (LCRB) |
| 1 |  | 668 MHz/ 133600 | 16@17 | 2672 MHz/ 3270 | 10 | REFSENS (NOTE 2) |
| 2 | 10 |  | 25@17 |  | 15 | REFSENS (NOTE 2) |
| 3 |  |  | 25@17 |  | 20 | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 6 and 7 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.4.1-1 in TS 36.521-1 [10] which defines uplink RB configuration and start RB location for each channel BW and E-UTRA band. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_24: Void

Table 7.3B.2.3.4.2.1-2_25: Test configurations table for exceptions due to UL harmonic interference for EN-DC 12_n78 (Test frequencies are selected to fulfil Requirement of Note 4 and 5 in Table 7.3B.2.0.3.1-1)

|  | E-UTRA Band 12 |  |  | NR Band 78 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) / NUL | UL allocation (LCRB) | NR FC (DL) (MHz) / NDL | NR CBW (MHz) | UL allocation (LCRB) |
|  |  |  | 10@10 |  | 10 |  |
| 1 | 10 | 711 MHz / 23130 | 15@22 | Mid | 15 | REFSENS (NOTE 2) |
|  |  |  | 20@15 |  | 20 |  |
|  |  |  | 25@12 |  | >=40 |  |
| NOTE 1: Test frequencies are selected to fulfil Note 4 and 5 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. NOTE 3: Only Highest NR channel bandwidths supported by the UE are tested. |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-2_26: Void

Table 7.3B.2.3.4.2.1-2_27: Void

Table 7.3B.2.3.4.2.1-2_28: Test configurations table for exceptions due to UL harmonic interference for EN-DC 28_n51 (Victim n51) (Requirement of Note 2, 13)

| E-UTRA Band B28 |  |  |  | NR Band n51 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test ID | Channel BW (MHz) | FC (UL) (MHz) NUL | UL allocation (LCRB) | FC (DL) (MHz) NDL | CBW (MHz) | UL allocation (LCRB) |
| 1 | 5 | 714.8 MHz/ 27328 | 12@6 | 1429.6 MHz/ 285920 | 5 | REFSENS (NOTE 2) |
| NOTE 1: Test frequencies are selected to fulfil Note 2 and 13 in Table 7.3B.2.0.3.1-1. NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |  |  |

The initial test configurations for E-UTRA band and NR band consist of environmental conditions, test frequencies, and channel bandwidths and RB allocations for exceptional test scenarios due to receiver harmonic mixing for EN-DC in NR FR1 are specified in Table 7.3B.2.3.4.2.1-3, to Table 7.3B.2.3.4.2.1-3j.

Table 7.3B.2.3.4.2.1-3: Default test conditions for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| Default Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1 E-UTRA Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1 |  |  |  | Mid range for E-UTRA and NR, unless otherwise specified in Table 7.3B.2.3.4.2.1-3a to Table 7.3B.2.3.4.2.1-3k |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1 |  |  |  | Highest NRB_agg, unless otherwise specified in Table 7.3B.2.3.4.2.1-3a to Table 7.3B.2.3.4.2.1-3k |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest |  |  |  |
| Test Parameters |  |  |  |  |  |  |  |
| Downlink Configuration |  |  |  | Uplink Configuration |  |  |  |
| Higher Frequency Band |  | Lower Frequency Band |  | Higher Frequency Band |  | Lower Frequency Band |  |
| Modulation | RB allocation | Modulation | RB allocation | Modulation | RB allocation | Modulation | RB allocation |
| (NOTE 1) | Full RB | (NOTE 1) | Full RB | (NOTE 2) | (NOTE 3) | (NOTE 2) | REFSENS (NOTE 4) |
| NOTE 1: QPSK for E-UTRA band or CP-OFDM QPSK for NR band. NOTE 2: QPSK for E-UTRA band or DFT-s-OFDM QPSK for NR band. NOTE 3: Uplink RB configuration as defined in Table 7.3B.2.0.3.2-2, unless otherwise specified in Table 7.3B.2.3.4.2.1-3a to Table 7.3B.2.3.4.2.1-3k. NOTE 4: REFSENS refers to Table 7.3.2.4.1-3 in TS 38.521-1 [8] if higher frequency band is NR band, which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band; REFSENS refers to Table 7.3.4.1-1 in TS 36.521-1 [10] if higher frequency band is E-UTRA band, which defines uplink RB configuration and start RB location for each channel BW, unless otherwise specified in Table 7.3B.2.3.4.2.1-3a to Table 7.3B.2.3.4.2.1-3k. |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-3a: Void

Table 7.3B.2.3.4.2.1-3aa: Void

Table 7.3B.2.3.4.2.1-3b: Void

Table 7.3B.2.3.4.2.1-3c: Void

Table 7.3B.2.3.4.2.1-3d: Void

Table 7.3B.2.3.4.2.1-3e: Void

Table 7.3B.2.3.4.2.1-3f: Void

Table 7.3B.2.3.4.2.1-3g: Void

Table 7.3B.2.3.4.2.1-3h: Void

Table 7.3B.2.3.4.2.1-3i: Void

Table 7.3B.2.3.4.2.1-3j: Void

Table 7.3B.2.3.4.2.1-3k: Void

The initial test configurations for E-UTRA band and NR band consist of environmental conditions, test frequencies, and channel bandwidths and RB allocations for exceptional test scenarios due to cross band isolation for EN-DC in NR FR1 are specified in Table 7.3B.2.3.4.2.1-4, to Table 7.3B.2.3.4.2.1-4n.

Table 7.3B.2.3.4.2.1-4: Test configurations table for exceptions due to cross band isolation for EN-DC FR1

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1 E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  | Specified in Table 7.3B.2.3.4.2.1-4a to Table 7.3B.2.3.4.2.1-4n. |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1 |  |  |  | Specified in Table 7.3B.2.3.4.2.1-4a to Table 7.3B.2.3.4.2.1-4n. |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 |  |  |  | 30 KHz |  |  |  |
| NR Test Parameters |  |  |  |  |  |  |  |
| Downlink Configuration |  |  |  | Uplink Configuration |  |  |  |
| NR Modulation | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation | NR Modulation | NR RB allocation | E-UTRA Modulation | E-UTRA RB allocation |
| CP-OFDM QPSK | Full RB (NOTE 1) | QPSK | Full RB | DFT-s-OFDM QPSK | Specified in Table 7.3B.2.3.4.2.1-4a to Table 7.3B.2.3.4.2.1-4n | QPSK | Table 7.3.4.1-1 in TS 36.521-1 [10] |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 of TS 38.521-1 [8]. |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4a: Void

Table 7.3B.2.3.4.2.1-4b: Test configurations for exceptions due to cross band isolation for EN-DC 25_n41 (Victim band LTE)

| Aggressor UL |  |  | E-UTRA B25 Ch BW/Frequency range |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW/RB allocation |  |  | 10MHz | 15 MHz | 20MHz |
|  |  |  | High range | High range | High range |
| NR | 100 MHz 160@0 | low range | X1- | X1- | X1 |
| n41 | 100 MHz 160@0 | mid range | - | - | X1 |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: NR UL RB configuration shall set per Table 7.3.2.4.1-3 in TS 38.521-1 [8]. |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4c: Void

Table 7.3B.2.3.4.2.1-4d: Test configurations for exceptions due to cross band isolation for EN-DC 38_n78 (Victim band LTE)

| Aggressor UL |  |  | E-UTRA B38 DL CBM/RB allocation |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW/RB allocation |  |  | 10MHz | 15 MHz | 20MHz |
|  |  |  | Low, Mid High range | Low, Mid High range | Low, Mid High range |
| NR Band | 100 MHz 270@0 | Low range | X | X | X |
| n78 | 100 MHz 270@0 | High range | - | - | X |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: NR UL RB configuration shall be further limited to that specified per Table 7.3.2.4.1-3 in TS 38.521-1 [8]. |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4e: Void

Table 7.3B.2.3.4.2.1-4f: Void

Table 7.3B.2.3.4.2.1-4g: Void

Table 7.3B.2.3.4.2.1-4h: Void

Table 7.3B.2.3.4.2.1-4i: Test configurations table for exceptions due to cross band isolation for EN-DC 66_n41(Victim band LTE)

| Aggressor UL |  |  | E-UTRA B66 Ch BW/Frequency range |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW/RB allocation |  |  | 10MHz | 15 MHz | 20MHz |
|  |  |  | High range | High range | High range |
| NR | 100 MHz 128@0 | low range | X1 | X1 | X1 |
| n41 | 100 MHz 128@0 | mid range | - | - | X1 |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: NR UL RB configuration shall be further limited to that specified per Table 7.3.2.4.1-3 in TS 38.521-1 [8]. NOTE 3: Applicable only when harmonic mixing MSD for this combination is not applied. |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4j: Test configurations table for exceptions due to cross band isolation for EN-DC 1_n40 (Victim band NR)

| Aggressor UL |  |  | NR n40 Ch BW/Frequency range |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW/RB allocation |  |  | 10MHz | 30 MHz | 80 MHz |
|  |  |  | Low range | Low range | Low range |
| E-UTRA | 20 MHz 100@0 | Mid range | X1 | X1 | X1 |
| B1 | 20 MHz 100@0 | High range | - | - | X1 |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: NR UL RB configuration shall be further limited to that specified per Table 7.3.2.4.1-3 in TS 38.521-1 [8]. |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4k: Test configurations table for exceptions due to cross band isolation for EN-DC 1_n40 (Victim band LTE)

| Aggressor UL |  |  | E-UTRA B1 Ch BW/Frequency range |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW/RB allocation |  |  | 10MHz | 15 MHz | 20MHz |
|  |  |  | Low range | Mid range | High range |
| NR | 20 MHz 100@0 | low range | X1 | X1 | X1 |
| n40 | 20 MHz 100@0 | mid range | - | - | X1 |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: NR UL RB configuration shall be further limited to that specified per Table 7.3.2.4.1-3 in TS 38.521-1 [8]. |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4l: Test configurations table for exceptions due to cross band isolation for EN-DC 3_n51 (Victim band NR)

| Aggressor UL |  |  | NR n51 Ch BW/Frequency range |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW/RB allocation |  |  | 5 MHz | 5 MHz | 5 MHz |
|  |  |  | Low range | Mid range | High range |
| E-UTRA | 5 MHz 25@0 | low range | X1 | - | - |
| B3 | 5 MHz 25@0 | Mid range | X1 | - | - |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: NR UL RB configuration shall be further limited to that specified per Table 7.3.2.4.1-3 in TS 38.521-1 [8]. |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4m: Test configurations table for exceptions due to cross band isolation for EN-DC 30_n66 (Victim band NR)

| Aggressor UL |  |  | NR n66 Ch BW/Frequency range |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW/RB allocation |  |  | 20 MHz | 20 MHz | 20 MHz |
|  |  |  | Low range | Mid range | High range |
| E-UTRA | 5 MHz 25@0 | low range | X1 | X1 | X1 |
| B30 | 5 MHz 25@0 | Mid range | - | - | X1 |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: NR UL RB configuration shall be further limited to that specified per Table 7.3.2.4.1-3 in TS 38.521-1 [8]. |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-4n: Test configurations table for exceptions due to cross band isolation for EN-DC 46_n78 (Victim band LTE)

| Aggressor UL |  |  | E-UTRA B46 Ch BW/Frequency range |  |  |
| --- | --- | --- | --- | --- | --- |
| CBW/RB allocation |  |  | 20 MHz | 20 MHz | 20 MHz |
|  |  |  | Low range | Mid range | High range |
| NR | 50 MHz 270@0 | mid range | X1 | X1 | X1 |
| N78 | 50 MHz 270@0 | high range | X1 | - | - |
| NOTE 1: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: NR UL RB configuration shall be further limited to that specified per Table 7.3.2.4.1-3 in TS 38.521-1 [8]. |  |  |  |  |  |

The initial test configurations for E-UTRA band and NR band consist of environmental conditions, test frequencies, and channel bandwidths and RB allocations for exceptional test scenarios due to dual uplink operation for EN-DC in NR FR1 (two bands) are specified in Table 7.3B.2.3.4.2.1-5.

Table 7.3B.2.3.4.2.1-5: Test Configuration Table Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1 E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  | For test frequencies refer to "Range" columns. |  |  |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1. |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | DC Configuration / NRB_agg |  |  |  |  |  | DL Allocation |  |  | UL Allocation (Note 2,3) |  |  |
|  | DC Configuration |  |  |  | E-UTRA Ch BW | NR Ch BW | CC MOD E-UTRA/NR | E-UTRA & NR RB allocation |  | CC MOD E-UTRA/NR | E-UTRA & NR allocations (LCRB @ RBstart) |  |
|  | E-UTRA |  | NR |  |  |  |  | PCC | SCC |  |  |  |
|  | Band | Range | Band | Range |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA_nYA Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid | Y | Mid | 5 | Mid | QPSK /CP-OFDM QPSK | All RBs |  | QPSK/ DFT-s-OFDM QPSK | REFSENS | REFSENS |
| Test Settings for DC_3A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | Note 5 | 7 | Note 5 | 5 | 10 | Note 7 | All RBs |  | Note 7 | 25@0 | 50@0 |
| Test Settings for DC_3A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | 1 | 1 | 1 |
| Test Settings for DC_3A_n20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 | Note 5 | 20 | Note 5 | 5 | 5 | Note 7 | All RBs |  | Note 7 | 25@0 | 25@0 |
| Test Settings for DC_7A-n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | Note 5 | 66 | Note 5 | 10 | 5 | Note 7 | All RBs |  | Note 7 | 50@0 | 25@0 |
| Test Settings for DC_7A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | Low | 77 | High | 5 | 10 | Note 7 | All RBs |  | Note 7 | 25@0 | 50@0 |
| Test Settings for DC_8A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 8 | Low | 41 | High | 5 | 10 | Note 7 | All RBs |  | Note 7 | 25@0 | 50@0 |
| Test Settings for DC_12A-n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 12 | Note 5 | 78 | Note 5 | 5 | 10 | Note 7 | All RBs |  | All RBs | 25@0 | 50@0 |
| Test Settings for DC_20A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 | Note 5 | 77 | Note 5 | 5 | 10 | Note 7 | All RBs |  | Note 7 | 25@0 | 50@0 |
| 2 | 20 | Note 5 | 77 | Note 5 | 5 | 10 | Note 7 | All RBs |  | Note 7 | 25@0 | 50@0 |
| Test Settings for DC_28A_n50A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 | Note 5 | 50 | Note 5 | 10 | 10 | Note 7 | All RBs |  | Note 7 | 50@0 | 50@0 |
| Test Settings for DC_66A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | Note 5 | 5 | Note 5 | 5 | 5 | Note 7 | All RBs |  | Note 7 | 25@0 | 25@0 |
| Test Settings for DC_66A_n25A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 66 | Note 5 | 5 | Note 5 | 5 | 5 | Note 7 | All RBs |  | Note 7 | 25@0 | 25@0 |
| Test Settings for CA_7A_20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 | Note 5 | 20 | Note 5 | 10 | 5 | QPSK | All RBs |  | QPSK | 50@0 | 25@0 |
| Note 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. In case Single UL is allowed and the UE only indicates support of "Single UL" the output power of the active UL shall be set at PCMAX_L,c or set to the maximum output power according to the UE power scaling capability. Note 2: Use DC Configuration – specific test points if present in the table, otherwise use test points from matching Group Test Settings, if present in the table. Otherwise use the Default Test Settings test points. Note 3: X,Y correspond to the different bands in the DC Configuration. E.g. for DC_1A-n3A, X=1, Y=3. Note 4: REFSENS refers to the E_UTRA bands and NR band NRB 's single carrier Uplink RB allocation for reference sensitivity according to table 7.3.5-2 of TS 36.521-1 [10] and Table 7.3.2.4.1-3 of TS 38.521-1 [8], respectively Note 5: Test frequency for each DC configuration shall follow Table 7.3B.2.0.3.5.1-1. If test configurations of each ID in a DC configuration are same, test frequency shall follow the order of Table 7.3B.2.0.3.5.1-1. Note 6: Not applicable if the UE only supports Bandwidth Combination Set 1. Note 7: Same as default. Note 8: RBSTART = 0 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.4.2.1-6: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions (two bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | For test frequencies refer to “Range” columns. |  |  |  |  |  |  |  |  |  |  |  |  |
| Test EN-DC channel bandwidth as specified in TS 36.508 [11] clause 4.3.1 and TS 38.508-1 [6] clause 4.3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB " columns |  |  |  |  |  |  |  |  |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Lowest supported SCS |  |  |  |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SCG -NR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  |  |  |  | Range |  |  |  |  |  | NRB |  |  |  |  |  |  |  |  |  |  | Band |  |  |  |  |  | Range |  |  |  | NRB |  |  |  |  |
|  | UL MOD |  |  |  |  | DL MOD |  |  |  |  |  | CH BW |  |  |  |  |  |  | DLalloc / UL alloc |  |  |  | UL MOD |  |  |  |  |  | DL MOD |  |  |  | UL/DL Ch BW | DLalloc / UL alloc |  |  |  |
| Test Settings for a DC_1A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12,11 | 1 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz | All RBs / 0 |  |  |  |
| 24,10 | 1 |  |  |  |  | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  | UL 1760 / DL 1855 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| 36 | 1 |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  | High |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz | All RBs / 0 |  |  |  |
| Test Settings for a DC_1A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 1 |  |  |  |  | UL 1965 / DL 2155 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  |  | UL 887.5 / DL 932.5 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_1A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 1 |  |  |  |  | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3900 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 23 | 1 |  |  |  |  | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3870 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz | All RBs / 0 |  |  |  |  |
| 34 | 1 |  |  |  |  | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 4090.005 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 44 | 1 |  |  |  |  | UL 1950/ DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3709.005 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_1A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 1 |  |  |  |  | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3709.995 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_2A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 2 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_LTE |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_3 |  |  |  |
| Test Settings for a DC_2A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  | 2 |  |  |  | UL 1855 / DL 1935 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | DL 2175 |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 24 |  | 2 |  |  |  | UL 1883.3 /DL1963.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | DL 2150 |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_2A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 |  |  |  |  | DL 1980 MHZ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n71 |  |  |  |  |  | UL 665.5 MHz |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_1 |  |  |  |  |
| 23 | 2 |  |  |  |  | DL1980 MHZ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n71 |  |  |  |  |  | UL 673 MHz |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_1 |  |  |  |  |
| 35 | 2 |  |  |  |  | UL 1881 MHZ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n71 |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz | All RBs / 0 |  |  |  |  |
| Test Settings for a DC_2A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 |  |  |  |  | UL 1860 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3720 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 23 | 2 |  |  |  |  | UL 1860 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3700.005 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz | All RBs / 0 |  |  |  |  |
| 3 8 | 2 |  |  |  |  | UL 1860 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3400.005 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz | All RBs / 0 |  |  |  |  |
| 45 | 2 |  |  |  |  | DL Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3920.01 |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 4a 9 | 2 |  |  |  |  | DL 1950 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3999.99 |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 54 | 2 |  |  |  |  | UL 1855/DL 1935 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3790.005 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 64 | 2 |  |  |  |  | UL 1900/DL 1980 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3720 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 74 | 2 |  |  |  |  | UL 1885/DL 1965 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3810 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_2A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 2 |  |  |  |  | UL 1870 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3740.01 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 23 | 2 |  |  |  |  | UL 1885 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3739.995 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz | All RBs / 0 |  |  |  |  |
| 34 | 2 |  |  |  |  | UL 1855/ DL 1935 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3790.005 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 44 | 2 |  |  |  |  | UL 1885/ DL 1965 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3690 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_3A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1,11 | 3 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |  |
| 26 | 3 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |  |
| 34 | 3 |  |  |  |  | UL 1760 / DL 1855 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  |  | UL 1950 / DL 2140 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_3A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 3 |  |  |  |  | UL 1771 / DL 1866 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  |  | UL 838 / DL 883 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| 24 | 3 |  |  |  |  | UL 1721 / DL 1816 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  |  | UL 838 / DL 883 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_3A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 3 |  |  |  |  | UL 1730 / DL 1825 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n7 |  |  |  |  |  | UL 2535 / DL 2655 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_3A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 3 |  |  |  |  | UL 1755 / DL 1850 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  |  | UL 900 / DL 945 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| 24 | 3 |  |  |  |  | UL 1747.5 / DL 1842.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  |  | UL 897.5 / DL 942.5 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_3A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 3 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 26 | 3 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 50 MHz | All RBs / REFSENS_ENDC_3 |  |  |  |
| 34, 10 | 3 |  |  |  |  | UL 1740 / DL 1835 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  |  | UL/DL 2657.5 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for DC_3A_n77A and DC_3A_n78A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 3 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3495 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 23 | 3 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3525 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz | All RBs / 0 |  |  |  |
| 35 | 3 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3685.005 |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |
| 39 | 3 |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | High |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs/0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |
| 44 | 3 |  |  |  |  | UL 1740 / DL 1835 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3574.995 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| 54 | 3 |  |  |  |  | UL 1765 / DL 1860 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3435 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_5A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 5 |  |  |  |  | UL 838/ DL 883 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  | UL 1721/ DL 2121 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | N/A |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_5A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 5 |  |  |  |  | UL 837.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3350.01 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 23 | 5 |  |  |  |  | UL 829 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 4145.01 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 38 | 5 |  |  |  |  | UL 840 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3600 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 44, 13 | 5 |  |  |  |  | UL 844/DL 889 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3421.005 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| 54, 13 | 5 |  |  |  |  | UL 826.5/DL 871.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 4177.5 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for DC_5A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 5 |  |  |  |  | UL 840 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3360 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 28 | 5 |  |  |  |  | UL 840 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3560.01 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 34 | 5 |  |  |  |  | UL 844/DL 889 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3421.005 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_7A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 |  |  |  |  | DL 2655 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  | DL 1825 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | N/A |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_7A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 7 |  |  |  |  | DL 2667 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  |  | DL 879 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | N/A |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_7A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 7 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |
| Test Settings for a DC_8A_n1A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 |  |  |  |  | DL 932.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  |  | DL 2155 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | N/A |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_8A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 |  |  |  |  | DL 945 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  | DL 1850 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | N/A |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| 24 | 8 |  |  |  |  | DL 942.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  | DL 1842.5 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | N/A |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | N/A |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_8A_n20A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 8 |  |  |  |  | UL 890.5 / DL 935.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n20 |  |  |  |  |  | UL 849.5 / DL 808.5 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| 24 | 8 |  |  |  |  | UL 892.5 / DL 937.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n20 |  |  |  |  |  | UL 847.5 / DL 806.5 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for DC_8A_n77A and DC_8A_n78A Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 8 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3590.01 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 2,8 | 8 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3520.005 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 34 | 8 |  |  |  |  | UL 897.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3634.995 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for DC_8A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 8 |  |  |  |  | UL 900 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  | UL/DL 4500 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 2,8 | 8 |  |  |  |  | UL 900 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  | UL/DL 4299.99 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 34 | 8 |  |  |  |  | UL 897.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  | UL/DL 4532.505 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_11A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 11 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  | UL/DL 4457.7 |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |
| 2,9 | 11 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  | UL/DL 4512.69 |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |
| Test Settings for a DC_13A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 13 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3909.99 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 18 | 13 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3600 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |  |
| 25 | 13 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3909.99 |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 29 | 13 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3834 |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 34 | 13 |  |  |  |  | UL 784.5/DL 753.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  | UL/DL 3891.495 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |  |
| Test Settings for a DC_19A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 19 |  |  |  |  | DL 884 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |
| 2,9 | 19 |  |  |  |  | DL 884 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  | UL/DL 4445.025 |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |
| Test Settings for a DC_20A_n3A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 |  |  |  |  | UL 840 / DL 799 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  | UL 1775 / DL 1870 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| 24 | 20 |  |  |  |  | UL 847 / DL 806 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  |  | UL 1735 / DL 1830 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_20A_n7A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 |  |  |  |  | UL 851 / DL 810 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n7 |  |  |  |  |  | UL 2512 / DL 2632 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_20A_n8A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | 20 |  |  |  |  | UL 849.5 / DL 808.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  |  | UL 892.5 / DL 937.5 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_20A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1,8 | 20 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | Mid |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 23 | 20 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3387.99 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  | N/A |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz | All RBs / 0 |  |  |  |
| 34 | 20 |  |  |  |  | UL 850 / DL 809 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  | UL/DL 3358.995 |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_21A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 21 |  |  |  |  | DL 1506 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  | UL/DL 4518 |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 29 | 21 |  |  |  |  | DL 1506 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  | UL/DL 4800 |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | QPSK |  |  |  | 100 MHz | All RBs / REFSENS_ENDC_2 |  |  |  |  |
| 34 | 21 |  |  |  |  | UL 1457.5/ DL 1505.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  |  |  |  | UL/DL 4420.5 |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz | All RBs / REFSENS_ENDC_4 |  |  |  |
| Test Settings for a DC_26A_n41A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 26 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  | UL/DL 2524.5 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 15 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  |  | N/A |  |  | CP-OFDM QPSK |  |  |  | 50 MHz |  |  |  |  | All RBs / 0 |  |
| 2,8 | 26 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  | UL/DL 2572 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 15 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  | 50 MHz |  |  |  |  | All RBs / 0 |  |
| 34 | 26 |  |  |  |  | UL 839 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  | UL/DL 2562 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| 45 | 26 |  |  |  |  | DL 886.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  | UL/DL 2660 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 15 MHz |  |  |  |  |  | All RBs / REFSENS_LTE |  |  |  |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |
| Test Settings for a DC_26A_n77A/DC_26A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 26 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3366 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 15 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  |  | N/A |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / 0 |  |
| 2,8 | 26 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3446.01 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 15 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  |  | N/A |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / 0 |  |
| 34 | 26 |  |  |  |  | UL 836.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3391.005 |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_26A_n79A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 26 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4432.5 |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  | 15 MHz |  |  |  |  |  | All RBs / 0 |  |  |  |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  | 60 MHz |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |
| 22,9 | 26 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n79 |  |  | UL/DL 4470.51 |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  | 15 MHz |  |  |  |  |  | All RBs / 0 |  |  |  |  |  |  | CP-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  | 60 MHz |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |
| Test Settings for a DC_28A_n5A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | 28 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  | Low |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 20MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  |  |  |  | N/A |  |  | CP-OFDM QPSK |  |  |  | 20 MHz |  |  |  |  | All RBs / 0 |  |
| Test Settings for a DC_28A_n77A/DC_28A_n78A Configuration (Test ID 4 only apply to DC_28A_n77A) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | 28 |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  |  | UL/DL 3540 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / 0 |  |  |  |
| 28 |  |  | 28 |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  |  | N/A |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / 0 |  |  |  |
| 34 |  |  | 28 |  |  | UL 705.5 / DL 760.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  |  | UL/DL 3582.495 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | QPSK |  |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |  |  | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |  |
| 45 |  |  | 28 |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  | UL/DL 3815.01 |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | N/A |  |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  | All RBs / 0 |  |  |  |  |  | CP-OFDM QPSK |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |  |
| Test Settings for a DC_40A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 40 |  |  |  |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  | UL/DL 3525 |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  | 20 MHz |  |  |  |  |  | All RBs / 0 |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  | 20 MHz |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |
| 2,9 | 40 |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  | Mid |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 20 MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |
| Test Settings for a DC_41A_n77A/DC_41A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 41 |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | UL/DL 3750 |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  | 20 MHz |  |  |  |  |  | All RBs / 0 |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / REFSENS_ENDC_2 |  |
| 26 | 41 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | Low |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  |  |  |  | QPSK |  |  |  |  | 20 MHz |  |  |  |  |  | All RBs / 0 |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |
| 36 | 41 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n77/n78 |  |  | Low |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  |  | QPSK |  |  |  |  | 20MHz |  |  |  |  |  | All RBs / REFSENS_ENDC_3 |  |  |  |  |  |  | N/A |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / 0 |  |
| Test Settings for a DC_48A_n66A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  | 48 |  |  |  |  | UL/DL 3555 |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  |  | UL 1760 MHZ |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |
| 23 |  |  |  | 48 |  |  |  |  | UL/DL 3565 |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  |  | UL 1770 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |
| 38 |  |  |  | 48 |  |  |  |  | UL/DL 3600 |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  |  | UL1760 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | N/A |  |  |  |  | QPSK |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 40 MHz |  |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |
| 44 |  |  |  | 48 |  |  |  |  | UL/DL 3630 |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  |  |  | UL 1715 / DL 2115 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | QPSK |  |  |  |  | QPSK |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_66A_n2A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14, 10 |  |  |  | 66 |  |  |  |  | UL1775/ DL2175 |  |  |  |  |  |  |  |  |  |  | n2 |  |  |  |  |  |  | UL 1855 / DL 1935 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | QPSK |  |  |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |
| 24, 10 |  |  |  | 66 |  |  |  |  | UL 1750/ DL 2150 |  |  |  |  |  |  |  |  |  |  | n2 |  |  |  |  |  |  | UL 1883.3 / DL 1963.3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | QPSK |  |  |  |  | QPSK |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_66A_n71A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  |  | 66 |  |  | UL 1750/ DL 2150 |  |  |  |  |  |  |  |  |  |  |  |  | n71 |  |  |  |  |  |  | UL 675/ DL 629 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |  |
| Test Settings for a DC_66A_n77A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  |  | 66 |  |  | UL 1720 |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3440.01 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 20 |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / 0 |  |
| 23 |  |  |  |  | 66 |  |  | UL 1720 |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3420 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 20 |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz |  |  |  |  | All RBs / 0 |  |
| 38 |  |  |  |  | 66 |  |  | UL 1720 |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3600 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 20 |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / 0 |  |
| 44 |  |  |  |  | 66 |  |  | UL 1775/ DL 2175 |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3949.995 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| 54 |  |  |  |  | 66 |  |  | UL 1760 / DL 2160 |  |  |  |  |  |  |  |  |  |  |  |  | n77 |  |  |  |  |  |  | UL/DL 3720 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| Test Settings for a DC_66A_n78A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  |  | 66 |  |  | UL 1755 |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3510 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 100 MHz |  |  |  |  | All RBs / 0 |  |
| 23 |  |  |  |  | 66 |  |  | UL 1755 |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3480 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 20 MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  | N/A |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 20 MHz |  |  |  |  | All RBs / 0 |  |
| 34 |  |  |  |  | 66 |  |  | UL 1730/ DL2130 |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  | UL/DL 3660 |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | QPSK |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / REFSENS_ENDC_4 |  |  | DFT-s-OFDM QPSK |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  | 10 MHz |  |  |  |  | All RBs / REFSENS_ENDC_4 |  |
| NOTE 1: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-2. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-2. REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation for ENDC according to table 7.3B.2.0.3.5.1-1 for PC3 and table 7.3B.2.0.3.5.1-1a for PC2. NOTE 2: Void NOTE 3: Test ID with UL harmonic exception NOTE 4: Test ID with 2UL intermodulation exception NOTE 5: Test ID with UL receiver harmonic mixing NOTE 6: Test ID with UL cross band isolation NOTE 7: Void NOTE 8: Test ID with UL harmonic exception avoided NOTE 9: Test ID with UL receiving harmonic mixing exception avoided NOTE 10: Only applicable to UEs not supporting UE capability singleUL-Transmission. NOTE 11: Test ID with Cross band isolation exception avoided, which is only applicable to DC_1_n3, DC_3_n1 and DC_3_n84. NOTE 12: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 14: If the NR frequency does not match to a valid NR-ARFCN, apply the closest NR frequency with a valid NR-ARFCN. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, C.2, C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1 of TS 38.521-1 [8].

5.	E-UTRA downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1 [10].

6.	The DL and UL Reference Measurement channels are set according to Tables 7.3B.2.3.4.2.1-0 to 7.3B.2.3.4.2.1-6 for E-UTRA CG and NR CG.

7.	NR propagation conditions are set according to Annex B.0 of TS 38.521-1 [8]. E-UTRA propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

8.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause7.3B.2.3.4.2.3.

9.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6. Step 9 only applicable to the test configuration in Table 7.3B.2.3.4.2.1-0.

7.3B.2.3.4.2.2	Test procedure

For test points in Table 7.3B.2.3.4.2.1-0:

1.	NR SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3B.2.3.4.2.1-0 on the NR CC. The NR SS sends downlink MAC padding bits on the DL RMC.

2.	NR SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3B.2.3.4.2.1-0 Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the appropriate REFSENS value defined in TS 38.521-1 [8] Table 7.3.2.5-1 for NR band. Send continuously uplink power control "up" commands in the uplink scheduling information to both carriers to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

4.	Measure the average throughput of the NR carrier for a duration sufficient to achieve statistical significance according to Annex H.2 of TS 38.521-1 [8] for NR band.

For test points in Table 7.3B.2.3.4.2.1-6:

1.	SS transmits PDSCH via PDCCH DCI format 1A and PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3B.2.3.4.2.1-6 on the E-UTRA CC and NR CC. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 and DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3B.2.3.4.2.1-6 on the E-UTRA CC and NR CC. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level for the E-UTRA CC and NR CC to the appropriate REFSENS value defined in Tables 7.3B.2.3.5-1 to 7.3B.2.3.5-4. Send continuously uplink power control "up" commands in the uplink scheduling information to both carriers to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

4.	Measure the average throughput of both NR and E-UTRA for a duration sufficient to achieve statistical significance according to Annex H.2 of TS 38.521-1 [8] for NR band, and Annex G.2 of TS 36.521-1 [10] for EUTRA band.

7.3B.2.3.4.2.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band.

Message contents exceptions for E-UTRA band are according to TS 36.521-1 [10] clause 7.3.4.3 for each network signalling value. Message contents exceptions for NR band are according to TS 38.521-1 [8] clause 7.3.2.4.3 for each network signalling value.

For test points with NOTE 4 in Table 7.3B.2.3.4.2.1-6, the following message exception applies:

Table 7.3B.2.3.4.2.3-1: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

Table 7.3B.2.3.4.2.3-2: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

7.3B.2.3.5	Test requirement

For inter-band EN-DC configurations affected by reference sensitivity exceptions, when test points without NOTE 8, 9 and 11 in Table 7.3B.2.3.4.2.1-6 are tested, the throughput of each CG shall be ≥ 95% of the maximum throughput for the reference receive power level specified in Table 7.3B.2.3.5-1, Table 7.3B.2.3.5-2, Table 7.3B.2.3.5-3, Table 7.3B.2.3.5-3a, Table 7.3B.2.3.5-4 and Table 7.3B.2.3.5-5 for MSDs due to uplink harmonic, harmonic mixing, cross band isolation for PC3 EN-DC, cross band isolation for PC2 EN-DC, dual uplinks for PC3 EN-DC and dual uplink operation for PC2 EN-DC, respectively. For a given EN-DC combo, if more than one category of MSD applies, UE shall pass all requirement.

For test points with NOTE 8, 9 or 11 in Table 7.3B.2.3.4.2.1-6, reference sensitivity test requirements are specified in Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC, and Table 7.3.5-1 in TS 36.521-1 [10] for E-UTRA CC.

Reference sensitivity test requirements for test points in Table 7.3B.2.3.4.2.1-0 are specified in Table 7.3.2.5-1 in TS 38.521-1 [8].

For the UE which supports inter-band EN-DC, the minimum requirement for reference sensitivity in Table 7.3.2.5-1 of TS 38.521-1 [8] for NR band and Table 7.3.5-1 of TS 36.521-1 [10] for EUTRA band, shall be increased by the amount given in ΔRIB,c defined in clause 7.3B.3.3 for the applicable two, three, four and five bands operation.

Table 7.3B.2.3.5-1: Reference sensitivity due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 3 | n772 | 15 |  | -71.4 +TT | -71.4 +TT | -71.3 +TT |  |  | -71.2 +TT | -71.3 +TT |  |  |  |  |
|  |  | 30 |  | -71.7 +TT | -71.5 +TT | -71.5 +TT |  |  | -71.3 +TT | -71.4 +TT | -71.4 +TT | -71.3 +TT | -71.3 +TT | -71.3 +TT |
|  |  | 60 |  | -72.1 +TT | -71.8 +TT | -71.7 +TT |  |  | -71.5 +TT | -71.5 +TT | -71.5 +TT | -71.4 +TT | -71.4 +TT | -71.4 +TT |
|  | n773 | 15 |  | -94.2 +TT | -92.7 +TT | -91.9 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.5 +TT | -92.8 +TT | -92.1 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -94.9 +TT | -93.1 +TT | -92.3 +TT |  |  |  |  |  |  |  |  |
| 2 | n772, 13 | 15 |  | -71.4 +TT | -71.4 +TT | -71.3 +TT |  |  | -71.2 +TT | -71.3 +TT |  |  |  |  |
|  |  | 30 |  | -71.7 +TT | -71.5 +TT | -71.5 +TT |  |  | -71.3 +TT | -71.4 +TT | -71.4 +TT | -71.3 +TT | -71.3 +TT | -71.3 +TT |
|  |  | 60 |  | -72.1 +TT | -71.8 +TT | -71.7 +TT |  |  | -71.5 +TT | -71.5 +TT | -71.5 +TT | -71.4 +TT | -71.4 +TT | -71.4 +TT |
|  | n773 | 15 |  | -94.2 +TT | -92.7 +TT | -91.9 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.5 +TT | -92.8 +TT | -92.1 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -94.9 +TT | -93.1 +TT | -92.3 +TT |  |  |  |  |  |  |  |  |
| 2 | n782 | 15 |  | -71.9+TT | -71.9 +TT | -71.8 +TT |  |  | -71.7 +TT | -71.8 +TT |  |  |  |  |
|  |  | 30 |  | -72.2 +TT | -72.0 +TT | -72.0 +TT |  |  | -71.8 +TT | -71.9 +TT | -71.9 +TT | -71.8 +TT | -71.8 +TT | -71.8 +TT |
|  |  | 60 |  | -72.6 +TT | -72.3 +TT | -72.2 +TT |  |  | -72.0 +TT | -72.0 +TT | -72.0 +TT | -71.9 +TT | -71.9 +TT | -71.9 +TT |
| 2 | n783 | 15 |  | -94.7 +TT | -93.2 +TT | -92.4 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.9 +TT | -93.3 +TT | -92.6 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | --95.4 +TT | -93.6 +TT | -92.8 +TT |  |  |  |  |  |  |  |  |
| 3 | n782 | 15 |  | -71.9 +TT | -71.9 +TT | -71.8 +TT |  |  | -71.7 +TT | -71.8 +TT |  |  |  |  |
|  |  |  |  | -74.1 +TT15 | -74.1 +TT15 | -74.0 +TT15 |  |  | -73.9 +TT15 | -74.0 +TT15 |  |  |  |  |
|  |  | 30 |  | -72.2 +TT | -72.0 +TT | -72.0 +TT |  |  | -71.8 +TT | -71.9 +TT | -71.9 +TT | -71.8 +TT | -71.8 +TT | -71.8 +TT |
|  |  |  |  | -74.4 +TT15 | -74.2 +TT15 | -74.2 +TT15 |  |  | -74.0 +TT15 | -74.1 +TT15 | --74.1 +TT15 | -74.0 +TT15 | -74.0 +TT15 | -74.0 +TT15 |
|  |  | 60 |  | -72.6 +TT | -72.3 +TT | -72.2 +TT |  |  | -72.0 +TT | -72.0 +TT | -72.0 +TT | -71.9 +TT | -71.9 +TT | -71.9 +TT |
|  |  |  |  | -74.8 +TT15 | -74.5 +TT15 | -74.4 +TT15 |  |  | -74.2 +TT15 | -74.2 +TT15 | -74.2 +TT15 | -74.1 +TT15 | -74.1 +TT15 | -74.1 +TT15 |
|  | n783 | 15 |  | -94.7 +TT | -93.2 +TT | -92.4 +TT |  |  |  |  |  |  |  |  |
|  |  |  |  | -96.9 +TT15 | -95.4 +TT15 | -94.6 +TT15 |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -95.0 +TT | -93.3 +TT | -92.6 +TT |  |  |  |  |  |  |  |  |
|  |  |  |  | -97.2 +TT15 | -95.5 +TT15 | -94.8 +TT15 |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -95.4 +TT | -93.6 +TT | -92.8 +TT |  |  |  |  |  |  |  |  |
|  |  |  |  | -97.6 +TT15 | -95.8 +TT15 | -95.0 +TT15 |  |  |  |  |  |  |  |  |
| 5 | n78 | 15 |  | -84.2 +TT | -84.4 +TT |  |  |  | -84.2 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -84.3 +TT | -84.5 +TT | -84.4 +TT |  |  | -84.3 +TT | -84.5 +TT | -84.4 +TT | -84.3 +TT | -84.0 +TT | -84.2 +TT |
|  |  | 60 |  | -84.5 +TT | -84.6 +TT | -84.5 +TT |  |  | -84.5 +TT | -84.6 +TT | -84.5 +TT | -84.4 +TT | -84.1 +TT | -84.3 +TT |
| 8 | n776,7 n786,7 | 15 |  | -84.5 +TT | -84.4 +TT | -84.2 +TT |  |  | -84.0 +TT | -83.9 +TT |  |  |  |  |
|  |  | 30 |  | -84.8 +TT | -84.5 +TT | -84.4 +TT |  |  | -84.1 +TT | -84.0 +TT | -83.9 +TT | -83.8 +TT | -83.5 +TT | -83.7 +TT |
|  |  | 60 |  | -85.2 +TT | -84.8 +TT | -84.6 +TT |  |  | -84.3 +TT | -84.1 +TT | -84.0 +TT | -83.9 +TT | -83.6 +TT | -83.8 +TT |
| 8 | n41 | 15 |  | -81.8 +TT | -81.7 +TT | -81.7 +TT |  |  | -81.6 +TT | -81.5 +TT |  |  |  |  |
|  |  | 30 |  | -82.1 +TT | -81.8 +TT | -81.9 +TT |  |  | -81.7 +TT | -81.6 +TT | -81.4 +TT | -81.3 +TT | -81.2 +TT | -81.2 +TT |
|  |  | 60 |  | -82.5 +TT | -82.1 +TT | -81.1 +TT |  |  | -81.9 +TT | -81.7 +TT | -81.6 +TT | -81.3 +TT | -81.2 +TT | -81.2 +TT |
| 8 | n794,5 | 15 |  |  |  |  |  |  | -82.8 +TT | -82.4 +TT |  |  |  |  |
|  |  | 30 |  |  |  |  |  |  | -82.9 +TT | -82.5 +TT | -82.3 +TT | -81.7 +TT |  | -81.2 +TT |
|  |  | 60 |  |  |  |  |  |  | -83.1 +TT | -82.6 +TT | -82.4 +TT | -81.8 +TT |  | -81.3 +TT |
| 12 | n66 | 15 | -89.5 +TT | -88.8 +TT | -88.3 +TT | -87.8 +TT |  |  | -87.7 +TT |  |  |  |  |  |
|  |  | 30 |  | -89.1 +TT | -88.4 +TT | -88.0 +TT |  |  | -87.8 +TT |  |  |  |  |  |
|  |  | 60 |  | -89.5 +TT | -88.7 +TT | -88.2 +TT |  |  | -87.9 +TT |  |  |  |  |  |
| 12 | n78 | 15 |  | -85.4 +TT | -85.1 +TT | -84.9 +TT |  |  | -84.9 +TT | -84.9 +TT |  |  |  |  |
|  |  | 30 |  | -85.7 +TT | -85.2 +TT | -85.1 +TT |  |  | -85.0 +TT | -85.0 +TT | -84.9 +TT | -84.9 +TT | -84.9 +TT | -84.9 +TT |
|  |  | 60 |  | -86.1 +TT | -85.5 +TT | -85.3 +TT |  |  | -85.2 +TT | -85.1 +TT | -85.0 +TT | -85.0 +TT | -85.0 +TT | -85.0 +TT |
| 13 | n774, 5 | 15 |  | -84.9 +TT | -84.6 +TT | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -85.2 +TT | -84.7 +TT | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  | -85.6 +TT | -85 +TT | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 18， 19 | n774,5 | 15 |  | -84.9 +TT | -84.6 +TT | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -85.2 +TT | -84.7 +TT | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  | -85.6 +TT | -85.0 +TT | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 28 | n774,5 n784,5 | 15 |  | -84.9 +TT | -84.6 +TT | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -85.2 +TT | -84.7 +TT | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT | -84.4 +TT |
|  |  | 60 |  | -85.6 +TT | -85.0 +TT | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT | -84.5 +TT |
| 20 | n786,7 | 15 |  | -84.5 +TT | -84.4 +TT | -84.2 +TT |  |  | -83.1 +TT |  |  |  |  |  |
|  |  | 30 |  | -84.8 +TT | -84.5 +TT | -84.4 +TT |  |  | -83.2 +TT |  |  |  |  |  |
|  |  | 60 |  | -85.2 +TT | -84.8 +TT | -84.6 +TT |  |  | -83.4 +TT |  |  |  |  |  |
| 26 | n41 | 15 |  | -84.5 +TT | -84.6 +TT | -84.4 +TT |  |  | -83.6 +TT | -83.3 +TT | 3.9 +TT | 3.1 +TT | 2.7 +TT |  |
|  |  | 30 |  | -84.8 +TT | -84.7 +TT | -84.6 +TT |  |  | -83.7 +TT | -83.4 +TT | -83.0 +TT | -82.5 +TT | -82.4 +TT |  |
|  |  | 60 |  | -85.2 +TT | -85.0 +TT | -84.8 +TT |  |  | -83.9 +TT | -83.5 +TT | -83.2 +TT | -82.5 +TT | -82.4 +TT |  |
| 26 | n776,7 n786,7 | 15 |  | -85.0 +TT | -84.9 +TT | -84.7 +TT |  |  | -83.6 +TT | -84.6 +TT |  |  |  |  |
|  |  | 30 |  | -85.3 +TT | -85.0+TT | -84.9 +TT |  |  | -83.7 +TT | -84.7 +TT | -84.7 +TT | -84.6 +TT | -84.6 +TT | -84.6 +TT |
|  |  | 60 |  | -85.7 +TT | --85.3 +TT | --85.1 +TT |  |  | -83.9 +TT | -84.8 +TT | -84.8 +TT | -84.7 +TT | -84.7 +TT | -84.7 +TT |
| 26 | n774,5 | 15 |  | -84.9 +TT | -84.6 +TT | -84.4 +TT |  |  | -84.4 +TT | -84.4 +TT |  |  |  |  |
|  |  | 30 |  | -85.2 +TT | -84.7 +TT | -84.6 +TT |  |  | -84.5 +TT | -84.5 +TT | -84.4 +TT | -84.4 +TT | -85.6 +TT | -84.4 +TT |
|  |  | 60 |  | -85.6 +TT | -85.0 +TT | -84.8 +TT |  |  | -84.7 +TT | -84.6 +TT | -84.5 +TT | -84.5 +TT | -85.7 +TT | -84.5 +TT |
| n28 | 18,9,10 | N/A | -89.1 | -88.7 | -88.3 | -88.0 |  |  |  |  |  |  |  |  |
| n71 | 211 | N/A | -92.7 | -93.3 | -91.8 | -90.7 |  |  |  |  |  |  |  |  |
| n71 | 212 | N/A | -95.6 | -93.3 | -91.8 | -90.7 |  |  |  |  |  |  |  |  |
| n71 | 7 | N/A | -82.7 | -82.6 | -82.4 | -82.3 |  |  |  |  |  |  |  |  |
| 66 | n772, 13 | 15 |  | -71.4 +TT | -71.4 +TT | -71.3 +TT |  |  | -71.2 +TT | -71.3 +TT |  |  |  |  |
|  |  | 30 |  | -71.7 +TT | -71.5 +TT | -71.5 +TT |  |  | -71.3 +TT | -71.4 +TT | -71.4 +TT | -71.3 +TT | -71.3 +TT | -71.3 +TT |
|  |  | 60 |  | -72.1 +TT | -71.8 +TT | -71.7 +TT |  |  | -71.5 +TT | -71.5 +TT | -71.5 +TT | -71.4 +TT | -71.4 +TT | -71.4 +TT |
|  | n773 | 15 |  | -94.2 +TT | -92.7 +TT | -91.9 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -94.5 +TT | -92.8 +TT | -92.1 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -94.9 +TT | -93.1 +TT | -92.3 +TT |  |  |  |  |  |  |  |  |
| 66 | n782 | 15 |  | -71.9 +TT | -71.9 +TT | -71.8 +TT |  |  | -71.7 +TT | -71.8 +TT |  |  |  |  |
|  |  | 30 |  | -72.2+TT | -72.0 +TT | -72.0 +TT |  |  | -71.8 +TT | -71.9 +TT | -71.9 +TT | -71.8 +TT | -71.8 +TT | -71.8 +TT |
|  |  | 60 |  | -72.6 +TT | -72.3 +TT | -72.2 +TT |  |  | -72.0 +TT | -72.0 +TT | -72.0 +TT | -71.9 +TT | -71.9 +TT | -71.9 +TT |
|  | n783 | 15 |  | -94.7 +TT | -93.2 +TT | -92.4 +TT |  |  |  |  |  |  |  |  |
|  |  | 30 |  | -95.0 +TT | -93.3 +TT | -92.6 +TT |  |  |  |  |  |  |  |  |
|  |  | 60 |  | -95.4 +TT | -93.6 +TT | -92.8 +TT |  |  |  |  |  |  |  |  |
| n66 | 48,2, 13 | N/A | -70.7 | -70.6 | -70.8 | -70.8 |  |  |  |  |  |  |  |  |
| n66 | 48,3 | N/A | -96.1 | -93.6 | -92.3 | -91.6 |  |  |  |  |  |  |  |  |
| NOTE 1: Void. NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , where andare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 9 The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 10: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. NOTE 11: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 12: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 13: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the EN-DC band combination: ∆FHD = 10 MHz for DC_1_n77, DC_2_n48, DC_2_n77, DC_42_n3, DC_48_n25, DC_48_n66, DC_66_n48, DC_66_n77, DC_3_n77, DC_3_n78, DC_11_n28 and DC_28_n50, DC_28_n51, DC_66_n78, DC_25_n77, DC_25_n78. NOTE 14: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 15: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 16: Void NOTE 17: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-1a: Test Tolerance (TT) for RX sensitivity level

| f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0 GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

Reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1, are specified in Table 7.3B.2.3.5-2 with uplink configuration specified in Table 7.3B.2.3.4.2.1-3a to Table 7.3B.2.3.4.2.1-3k for each specific EN-DC combination scenario.

Table 7.3B.2.3.5-2: Reference sensitivity due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | n714 | 15 | -70.4 +TT | -70.4 +TT | -70.4 +TT | -70.4 +TT |  |  |  |  |  |  |  |
|  |  | 30 |  | -70.7 +TT | -70.7 +TT | -71.8 +TT |  |  |  |  |  |  |  |
|  |  | 60 |  | -72.4 +TT | -72.7 +TT | -77.0 +TT |  |  |  |  |  |  |  |
| n41 | 264 | N/A | -72.5 | -69.5 | -69.5 | N/A |  |  |  |  |  |  |  |
| n77 | 2 | N/A | -91.2 | -89.3 | -88.5 | -87.6 |  |  |  |  |  |  |  |
|  |  |  | -93.910 | -9210 | -91.210 | -90.310 |  |  |  |  |  |  |  |
| n77 | 13 | N/A |  | -65.3 | -65.3 |  |  |  |  |  |  |  |  |
| n77 | 418 | N/A | -86.9 | -83.9 | -82.1 | -80.9 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| n77 | 282 | N/A | -69.8 | -69.8 | -69.8 | -68.3 |  |  |  |  |  |  |  |
| n78 | 3 | N/A | -90.6 | -89.3 | -88.5 | -87.6 |  |  |  |  |  |  |  |
|  |  |  | -93.310 | -92.010 | -91.210 | -90.310 |  |  |  |  |  |  |  |
| n78 | 40 | N/A | -86.9 | -83.9 | -82.1 | -80.9 |  |  |  |  |  |  |  |
| n78 | 418 | N/A | -86.9 | -83.9 | -82.1 | -80.9 | N/A | N/A | N/A | N/A | N/A | N/A |  |
| n79 | 192 | N/A | -69.8 | -69.8 | -69.8 |  |  |  |  |  |  |  |  |
| n79 | 262 | N/A | -69.8 | -69.8 | -69.8 | N/A | N/A | N/A | N/A | N/A | N/A |  | N/A |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (higher) band for which the mixing product due to harmonic of victim (lower) band LO with leakage of aggressor (higher) band is within the downlink transmission bandwidth of a victim (lower) band. NOTE 2: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. NOTE 3: Void. NOTE 4: The requirements should be verified for DL EARFCN or NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band andfULHB the UL carrier frequency in the higher band, both in MHz. NOTE 5: Void. NOTE 6: Void. NOTE 7: The requirements should be verified for DL EARFCN of the victim (higher) band (superscript HB) such that fDLHB=50×fULLB0.03 with fDLHB the DL carrier frequency in the higher band and fULLB the UL carrier frequency in the lower band, both in MHz. NOTE 8: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1fDLLB=fULHB/0.150.1 with the DL carrier frequency in the lower band andfULHBfULHBfDLLB=fULHB/0.150.1 the UL carrier frequency in the higher band, both in MHz. NOTE 9: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 10: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Reference sensitivity exceptions due to cross band isolation for EN-DC in NR FR1, are specified in Table 7.3B.2.3.5-3 with uplink configuration specified in Table 7.3B.2.3.4.2.1-4a to Table 7.3B.2.3.4.2.1-4n.

Table 7.3B.2.3.5-3: Reference sensitivity exceptions due to cross band isolation for PC3 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| n1 | 3 | N/A | -93.3 | -91 | -89.5 | -88.5 |  |  |  |  |  |  |  |  |  |
| 1 | n3 | 15 | -94 +TT | -91.6 +TT | -90.1 +TT | -89.1 +TT | -88.1 +TT | -87.4 +TT | -86.2 +TT |  |  |  |  |  |  |
| 1 | n40 | 15 | -93.4 +TT | -90.2 +TT | -88.4 +TT | -87.2 +TT | -86.1 +TT |  | -85.3 +TT | -84.0 +TT |  |  |  |  |  |
| 3 | n41 | 30 |  | -94.4 +TT | -92.4 +TT | -91.3 +TT |  | -89.3 +TT | -88.0 +TT | -87 +TT | -86.2 +TT |  | -84.9 +TT | -84.4 +TT | -84.0 +TT |
|  |  |  |  | -97.1 +TT4 | -95.1 +TT4 | -94 +TT4 |  | -92.0 +TT4 | -90.7 +TT4 | -89.7 +TT4 | -88.9 +TT4 |  | -87.6 +TT4 | -87.1 +TT4 | -86.7 +TT4 |
| n5 | 28 | 15 | -93.3 | -91.8 | -90.8 | -90 |  |  |  |  |  |  |  |  |  |
| n40 | 1 | N/A | -91.0 | -88.0 | -86.2 | -85.0 |  |  |  |  |  |  |  |  |  |
| n41 | 2 | N/A | -96.7 | -93.7 | -91.9 | -90.7 |  |  |  |  |  |  |  |  |  |
| n41 | 3 | N/A | 3 | n41 | 30 | 3 |  |  |  |  |  |  |  |  |  |
| n41 | 25 | N/A | -95.2 | -92.2 | -90.4 | -89.2 |  |  |  |  |  |  |  |  |  |
| n77 | 411 | N/A | -92.8 | -89.8 | -88.0 | -86.8 |  |  |  |  |  |  |  |  |  |
| n41 | 66 | N/A | -95.3 | -92.3 | -90.5 | -89.3 |  |  |  |  |  |  |  |  |  |
| 41 | n773 | 30 |  | -87.3 +TT | -85.3 +TT | -84.1 +TT |  |  | -82.9 +TT | -82.9 +TT | -82.9 +TT |  | -82.1 +TT | -81.7 +TT | -81.3 +TT |
| 30 | n66 | 15 | -91.2 +TT | -88.0 +TT | -86.2 +TT | -85.0 +TT |  |  |  | -81.8 +TT |  |  |  |  |  |
| n78 | 71 | N/A | -93.5 | -90.5 | -88.7 | -87.5 |  |  |  |  |  |  |  |  |  |
| n78 | 38 | N/A | -96.7 | -93.7 | -91.9 | -90.7 |  |  |  |  |  |  |  |  |  |
| n78 | 411 | N/A | -92.8 | -89.8 | -88.0 | -86.8 |  |  |  |  |  |  |  |  |  |
| 41 | n78 | 30 |  | -87.8 +TT | -85.8 +TT | -84.6 +TT |  |  | -83.4 +TT | -83.4 +TT | -83.4 +TT |  | -82.6 +TT | -82.2 +TT | -81.8 +TT |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 3: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 4: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-3a: Reference sensitivity exceptions due to cross band isolation for PC2 EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 70 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| 3 | n41 | 30 |  | -94.4 +TT | -92.4 +TT | -91.3 +TT |  | -89.3 +TT | -88.0 +TT | -87.0 +TT | -86.2 +TT |  | -84.9 +TT | -84.4 +TT | -84.0 +TT |
|  |  |  |  | -97.1 +TT3 | -95.1 +TT3 | -94 +TT3 |  | -92.0 +TT3 | -90.7 +TT3 | -89.7 +TT3 | -88.9 +TT3 |  | -87.6 +TT3 | -87.1 +TT3 | -86.7 +TT3 |
| n41 | 3 | N/A | -94.0 | -91.0 | -89.2 | -88.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | -96.73 | -93.73 | -91.93 | -90.73 |  |  |  |  |  |  |  |  |  |
| NOTE 1: Applicable only when harmonic mixing MSD for this combination is not applied. NOTE 2: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 3: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1, are specified in Table 7.3B.2.3.5-4 with uplink configuration specified in Table 7.3B.2.3.4.2.1-6.

Table 7.3B.2.3.5-4: Reference sensitivity exceptions due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 40 MHz (dBm) | IMD order | Duplex mode |
| DC_1A_n3A | 1 | N/A | [-76.3] | - | - | - | - | IMD3 | FDD |
|  | n3 | 15 | REFSENS |  | - | - | - | N/A | TDD |
| DC_1A_n8A | 1 | N/A | -93.3 | - | - | - | - | IMD4 | FDD |
|  | n8 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_1A_n77A | 1 | N/A | -69.5 | - | - | - | - | IMD23 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_1A_n77A | 1 | N/A | -91.3 | - | - | - | - | IMD4- | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_1A_n78A | 1 | N/A | -91.3 | - | - | - | - | IMD4 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A_n66A | 2 | N/A | -77.3 | - | - | - | - | IMD3 |  |
|  | n66 | 15 | REFSENS | - | - | - | - | N/A |  |
| DC_2A_n66A | 2 | N/A | REFSENS | - | - | - | - | N/A |  |
|  | n66 | 15 | -95.5 +TT | - | - | - | - | IMD5 |  |
| DC_2A_n77A | 2 | N/A | -71.3 | - | - | - | - | IMD2 | FDD |
| DC_2A-2A_n77A | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 2 | N/A | -89.3 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 2 | N/A | -92.3 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A_n78A | 2 | N/A | -71.37 | - | - | - | - | IMD23 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | - | TDD |
| DC_2A_n78A | 2 | N/A | -89.37 | - | - | - | - | N/A | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | IMD43 | TDD |
| DC_3A_n1A | 3 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n1 | 15 | -74.0+TT | - | - | - | - | IMD3 | FDD |
| DC_3A_n5A | 3 | N/A |  | -89.3 | - | - | - | IMD4 | FDD |
|  | n5 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 3 | N/A |  | REFSENS | - | - | - | N/A | FDD |
|  | n5 | 15 | -74.0+TT | - | - | - | - | IMD23 | FDD |
| DC_3A_n7A | 3 | N/A | REFSENS | - | - | - | - | - |  |
|  | n7 | 15 | - | -84.6 +TT5 | - | - | - | - |  |
| DC_3A_n8A | 3 | N/A | - | REFSENS | - | - | - | N/A | FDD |
|  | n8 | 15 | -88.3 | - | - | - | - | IMD43 | FDD |
|  | 3 | N/A | - | -86.9 | - | - | - | IMD5 | FDD |
|  | n8 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| CA_3A-n20A | 3 | N/A | -92.3 | - | - | - | - | IMD4 | FDD |
|  | n20 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 3 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n20 | 15 | -88.0+TT | - | - | - | - | IMD4 | FDD |
| DC_3A_n41A | 3 | N/A | -88.1 | - | - | - | - | IMD4 | FDD |
|  |  |  | -90.87 | - | - | - | - |  |  |
| DC_3A_n77A, DC_3A_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | N/A | -70.3 | - | - | - | - | IMD23 | FDD |
|  |  |  | -70.67 | - | - | - | - |  |  |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n77A, DC_3A_n77(2A), DC_3C_n77A, DC_3C_n77(2A), DC_3A_SUL_n77A-n80A, DC_3A_n78A, DC_3A_SUL_n78A-n80A, DC_3A_n78(2A), DC_3C_n78A DC_3C_n78(2A) | 3 | N/A | -88.3 | - | - | - | - | IMD43 | FDD |
|  |  |  | -88.37 | - | - | - | - |  |  |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_5A_n66A | 5 | N/A | -67.3 | - | - | - | - | IMD23 | FDD |
|  | n66 |  | REFSENS | - | - | - | - | N/A | FDD |
| DC_5A_n77A8 | 5 | N/A | -89 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 5 | N/A | -91.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_5A_n78A | 5 | N/A | -89.0 | - | - | - | - | IMD4 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_7A_n3A | 7 | N/A | - | -81.3 | - | - | - | IMD4 | FDD |
|  | n3 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_7A_n5A | 7 | N/A |  | REFSENS | - | - | - | N/A | FDD |
|  | n5 | 15 | -86.0+TT | - | - | - | - | IMD33 | FDD |
| DC_7A_n66A | 7 | N/A | - | -79.3 | - | - | - | IMD4 | FDD |
|  | n66 | 15 | REFSENS | - |  | - | - | N/A | TDD |
| DC_7A_n77A | 7 | N/A | -90.2 | - | - | - | - | IMD4 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_8A_n1A | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n1 | 15 | -94.0 + TT | - | - | -- | - | IMD4 | FDD |
| DC_8A_n3A | 8 | N/A | -88.3 | - | - | - | - | IMD43 | FDD |
|  | n3 | 15 | - | REFSENS | - | - | - | N/A | FDD |
|  | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n3 | 15 | - | -87.4+ TT | - | - | - | IMD5 | FDD |
| DC_8A_n20A | n20 | 15 | -71.3 | - | - | - | - | IMD3 | FDD |
|  | 8 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n20 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 8 | N/A | -71.3 | - | - | - | - | IMD3 | FDD |
| DC_8A_n41A | 8 | N/A | -84.2 | - | - | - | - | IMD33 | FDD |
|  | n41 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_8A_n77A DC_8A_n78A DC_8A-SUL_n78A-n81A | 8 | N/A | -88.0 | - | - | - | - | IMD4 | FDD |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | H4 | TDD |
| DC_8A_n79A DC_8A-SUL_n79A-n81A | 8 | N/A | -91.5 | - | - | - | - | IMD5 | FDD |
|  | n79 | 15 | - | - | - | - | REFSENS | N/A | TDD |
| DC_12_n78 | 12 | N/A | -90.8 | - | - | - | - | IMD5 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_13A_n77A | 13 | N/A | -90.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_20A_n3A | 20 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n3 | 15 | -93.0 +TT | - | - | - | - | IMD4 | FDD |
|  | 20 | N/A | -87.3 | - | - | - | - | IMD4 | FDD |
|  | n3 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_20_n7 | 20 | N/A | -84.3 | - | - | - | - | IMD3 | FDD |
|  | n7 | 15 | - | REFSENS | - | - | - | N/A | FDD |
| DC_20_n8 | 20 | N/A | -71.3 | - | - | - | - | IMD3 | FDD |
|  | n8 | 15 | -71.3 | - | - | - | - | IMD3 | FDD |
| DC_20A_n78A, DC_20A-SUL_n78A-n82A | 20 | N/A | -74.6 | - | - | - | - | IMD44 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_21A_n79A | 21 | N/A | -80.9 | - | - | - | - | IMD3 | FDD |
|  | n79 | 15 | - | - | - | - | REFSENS | N/A | TDD |
| DC_28_n50 | 28 | N/A | - | -79.5 | - | - | - | IMD 2 | FDD |
|  | n50 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 28 | N/A | - | -88.8 | - | - | - | IMD 4 | FDD |
|  | n50 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 28 | N/A | - | -94.3 | - | - | - | IMD 5 | FDD |
|  | n50 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| CA_28A_n77A, CA_28A_n78A, DC_28A-SUL_n78A-n83A | 28 | N/A | -92.3 | - | - | - | - | IMD5 | FDD |
|  | n77, n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_48A_n66A | 48 | N/A | - | REFSENS | - | - | - | N/A | TDD |
|  | n66 | 15 | -95.5 +TT | - | - | - | - | IMD5 | FDD |
| DC_66A_n2A | 66 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n2 | 15 | -78.0+TT | - | - | - | - | IMD3 | FDD |
|  | 66 | N/A | -94.8 | - | - | - | - | IMD5 | FDD |
|  | n2 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n5A | 66 | N/A | -68.8 | - | - | - | - | IMD23 | FDD |
|  | n5 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n25A | 66 | N/A | REFSENS | - | - | - | - | N/A | FDD |
|  | n25 | 15 | -76.5+TT | - | - | - | - | IMD3 | FDD |
|  | 66 | N/A | -75.8 | - | - | - | - | IMD3 | FDD |
|  | n25 | 15 | REFSENS | - | - | - | - | N/A | FDD |
|  | 66 | N/A | -94.8 | - | - | - | - | IMD5 | FDD |
|  | n25 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n71A | 66 | N/A | -93.8 | - | - | - | - | IMD4 | FDD |
|  | n71 | 15 | REFSENS | - | - | - | - | N/A | FDD |
| DC_66A_n77A | 66 | N/A | -67.8 | - | - | - | - | IMD2 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
|  | 66 | N/A | -93.8 | - | - | - | - | IMD5 | FDD |
|  | n77 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| DC_66A_n78A | 66 | N/A | -93.8 | - | - | - | - | IMD5 | FDD |
|  | n78 | 15 | - | REFSENS | - | - | - | N/A | TDD |
| NOTE 1: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2.5-2 for 4 antenna port NR band. NOTE 5: Void NOTE 6: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 7: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3.5-5: Reference sensitivity exceptions due to dual uplink operation for PC2 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band |  | SCS (kHz) |  | 5 MHz (dBm) |  | 10 MHz (dBm) | 15 MHz (dBm)) | 20 MHz (dBm) | 40 MHz (dBm) | IMD order) | Duplex mode |
| DC_1A_n78A | 1 |  | N/A |  | REFSENS-17.8 |  | - | - | - | - | IMD4 |  |
|  | n78 |  | 15 |  | - |  | REFSENS | - | - | - | N/A | TDD |
| DC_2A_n77A |  | 2 |  | N/A |  | -65.2 | - | - | - | - | IMD2 | FDD |
| DC_2A-2A_n77A |  |  |  |  |  | -65.157 |  |  |  |  |  |  |
| DC_2A_n77C |  | n77 |  | 15 |  | - | REFSENS | - | - | - | N/A | TDD |
| DC_2A-2A_n77C |  | 2 |  | N/A |  | -78.2 | - | - | - | - | IMD49 | FDD |
|  |  |  |  |  |  | -78.157 |  |  |  |  |  |  |
|  |  | n77 |  | 15 |  | - | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n41A | 3 |  | N/A |  | -77.9 |  | - | - | - | - | IMD4 | FDD |
|  |  |  |  |  | -80.67 |  | - | - | - | - |  |  |
|  | n41 |  | 15 |  | - |  | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n78A | 3 |  | N/A |  | -64.4 |  | - | - | - | - | IMD2 | FDD |
|  |  |  |  |  | -67.17 |  | - | - | - | - |  |  |
|  | n78 |  | 15 |  | - |  | REFSENS | - | - | - | N/A | TDD |
| DC_3A_n78A | 3 |  | N/A |  | -77.8 |  | - | - | - | - | IMD4 | FDD |
|  |  |  |  |  | -80.57 |  | - | - | - | - |  |  |
|  | n78 |  | 15 |  | - |  | REFSENS | - | - | - | N/A | TDD |
| DC_5A_n77A8 |  | 5 |  | N/A |  | -78.7 | - | - | - | - | IMD49 | FDD |
| DC_5A_n77C8 |  | n77 |  | 15 |  | - | REFSENS | - | - | - | N/A | TDD |
| DC_13A_n77A |  | 13 |  | N/A |  | -80.93 | - | - | - | - | IMD5 | FDD |
| DC_13A_n77C |  | n77 |  | 15 |  | - | REFSENS | - | - | - | N/A | TDD |
| DC_66A_n77A DC_66A-66A_n77A DC_66A-66A-66A_n77A DC_66A_n77C DC_66A-66A_n77C DC_66A-66A-66A_n77C |  | 66 |  | N/A |  | -64.47 | - | - | - | - | IMD2 | FDD |
|  |  | n77 |  | 15 |  | - | REFSENS | - | - | - | N/A | TDD |
|  |  | 66 |  | N/A |  | -87.53 | - | - | - | - | IMD5 | FDD |
|  |  | n77 |  | 15 |  | - | REFSENS | - | - | - | N/A | TDD |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. In case Single UL is allowed and the UE only indicates support of "Single UL" the output power of the active UL shall be set at PCMAX_L,c or set to the maximum output power according to the UE power scaling capability. NOTE 2: RBSTART = 0 NOTE 3: This band is subject to IMD5 also which MSD is not specified. NOTE 4: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2.5-2 for 4 antenna port NR band. NOTE 5: For UEs only indicating support of Single UL, this requirement is verified with non-simultaneous uplink transmissions on the E-UTRA and NR CGs. NOTE 6: TT is the same as defined in Table 7.3B.2.3.5-1a. NOTE 7: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 from TS 38.101-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 9: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |  |  |  |  |

#### 7.3B.2.3_1	Reference sensitivity for EN-DC within FR1 (>2 CCs)

##### 7.3B.2.3_1.1	Reference sensitivity for EN-DC within FR1 (3 CCs)

Editor's note:	The test requirements section of this test case contains some EN-DC configurations that have no test points defined meaning these cannot be tested.

Editor's note:	For EN-DC configurations with >1NR CC, test points in Table 7.3B.2.3_1.1.4.1-0 only need to be tested if the configuration is a max NR CC of the UE, where detailed execution rules are FFS.

7.3B.2.3_1.1.1	Test purpose

To verify the ability of UE that support EN-DC configurations to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise when no CA exceptions are allowed and single carrier requirements apply whenever possible. A UE unable to meet the throughput requirement under these conditions will decrease the effective coverage area.

7.3B.2.3_1.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 3CC EN-DC.

7.3B.2.3_1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

7.3B.2.3_1.1.4	Test description

7.3B.2.3_1.1.4.1	Initial conditions

Same initial conditions as in clause 7.3B.2.3.4.1 with following exceptions:

The initial test configurations for E-UTRA band and NR band consist of environmental conditions, test frequencies, and channel bandwidths and RB allocations for exceptional test scenarios are specified in Table 7.3B.2.3_1.1.4.1-0 and 7.3B.2.3_1.1.4.1-1.

Table 7.3B.2.3_1.1.4.1-0: Test Configuration Table for all EN-DC configurations for FR1 non-exception requirements

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  | DC_XA_nYA-nZA Mid range for MCG and SCG DC_XA_nYC: Mid range for XA Low range, High Range for nXC DC_(n)XCA: Low range, High Range DC_XA_nY(2A): TBD |  |  |  |  |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1. |  |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB" columns |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  | SCC1 – EUTRA/NR |  |  |  |  |  | SCG -NR |  |  |  |
|  | Band | Range | NRB |  | Band | Range |  |  | NRB |  | Band | Range | NRB |  |
|  | UL MOD | DL MOD | CH BW | DLalloc/UL alloc | UL MOD | DL MOD |  |  | UL/DL Ch BW | DLalloc/ ULalloc | UL MOD | DL MOD | UL/DL Ch BW | DLalloc/UL alloc |
| Test Settings for a DC_(n)XCA Configuration (Intra-band contiguous EN-DC with LTE CA)– Note 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | X | default |  |  |  |  | nX | default |  |  |
|  | QPSK | QPSK | Highest | All RBs / REFSENS_LTE | N/A | QPSK |  |  | Highest | All RBs / 0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | Highest | All RBs / REFSENS_NR |
| Test Settings for a DC_XC_nXA Configuration (Intra-band non-contiguous EN-DC with LTE CA)– Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.2 |  |  |  |  |  |
| Test Settings for a DC_XA-XA_nXA/ DC_XA_(n)XAA Configuration (Intra-band non-contiguous EN-DC with LTE CA)– Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.2 |  |  |  |  |  |
| Test Settings for a DC_XA_nYC Configuration (Inter-band EN-DC with NR CA, 2 bands) – Note 1,5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | nY | default |  |  |  |  | nY | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | N/A | CP-OFDM QPSK |  |  | Highest | All RBs / 0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | Highest | All RBs / REFSENS_NR |
| Test Settings for a DC_XA_nY(2A) Configuration (Inter-band EN-DC with NR CA, 2 bands) – Note 1,5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | default |  |  | nY | default |  |  |  |  | nY | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | N/A | CP-OFDM QPSK |  |  | Highest | All RBs / 0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | Highest | All RBs / REFSENS_NR |
| Test Settings for a DC_XC_nYA, DC_XA-XA_nYA Configuration (Inter-band EN-DC with LTE CA, 2 bands) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |
| Default Test Settings for a DC_XA-YA_nZA Configuration (Inter-band EN-DC with LTE CA, 3 bands) – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |
| Default Test Settings for a DC_XA_nYA-nZA Configuration (Inter-band EN-DC with NR CA, 3 bands) – Note 1,5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | X | Mid |  |  | nY | default |  |  |  |  | nZ | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | N/A | CP-OFDM QPSK |  |  | Highest | All RBs / 0 | DFT-s-OFDM QPSK | CP-OFDM QPSK | Highest | All RBs / REFSENS_NR |
| 2 | X | Mid |  |  | nY | default |  |  |  |  | nZ | default |  |  |
|  | N/A | N/A | 5 MHz | 0/0 | DFT-s-OFDM QPSK | CP-OFDM QPSK |  |  | Highest | All RBs / REFSENS_NR | N/A | CP-OFDM QPSK | Highest | All RBs / 0 |
| Note 1: LTE anchor agnostic configuration Note 2: Not LTE anchor agnostic configuration due to exception requirement for intra-band contiguous CA in clause 7.4B, 7.5B, 7.6B.2, 7.6B.3, 7.6B.4, 7.7B, 7.8B, 7.9B test cases Note 3: LTE anchor agnostic configuration with LTE CA where LTE CA fallback to 1CC is sufficient to test Note 4: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. Note 5: For UE supporting multiple EN-DC configurations with the same NR CA combination, only one EN-DC configuration is tested. Note 6: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3_1.1.4.1-1: Test Configuration Table for EN-DC configurations affected by Reference sensitivity exceptions

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR Test Frequencies as specified in TS 38.508-1 [6] clause4.3.1, E-UTRA Test Frequencies as specified in TS 36.508 [11] clause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | EN-DC configurations containing the following band combinations: 20-n78: Mid in band 20 and Mid in band 78. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NR Test Channel Bandwidths as specified in TS 38.508-1 [6] clause 4.3.1 E-UTRA Test Channel Bandwidths as specified in TS 36.508 [11] clause 4.3.1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Refer to "NR NRB" and "E-UTRA NRB" columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Network signalling value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test SCS for the NR cell as specified in TS 38.521-1 [8] Table 5.3.5-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Parameters for DC Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID | PCC – E-UTRA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SCC1 – EUTRA/NR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CG -NR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Band |  |  |  | Range | NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Band |  |  | Range |  |  |  |  |  |  |  | NRB |  |  |  |  |  |  |  |  |  |  |  |  | Band |  |  |  |  | Range |  |  |  |  |  |  |  |  |  | NRB |  |  |  |  |  |  |  |
|  | UL MOD |  |  |  | DL MOD | CH BW |  |  |  |  |  | DLalloc/UL alloc |  |  |  |  |  |  |  |  | UL MOD |  |  | DL MOD |  |  |  |  |  |  |  | UL/DL Ch BW |  |  |  |  |  |  |  |  | DLalloc/ ULalloc |  |  |  | UL MOD |  |  |  |  | DL MOD |  |  |  |  |  |  |  |  |  | UL/DL Ch BW |  |  | DLalloc/UL alloc |  |  |  |  |
| Default Test Settings for a DC_XA_nYC, DC_XA_nY(2A) Configuration (Inter-band EN-DC with NR CA, 2 bands) – Note 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No test required, NR 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XC_nYA, DC_XA-XA_nYA Configuration (Inter-band EN-DC with LTE CA, 2 bands) – Note 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA-YA_nZA Configuration (Inter-band EN-DC with LTE CA, 3 bands) – Note 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No test required, LTE 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA_nYA-nZA Configuration (Inter-band EN-DC with NR CA, 3 bands) – Note 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | No test required, NR 1CC fallback is tested in 7.3B.2.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Settings for DC_1A-3A_n28A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  |  |  | UL 1975 / DL 2165 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  | DL 1818.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 710.5 / DL 765.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  | QPSK | 5 MHz |  |  |  |  |  | All RBs / All RBs |  |  |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / All RBs |  |  |  |  |  |  |
| 2 | 3 |  |  |  | UL 1780 / DL 1875 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  | DL 2139 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 710.5 / DL 765.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  | QPSK | 5 MHz |  |  |  |  |  | All RBs / All RBs |  |  |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s- OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / All RBs |  |  |  |  |  |  |
| Test Settings for DC_1A-7A_n28A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  |  |  | UL 1935 / DL 2125 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  | DL 2653 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 718 / DL 773 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  | QPSK | 5 MHz |  |  |  |  |  | All RBs / All RBs |  |  |  |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  |  | All RBs / 0 |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  |  | 5 MHz |  |  | All RBs / All RBs |  |  |  |  |
| Test Settings for DC_1A-3A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | UL 1950 / DL 2140 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  | DL 1807.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3757.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 1 |  | DL 2125 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  | UL 1775 / DL 1870 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3725 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_1A-5A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | DL 2122 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |  |  |  |  |  | UL 829 / DL 874 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3780 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 1 |  | UL 1975 / DL 2165 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |  |  |  |  |  | DL 885 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3405 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_1A-7A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | UL 1977.5 / DL 2167.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  | DL 2627.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3305 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 1 |  | DL 2140 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  | UL 2510 / DL 2630 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3580 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 10MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_1A-20A_n8A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | UL 1925/ DL 2115 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | DL 805 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  | UL 910 / DL 955 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_1A-20A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | DL 2120 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | UL 835 / DL 794 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3790 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 1 |  | UL 1950 / DL 2140 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | DL 810 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3330 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_1A-28A_n3A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | DL 2139 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  | UL 710.5 / DL 765.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | UL 1780 / DL 1875 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_1A_n28A-n78A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 1 |  | UL 1950 / DL 2140 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  |  |  | 3416 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  |  |  |  | UL 733 / DL 788 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | N/A |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / All RBs |  |  |
| 2 | 1 |  | UL 1950 / DL 2140 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  |  |  |  | DL 790 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  |  |  | 3320 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | N/A |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / All RBs |  |  |
| Test Settings for DC_1A-28A_n78A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 |  | UL 740 / DL 795 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  | DL 2150 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3630 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 1 |  | UL 1970 / DL 2160 |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  | DL 794 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3352 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_2A-66A_n41A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 |  | UL 1860 / DL 1940 MHz |  |  |  | 5 MHz |  |  |  |  |  |  |  |  |  | 66 |  |  |  |  |  |  |  | UL 1715 / DL 2115 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n41 |  |  |  |  | 2685 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_3A-7A_n5A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | UL 1780/ DL 1875 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  | DL 2625 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  | UL 845 / DL 890 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_3A-7A_n8A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | UL 1780/ DL 1875 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  | DL 2670 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  | UL 890 / DL 935 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_3A-7A_n28A – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  |  |  | UL 1712.5 / DL 1807.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  | DL 2682 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  | UL 743 / DL 798 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  | QPSK |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 10 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |
| 2 | 7 |  |  |  | UL 2543 / DL 2663 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  | DL 1832.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  | UL 710.5 / DL 765.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  |  |  | QPSK |  |  |  |  | 10 MHz |  |  |  | All RBs / All RBs |  |  |  |  |  | N/A |  |  | QPSK |  |  |  |  |  | 5 MHz |  |  |  |  |  |  | All RBs / 0 |  |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |
| Test Settings for DC_3A-7A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | DL 1820 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  | UL 2565 / DL 2685 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3310 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 3 |  | DL 1820 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  | UL 2565 / DL 2685 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3475 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_3A-8A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | DL 1820 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  | UL 910 / DL 955 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3640 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_3A-20A_n8A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | DL 1860 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | UL 840 MHz / DL 799 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  | UL 900 / DL 945 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_3A-20A_n28A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | DL 1828 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | UL 852 / DL 811 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 728 / DL 783 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_3A-20A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | DL 1820 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | UL 845 / DL 804 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3510 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_3A-28A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 |  | UL 740 / DL 760 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  | UL 1775 / DL 1870 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3350 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_3A_n28A-n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | UL 1750/ DL 1845 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  |  | 3764 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 743 / DL 798 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_3A-40A_n1A Configuration – Note 3, 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 3 |  | UL 1735 / DL 1830 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 |  |  |  |  |  |  |  | DL 2380 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1A |  |  |  |  | UL 1950 / DL 2140 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_5A-7A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 5 |  | UL 844 / DL 889 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  | DL 2645 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3489 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 5 |  | DL 879 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  | UL 2550 / DL 2670 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3429 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 3 | 5 |  | DL 875 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  | UL 2525 / DL 2645 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3350 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_7A-8A_n3A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | UL 2530 / DL 2650 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  | DL 940 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | UL 1735 / DL 1830 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 7 |  | DL 2670 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  | UL 890 / DL 935 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | UL 1780 / DL 1875 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_7A-20A_n1A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | UL 2510 / DL 2630 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | 800 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n1 |  |  |  |  | UL 1940 / DL 2130 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 10 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_7A-20A_n3A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | UL 2543 / DL 2663 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | DL 806 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | UL 1737 / DL 1832 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 7 |  | DL 2630 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | UL 855 / DL 896 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | UL 1775 / DL 1870 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_7A-20A_n8A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | UL 2565 / DL 2685 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | DL 795 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  | UL 885 / DL 930 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 7 |  | DL 2640 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | UL 840 / DL 799 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  | UL 900 / DL 945 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 3 | 7 |  | DL 2624 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | UL 857 / DL 816 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n8 |  |  |  |  | UL 910 / DL 955 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_7A-20A_n28A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 20 |  | UL 842/ DL 801 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  | DL 2640 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 728 / DL 783 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  | N/A |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  |  | All RBs / All RBs |  |
| Test Settings for DC_7A-20A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | UL 2560 / DL 2680 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | DL 810 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3370 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 7 |  | UL 2560 / DL 2680 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | DL 810 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3435 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 3 | 7 |  | DL 2675 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 |  |  |  |  |  |  |  | UL 845 / DL 804 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3520 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_7A-28A_n3A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | UL 2543 / DL 2663 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  | DL 796 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | 1842 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 7 |  | DL 2685 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  | UL 745 / DL 800 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n3 |  |  |  |  | 1810 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_7A-28A_n5A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | UL 2540 / DL 2660 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  | DL 776 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  | UL 829 / DL 874 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 7 |  | DL 2630 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  | UL 730 / DL 785 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n5 |  |  |  |  | UL 840 / DL 874 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_7A-28A_n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | UL 2567.5 / DL 2687.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  | DL 782.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3350 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 7 |  | UL 2567.5 / DL 2687.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  | DL 782.5 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3460 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 3 | 7 |  | DL 2650 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 28 |  |  |  |  |  |  |  | UL 740 / DL 795 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3390 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_7A_n28A-n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 7 |  | UL 2565/ DL 2685 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  |  | 3310 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 745 / DL 800 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 7 |  | UL 2565/ DL 2685 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  |  |  | DL 800 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3365 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_2A-14A_n66A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 |  | DL 1954 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 14 |  |  |  |  |  |  |  | UL 793 / DL 763 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n66 |  |  |  |  | UL 1770 / DL 2170 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | N/A |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / 0 |  | QPSK |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / All RBs |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_14A-66A_n2A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 14 |  | DL 763 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | 66 |  |  |  |  |  |  |  | UL 1762 / DL 2162 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n2 |  |  |  |  | UL 1874 / DL 1954 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_20A_n28A-n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 |  | UL 857 / DL 816 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  |  | 3314 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  | UL 743 / DL 798 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  | 10MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 20 |  | UL 837 / DL 796 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  |  |  | UL 744 / DL 799 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3310 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  | 5MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Test Settings for DC_20A_n28A-n75A Configuration |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 20 |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n75 |  |  |  |  | DL 1476MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20MHz |  |  |  |  |  |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  | 20MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 20MHz |  |  |  |  |  |  |  | All RBs / N/A |  |  |  |
| 2Note 8 | 20 |  | Mid |  |  |  |  |  |  |  |  |  |  |  |  |  | n28 |  |  |  |  |  |  |  | Low |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n75 |  |  |  |  | High |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 20MHz |  |  |  |  |  |  |  | All RBs / 0 |  | DFT-s-OFDM QPSK |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  | 20MHz |  |  |  | All RBs / REFSENS_ENDC_1 |  |  |  |  | N/A |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 20MHz |  |  |  |  |  |  |  | All RBs / N/A |  |  |  |
| Test Settings for DC_28A_n7A-n78A Configuration – Note 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 28 |  | UL 745 / DL 8005 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  |  |  |  | 3310 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n7 |  |  |  |  | UL 2565 / DL 2685 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  | 10 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| 2 | 28 |  | UL 745 / DL 8005 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  | n7 |  |  |  |  |  |  |  | DL 2650 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | n78 |  |  |  |  | 3390 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | QPSK |  | QPSK |  |  |  | 5 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  | N/A |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  |  | 5 MHz |  |  |  | All RBs / 0 |  |  |  |  | DFT-s-OFDM QPSK |  |  |  |  | CP-OFDM QPSK |  |  |  |  |  |  |  | 10 MHz |  |  |  |  |  |  |  | All RBs / All RBs |  |  |  |
| Note 1: Void Note 2: Void Note 3: EN-DC configuration affected by 2UL intermodulation exception. The exceptions always apply for a certain UL configuration. Note 4: LTE CA fallback to 1CC is sufficient to test, unless both LTE cells are part of the exception requirement in which case the configuration need to be tested (using configuration specific test settings and not default). Note 5: In an E-UTRA band or FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. Note 6: Test only applicable for to UEs not supporting UE capability singleUL-Transmissionn for the correspondent uplink configuration. Note 7: NR CA fallback to 1CC is sufficient to test, unless both NR cells are part of the exception requirement in which case the configuration need to be tested (using configuration specific test settings and not default). Note 8: Test ID with UL harmonic exception avoided. Note 9: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. REFSENS_ENDC_1 refers to the Uplink RB allocation for reference sensitivity exceptions due to UL harmonic interference according to table 7.3B.2.0.3.1-2. REFSENS_ENDC_2 refers to the Uplink RB allocation for reference sensitivity exceptions due to receiver harmonic mixing according to table 7.3B.2.0.3.2-2. REFSENS_ENDC_3 refers to the Uplink RB allocation for reference sensitivity exceptions due to cross band isolation according to table 7.3B.2.0.3.4-2. REFSENS_ENDC_4 refers to the Uplink RB allocation for reference sensitivity exceptions due to dual uplink operation for ENDC according to table 7.3B.2.0.3.5.1-1 for PC3 and table 7.3B.2.0.3.5.1-1a for PC2. NOTE 10: If the NR frequency does not match to a valid NR-ARFCN, apply the closest NR frequency with a valid NR-ARFCN. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

7.3B.2.3_1.1.4.2	Test procedure

Same as in clause 7.3B.2.3.4.2.

7.3B.2.3_1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band.

Message contents exceptions for E-UTRA band are according to TS 36.521-1 [10] clause 7.3.4.3 for each network signalling value. Message contents exceptions for NR band are according to TS 38.521-1 [8] clause 7.3.2.4.3 for each network signalling value.

For test points with Note 3 in Table 7.3B.2.3_1.1.4.1-1, the following message exception applies:

Table 7.3B.2.3.4.2.3-1: RRCConnectionReconfiguration: nr-Config-r15

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-MaxEUTRA-r15 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

Table 7.3B.2.3.4.2.3-2: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6] Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 2 UE |
|  | 20 |  | Power Class 3 UE |

7.3B.2.3_1.1.5	Test requirement

Reference sensitivity test requirements for EN-DC configurations affected by 3 band 2UL intermodulation interference, are specified in Table 7.3B.2.3_1.1.5-1 and Table 7.3B.2.3_1.1.5-2 with uplink configuration specified in Table 7.3B.2.3_1.1.4.1-1.

Reference sensitivity test requirements for test points in Table 7.3B.2.3_1.1.4.1-1, are specified in Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC.

Table 7.3B.2.3_1.1.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | Test ID |  | EUTRA/ NR band |  | SCS (kHz) |  | 5 MHz (dBm) |  | 10 MHz (dBm) |  | 20 MHz (dBm) |  | 40 MHz (dBm) |  | Duplex mode |  | IMD order |  | Single UL allowed |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A-3A_n28A |  | 1 |  | 1 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n28 |  | 15 |  | REFSENS |  | - |  | - |  | - |  |  |  | N/A |  |  |  |
|  |  |  |  | 3 |  | N/A |  | -92.3 |  | - |  | - |  | - |  |  |  | IMD5 |  |  |  |
|  |  | 2 |  | 3 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n28 |  | 15 |  | REFSENS |  | - |  | - |  | - |  |  |  | N/A |  |  |  |
|  |  |  |  | 1 |  | N/A |  | -88.3 |  | - |  | - |  | - |  |  |  | IMD4 |  |  |  |
| DC_1A-7A_n28A | 1 |  | 1 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n28 |  | 15 |  | REFSENS |  | - |  | - |  | - |  |  |  | N/A |  |  |  |
|  |  |  | 7 |  | N/A |  | - |  | -64.3 |  | - |  | - |  |  |  | IMD2 |  |  |  |
| DC_1A-3A_n77A | 1 |  | 1 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 3 |  | N/A |  | -64.8 |  | - |  | - |  | - |  |  |  | IMD2 |  |  |  |
|  |  |  | n77 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 2 |  | 1 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 3 |  | N/A |  | -87.8 |  | - |  | - |  | - |  |  |  | IMD4 |  |  |  |
|  |  |  | n77 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 3 |  | 1 |  | N/A |  | -68.3 |  | - |  | - |  | - |  | FDD |  | IMD2 |  |  |  |
|  |  |  | 3 |  | N/A |  | REFSENS |  | - |  | - |  | - |  |  |  | N/A |  |  |  |
|  |  |  | n77 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_1A-3A_n78A DC_1A-3C_n78A | 1 |  | 1 |  | N/A |  | -91.0 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  | 3 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 2 |  | 1 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 3 |  | N/A |  | -65.1 |  | - |  | - |  | - |  |  |  | IMD2 |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 3 |  | 1 |  | N/A |  | -96.5 |  | - |  | - |  | - |  | FDD |  | IMD5 |  |  |  |
|  |  |  | 3 |  | N/A |  | REFSENS |  | - |  | - |  | - |  |  |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_1A-5A_n78A | 1 |  | 1 |  | N/A |  | -91.0 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  | 5 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 2 |  | 1 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 5 |  | N/A |  | -89.0 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 3 |  | 1 |  | N/A |  | -81.2 |  | - |  | - |  | - |  | FDD |  | IMD3 |  |  |  |
|  |  |  | 5 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 4 |  | 1 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 5 |  | N/A |  | -94.2 |  | - |  | - |  | - |  | FDD |  | IMD5 |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_1A-7A_n78A | 1 |  | 1 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 7 |  | N/A |  | -88.2 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 2 |  | 1 |  | N/A |  | -90.6 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  | 7 |  | N/A |  | - |  | REFSENS |  | - |  |  |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_1A-20A_n8A | 1 |  | 1 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 20 |  | N/A |  | -87.8 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  | n8 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
| DC_1A-20A_n78A | 1 |  | 1 |  | N/A |  | -79.0 |  | - |  | - |  | - |  | FDD |  | IMD3 |  |  |  |
|  |  |  | 20 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_1A-20A_n78A | 2 |  | 1 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 20 |  | N/A |  | -93.3 |  | - |  | - |  | - |  | FDD |  | IMD5 |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_1A-28A_n3A | 1 |  | 28 |  | N/A |  | REFSENS |  |  |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n3 |  | 15 |  | REFSENS |  |  |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 1 |  | N/A |  | -88.3 |  |  |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
| DC_1A_n28A-n78A |  | 1 |  | 1 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n28 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n78 |  | 15 |  | - |  | -80.1 + TT |  | - |  | - |  | TDD |  | IMD3 |  |  |  |
|  |  | 2 |  | 1 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  |  |  |  | n28 |  | 15 |  | -95.2 + TT |  | - |  | - |  | - |  | FDD |  | IMD5 |  |  |  |
| DC_1A-28A_n78A | 1 |  | 1 |  | N/A |  | -83.6 |  | - |  | - |  | - |  | FDD |  | IMD3 |  |  |  |  |
|  |  |  | 28 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |  |
|  | 2 |  | 1 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |  |
|  |  |  | 28 |  | N/A |  | -93.6 |  | - |  | - |  |  |  | FDD |  | IMD5 |  |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |  |
| DC_2A-14A_n66A | 1 |  | 2 |  | N/A |  | -90.1 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  | 14 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n66 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
| DC_2A-66A_n41A | 1 |  | 2 |  | N/A |  | -74.4 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  | 66 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n41 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_3A-5A_n78A | 1 |  | 3 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 5 |  | N/A |  | -89.0 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 2 |  | 3 |  | N/A |  | -70.3 |  | - |  | - |  | - |  | FDD |  | IMD2 |  |  |  |
|  |  |  | 5 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 3 |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  |  |  | 3 |  | N/A |  | -88.3 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  |  |  |  |  | [TBD] |  |  |  |  |  | 10.75 |  |  |  |  |  |  |  |
| DC_3A-7A_n5A |  | 1 |  | 3 |  | N/A |  | - |  | REFSENS |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | 7 |  | N/A |  | - |  | - 64.3 |  | - |  | - |  | FDD |  | IMD2XX |  |  |  |
|  |  |  |  | n5 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
| DC_3A-7A_n8A |  | 1 |  | 3 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | 7 |  | N/A |  | - |  | -65.3 |  | - |  | - |  | FDD |  | IMD2 IMD38 |  |  |  |
|  |  |  |  | n8 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
| DC_3A-7A_n28A |  | 1 |  | 3 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n28 |  | 15 |  | REFSENS |  | - |  | - |  | - |  |  |  | N/A |  |  |  |
|  |  |  |  | 7 |  | N/A |  | - |  | -77.4 |  | - |  | - |  |  |  | IMD3 |  |  |  |
|  |  | 2 |  | 7 |  | N/A |  | - |  | REFSENS |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n28 |  | 15 |  | REFSENS |  | - |  | - |  | - |  |  |  | N/A |  |  |  |
|  |  |  |  | 3 |  | N/A |  | -70.3 |  | - |  | - |  | - |  |  |  | IMD2 |  |  |  |
| DC_3C-7C_n78A | 1 |  | 3 |  | N/A |  | -78.7 |  | - |  | - |  | - |  | FDD |  | IMD3 |  |  |  |
|  |  |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 2 |  | 3 |  | N/A |  | -87.7 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_3A-7A_n78A DC_3C-7A_n78A | 1 |  | 3 |  | N/A |  | -78.7 |  | - |  | - |  | - |  | FDD |  | IMD3 |  |  |  |
|  |  |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_3A-7A_n78A DC_3C-7A_n78A | 2 |  | 3 |  | N/A |  | -87.7 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_3A-8A_n78A | 1 |  | 3 |  | N/A |  | -79.8 |  | - |  | - |  | - |  | FDD |  | IMD3 |  |  |  |
|  |  |  | 8 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_3A-20A_n8A |  | 1 |  | 3 |  | N/A |  | -81.8 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  |  | 20 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n8 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
| DC_3A-20A_n28A |  | 1 |  | 20 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n28 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | 3 |  | N/A |  | -86.9 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
| DC_3A-20A_n78A DC_3C-20A_n78A | 1 |  | 3 |  | N/A |  | -79.0 |  | - |  | - |  | - |  | FDD |  | IMD3 |  |  |  |
|  |  |  | 20 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_3A_n28A-n78A | 1 |  | 3 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |  |
|  |  |  | n28 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | -91.3+TT |  | - |  | - |  | TDD |  | IMD5 |  |  |  |  |
| DC_3A-28A_n78A | 1 |  | 3 |  | N/A |  | -79 |  | - |  | - |  | - |  | FDD |  | IMD3 |  |  |  |
|  |  |  | 28 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_3A-40A_n1A | 1 |  | n1 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 3 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 40 |  | N/A |  | -91.3 |  | - |  | - |  | - |  | TDD |  | IMD5 |  |  |  |
| DC_5A-7A_n78A | 1 |  | 5 |  | N/A |  | -89.0 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 2 |  | 5 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 7 |  | N/A |  | -67.2 |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 3 |  | 5 |  | N/A |  | -67.1 |  | - |  | - |  | - |  | FDD |  | IMD2 |  |  |  |
|  |  |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 4 |  | 5 |  | N/A |  | -94.0 |  | - |  | - |  | - |  | FDD |  | IMD5 |  |  |  |
|  |  |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_7A-8A_n3A | 1 |  | 7 |  | N/A |  | - |  | REFSENS |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 8 |  | N/A |  | -78.3 |  | - |  | - |  | - |  | FDD |  | IMD3 |  |  |  |
|  |  |  | n3 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  | 2 |  | 7 |  | N/A |  | - |  | -65.3 |  | - |  | - |  | FDD |  | IMD2+ IMD3YY |  |  |  |
|  |  |  | 8 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n3 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
| DC_7A-20A_n1A | 1 |  | 7 |  |  |  |  |  | REFSENS |  |  |  |  |  | FDD |  | N/A |  |  |  |
|  |  |  | 20 |  |  |  | -88.8 |  |  |  |  |  |  |  | FDD |  | IMD5 |  |  |  |
|  |  |  | n1 |  |  |  | REFSENS |  |  |  |  |  |  |  | FDD |  | N/A |  |  |  |
| DC_7A-20A_n3A | 1 |  | 7 |  | N/A |  | - |  | REFSENS |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 20 |  | N/A |  | - |  | -82.8 |  | - |  | - |  | FDD |  | IMD2 |  |  |  |
|  |  |  | n3 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  | 2 |  | 7 |  | N/A |  | - |  | -68.3 |  | - |  | - |  | FDD |  | IMD2 |  |  |  |
|  |  |  | 20 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n3 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | FDD |  | N/A |  |  |  |
| DC_7A-20A_n8A | 1 |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 20 |  | N/A |  | -78.9 |  | - |  | - |  | - |  | FDD |  | IMD3 |  |  |  |
|  |  |  | n8 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  | 2 |  | 7 |  | N/A |  | -76.2 |  | - |  | - |  | - |  | FDD |  | IMD3 |  |  |  |
|  |  |  | 20 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n8 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  | 3 |  | 7 |  | N/A |  | -78.5 |  | - |  | - |  | - |  | FDD |  | IMD3 |  |  |  |
|  |  |  | 20 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n8 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
| DC_7A-20A_n28A |  | 1 |  | 7 |  | N/A |  | - |  | -91.4 |  | - |  | - |  | FDD |  | IMD5 |  |  |  |
|  |  |  |  | 20 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n28 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
| DC_7A-20A_n78A | 1 |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 20 |  | N/A |  | -65.8 |  | - |  | - |  | - |  | FDD |  | IMD2 |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_7A-20A_n78A | 2 |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 20 |  | N/A |  | -93.3 |  | - |  | - |  | - |  | FDD |  | IMD5 |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_7A-20A_n78A | 3 |  | 7 |  | N/A |  | -66.5 |  | - |  | - |  | - |  | FDD |  | IMD2 |  |  |  |
|  |  |  | 20 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_7A-28A_n3A | 1 |  | 7 |  | N/A |  | REFSENS |  |  |  |  |  |  |  | FDD |  | N/A |  |  |  |
|  |  |  | 28 |  | N/A |  | -77.8 |  |  |  |  |  |  |  | FDD |  | IMD2 |  |  |  |
|  |  |  | n3 |  | 15 |  | REFSENS |  |  |  |  |  |  |  | FDD |  | N/A |  |  |  |
|  | 2 |  | 7 |  | N/A |  | -79.3 |  |  |  |  |  |  |  | FDD |  | IMD3 |  |  |  |
|  |  |  | 28 |  | N/A |  | REFSENS |  |  |  |  |  |  |  | FDD |  | N/A |  |  |  |
|  |  |  | n3 |  | 15 |  | REFSENS |  |  |  |  |  |  |  | FDD |  | N/A |  |  |  |
| DC_7A-28A_n5A | 1 |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 28 |  | N/A |  | -93.4 |  | - |  | - |  | - |  | FDD |  | IMD5 |  |  |  |
|  |  |  | n5 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  | 2 |  | 7 |  | N/A |  | -91.9 |  | - |  | - |  | - |  | FDD |  | IMD5 |  |  |  |
|  |  |  | 28 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | n5 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
| DC_7A-28A_n78A | 1 |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 28 |  | N/A |  | -89.5 |  | - |  | - |  | - |  |  |  | IMD2 |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 2 |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 28 |  | N/A |  | -94.8 |  | - |  | - |  | - |  |  |  | IMD5 |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  | 3 |  | 7 |  | N/A |  | -66.8 |  | - |  | - |  | - |  | FDD |  | IMD2 |  |  |  |
|  |  |  | 28 |  | N/A |  | REFSENS |  | - |  | - |  | - |  |  |  | N/A |  |  |  |
|  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_7A_n28A-n78A |  | 1 |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n28 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n78 |  | 15 |  | - |  | -66.1+TT |  | - |  | - |  | TDD |  | IMD2 |  |  |  |
|  |  | 2 |  | 7 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n28 |  | 15 |  | -69.7+TT |  | - |  | - |  | - |  | FDD |  | IMD2 |  |  |  |
|  |  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_20A_n28A-n78A |  | 1 |  | 20 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n28 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n78 |  | 15 |  | - |  | -87.1+TT |  | - |  | - |  | TDD |  | IMD4 |  |  |  |
|  |  | 2 |  | 20 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
|  |  |  |  | n28 |  | 15 |  | -89.1+TT |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
| DC_28A_n7A-n78A |  | 1 |  | 28 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n7 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n78 |  | 15 |  | - |  | -66.1+TT |  | - |  | - |  | TDD |  | IMD2 |  |  |  |
|  |  | 2 |  | 28 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  |  | n7 |  | 15 |  | -67.5+TT |  | - |  | - |  | - |  | FDD |  | IMD2 |  |  |  |
|  |  |  |  | n78 |  | 15 |  | - |  | REFSENS |  | - |  | - |  | TDD |  | N/A |  |  |  |
| DC_14A-66A_n2A | 1 |  | 14 |  | N/A |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
|  |  |  | 66 |  | N/A |  | -91.2 |  | - |  | - |  | - |  | FDD |  | IMD4 |  |  |  |
|  |  |  | n2 |  | 15 |  | REFSENS |  | - |  | - |  | - |  | FDD |  | N/A |  |  |  |
| NOTE 1: E-UTRA carrier shall be set to min(+20 dBm, PCMAX_L_E-UTRA,c) and NR carrier shall be set to min(+20 dBm, PCMAX_L,f,c,NR) as defined in clause 6.2B.4.1.3. NOTE 2: RBSTART = 0 NOTE 3: Void NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: The symbol "REFSENS" in this table refers to the reference sensitivity values for single carrier specified in Table 7.3.5-1 of TS 36.521-1 [10] for 2 antenna port E-UTRA band, Table 7.3_1.5-1 of TS 36.521-1 [10] for 4 antenna port E-UTRA band, Table 7.3.2.5-1 for 2 antenna port NR band and Table 7.3.2.5-2 for 4 antenna port NR band. NOTE 6: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the Band 46. The reference sensitivity should only be verified when this is not the case (the requirements for Band 46 specified in the CA_7A-46A in clause 7.3.1 of TS 36.101 [5] apply). NOTE 7: This band is subject to IMD3 also which MSD is not specified. NOTE 8: This MSD requirement apply with both IMD2 and IMD3 products should be generated. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3_1.1.5-2: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| EN-DC Configuration | EUTRA/ NR band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 20 MHz (dBm) | 40 MHz (dBm) | Duplex mode | IMD order | Single UL allowed |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n78C | 1 | N/A | -92.0+TT | - | - | - | FDD | IMD4 |  |
|  | n78 | 15 | - | REFSENS | - | - | TDD | N/A |  |
|  | n78 | 15 | - | REFSENS | - | - |  | N/A |  |
| NOTE 1: Both of the transmitters shall be set min (+20 dBm, PCMAX_L,c) as defined in clause 6.2.5A. In case Single UL is allowed and the UE only indicates support of "Single UL" the output power of the active UL shall be set at PCMAX_L,c or set to the maximum output power according to the UE power scaling capability. NOTE 2: RBSTART = 0 |  |  |  |  |  |  |  |  |  |

Test tolerance is the same as given in Table 7.3B.2.3.5-2.

##### 7.3B.2.3_1.2	Reference sensitivity for EN-DC within FR1 (4 CCs)

7.3B.2.3_1.2.1	Test purpose

Same as in clause 7.3B.2.3.1.

7.3B.2.3_1.2.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 4 CCs EN-DC.

7.3B.2.3_1.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0.

For EN-DC combinations with no exception requirements applicable to NR or E-UTRA, LTE anchor agnostic approach is applied.

For EN-DC combinations with exceptional requirements, LTE anchor agnostic approach is not applied.

7.3B.2.3_1.2.4	Test description

7.3B.2.3_1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations for E-UTRA consist of the test frequency based on E-UTRA operating band and test channel bandwidth as specified in Table 4.6-1, with the exception that the E-UTRA channel bandwidth is the lowest supported value in Table 5.3B.1.3-1 for the EN-DC non-contiguous configuration under test.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.5B.2-1. The 4CC EN-DC configurations listed in table 7.3B.2.3_1.2.4.1-0 shall not be tested according to TR 38.905 [7] test point analysis. The other 4CC configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Tables 7.3B.2.3.4.2.1-0 to 7.3B.2.3.4.2.1-1 for NR band. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in TS 38.521-1 [8] Annex C2.

Table 7.3B.2.3_1.2.4.1-0: 4CC EN-DC configurations that shall not be tested

| EN-DC type |  | E-UTRA CA | NR CA | Notation |
| --- | --- | --- | --- | --- |
| Intra-band non-contiguous EN-DC (1 band) |  | Yes (cont) | No | DC_XD_nXA |
|  |  | Yes (non-cont) | No | Note 4 |
| Inter-band EN-DC | Yes (all types) | No | DC_XD_nYA, DC_XA-YC_nZA, DC_XA-XA-YA_nZA, DC_XA-YA-ZA_nRA |
|  | Yes (all types) | Yes (cont) | DC_XC_nYC, DC_XA-XA_nYC, DC_XA-YA_nYC |
|  |  | Yes (non-cont) | DC_XC_nY(2A), DC_XA-YA_nY(2A) |
|  |  | Yes (inter) | DC_XC_nYA-nZA, DC_XA-XA_nYA-nZA, DC_XA-YA_nZA-nRA |
| Inter-band + Intra-band contiguous EN-DC (2-3 band) | Yes | No | DC_XA-YA_(n)ZAA, DC_XC_(n)YAA |
|  | Yes | Yes | DC_(n)XCA-nYA |
| Intra-band non contiguous EN-DC (2-3 band) | Yes | No | DC_XA-YA-ZA_nZA |
|  | Yes | Yes | DC_XA-YA_nYA-nZA |
| NOTE 1: X, Y and Z in this table correspond to different bands i.e. X != Y != Z. NOTE 2: The table apply to all band numbers, i.e. all values of X, Y and Z defined in TS38.101-3 NOTE 3: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. DC_YA-(n)XAA represents the set of DC_YA-(n)XAA and DC_(n)XAA-YA. NOTE 4: No such config in TS 38.101-3 [7] V16.5.0x |  |  |  |

Table 7.3B.2.3_1.2.4.1-1: Test Configuration Table for all EN-DC configurations for FR1 non-exception requirements

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  |  | DC_(n)XDA: Low range, High Range DC_XA_nYA-nZC: Mid range for X and nY Low range, High Range for nY DC_XA_nY(2A)-nZA: Mid range for X, nY, nZ |  |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  |  | Refer to "NR NRB"and "E-UTRA NRB" columns |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Lowest SCS per Channel Bandwidth |  |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default Unless given by Table 7.3.3-3 in TS 36.521-1 [10] for the E-UTRA band and Table 7.3.2.3-4 in TS 38.521-1 [8] for the NR band. |  |  |  |  |
| Test Parameters for EN-DC Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CC (NOTE1) | Band | SCS | NRB | Range/ Wgap |  | UL MOD | DL MOD | UL Alloc | DL Alloc |
| Default Test Settings for a DC_(n)XDA Configuration – Note 5 |  |  |  |  |  |  |  |  |  |  |
| 1 (Note 6) | PCC(M) | X |  | Highest NRB | default |  | QPSK | QPSK | REFSENS_LTE | All RBs |
|  | SCC1(M) | X |  | Highest NRB | Default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(M) | X |  | Highest NRB | Default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | nX |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS_NR | All RBs |
| 2 (Note 7) | PCC(M) | X |  | Highest NRB | default |  | N/A | QPSK | 0 | All RBs |
|  | SCC1(M) | X |  | Highest NRB | Default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(M) | X |  | Highest NRB | Default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | nX |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS_NR | All RBs |
| Default Test Settings for a DC_XA_nY(2A)-nZA Configuration – Note 4 |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X |  | 5 MHz | default |  | N/A | QPSK | 0 | All RBs |
|  | SCC1(S) | nY |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | nZ |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | nY |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS_NR | All RBs |
| 2 | PCC(M) | X |  | 5 MHz | default |  | N/A | QPSK | 0 | All RBs |
|  | SCC1(S) | nY |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | nY |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | nZ |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS_NR | All RBs |
| Default Test Settings for a DC_XA_nYA-nZC Configuration– Note 4 |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X |  | 5 MHz | default |  | N/A | QPSK | 0 | All RBs |
|  | SCC1(S) | nZ |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | nZ |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | nY |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS_NR | All RBs |
| 2 | PCC(M) | X |  | 5 MHz | default |  | N/A | QPSK | 0 | All RBs |
|  | SCC1(S) | nZ |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(S) | nY |  | Highest NRB | default |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | nZ |  | Highest NRB | default |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| NOTE 1: (M) and (S) indicate MCG and SCG respectively. NOTE 2: X, Y and Z in this table correspond to different bands i.e. X != Y != Z. NOTE 3: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. DC_YA-(n)XAA represents the set of DC_YA-(n)XAA and DC_(n)XAA-YA. NOTE 4: LTE anchor agnostic configuration NOTE 5: Not LTE anchor agnostic configuration due to exception requirement for intra-band contiguous CA in clause 7.4B, 7.5B, 7.6B.2, 7.6B.3, 7.6B.4, 7.7B, 7.8B, 7.9B test cases NOTE 6: Test point for UE supporting dual UL NOTE 7: Test point for UE supporting single UL NOTE 8: In a FR1 band where UE supports 4Rx, the test shall be performed only with 4Rx antennas ports connected. NOTE 9: UE supporting multiple EN-DC configurations with the same NR CA combination, only one EN-DC configuration is tested. NOTE 10: REFSENS_LTE refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.3-2 of TS 36.521-1 [10]. REFSENS_NR refers to the single carrier Uplink RB allocation for reference sensitivity according to table 7.3.2.4.1-3 of TS 38.521-1 [8]. |  |  |  |  |  |  |  |  |  |  |

For 4CC EN-DC configurations affected by exceptions, exception testing can be covered by 2CC or 3CC fallback configurations. No exception test points are needed in this test case.

Table 7.3B.2.3_1.2.4.1-2: Void

Table 7.3B.2.3_1.2.4.1-3: Void

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, Figure A.3.1.1.1 for TE diagram and clause A.3.2 for UE diagram.

2.	The parameter settings for NR cell are set up according to TS 38.508-1 [6] clause 4.4.3.

3.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

4.	NR downlink signals are initially set up according to Annex C.0, C.1, C.2, C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1 of TS 38.521-1 [8].

5.	E-UTRA downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1 [10].

6.	The DL and UL Reference Measurement channels are set according to Tables 7.3B.2.3.4.2.1-0 to 7.3B.2.3.4.2.1-6 for E-UTRA CG and NR CG.

7.	NR propagation conditions are set according to Annex B.0 of TS 38.521-1 [8]. E-UTRA propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

8.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 7.3B.2.3_1.2.4.3.

9.	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.6-1 under clause 4.6.

7.3B.2.3_1.2.4.2	Test procedure

1.	NR SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.3B.2.3_1.2.4.2.1-1 on the NR CC. The NR SS sends downlink MAC padding bits on the DL RMC.

2.	NR SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.3B.2.3_1.2.4.2.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the appropriate REFSENS value defined in TS 38.521-1 [8], Table 7.3.2.5-1 for NR band. Send continuously uplink power control "up" commands in the uplink scheduling information to NR carrier to ensure the UE transmits PUMAX level for at least the duration of the Throughput measurement.

4.	Measure the average throughput of the NR carrier for a duration sufficient to achieve statistical significance according to Annex H.2 of TS 38.521-1 [8] for NR band.

7.3B.2.3_1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band.

Message contents exceptions for E-UTRA band are according to TS 36.521-1 [10] clause 7.3.4.3 for each network signalling value. Message contents exceptions for NR band are according to TS 38.521-1 [8] clause 7.3.2.4.3 for each network signalling value.

7.3B.2.3_1.2.5	Test Requirement

Reference sensitivity test requirements for EN-DC configurations are specified in Table 7.3.5-1 in TS 36.521-1 [10] for the LTE CC, and Table 7.3.2.5-1 in TS 38.521-1 [8] for the NR CC.

Table 7.3B.2.3_1.2.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (two bands)

FFS

Table 7.3B.2.3_1.2.5-2: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

FFS

##### 7.3B.2.3_1.3	Reference sensitivity for EN-DC within FR1 (5 CCs)

7.3B.2.3_1.3.1	Test purpose

Same as in clause 7.3B.2.3.1.

7.3B.2.3_1.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting 5 CCs inter-band EN-DC.

7.3B.2.3_1.3.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.3B.2.0

Exception requirements for both NR and E-UTRA are defined for this test and therefore LTE anchor agnostic approach is not applied. E-UTRA test point analysis is included and E-UTRA measurements are performed.

7.3B.2.3_1.3.4	Test description

7.3B.2.3_1.3.4.1	Initial conditions

Same initial conditions as in clause 7.3B.2.3.4.1 with following exceptions:

The initial test configurations for E-UTRA band and NR band consist of environmental conditions, test frequencies, and channel bandwidths and RB allocations for exceptional test scenarios are specified in Table 7.3B.2.3_1.3.4.1-1 and Table 7.3B.2.3_1.3.4.1-2

Table 7.3B.2.3_1.3.4.1-1: Test Configuration Table Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (four bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  |  | For test frequencies refer to "Range" columns. For mapping within Band refer to "CC" columns |  |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  |  | Refer to "NRB" column |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Refer to "SCS" column |  |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |
| Test Parameters for EN-DC Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CC (NOTE1) | Band | SCS | NRB | Range/ Wgap |  | UL MOD | DL MOD | UL Alloc | DL Alloc |
| Default Test Settings for a DC_XA-YA_ZC_nRA Configuration |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X | N/A | Highest NRB | Mid |  | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(M) | Y | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(M) | Z | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | SCC3(M) | Z | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | R | 15 kHz | Highest NRB | Mid |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| NOTE 1: (M) and (S) indicate MCG and SCG respectively. NOTE 2: X, Y and Z in this table correspond to different bands i.e. X != Y != Z NOTE 3: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. DC_YA-(n)XAA represents the set of DC_YA-(n)XAA and DC_(n)XAA-YA |  |  |  |  |  |  |  |  |  |  |

Table 7.3B.2.3_1.3.4.1-2: Test Configuration Table Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (five bands)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] clause 4.1 |  |  |  |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [6] clause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  |  | For test frequencies refer to "Range" columns. For mapping within Band refer to "CC" columns |  |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  |  | Refer to "NRB " column |  |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1 [8] |  |  |  |  |  | Refer to "SCS" column |  |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default, exceptions listed in Table 7.3.3-3, dependent on PCC Band |  |  |  |  |
| Test Parameters for EN-DC Configurations |  |  |  |  |  |  |  |  |  |  |
| ID | CC (NOTE1) | Band | SCS | NRB | Range/ Wgap |  | UL MOD | DL MOD | UL Alloc | DL Alloc |
| Default Test Settings for a DC_XA-YA-ZA-SA_nRA Configuration |  |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X | N/A | Highest NRB | Mid |  | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(M) | Y | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | SCC2(M) | Z | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | SCC3(M) | S | N/A | Highest NRB | Mid |  | N/A | QPSK | N/A | All RBs |
|  | PCC(S) | R | 15 kHz | Highest NRB | Mid |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| NOTE 1: (M) and (S) indicate MCG and SCG respectively. NOTE 2: X, Y and Z in this table correspond to different E-UTRA bands i.e. X != Y != Z, R corresponds to NR band. NOTE 3: For inter-band EN-DC 4CCs configuration with four bands, if there is no additional exceptional test point is defined besides the requirement for default uplink EN-DC configurations defined in Table 5.5B.4.3-1, testing for 4CC with four band can be skipped. |  |  |  |  |  |  |  |  |  |  |

7.3B.2.3_1.3.4.2	Test procedure

Same as in clause 7.3B.2.3.4.2.

7.3B.2.3_1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.

7.3B.2.3_1.3.5	Test requirement

For inter-band EN-DC configurations, the throughput of each CG shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2 with reference receive power level specified in Tables Table 7.3.2.5-1 in TS 38.521-1 [8] and parameters specified Tables 7.3.2.4.1-1, Tables 7.3.2.4.1-2 and Tables 7.3.2.4.1-3 in TS 38.521-1 [8] for NR band.

Each EN-DC combination defined in Table 5.5B.4.3-1 shall be tested in anchor-agnostic mode as described in this clause. If a test point is overlapped with uplink EN-DC combo MSD test points, the requirement with MSD shall apply.

For the UE which supports inter-band EN-DC, the minimum requirement for reference sensitivity in Table 7.3.2.5-1 of TS 38.521-1 [8] for NR band and Table 7.3.5-1 of TS 36.521-1 [10] for EUTRA band, shall be increased by the amount given in ΔRIB,c defined in clause 7.3B.3.3 for the applicable for two, three, four and five bands operation.

Reference sensitivity exceptions for intermodulation interference due to dual uplink operation for EN-DC in NR FR1, are specified in Table 7.3B.2.3_1.3.5-1 for four bands and Table 7.3B.2.3_1.3.5-2 for five bands with uplink configuration specified in Table 7.3B.2.3_1.3.4.1-1 for four bands and 7.3B.2.3_1.3.4.1-2 for five bands, respectively.

For a given inter-band EN-DC 5CCs configuration defined in Table 5.5B.4.3-1 and Table 5.5B.4.4-1, if there is no additional exceptional test point is defined besides the requirement for default uplink EN-DC configurations defined in Table 5.5B.4.3-1 and Table 5.5B.4.4-1, the test requirement for default fallback uplink EN-DC configurations defined in Table 5.5B.4.3-1 applies.

Table 7.3B.2.3_1.3.5-1 Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (four bands)

TBD

Table 7.3B.2.3_1.3.5-2 Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (five bands)

TBD

##### 7.3B.2.3_1.4	Reference sensitivity for EN-DC within FR1 (6 CCs)

TBD

#### 7.3B.2.4	Reference sensitivity for Inter-band EN-DC including FR2 (1 NR CC)

Editor's note:	The following aspects are either missing or not yet determined:

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

7.3B.2.4.1	Test purpose

Same test purpose as in clause 7.3.2.1 in TS 38.521-2 [9] for the NR carrier.

7.3B.2.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting inter-band EN-DC including FR2 with 1 NR DL CC.

7.3B.2.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3B.2.0.4 TS 3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

The normative reference for this requirement is TS 38.101-3 [4] clause 7.3B.2.4.

7.3B.2.4.4	Test description

7.3B.2.4.4.1	Initial conditions

Same test description as in clause 7.3.2.4 in TS 38.521-2 [9] for the NR carrier with the following exception:

The initial test configurations for E-UTRA band consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 4.6-1. For Initial conditions as in clause 7.3.2.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are set according to Table 4.7-1 and propagation conditions are set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.3.2.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

Same test procedure as in clause 7.3.2.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

7.3B.2.4.5	Test requirement

Same test requirement as in clause 7.3.2.5 in TS 38.521-2 [9] for the NR carrier.

#### 7.3B.2.4_1	Reference sensitivity for Inter-band EN-DC including FR2 (>1 NR CC)

##### 7.3B.2.4_1.1	Reference sensitivity for Inter-band EN-DC including FR2 (2 NR CCs)

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz and for intra-band non-contiguous CA are TBD.

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

- In case of frequency separation larger than 800 MHz and in case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

- Testing of extreme conditions for FR2 is FFS.

7.3B.2.4_1.1.1	Test purpose

Same test purpose as in clause 7.3B.2.4.1.

7.3B.2.4_1.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC including FR2 with 2 NR DL CCs in either intra-band contiguous or intra-band non-contiguous configuration.

7.3B.2.4_1.1.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3B.1.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.3B.2.4_1.1.4	Test description

For inter-band EN-DC including FR2 UE configured as 2 NR DL CCs and 1 LTE DL CC, the test description of 2DL FR2 CA for reference sensitivity is the same as in corresponding clause of clause 7.3A.2.1.4 in TS 38.521-2 [9] with the exceptions described below.

7.3B.2.4_1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies and channel bandwidths based on EN-DC operating bands specified in clause 5.5B.5.1, channel bandwidths and sub-carrier spacings for the NR cell specified in TS 38.521-2 [9] clause 5.3A and channel bandwidth for the E-UTRA cell are specified in TS 36.521-1 [10] clause 5.4.2 or clause 5.4.2A. All valid configurations shall be tested with applicable test parameters for inter-band EN-DC including FR2 configuration specified in clause 5.5B.5, and the configuration for NR carrier are shown in TS 38.521-2 [9] Table 7.3A.2.1.4.1-1.

For Initial conditions as in clause 7.3A.2.1.4.1 in TS 38.521-2 [9], the following steps will be added to configure E-UTRA component:

2.1.	The parameter settings for E-UTRA cell are set up according to TS 36.508 [11] clause 4.4.3.

3.1.	The E-UTRA downlink signal level, uplink signal level are configured as per clause 4.7 with parameters set according to Table 4.7-1 and propagation conditions set according to Annex B.0 of TS 36.521-1 [10].

Step 6 of Initial conditions as in clause 7.3A.2.1.4.1 in TS 38.521-2 [9] is replaced by:

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [6] clause 4.5.

7.3B.2.4_1.1.4.2	Test Procedure

Same test procedure as in clause 7.3A.2.1.4.2 in TS 38.521-2 [9] with the following steps added for E-UTRA component:

1.1	On the E-UTRA carrier, disable periodic and aperiodic CQI reports, disable SRS, set TimeAlignmentTimerDedicated IE to infinity and disable downlink and uplink scheduling, all as per Table 4.7-1 under clause 4.7.

7.3B.2.4_1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1.

7.3B.2.4_1.1.5	Test requirement

For each NR component carrier, the test requirement is the same as in clause 7.3A.2.1.5 in TS 38.521-2 [9].

##### 7.3B.2.4_1.2	Reference sensitivity for Inter-band EN-DC including FR2 (3 NR CCs)

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz and for intra-band non-contiguous CA are TBD.

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

- In case of frequency separation larger than 800 MHz and in case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

- Testing of extreme conditions for FR2 is FFS.

7.3B.2.4_1.2.1	Test purpose

Same test purpose as in clause 7.3B.2.4_1.1.1.

7.3B.2.4_1.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC including FR2 with 3 NR DL CCs in either intra-band contiguous or intra-band non-contiguous configuration.

7.3B.2.4_1.2.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3B.2.4_1.1.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.3B.2.4_1.2.4	Test description

For inter-band EN-DC including FR2 UE configured as 3 NR DL CCs and 1 LTE DL CC, the test description of 3DL FR2 CA for reference sensitivity is the same as in corresponding clause of clause 7.3A.2.2.4 in TS 38.521-2 [9], with the exceptions described in clause 7.3B.2.4_1.1.4.1 and clause 7.3B.2.4_1.1.4.2.

7.3B.2.4_1.2.5	Test requirement

For each NR component carrier, the test requirement is the same as in clause 7.3A.2.1.5 in TS 38.521-2 [9].

##### 7.3B.2.4_1.3	Reference sensitivity for Inter-band EN-DC including FR2 (4 NR CCs)

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances for intra-band contiguous CA supporting aggregated BW > 400MHz and for intra-band non-contiguous CA are TBD.

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

- In case of frequency separation larger than 800 MHz and in case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

- Testing of extreme conditions for FR2 is FFS.

7.3B.2.4_1.3.1	Test purpose

Same test purpose as in clause 7.3B.2.4_1.1.1.

7.3B.2.4_1.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC including FR2 with 4 NR DL CCs in either intra-band contiguous or intra-band non-contiguous configuration.

7.3B.2.4_1.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3B.2.4_1.1.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.3B.2.4_1.3.4	Test description

For inter-band EN-DC including FR2 UE configured as 4 NR DL CCs and 1 LTE DL CC, the test description of 4DL FR2 CA for reference sensitivity is the same as in corresponding clause of clause 7.3A.2.3.4 in TS 38.521-2 [9], with the exceptions described in clause 7.3B.2.4_1.1.4.1 and clause 7.3B.2.4_1.1.4.2.

7.3B.2.4_1.3.5	Test requirement

For each NR component carrier, the test requirement is the same as in clause 7.3A.2.1.5 in TS 38.521-2 [9].

##### 7.3B.2.4_1.4	Reference sensitivity for Inter-band EN-DC including FR2 (5 NR CCs)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS.

- In case of frequency separation larger than 800 MHz and in case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

- Testing of extreme conditions for FR2 is FFS.

7.3B.2.4_1.4.1	Test purpose

Same test purpose as in clause 7.3B.2.4_1.1.1.

7.3B.2.4_1.4.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC including FR2 with 5 NR DL CCs in either intra-band contiguous or intra-band non-contiguous configuration.

7.3B.2.4_1.4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3B.2.4_1.1.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.3B.2.4_1.4.4	Test description

For inter-band EN-DC including FR2 configured as 5 NR DL CCs and 1LTE DL CC, the test description of 5DL FR2 CA for reference sensitivity is the same as in corresponding clause of clause 7.3A.2.4.4 in TS 38.521-2 [9], with the exceptions described in clause 7.3B.2.4_1.1.4.1 and clause 7.3B.2.4_1.1.4.2.

7.3B.2.4_1.4.5	Test requirement

For each NR component carrier, the test requirement is the same as in clause 7.3A.2.1.5 in TS 38.521-2 [9].

##### 7.3B.2.4_1.5	Reference sensitivity for Inter-band EN-DC including FR2 (6 NR CCs)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS.

- In case of frequency separation larger than 800 MHz and in case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

- Testing of extreme conditions for FR2 is FFS.

7.3B.2.4_1.5.1	Test purpose

Same test purpose as in clause 7.3B.2.4_1.1.1.

7.3B.2.4_1.5.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC including FR2 with 6 NR DL CCs in either intra-band contiguous or intra-band non-contiguous configuration.

7.3B.2.4_1.5.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3B.2.4_1.1.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.3B.2.4_1.5.4	Test description

For inter-band EN-DC including FR2 configured as 6 NR DL CCs and 1LTE DL CC, the test description of 6DL FR2 CA for reference sensitivity is the same as in corresponding clause of clause 7.3A.2.5.4 in TS 38.521-2 [9], with the exceptions described in clause 7.3B.2.4_1.1.4.1 and clause 7.3B.2.4_1.1.4.2.

7.3B.2.4_1.5.5	Test requirement

For each NR component carrier, the test requirement is the same as in clause 7.3A.2.1.5 in TS 38.521-2 [9].

##### 7.3B.2.4_1.6	Reference sensitivity for Inter-band EN-DC including FR2 (7 NR CCs)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS.

- In case of frequency separation larger than 800 MHz and in case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

- Testing of extreme conditions for FR2 is FFS.

7.3B.2.4_1.6.1	Test purpose

Same test purpose as in clause 7.3B.2.4_1.1.1.

7.3B.2.4_1.6.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC including FR2 with 7 NR DL CCs in either intra-band contiguous or intra-band non-contiguous configuration.

7.3B.2.4_1.6.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3B.2.4_1.1.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.3B.2.4_1.6.4	Test description

For inter-band EN-DC including FR2 configured as 7 NR DL CCs and 1LTE DL CC, the test description of 7DL FR2 CA for reference sensitivity is the same as in corresponding clause of clause 7.3A.2.6.4 in TS 38.521-2 [9], with the exceptions described in clause 7.3B.2.4_1.1.4.1 and clause 7.3B.2.4_1.1.4.2.

7.3B.2.4_1.6.5	Test requirement

For each NR component carrier, the test requirement is the same as in clause 7.3A.2.1.5 in TS 38.521-2 [9].

##### 7.3B.2.4_1.7	Reference sensitivity for Inter-band EN-DC including FR2 (8 NR CCs)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS.

- In case of frequency separation larger than 800 MHz and in case the device manufacturer does not explicitly declare that the beam peak for a reference (frequency band, CBW) or (frequency band combination, CA BW class) is applicable for a group of other intra-band contiguous combinations and CA BW classes, according to Table A.4.3.9-6 in 38.508-2, following aspect of beam peak search procedures for CA is FFS: RB allocation, power level, channel bandwidth configuration, per CC approach or all CC combined approach, etc

- Testing of extreme conditions for FR2 is FFS.

7.3B.2.4_1.7.1	Test purpose

Same test purpose as in clause 7.3B.2.4_1.1.1.

7.3B.2.4_1.7.2	Test applicability

This test case applies to all types of E-UTRA UE release 15 and forward, supporting EN-DC including FR2 with 8 NR DL CCs in intra-band contiguous configuration.

7.3B.2.4_1.7.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 7.3B.2.4_1.1.3.

No exception requirements applicable to NR or E-UTRA. LTE anchor agnostic approach is applied.

7.3B.2.4_1.7.4	Test description

For inter-band EN-DC including FR2 configured as 8 NR DL CCs and 1LTE DL CC, the test description of 8DL FR2 CA for reference sensitivity is the same as in corresponding clause of clause 7.3A.2.7.4 in TS 38.521-2 [9], with the exceptions described in clause 7.3B.2.4_1.1.4.1 and clause 7.3B.2.4_1.1.4.2.

7.3B.2.4_1.7.5	Test requirement

For each NR component carrier, the test requirement is the same as in clause 7.3A.2.1.5 in TS 38.521-2 [9].

### 7.3B.2.4D	Reference sensitivity for inter-band EN-DC including FR2 for UL MIMO

No exception requirements applicable to NR or LTE. LTE anchor agnostic approach can be applied and only NR carriers need to be tested.

No test case details are specified. Given UE’s Rx performance would not be impacted by the Tx configuration in NR FR2 TDD bands, the requirements in this test case can be well covered in 7.3B.2.4 and 7.3B.2.4_1 and don’t need to be tested again.

#### 7.3B.2.5	Reference sensitivity for Inter-band EN-DC including FR1 and FR2 (3 CCs)

7.3B.2.5.1	Test purpose

Same test purpose as in 7.3B.2 in TS 38.521-1 [8] for NR FR1 carrier(s) and 7.3.2 in TS 38.521-2 [9] for NR FR2 carrier(s).

7.3B.2.5.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 and E-UTRA in conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The EN-DC requirements for reference sensitivity apply and are tested as part of the EN-DC within FR1 and EN-DC including FR2 test cases in clause 7.3B.

#### 7.3B.2.5D	Reference sensitivity for inter-band EN-DC including FR1 and FR2 for UL MIMO

7.3B.2.5D.1	Test purpose

Same test purpose as in clause 7.3D in TS 38.521-1 [8] for NR FR1 carrier and 7.3D in TS 38.521-2 [9] for NR FR2 carrier.

7.3B.2.5D.2	Test applicability

The requirements in this test are not testable due to issues with combined testing of NR FR1 and E-UTRA in conducted mode with NR FR2 in radiated mode. Therefore, the conducted and radiated requirements are tested separately.

No test case details are specified. The EN-DC requirements for reference sensitivity apply and are tested as part of the EN-DC within FR1 as in clause 7.3 in TS 38.521-1 [8] and EN-DC within FR2 as in clause 7.3 in TS 38.521-2 [9].

#### 7.3B.2.6	Void

### 7.3B.3	ΔRIB,c ΔRIBNC for EN-DC

#### 7.3B.3.0	General

<< Unchanged sections skipped>>

<< End of changes >>