3GPP TSG-RAN5 Meeting #97	R5-227528

# Toulouse, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3319 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to NR TC 7.1.2.3.5 to test RedCap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - RedCap UE use 12bit SN for PDCP, so the PDCP header is 2 bytes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update Note 4 for RedCap UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the TC or a non-conformant UE may pass the TC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-226946r1. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.1.2.3.5	AM RLC / 12-bit SN / Control of transmit window / Control of receive window

7.1.2.3.5.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state with AM RLC 12 bit SN and pending uplink data for transmission }

ensure that {

when { AMD PDUs in transmission buffer fall outside TX_Next_Ack <= SN < TX_Next_Ack + AM_Window_Size }

then { UE does not transmit these AMD PDUs }

}

(2)

with { UE in RRC_CONNECTED state with AM RLC 12 bit SN and pending uplink data for transmission }

ensure that {

when { receiving a STATUS PDU where ACK_SN acknowledges at least one AMD PDU not yet acknowledged }

then { UE transmits AMD PDUs within updated window range }

}

(3)

with { UE in RRC_CONNECTED state with AM RLC 12 bit SN }

ensure that {

when { the UE receives AMD PDUs with SN outside the upper boundary of the receive window }

then { the UE discards these AMD PDUs }

}

(4)

with { UE in RRC_CONNECTED state with AM RLC 12 bit SN }

ensure that {

when { the receive window has been moved }

then { UE continues accepting AMD PDUs within updated window range }

}

7.1.2.3.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.322, clauses 5.2.3.2.1, 5.2.3.2.2, 5.2.3.2.3 and 7.2. Unless otherwise stated these are Rel-15 requirements.

[TS 38.322, clause 5.2.3.2.2]

When an AMD PDU is received from lower layer, where the AMD PDU contains byte segment numbers y to z of an RLC SDU with SN = x, the receiving side of an AM RLC entity shall:

-	if x falls outside of the receiving window; or

-	if byte segment numbers y to z of the RLC SDU with SN = x have been received before:

-	discard the received AMD PDU.

-	else:

-	place the received AMD PDU in the reception buffer;

-	if some byte segments of the RLC SDU contained in the AMD PDU have been received before:

-	discard the duplicate byte segments.

[TS 38.322, clause 5.2.3.2.3]

When an AMD PDU with SN = x is placed in the reception buffer, the receiving side of an AM RLC entity shall:

-	if x >= RX_Next_Highest

-	update RX_Next_Highest to x+ 1.

-	if all bytes of the RLC SDU with SN = x are received:

-	reassemble the RLC SDU from AMD PDU(s) with SN = x, remove RLC headers when doing so and deliver the reassembled RLC SDU to upper layer;

-	if x = RX_Highest_Status,

-	update RX_Highest_Status to the SN of the first RLC SDU with SN > current RX_Highest_Status for which not all bytes have been received.

-	if x = RX_Next:

-	update RX_Next to the SN of the first RLC SDU with SN > current RX_Next for which not all bytes have been received.

-	if t-Reassembly is running:

-	if RX_Next_Status_Trigger = RX_Next; or

-	if RX_Next_Status_Trigger = RX_Next + 1 and there is no missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU; or

-	if RX_Next_Status_Trigger falls outside of the receiving window and RX_Next_Status_Trigger is not equal to RX_Next + AM_Window_Size:

-	stop and reset t-Reassembly.

-	if t-Reassembly is not running (includes the case t-Reassembly is stopped due to actions above):

-	if RX_Next_Highest> RX_Next +1; or

-	if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU:

-	start t-Reassembly;

-	set RX_Next_Status_Trigger to RX_Next_Highest.

[TS 38.322, clause 7.2]

a) AM_Window_Size

This constant is used by both the transmitting side and the receiving side of each AM RLC entity. AM_Window_Size = 2048 when a 12 bit SN is used, AM_Window_Size = 131072 when an 18 bit SN is used.

7.1.2.3.5.3	Test description

7.1.2.3.5.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.2.1.1 with the exception that the AM DRB is configured according to Table 7.1.2.3.5.3.1-1.

Table 7.1.2.3.5.3.1-1: RLC parameters

| t-PollRetransmit | ms300 |
| --- | --- |
| pollPDU | infinity |
| pollByte | infinity |
| sn-FieldLength(UL-AM-RLC) | size12 |
| sn-FieldLength(DL-AM-RLC) | size12 |

7.1.2.3.5.3.2	Test procedure sequence

Table 7.1.2.3.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS does not allocate any uplink grant. | - | - | - | - |
| - | EXCEPTION: The SS is configured for step 1 500 ms in advance. The transmissions are performed every second radio frame. Step 2 is started 100 ms after the first DL AMD PDU has been transmitted in step 1. | - | - | - | - |
| - | EXCEPTION: Step 1 a1 shall be repeated from j=0 to j= FLOOR((Maximum_RLC_SN/iteration size) -1, and the last repetition shall execute Step 1b1. (Note 1) (Note 3) (Note 4) (Note 6) | - | - | - | - |
| 1a1 | The SS transmits several RLC PDUs in a RLC PDU List, the number of RLC PDUs sent is defined by the iteration_size. Each RLC Data PDU contains one RLC SDU. (Note 8) (Note 9) (Note 5) | <-- | RLC Data PDU (SN = j*iteration_ size, SN=(((j+1)*iteration_size)-1) | - | - |
| 1b1 | The SS transmits several RLC PDUs in a RLC PDU List, the number of RLC PDUs sent is defined by the iteration_size+1. Each RLC Data PDU contains one RLC SDU. The SS transmits AMD PDU(SN=W+1) as last packet (Note 8) (Note 9) (Note 5) | <-- | RLC Data PDU (SN = j*iteration_ size, SN=(((j+1)*iteration_size)) | - | - |
| 1A | Void | - | - | - | - |
| 2 | In the following steps the SS transmits 1 UL grant in every second radio frame to enable the UE to return each received AMD PDU in one looped back AMD PDU. (Note 2) | <-- | (UL grants) | - | - |
| - | EXCEPTION: Step 2A shall be repeated from j=0 to j=FLOOR((Maximum_RLC_SN/iteration size). (Note 1) (Note 3) (Note 4) (Note 6) | - | - | - | - |
| - | EXCEPTION: In Step 2A, SS shall receive a RLC PDU and step 2A is repeated from SN=j*iteration_size to SN=(((j+1)*iteration_ size)-1). (Note 1) (Note 3) (Note 4) (Note 6) (Note 8) (Note 9) | - | - | - | - |
| 2A | Check: Does UE transmit a RLC Data PDU with the Poll bit not set and with SN=0 for the first RLC Data PDU and then incremented by 1 at each RLC Data PDU? (Note 7)(Note 10) | --> | RLC Data PDU (SN = j*iteration_ size, SN=(((j+1)*iteration_size)-1) | 1 | P |
| 3 | Check: Does the UE transmit the (W)st AMD PDU with the Poll bit set and with the contents of the SDU? | --> | AMD PDU(SN=W-1), Poll | 1 | P |
| 4 | The SS starts the UL default grant transmission. | - | - | - | - |
| 5 | Check: Does the UE transmit an AMD PDU within t-PollRetransmit/2? | --> | AMD PDU | 1 | F |
| 6 | The SS transmits a STATUS PDU to acknowledge the W uplink AMD PDUs with SN=0 to SN=W-1. ACK_SN = W. | <-- | STATUS PDU | - | - |
| 7 | Check: Does the UE transmit an AMD PDU with the Poll bit set and with the contents of the SDU? | --> | AMD PDU(SN=W), Poll | 2 | P |
| 8 | The SS transmits a STATUS PDU with ACK_SN = W+1. | <-- | STATUS PDU | - | - |
| 9 | The SS transmits the (W+2)nd AMD PDU containing a SDU to the UE with the Sequence Number field set to ((2W+1 mod AM_Modulus) = 1) and the Polling bit set. (Note 3) (Note 5) | <-- | AMD PDU | - | - |
| 10 | Check: Does the UE transmit a STATUS PDU acknowledging W+1 SDUs? (ACK_SN = W+1). (Note 1) | --> | STATUS PDU | 3 | P |
| 11 | The SS transmits the (W+2)nd AMD PDU to the UE with the Sequence Number field set to W+1 and the Polling bit set. (Note 5) | <-- | AMD PDU | - | - |
| - | EXCEPTION: Steps 12 and 13 can happen in any order | - | - | - | - |
| 12 | Check: Does the UE transmit a STATUS PDU acknowledging W+1 PDUs? (ACK_SN field = W+2). (Note 11) | --> | STATUS PDU | 4 | P |
| 13 | Check: Does the UE transmit an AMD PDU with the same data as received in the corresponding DL AMD PDU in step 11? (Note 11) | --> | AMD PDU | 4 | P |
| 14 | The SS transmits a STATUS PDU with ACK_SN = W+2. | <-- | STATUS PDU | - | - |
| Note 1: PDUs are numbered 1,2, …, W+2. Note 2: 20 ms gap between transmissions both in DL and UL respectively allows TTCN to tolerate one HARQ retransmission (FDD/TDD) per transport block. Note 3: AM_Modulus is 4096 resp 262144 for SN size is size12 or size18. Note 4: The RLC SDU size shall be 4 octets(3 octets of PDCP header + 1 octet PDCP SDU) for pc_supportOfRedCap_r17=false or 4 octets(2 octets of PDCP header + 2 octets PDCP SDU) for pc_supportOfRedCap_r17=true. If SN size is size18 is used the RLC SDU size shall be 7 octets. With 2 octets of BSR or padding, 2 octets of MAC header and 3 octets of RLC header (without SO) the RLC PDU consists of 56 bits and a TBS of 112 bits shall be allocated Note 5: PDCP SN=W+1 Note 6: Maximum_RLC_SN = W-1. Note 7: The verdict shall be provided each time (SN+1) mod 256 = 0 resp. (SN+1) mod 4096 = 0, if SN size is size12 or size18. Note 8: Iteration will be 211 in case of len12bits. Small RLC SDU size will be used and no repetition will be needed. Note 9: Iteration will be incremented by iteration_size of 211 for SN len18bits. Small RLC SDU size will be used and it shall be repeated FLOOR(Maximum_RLC_SN/iteration_size). Note 10: -2 for the last iteration, as the last reception will be handled by step 3. Note 11: STATUS PDU at step 12 and AMD PDU at step 13 may be received by SS in the same slot or in multiple slots |  |  |  |  |  |

7.1.2.3.5.3.3	Specific message contents

None

<End of modified section 1>