3GPP TSG-RAN5 Meeting #97	R5-227908

Toulouse, France, 14th November – 18th November 2022

Title: 	Discussion on the need to add new test cases under mpr-Power Boost conditions

Source:	        Keysight Technologies UK Ltd

Agenda Item:	5.3.13.8

Document for:	Discussion and Endorsement

# 1.	Introduction

In RAN5#96-e, new test case 6.2.4_1 Configured transmitted power with power boost was added into TS 38.521-2 ([1]) via discussion paper in [2] and CR in [3].

As part of the above discussion, a concern was raised on whether other test cases as EVM or emissions test cases should be checked under power boost conditions. Hence AP#96e.28 was raised:

| Action ID | sWG | Action | Responsible | Relevant Tdoc | Deadline | Status |
| --- | --- | --- | --- | --- | --- | --- |
| AP#96e.28 | RF | Study which additional signal quality and RF emission requirements need to be verified with mpr-PowerBoost-FR2-r16 configured | Ericsson, Qualcomm, KEYS, Apple | R5-224046 | RAN5#97 | Open |

This document makes proposals to enable AP#96.28 closing.

# 2.	Discussion

According to core requirements, a UE will transmit PIBE higher under mpr-Power Boost conditions:

The configured UE maximum output power PCMAX,f,c for carrier f of a serving cell c shall be set such that the corresponding measured peak EIRP PUMAX,f,c is within the following bounds PPowerclass + DPIBE – MAX(MAX(MPRf,c, A- MPRf,c,) + ΔMBP,n, P-MPRf,c) – MAX{T(MAX(MPRf,c, A- MPRf,c,)), T(P-MPRf,c)} ≤ PUMAX,f,c ≤ EIRPmax while the corresponding measured total radiated power PTMAX,f,c is bounded by PTMAX,f,c ≤ TRPmax with PPowerclass the UE minimum peak EIRP as specified in sub-clause 6.2.1.1.3, EIRPmax the applicable maximum EIRP as specified in sub-clause 6.2.1.1.3, MPRf,c as specified in sub-clause 6.2.2.3, A-MPRf,c as specified in sub-clause 6.2.3.3, ΔMBP,n the peak EIRP relaxation as specified in section 6.2.1.1.3 and TRPmax the maximum TRP for the UE power class as specified in sub-clause 6.2.1.1.3. DPIBE is 1.0 dB if UE declares support for mpr-PowerBoost-FR2-r16, UL transmission is QPSK, MPRf,c = 0 and when NS_200 applies and the network configures the UE to operate with mpr-PowerBoost-FR2-r16, otherwise DPIBE is 0.0 dB. The requirement is verified in beam peak direction.

It is well known that EVM and emissions core requirements are dependent on UE measured peak EIRP PUMAX,f,c.

RAN4 has not updated EVM and emissions requirements under mpr-PowerBoost conditions, which means that existing core requirements apply.

Observation 1: RAN4 has not updated EVM and emissions requirements under mpr-PowerBoost conditions, which means that existing core requirements apply.

Proposal 1: It is proposed to add the following test cases to the UE Conformance Test Aspects - NR RF requirement enhancements for frequency range 2 (FR2) Work Plan [4]:

- 6.4.2.1_1 Error vector magnitude with Power Boost

- 6.5.2.1_1 Spectrum Emission Mask with Power Boost​

- 6.5.3.1_1 Transmitter Spurious emissions with Power Boost

- 6.5.3.2_1 Spurious emission band UE co-existence with Power Boost​

- 6.5.3.3_1 Additional spurious emissions with Power Boost

Skipping those measurements with relative power requirements (ACLR, carrier leakage) could be a good trade-off (although they could be added if requested).​

In order to exercise mpr-PowerBoost conditions, test points analysis shall be the same as those defined for 6.2.4_1 Configured transmitted power with Power Boost in terms of modulation, waveform and RB allocation while keeping other parameters in test point analysis as per corresponding Rel-15 single CC corresponding test cases [5].​

Proposal 2: Define test point analysis for 6.4.2.1_1 equal to those for 6.2.4_1.

Proposal 2a: Define test point analysis for 6.5.2.1_1, 6.5.3.1_1, 6.5.3.2_1 and 6.5.3.3_1 equal to those for 6.2.4_1 in terms of modulation, waveform and RB allocation while keeping other parameters in test point analysis as per corresponding Rel-15 single CC corresponding test cases.

As in the case for of test case 6.2.4_1, since new Rel-16 signalling and new Rel-16 UE capabilities are required to exercise Power Boost requirements, Power Boost associated testing should be applicable from Rel-16 and onwards for devices supporting mpr-PowerBoost-FR2-r16 UE capability.

Proposal 3: Test case applicability for 6.4.2.1_1, 6.5.2.1_1, 6.5.3.1_1, 6.5.3.2_1 and 6.5.3.3_1 will be for all types of NR UE release 16 and forward supporting mpr-PowerBoost-FR2-r16 UE capability.​

Regarding test procedure, Rel-15 single carrier equivalent test procedures could be leveraged except for the fact that mpr-PowerBoost signalling as described in message contents in 6.2.4_1.4.3 is required.

Proposal 4: Leverage Rel-15 single carrier equivalent test procedures with message contents as per 6.2.4_1.4.3 for 6.4.2.1_1, 6.5.2.1_1, 6.5.3.1_1, 6.5.3.2_1 and 6.5.3.3_1.

Corresponding DC test cases should be added into TS 38.521-3 [6]:

- 6.4B.2.4.1a Error Vector Magnitude for inter-band EN-DC including FR2 (1 NR CC) with Power Boost​

- 6.5B.2.4.1a Spectrum emissions mask for Inter-band EN-DC including FR2 (1 NR CC) with Power Boost​

- 6.5B.3.4.1a General Spurious Emissions for Inter-band including FR2 (1 NR CC) with Power Boost

- 6.5B.3.4.2a Spurious emission band UE co-existence for Inter-band including FR2 (1 NR CC) with Power Boost

- 6.5B.4.4 a Additional Spurious Emissions for Inter-band including FR2 (1 NR CC) with Power Boost

Proposal 5: It is proposed to add the following test cases to the UE Conformance Test Aspects - NR RF requirement enhancements for frequency range 2 (FR2) Work Plan [4]:

- 6.4B.2.4.1a Error Vector Magnitude with Power Boost for inter-band EN-DC including FR2 (1 NR CC)

- 6.5B.2.4.1a Spectrum emissions mask with Power Boost for Inter-band EN-DC including FR2 (1 NR CC)

- 6.5B.3.4.1a General Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC)​

- 6.5B.3.4.2a Spurious emission band UE co-existence with Power Boost for Inter-band including FR2 (1 NR CC)

- 6.5B.4.4a Additional Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC)

# 3. 	Conclusion

This document makes proposals to enable AP#96.28 closing on regarding the need to add new test cases under mpr-Power Boost conditions.

The following proposals and observations are made:

Observation 1: RAN4 has not updated EVM and emissions requirements under mpr-PowerBoost conditions, which means that existing core requirements apply.

Proposal 1: It is proposed to add the following test cases to the UE Conformance Test Aspects - NR RF requirement enhancements for frequency range 2 (FR2) Work Plan [4]:

- 6.4.2.1_1 Error vector magnitude with Power Boost

- 6.5.2.1_1 Spectrum Emission Mask with Power Boost​

- 6.5.3.1_1 Transmitter Spurious emissions with Power Boost

- 6.5.3.2_1 Spurious emission band UE co-existence with Power Boost​

- 6.5.3.3_1 Additional spurious emissions with Power Boost

Proposal 2: Define test point analysis for 6.4.2.1_1 equal to those for 6.2.4_1.

Proposal 2a: Define test point analysis for 6.5.2.1_1, 6.5.3.1_1, 6.5.3.2_1 and 6.5.3.3_1 equal to those for 6.2.4_1 in terms of modulation, waveform and RB allocation while keeping other parameters in test point analysis as per corresponding Rel-15 single CC corresponding test cases.

Proposal 3: Test case applicability for 6.4.2.1_1, 6.5.2.1_1, 6.5.3.1_1, 6.5.3.2_1 and 6.5.3.3_1 will be for all types of NR UE release 16 and forward supporting mpr-PowerBoost-FR2-r16 UE capability.​

Proposal 4: Leverage Rel-15 single carrier equivalent test procedures with message contents as per 6.2.4_1.4.3 for 6.4.2.1_1, 6.5.2.1_1, 6.5.3.1_1, 6.5.3.2_1 and 6.5.3.3_1.

Proposal 5: It is proposed to add the following test cases to the UE Conformance Test Aspects - NR RF requirement enhancements for frequency range 2 (FR2) Work Plan [4]:

- 6.4B.2.4.1a Error Vector Magnitude with Power Boost for inter-band EN-DC including FR2 (1 NR CC)

- 6.5B.2.4.1a Spectrum emissions mask with Power Boost for Inter-band EN-DC including FR2 (1 NR CC)

- 6.5B.3.4.1a General Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC)​

- 6.5B.3.4.2a Spurious emission band UE co-existence with Power Boost for Inter-band including FR2 (1 NR CC)

- 6.5B.4.4a Additional Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC)

These proposals are implemented in [7]-[19].

# 4.	References

- 3GPP TS 38.521-2 v16.12.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone

- R5-224046, “Discussion on Rel-16 Power Boost test definition”, Keysight Technologies UK Ltd, RAN5#96-e

- R5-225658, “New test case addition: 6.2.4_1 Configured transmitted power with Power Boost”, Keysight Technologies UK Ltd, RAN5#96-e

- R5-224144, “WP UE Conformance Test Aspects for NR RF Requirement Enhancements for FR2”, Nokia, Apple, RAN5#96

- R5-225745, “Test point analysis for test 6.2.4_1 Configured transmitted power with Power Boost”, Keysight Technologies UK Ltd, RAN5#96

- 3GPP TS 38.521-3 v17.6.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios

- R5-227911, “New test case addition: 6.4.2.1_1 Error vector magnitude with Power Boost”, Keysight Technologies UK Ltd, RAN5#97

- R5-227910, “New test case addition: 6.5.2.1_1 Spectrum Emission Mask with Power Boost​”, Keysight Technologies UK Ltd, RAN5#97

- R5-228037, “New test case addition: 6.5.3.1_1 Transmitter Spurious emissions with Power Boost”, Keysight Technologies UK Ltd, RAN5#97

- R5-228038, “New test case addition: 6.5.3.2_1 Spurious emission band UE co-existence with Power Boost”, Keysight Technologies UK Ltd, RAN5#97

- R5-228039, “New test case addition: 6.5.3.3_1 Additional spurious emissions with Power Boost”, Keysight Technologies UK Ltd, RAN5#97

- R5-227909, “Test point analysis for mpr-PowerBoost tests 6.4.2.1_1, 6.5.2.1_1, 6.5.3.1_1, 6.5.3.2_1 and 6.5.3.3_1”, Keysight Technologies UK Ltd, RAN5#97

- R5-227819, “Measurement uncertainties and test tolerances for mpr-PowerBoost tests 6.4.2.1_1, 6.5.2.1_1, 6.5.3.1_1, 6.5.3.2_1 and 6.5.3.3_1”, Keysight Technologies UK Ltd, RAN5#97

- R5-226886, “New test case addition: 6.4B.2.4.1a Error Vector Magnitude with Power Boost for inter-band EN-DC including FR2 (1 NR CC)”, Keysight Technologies UK Ltd, RAN5#97

- R5-226887, “New test case addition: 6.5B.2.4.1a Spectrum emissions mask with Power Boost for Inter-band EN-DC including FR2 (1 NR CC)​”, Keysight Technologies UK Ltd, RAN5#97

- R5-226888, “New test case addition: 6.5B.3.4.1a General Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC)”, Keysight Technologies UK Ltd, RAN5#97

- R5-226889, “New test case addition: 6.5B.3.4.2a Spurious emission band UE co-existence with Power Boost for Inter-band including FR2 (1 NR CC)”, Keysight Technologies UK Ltd, RAN5#97

- R5-226890, “New test case addition: 6.5B.4.4a Additional Spurious Emissions with Power Boost for Inter-band including FR2 (1 NR CC)”, Keysight Technologies UK Ltd, RAN5#97

- R5-226941, “Measurement uncertainties and test tolerances for mpr-PowerBoost tests 6.4B.2.4.1a, 6.5B.2.4.1a, 6.5B.3.4.1a, 6.5B.3.4.2a and 6.5B.4.4a”, Keysight Technologies UK Ltd, RAN5#97