3GPP TSG-RAN5 Meeting #98	R5-230056

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0622 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to periodic CQI reporting with Table 3 cases 6.2.2.1.1.2, 6.2.2.2.1.2, 6.2.3.1.1.2 and 6.2.3.2.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2022-2-2 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The test applicability of periodic CQI reporting with Table 3 cases only involved NR release16 UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the test applicability to involve more UE types. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Only NR release16 UE can be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.1.1.2, 6.2.2.2.1.2, 6.2.3.1.1.2, 6.2.3.2.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

6.2.2.1.1.2	2Rx FDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Whether to adopt minimum sample for pass/fail verdict needs further study.

6.2.2.1.1.2.1	Test Purpose

The purpose of this test is to verify the variance of the wideband CQI reports is within the limits defined and a PDSCH BLER of 10-5 falls between the transport format based median CQI-1 and median CQI or the transport format based median CQI and median CQI +1.

6.2.2.1.1.2.2	Test Applicability

This test applies to all types of NR UE release 16 and forward supporting cqi-TableAlt.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and cqi-TableAlt.

6.2.2.1.1.2.3	Minimum requirement for periodic CQI reporting with Table 3

For the parameters specified in Table 6.2.2.1.1.2.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 10-5, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 10-5. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 10-5, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 10-5.

c)	The reported CQI value according to the reference channel shall be ≥ 1.

Table 6.2.2.1.1.2.3-1: CQI reporting test parameters

| Parameter |  |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 |  |
| Duplex Mode |  |  |  | FDD |  |
| Subcarrier spacing |  |  | kHz | 15 |  |
| SNR |  |  | dB | 1 | 2 |
| Propagation channel |  |  |  | AWGN |  |
| Antenna configuration |  |  |  | 1×2 with static channel specified in Annex B.1 |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |
|  | CDM Type |  |  | FD-CDM2 |  |
|  | Density (ρ) |  |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |
|  | CSI-RS periodicity and offset |  | slot | 5/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 1 |  |
|  | CDM Type |  |  | No CDM |  |
|  | Density (ρ) |  |  | 3 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 1,(0,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |
|  | CSI-IM RE pattern |  |  | 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 |  |
| ReportConfigType |  |  |  | Periodic |  |
| CQI-table |  |  |  | Table 3 |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |
| Sub-band Size |  |  | RB | 8 |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  |  | slot | 5/0 |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |
|  |  | Codebook Mode |  | 1 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |
|  |  | CodebookSubsetRestriction |  | 000001 |  |
|  |  | RI Restriction |  | N/A |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |
| CQI/RI/PMI delay |  |  | ms | 8 |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |
| Measurement channel |  |  |  | As specified in Table A.4-4, TBS.4-1 |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.2.2.1.1.2.

6.2.2.1.1.2.4	Test Description

6.2.2.1.1.2.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the NR cell are set up according to Table 6.1.2-1 and 6.2.2.1.1.2.3-1 as appropriate.

3.	Downlink signals for the NR cell are initially set up according to Annexes C.0, C.1, C.2, C.3.1 , and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions for the NR cell are set according to Annex B.1.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 6.2.2.1.1.2.4.3.

6.2.2.1.1.2.4.2	Test Procedure

1.	Set the parameters of bandwidth, SCS, reference Channel, the propagation condition, antenna configuration and the SNR according to Table 6.2.2.1.1.2.3-1.

2.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to CQI value 2 and keep it regardless of the wideband CQI value sent by the UE. The SS sends downlink MAC padding bits on the DL RMC. Continue transmission of the PDSCH until 5000 wideband CQI reports have been gathered. In this process the SS collects wideband CQI reports every 5 ms and also cases where UE transmits nothing in its CQI timing are also counted as wideband CQI reports.

3.	Set up a relative frequency distribution for the reported wideband CQI-values, Calculate the median value (wideband Median CQI is the wideband CQI that is at or crosses 50% distribution from the lower wideband CQI side). This CQI-value is declared as wideband Median CQI value.

4.	If Median CQI is not equal to 1 or 15 and 4500 or more of the wideband CQI values are in the range (Median CQI - 1) ≤ Median CQI ≤ ( Median CQI + 1) then continue with step 5, otherwise go to step 8.

5.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband median-CQI value and shall not react to the UE’s wideband CQI reports. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH transmitted by the SS, record the associated ACK, NACK and statDTX responses. The responses are then filtered as follows: for the sequence of responses for each HARQ process, discard all the statDTX responses. Measure the BLER for a duration sufficient to achieve statistical significance according to Annex G.4.

For the filtered ACK and NACK responses if the ratio (NACK / (ACK + NACK)) ≤ 10-5 then go to step 6, otherwise go to step 7.

6.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband median-CQI+1 value and shall not react to the UE’s wideband CQI reports. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH, transmitted by the SS, record and filter the ACK, NACK and statDTX responses as in step 5, and measure the BLER for a duration sufficient to achieve statistical significance according to Annex G.4.

If the ratio (NACK /(ACK + NACK)) > 10-5

then pass the UE for this test, otherwise go to step 8.

7.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband median-CQI-1 value and shall not react to the UE’s wideband CQI reports. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH, transmitted by the SS, record and filter the ACK, NACK and statDTX responses as in step 5, and measure the BLER for a duration sufficient to achieve statistical significance according to Annex G.4.

If the ratio (NACK /ACK + NACK) ≤10-5

then pass the UE for this test, otherwise go to step 8.

8.	If both SNR points of the test have not been tested, then repeat the same procedure (steps 1 to 7) for the other SNR point as appropriate. Otherwise fail the UE.

6.2.2.1.1.2.4.3	Message contents

Message contents are according to TS 38.508 [6] clause 5.4.2 with the following exceptions:

6.2.2.1.1.2.4.3_1	Message exceptions for SA

Table 6.2.2.1.1.2.4.3_1-1: NZP CSI-RS-ResourceMapping

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0000 | K0=0, row 1, | 1Tx test cases |
| } |  |  |  |
| nrofPorts | p1 |  |  |
| firstOFDMSymbolInTimeDomain | 13 |  |  |
| CDM Type | noCDM |  |  |
| density CHOICE { |  |  |  |
| three | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.2.1.1.2.4.4_1-3: CSI-IM-Resource

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.4-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-Resource ::= SEQUENCE { |  |  |  |
| periodicityAndOffset SEQUENCE { |  |  |  |
| slot5 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.2.1.1.2.4.3_1-2: NZP-CSI-RS-Resource

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| slot5 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.2.1.1.2.4.4_1-3: CSI-ReportConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.4-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| cqi-Table | table3 |  |  |
| } |  |  |  |

Table 6.2.2.1.1.2.4.4_1-4: CodebookConfig

| Derivation Path: TS38.508-1 [6], Table 5.4.2.4-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CodebookConfig ::= SEQUENCE { |  |  |  |
| codebookType CHOICE { |  |  |  |
| type1 SEQUENCE { |  |  |  |
| subType CHOICE { |  |  |  |
| typeI-SinglePanel SEQUENCE { |  |  |  |
| nrOfAntennaPorts CHOICE { |  |  |  |
| Two SEQUENCE { |  |  |  |
| twoTX-codebookSubsetRestriction | 000001 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.2.1.1.2.4.4_1-5: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-100 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| mcs-Table | qam64LowSE |  |  |
| } |  |  |  |

6.2.2.1.1.2.4.3_2	Message exceptions for NSA

Same as specified in 6.2.2.1.1.2.4.3_1.

6.2.2.1.1.2.5	Test Requirements

The pass fail decision is as specified in the test procedure in clause 6.2.2.1.1.2.4.2.

There are no parameters in the test setup or measurement process whose variation impacts the results so there are no applicable test tolerances for this test.

6.2.2.2.1.2	2Rx TDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Whether to adopt minimum sample for pass/fail verdict needs further study.

6.2.2.2.1.2.1	Test Purpose

The purpose of this test is to verify the variance of the wideband CQI reports is within the limits defined and a PDSCH BLER of 10-5 falls between the transport format based median CQI-1 and median CQI or the transport format based median CQI and median CQI +1.

6.2.2.2.1.2.2	Test Applicability

This test applies to all types of NR UE release 16 and forward supporting cqi-TableAlt.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and cqi-TableAlt.

6.2.2.2.1.2.3	Minimum requirement for periodic CQI reporting with Table 3

For the parameters specified in Table 6.2.2.2.1.2.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 10-5, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 10-5. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 10-5, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 10-5.

c)	The reported CQI value according to the reference channel shall be ≥ 1.

Table 6.2.2.2.1.2.3-1: CQI reporting test parameters

| Parameter |  |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |  |
| Subcarrier spacing |  |  | kHz | 30 |  |
| Duplex Mode |  |  |  | TDD |  |
| TDD UL-DL pattern |  |  |  | FR1.30-1 |  |
| SNR |  |  | dB | 1 | 2 |
| Propagation channel |  |  |  | AWGN |  |
| Antenna configuration |  |  |  | 1×2 with static channel specified in Annex B.1 |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |
|  | CDM Type |  |  | FD-CDM2 |  |
|  | Density (ρ) |  |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 1 |  |
|  | CDM Type |  |  | No CDM |  |
|  | Density (ρ) |  |  | 3 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 1,(0,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 1 |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |
|  | CSI-IM RE pattern |  |  | 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |  |
| ReportConfigType |  |  |  | Periodic |  |
| CQI-table |  |  |  | Table 3 |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |
| Sub-band Size |  |  | RB | 16 |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |
|  |  | Codebook Mode |  | 1 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |
|  |  | CodebookSubsetRestriction |  | 000001 |  |
|  |  | RI Restriction |  | N/A |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |
| Measurement channel |  |  |  | As specified in Table A.4-4, TBS.4-2 |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.2.2.2.1.2.

6.2.2.2.1.2.4	Test Description

6.2.2.2.1.2.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the NR cell are set up according to Table 6.1.2-1 and 6.2.2.2.1.2.3-1 as appropriate.

3.	Downlink signals for the NR cell are initially set up according to Annexes C.0, C.1, C.2, C.3.1 , and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions for the NR cell are set according to Annex B.1.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 6.2.2.2.1.2.4.3.

6.2.2.2.1.2.4.2	Test Procedure

1.	Set the parameters of bandwidth, SCS, reference Channel, the propagation condition, antenna configuration and the SNR according to Table 6.2.2.2.1.2.3-1.

2.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to CQI value 2 and keep it regardless of the wideband CQI value sent by the UE. The SS sends downlink MAC padding bits on the DL RMC. Continue transmission of the PDSCH until 5000 wideband CQI reports have been gathered. In this process the SS collects wideband CQI reports every 10 ms and also cases where UE transmits nothing in its CQI timing are also counted as wideband CQI reports.

3.	Set up a relative frequency distribution for the reported wideband CQI-values, Calculate the median value (wideband Median CQI is the wideband CQI that is at or crosses 50% distribution from the lower wideband CQI side). This CQI-value is declared as wideband Median CQI value.

4.	If Median CQI is not equal to 1 or 15 and 4500 or more of the wideband CQI values are in the range (Median CQI - 1) ≤ Median CQI ≤ ( Median CQI + 1) then continue with step 5, otherwise go to step 8.

5.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband median-CQI value and shall not react to the UE’s wideband CQI reports. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH transmitted by the SS, record the associated ACK, NACK and statDTX responses. The responses are then filtered as follows: for the sequence of responses for each HARQ process, discard all the statDTX responses. Measure the BLER for a duration sufficient to achieve statistical significance according to Annex G.4.

For the filtered ACK and NACK responses if the ratio (NACK / (ACK + NACK)) ≤ 10-5 then go to step 6, otherwise go to step 7.

6.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband median-CQI+1 value and shall not react to the UE’s wideband CQI reports. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH, transmitted by the SS, record and filter the ACK, NACK and statDTX responses as in step 5, and measure the BLER for a duration sufficient to achieve statistical significance according to Annex G.4.

If the ratio (NACK / (ACK + NACK)) > 10-5

then pass the UE for this test, otherwise go to step 8.

7.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband median-CQI-1 value and shall not react to the UE’s wideband CQI reports. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH, transmitted by the SS, record and filter the ACK, NACK and statDTX responses as in step 5, and measure the BLER for a duration sufficient to achieve statistical significance according to Annex G.4.

If the ratio (NACK / ACK + NACK) ≤10-5

then pass the UE for this test, otherwise go to step 8.

8.	If both SNR points of the test have not been tested, then repeat the same procedure (steps 1 to 7) for the other SNR point as appropriate. Otherwise fail the UE.

6.2.2.2.1.2.4.3	Message contents

Message contents are according to TS 38.508 [6] clause 5.4.2 with the following exceptions:

6.2.2.2.1.2.4.3_1	Message exceptions for SA

Table 6.2.2.2.1.2.4.3_1-1: NZP CSI-RS-ResourceMapping

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row1 | 0000 | K0=0, row 1, | 1Tx test cases |
| } |  |  |  |
| nrofPorts | p1 |  |  |
| firstOFDMSymbolInTimeDomain | 1 |  |  |
| CDM Type | noCDM |  |  |
| density CHOICE { |  |  |  |
| three | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.2.2.1.2.4.3_1-2: NZP-CSI-RS-Resource

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| slot10 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.2.2.1.2.4.4_1-3: CSI-IM-Resource

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.4-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-IM-Resource ::= SEQUENCE { |  |  |  |
| periodicityAndOffset SEQUENCE { |  |  |  |
| slot10 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.2.2.1.2.4.4_1-4: CSI-ReportConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.4-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| cqi-Table | table3 |  |  |
| } |  |  |  |

Table 6.2.2.2.1.2.4.4_1-5: CodebookConfig

| Derivation Path: TS38.508-1 [6], Table 5.4.2.4-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CodebookConfig ::= SEQUENCE { |  |  |  |
| codebookType CHOICE { |  |  |  |
| type1 SEQUENCE { |  |  |  |
| subType CHOICE { |  |  |  |
| typeI-SinglePanel SEQUENCE { |  |  |  |
| nrOfAntennaPorts CHOICE { |  |  |  |
| Two SEQUENCE { |  |  |  |
| twoTX-codebookSubsetRestriction | 000001 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.2.2.1.2.4.4_1-6: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-100 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| mcs-Table | qam64LowSE |  |  |
| } |  |  |  |

6.2.2.2.1.2.4.3_2	Message exceptions for NSA

Same as specified in 6.2.2.2.1.2.4.3_1.

6.2.2.2.1.2.5	Test Requirements

The pass fail decision is as specified in the test procedure in clause 6.2.2.2.1.2.4.2.

There are no parameters in the test setup or measurement process whose variation impacts the results so there are no applicable test tolerances for this test.

6.2.3.1.1.2	4Rx FDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Whether to adopt minimum sample for pass/fail verdict needs further study.

6.2.3.1.1.2.1	Test Purpose

The purpose of this test is to verify the variance of the wideband CQI reports is within the limits defined and a PDSCH BLER of 10-5 falls between the transport format based median CQI-1 and median CQI or the transport format based median CQI and median CQI +1.

6.2.3.1.1.2.2	Test Applicability

This test applies to all types of NR UE release 16 and forward supporting cqi-TableAlt.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and cqi-TableAlt.

6.2.3.1.1.2.3	Minimum requirement for periodic CQI reporting with Table 3

For the parameters specified in Table 6.2.3.1.1.2.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 10-5, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 10-5. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 10-5, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 10-5.

c)	The reported CQI value according to the reference channel shall be ≥ 1.

Table 6.2.3.1.1.2.3-1: CQI reporting test parameters

| Parameter |  |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 |  |
| Subcarrier spacing |  |  | kHz | 15 |  |
| Duplex Mode |  |  |  | FDD |  |
| SNR |  |  | dB | -2 | -1 |
| Propagation channel |  |  |  | AWGN |  |
| Antenna configuration |  |  |  | 1×4 with static channel specified in Annex B.1 |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |
|  | CDM Type |  |  | FD-CDM2 |  |
|  | Density (ρ) |  |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |
|  | CSI-RS periodicity and offset |  | slot | 5/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 1 |  |
|  | CDM Type |  |  | No CDM |  |
|  | Density (ρ) |  |  | 3 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 1,(0,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 1 |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |
|  | CSI-IM RE pattern |  |  | 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 |  |
| ReportConfigType |  |  |  | Periodic |  |
| CQI-table |  |  |  | Table 3 |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |
| Sub-band Size |  |  | RB | 8 |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  |  | slot | 5/0 |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |
|  |  | Codebook Mode |  | 1 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |
|  |  | CodebookSubsetRestriction |  | 000001 |  |
|  |  | RI Restriction |  | N/A |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |
| CQI/RI/PMI delay |  |  | ms | 8 |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |
| Measurement channel |  |  |  | As specified in Table A.4-4, TBS.4-1 |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.2.3.1.1.2.

6.2.3.1.1.2.4	Test Description

6.2.3.1.1.2.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the NR cell are set up according to Table 6.1.2-1 and 6.2.3.1.1.2.3-1 as appropriate.

3.	Downlink signals for the NR cell are initially set up according to Annexes C.0, C.1, C.2, C.3.1 , and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions for the NR cell are set according to Annex B.1.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 6.2.3.1.1.2.4.3.

6.2.3.1.1.2.4.2	Test Procedure

1.	Set the parameters of bandwidth, SCS, reference Channel, the propagation condition, antenna configuration and the SNR according to Table 6.2.3.1.1.2.3-1.

2.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to CQI value 2 and keep it regardless of the wideband CQI value sent by the UE. The SS sends downlink MAC padding bits on the DL RMC. Continue transmission of the PDSCH until 5000 wideband CQI reports have been gathered. In this process the SS collects wideband CQI reports every 5 ms and also cases where UE transmits nothing in its CQI timing are also counted as wideband CQI reports.

3.	Set up a relative frequency distribution for the reported wideband CQI-values, Calculate the median value (wideband Median CQI is the wideband CQI that is at or crosses 50% distribution from the lower wideband CQI side). This CQI-value is declared as wideband Median CQI value.

4.	If Median CQI is not equal to 1 or 15 and 4500 or more of the wideband CQI values are in the range (Median CQI - 1) ≤ Median CQI ≤ ( Median CQI + 1) then continue with step 5, otherwise go to step 8.

5.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband median-CQI value and shall not react to the UE’s wideband CQI reports. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH transmitted by the SS, record the associated ACK, NACK and statDTX responses. The responses are then filtered as follows: for the sequence of responses for each HARQ process, discard all the statDTX responses. Measure the BLER for a duration sufficient to achieve statistical significance according to Annex G.4.

For the filtered ACK and NACK responses if the ratio (NACK / (ACK + NACK)) ≤ 10-5 then go to step 6, otherwise go to step 7.

6.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband median-CQI+1 value and shall not react to the UE’s wideband CQI reports. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH, transmitted by the SS, record and filter the ACK, NACK and statDTX responses as in step 5, and measure the BLER for a duration sufficient to achieve statistical significance according to Annex G.4.

If the ratio (NACK /(ACK + NACK)) > 10-5

then pass the UE for this test, otherwise go to step 8.

7.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband median-CQI-1 value and shall not react to the UE’s wideband CQI reports. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH, transmitted by the SS, record and filter the ACK, NACK and statDTX responses as in step 5, and measure the BLER for a duration sufficient to achieve statistical significance according to Annex G.4.

If the ratio (NACK /ACK + NACK) ≤10-5

then pass the UE for this test, otherwise go to step 8.

8.	If both SNR points of the test have not been tested, then repeat the same procedure (steps 1 to 7) for the other SNR point as appropriate. Otherwise fail the UE.

6.2.3.1.1.2.4.3	Message contents

Message contents are according to TS 38.508 [6] clause 5.4.2 with the following exceptions:

6.2.3.1.1.2.4.3_1	Message exceptions for SA

Same as specified in clause 6.2.2.1.1.2.4.3_1.

6.2.3.1.1.2.4.3_2	Message exceptions for NSA

Same as specified in 6.2.3.1.1.2.4.3_1.

6.2.3.1.1.2.5	Test Requirements

The pass fail decision is as specified in the test procedure in clause 6.2.3.1.1.2.4.2.

There are no parameters in the test setup or measurement process whose variation impacts the results so there are no applicable test tolerances for this test.

6.2.3.2.1.2	4Rx TDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA

Editor's note:	This clause is incomplete. The following aspects are either missing or not yet determined:

-	Whether to adopt minimum sample for pass/fail verdict needs further study.

6.2.3.2.1.2.1	Test Purpose

The purpose of this test is to verify the variance of the wideband CQI reports is within the limits defined and a PDSCH BLER of 10-5 falls between the transport format based median CQI-1 and median CQI or the transport format based median CQI and median CQI +1.

6.2.3.2.1.2.2	Test Applicability

This test applies to all types of NR UE release 16 and forward supporting cqi-TableAlt.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and cqi-TableAlt.

6.2.3.2.1.2.3	Minimum requirement for periodic CQI reporting with Table 3

For the parameters specified in Table 6.2.3.2.1.2.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 10-5, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 10-5. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 10-5, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 10-5.

c)	The reported CQI value according to the reference channel shall be ≥ 1.

Table 6.2.3.2.1.2.3-1: CQI reporting test parameters

| Parameter |  |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |  |
| Subcarrier spacing |  |  | kHz | 30 |  |
| Duplex Mode |  |  |  | TDD |  |
| TDD UL-DL pattern |  |  |  | FR1.30-1 |  |
| SNR |  |  | dB | -2 | -1 |
| Propagation channel |  |  |  | AWGN |  |
| Antenna configuration |  |  |  | 1×4 with static channel specified in Annex B.1 |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |
|  | CDM Type |  |  | FD-CDM2 |  |
|  | Density (ρ) |  |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 1 |  |
|  | CDM Type |  |  | No CDM |  |
|  | Density (ρ) |  |  | 3 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 1,(0,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 1 |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |
|  | CSI-IM RE pattern |  |  | 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |  |
| ReportConfigType |  |  |  | Periodic |  |
| CQI-table |  |  |  | Table 3 |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |
| Sub-band Size |  |  | RB | 16 |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |
|  |  | Codebook Mode |  | 1 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |
|  |  | CodebookSubsetRestriction |  | 000001 |  |
|  |  | RI Restriction |  | N/A |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |
| Measurement channel |  |  |  | As specified in Table A.4-4, TBS.4-2 |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.2.3.2.1.2.

6.2.3.2.1.2.4	Test Description

6.2.3.2.1.2.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the NR cell are set up according to Table 6.1.2-1 and 6.2.3.2.1.2.3-1 as appropriate.

3.	Downlink signals for the NR cell are initially set up according to Annexes C.0, C.1, C.2, C.3.1 , and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions for the NR cell are set according to Annex B.1.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 6.2.3.2.1.2.4.3.

6.2.3.2.1.2.4.2	Test Procedure

1.	Set the parameters of bandwidth, SCS, reference Channel, the propagation condition, antenna configuration and the SNR according to Table 6.2.3.2.1.2.3-1.

2.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to CQI value 2 and keep it regardless of the wideband CQI value sent by the UE. The SS sends downlink MAC padding bits on the DL RMC. Continue transmission of the PDSCH until 5000 wideband CQI reports have been gathered. In this process the SS collects wideband CQI reports every 10 ms and also cases where UE transmits nothing in its CQI timing are also counted as wideband CQI reports.

3.	Set up a relative frequency distribution for the reported wideband CQI-values, Calculate the median value (wideband Median CQI is the wideband CQI that is at or crosses 50% distribution from the lower wideband CQI side). This CQI-value is declared as wideband Median CQI value.

4.	If Median CQI is not equal to 1 or 15 and 4500 or more of the wideband CQI values are in the range (Median CQI - 1) ≤ Median CQI ≤ ( Median CQI + 1) then continue with step 5, otherwise go to step 8.

5.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband median-CQI value and shall not react to the UE’s wideband CQI reports. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH transmitted by the SS, record the associated ACK, NACK and statDTX responses. The responses are then filtered as follows: for the sequence of responses for each HARQ process, discard all the statDTX responses. Measure the BLER for a duration sufficient to achieve statistical significance according to Annex G.4.

For the filtered ACK and NACK responses if the ratio (NACK / (ACK + NACK)) ≤ 10-5 then go to step 6, otherwise go to step 7.

6.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband median-CQI+1 value and shall not react to the UE’s wideband CQI reports. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH, transmitted by the SS, record and filter the ACK, NACK and statDTX responses as in step 5, and measure the BLER for a duration sufficient to achieve statistical significance according to Annex G.4.

If the ratio (NACK / (ACK + NACK)) > 10-5

then pass the UE for this test, otherwise go to step 8.

7.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband median-CQI-1 value and shall not react to the UE’s wideband CQI reports. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH, transmitted by the SS, record and filter the ACK, NACK and statDTX responses as in step 5, and measure the BLER for a duration sufficient to achieve statistical significance according to Annex G.4.

If the ratio (NACK / ACK + NACK) ≤10-5

then pass the UE for this test, otherwise go to step 8.

8.	If both SNR points of the test have not been tested, then repeat the same procedure (steps 1 to 7) for the other SNR point as appropriate. Otherwise fail the UE.

6.2.3.2.1.2.4.3	Message contents

Message contents are according to TS 38.508 [6] clause 5.4.2 with the following exceptions:

6.2.3.2.1.2.4.3_1	Message exceptions for SA

Same as specified in clause 6.2.2.2.1.2.4.3_1.

6.2.3.2.1.2.4.3_2	Message exceptions for NSA

Same as specified in 6.2.3.2.1.2.4.3_1.

6.2.3.2.1.2.5	Test Requirements

The pass fail decision is as specified in the test procedure in clause 6.2.3.2.1.2.4.2.

There are no parameters in the test setup or measurement process whose variation impacts the results so there are no applicable test tolerances for this test.

<< End of changes >>