3GPP TSG-RAN5 Meeting #98	R5-231029

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5185 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of IoT NTN TC 6.1.1.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To avoid fake pass, need to add step to check that UE is in Idle on Cell 11 in T1. - TAC-1 needs to be added in SIB1-BR of Cell 1 and SIB1-BR of Cell 11 needs to be specified. - UE postion needs to be pre-configured in the preamble. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Step 3 was added to paging the UE. - SIB1-BR of Cell 1 and Cell 11 were updated. - UE postion was pre-configured in the preamble. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A unconformat UE may pass this case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

6.1.1.11	eMTC / NTN / Multi-TAC

6.1.1.11	Test Purpose (TP)

(1)

with { UE in EMM-REGISTERED/RRC-IDLE and UE finds a cell which provides access by E-UTRA RAT }

ensure that {

when { systemInformationBlockType1 broadcast more than one TAC per PLMN }

then { UE forwards the trackingAreaList to upper layers for mobility management procedure }

}

6.1.1.11.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.300 clause 23.21.4.1 and TS 36.331 clauses 5.2.2.7 and 6.2.2. Unless otherwise stated these are Rel-17 requirements.

[TS 36.300, clause 23.21.4.1]

The network may broadcast more than one TAC per PLMN in a cell in order to reduce the signalling load at cell edge in NTN, in particular for Earth-moving cell coverage. The AS layer indicates all received TACs for the selected PLMN to the NAS layer. The network may update the UEs upon TAC removal. UEs may by UE implementation also check whether a TAC has been removed.

[TS 36.331, clause 5.2.2.7]

Upon receiving the SystemInformationBlockType1 or SystemInformationBlockType1-BR either via broadcast or via dedicated signalling, the UE shall:

…

1>	else:

2>	if the frequency band indicated in the freqBandIndicator is part of the frequency bands supported by the UE and it is not a downlink only band; or

2>	if the UE supports multiBandInfoList, and if one or more of the frequency bands indicated in the multiBandInfoList are part of the frequency bands supported by the UE and they are not downlink only bands:

3>	forward the cellIdentity to upper layers;

3>	forward the trackingAreaCode to upper layers;

3>	forward the trackingAreaList to upper layers, if present;

3>	forward the PLMN identity to upper layers;

[TS 36.331, clause 6.2.2]

trackingAreaList A list of tracking area codes for the PLMN listed. For the first entry in plmn-IdentityList-v1700: If this field is present, the list of tracking area codes include the tracking area code in trackingAreaCode (without suffix) and the tracking area codes in trackingAreaList. If this field is absent, trackingAreaCode (without suffix) applies. For other entries in plmn-IdentityList-v1700: If this field is present, the list of tracking area codes include the tracking area codes in trackingAreaList. If this field is absent, the list of tracking area codes of the preceding entry in plmn-IdentityList-v1700 applies. The total number of signalled tracking area codes across all PLMNs cannot be more than maxTAC-r17.

6.1.1.11.3	Test description

6.1.1.11.3.1	Pre-test conditions

System Simulator:

-	Cell 1 (belongs to TAI-1, TAI-2) and Cell 11.

-	System information combination 33 as defined in TS 36.508 [18] clause 4.4.3.1.1-1 is used for both cells.

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE's positioning engine (e.g., standalone GNSS receiver) should be enabled to allow it to acquire the position. This could be done by use of AT command, and the detailed UE location is defined in TS 36.508 [18] Clause 4.xx.

Preamble:

-	UE is in state Registered, Idle Mode (state 2) on Cell 1 (serving cell) according to TS 36.50 [18].

6.1.1.11.3.2	Test procedure sequence

Table 6.1.1.11.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 6.1.1.11.3.2-2. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 36.508 [18].

Table 6.1.1.11.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Cell 1 | Cell 11 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T1 | RS EPRE | dBm/15kHz | -85 | “Off” | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |
| T2 | RS EPRE | dBm/15kHz | “Off” | -85 | Power level “Off” is defined in TS 36.508 Table 6.2.2.1-1 |

Table 6.1.1.11.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell levels according to row T2 of table 6.1.1.11.3.2-1. | - | - | - | - |
| 2 | Check: Does the UE send an RRCConnectionRequest on cell 11 within 3 minutes? | --> | RRCConnectionRequest | 1 | F |
| 3 | Check: Does the test result of generic test procedure in TS 36.508 [18] subclause 6.4.2.7 indicate that the UE is camped on Cell 11? | - | - | 1 | P |

6.1.1.11.3.3	Specific message contents

Table 6.1.1.11.3.3-1: SystemInformationBlockType1-BR-r13 on Cell 1 (preamble and all steps, Table 6.1.1.11.3.2-2)

| Derivation Path: TS 36.508 [18], Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-v1700-IEs SEQUENCE { |  |  |  |
| cellAccessRelatedInfo-NTN-r17 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| cellBarred-NTN-r17 | Not Barred |  |  |
| PLMN-IdentityList-v1700 SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo-v1700 SEQUENCE { | 1 entry |  |  |
| PLMN-IdentityInfo-v1700 SEQUENCE { |  | Entry 1 |  |
| TrackingAreaList-r17 SEQUENCE (SIZE (1..maxTAC-r17)) OF TrackingAreaCode SEQUENCE { | 2 entries |  |  |
| TrackingAreaCode[1] | 1 |  |  |
| TrackingAreaCode[2] | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.1.11.3.3-2: SystemInformationBlockType1-BR-r13 on Cell 11 (preamble and all steps, Table 6.1.1.11.3.2-2)

| Derivation Path: TS 36.508 [18], Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-v1700-IEs SEQUENCE { |  |  |  |
| cellAccessRelatedInfo-NTN-r17 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| cellBarred-NTN-r17 | Not Barred |  |  |
| PLMN-IdentityList-v1700 SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo-v1700 SEQUENCE { | 1 entry |  |  |
| PLMN-IdentityInfo-v1700 SEQUENCE { |  | Entry 1 |  |
| TrackingAreaList-r17 SEQUENCE (SIZE (1..maxTAC-r17)) OF TrackingAreaCode SEQUENCE { | 1 entry |  |  |
| TrackingAreaCode[1] | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

[End of change]