3GPP TSG-RAN5 Meeting #98	R5-231214

Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5201 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EIEI test case 11.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. When eCall over IMS is triggered in eCall only mode, UE may initiate emergency PDN bring up while IMS registration procedure is going on. 2. Specific message content for Tracking Area Update Accept message is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Added steps 3A, 3B and modified main behavior to have parallel handling of IMS registration procedure and emergency PDN bring up. 2. Added specific message content Table 11.3.1.3.3-4A. 3. Fixed formatting issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

### 11.3.1	eCall Only mode / T3444 / eCall inactivity procedure / Removal of eCall only restriction after an eCall over IMS

#### 11.3.1.1	Test Purpose (TP)

(1)

with { UE is switched ON with eCall only enabled USIM}

ensure that {

when { UE reads a ims-EmergencySupport-r9 and eCallOverIMS-Support-r14 from SystemInformationBlockType1}

then {UE enters substate EMM-DEREGISTERED.eCALL-INACTIVE and shall not start registration process.}

}

(2)

with { The UE is in the state EMM-DEREGISTERED.eCALL-INACTIVE}

ensure that {

when { UE is requested to make a manual eCall}

then { UE sends ATTACH REQUEST message with EPS attach type IE 'combined EPS/IMSI attach'}

}

(3)

with { UE receives ATTACH ACCEPT message with EPS attach result 'combined EPS/IMSI attach' and

IMS voice over PS sessions is supported }

ensure that {

when { UE is in the EMM-REGISTERED.NORMAL-SERVICE state and an initial IMS registration is performed }

then { the UE transmits a PDN CONNECTIVITY REQUEST message with the request type set to

"emergency" }

}

(4)

with { UE receives an RRCConnectionRelease message and enters in RRC_IDLE state }

ensure that {

when { SS sends paging request for IMS MT speech call }

then {the UE answers the paging request for IMS MT speech call  }

}

(5)

with { UE receives an RRCConnectionRelease message and enters in RRC_IDLE state }

ensure that {

when { UE starts T3444 for 12Hours and the periodic tracking area updating timer T3412 expires }

then { UE sends TRACKING AREA UPDATE REQUEST message with EPS update type = ’Periodic updating' }

}

(6)

with { UE is in RRC_IDLE state }

ensure that {

when { eCall Inactivity timer T3444 expires }

then { The UE performs Detach procedure }

}

#### 11.3.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.301, clauses 5.3.1.2.1, 5.5.3.4 and 10.2 and TS 36.331, clauses 6.2.2, 5.2.2.7

[TS 24.301 clause 5.3.1.2.1]

The signalling procedure for the release of the NAS signalling connection is initiated by the network.

In S1 mode, when the RRC connection has been released, the UE shall enter EMM-IDLE mode and consider the NAS signalling connection released.

If the UE is configured for eCall only mode as specified in 3GPP TS 31.102 [17] then:

-	if the NAS signalling connection that was released had been established for eCall over IMS, the UE shall start timer T3444; and

-	if the NAS signalling connection that was released had been established for a call to an HPLMN designated non-emergency MSISDN or URI for test or terminal reconfiguration service, the UE shall start timer T3445.

[TS 24.301 clause 5.5.3.4]

The eCall inactivity procedure is applicable only to a UE configured for eCall only mode as specified in 3GPP TS 31.102 [17]. The procedure shall be started when timer T3444 or timer T3445 expires or is found to have already expired in any EMM-REGISTERED substate except substates EMM-REGISTERED.PLMN-SEARCH or EMM-REGISTERED.NO-CELL-AVAILABLE. The UE shall then perform the following actions:

- stop other running timers (e.g. T3411, T3412);

- if the UE is currently registered to EPS services only, perform a detach procedure for EPS services only;

-	if the UE is currently registered for both EPS services and non-EPS services, perform a combined detach procedure for EPS services and non-EPS services;

- delete any GUTI, TAI list, last visited registered TAI, list of equivalent PLMNs, and KSI; and

- enter EMM-DEREGISTERED.eCALL-INACTIVE state.

[TS 24.301 clause 10.2]

| TIMER NUM. | TIMER VALUE | STATE | CAUSE OF START | NORMAL STOP | ON EXPIRY |
| --- | --- | --- | --- | --- | --- |
| T3444 | 12 hours | All except EMM-NULL | UE configured for eCall only mode enters EMM-IDLE mode after an eCall over IMS | Removal of eCall only restriction | Perform eCall inactivity procedure as described in subclause 5.5.3.4 |
| T3445 | 12 hours | All except EMM-NULL | UE configured for eCall only mode enters EMM-IDLE mode after a call to a non-emergency MSISDN or URI for test or terminal reconfiguration service | Removal of eCall only restriction | Perform eCall inactivity procedure as described in subclause 5.5.3.4 |

[TS 36.331 clause 5.2.2.7]

Upon receiving the SystemInformationBlockType1 or SystemInformationBlockType1-BR either via broadcast or via dedicated signalling, the UE shall:

1>	if in RRC_IDLE or in RRC_CONNECTED while T311 is running; and

…

3>	forward the eCallOverIMS-Support to upper layers, if present;

[TS 36.331 clause 6.2.2]

eCallOverIMS-Support

Indicates whether the cell supports eCall over IMS services for UEs as defined in TS 23.401 [41]. If absent, eCall over IMS is not supported by the network in the cell. NOTE 2.

#### 11.3.1.3	Test description

11.3.1.3.1	Pre-test conditions

System Simulator:

-	Cell A is configured according to Table 6.3.2.2-1 in [18] and is the serving cell.

UE:

-	the eCall capable UE is equipped with ‘eCall only’ enabled USIM configured as per TS 36.508 [18] Table 4.9.3.5-2.

-	The UE is equipped with a USIM containing values shown in Table 11.3.1.3.1-1.

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

11.3.1.3.2	Test procedure sequence

Table 11.3.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Check: Does UE send an RRCConnectionRequest on Cell 1 within 120 seconds? | --> | RRCConnectionRequest | 1 | F |
| 3 | A manual eCall is initiated. (See Note 1) | - | - | - | - |
| 3A | Check: Does UE transmits a RRCConnectionRequest message with 'establishmentCause' set to ' emergency'? | --> | RRCConnectionRequest | 2 | P |
| 3B | SS transmits an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| 4 | Check: Does the UE sends an ATTACH REQUEST message with a PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN. EPS attach type = "combined EPS/IMSI attach"? | --> | ATTACH REQUEST | 2 | P |
| 5-13 | Steps 5-13 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 14 | The SS transmits an ATTACH ACCEPT message with the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message and SS assigns value of 186 minutes for the timer T3412. | <-- | ATTACH ACCEPT | - | - |
| 15 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of default bearer. | - | - | - | - |
| - | EXCEPTION: In parallel to the event described in step 16 below, if initiated by the UE the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in steps 16-25 below the generic procedure for IMS signalling in the Uplane specified in subclause 4.5A.3 takes place if requested by the UE. | - | - | - | - |
| 16 | Step 16 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 is performed. | - | - | - | - |
| 17-18b1 | Void | - | - | - | - |
| 19 | Check: Does UE transmit a PDN connectivity REQUEST message to request an additional PDN, with 'Request type' set to 'emergency (‘0100’B)’? | --> | PDN CONNECTIVITY REQUEST | 3 | P |
| 20-25 | The UE performs Steps 20-25 of the generic test procedure for eCall over IMS establishment in EUTRA: eCall only mode described in TS 36.508 subclause 4.5A.27.3. | - | - | - | - |
| 26-29 | Release eCall over IMS using the generic procedure described in TS 34.229-1 [35] subclause C.33 | - | - | - | - |
| 30 | Void | - | - | - | - |
| 31 | The SS releases the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 32 | The SS waits 1 minute. | - | - | - | - |
| 33-56 | Steps 1 to 24 of the generic test procedure for IMS MT speech call specified in TS 36.508 subclause 4.5A.7.3 | - | - | 4 | P |
| 57-60 | Generic test procedure for MT release of IMS call as described in annex C.33 of TS 34.229-1 [35] takes place. | - | - | - | - |
| 61 | The SS releases the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 62 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message at the expiry of T3412 periodically for every 186 minutes and remains registered until T3444 expires? (See Note 2) | --> | TRACKING AREA UPDATE REQUEST | 5 | P |
| 63 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 64 | Check: Does the UE transmit a DETACH REQUEST message at expiry of T3444? | --> | DETACH REQUEST | 6 | P |
| 65 | The SS responds with DETACH ACCEPT message. | <-- | DETACH ACCEPT | - | - |
| 66 | The SS releases the RRC connection. | --> | RRCConnectionRelease | - | - |
| Note 1: The request to originate a manual eCall may be performed by MMI or AT command. Note 2: Timer T3412 is reset and started with its initial value, when the UE changes from EMM-CONNECTED to EMM-IDLE mode so the timer T3412 should be started after step 61 and T3444 should be started after step 31. |  |  |  |  |  |

11.3.1.3.3	Specific message contents

Table 11.3.1.3.3-1: Message SystemInformationBlockType1 (Preamble)

Derivation path: 36.508 table 4.4.3.2-3 Condition eCalloverIMS

Table 11.3.1.3.3-2: Message ATTACH ACCEPT (step 14, Table 11.3.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS attach result | '010'B | ''Combined EPS/IMSI attach'' |  |
| T3412 value |  |  |  |
| Unit | '010' | ''value is incremented in multiples of decihours'' |  |
| Timer value | '11111' | ''186 minutes'' |  |
| GUTI | GUTI-1 |  |  |
| EPS network feature support | '0000 0011'B | emergency bearer services in S1 mode and IMS voice over PS session in S1 mode are supported |  |

Table 11.3.1.3.3-3: Message PDN CONNECTIVITY REQUEST (step 19, Table 11.3.1.3.2-1)

| Derivation Path: 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Request type | '0100'B | emergency |  |
| Access point name | Not present |  |  |

Table 11.3.1.3.3-4: Message TRACKING AREA UPDATE REQUEST (step 62, Table 11.3.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type value | '011'B | ''Periodic updating'' |  |
| Old GUTI | GUTI-1 |  |  |

Table 11.3.1.3.3-4A: Message TRACKING AREA UPDATE ACCEPT (step 63, Table 11.3.1.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| T3412 value |  |  |  |
| Unit | '010' | ''value is incremented in multiples of decihours'' |  |
| Timer value | '11111' | ''186 minutes'' |  |
| EPS network feature support | '0000 0011'B | IMS voice over PS session in S1 mode is supported emergency bearer services in S1 mode supported |  |

Table 11.3.1.3.3-5: DETACH REQUEST (Step 64, Table 11.3.1.3.2-1)

| Derivation Path: 36.508, Table 4.7.2-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Detach type |  |  |  |
| Type of detach | 001 | combined EPS/IMSI detach |  |
| Switch off | 0 | normal detach |  |
| GUTI or IMSI | GUTI-1 |  |  |

<End of modified section>