3GPP TSG-RAN5 Meeting #98	R5-230045

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2148 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of minimum requirements for FR1 6.6.18.0 - concurrent gaps |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add minimum requirements for FR1 concurrent gaps TCs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Minimum requirements for FR1 concurrent gaps TCs are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplet. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, 6.6.18 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

2	References

The following documents contain provisions, which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[4]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[5]	3GPP TS 38.101-4: "NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements".

[6]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[7]	3GPP TS 38.211: "NR; Physical channels and modulation".

[8]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[9]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[10]	3GPP TS 38.215: "NR; Physical layer measurements".

[11]	3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities".

[12]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[13]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[14]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment".

[15]	3GPP TS 38.508-2: "5GS; User Equipment (UE) conformance specification; Part 2: Common Implementation Conformance Statement (ICS) proforma".

[16]	3GPP TS 38.509: "5GS; Special Conformance Testing Functions for UE".

[17]	3GPP TS 38.521-1: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone".

[18]	3GPP TS 38.521-2: " NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone".

[19]	3GPP TS 38.521-3: " NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[20]	3GPP TS 38.521-4: "NR; User Equipment (UE) conformance specification; Part 4: Performance".

[21]	3GPP TS 38.522: "NR; User Equipment (UE) conformance specification; Applicability of radio transmission, radio reception and radio resource management test cases".

[22]	3GPP TS 38.903: "NR; Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases".

[23]	3GPP TS 36.133: "E-UTRA requirements for support of radio resource management".

[24]	3GPP TS 36.211: "E-UTRA Physical Channels and Modulation".

[25]	3GPP TS 36.508: "Common test environments for User Equipment (UE)".

[26]	3GPP TS 36.521-3: "E-UTRA; UE conformance specification; Radio transmission and reception; Part 3: Radio Resource Management (RRM) conformance testing"

[27]	3GPP TS 36.101: "E-UTRA UE radio transmission and reception".

[28]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[29]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Resource Control (RRC) Protocol Specification".

[30]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[31]	3GPP TS 38.212 "NR; Multiplexing and channel coding".

[32]	3GPP TR 38.810: "Study on test methods for New Radio".

[33]	3GPP TR 36.213: "E-UTRA; Physical layer procedures".

[34]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[35]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[36]	3GPP TS 37.106: "User Equipment (UE) requirements for shared spectrum channel access".

[XX]	3GPP TS 37.355: "LTE Positioning Protocol (LPP) ".

<< Skip unchanged text >>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AoA	Angle of Arrival

AoD	Angle of Departure

BFD	Beam Failure Detection

BFD-RS	BFD Reference Signal

BS	Base Station

BWP	Bandwidth Part

CBD	Candidate Beam Detection

CDF	Cumulative Distribution Function

CC	Component Carrier

CLI	Cross Link Interference

CMR	Channel Measurement Resource

CORESET	Control Resource Set

CP	Cyclic Prefix

CSI	Channel-State Information

CSI-RS	CSI Reference Signal

CSI-RSRP	CSI Reference Signal based Reference Signal Received Power

CSI-RSRQ	CSI Reference Signal based Reference Signal Received Quality

CSI-SINR	CSI Reference Signal based Signal to Noise and Interference Ratio

DC	Dual Connectivity

DCI	Downlink Control Information

DL	Downlink

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

E-CID	Enhanced Cell ID

E-UTRA	Evolved UTRA

E-UTRA/5GC	E-UTRA connected to 5GC

E-UTRAN	Evolved UTRAN

EN-DC	E-UTRA – NR Dual Connectivity

FR	Frequency Range

FR1	Frequency Range 1

FR2	Frequency Range 2

HARQ	Hybrid Automatic Repeat Request

HO	Handover

IMR	Interference Measurement Resource

L1-RSRP	Layer 1 RSRP

LMF	Location Management Function

LPP	LTE Positioning Protocol

MAC	Medium Access Control

MCG	Master Cell Group

MG	Measurement Gap

MGL	Measurement Gap Length

MGRP	Measurement Gap Repetition Period

MIB	Master Information Block

MN	Master Node

MR-DC	Multi-Radio Dual Connectivity

NE-DC	NR-E-UTRA Dual Connectivity

NGEN-DC	NG-RAN E-UTRA-NR Dual Connectivity

NR	New Radio

NR/5GC	NR connected to 5GC

NR-DC	NR-NR Dual Connectivity

NSA	Non-Standalone

OCNG	OFDMA Channel Noise Generator

OFDM	Orthogonal Frequency Division Multiplexing

OFDMA	Orthogonal Frequency Division Multiple Access

PCC	Primary Component Carrier

PCell	Primary Cell

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PLMN	Public Land Mobile Network

PRACH	Physical RACH

PRS	Positioning Reference Signal

PSCell	Primary Secondary Cell

PSS	Primary Synchronization Signal

pTAG	Primary Timing Advance Group

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

QCL	Quasi Co-Location

RACH	Random Access Channel

RAT	Radio Access Technology

RedCap	Reduced Capabilities

RLM	Radio Link Monitoring

RLM-RS	Reference Signal for RLM

RMC	Reference Measurement Channel

RMSI	Remaining Minimum System Information

RRC	Radio Resource Control

RRM	Radio Resource Management

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

RTT	Round Trip Time

SA	Standalone

SCC	Secondary Component Carrier

SCell	Secondary Cell

SCG	Secondary Cell Group

SCS	Subcarrier Spacing

SCSSSB	SSB subcarrier spacing

SDL	Supplementary Downlink

SFN	System Frame Number

SFTD	SFN and Frame Timing DifferenceSI	System Information

SIB	System Information Block

SMTC	SSB-based Measurement Timing configuration

SpCell	Special Cell

SRS	Sounding Reference Signal

SRS-RSRP	Sounding Reference Signal based Reference Signal Received Power

SS	System Simulator

SS-RSRP	Synchronization Signal based Reference Signal Received Power

SS-RSRQ	Synchronization Signal based Reference Signal Received Quality

SS-SINR	Synchronization Signal based Signal to Noise and Interference Ratio

SSB	Synchronization Signal Block

SSB_RP	Received (linear) average power of the resource elements that carry NR SSB signals and channels, measured at the UE antenna connector.

SSS	Secondary Synchronization Signal

sTAG	Secondary Timing Advance Group

SUL	Supplementary Uplink

TA	Timing Advance

TAG	Timing Advance Group

TCI	Transmission Configuration Indicator

TTI	Transmission Time Interval

UL	Uplink

<< Skip unchanged text >>

6.6.18	SA event triggered reporting tests with concurrent gaps

6.6.18.0	Minimum conformance requirements

6.6.18.0.1	Minimum conformance requirements for Intra-frequency measurement

The UE shall be able to identify a new detectable intra-frequency cell within the time period specified in TS 38.133 [6].

Intra-frequency measurements without measurement gaps:

When UE supports concurrentMeasGap-r17 and is configured with concurrent measurement gaps,

Kp is the scaling factor for an SSB frequency layer to be measured without measurement gaps. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(SMTC period, MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap and/or per-FR measurement gap within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions within the window, including those overlapped with measurement gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion within the window W, after accounting for measurement gap collisions by applying the measurement gap collision rule in TS 38.133 [6] section 9.1.8.3.

Kp = 1 when Navailable = 0.

-	Otherwise, when UE is not configured with or UE does not support concurrent measurement gaps:

When intra-frequency SMTC is fully non overlapping with measurement gaps or intra-frequency SMTC is fully overlapping with MGs, Kp =1

When intra-frequency SMTC is partially overlapping with measurement gaps, Kp = 1/(1- (SMTC period /MGRP)), where SMTC period < MGRP. When intra-frequency SMTC is partially overlapping with the ML of NCSG, Kp = 1/(1- (SMTC period /VIRP)), where SMTC period < VIRP. For calculation of Kp, if the high layer signalling (TS 38.331 [13]) of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, the SMTC periodicity corresponds to the value of higher layer parameter smtc2; for the other cells, the SMTC periodicity corresponds to the value of higher layer parameter smtc1.

If the higher layer signaling in TS38.331 [13] signalling of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, time period requirements are not specified to identify a new detectable intra-frequency cell.

Intra-frequency measurements with measurement gaps:

Kgap is the scaling factor for a SSB frequency layer to be measured within an associated measurement gap pattern. Kgap = 1 when the UE is not configured with concurrent measurement gaps or not supporting [concurrent measurement gaps]. Otherwise, Kgap = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

For a window W of duration max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap and per-FR measurement gap within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion:

--	Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with other measurement gap occasions within the window, and

Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated measurement gap within the window W after accounting for measurement gap collisions by applying the measurement gap collision rule in TS 38.133 [6] section 9.1.8.3.

When concurrent measurement gaps are configured, requirements in this clause do not apply ifNavailable =0.

If the higher layer signaling in TS 38.331 [13] of smtc2 is present and smtc1 is fully overlapping with measurement gaps and smtc2 is partially overlapping with measurement gaps, time period requirements are not specified for identify a new detectable intra-frequency cell.

The normative reference for this requirement is TS 38.133 [6] clauses 9.2.2, 9.2.4.3, 9.2.5 and 9.2.6.

6.6.18.0.2	Minimum conformance requirements for Inter-frequency measurement

The UE shall be able to identify a new detectable inter-frequency cell within the time period specified in TS 38.133 [6].

Inter-frequency measurements without measurement gaps:

When UE supports concurrentMeasGap-r17 and is configured with concurrent measurement gaps,

Kp is a scaling factor for an SSB frequency layer to be measured without measurement gaps. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

For a window W of duration max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the SSB frequency layer, and starting at the beginning of any SMTC occasion:

Ntotal is the total number of SMTC occasions within the window, including those overlapped with MG occasions within the window, and

Navailable is the number of SMTC occasions that are not overlapped with any MG occasion within the window W, after accounting for MG collisions by applying the selected gap collision rule provided that concurrent measurement gaps are configured.

Kp = 1 when Navailable = 0.

Otherwise, when UE is not configured with or UE does not support concurrent measurement gaps:

When inter-frequency SMTC is fully non overlapping with measurement gaps or inter-frequency SMTC is fully overlapping with MGs, Kp =1.

When inter-frequency SMTC is partially overlapping with measurement gaps, Kp = 1/(1- (SMTC period /MGRP)), where SMTC period < MGRP. When inter-frequency SMTC is partially overlapping with the VIL of NCSG, Kp = 1/(1- (SMTC period /VIRP)), where SMTC period < VIRP.

Inter-frequency measurements with measurement gaps:

Kgap is a scaling factor for a SSB frequency layer to be measured within an associated measurement gap pattern. Kgap = 1 when the UE is not configured with concurrent measurement gaps. Otherwise, Kgap = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(SMTC period,  MGRP_max), where MGRP_max is the maximum MGRP across all configured per-UE measurement gap(s) and per-FR measurement gap(s) within the same FR, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions that are covered by instances of the associated measurement gap within the window W, including those overlapped with other measurement gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated measurement gap within the window W, after accounting for collisions between the measurement gaps by applying the measurement gap collision rule in TS 38.133 [6] section 9.1.8.3.

Kgap is only applicable for UE supporting concurrentMeasGap-r17. When concurrent measurement gaps are configured, requirements in this clause do not apply if Navailable =0.

The normative reference for this requirement is TS 38.133 [6] clause 9.3.2, 9.3.4, 9.3.5, 9.3.6.3, 9.3.9.

6.6.18.0.3	Minimum conformance requirements for Inter-RAT measurement

The requirements are application for NR−E-UTRAN FDD and NR−E-UTRAN TDD when DRX is used or not.

When the UE requires measurement gaps to identify and measure inter-RAT cells and an appropriate measurement gap pattern is scheduled, or when the UE is capable of concurrent measurement gap patterns and concurrent measurement gap patterns are scheduled, or the UE supports capability of conducting such measurements without gaps, the UE shall be able to identify a new detectable FDD or TDD cell within the time period specified in TS 38.133 [6].

For a UE supporting and configured with concurrent measurement gaps, Kgap_EUTRA: it is the scaling factor for an E-UTRAN frequency layer to be measured within the associated measurement gap pattern. Kgap = 1 when the UE is not configured with concurrent measurement gaps. Otherwise, Kgap_EUTRA = Ntotal / Navailable for UE configured with concurrent measurement gaps.

For a window W of duration MGRP_max, where MGRP_max is the maximum MGRP across all configured per-UE measurement gap(s) and per-FR measurement gap(s) for FR1, and starting from the beginning of any associated gap occasion:

Ntotal is the total number of associated gap occasions within the window, including those overlapped with other MG occasions within the window, and

Navailable is the number of non-dropped associated measurement gap occasions after accounting for collisions between the measurement gaps by applying the measurement gap collision rule in TS 38.133 [6] section 9.1.8.3.

Requirements do not apply for UE configured with concurrent measurement gaps, if Navailable=0

The normative reference for this requirement is TS 38.133 [6] clause 9.4.2, 9.4.3.

6.6.18.0.4	Minimum conformance requirements for PRS measurement

When physical layer receives last of NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message from LMF via LPP specified in TS 37.355 [XX], the UE shall be able to measure multiple (up to the UE capability specified in TS 38.133 [6] Clause 9.9.2.3) DL RSTD measurements, defined in TS 38.215 [10], during the measurement period defined in TS 38.133 [6] Clause 9.9.2.5

Kp,PRS,i is a scaling factor for a positioning frequency layer to be measured within the associated measurement gap pattern, which is defined as Kp,PRS,i = Ntotal / Navailable for UE configured with concurrent measurement gap, and Kp,PRS,i = 1 for UE not configured with concurrent measurement gap.

-	For a window W of duration max(TPRS,i,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE MG and per-FR MG within the same FR as the positioining frequency layer, and starting at the beginning of any associated gap occasions covering the PRS occasion:

-	Ntotal is the total number of associated gap occasions covering PRS occasions within the window, including those overlapped with other MG occasions within the window, and

-	Navailable is the number of non-dropped associated gap occasions covering PRS occasions within the window W, after further accounting for MG collisions by applying the selected gap collision rule

-	Requirements do not apply if Navailable =0.

The normative reference for this requirement is TS 38.133 [6] clause 9.9.2.

<< End of changes >>