3GPP TSG-RAN5 Meeting #98	R5-230104

# Athens, Greece, 27th  Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2969 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RedCap: Test Model updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To implement the RedCap-specific test cases, the NR/5GC test model needs to be enhanced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added clarification in the NR/5GC physical layer aspects in clause 7.3.2 to cover RedCap-specific initial uplink/downlink BWP. - Added description of 5G eDRX in the Idle or Inactive Mode in clause 7.3.4. Removed deprecated sentence in clause 7.1.5.2 and updated Note 1 in Table 7.1.5.2-1. Added SS requirements in clause 7.3.3.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will not be possible to implement the RedCap-specific test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 7.1.5.2, 7.3.2, 7.3.3.1, 7.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

ASP	Abstract Service Primitive

ATS	Abstract Test Suite

AWGN	Additive White Gaussian Noise

BWP	Bandwidth Part

CA	Carrier Aggregation

CBRA	Contention Based Random Access

CC	Component Carriers

CCE	Control Channel Element

CFRA	Contention Free Random Access

CORESET	Control Resource Set

DAI	Downlink Assignment Index

DCI	Downlink Control Information

DL	Downlink

DL-SCH	Downlink Shared Channel

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

EHC	Ethernet Header Compression

EN-DC	E-UTRA-NR Dual Connectivity

EPC	Evolved Packet Core

FR1	Frequency Range 1

FR2	Frequency Range 2

HO	Handover

H-SFN	Hyper SFN

ICS	Implementation Conformance Statement

IUT	Implementation Under Test

IXIT	Implementation eXtra Information for Testing

LSB	Least Significant Bit

MCG	Master Cell Group

MN	Master Node

MSB	Most Significant Bit

NCD-SSB	Non Cell Defining SSB

NE-DC	NR-E-UTRA Dual Connectivity

NR	NR Radio Access

NR-DC	NR-NR Dual Connectivity

PCell	Primary Cell

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PH	Paging Hyperframe

PO	Paging Occasion

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PSCell	Primary SCG Cell

PSS	Primary Synchronisation Signal

PTW	Paging Time Window

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

RA	Random Access

RACH	Random Access Channel

RAR	Random Access Response

RAT	Radio Access Technology

RMSI	Remaining Minimum SI

SCell	Secondary Cell

SCG	Secondary Cell Group

SFN	System Frame Number

SN	Secondary Node

SRS	Sounding Reference Signal

SS	System Simulator

SSB	Synchronization Signal and PBCH block

SSS	Secondary Synchronisation Signal

TC	Test Case

UL	Uplink

UL-SCH	Uplink Shared Channel

UT	Upper Tester

VNG	Virtual Noise Generator

<End of modified section>

<Start of modified section>

#### 7.1.5.2	Cell(s) timing

The timing of E-UTRA cells is specified in TS 36.523-3 [12] subclause 7.4.3.1.

The timing information used in an NR cell is specified in terms of H-SFN / SFN / subframe / slot.

The DL timing offset of each NR cell is specified in Table 7.1.5.2-1.

Table 7.1.5.2-1: DL timing parameters of simulated NR cells

| NR cell Id | H-SFN-offset (note 1) | SFN-offset for FDD (note 2) | SFN-offset for TDD (note 2) | Tcell (note 3) | Tc-offset (note 4) |
| --- | --- | --- | --- | --- | --- |
| NR Cell 1 | 0 | 0 | 0 | 0 | 0 |
| NR Cell 2 | 0 | 124 | 0 | 0 | 0 |
| NR Cell 3 | 0 | 257 | 257 | 0 | 0 |
| NR Cell 4 | 0 | 1000 | 0 | 0 | 0 |
| NR Cell 6 | 0 | 656 | 656 | 0 | 0 |
| NR Cell 10 | 0 | 129 | 129 | 0 | 0 |
| NR Cell 11 | 0 | 956 | 0 | 0 | 0 |
| NR Cell 12 | 0 | 1015 | 257 | 0 | 0 |
| NR Cell 13 | 0 | 890 | 656 | 0 | 0 |
| NR Cell 14 | 0 | 680 | 680 | 0 | 0 |
| NR Cell 23 | 0 | 383 | 257 | 0 | 0 |
| NR Cell 28 | 0 | 890 | 890 | 0 | 0 |
| NR Cell 29 | 0 | 680 | 890 | 0 | 0 |
| NR Cell 30 | 0 | 1015 | 129 | 0 | 0 |
| NR Cell 31 | 0 | 53 | 129 | 0 | 0 |
| NOTE 1: H-SFN-offset corresponds to the offset applied on hyper system frame number (0..1023) NOTE 2: SFN-offset corresponds to the offset applied on system frame number (0..1023). NOTE 3: Tcell corresponds to the timing offset in Ts. Ts = 1/(15000 * 2048) as for E-UTRA. NOTE 4: Tc-offset corresponds to the timing offset in Tc. = Ts/Tc = 64 with Tc = 1/(480000 * 4096). See TS 38.211 [19] subclause 4.1 and TS 36.211 [23] subclause 4). NOTE 5: For each NR cell, its SSB-Index is specified in TS 38.508-1 [5] Table 4.4.2-2. |  |  |  |  |  |

<End of modified section>

<Start of modified section>

### 7.3.2	Physical layer aspects

The physical layer aspects specified in clause 7.1.2 apply, unless explicitly specified differently in the present clause.

When RedCap-specific initial downlink BWP is configured, TTCN configures both the default initial DL BWP and the RedCap-specific initial DL BWP in SS. Each DCI is assigned to either the initial DL BWP or the RedCap-specific initial DL BWP depending on the test case configuration.

NOTE: The configuration of NCD-SSB is not covered in the current test model.

When RedCap-specific initial uplink BWP is configured, TTCN configures both the default initial UL BWP and the RedCap-specific initial UL BWP in SS.

<End of modified section>

<Start of modified section>

#### 7.3.3.1	General SS requirements

TTCN provides the complete system information and scheduling information to the SS as a structured ASN.1 type using a single control ASP (NR_SYSTEM_CTRL_REQ). The following rules apply:

-	The system information is sent to SS using the asn.1 types. The SS shall encode each SI message as specified in Table 8.1-1 and add the necessary padding bits as specified in TS 38.331 [16], clause 8.5.

-	The SS shall start scheduling all system information from the same SFN.

-	The scheduling information sent to SS is the same as the scheduling information sent to the UE. For each SI message (including SI messages containing posSIBs), the slotOffset list in NR_SYSTEM_CTRL_REQ indicates the exact point in time in the SI-window at which SS shall transmit the related SI to the UE.

-	SI-window shall be calculated as per TS 38.331[16] clause 5.2.2.3.2 for all SI messages. For SI messages containing posSIBs, the parameter OffsetToSI_Used indicates to SS the SI-window calculation to be applied.

- 	In the present version of this specification, encryption of the content of SI messages containing posSIBs is not supported.

-	If the H-SFN value is included in SIB1 sent from the TTCN, the SS shall set the hyperSFN in this message to the current H-SFN. In this case, a dummy value is provided by TTCN.

-	If the H-SFN is not included in the SIB1 sent from the TTCN, it shall not be added by the SS.

<End of modified section>

<Start of modified section>

### 7.3.4	Paging and Short Message

SS can be configured with a DCI including Short Messages. In that case, when SS is triggered to transmit a Paging message, the Short Message indication shall be included in the DCI.

SS can be triggered to transmit Short Messages alone; this is achieved in TTCN with the DciTrigger type. SS is triggered to send the Paging message or a Short Message indication at a calculated Paging Occasion provided in the activation time and optionally a list of slot offsets when multiple paging occasions are applied. Discontinuous Reception (DRX) or extended DRX (eDRX) is applied for the transmission of a Paging message or a Short Message indication to the UE in the RRC_IDLE or RRC_INACTIVE states. The paging frame calculation is according to TS 38.304 [24] clause 7. The following default values are provided in TS 38.508-1[5]:

-	defaultPagingCycle = 128

-	Ns = one

-	N = 64

-	PF_Offset = 0

When these default values are applied, the Paging Frame calculation is:

-	Paging Frame: SFN mod 128 =2*(UE_ID mod 64) so depending on UE_ID, the Paging Frame is set to 0,2,4,6,… 126 of the  paging cycle.

When UE is in RRC_IDLE or RRC_INACTIVE and when PDCCH monitoring occasions with default association is applied, the Paging Occasion is set to:

-	slot#2 of the Paging Frame when the cell is configured with SSB#1 or

-	slot#1 of the Paging Frame when the cell is configured with SSB#0.

To notify system information modification when UE is in RRC_CONNECTED and when PDCCH monitoring occasions with default association is applied, the Paging Occasion is set to:

-	slot#1 and slot#2 of the Paging Frame when the cell is configured with SSB#1 or

-	slot#0 and slot#1 of the Paging Frame when the cell is configured with SSB#0.

The UE operates in eDRX for CN paging in RRC_IDLE or RRC_INACTIVE states if the UE is configured for eDRX by upper layers and eDRX-AllowedIdle is signalled in SIB1. The UE operates in eDRX for RAN paging in RRC_INACTIVE state if the UE is configured for eDRX by RAN and eDRX-AllowedInactive is signalled in SIB1. The parameter used is the Paging Hyperframe (PH) and refers to the hyper SFN (H-SFN) in which the UE starts monitoring POs during a Paging Time Window (PTW). The H-SFN is broadcasted by the cell and increments by one when the SFN wraps around.

When a change notification is received, the UE acquires the updated system information at the next H-SFN boundary defined by:

H-SFN mod 1024 = 0

If the UE is configured with extended DRX cycle no longer than 1024 radio frames, it monitors POs as defined on normal DRX with parameter TeDRX, CN, otherwise it will monitor POs calculated as normal DRX only within the periodic PTW configured for the UE. The calculation of the bounds for the PTW are UE-specific and described on TS 38.304 [24] clause 7.4

Figure 7.3.4-1: POs in RRC_IDLE or RRC_INACTIVE state when eDRX cycle > 1024 radio frames

<End of modified section>