3GPP TSG-RAN5 Meeting #98	R5-230204

Electronic Meeting, 22 February – 5 March 2021

Agenda item:	5.3.17.9

Source:	Keysight Technologies

Title:	PC5 measurement grids including the alternate antenna array assumptions

Document for:	Discussion and Endorsement

# Introduction

This contribution defines the measurement grids for PC5.

# Measurement Grids for PC5

In RAN5#97 [1], it was agreed that the antenna assumption for MTSU/MU/TT definitions is based on the same antenna assumptions as PC1 [2][3]

Proposal1: Endorse PC5 FWA device and antenna assumptions as listed in Table 2.1.

It is therefore proposed to leverage the measurement grids defined for PC1 [4] for PC5 as default measurement grids.

Proposal 1: Leverage the measurement grids defined for PC1 for PC5 as default measurement grids

Additionally, in RAN5#97 [1], an 6x6 alternate antenna array assumptions for PC5 was endorsed to complement the PC1 12x12 antenna array assumption [2][3] to support test time reduction.

Proposal2: For MU simulations, in addition to 12x12 antenna array, assume a 6x6 antenna array which a UE is allowed to optionally declare via vendor declarations.

This approach is aligned with the PC3 antenna array assumptions:

- Default (used for all MU and TT analyses): 8x2 worst case antenna array assumption

- Optional (can be selected based on vendor declaration): 4x2 antenna array assumption

The single-element array patterns for the 6x6 array configuration are based on Table 5.2.3.3-1 of TR 38.803 and summarized in Table 1.

Table 1: Single-Element Antenna Array Assumptions

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=90°,SLAV=25dB |
| Antenna element horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=90°,Am=25dB |
| Combining method for 3D antenna element pattern (dB) | A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am |
| Maximum directional gain of an antenna element GE,max | 5 dBi |
| M x N array | 6 x 6 |
| (dv, dh) | (0.5λ, 0.5λ) |

The resulting antenna patterns are highlighted in the 3D pattern plots of the 6x6 antenna array in Figure 1. The pattern has an HPBW of ~16.8o in both principal planes.

Figure 1: 6x6 Antenna Pattern.

# Spherical Coverage Measurement Grid for 6x6 Antenna Array Assumption

In the absence of beam steering assumptions provided in [1] specifically for the 6x6 array configuration, the same assumptions as for 12x12 were assumed. The cumulative antenna pattern, i.e., the maximum achievable EIRP in 3D, is shown in Figure 2

Figure 2: Illustration of the cumulative 6x6 Antenna Pattern including beam steering

The simulation assumptions including the antenna patterns for the spherical coverage measurement grids are the same as those presented in [4] except the 6x6 antenna array assumption [1] instead of 12x12 assumptions.

The reference CDF curve, which utilized scaling the PDFs by sin(), was determined with a very fine constant step size measurement grid using a 1o step size in  and  is highlighted in Figure 3.

Figure 3: Sample CDF Analyses for a very fine 1o constant step size measurement grid with the sin() scaling of the PDF.

At the 85%-tile CDF, i.e., the target CDF for Power Class 5, statistical analyses of all 10000 EIRPs, EIRP85%CDF, is performed. For the example of the 15o constant step size grid, the histogram is shown in Figure 4.

Figure 4: Sample Histogram of the 10000 min EIRPs at the 85%-tile CDF for a 15º constant step size measurement grid

The simulations in this contribution were only for the case where the beam peak is oriented in completely random orientations, i.e., the beam peak is not always aligned to a grid point. It is understood that the CDF curve cannot be used to accurately determine the TX beam peak (100%-tile CDF)

The results for various constant-step size measurement grids are tabulated in Table 2.

Table 2: Statistical results of EIRP85%CDF for the 6x6 antenna array for constant step size measurement grids and the beam peak oriented in completely random orientations.

| Step Size [o] | Number of unique grid points | Std. Dev [dB] | ¦Mean Error¦ [dB] |
| --- | --- | --- | --- |
| 10 | 614 | 0.10 | 0.05 |
| 12 | 422 | 0.10 | 0.03 |
| 15 | 266 | 0.12 | 0.07 |
| 20 | 146 | 0.24 | 0.06 |

Similar results for the constant-density measurement grids are tabulated in Table 3.

Table 3: Statistical results of EIRP85%CDF for the 6x6 antenna array for constant density measurement grids and the beam peak oriented in completely random orientations.

| Number of unique grid points | Std. Dev [dB] | ¦Mean Error¦ [dB] |
| --- | --- | --- |
| 200 | 0.13 | 0.04 |
| 210 | 0.12 | 0.04 |
| 220 | 0.11 | 0.04 |
| 230 | 0.09 | 0.05 |
| 240 | 0.10 | 0.05 |
| 250 | 0.10 | 0.04 |
| 400 | 0.08 | 0.03 |

In order to match the MUs of the spherical coverage grids for the 6x6 array assumption with those of the 12x12 default array assumption, it is recommended to use the following recommendation in terms of min. number of grid points, standard deviation, and mean error for spherical coverage grids:

- constant density grid (using the charged particle implementation) with at least 200 grid points: standard deviation (MU element ‘Influence of spherical coverage grid’) of 0.13dB

- constant step size grid with at least 266 grid points: standard deviation (MU element ‘Influence of spherical coverage grid’) of 0.12dB

- the MU element ‘Systematic error related to EIS spherical coverage’ is the DL step size, i.e., 0.2dB.

When compared to the default 12x12 antenna assumptions, the spherical coverage measurement grids for the alternate 6x6 antenna assumption are the same with very similar MUs.

Proposal 2: Adopt the proposed measurement grids for spherical coverage measurement grids for PC5 UEs with the alternate 6x6 antenna array assumption which match the spherical coverage measurement grids and MU with the 12x12 array assumption.

# TRP Measurement Grid for 6x6 Antenna Array Assumption

The simulation assumptions including the antenna patterns for the TRP measurement grids are those agreed in [1].

The results tabulated in this section outline the results of statistical analyses with the positioning concept taken into account, i.e., the analyses were performed with and without the assumption that the beam peak direction is oriented away from the hemisphere towards the pole at = 180o. Additionally, the standard deviations are presented when ranges of pattern values are disregarded (zeroed out). For the constant-step size measurement grids, separate investigations were performed to determine how many latitudes (grid points with constant q) towards the back pole/positioner can be skipped/disregarded. As the positioner/mast is commonly blocking the measurement direction towards q=180°, at least this one grid point must be skipped for TRP. The results with the re-positioning concept applied are summarized in Table 4 for the sin(theta) and the Clenshaw-Curtis quadratures while the results without the re-positioning concept applied are summarized in Table 5.

For the constant density measurement grids, a similar investigation was performed using the Charged Particle implementation. Here, the effect of disregarding grids points within specific angular regions near the back pole/positioner was also studied. The results with the re-positioning concept applied are summarized in Table 6 for the Charged Particle implementation while the results without the re-positioning concept applied are summarized in Table 7.

Similar to PC1 and PC3 TRP grids, it is assumed that the standard deviation shall not exceed 0.25dB. Thus, the following number of points shall be included in the alternate TRP measurement grids for PC5 UEs (based on the 6x6 antenna array assumption).

If the re-positioning concept is not applied to TRP test cases:

-	150 measurement grid points for constant density grid – Charged Particle implementation, with standard deviation of 0.13 dB

-	13 latitudes and 24 longitudes (266 unique grid points) for constant step size grid – sin (theta) weights integration approach, with standard deviation of 0.20dB with the allowance to skip and interpolate measurements at the pole at =180o, see Annex M.4.4 [4]

-	13 latitudes and 24 longitudes (266 unique grid points) for constant step size grid – Clenshaw Curtis weights integration approach, with standard deviation of 0.15dB with the allowance to skip and interpolate measurements at the pole at =180o, see Annex M.4.4 [4]

If the re-positioning concept is applied to TRP test cases:

-	150 measurement grid points for constant density grid – Charged Particle implementation, with standard deviation of 0.13 dB with the allowance to skip and interpolate measurements beyond 150o in , see Annex M.4.4 [4]

-	13 latitudes and 24 longitudes (266 unique grid points) for constant step size grid – sin (theta) weights integration approach, with standard deviation of 0.19dB with the allowance to skip and interpolate measurements beyond 150o in , see Annex M.4.4 [4]

-	13 latitudes and 24 longitudes (266 unique grid points) for constant step size grid – Clenshaw-Curtis weights integration approach, with standard deviation of 0.04dB with the allowance to skip and interpolate measurements beyond 150o in , see Annex M.4.4 [4]

Table 4: Statistics of quadrature approaches for constant step size measurement grids for the 6x6 alternate antenna array with the re-positioning concept applied.

| Number of |  | Step Size Dq=Df | Allowance to skip and interpolate measurements for q≥ | ¦Mean Error¦ [dB] | Std. Dev [dB] | Quadrature | Re-Positioning Concept Applied |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Latitudes | Longitudes |  |  |  |  |  |  |
| 25 | 48 | 7.5 | 180 | 0.01 | 0.03 | Sin(theta) | Yes |
| 25 | 48 | 7.5 | 180 | 0.00 | 0.00 | Clenshaw-Curtis | Yes |
| 25 | 48 | 7.5 | 172.5 | 0.01 | 0.04 | Sin(theta) | Yes |
| 25 | 48 | 7.5 | 172.5 | 0.00 | 0.00 | Clenshaw-Curtis | Yes |
| 25 | 48 | 7.5 | 165 | 0.01 | 0.04 | Sin(theta) | Yes |
| 25 | 48 | 7.5 | 165 | 0.00 | 0.00 | Clenshaw-Curtis | Yes |
| 25 | 48 | 7.5 | 157.5 | 0.01 | 0.04 | Sin(theta) | Yes |
| 25 | 48 | 7.5 | 157.5 | 0.00 | 0.00 | Clenshaw-Curtis | Yes |
| 25 | 48 | 7.5 | 150 | 0.01 | 0.04 | Sin(theta) | Yes |
| 25 | 48 | 7.5 | 150 | 0.00 | 0.00 | Clenshaw-Curtis | Yes |
| 21 | 40 | 9 | 180 | 0.01 | 0.05 | Sin(theta) | Yes |
| 21 | 40 | 9 | 180 | 0.00 | 0.00 | Clenshaw-Curtis | Yes |
| 21 | 40 | 9 | 171 | 0.01 | 0.05 | Sin(theta) | Yes |
| 21 | 40 | 9 | 171 | 0.00 | 0.00 | Clenshaw-Curtis | Yes |
| 21 | 40 | 9 | 162 | 0.01 | 0.05 | Sin(theta) | Yes |
| 21 | 40 | 9 | 162 | 0.00 | 0.00 | Clenshaw-Curtis | Yes |
| 21 | 40 | 9 | 153 | 0.01 | 0.05 | Sin(theta) | Yes |
| 21 | 40 | 9 | 153 | 0.00 | 0.00 | Clenshaw-Curtis | Yes |
| 19 | 36 | 10 | 180 | 0.01 | 0.06 | Sin(theta) | Yes |
| 19 | 36 | 10 | 180 | 0.00 | 0.00 | Clenshaw-Curtis | Yes |
| 19 | 36 | 10 | 170 | 0.01 | 0.07 | Sin(theta) | Yes |
| 19 | 36 | 10 | 170 | 0.00 | 0.00 | Clenshaw-Curtis | Yes |
| 19 | 36 | 10 | 160 | 0.01 | 0.07 | Sin(theta) | Yes |
| 19 | 36 | 10 | 160 | 0.00 | 0.00 | Clenshaw-Curtis | Yes |
| 16 | 30 | 12 | 180 | 0.02 | 0.10 | Sin(theta) | Yes |
| 16 | 30 | 12 | 180 | 0.00 | 0.00 | Clenshaw-Curtis | Yes |
| 16 | 30 | 12 | 168 | 0.02 | 0.10 | Sin(theta) | Yes |
| 16 | 30 | 12 | 168 | 0.00 | 0.00 | Clenshaw-Curtis | Yes |
| 13 | 24 | 15 | 180 | 0.03 | 0.19 | Sin(theta) | Yes |
| 13 | 24 | 15 | 180 | 0.00 | 0.04 | Clenshaw-Curtis | Yes |
| 13 | 24 | 15 | 165 | 0.03 | 0.20 | Sin(theta) | Yes |
| 13 | 24 | 15 | 165 | 0.00 | 0.04 | Clenshaw-Curtis | Yes |
| 13 | 24 | 15 | 150 | 0.03 | 0.19 | Sin(theta) | Yes |
| 13 | 24 | 15 | 150 | 0.00 | 0.04 | Clenshaw-Curtis | Yes |

Table 5: Statistics of quadrature approaches for constant step size measurement grids for the 6x6 alternate antenna array without the re-positioning concept applied.

| Number of |  | Step Size Dq=Df | Allowance to skip and interpolate measurements for q≥ | ¦Mean Error¦ [dB] | Std. Dev [dB] | Quadrature | Re-Positioning Concept Applied |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Latitudes | Longitudes |  |  |  |  |  |  |
| 25 | 48 | 7.5 | 180 | 0.01 | 0.03 | Sin(theta) | No |
| 25 | 48 | 7.5 | 180 | 0.00 | 0.03 | Clenshaw-Curtis | No |
| 25 | 48 | 7.5 | 172.5 | 0.05 | 0.32 | Sin(theta) | No |
| 25 | 48 | 7.5 | 172.5 | 0.05 | 0.32 | Clenshaw-Curtis | No |
| 25 | 48 | 7.5 | 165 | 0.09 | 0.37 | Sin(theta) | No |
| 25 | 48 | 7.5 | 165 | 0.09 | 0.38 | Clenshaw-Curtis | No |
| 25 | 48 | 7.5 | 157.5 | 0.13 | 0.42 | Sin(theta) | No |
| 25 | 48 | 7.5 | 157.5 | 0.13 | 0.42 | Clenshaw-Curtis | No |
| 25 | 48 | 7.5 | 150 | 0.17 | 0.47 | Sin(theta) | No |
| 25 | 48 | 7.5 | 150 | 0.17 | 0.47 | Clenshaw-Curtis | No |
| 21 | 40 | 9 | 180 | 0.01 | 0.05 | Sin(theta) | No |
| 21 | 40 | 9 | 180 | 0.01 | 0.04 | Clenshaw-Curtis | No |
| 21 | 40 | 9 | 171 | 0.08 | 0.48 | Sin(theta) | No |
| 21 | 40 | 9 | 171 | 0.08 | 0.48 | Clenshaw-Curtis | No |
| 21 | 40 | 9 | 162 | 0.14 | 0.51 | Sin(theta) | No |
| 21 | 40 | 9 | 162 | 0.14 | 0.51 | Clenshaw-Curtis | No |
| 21 | 40 | 9 | 153 | 0.20 | 0.57 | Sin(theta) | No |
| 21 | 40 | 9 | 153 | 0.19 | 0.57 | Clenshaw-Curtis | No |
| 19 | 36 | 10 | 180 | 0.01 | 0.06 | Sin(theta) | No |
| 19 | 36 | 10 | 180 | 0.01 | 0.05 | Clenshaw-Curtis | No |
| 19 | 36 | 10 | 170 | 0.11 | 0.62 | Sin(theta) | No |
| 19 | 36 | 10 | 170 | 0.10 | 0.60 | Clenshaw-Curtis | No |
| 19 | 36 | 10 | 160 | 0.18 | 0.59 | Sin(theta) | No |
| 19 | 36 | 10 | 160 | 0.17 | 0.61 | Clenshaw-Curtis | No |
| 16 | 30 | 12 | 180 | 0.02 | 0.10 | Sin(theta) | No |
| 16 | 30 | 12 | 180 | 0.01 | 0.08 | Clenshaw-Curtis | No |
| 16 | 30 | 12 | 168 | 0.17 | 0.87 | Sin(theta) | No |
| 16 | 30 | 12 | 168 | 0.16 | 0.84 | Clenshaw-Curtis | No |
| 13 | 24 | 15 | 180 | 0.03 | 0.20 | Sin(theta) | No |
| 13 | 24 | 15 | 180 | 0.02 | 0.15 | Clenshaw-Curtis | No |
| 13 | 24 | 15 | 165 | 0.31 | 1.33 | Sin(theta) | No |
| 13 | 24 | 15 | 165 | 0.29 | 1.31 | Clenshaw-Curtis | No |

Table 6: Statistics for constant density measurement grid types for the 6x6 alternate antenna array with the re-positioning concept applied (charged particle implementation only)

| Number of Grid Points | Range of Angles disregarded | ¦Mean Error¦ [dB] | Std. Dev [dB] | Re-Positioning Concept Applied |
| --- | --- | --- | --- | --- |
| 300 | None | 0.01 | 0.03 | No |
| 275 | None | 0.01 | 0.03 | No |
| 250 | None | 0.01 | 0.04 | No |
| 225 | None | 0.01 | 0.05 | No |
| 200 | None | 0.01 | 0.06 | No |
| 175 | None | 0.01 | 0.07 | No |
| 150 | None | 0.01 | 0.13 | No |
| 125 | None | 0.02 | 0.27 | No |
| 100 | None | 0.05 | 0.63 | No |
| 75 | None | 0.24 | 1.43 | No |
| 300 | 165o-180o | 0.14 | 0.71 | No |
| 300 | 150o-180o | 0.50 | 1.79 | No |

Table 7: Statistics for constant density measurement grid types for the 6x6 alternate antenna array without the re-positioning concept applied (charged particle implementation only)

| Number of Grid Points | Range of Angles disregarded | ¦Mean Error¦ [dB] | Std. Dev [dB] | Re-Positioning Concept Applied |
| --- | --- | --- | --- | --- |
| 300 | None | 0.01 | 0.03 | Yes |
| 275 | None | 0.01 | 0.03 | Yes |
| 250 | None | 0.01 | 0.04 | Yes |
| 225 | None | 0.01 | 0.04 | Yes |
| 200 | None | 0.01 | 0.05 | Yes |
| 175 | None | 0.01 | 0.07 | Yes |
| 150 | None | 0.01 | 0.13 | Yes |
| 125 | None | 0.02 | 0.27 | Yes |
| 100 | None | 0.06 | 0.63 | Yes |
| 75 | None | 0.26 | 1.43 | Yes |
| 300 | 165o-180o | 0.01 | 0.03 | Yes |
| 275 | 165o-180o | 0.01 | 0.03 | Yes |
| 250 | 165o-180o | 0.01 | 0.04 | Yes |
| 225 | 165o-180o | 0.01 | 0.04 | Yes |
| 200 | 165o-180o | 0.01 | 0.05 | Yes |
| 175 | 165o-180o | 0.01 | 0.07 | Yes |
| 150 | 165o-180o | 0.01 | 0.13 | Yes |
| 125 | 165o-180o | 0.02 | 0.27 | Yes |
| 100 | 165o-180o | 0.06 | 0.63 | Yes |
| 75 | 165o-180o | 0.26 | 1.43 | Yes |
| 300 | 150o-180o | 0.01 | 0.03 | Yes |
| 275 | 150o-180o | 0.01 | 0.03 | Yes |
| 250 | 150o-180o | 0.01 | 0.04 | Yes |
| 225 | 150o-180o | 0.01 | 0.04 | Yes |
| 200 | 150o-180o | 0.01 | 0.05 | Yes |
| 175 | 150o-180o | 0.01 | 0.07 | Yes |
| 150 | 150o-180o | 0.01 | 0.13 | Yes |
| 125 | 150o-180o | 0.02 | 0.27 | Yes |
| 100 | 150o-180o | 0.06 | 0.63 | Yes |
| 75 | 150o-180o | 0.25 | 1.43 | Yes |

Proposal 3: Adopt the revised measurement grids for TRP for PC5 UEs with the alternate 6x6 antenna array assumption

# Beam Peak Search Measurement Grid for 6x6 Antenna Array Assumption

The simulation assumptions including the antenna patterns for the TRP measurement grids are the same as [4] except a 6x6 antenna array assumption instead of the 12x12 array assumption.

For the simulations, the relative orientation of the simulated antenna array and the measurement grid was altered randomly. The statistical results from simulations using 50,000 random orientations are then used to determine mean error, standard deviation and percentile analysis on CDF curve of all maximum EIRPs for each measurement grid. The EIRPs are normalized by the known 6x6 antenna peak antenna gain.

Sample histograms and CDF distributions for the beam peak error for constant step-size measurement grids are shown in Figure 5 and for the constant density measurement grid (based on the charged particle implementation) in Figure 6. The histograms show a half-normal distribution.

Given the half-normal distribution, the MU term should be based on the determination of the offset from the beam peak that contains 95% of the distribution (alternatively, the value at which the CDF is 5%).  This offset shall be considered a systematic error in the MU budget. The various statistical metrics are illustrated in Figure 7.

Figure 5: Histogram of maximum beam peak errors for sample constant-step size meausurement grids (left: 2.5o, right: 7.5o step size)

Figure 6: Histogram of maximum beam peak errors for sample constant density measurement grids (left: 8000, right: 750 grid points)

Figure 7: Statistical metrics for a sample half-normal distribution

The mean error and the standard deviation, and the offset at which the CDF is 5% are tabulated in Table 8 for the constant step size grids Table 9 for the constant density grids.

Table 8: Statistical Analyses of the 50k simulations for the constant step size grids

| Angular Step Size [o] | Number of unique grid points | Mean Error [dB] | STD [dB] | Offset5%CDF [dB] |
| --- | --- | --- | --- | --- |
| 2.5 | 10226 | 0.03 | 0.02 | 0.08 |
| 3.0 | 7082 | 0.05 | 0.03 | 0.11 |
| 3.6 | 4902 | 0.07 | 0.05 | 0.16 |
| 4.0 | 3962 | 0.09 | 0.06 | 0.20 |
| 4.5 | 3122 | 0.11 | 0.08 | 0.25 |
| 5.0 | 2522 | 0.14 | 0.10 | 0.32 |
| 6.0 | 1742 | 0.20 | 0.14 | 0.45 |
| 7.5 | 1106 | 0.31 | 0.22 | 0.71 |
| 9.0 | 762 | 0.45 | 0.31 | 1.03 |
| 10.0 | 614 | 0.56 | 0.39 | 1.27 |

Table 9: Statistical Analyses of the 50k simulations for the constant-density grids

| Number of unique grid points | Mean Error [dB] | STD [dB] | Offset5%CDF [dB] |
| --- | --- | --- | --- |
| 8000 | 0.03 | 0.02 | 0.07 |
| 7000 | 0.04 | 0.02 | 0.07 |
| 6000 | 0.04 | 0.03 | 0.09 |
| 6500 | 0.04 | 0.02 | 0.08 |
| 5000 | 0.05 | 0.03 | 0.10 |
| 4500 | 0.06 | 0.03 | 0.12 |
| 4000 | 0.07 | 0.04 | 0.13 |
| 3500 | 0.08 | 0.04 | 0.15 |
| 3000 | 0.09 | 0.05 | 0.17 |
| 2000 | 0.13 | 0.08 | 0.26 |
| 1500 | 0.18 | 0.10 | 0.35 |
| 1250 | 0.21 | 0.13 | 0.42 |
| 1100 | 0.24 | 0.14 | 0.48 |
| 1000 | 0.26 | 0.16 | 0.53 |
| 900 | 0.30 | 0.17 | 0.59 |
| 800 | 0.33 | 0.20 | 0.66 |
| 750 | 0.35 | 0.21 | 0.71 |
| 700 | 0.38 | 0.23 | 0.76 |
| 600 | 0.44 | 0.26 | 0.89 |

In Table 10, the minimum number of unique grid points are listed for each grid type investigated for sample systematic errors of ‘Beam Peak Search’ of 0.5 and 0.7dB. The option with the 0.7dB was selected for PC1 as it seemed to be best compromise in terms of MU and test points/test time. It should be noted that for PC3 devices, a systematic error of 0.5dB was selected.

Table 10: Minimum number of unique grid points for sample systematic errors

| Systematic Error of ‘Beam Peak Search’: Offset from Beam Peak at which CDF is 5% | Minimum Number of Unique Grid Points for Constant Step Size Grid | Minimum Number of Unique Grid Points for Constant Density Grid |
| --- | --- | --- |
| 0.5dB | 1742 (6ostep size) | 1100 |
| 0.7dB | 1106 (7.5o step size) | 750 |

Taking into account simulation results above and in order to make a reasonable trade-off with measurement uncertainties, it is recommended to use for beam peak search the following measurement grids leading to a systematic error of “Beam Peak Search” of 0.7 dB:

-	Constant density grid (using the charged particle implementation) with at least 750 grid points.

-	Constant step size grid with at least 1106 grid points, corresponding to an angular step size of 7.5º.

Proposal 4: Adopt the measurement grids highlighted in Table 10 for Beam Peak Search for PC5 UEs based on the alternate 6x6 antenna assumption with a systematic error of ‘Beam Peak Search’ of 0.7dB.

# Conclusion

Proposal 1: Leverage the measurement grids defined for PC1 for PC5 as default measurement grids

Proposal 2: Adopt the proposed measurement grids for spherical coverage measurement grids for PC5 UEs with the alternate 6x6 antenna array assumption which match the spherical coverage measurement grids and MU with the 12x12 array assumption.

Proposal 3: Adopt the revised measurement grids for TRP for PC5 UEs with the alternate 6x6 antenna array assumption

Proposal 4: Adopt the measurement grids highlighted in Table 10 for Beam Peak Search for PC5 UEs based on the alternate 6x6 antenna assumption with a systematic error of ‘Beam Peak Search’ of 0.7dB.

# References

- R5-227982, PC5 device assumptions, Qualcomm, 3GPP TSG-RAN5 Meeting #97, November 2022

- R5-206618, PC1 Assumptions for CDF Curve for MOP EIRP Spherical Coverage, Qualcomm, 3GPP TSG-RAN5 Meeting #89-e, November 2020

- R5-198203, MU/MTSU Assumptions for PC1/Fixed Wireless Access (FWA) UE, Qualcomm, 3GPP TSG-RAN WG5 #85, November 2019

- TS 38.521-2, User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone, V16.20 (2019-12)

- R5-211189, PC1 MUs based on the revised antenna array assumptions, Keysight Technologies, 3GPP TSG-RAN5 Meeting #90-e, February 2021

- R5-198265, Proposals for Beam Peak and TRP Measurement Grids for PC1 UEs, Keysight Technologies, 3GPP TSG-RAN WG5 #85, November 2019

- R5-199530, Proposals for Spherical Coverage Measurement Grids for PC1 UEs, Keysight Technologies, 3GPP TSG-RAN WG5 #85, November 2019

- R5-202845, On PC1 MUs, Keysight Technologies, 3GPP TSG-RAN5 Meeting #87-e, May 2020

- TR38.810, Study on test methods, V16.5.0 (2019-12)

- TR 38.903, Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases, V16.2.0 (2019-12)

- R5-191628, MU Elements: documentation of justification and interdependence, Keysight Technologies, 3GPP TSG-RAN5 Meeting #82, February 2019

- R5-190389, On Positioning Misalignment MU, Keysight Technologies, 3GPP TSG-RAN WG5 5G NR AH #4, January 2019

- R5-190387, On MU Element for DUT Repositioning, Keysight Technologies, 3GPP TSG-RAN WG5 5G NR AH #4, January 2019

- TS 38.101-2, User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone, V16.2.0 (2019-12)

- R4-1713677, Test Equipment Measurement Uncertainties for NR UE RF Baseline System, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #85, November 2017

- R5-190804, MU contribution of “Uncertainty of the RF power measurement equipment” (2), Anritsu, 3GPP TSG-RAN5 #4-5G-NR Adhoc, January 2019

- R5-191616, On Calibration VNA uncertainty, Keysight Technologies, 3GPP TSG-RAN5 #82, February 2019