Title: 	Discussion on test point selection for Minimum Output Power in FR1 for NR UEs supporting satellite access operation

Source: 	Keysight Technologies

# Introduction

As agreed in RAN5#78, each test case would require specific analysis to conform its test configuration table based on the study for Frequency, Channel BW, SCS, RB allocation (including modulations).

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for Minimum Output Power test case for NR UEs supporting satellite access operation defined in 38.521-5 [5].

# Discussion

Core requirements specification for NR UEs supporting satellite access operation is made in 38.101-5 [4].

The operating bands defined there for satellite access operation are within FR1 range and hence the following general statement is made:

## 4.1 Relationship between minimum requirements and test requirements The present document is a Single-RAT specification for satellite NR UE, covering RF characteristics and minimum performance requirements. Conformance to the present specification is demonstrated by fulfilling the test requirements specified in the conformance specification 3GPP TS 38.521-1 [2]. The Minimum Requirements given in this specification make no allowance for measurement uncertainty. The test specification 3GPP TS 38.521-1 [2] defines test tolerances. These test tolerances are individually calculated for each test. The test tolerances are used to relax the minimum requirements in this specification to create test requirements. For some requirements, including regulatory requirements, the test tolerance is set to zero. The measurement results returned by the test system are compared - without any modification - against the test requirements as defined by the shared risk principle. The shared risk principle is defined in Recommendation ITU-R M.1545 [3].

It means that as a general rule, the test point analysis for the equivalent test cases in 38.521-1 [3] could and should be reused for test cases defined in 38.521-5 [5].

For the particular case of minimum output power test, none of the peculiarities applicable to a NR UEs supporting satellite access operation versus a NR FR1 UE, suggests the need of a different test point selection or any exception in terms of test environment, SCS, frequencies, CBW, modulation or UL configuration to test.

Proposal 1: For NR UEs supporting satellite access operation, for minimum output power test case 6.3.1, to reuse the test points already defined for NR FR1 UEs in clause 6.3.1 in 38.521-1 [3] regarding test environment, SCS, frequencies, CBW, modulation and UL configuration.

# Conclusions

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on Minimum Output Power test case definition for NR UEs supporting satellite access operation:

Proposal 1: For NR UEs supporting satellite access operation, for minimum output power test case 6.3.1, to reuse the test points already defined for NR FR1 UEs in clause 6.3.1 in 38.521-1 [3] regarding test environment, SCS, frequencies, CBW, modulation and UL configuration.

# References

[1] R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[2] 3GPP TS 38.101-1 v17.7.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 17)

[3] 3GPP TS 38.521-1 v17.7.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone (Release 17)

[4] 3GPP TS 38.101-5 v17.1.0 User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements (Release 17)

[5] 3GPP TS 38.521-5 v0.0.1 User Equipment (UE) conformance specification; Radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements (Release 17)